3GPP TSG-RAN WG1 #108-e			R1-2202434

e-Meeting, February 21 – March 3, 2022

Title:	DRAFT LS reply about the MBS issues

Response to:	R1-2200882/R2-2201830

Release:	Rel-17

Work Item:	NR_MBS-Core

Source:	Huawei (RAN1)

To:	RAN2

Cc:

Contact Person:

#### Name:	Jinhuan XIA

Tel. Number:

E-mail Address:	jinhuan.xia@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

- Overall Description:

RAN1 thanks RAN2 for the LS regarding the question whether only a single CFR configured for MCCH/MTCH is RAN1’s understanding:

Question: Currently, RAN2 running RRC design assumes that only a single CFR (indicated by locationAndBandwidth-Broadcast) is configured for MCCH/MTCH reception of MBS broadcast and it is common for MCCH and all MTCHs. RAN2 would like to confirm this understanding with RAN1.

The CFR in RAN2’s question is about the common frequency resource indicated by locationAndBandwidth-Broadcast. The following are the relevant agreements reached in RAN1:

Agreement:

For broadcast reception, RRC_IDLE/RRC_INACTIVE UEs can use the same bandwidth configurations for the CFR of GC-PDCCH/PDSCH carrying MCCH and the CFR of GC-PDCCH/PDSCH carrying MTCH.

- FFS: use of different bandwidth configurations for the CFR of GC-PDCCH/PDSCH carrying MCCH and the CFR of GC-PDCCH/PDSCH carrying MTCH

Agreement

For broadcast reception with RRC_IDLE/RRC_INACTIVE Ues:

- The CFR frequency resources used for MCCH and MTCH are configured by SIBx;

- PDCCH-config/PDSCH-config for broadcast reception with GC-PDCCH/PDSCH carrying MCCH is configured by SIBx

- PDCCH-config/PDSCH-config for broadcast reception with GC-PDCCH/PDSCH carrying MTCH is configured by MCCH. If the PDCCH-config/PDSCH-config for MTCH is not configured, the PDCCH-config/PDSCH-config for GC-PDCCH/PDSCH carrying MCCH configured by SIBx is reused for GC-PDCCH/PDSCH carrying MTCH.

It has been controversial in RAN1 whether to support different CFRs in terms of locationAndBandwidth configured for MCCH and MTCH at a given time from UE perspective. There is no consensus to support it in RAN1.

Therefore, based on the agreements achieved so far, RAN1 confirmed that only a single CFR (indicated by locationAndBandwidth-Broadcast) is configured for MCCH/MTCH reception of MBS broadcast and it is common for MCCH and all MTCHs.

2. Actions:

To RAN2

ACTION: 	RAN1 respectfully requests RAN2 to take RAN1’s response into account.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #109-e 	16 – 27 May 2022	                       E-Meeting

TSG-RAN WG1 Meeting #110-e	22 - 26 August 2022	               Toulouse, France