3GPP TSG RAN WG1 Meeting #108-e	R1-2202462

e-Meeting, February 21 – March 3, 2022

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion on UL BWP with PRACH resources only for RACH with Msg3 repetition

Document for:	Discussion and Decision

Introduction

In the last meeting, an LS [1] from RAN2 is received. In the LS, RAN1 is asked to discuss the following working assumption and then inform RAN2 whether such a configuration for RACH with Msg3 repetition is feasible or not.

1. Overall Description: RAN2 discussed the issue whether network can configure PRACH resources only for RACH with Msg3 repetition in a dedicated UL BWP (i.e. for use in RRC_CONNECTED) and made the following working assumption: - “Working assumption: From RAN2’s perspective, a dedicated UL BWP can be configured with only CE RACH resources. Its feasibility is to be confirmed by RAN1.” However, RAN2 did not reach consensus on whether such a configuration is feasible from RAN1’s perspective. 2. Actions: To RAN1 ACTION: RAN2 respectfully asks RAN1 to discuss the above working assumption and then inform RAN2 whether such a configuration for RACH with Msg3 repetition is feasible or not.

In this contribution, our view on the working assumption is provided.

Discussion

Because regardless of whether a UE requests for Msg3 repetition or not, the PRACH resource selected by the UE is capable to provide full functionality for PRACH procedure. In our understanding, the situation is similar to the configuration of PRACH resources for 2-step RACH where a dedicated UL BWP can be configured with only 2-step RACH resources in Rel-16. Therefore, we have the following proposal,

Proposal: confirm that the working assumption in R1-2200885 is feasible from RAN1 perspective. Adopt the draft reply LS in Appendix.

Conclusion

According to the above discussions, we have the following proposal:

Proposal: confirm that the working assumption in R1-2200885 is feasible from RAN1 perspective. Adopt the draft reply LS in Appendix.

- References

R1-2200885, “LS on UL BWP with PRACH resources only for RACH with Msg3 repetition”.

- Appendix

Title:	Draft reply LS on UL BWP with PRACH resources only for RACH with Msg3 repetition

Response to:	R1-2200885/R2-2201982

Release:	Rel-17

Work Item:	NR_cov_enh-Core

Source:	Huawei [RAN WG1]

To:	RAN WG2

Cc:

Contact Person:

Name:	Frank Long

E-mail Address:	frank.longyi{at }huawei.com

Attachments: 	-

1. Overall Description:

RAN1 thanks RAN2 for the LS on configuration of PRACH resources for RACH with Msg3 repetition.

Regarding the working assumption in the LS, from RAN1’s perspective, it is feasible for gNB to configure only CE RACH resources in a dedicated UL BWP, just as a dedicated UL BWP can be configured with only 2-step RACH resources in Rel-16. In such configuration, the procedure for a UE to select RACH resource should be discussed by RAN2.

2. Actions:

To: RAN2

ACTION: RAN1 respectfully requests RAN2 to take the above responses into account in their further work.

3. Date of Next Meetings:

3GPP RAN1#109-e				16 - 27, May 2022	                Online

3GPP RAN1#110				22 - 26, August 2022	        Toulouse, FR