3GPP TSG-RAN WG1 Meeting #108-e											R1-2202505

e-Meeting, February 21st - March 3rd, 2022

Agenda Item:	7.2.1

Source:			Moderator (ZTE)

Title:			Summary of preparation phase email discussion for Rel-16 2-step RACH

Document for:	Discussion

Introduction

This document contains the summary of issues related to the maintenance of Rel-16 2-step RACH WI in RAN1#108-e meeting. The following email discussion is initiated to collect companies’ views:

[108-e-Prep-AI7.2.1] Preparation phase for Rel-16 NR 2-step RACH maintenance

Preparation phase discussion

The following 2 CRs are submitted to the maintenance of Rel-16 2-step RACH in RAN1#108-e.

| Issue # | Description | Affected spec | TDoc # |
| --- | --- | --- | --- |
| 1 | In TS38.213 section 6, it is mentioned that the PDCCH reception that determines the completion of CBRA procedure is described in clause 5.1.5[11, TS38.321], but the quoted clause is for 4 step RACH only, it’s suggested to add clause 5.1.4a for 2 step RACH as well. Note: these 2 contributions are for Rel-16 and Rel-17 specs respectively. | TS38.213 | R1-2201065 vivo R1-2201066 vivo |

To share the views on the necessity of the above issues, please fill in ‘Yes/No/Editorial’ to the table below. If it’s considered as Editorial, please also indicate whether you agree with the editorial change.

| Company | Issue #1 | Comments |
| --- | --- | --- |
| Qualcomm | Editorial | OK with the editorial changes for TS 38.213 suggested in R1-2201065/6. |
| vivo | Editorial |  |
| Intel | Editorial | Can be considered as alignment CR. |
| Samsung | Editorial |  |
| New H3C | Editorial |  |
| Nokia, Nokia Shanghai Bell | Editorial | Can be considered as alignment CR. In general we are OK with the proposed addition/clarification. |
| Apple | Editorial |  |
| ZTE | Editorial | OK with the editorial change. |

Summary

All 8 companies agree that the issue introduced in R1-2201065/R1-2201066 is editorial and it seems all companies are OK with the editorial change. Given that the editorial change is quite straightforward, Moderator suggests the following:

Proposal:

The CRs proposed in R1-2201065/R1-2201066 are agreeable to be incorporated into alignment CR, no email discussion is needed.

Per Mr.Chairman’s guidance, “Since this week is only preparation phase, let’s have a small email discussion next week for the draft CRs.”

[108-e-R16-2Step-RACH-01] Email discussion on draft CSs R1-2201065/R1-2201066 for recommendation to the editors for alignment CRs by February 22 - Zhipeng (vivo)

Any other comments?

| Company | Comment |
| --- | --- |

References

- R1-2201065 Draft CR on BFR with 2-step RACH (Rel-16)                 vivo

- R1-2201066 Draft CR on BFR with 2-step RACH (Rel-17)                 vivo

Appendix

List of proposals in the submitted contributions.

| TDoc | Proposals |
| --- | --- |
| R1-2201065, vivo | Reason for change: The PDCCH reception that determines the completion of the contention based random access procedure for 4-step RACH and 2-step RACH is described in section 5.1.5 and 5.1.4a of 38.321 respectively. However, this PDCCH reception is said to be also described in section 5.1.5 of 38.321 for 2-step RACH, which is wrong. Summary of change: Clarify that MsgB reception for completion of the CBRA is described in 5.1.4a of 38.321 instead of section 5.1.5 where Msg4 is described. Consequences if not approved: The PDCCH reception that determines the completion of the contention based random access procedure for 2-step RACH is assumed to be described in section 5.1.5 of 38.321, which is wrong. ========CR to Rel-16 TS38.213======= 6 Link recovery procedures *** Unchanged text omitted *** For the PCell or the PSCell, if BFR MAC CE [11, TS 38.321] is provided in Msg3 or MsgA of contention based random access procedure, and if a PUCCH resource is provided with PUCCH-SpatialRelationInfo, after 28 symbols from the last symbol of the PDCCH reception that determines the completion of the contention based random access procedure as described in clause 5.1.5 or clause 5.1.4a of [11, TS38.321], the UE transmits the PUCCH on a same cell as the PRACH transmission using - a same spatial filter as for the last PRACH transmission - a power determined as described in clause 7.2.1 with qu=0, qd=qnew, and l=0, where qnew is the SS/PBCH block index selected for the last PRACH transmission. *** Unchanged text omitted *** |
| R1-2201066, vivo | Reason for change: The PDCCH reception that determines the completion of the contention based random access procedure for 4-step RACH and 2-step RACH is described in section 5.1.5 and 5.1.4a of 38.321 respectively. However, this PDCCH reception is said to be also described in section 5.1.5 of 38.321 for 2-step RACH, which is wrong. Summary of change: Clarify that MsgB reception for completion of the CBRA is described in 5.1.4a of 38.321 instead of section 5.1.5 where Msg4 is described. Consequences if not approved: The PDCCH reception that determines the completion of the contention based random access procedure for 2-step RACH is assumed to be described in section 5.1.5 of 38.321, which is wrong. ========CR to Rel-17 TS38.213======= 6 Link recovery procedures *** Unchanged text omitted *** For the PCell or the PSCell, if BFR MAC CE [11, TS 38.321] is provided in Msg3 or MsgA of contention based random access procedure, and if a PUCCH resource is provided with PUCCH-SpatialRelationInfo, after 28 symbols from the last symbol of the PDCCH reception that determines the completion of the contention based random access procedure as described in clause 5.1.5 or clause 5.1.4a of [11, TS38.321], the UE transmits the PUCCH on a same cell as the PRACH transmission using - a same spatial filter as for the last PRACH transmission - a power determined as described in clause 7.2.1 with qu=0, qd=qnew, and l=0, where qnew is the SS/PBCH block index selected for the last PRACH transmission. *** Unchanged text omitted *** |