3GPP TSG RAN WG1 #108-e	R1-2202614

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	7.2.4

Source: 	Moderator (OPPO)

Title: 	Summary for email discussion [108-e-R16-V2X-05]

Document for:	Discussion and Decision

Introduction

This document provides a summary of [108-e-R16-V2X-05] Type 1 codebook generation in PUCCH; considering R1-2201253.

Discussion

The draft CR [1] proposes a change in Type-1 HARQ-ACK codebook in physical uplink control channel (TS 38.213 section 16. 5. 1.1): change 2μUL-μSL to 2μSL-μUL, to use the same scale factor during type 1 codebook generation procedure. During preparation phase, two companies expressed that current spec is correct and this CR is not necessary. The comments are as follows:

- Company 1: this part is reusing the Uu specification. Both (μUL-μDL) and (μDL-μSL) are used in in section 9.1.2.1 to handle different cases.

- Company 2: the current spec is reusing the logic in Uu, and the point is to ensure one and only one valid PUCCH slot to be used for feedback within any the time duration of any SL slot for data, so if μUL > μSL, only the first UL slot within a SL slot is used. And in case a UL slot comes with larger granularity (μUL < μSL), enumerate all SL slots within the duration of a UL slot (hence "while nS<max2μSL-μUL,1”). In summary, use of both 2μUL-μSL and 2μSL-μUL in current spec is correct and there is no such “typo" as mentioned abvoe (and hence no change is necessary).

The general principle of codebook generation procedure in PUCCH for SL HARQ feedback reporting is to reuse NR Uu codebook generation procedure as much as possible. The key modification is changing DL related parameters (in section 9.1.2.1) to SL related parameters (in section 16. 5. 1.1). It is moderator’s view that the above comments are valid and this modification in [1] is not necessary.

Companies view:

Q1: Do you think the modification in R1-2201253 is necessary?

| Company | Yes/No | Views |
| --- | --- | --- |
| vivo | no | After checking the spec, we agree with Sharp and NEC that the current spec is correct, the text max2μUL-μSL,1 is not a typo, it is to find the UL slot corresponding to a given K1,k value expressed in UL SCS. if SL SCS(e.g., 30kHz)>UL SCS(e.g., 15kHz): max2μUL-μSL,1 is always 1 mod nU-K1,k+1,max2μUL-μSL,1is always 0 Then UE enters the ‘while’ loop for each UL slot nU-K1,k and iterate through the nS<max2μSL-μUL,1=2 SL slots overlapped with the UL slot nU-K1,k . An example is as below. If SL SCS(e.g., 15kHz)<UL SCS(e.g., 30kHz), it can be seen that a SL slot is overlapped with two UL slot corresponding to two different K1,k respectively, but only one of the UL slot would trigger the ‘while’ loop. An example is as below. max2μUL-μSL,1=max2,1=2 In this example, assuming PUCCH is at slot nU=4, only the UL slot nU-K1,k with K1,k=1would meet the condition mod nU-K1,k+1,max2,1=mod nU-K1,k+1, 2=0 and enter the ‘while’ loop, and there is only nS<max2μSL-μUL,1max0.5,1=1 SL slot to be checked Thus, the spec is correct. |
| Intel | No | After checking the spec we also agree with arguments from Sharp, NEC, and vivo |
| LG Electronics | No | We tend to agree with the analysis of current specification from vivo. |
| ZTE, Sanechips | No | The same view as above companies. |
| Qualcomm | No |  |
| Sharp | No |  |
| NTT DOCOMO | No |  |
| CATT, GOHIGH | No |  |
| Samsung | No |  |
| Ericsson | No | The change is not necessary. |
| Huawei, HiSilicon | No | Share similar views with moderator and other companies, which the modification in R1-2201253 is not necessary |
| NEC | NO | As stated in preparation phase, (μ_UL-μ_SL) is designedly captured. |
| Nokia, NSB | No | Current spec is correct |

Summary:

Thanks all companies for the input. According to comment, the current spec is correct and the draft CR is not necessary. Then the following conclusion is given.

Conclusion: The draft CR in R1-2201253 is not necessary.

References

- R1-2201253 “Corrections for Type-1 codebook generation in PUCCH,” OPPO, RAN1 #108-e.