3GPP TSG-RAN WG1 Meeting #108-e	R1-2202636

e-Meeting, February 21th – March 3rd, 2022

Agenda item:	8.1.2.1

Source: 	Moderator (Nokia)

Title: 	Summary for mTRP PUCCH/PUSCH Enhancements

Document for:	Discussion/Decision

# Offline agreements (Issue 6,9,10,11,13,15)

Offline agreement 1.6: Adopt the following TP in Clause 7.1.1 of TS 38.213,

Clause 7.1.1 of TS 38.213 --------------Text omitted---------------- - If the UE is provided enablePL-RS-UpdateForPUSCH-SRS, a mapping between sri-PUSCH-PowerControlId and PUSCH-PathlossReferenceRS-Id values can be updated by a MAC CE as described in [11, TS 38.321] - For a PUSCH transmission scheduled by a DCI format that does not include an SRI field, or for a PUSCH transmission configured by ConfiguredGrantConfig and activated, as described in clause 10.2, by a DCI format that does not include an SRI field, the UE determines a RS resource index qd from the PUSCH-PathlossReferenceRS-Id mapped to sri-PUSCH-PowerControlId = 0. If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’, the UE determines first and second RS resource indexes qd from respective PUSCH-PathlossReferenceRS-Id mapped to sri-PUSCH-PowerControlId = 0 of sri-PUSCH-MappingToAddModList and sri-PUSCH-PowerControlId = 0 of sri-PUSCH-MappingToAddModList2 respectively. --------------Text omitted----------------

Offline agreement 1.9: Adopt the following TP in Clause 6.1 of TS 38.214,

Clause 6.1 of TS 38.214 --------------Text omitted---------------- For PUSCH scheduled by DCI format 0_0 on a cell, the UE shall transmit PUSCH according to the spatial relation, if applicable, corresponding to the dedicated PUCCH resource with the lowest ID within the active UL BWP of the cell, as described in Clause 9.2.1 of [6, TS 38.213]. If the dedicated PUCCH resource with the lowest ID within the active UL BWP of the cell corresponds to two spatial relations, the UE shall transmit the PUSCH according to the spatial relation with the lower ID. --------------Text omitted----------------

Offline agreement 1.10: Adopt the following TP in Clause 6.1.2.3 of TS 38.214,

Clause 6.1.2.3 in TS 38.214 --------------Text omitted---------------- For PUSCH transmissions with a Type 2 configured grant, when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, the SRS resource set association to (nominal) repetitions follows MappingPattern in ConfiguredGrantConfig as defined in Clause 6.1.2.1 for PUSCH scheduled by DCI format 0_1 and 0_2. For PUSCH transmissions with a Type 1 configured grant, when two SRS resource sets with usage set to ‘codebook’ or ‘noncodebook’ are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, if two SRS resource indicators and two precoding information are provided, the SRS resource set association to (nominal) repetitions is determined as follows. When K = 2, the first and second SRS resource sets are applied to the first and second (nominal) repetitions, respectively. - When K > 2 and cyclicMapping in ConfiguredGrantConfig is enabled, the first and second SRS resource sets are applied to the first and second (nominal) repetitions, respectively, and the same SRS resource set mapping pattern continues to the remaining (nominal) repetitions. - When K > 2 and sequentialMapping in ConfiguredGrantConfig is enabled, first SRS resource set is applied to the first and second (nominal) repetitions, and the second SRS resource set is applied to the third and fourth (nominal) repetitions, and the same SRS resource set mapping pattern continues to the remaining (nominal) repetitions. --------------Text omitted----------------

Offline agreement 1.11: Adopt the following TP in Clause 7.2.1 of TS 38.213,

Clause 7.2.1 of TS 38.213 --------------Text omitted---------------- If the UE transmits the PUCCH with NPUCCHrepeat>1 repetitions, as described in clause 9.2.6, and the UE is provided twoPUCCH-PC-AdjustmentStates via higher layer parameter pucch-PowerControl - If the DCI format includes two TPC command values and the PUCCH resource of the PUCCH transmission is associated with l=0 and l=1, the UE applies the first TPC command value for l=0 and applies the second TPC command value for l=1 - If the DCI format includes two TPC command values and the PUCCH resource of the PUCCH transmission is associated with l=0, the UE applies the first TPC command value for l=0 and ignores the second TPC command value - If the DCI format includes two TPC command values and the PUCCH resource of the PUCCH transmission is associated with l=1, the UE applies the second TPC command value for l=1 and ignores the first TPC command value - If the DCI format includes one TPC command value, the UE applies the TPC command value for all l associated with the PUCCH resource of the PUCCH transmission --------------Text omitted----------------

Offline agreement 1.13: Adopt the following TP in Clause 6.1.2.3 of TS 38.214,

Clause 6.1.2.3 in TS 38.214 --------------Text omitted---------------- - Number of DM-RS CDM groups, DM-RS ports, SRS resource indication and DM-RS sequence initialization are determined as in Clause 7.3.1.1.2 of [5, TS 38.212], and the antenna port value, the bit value for DM-RS sequence initialization, precoding information and number of layers, SRS resource indicator are provided by antennaPort, dmrs-SeqInitialization, precodingAndNumberOfLayers, and srs-ResourceIndicator respectively; When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, precoding information and number of layers (applicable when higher layer parameter usage in SRS-ResourceSet set to ‘codebook’) associated with the first and second SRS resource set is provided by precodingAndNumberOfLayers and precodingAndNumberOfLayers2, respectively, and SRS resource indicators associated with the first and second SRS resource sets are provided by srs-ResourceIndicator and srs-ResourceIndicator2, respectively. When both srs-ResourceSetToAddModList and srs-ResourceSetToAddModListDCI-0-2 are configured with two SRS resource sets, the two SRS resource sets configured by srs-ResourceSetToAddModList is used to determine the SRS resource indications by srs-ResourceIndicator and srs-ResourceIndicator2. --------------Text omitted----------------

Offline agreement 1.15: Adopt the following TP in Clause 6.1.2.3 of TS 38.214,

Clause 6.1.2.3 in TS 38.214 --------------Text omitted---------------- For PUSCH transmissions with a Type 1 configured grant, when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, if configuredGrantConfig contains only one srs-ResourceIndicator and precodingAndNumberOfLayers (applicable when higher layer parameter usage in SRS-ResourceSet set to ‘codebook’), PUSCH repetitions are associated only with the first SRS resource set. If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’, and the UE is not provided p0-PUSCH-Alpha2 and powerControlLoopToUse2, for a retransmission of a configured grant Type 1 PUSCH, or for activation or retransmission of a configured grant Type 2 PUSCH, scheduled by a DCI format that includes an SRS resource set indicator field, the UE expects the value of the SRS resource set indicator field to be set to ‘00’, and PUSCH repetitions are associated only with the first SRS resource set. The UE shall not transmit anything on the resources configured by configuredGrantConfig if the higher layers did not deliver a transport block to transmit on the resources allocated for uplink transmission without grant. --------------Text omitted----------------

# Summary of Discussion

| # | Summary of the issue | Moderator’s Comments |
| --- | --- | --- |
| Issue 1 [1] | TP for 38.213 on SRI based power control mechanism for mTRP PUSCH repetition | This is editorial (missing in 38.213), we can discuss the TP directly in Section 1.1 |
| Issue 2 [2] | The below agreement (mainly pathlossReferenceIndex) is not captured in TS 38.213. [2] also suggest a text change for 38.214 (even though it captures 'srs-ResourceIndicator', 'precodingAndNumberOfLayers'). Agreement For a BWP configured with two SRS resource sets for CB or NCB based mTRP PUSCH repetition with Type 1 CG configuration, - If the CG is configured with only one field for each of ‘pathlossReferenceIndex’, 'srs-ResourceIndicator', 'precodingAndNumberOfLayers', 'p0-PUSCH-Alpha' and 'powerControlLoopToUse', PUSCH repetitions are associated with the first SRS resource set. | From FL view, only the first TP (change to 38.213 section 7.1.1) is needed, and 38.214 does not need to cover UE power control aspects. We can discuss the TP directly in Section 1.2 |
| Issue 3 [4] | For repetition number equal to 2, PUCCH repetition procedure (beam/power control set mapping) is not fully defined in 38.213. | This is editorial (missing in 38.213), we can discuss the TP directly in Section 1.3 |
| Issue 4 [4], [15] | Below agreement is not captured in 38.212 Agreement For mTRP PUSCH repetition scheduled with DCI format 0_2, the value of the NSRS,0_2 in two SRS resource sets configured by higher layer parameter srs-ResourceSetToAddModListDCI-0-2 should be the same. | This is editorial (missing in 38.212), we can discuss the TP directly in Section 1.4 |
| Issue 5 [5] | Proposal 4/5 in [5] highlight an issue in the existing text in 38.213 on resetting accumulation of a PUSCH power control adjustment state (considering mTRP operation). | Two proposals (4 and 5 in [5]) need further discussion. Provide your views in Section 1.5 |
| Issue 6 [7] | Implementing a change in 38.213, which is based on a RAN2 agreement to support two sri-PUSCH-MappingToAddModList. RAN2 #116b-e Add second sri-PUSCH-MappingToAddModList, and select two SRI-PUSCH-PowerControl from two sri-PUSCH-MappingToAddModList | This is editorial (need to update 38.213 based on RAN2 agreement), we can discuss the TP directly in Section 1.6 |
| Issue 7 [7] | Clarify the interpretation of PT-RS-DMRS association for sTRP/mTRP modes. | This is editorial (need to update 38.212), we can discuss the TP directly in Section 1.7 |
| Issue 8 [7], [11] | Extend the enhancements agreed in Rel-17 WI on coverage enhancement also to mTRP PUSCH repetition. | Interaction between mTRP PUSCH repetition and some enhancements (TBoMS, available slot counting) in Cov. Env should be clarified. Please check the proposals in Section 1.8 |
| Issue 9 [15] | TP for 38.214 on default beam for PUSCH. | This is a small editorial change, we can discuss the TP directly in Section 1.9 |
| Issue 10 [15] | TP for 38.214 on beam mapping for CG PUSCH | This is a small editorial change, we can discuss the TP directly in Section 1.10 |
| Issue 11 [17] | TP for 38.212 on second TPC field. TPC field interpretation for sTRP PUCCH transmission is not defined in the current version of TS 38.213. | This is editorial (need to update 38.213), we can discuss the TP directly in Section 1.11 |
| Issue 12 [18] | TP for 38.213 on twoPUCCH-PC-AdjustmentStates (proposal 4 in [18]) | This is a small editorial change, we can discuss the TP directly in Section 1.12 |
| Issue 13 [18] | Error in the spec and not consistent with the endorsed CR in R1-2112949 (proposal 5 in [18]) | This is clearly an error. We can change either highlighting this to the Editor or with a new TP. Section 1.13 |
| Issue 14 [18] | Interpretation of srs-ResourceIndicator in CG Type 1 for the case of only srs-ResourceSetToAddModListDCI-0-2 configured with two resource sets. | Cases highlighted in [18] , proposal 6, require some discussion. Provide your views in Section 1.14 |
| Issue 15 [18] | TP for 38.213 on CG PUSCH (proposal 7 in [18]). The following agreement is not implemented, Agreement When a DCI that includes the new 2-bits DCI field for dynamic switching activates a type 2 CG or schedules a retransmission of a type 1 or type 2 CG, and the CG configuration is RRC-configured with only one set of power control parameters (one ‘p0-PUSCH-Alpha’ and ‘powerControlLoopToUse’): - The UE expects the new DCI field for dynamic switching is set to “00”, and all PUSCH repetitions are associated with the first SRS resource set. | This is a small editorial change, we can discuss the TP directly in Section 1.15 |
| Issue 16 [19] | Issue is related to number of power control parameters. RAN2 asking the maximum number of power control parameter sets ([19] suggest 8) that shall be considered in MAC-CE design. RAN2 also agreed to a new IE for mTRP FR1 operation, [19] has a TP to implement FR1 mTRP PUCCH operation. RAN2#116e-bis agreements: add a new IE for power control for mTRP FR1 operation and consult on the number of power control sets to be configured. [060] Introduce the new MAC CE(s) to support PUCCH Power control set update (with power control) for FR1 cases. FFS, detail MAC CE design based on new RRC IE for FR1-dedicated power control set. | This needs some discussion as RAN1 shall reply to RAN2 LS on maximum number of power control parameter sets. Also, suggested TP in [19] (proposal 8) needs some discussion. Provide your views in Section 1.16 |
| Issue 17 [21] | Proposal 1 in [21] discuss whether M-TRP PUCCH operation is supported for all UCI types. It is mainly ask for clarification on whether mTRP PUCCH repetition is supported for other UCI types (e.g. SPS HARQ-ACK (without a corresponding PDCCH), SR, and/or CSI) | Some discussion is needed on this. Provide your views in Section 1.17 |
| Issue 18 [21] | Proposal 2 in [21] discussing S-TRP PUCCH operation for a PUCCH resource with two spatial or power control settings. | The case seems to be not defined in the spec and can consider as an error case. Provide your views in Section 1.18 |
| Other issues |  |  |
| Issue 19 [3] | TP for 38.213 on default power control setting – suggest deleting the condition that DCI includes an SRS resource set indicator field with value 10 or 11 (see Proposal 3 & 4 in [3]) | This TP is not needed to FL reading as the text change suggested in the TP cannot be generalized for sTRP operation. |
| Issue 20 [3] | Editorial errors in TP for 38.213 : Proposal 5 in [3]. | Other than italic font change, it is not clear the suggested editorial here. Font change should be able to handle by the Editor. |
| Issue 21 [3] | UE does not expect to be indicated with SRS resource set indication field codepoint 10/11 when repetition number is 1. | This seems an error case that can be handled by the gNB. |
| Issue 22 [4] | Suggest capturing a TP on the following conclusion. Conclusion For mTRP PUSCH repetition scheduled with DCI format 0_2, when only one SRS resource set is configured in srs-ResourceSetToAddModListDCI, the UE is not expected to configure with two SRS resource sets in srs-ResourceSetToAddModList-0-2. | This is a conclusion and does not require any text captured in the spec. |
| Issue 23 [4] | For mTRP PUSCH, when the repetition number is equal to 2, mapping is not fully defined - [4] suggest few changes to 38.214. | FL do not fully get the motivation as the agreement on this already captured in 38.214. No spec change considered necessary. |
| Issue 24 [6] | - Use of two srs-ResourceIndicator and/or precodingAndNumberOfLayers as the indicator of mTRP CG PUSCH transmission : few proposals (e.g. 4,5,6, 8) in [6] | These proposals are not inline with the agreements and how the spec is identifying mTRP CG PUSCH. |
| Issue 25 [6] | Type 1 and 3 PHR discussion | FL do not fully get the proposal 9 in [6]. No discussion unless proponents clarify. |
| Issue 26 [6] | - Changes to 38.212 in [6] (proposal 10) mentioning that Spec text is misleading. | FL thinks that there is no misleading in the wording used in the spec, and not consider as an essential correction. |
| Issue 27 [7] | - For Type 1 CG mTRP PUSCH, [7] highlight that the ‘p0-PUSCH-Alpha’ and ‘powerControlLoopToUse’ association to SRS resource sets is not defined (proposal 4). - Also, a TP to capture missing description in TS38.213 is suggested (spec does not support two fields of 'p0-PUSCH-Alpha' and 'powerControlLoopToUse' in 'ConfiguredGrantConfig’ for type 1 CG based multi-TRP PUSCH transmission.) | FL do not think that the proposal 4 is needed. The proposed TP needs further discussion, but it seems that 38.213 text is already covering the case of Type 1 CG PUSCH transmission. |
| Issue 28 [11], [19] | - Clarifications on the number of repetitions - [11] highlight that the legacy PUCCH repetition (Rel-15/16) applicable to only formats 1, 3 and 4 appears to have been changed – this should be revised such that legacy behaviour is applicable to only formats 1, 3 and 4 but Rel-17 repetition behaviour is applicable to all formats [19] suggest to have some clarifications on what is supported. | To FL reading, all PUCCH formats support PUCCH repetition, Agreement Support slot-based PUCCH repetition for PUCCH Format 0 and Format 2 also for single TRP operation. - The support is subject to independent UE capability indication Also, 38.213 capture PUCCH repetition is general form such that enhancements on number of repetitions are valid for mTRP, IIoT, and Cov. Enh. |
| Issue 29 [14] | - Proposals 1 & 2 in [14] suggest changes to the text applied for mTRP sub-slot repetition in 38.213 | FL views that 38.213 description correctly capture the mTRP intra-slot operation (agreed to follow Rel-17 IIoT agreements on sub-slot repetition). |
| Issue 30 [14] | - Proposal 3 in [14] has a TP to capture an associated CSI-RS resource configured to each one of two SRS resource sets for non-codebook based PUSCH transmission explicitly in TS 38.214. | Not an essential correction as there can not be any other interpretation (spec not having multiple associated CSI-RS resources per SRS resource set). |
| Issue 31 [17] | - Proposal 3 in [17] has a TP to capture explicit text on UL PT-RS port determination for CB based mTRP PUSCH transmission. | The suggested text change may be explicit, but current text in 38.214 seems also covering the same behaviour. FL view that the TP is not necessary. |
| Issue 32 [22] | - Proposal 3 in [21] asking for clarification whether the multi-TRP PUSCH/PUCCH repetition operation is not restricted when pathlossReferenceLinking is provided. | FL views that pathlossReferenceLinking is applied for mTRP PUSCH repetition based on the current spec. |

Note : Some issues that RAN1 already discussed before (with no outcome) are not listed as issues. If companies wish to indicate the importance of such proposals, please use the email discussion to comment.

## Issue 1: TP in [1] to address SRS resource set indicator

The following editorial TP (in [1]) is to make the description of SRI based power control mechanism for mTRP PUSCH repetition to be more accurate (capturing applicability of separate power control parameters only for mTRP PUSCH when the SRS resource set indicator set to “10” or “11”).

Section 7.1.1 of TS 38.213 < Unchanged parts are omitted > - if the DCI format scheduling the PUSCH transmission includes two SRI fields and the SRS resource set indicator is “10” or “11” and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the two SRI fields and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values, and determines first and second values of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId values that are mapped to the values of the first and second SRI fields, respectively. - if the DCI format scheduling the PUSCH transmission includes two SRI fields and the SRS resource set indicator is “10” or “11” and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between < Unchanged parts are omitted > - If the UE is provided by SRI-PUSCH-PowerControl more than one values of p0-PUSCH-AlphaSetId - if a DCI format scheduling the PUSCH transmission includes two SRI fields and the SRS resource set indicator is “10” or “11” and an open-loop power control parameter set indication field and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' - if a value of the open-loop power control parameter set indication field is '0', the UE determines two values of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId values in SRI-PUSCH-PowerControl that are mapped to the two SRI values corresponding to each SRS resource set with usage set to ‘codebook’. - if a value of the open-loop power control parameter set indication field is '1', the UE determines two values of PO_UE_PUSCH,b,f,cj from first values in P0-PUSCH-Set in P0-PUSCH-SetList and P0-PUSCH-Set in P0-PUSCH-SetList2 with p0-PUSCH-SetId values mapped to the two SRI values corresponding to each SRS resource set with usage set to ‘codebook’, respectively. - if a DCI format scheduling the PUSCH transmission includes two SRI fields and the SRS resource set indicator is “10” or “11” and an open-loop power control parameter set indication field and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', - if a value of the open-loop power control parameter set indication field is '0', the UE determines two values of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId values in SRI-PUSCH-PowerControl that are mapped to the first SRI field value corresponding to the first SRS resource set with usage set to ‘nonCodebook’ and to a second value, that is associated with the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value, corresponding to the second SRS resource set with usage set to ‘nonCodebook’. < Unchanged parts are omitted > - if a DCI format scheduling the PUSCH transmission includes two SRI fields and the SRS resource set indicator is “10” or “11” and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook', the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for - the two SRI fields and a set of indexes provided by P0-PUSCH-AlphaSetId that map to P0-PUSCH-AlphaSet values, and determines first and second values of αb,f,cj from the P0-PUSCH-AlphaSetID values that are mapped to the values of the first and second SRI field values, respectively. - if a DCI format scheduling the PUSCH transmission includes two SRI fields and the SRS resource set indicator is “10” or “11” and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for - the first SRI field and a set of indexes provided by P0-PUSCH-AlphaSetId that map to P0-PUSCH-AlphaSet values, and determines first value of αb,f,cj from the P0-PUSCH-AlphaSetID value that is mapped to the first SRI field value, and - the second value, associated with the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value, and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values, and determines the second value of αb,f,cj from the p0-PUSCH-AlphaSetId value that is mapped to the second SRI field value

Please provide your views,

| Company | Comments |
| --- | --- |
| Apple | We failed to see the necessity for the TP. It can be discussed during CR review stage. |
| Lenovo | Support. |
| NTT Docomo | Support the TP. |
| QC | Ok. |
| Samsung | We are fine with TP. |
| ZTE | We think this TP is not needed. Basically, we agree with FL’s understanding in principle that this part is used to specify the SRI based power control mechanism for mTRP PUSCH repetition. However, it worth noting that STRP/MTRP dynamic switching is supported in MTRP PUSCH operation, hence this TP will cause the case when switching to STRP operation to be lost. On the other hand, we think the current description is sufficient to clearly capture the association between SRS resource sets and power control parameter sets for MTRP operation, because the newly added text in 38.211 of “SRS resource set indicator” could further specify the separate power control parameters for each TRP, no matter STRP or MTRP PUSCH operation when considering STRP/MTRP dynamic switching. |
| MediaTek | Agree with ZTE that the TP is not needed. |
| OPPO | We tend to agree with ZTE |
| vivo | We share similar view as ZTE. If it has to be captured, a complete text is preferred to have all possible codepoints of SRS resource set indicator for both STRP and MTRP included. |
| LG | We share similar view as ZTE and vivo. |
| Futurewei | Fine with the TP and fine with more complete text to describe all codepoints. |
| CATT | We are fine to have a TP to make the spec clear. The TP is OK to capture the SRI based power control mechanism for the cases that SRS resource set indicator is “10” or “11”. However, the SRI based power control mechanism for the cases that the SRS resource set indicator is “00” or “01” is missing. In order to capture all cases, we suggest to change the TP to be the one provided by us in section 1.20. |
| Ericsson | We tend to agree with ZTE. If needed, a TP could be discussed later that covers both STRP/MTRP modes when 2 SRS resource sets are configured. |
| Xiaomi | Agree with ZTE that the TP is not needed. |
| Intel | Tend to agree with ZTE, TP not needed |
| Huawei, HiSilicon | We support the TP, and prefer to have a clearer description for SRI based PC due to dynamic switching between sTRP and mTRP. When DCI includes two SRI fields and mTRP mode is indicated, two pc parameters are used for the related two SRIs, while, when DCI includes two SRI fields and sTRP mode is indicated, only one of the pc parameters are used for the actually used SRI. |
| Nokia, NSB | We share similar views as ZTE and vivo. |

### Update (discussion closed)

Majority of companies think that the TP is not needed. No further discuss is triggered on this TP.

## Issue 2: TP in [2] to address CG PUSCH

Description related to the below agreement (for pathlossReferenceIndex) is not captured in TS 38.213.

Agreement

For a BWP configured with two SRS resource sets for CB or NCB based mTRP PUSCH repetition with Type 1 CG configuration,

- If the CG is configured with only one field for each of ‘pathlossReferenceIndex’, 'srs-ResourceIndicator', 'precodingAndNumberOfLayers', 'p0-PUSCH-Alpha' and 'powerControlLoopToUse', PUSCH repetitions are associated with the first SRS resource set.

[2] provides the below TP to capture changes,

Section 7.1.1 of TS 38.213 - For a PUSCH transmission configured by ConfiguredGrantConfig, if rrc-ConfiguredUplinkGrant is included in ConfiguredGrantConfig, a RS resource index qd is provided by a value of pathlossReferenceIndex included in rrc-ConfiguredUplinkGrant if the UE is provided one SRS resource set in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’. where the RS resource is either on serving cell c or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking. For a PUSCH transmission configured by ConfiguredGrantConfig, if rrc-ConfiguredUplinkGrant is included in ConfiguredGrantConfig, and if If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’ and for configured grant Type 1 PUSCH, first and second RS resource indexes qd are provided by respective values of pathlossReferenceIndex and pathlossReferenceIndex2 in rrc-ConfiguredUplinkGrant if both values are configured. For a PUSCH transmission configured by ConfiguredGrantConfig, if rrc-ConfiguredUplinkGrant is included in ConfiguredGrantConfig, and if the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’ and for configured grant Type 1 PUSCH, a RS resource index qd is provided by a value of pathlossReferenceIndex in rrc-ConfiguredUplinkGrant if pathlossReferenceIndex2 is not configured. The RS resource(s) is either on serving cell c or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking. - For a PUSCH transmission configured by ConfiguredGrantConfig that does not include rrc-ConfiguredUplinkGrant, - If the UE is provided one SRS resource set in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’, the UE determines a RS resource index qd from a value of PUSCH-PathlossReferenceRS-Id that is mapped to a SRI field value in a DCI format activating the PUSCH transmission. - If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ and the DCI format activating the PUSCH transmission includes two SRI fields, the UE determines first and second RS resource indexes qd from respective first and second values of PUSCH-PathlossReferenceRS-Id that are mapped to the first and second SRI values corresponding to each SRS resource set with usage set to ‘codebook’, respectively. - If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘nonCodebook’ and the DCI format activating the PUSCH transmission includes two SRI fields, the UE determines first and second RS resource indexes qd from respective first and second values of PUSCH-PathlossReferenceRS-Id that are mapped to the first SRI value corresponding to the first SRS resource set with usage set to ‘nonCodebook’, and the value, associated with the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value, corresponding to the second SRS resource set with usage set to ‘nonCodebook’. - If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’ and the DCI format activating the PUSCH transmission does not include an SRI field, the UE determines first and second RS resource indexes qd with respective first and second PUSCH-PathlossReferenceRS-Id value being equal to zero and one. - If the DCI format activating the PUSCH transmission does not include an SRI field, the UE determines a RS resource index qd with a respective PUSCH-PathlossReferenceRS-Id value being equal to zero where the RS resource is either on serving cell c or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking …

FL views that the TP in [2] can be improved to make changes to be minimal. Please indicate your views about this issue.

| Company | Comments |
| --- | --- |
| Apple | We failed to see the necessity for the TP. It can be discussed during CR review stage. |
| Lenovo | Support. |
| QC | We think 38.214 may be a better place to capture the agreement above because it is about association of PUSCH repetitions with the first SRS resource set, and it is not so much related to power control. |
| Samsung | We are fine with this clarification. |
| ZTE | Support. |
| MediaTek | Support in principle. |
| OPPO | We are fine with the TP |
| vivo | We share similar views as QC. It is simpler to be captured in TS 38.214 as following. Proposed TP in Clause 6.1.2.3 of TS 38.214 For PUSCH transmissions with a Type 1 configured grant, when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, if configuredGrantConfig contains only one pathlossReferenceIndex, p0-PUSCH-Alpha, powerControlLoopToUse, srs-ResourceIndicator and precodingAndNumberOfLayers (applicable when higher layer parameter usage in SRS-ResourceSet set to 'codebook'), PUSCH repetitions are associated only with the first SRS resource set. |
| LG | We share the same view with QC and vivo. |
| Futurewei | Support. The agreement is relevant to power control since it is related to PL reference index and PC loop to use. We are fine with capturing the agreement in 214, but we also think the complete description is needed in 213 as well. |
| CATT | Support and we are also fine to capture the agreement above in TS38.214. |
| Ericsson | We have some concern on this TP although we understand the motivation. For instance, we have the following two sentences in the modified TP: “For a PUSCH transmission configured by ConfiguredGrantConfig, if rrc-ConfiguredUplinkGrant is included in ConfiguredGrantConfig, a RS resource index qd is provided by a value of pathlossReferenceIndex included in rrc-ConfiguredUplinkGrant if the UE is provided one SRS resource set in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’.” “ For a PUSCH transmission configured by ConfiguredGrantConfig, if rrc-ConfiguredUplinkGrant is included in ConfiguredGrantConfig, and if If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’ and for configured grant Type 1 PUSCH, first and second RS resource indexes qd are provided by respective values of pathlossReferenceIndex and pathlossReferenceIndex2 in rrc-ConfiguredUplinkGrant if both values are configured.” Now, for example if srs-ResourceSetToAddModList is configured with 2 SRS resource sets and srs-ResourceSetToAddModListDCI-0-2 is configured with 1 SRS resource set, then which of the above two sentences apply? It seems both sentences apply in this case which would create ambiguity? |
| Xiaomi | Support in principle, but better capture this in 214. |
| Intel | Support in principle |
| Huawei, HiSilicon | We are fine with the TP. |
| Nokia, NSB | Ok with the TP and some wording may need to revise to address the point mentioned by E///. |

### Update 1 :

Majority of companies think that a TP is needed. Also, capturing a TP in 38.214 (instead of partly capturing it in two specs) is preferred by multiple companies. Vivo provided a draft TP capturing this in 38.214, and it seems like a better way. Let’s try the following proposal,

Proposal 1.2: Adopt the following TP in Clause 6.1.2.3 of TS 38.214,

Clause 6.1.2.3 of TS 38.214 --------------Text omitted---------------- For PUSCH transmissions with a Type 1 configured grant, when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, if configuredGrantConfig contains only one pathlossReferenceIndex, p0-PUSCH-Alpha, powerControlLoopToUse, srs-ResourceIndicator and precodingAndNumberOfLayers (applicable when higher layer parameter usage in SRS-ResourceSet set to 'codebook’), PUSCH repetitions are associated only with the first SRS resource set. --------------Text omitted----------------

| Company | Comments |
| --- | --- |
| ZTE | Support this TP, which is a proper way to capture this in 38.214 because it can avoid the issue raised by E///. @E///, I understand your motivation, but we haven’t discussed whether the number of SRS resource sets configured by srs-ResourceSetToAddModList and srs-ResourceSetToAddModListDCI-0-2 can be different, right? Please see our comment of issue 14. |
| QC | Support. |
| LG | Support. |
| Lenovo | Support |
| OPPO | Support |
| CATT | Support the TP in principle. We have a question that does “ configuredGrantConfig contains only one pathlossReferenceIndex, p0-PUSCH-Alpha, powerControlLoopToUse, srs-ResourceIndicator and precodingAndNumberOfLayers (applicable when higher layer parameter usage in SRS-ResourceSet set to 'codebook’)” mean all of pathlossReferenceIndex, p0-PUSCH-Alpha, powerControlLoopToUse, srs-ResourceIndicator and precodingAndNumberOfLayers (applicable when higher layer parameter usage in SRS-ResourceSet set to 'codebook’) have to be configured? Since srs-ResourceIndicator is optional in TS38.331, no srs-ResourceIndicator (e.g., when only one SRS resource is configured, srs-ResourceIndicator is not needed) also should be allowed in Rel-17. Besides, we propose to change “PUSCH repetitions are associated only with the first SRS resource set” to “ PUSCH repetitions are associated only with the first SRS resource set of srs-ResourceSetToAddModList” to make the spec clear. |

## Issue 3: TP in [4] to address PUCCH repetition number

The following agreement is not fully reflected in 38.213.

Agreement

For multi-TRP PUCCH (scheme 1 and 3) and PUSCH (Type A and B) repetition, when the number of repetitions is equal to two, the first and second transmission occasion shall be associated with two TRPs, respectively (two UL beams or Power control parameter sets), regardless of the configured mapping pattern.

- Note: For M-TRP PUSCH type B, the number of repetitions refers to ‘nominal’ repetition.

[4] has the following suggestion,

38.213, Section 9.2.6 <Unchanged part omitted> When a PUCCH resource with one of PUCCH formats 0, 1, 2, 3 or 4 used for repetitions of a PUCCH transmission by a UE includes first and second spatial settings, or first and second sets of power control parameters, as described in [11, TS 38.321] and in clause 7.2.1, the UE - uses the first and second spatial settings, or the first and second sets of power control parameters, for first and second repetitions of the PUCCH transmission, respectively, when NPUCCHrepeat=2 if mappingPattern = ‘cyclicMapping’, - uses the first spatial setting or the first set of power control parameters for first and second repetitions for the PUCCH transmission when NPUCCHrepeat=2 if mappingPattern = ‘sequentialMapping’, - alternates between the first and second spatial settings, or between the first and second sets of power control parameters, respectively, per NPUCCHswitch repetitions of the PUCCH transmission, where NPUCCHswitch=1 if mappingPattern = ‘cyclicMapping’; else, NPUCCHswitch=2. <Unchanged part omitted>

FL views that mentioning of the PUCCH formats is not necessary (editor already generalized that in the CR), Also, mention of “if mappingPattern = ‘cyclicMapping” and “if mappingPattern = ‘sequentialMapping” with two bullets may not be needed and we can have a general statement as in the original agreement.

Please indicate your views about this issue.

| Company | Comments |
| --- | --- |
| Apple | We do not think the TP is needed. |
| Lenovo | Not support since it made a mistake. As an agreement in R1#105e said that “ Agreement For multi-TRP PUCCH (scheme 1 and 3) and PUSCH (Type A and B) repetition, when the number of repetitions is equal to two, the first and second transmission occasion shall be associated with two TRPs, respectively (two UL beams or Power control parameter sets), regardless of the configured mapping pattern. - Note: For M-TRP PUSCH type B, the number of repetitions refers to ‘nominal’ repetition.” It said that when the repetition number is equal to 2, the first and second transmission occasion should be associated with two TRPs, therefore, the original TS38.213 covered this agreement, while the TP does not. |
| NTT Docomo | Do not support the TP. Same view as Lenovo. we had an agreement that when repetition number is 2, the two repetitions are associated with two TRPs (cyclic mapping), regardless of mapping pattern. Agreement For multi-TRP PUCCH (scheme 1 and 3) and PUSCH (Type A and B) repetition, when the number of repetitions is equal to two, the first and second transmission occasion shall be associated with two TRPs, respectively (two UL beams or Power control parameter sets), regardless of the configured mapping pattern. |
| QC | Similar view as Lenovo. We are confused by this TP. The original spec language seems to be aligned with the agreement. The TP is different than the agreement as in the case of 2 repetitions, the configured mapping pattern is irrelevant (which is the current spec). Can ZTE clarify the intention further? |
| Samsung | We can share the same view as Lenovo. The original text can cover the agreement. |
| ZTE | Thanks for FL’s reminder with the above agreement. Based on that, the current text proposed by the editor is accurate to our understanding, so we can be fine not to adopt this TP. |
| MediaTek | Do not support. The current implementation in TS 38.213 of the mentioned agreement is fine. |
| OPPO | Share similar view as Lenovo. The original version is more concise and has capture the agreement |
| vivo | Similar view as Lenovo. Current spec is clear and precise. |
| LG | Do not need this CR. The proposed TP is not aligned with the agreement, which applies the same beam mapping pattern regardless of configured mappingPattern when repetition number is 2. |
| Futurewei | Similar view as the majority. |
| CATT | Similar view as the majority. |
| Ericsson | This TP is not needed. We share same view as Lenovo. |
| Xiaomi | Similar view as the majority. |
| Intel | We have similar view with Lenovo, and do not support this TP. |
| Huawei, HiSilicon | This is not needed. |
| Nokia, NSB | Agree with FL and Lenovo’s assessments. |

### Update (discussion closed)

Majority of companies think that the TP is not needed. No further discuss is triggered on this TP.

## Issue 4: TP in [4] to address srs-ResourceSetToAddModListDCI-0-2

The below agreement is not implemented in 38.212.

Agreement

For mTRP PUSCH repetition scheduled with DCI format 0_2, the value of the NSRS,0_2 in two SRS resource sets configured by higher layer parameter srs-ResourceSetToAddModListDCI-0-2 should be the same.

Proposed TP in [4],

38.212, Section 7.3.1.1 <Unchanged part omitted> Table 7.3.1.1.2-36: SRS resource set indication Bit field mapped to index; SRS resource set indication. 0; SRS resource indicator field and Precoding information and number of layers field are associated with the first SRS resource set; Second SRS resource indicator field and Second Precoding information field are reserved.. 1; SRS resource indicator field and Precoding information and number of layers field are associated with the second SRS resource set; Second SRS resource indicator field and Second Precoding information field are reserved.. 2; SRS resource indicator field and Precoding information and number of layers field are associated with the first SRS resource set; Second SRS resource indicator field and Second Precoding information field are associated with the second SRS resource set.. 3; SRS resource indicator field and Precoding information and number of layers field are associated with the first SRS resource set; Second SRS resource indicator field and Second Precoding information field are associated with the second SRS resource set.. NOTE 1: The first and the second SRS resource sets are respectively the ones with lower and higher srs-ResourceSetId of the two SRS resources sets configured by higher layer parameter srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, and associated with the higher layer parameter usage of value 'nonCodeBook' if txConfig=nonCodebook or 'codeBook' if txConfig=codebook. When only one SRS resource set is configured by higher layer parameter srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, and associated with the higher layer parameter usage of value 'codeBook' or 'nonCodeBook' respectively, the first SRS resource set is the SRS resource set. The association of the first and second SRS resource sets to PUSCH repetitions for each bit field index value is as defined in Clause 6.1.2.1 of [6, TS 38.214]. NOTE 2: For DCI format 0_2, the first and second SRS resource sets configured by higher layer parameter srs-ResourceSetToAddModListDCI-0-2 are composed of the first NSRS, 0_2 SRS resources together with other configurations in the first and second SRS resource sets configured by higher layer parameter srs-ResourceSetToAddModList, if any, and associated with the higher layer parameter usage of value 'codeBook' or 'nonCodeBook', respectively, except for the higher layer parameters ‘srs-ResourceSetId’ and ‘srs-ResourceIdList’. Where, the value of NSRS, 0_2 in two SRS resource sets configured by higher layer parameter srs-ResourceSetToAddModListDCI-0-2 is the same.. <Unchanged part omitted>

Please provide your views,

| Company | Comments |
| --- | --- |
| Apple | OK |
| Lenovo | Support |
| QC | Not necessary. This seems to be already captured in 38.214: When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook’, the UE is not expected to be configured with different number of SRS resources in the two SRS resource sets. … When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'nonCodebook’, the UE is not expected to be configured with different number of SRS resources in the two SRS resource sets. |
| Samsung | We can share similar view to Qualcomm. This TP is not necessary. |
| ZTE | We tend to agree with QC. |
| MediaTek | Not needed. We share the same view as QC. |
| OPPO | Share the similar as QC/Samsung. |
| vivo | Not needed. We share similar view as QC. |
| LG | Not needed. We share similar view as QC. |
| Futurewei | Fine with the TP but understood it’s not absolutely necessary |
| CATT | Similar view as QC. |
| Ericsson | TP is not needed. Similar view as QC. |
| Xiaomi | Not needed. We share similar view as QC. |
| Intel | We have similar view with QC. This TP is not necessary since it has been captured in TS 38.214. |
| Huawei, HiSilicon | Share the same view as QC. |
| Nokia, NSB | Not needed. |

### Update (discussion closed)

Majority of companies think that the TP is not needed. No further discuss is triggered on this TP.

## Issue 5: Reset of close loop index for PUSCH

Two proposal in [5] and the corresponding TP require some discussion. There seems a typo in proposal 4 in [5] (corrected it below)

Proposal 1.5-A: When twoPUSCH-PC-AdjustmentStates is configured and one SRS resource per SRS resource set is configured (i.e., when two SRI fields are absent in DCI formats 0_1 / 0_2), if the first P0-PUSCH-AlphaSet in P0-AlphaSet is reconfigured, closed-loop adjustment state with index l = 0 should be reset; if the PC parameters in the second P0-PUSCH-AlphaSet in P0-AlphaSet is reconfigured, closed-loop adjustment state with index l = 1 should be reset.

Proposal 1.5-B: When twoPUSCH-PC-AdjustmentStates is configured and DCI schedules a retransmission of CG-PUSCH for type 1 CG or type 2 CG (DCI with CRC scrambled with CS-RNTI and NDI=1) while the CG configuration is RRC-configured with two fields of power control parameters , if the first (legacy) RRC-configured fields ‘p0-PUSCH-Alpha’ is reconfigured, closed-loop adjustment state with the first  (legacy) ‘powerControlLoopToUse’ should be reset; if the second (new) RRC-configured fields ‘p0-PUSCH-Alpha’ is reconfigured, closed-loop adjustment state with the second (new) ‘powerControlLoopToUse’ should be reset.

In [5], the following TP is also provided to implement the proposals,

38.213, Section 7.1.1 (omitted part) If a UE transmits a PUSCH on active UL BWP b of carrier f of serving cell c using parameter set configuration with index j and PUSCH power control adjustment state with index l, the UE determines the PUSCH transmission power PPUSCH,b,f,c(i,j,qd,l) in PUSCH transmission occasion i as [dBm] where, (omitted part) - A UE resets accumulation of a PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c to fb,f,ck,l=0, k=0,1,…,i - If a configuration for a corresponding PO_UE_PUSCH,b,f,cj value is provided by higher layers - If a configuration for a corresponding αb,f,cj value is provided by higher layers where l is determined from the value of j as - If j>1 and the UE is provided higher SRI-PUSCH-PowerControl, l is the sri-PUSCH-ClosedLoopIndex value(s) configured in any SRI-PUSCH-PowerControl with the sri-P0-PUSCH-AlphaSetId value corresponding to j - If j>1 and the UE is not provided SRI-PUSCH-PowerControl or j=0, l=10 if PO_UE_PUSCH,b,f,cj value or αb,f,cj value is from second P0-PUSCH-AlphaSet in p0-AlphaSets, l=0 otherwise - If j=1, l is provided by the value of powerControlLoopToUse if PO_UE_PUSCH,b,f,c1 or αb,f,c(1) is provided by p0-PUSCH-Alpha in ConfiguredGrantConfig, while l is provided by the value of powerControlLoopToUse2 if PO_UE_PUSCH,b,f,c1 or αb,f,c(1) is provided by p0-PUSCH-Alpha2 in ConfiguredGrantConfig.

In general, it is not clear how often there can be reconfiguration of first and second P0-PUSCH-AlphaSet in P0-AlphaSet when operating with mTRP PUSCH. Anyways, clearly only the single TRP behavior is defined in 38.213 for such scenarios, and may need to consider the extension to support mTRP. Please indicate your view/support to the proposals and TP.

| Company | Comments |
| --- | --- |
| Apple | It seems the TP is not aligned with the proposal? The relevant spec is the paragraph starting with “ A UE resets accumulation of a PUSCH power control adjustment state…”, We are open to discuss, but it seems no spec change is needed. |
| NTT Docomo | Support the TP. |
| QC | Ok in principle. A simpler TP for the second change can be: - If j=1, l is provided by the value of powerControlLoopToUse or powerControlLoopToUse2 that is associated with PO_UE_PUSCH,b,f,cj or αb,f,cj |
| Samsung | We are open to discuss. Like single TRP behavior, behavior for mTRP seems to be specified. |
| ZTE | We can be fine with these two proposals and their TPs, and QC’s refinement looks better to us. |
| MediaTek | Support. We are also fine with QC’s version on the second change. |
| OPPO | Support. Regarding QC’s version, it is not clear when l is associated with powerControlLoopToUse, and when l is associated with powerControlLoopToUse2. In order to make the spec clearer, it is preferred to explicitly capture the conditions in the spec. |
| vivo | Support. OPPO’s version is clearer. |
| LG | We are open for further discussion whether to extend reset mechanism for closed loop index for MTRP case. |
| Futurewei | Support, the reset behavior is necessary. OPPO’s version seems working. |
| CATT | Support the principle as well as QC’s change. |
| Ericsson | Although we agree with the intention, we have a question whether this change should apply to the case where j=0? The exact TP may need further discussion. |
| Xiaomi | Support, prefer QC’s version on the second change. |
| Intel | Same view as Samsung – good to clarify resetting for mTRP as well |
| Huawei, HiSilicon | Fine with the TP. |
| OPPO2 | Reply to Ericsson: for the case j=0, the part “ l=0 otherwise” is always applied. Thus, there is no change for j=0. |
| Nokia, NSB | Fine with the proposals and TP. |

### Update 1:

Majority of companies are open for discussion.

- Few companies are ok with the TP, QC suggested a refined version but many prefer original version by OPPO.

- Proposals 1.5.A/B seems not necessary if we can agree on the TP as some companies do not fully see them as needed.

- Thanks to OPPO for provided responses to the questions raised by others, please check above.

Proposal 1.2: Adopt the following TP in Clause 7.1.1 of TS 38.213,

Clause 7.1.1 of TS 38.213 --------------Text omitted---------------- If a UE transmits a PUSCH on active UL BWP b of carrier f of serving cell c using parameter set configuration with index j and PUSCH power control adjustment state with index l, the UE determines the PUSCH transmission power PPUSCH,b,f,c(i,j,qd,l) in PUSCH transmission occasion i as [dBm] where, --------------Text omitted---------------- - A UE resets accumulation of a PUSCH power control adjustment state l for active UL BWP b of carrier f of serving cell c to fb,f,ck,l=0, k=0,1,…,i - If a configuration for a corresponding PO_UE_PUSCH,b,f,cj value is provided by higher layers - If a configuration for a corresponding αb,f,cj value is provided by higher layers where l is determined from the value of j as - If j>1 and the UE is provided higher SRI-PUSCH-PowerControl, l is the sri-PUSCH-ClosedLoopIndex value(s) configured in any SRI-PUSCH-PowerControl with the sri-P0-PUSCH-AlphaSetId value corresponding to j - If j>1 and the UE is not provided SRI-PUSCH-PowerControl or j=0, l=10 if PO_UE_PUSCH,b,f,cj value or αb,f,cj value is from second P0-PUSCH-AlphaSet in p0-AlphaSets, l=0 otherwise - If j=1, l is provided by the value of powerControlLoopToUse if PO_UE_PUSCH,b,f,c1 or αb,f,c(1) is provided by p0-PUSCH-Alpha in ConfiguredGrantConfig, while l is provided by the value of powerControlLoopToUse2 if PO_UE_PUSCH,b,f,c1 or αb,f,c(1) is provided by p0-PUSCH-Alpha2 in ConfiguredGrantConfig. --------------Text omitted----------------

| Company | Comments |
| --- | --- |
| ZTE | Fine with this TP. |
| QC | Ok with the TP. |
| LG | Ok with the TP. |
| OPPO | Support |
| CATT | Ok with the TP. |

## Issue 6: TP in [7] to address second sri-PUSCH-MappingToAddModList

As in [7], there is an agreement in RAN2 #116b-e that two sri-PUSCH-MappingToAddModList will be configured for multi-TRP PUSCH. The following TP is proposed in [7] to capture it.

38.213, Section 7.1.1 < Unchanged parts are omitted > - If the UE is provided enablePL-RS-UpdateForPUSCH-SRS, a mapping between sri-PUSCH-PowerControlId and PUSCH-PathlossReferenceRS-Id values can be updated by a MAC CE as described in [11, TS 38.321] - For a PUSCH transmission scheduled by a DCI format that does not include an SRI field, or for a PUSCH transmission configured by ConfiguredGrantConfig and activated, as described in clause 10.2, by a DCI format that does not include an SRI field, the UE determines a RS resource index qd from the PUSCH-PathlossReferenceRS-Id mapped to sri-PUSCH-PowerControlId = 0. If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', the UE determines first and second RS resource indexes qd from respective PUSCH-PathlossReferenceRS-Id mapped to sri-PUSCH-PowerControlId = 0 of sri-PUSCH-MappingToAddModList and sri-PUSCH-PowerControlId = 0 1 of sri-PUSCH-MappingToAddModList2 respectively. < Unchanged parts are omitted >

Please provide the views on above TP.

| Company | Comments |
| --- | --- |
| Apple | We failed to see the necessity for the TP, but we are open for discussion. |
| Lenovo | Sorry for a mistake in “ sri-PUSCH-PowerControlId = 0 1 of sri-PUSCH-MappingToAddModList2 respectively.” It should be “ sri-PUSCH-PowerControlId = 0 1 of sri-PUSCH-MappingToAddModList2 respectively.” And after the correction, support it. |
| NTT Docomo | We agree with the intention. But in our understanding the TP should be “If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', the UE determines first and second RS resource indexes qd from respective PUSCH-PathlossReferenceRS-Id mapped to sri-PUSCH-PowerControlId = 0 of sri-PUSCH-MappingToAddModList and sri-PUSCH-PowerControlId = 1 0 of sri-PUSCH-MappingToAddModList2 respectively.” |
| QC | Seems ok given the clarification from Lenovo above. |
| Samsung | According to RAN2’s RRC design for sri-PUSCH-PowerControl with two SRS resource sets (adding ‘sri-PUSCH-MappingToAddModList2-r17’ for the second SRS resource set), we are fine with Lenovo’s updated TP. |
| ZTE | Support of Lenovo’s modification. |
| MediaTek | Ok with Lenovo’s corrected version. |
| OPPO | We are fine with DCM’s version |
| vivo | Support Lenovo’s updated version. |
| LG | Support Lenovo’s updated version. |
| Futurewei | Ok with the correction. |
| CATT | OK with the change from Docomo and Lenovo. |
| Ericsson | Support the TP with updated correction from Lenovo. |
| Xiaomi | Ok with the updated version. |
| Intel | Support the Lenovo’s updated TP |
| Huawei, HiSilicon | Fine with the updated version from Lenovo. |
| Nokia, NSB | Fine with Lenovo’s updated version. |

### Update 1:

Companies are ok with the Lenovo update and look like an offline agreement.

Offline agreement 1.6: Adopt the following TP in Clause 7.1.1 of TS 38.213,

Clause 7.1.1 of TS 38.213 --------------Text omitted---------------- - If the UE is provided enablePL-RS-UpdateForPUSCH-SRS, a mapping between sri-PUSCH-PowerControlId and PUSCH-PathlossReferenceRS-Id values can be updated by a MAC CE as described in [11, TS 38.321] - For a PUSCH transmission scheduled by a DCI format that does not include an SRI field, or for a PUSCH transmission configured by ConfiguredGrantConfig and activated, as described in clause 10.2, by a DCI format that does not include an SRI field, the UE determines a RS resource index qd from the PUSCH-PathlossReferenceRS-Id mapped to sri-PUSCH-PowerControlId = 0. If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’, the UE determines first and second RS resource indexes qd from respective PUSCH-PathlossReferenceRS-Id mapped to sri-PUSCH-PowerControlId = 0 of sri-PUSCH-MappingToAddModList and sri-PUSCH-PowerControlId = 0 of sri-PUSCH-MappingToAddModList2 respectively. --------------Text omitted----------------

## Issue 7: TP to address DMRS-PTRS association

As agreed in RAN1, mTRP PUSCH repetition mode is only valid when the SRS resource indicator is set to ‘10’ or ‘11’, but this is not fully reflected in DMRS-PTRS association description in 38.212.

[7] propose the following TP.

38.212, Section 7.3.1.1.2 : Format 0_1 < Unchanged parts are omitted > - PTRS-DMRS association – number of bits determined as follows - 0 bit if PTRS-UplinkConfig is not configured in either dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB and transform precoder is disabled, or if transform precoder is enabled, or if maxRank=1; - 2 bits otherwise, where Table 7.3.1.1.2-25/7.3.1.1.2-25A and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field. When the SRS resource set indicator field is present and maxRank>2, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26. When the SRS resource set indicator field is present and is equal “10” and “11” and maxRank=2, the MSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A. When the SRS resource set indicator field is present and is equal “00” and “01” and maxRank=2, the field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator and/or Precoding information and number of layers field, according to Table 7.3.1.1.2-25. < Unchanged parts are omitted > Section 7.3.1.1.3 : Format 0_2 < Unchanged parts are omitted > - PTRS-DMRS association – number of bits determined as follows - 0 bit if PTRS-UplinkConfig is not configured in either dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB and transform precoder is disabled, or if transform precoder is enabled, or if maxRank=1; - 2 bits otherwise, where Table 7.3.1.1.2-25/7.3.1.1.2-25A and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field. When the SRS resource set indicator field is present and maxRank>2, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26. When the SRS resource set indicator field is present and is equal “10” and “11” and maxRank=2, the MSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A. When the SRS resource set indicator field is present and is equal “00” and “01” and maxRank=2, the field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator and/or Precoding information and number of layers field, according to Table 7.3.1.1.2-25. < Unchanged parts are omitted >

Please provide the views on above TP.

| Company | Comments |
| --- | --- |
| Apple | We failed to see the necessity for the TP. |
| Lenovo | Support. |
| NTT Docomo | Support the TP |
| QC | Ok with the change “ and is equal “10” and “11””. For the other change, we think it may not be necessary (the Rel-15/16 part applies) |
| Samsung | We are fine with this TP for the clarification. |
| ZTE | Support. |
| MediaTek | Support. |
| OPPO | We are fine with the TP |
| vivo | Support with wording correction as follows: is equals “10” and or “11” is equals “00” and or “01” |
| LG | Support TP with vivo’s editorial correction. |
| Futurewei | Support, and vivo’s version is better. |
| CATT | Support the TP with vivo’s editorial correction |
| Ericsson | We had a TP for the same issue in [19] which is missed in this summary. Our understanding is similar to QC that only change ‘and is equal ‘10’ and ‘11’’ is needed. Note that a similar change is also needed for the case of maxRank > 2. Our TP is copied below FYI: --------------- Start of TP2 for 38.212 v17.0.0------------- 7.3.1.1.2 Format 0_1 DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback information (CG-DFI) to a UE. ---unchanged text omitted --- PTRS-DMRS association – number of bits determined as follows - 0 bit if PTRS-UplinkConfig is not configured in either dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB and transform precoder is disabled, or if transform precoder is enabled, or if maxRank=1; - 2 bits otherwise, where Table 7.3.1.1.2-25/7.3.1.1.2-25A and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field. When the SRS resource set indicator field is present, and a value of 2 or 3 is indicated in the SRS resource set indicator field, and maxRank>2, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26. When the SRS resource set indicator field is present, and a value of 2 or 3 is indicated in the SRS resource set indicator field, and maxRank=2, the MSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A. ---unchanged text omitted --- 7.3.1.1.3 Format 0_2 DCI format 0_2 is used for the scheduling of PUSCH in one cell. The following information is transmitted by means of the DCI format 0_2 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI: ---unchanged text omitted --- - PTRS-DMRS association – number of bits determined as follows - 0 bit if PTRS-UplinkConfig is not configured in either dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB and transform precoder is disabled, or if transform precoder is enabled, or if maxRankDCI-0-2=1; - 2 bits otherwise, where Table 7.3.1.1.2-25/7.3.1.1.2-25A and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field. When the SRS resource set indicator field is present, and a value of 2 or 3 is indicated in the SRS resource set indicator field, and maxRankDCI-0-2>2, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26 field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26. When the SRS resource set indicator field is present, and a value of 2 or 3 is indicated in the SRS resource set indicator field, and maxRankDCI-0-2=2, the MSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A. ---unchanged text omitted --- -------------------End of TP2 for 38.212 ------------------ |
| Xiaomi | Fine with the TP. |
| Intel | We support this TP. |
| Nokia, NSB | Fine with the TP. |

### Update 1:

Majority of companies are ok with the TP and there are some suggestions,

- QC, E/// views that TP shall only consider “10” and “11”” and other instances (“00” and “01”) can be Rel-15/16 part.

- Vivo has a editorial suggestion and many are ok with it.

- E/// highlight that a similar change is needed for maxRank >2.

Let’s further discussion the following TP which is updated based on comments above.

Proposal 1.7: Adopt the following TP in Clause 7.1.1 of TS 38.212,

Clause 7.3.1.1.2 of TS 38.212 --------------Text omitted---------------- - PTRS-DMRS association – number of bits determined as follows - 0 bit if PTRS-UplinkConfig is not configured in either dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB and transform precoder is disabled, or if transform precoder is enabled, or if maxRank=1; - 2 bits otherwise, where Table 7.3.1.1.2-25/7.3.1.1.2-25A and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field. When the SRS resource set indicator field is present and equals “10” and “11” and maxRank>2, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26. When the SRS resource set indicator field is present and equals “10” and “11” and maxRank=2, the MSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A. --------------Text omitted---------------- Clause 7.3.1.1.3 of TS 38.212 --------------Text omitted---------------- - PTRS-DMRS association – number of bits determined as follows - 0 bit if PTRS-UplinkConfig is not configured in either dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB and transform precoder is disabled, or if transform precoder is enabled, or if maxRank=1; - 2 bits otherwise, where Table 7.3.1.1.2-25/7.3.1.1.2-25A and 7.3.1.1.2-26 are used to indicate the association between PTRS port(s) and DMRS port(s) when one PT-RS port and two PT-RS ports are configured by maxNrofPorts in PTRS-UplinkConfig respectively, and the DMRS ports are indicated by the Antenna ports field. When the SRS resource set indicator field is present and equals “10” and “11” and maxRank>2, this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator field and/or Precoding information and number of layers field according to Table 7.3.1.1.2-25 and 7.3.1.1.2-26. When the SRS resource set indicator field is present and equals “10” and “11” and maxRank=2, the MSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to SRS resource indicator and/or Precoding information and number of layers field, and the LSB of this field indicates the association between PTRS port(s) and DMRS port(s) corresponding to Second SRS resource indicator field and/or Second Precoding information field, according to Table 7.3.1.1.2-25A. --------------Text omitted----------------

| Company | Comments |
| --- | --- |
| ZTE | Support this TP. |
| QC | Support. |
| LG | We supports TP for maxRank=2 but no change is needed for maxRank>2. For “00” or “01” case, first SRI field and first TPMI field are still used instead of second SRI/TPMI fields. Therefore, this PTRS field can be used regardless of the value of SRS resource set indicator field. |
| Lenovo | Don’t support. From the updated specification, UE’s behavior is not clear when the SRS resource set indicator field is present and equals “00” and “01” and maxRank=2. Therefore, we suggest to capture all cases. Besides, we share same view as LG, no change is needed for maxRank>2. |
| OPPO | Vivo’s modification suggested in the 1st round should be included. |
| CATT | Similar view as LG. Only support changes for maxRank=2. |

## Issue 8: mTRP PUSCH repetition support for Cov. Enh

The following proposals are from [8] (UE capability proposal in [11] can be discussed later), which are related to applying enhancement introduced in coverage enhancement (available slot counting is supported for PUSCH repetition type A and TboMS) also for mTRP PUSCH operation.

Proposal 1.8-A: Support available slot counting for M-TRP PUSCH repetition type A.

Proposal 1.8-B: Support M-TRP TboMS.

- Two beams/power control parameters are mapped to multiple repetitions of TboMS. One beam is applied to each repetition of TboMS.

Please provide the views on above two proposals.

| Company | Comments |
| --- | --- |
| Apple | We think we need some discussion on mTRP+Cov enh. In addition to the two proposals, there is one additional proposal as follows in R1-2201759, which seems to be missed in the FL summary With regard to cross-slot channel estimation, support to configure the number of consecutive slots per beam for sequential mapping, where the candidate value is based on the candidate time domain bundling window size for cross-slot channel estimation. We support the proposals. |
| NTT Docomo | Support the two proposals |
| QC | This is not essential. These would be new features that at least require new UE capabilities (if not new RRC configurations). |
| Samsung | We don’t think it is appropriate to discuss in this session. That can be discussed in R17 CovEnh session. We should avoid the overlapped discussions. |
| ZTE | Do NOT support due to it is out of scope. It is proper to discuss these enhancements in CE WI, if needed. |
| OPPO | Share similar view as QC that is not essential. |
| Vivo | We don’t think it is appropriate to discuss in this session. |
| LG | We don’t think it is appropriate to discuss in maintenance session. If needed, we should first check potential impacts for cross item enhancement. |
| Futurewei | We are ok with further discussion. |
| Ericsson | This is not an essential correction. Do not support. |
| Xiaomi | Share similar view with QC that is not essential. |
| Intel | Support the 1st proposal. We are open to discuss the 2nd proposal. |
| Huawei, HiSilicon | We don’t see the change in the spec, if coverage is configured, it works already in the spec. |
| Nokia, NSB | This is not essential. |

### Update (discussion closed)

Majority of companies think that the discussion is not needed or not essential.

## Issue 9: TP in [15] to address default beam for PUSCH

To correct an editorial error, [15] propose the following TP.

------------------------------------- Start of text proposal of 6.1 in TS 38.214 ------------------------------------ For PUSCH scheduled by DCI format 0_0 on a cell, the UE shall transmit PUSCH according to the spatial relation, if applicable, corresponding to the dedicated PUCCH resource with the lowest ID within the active UL BWP of the cell, as described in Clause 9.2.1 of [6, TS 38.213]. If the dedicated PUCCH resource with the lowest ID within the active UL BWP of the cell corresponds to two spatial relations, the UE shall transmit the PUSCH according to the spatial relation with lower ID. -------------------------------------------------- End of text proposal ------------------------------------------------

Please provide the views on the TP.

| Company | Comments |
| --- | --- |
| Apple | OK, but it is better to be discussed during CR review phase |
| Lenovo | Support. |
| QC | Ok. |
| Samsung | We are fine with TP. |
| ZTE | Support. |
| MediaTek | Support. |
| OPPO | Ok |
| vivo | OK with the TP. |
| LG | Support. |
| Futurewei | OK, but should be “the lower ID”. |
| CATT | OK |
| Ericsson | OK with revision suggested by Futurewei. |
| Xiaomi | Ok with Futherwei’s editorial correction. |
| Intel | We are OK with the TP. |
| Huawei, HiSilicon | Support. |
| Nokia, NSB | OK |

### Update 1:

This is an offline agreement. We can use the FutureWei suggestion.

Offline agreement 1.9: Adopt the following TP in Clause 6.1 of TS 38.214,

Clause 6.1 of TS 38.214 --------------Text omitted---------------- For PUSCH scheduled by DCI format 0_0 on a cell, the UE shall transmit PUSCH according to the spatial relation, if applicable, corresponding to the dedicated PUCCH resource with the lowest ID within the active UL BWP of the cell, as described in Clause 9.2.1 of [6, TS 38.213]. If the dedicated PUCCH resource with the lowest ID within the active UL BWP of the cell corresponds to two spatial relations, the UE shall transmit the PUSCH according to the spatial relation with the lower ID. --------------Text omitted----------------

| Company | Comments |
| --- | --- |

## Issue 10: TP in [15] to address beam mapping for CG PUSCH

To correct an editorial error, [15] propose the following TP.

----------------------------------- Start of text proposal of 6.1.2.3 in TS 38.214 ---------------------------------- For PUSCH transmissions with a Type 2 configured grant, when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, the SRS resource set association to (nominal) repetitions follows MappingPattern in ConfiguredGrantConfig as defined in Clause 6.1.2.1 for PUSCH scheduled by DCI format 0_1 and 0_2. For PUSCH transmissions with a Type 1 configured grant, when two SRS resource sets with usage set to ‘codebook’ or ‘noncodebook’ are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, if two SRS resource indicators and two precoding information are provided, the SRS resource set association to (nominal) repetitions is determined as follows. When K = 2, the first and second SRS resource sets are applied to the first and second (nominal) repetitions, respectively. - When K > 2 and cyclicMapping in ConfiguredGrantConfig is enabled, the first and second SRS resource sets are applied to the first and second (nominal) repetitions, respectively, and the same SRS resource set mapping pattern continues to the remaining (nominal) repetitions. - When K > 2 and sequentialMapping in ConfiguredGrantConfig is enabled, first SRS resource set is applied to the first and second (nominal) repetitions, and the second SRS resource set is applied to the third and fourth (nominal) repetitions, and the same SRS resource set mapping pattern continues to the remaining (nominal) repetitions. -------------------------------------------------- End of text proposal ------------------------------------------------

Please provide the views on the TP.

| Company | Comments |
| --- | --- |
| Apple | OK, but it is better to be discussed during CR review phase |
| Lenovo | Support. |
| QC | Ok. |
| Samsung | We are fine with TP. |
| ZTE | Support. |
| MediaTek | Support. |
| OPPO | Ok |
| vivo | OK with the proposal. |
| LG | OK with the proposal. |
| Futurewei | Support |
| CATT | OK |
| Ericsson | OK |
| Xiaomi | Ok. |
| Intel | Support. |
| Huawei, HiSilicon | Support. |
| Nokia, NSB | OK |

### Update 1:

This is an offline agreement.

Offline agreement 1.10: Adopt the following TP in Clause 6.1.2.3 of TS 38.214,

Clause 6.1.2.3 in TS 38.214 --------------Text omitted---------------- For PUSCH transmissions with a Type 2 configured grant, when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, the SRS resource set association to (nominal) repetitions follows MappingPattern in ConfiguredGrantConfig as defined in Clause 6.1.2.1 for PUSCH scheduled by DCI format 0_1 and 0_2. For PUSCH transmissions with a Type 1 configured grant, when two SRS resource sets with usage set to ‘codebook’ or ‘noncodebook’ are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, if two SRS resource indicators and two precoding information are provided, the SRS resource set association to (nominal) repetitions is determined as follows. When K = 2, the first and second SRS resource sets are applied to the first and second (nominal) repetitions, respectively. - When K > 2 and cyclicMapping in ConfiguredGrantConfig is enabled, the first and second SRS resource sets are applied to the first and second (nominal) repetitions, respectively, and the same SRS resource set mapping pattern continues to the remaining (nominal) repetitions. - When K > 2 and sequentialMapping in ConfiguredGrantConfig is enabled, first SRS resource set is applied to the first and second (nominal) repetitions, and the second SRS resource set is applied to the third and fourth (nominal) repetitions, and the same SRS resource set mapping pattern continues to the remaining (nominal) repetitions. --------------Text omitted----------------

## Issue 11: TP in [17] to address second TPC field

Even though 38.213 section 7.2.1 captures below agreements in some form, those are not fully valid due to a condition (text) used in 38.213

Agreement

For per-TRP closed-loop power control,

- When the second TPC field is configured and the indicated PUCCH transmission in DCI formats 1_1/1_2  (or PUSCH transmission in DCI formats 0_1/0_2) is associated with one “closedLoopIndex” value for single TRP transmission, the other TPC field associated with the other “closedLoopIndex” value is unused.

- Note1: Each TPC field is for each closed-loop index value respectively (i.e., 1st /2nd TPC fields correspond to “closedLoopIndex” value = 0 and 1, respectively).

- Note2: When the other TPC field associated with the other “closedLoopIndex” value is unused, the unused TPC field is not applied for any legacy procedures of calculating sum of TPC command values.

Agreement

For mTRP PUCCH (or PUSCH) repetitions schemes,

- When the second TPC field is configured and the indicated PUCCH transmission in DCI formats 1_1/1_2 (or PUSCH transmission in DCI formats 0_1/0_2) is associated with the same “closedLoopIndex” value for mutli-TRP ransmission, the other TPC field associated with the other “closedLoopIndex” value is unused.

- Note: When the other TPC field associated with the other “closedLoopIndex” value is unused, the unused TPC field is not applied for any legacy procedures of calculating sum of TPC command values.

[17] propose the following TP.

### 38.213, Section 7.2.1 --- start of TP --- - If the UE transmits the PUCCH with NPUCCHrepeat>1 repetitions, as described in clause 9.2.6, using a PUCCH resource that includes first and second spatial settings, or first and second sets of power control parameters, and the UE is provided twoPUCCH-PC-AdjustmentStates for the PUCCH resource - If the DCI format includes two TPC command values and the PUCCH resource of the PUCCH transmission is associated with l=0 and l=1, the UE applies the first TPC command value for l=0 and applies the second TPC command value for l=1 - If the DCI format includes two TPC command values and the PUCCH resource of the PUCCH transmission is associated with l=0, the UE applies the first TPC command value for l=0 and ignores the second TPC command value - If the DCI format includes two TPC command values and the PUCCH resource of the PUCCH transmission is associated with l=1, the UE applies the second TPC command value for l=1 and ignores the first TPC command value - If the DCI format includes one TPC command value, the UE applies the TPC command value for all l associated with the PUCCH resource of the PUCCH transmission --- end of TP ---

Please provide the views on above two proposals.

| Company | Comments |
| --- | --- |
| Apple | We failed to see the necessity for the TP. |
| Lenovo | Support. |
| NTT Docomo | Support the TP |
| QC | Support the TP to also cover the sTRP case according to the agreement. |
| Samsung | With red marked parts, that section is only for the mTRP operation. Even though two TPC command values are included in DCI, the DCI can schedule PUCCH resource for sTRP transmission which includes one spatial setting or one set of power control parameters. Therefore, we need to delete the red part to cover the agreement for sTRP PUCCH transmission case. We think the suggested TP is enough without huge change. |
| ZTE | Basically, we agree that this part should capture the above agreements accurately, hence we also think the text is not fully valid. According to intention and description of the two agreement, these aim to specify how to use the two TPC fields when one single CLI in STRP operation and when two same CLIes in MTRP operation, so it is better to reflect the two cases of the two agreements in the specification clearly. Furthermore, the accurate condition should be “the PUCCH resource which activated with one or two spatial relation info/ power control parameter sets”. Hence we suggest: ### 38.213, Section 7.2.1 --- start of TP --- - If the UE transmits the PUCCH with NPUCCHrepeat>1 repetitions, as described in clause 9.2.6, using a PUCCH resource that includes first and second spatial settings, or first and second sets of power control parameters, and the UE is provided twoPUCCH-PC-AdjustmentStates for the PUCCH resource, and the PUCCH resource of the PUCCH transmission is activated with one or two spatial relation info/ power control parameter sets. |
| MediaTek | Support. Also fine with ZTE’s proposal. |
| OPPO | Support |
| vivo | Support. Also fine with ZTE’s proposal. |
| LG | Support |
| Futurewei | Ok with ZTE’s version. |
| CATT | Ok with ZTE’s version. |
| Ericsson | We agree with the TP except one more change may be needed. Note that twoPUCCH-PC-AdjustmentStates is not provided per PUCCH resource. It is provided by higher layer parameter pucch-PowerControl. Hence, the TP needs to be further revised as follows: ### 38.213, Section 7.2.1 --- start of TP --- - If the UE transmits the PUCCH with NPUCCHrepeat>1 repetitions, as described in clause 9.2.6, using a PUCCH resource that includes first and second spatial settings, or first and second sets of power control parameters, and the UE is provided twoPUCCH-PC-AdjustmentStates via higher layer parameter pucch-PowerControl for the PUCCH resource - If the DCI format includes two TPC command values and the PUCCH resource of the PUCCH transmission is associated with l=0 and l=1, the UE applies the first TPC command value for l=0 and applies the second TPC command value for l=1 - If the DCI format includes two TPC command values and the PUCCH resource of the PUCCH transmission is associated with l=0, the UE applies the first TPC command value for l=0 and ignores the second TPC command value - If the DCI format includes two TPC command values and the PUCCH resource of the PUCCH transmission is associated with l=1, the UE applies the second TPC command value for l=1 and ignores the first TPC command value - If the DCI format includes one TPC command value, the UE applies the TPC command value for all l associated with the PUCCH resource of the PUCCH transmission --- end of TP --- |
| Xiaomi | Support, fine with E///’s version. |
| Intel | Support |
| Huawei, HiSilicon | Fine with the TP. |
| Nokia, NSB | Support the TP. |

### Update 1:

Majority of companies are ok with the TP and there are some suggestions,

- ZTE has a suggestion that to mention a condition that “the PUCCH resource which activated with one or two spatial relation info/ power control parameter sets”, and some companies support it. However, it is not necessary as that is generally the case when we delete the suggested text in the original TP.

- E/// highlight that twoPUCCH-PC-AdjustmentStates is not provided er PUCCH resource. This seems also discussed in Issue 12 , and as we can combine the TPs, considered in this discussion.

Offline agreement 1.11: Adopt the following TP in Clause 7.2.1 of TS 38.213,

Clause 7.2.1 of TS 38.213 --------------Text omitted---------------- If the UE transmits the PUCCH with NPUCCHrepeat>1 repetitions, as described in clause 9.2.6, and the UE is provided twoPUCCH-PC-AdjustmentStates via higher layer parameter pucch-PowerControl - If the DCI format includes two TPC command values and the PUCCH resource of the PUCCH transmission is associated with l=0 and l=1, the UE applies the first TPC command value for l=0 and applies the second TPC command value for l=1 - If the DCI format includes two TPC command values and the PUCCH resource of the PUCCH transmission is associated with l=0, the UE applies the first TPC command value for l=0 and ignores the second TPC command value - If the DCI format includes two TPC command values and the PUCCH resource of the PUCCH transmission is associated with l=1, the UE applies the second TPC command value for l=1 and ignores the first TPC command value - If the DCI format includes one TPC command value, the UE applies the TPC command value for all l associated with the PUCCH resource of the PUCCH transmission --------------Text omitted----------------

| Company | Comments |
| --- | --- |
| ZTE | We strongly suggest to add the wording “, and the PUCCH resource of the PUCCH transmission is activated with one or two spatial relation info/ power control parameter sets” back, which could capture the following two agreements clearly as we elaborated in the first round. Agreement For per-TRP closed-loop power control, • When the second TPC field is configured and the indicated PUCCH transmission in DCI formats 1_1/1_2 (or PUSCH transmission in DCI formats 0_1/0_2) is associated with one “closedLoopIndex” value for single TRP transmission, the other TPC field associated with the other “closedLoopIndex” value is unused. • Note1: Each TPC field is for each closed-loop index value respectively (i.e., 1st /2nd TPC fields correspond to “closedLoopIndex” value = 0 and 1, respectively). • Note2: When the other TPC field associated with the other “closedLoopIndex” value is unused, the unused TPC field is not applied for any legacy procedures of calculating sum of TPC command values. Agreement For mTRP PUCCH (or PUSCH) repetitions schemes, - When the second TPC field is configured and the indicated PUCCH transmission in DCI formats 1_1/1_2 (or PUSCH transmission in DCI formats 0_1/0_2) is associated with the same “closedLoopIndex” value for mutli-TRP transmission, the other TPC field associated with the other “closedLoopIndex” value is unused. Note: When the other TPC field associated with the other “closedLoopIndex” value is unused, the unused TPC field is not applied for any legacy procedures of calculating sum of TPC command values.. |
| QC | Support the latest TP from FL. The additions by ZTE seem to be unnecessary since the red part does not add any information. |
| LG | Support the latest TP from FL. |
| Lenovo | Support the latest TP from FL. |
| CATT | Support the latest TP from FL. |
| Moderator | The latest version agreed over email. |

## Issue 12: TP in [18] to address twoPUCCH-PC-AdjustmentStates

As “twoPUCCH-PC-AdjustmentStates” is not configured per PUCCH resource, [18] propose the following TP.

============TP for 38.213 Section 7.2.1 ==================================== --Unchanged part omitted------------------------ - If the UE transmits the PUCCH with NPUCCHrepeat>1 repetitions, as described in clause 9.2.6, using a PUCCH resource that includes first and second spatial settings, or first and second sets of power control parameters, and the UE is provided twoPUCCH-PC-AdjustmentStates - If the DCI format includes two TPC command values and the PUCCH resource of the PUCCH transmission is associated with l=0 and l=1, the UE applies the first TPC command value for l=0 and applies the second TPC command value for l=1 - If the DCI format includes two TPC command values and the PUCCH resource of the PUCCH transmission is associated with l=0, the UE applies the first TPC command value for l=0 and ignores the second TPC command value - If the DCI format includes two TPC command values and the PUCCH resource of the PUCCH transmission is associated with l=1, the UE applies the second TPC command value for l=1 and ignores the first TPC command value - If the DCI format includes one TPC command value, the UE applies the TPC command value for all l associated with the PUCCH resource of the PUCCH transmission ===============================================================

Please provide the views on the TP.

| Company | Comments |
| --- | --- |
| Apple | OK |
| Lenovo | Support. |
| NTT Docomo | Support |
| QC | Support the TP. |
| Samsung | We are fine with TP. |
| ZTE | We agree with FL’s assessment. We suggest to revise this part as follow: ============TP for 38.213 Section 7.2.1 ==================================== --Unchanged part omitted------------------------ - If the UE transmits the PUCCH with NPUCCHrepeat>1 repetitions, as described in clause 9.2.6, using a PUCCH resource that includes first and second spatial settings, or first and second sets of power control parameters, and the UE is provided twoPUCCH-PC-AdjustmentStates for the PUCCH transmission. |
| MediaTek | Support. |
| OPPO | Ok |
| vivo | OK |
| LG | OK |
| Futurewei | Ok |
| CATT | Support |
| Ericsson | Ok. But we could say ‘twoPUCCH-PC-AdjustmentStates is provided by higher layer parameter pucch-PowerControl’ instead: - If the UE transmits the PUCCH with NPUCCHrepeat>1 repetitions, as described in clause 9.2.6, using a PUCCH resource that includes first and second spatial settings, or first and second sets of power control parameters, and the UE is provided twoPUCCH-PC-AdjustmentStates via higher layer parameter pucch-PowerControl |
| Xiaomi | Fine with E///’s version. |
| Intel | Support the TP. |
| Huawei, HiSilicon | Fine with the proposal. |
| Nokia, NSB | Fine with the TP or E/// version. |

### Update (discussion closed)

This has majority support and now considered with Issue 11. Please check the discussion there.

## Issue 13: TP in [18] to address an error in the latest 38.214

Endorsed CR in R1-2112949 is not fully captured in the latest 38.214 specification, [18] propose the following TP (to add back the endorsed text).

============TP for 38.214 Section 6.1.2.3 ==================================== --Unchanged part omitted------------------------ - Number of DM-RS CDM groups, DM-RS ports, SRS resource indication and DM-RS sequence initialization are determined as in Clause 7.3.1.1.2 of [5, TS 38.212], and the antenna port value, the bit value for DM-RS sequence initialization, precoding information and number of layers, SRS resource indicator are provided by antennaPort, dmrs-SeqInitialization, precodingAndNumberOfLayers, and srs-ResourceIndicator respectively; When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, precoding information and number of layers (applicable when higher layer parameter usage in SRS-ResourceSet set to ‘codebook’) associated with the first and second SRS resource set is provided by precodingAndNumberOfLayers and precodingAndNumberOfLayers2, respectively, and SRS resource indicators associated with the first and second SRS resource sets are provided by srs-ResourceIndicator and srs-ResourceIndicator2, respectively. When both srs-ResourceSetToAddModList and srs-ResourceSetToAddModListDCI-0-2 are configured with two SRS resource sets, the two SRS resource sets configured by srs-ResourceSetToAddModList is used to determine the SRS resource indications by srs-ResourceIndicator and srs-ResourceIndicator2. --Unchanged part omitted------------------------

Please provide the views on the TP.

| Company | Comments |
| --- | --- |
| Apple | OK |
| Lenovo | Support. |
| NTT Docomo | Support |
| QC | Support. |
| Samsung | We are fine with this TP. |
| ZTE | Support. |
| MediaTek | Support. |
| OPPO | Support |
| vivo | Support |
| LG | Support |
| Futurewei | Ok |
| CATT | Support |
| Ericsson | ok |
| Xiaomi | Support. |
| Intel | Support. |
| Huawei, HiSilicon | Support. |
| Nokia, NSb | Ok |

### Update 1:

This is an offline agreement.

Offline agreement 1.13: Adopt the following TP in Clause 6.1.2.3 of TS 38.214,

Clause 6.1.2.3 in TS 38.214 --------------Text omitted---------------- - Number of DM-RS CDM groups, DM-RS ports, SRS resource indication and DM-RS sequence initialization are determined as in Clause 7.3.1.1.2 of [5, TS 38.212], and the antenna port value, the bit value for DM-RS sequence initialization, precoding information and number of layers, SRS resource indicator are provided by antennaPort, dmrs-SeqInitialization, precodingAndNumberOfLayers, and srs-ResourceIndicator respectively; When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, precoding information and number of layers (applicable when higher layer parameter usage in SRS-ResourceSet set to ‘codebook’) associated with the first and second SRS resource set is provided by precodingAndNumberOfLayers and precodingAndNumberOfLayers2, respectively, and SRS resource indicators associated with the first and second SRS resource sets are provided by srs-ResourceIndicator and srs-ResourceIndicator2, respectively. When both srs-ResourceSetToAddModList and srs-ResourceSetToAddModListDCI-0-2 are configured with two SRS resource sets, the two SRS resource sets configured by srs-ResourceSetToAddModList is used to determine the SRS resource indications by srs-ResourceIndicator and srs-ResourceIndicator2. --------------Text omitted----------------

| Company | Comments |
| --- | --- |
| ZTE | Thanks for FL’s reminder in issue 14, so we cannot agree with this TP as of now. Please see our comment of issue 14. |
| Moderator | The latest version agreed over email. |

## Issue 14: Interpretation of srs-ResourceIndicator in CG Type 1

In [18], for CG type 1 PUSCH, the following two cases are suggested as valid configurations (which are not yet captured in 38.214)

- Case 1: One SRS resource set is configured in srs-ResourceSetToAddModListDCI-0-2 while there are two SRS resource sets configured in srs-ResourceSetToAddModList

- Case 2: Two SRS resource sets are configured in srs-ResourceSetToAddModListDCI-0-2 while there is only one SRS resource set configured in srs-ResourceSetToAddModList

RAN1 shall define how to interpret srs-ResourceIndicator and srs-ResourceIndicator2 for Case 1 and Case 2 above. Proposal in [18] is used for discussion,

Proposal 1.14: For mTRP Type 1 CG PUSCH,

- If srs-ResourceSetToAddModList is configured with two SRS resource sets, the two SRS resource sets are used to determine the SRS resource indications by srs-ResourceIndicator and srs-ResourceIndicator2.

- Otherwise, if srs-ResourceSetToAddModListDCI-0-2 is configured with two SRS resource sets, the two SRS resource sets are used to determine the SRS resource indications by srs-ResourceIndicator and srs-ResourceIndicator2.

[18] has the following TP,

============TP for 38.214 Section 6.1.2.3 (using TP of Issue 1 as baseline) =========================== --Unchanged part omitted------------------------ - Number of DM-RS CDM groups, DM-RS ports, SRS resource indication and DM-RS sequence initialization are determined as in Clause 7.3.1.1.2 of [5, TS 38.212], and the antenna port value, the bit value for DM-RS sequence initialization, precoding information and number of layers, SRS resource indicator are provided by antennaPort, dmrs-SeqInitialization, precodingAndNumberOfLayers, and srs-ResourceIndicator respectively; When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, precoding information and number of layers (applicable when higher layer parameter usage in SRS-ResourceSet set to ‘codebook’) associated with the first and second SRS resource set is provided by precodingAndNumberOfLayers and precodingAndNumberOfLayers2, respectively, and SRS resource indicators associated with the first and second SRS resource sets are provided by srs-ResourceIndicator and srs-ResourceIndicator2, respectively. - If when two SRS resource sets with usage set to ‘codebook’ or ‘noncodebook’ are configured in srs-ResourceSetToAddModList, the two SRS resource sets are used to determine the SRS resource indications by srs-ResourceIndicator and srs-ResourceIndicator2. - otherwise, the two SRS resource sets with usage set to ‘codebook’ or ‘noncodebook’ configured in srs-ResourceSetToAddModListDCI-0-2 are used to determine the SRS resource indications by srs-ResourceIndicator and srs-ResourceIndicator2. ===============================================================

Please provide the views on the proposal and the TP.

| Company | Comments |
| --- | --- |
| Apple | OK |
| QC | Support. BTW, this TP assumes TP of issue 13 as baseline. |
| Samsung | We are open to discuss. |
| ZTE | Do NOT support. We fail to see the motivation and use case to configure the different SRS resource set number of srs-ResourceIndicator and srs-ResourceIndicator2. Maybe one conclusion is needed for this issue. Hence we suggest: Conclusion 1.14 For MTRP PUSCH repetition, the number of SRS resource sets configured by srs-ResourceIndicator and srs-ResourceIndicator2 should be the same. |
| MediaTek | Support. |
| OPPO | We are fine with the TP |
| vivo | We think an agreement on this issue is needed first. Another option can be added for discussion as follows: Proposal 1.14-a: when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to ‘codebook’, - SRS resource indication by srs-ResourceIndicator field is determined by the first SRS resource set configured by srs-ResourceSetToAddModList; - SRS resource indication by srs-ResourceIndicator2 field is determined by the second SRS resource set configured by srs-ResourceSetToAddModList if any, otherwise by the second SRS resource set configured by srs-ResourceSetToAddModList DCI-0-2, and precodingAndNumberOfLayers and precodingAndNumberOfLayers2 are associated to srs-ResourceIndicator and srs-ResourceIndicator2 respectively. |
| LG | We are open for further discussion. |
| Futurewei | Open for further discussion. |
| CATT | Support. It is aligned with our proposal in R1-2201329. |
| Ericsson | We are open for further discussion. |
| Xiaomi | Support. |
| Intel | OK. |
| Huawei, HiSilicon | We are open to discuss and the motivation of different configurations for the two cases should be clarified. |
| Nokia, NSB | Support |

### Update 1:

Majority of companies are ok with the TP and there are some other views as well.

- TP in Issue 13 is considered in the base line for the TP. As issue 13 is an offline agreement (planned to endorse first), it should not be an issue to consider this on top of that if this get a separate agreement.

- It seems that only ZTE has concern on the TP. ZTE do not think that the TP is needed. They prefer to have conclusion but the conclusion is not fully clear. If the suggestion is to configure two SRS resource sets in  ResourceSetToAddModListDCI-0-2 and ResourceSetToAddModList, it will unnecessarily increase DCI overhead if network only seeking mTRP operation with DCI format 0-2.  Technically, the TP looks ok and suggest ZTE to reconsider.

Proposal 1.14: Adopt the following TP in Clause 6.1.2.3 of TS 38.214, (Note : This TP assumes TP in Issue 13 as baseline, where TP in issue 13 is correcting a copy/paste error in the endorsed 38.214 in R1-2112949).

Clause 6.1.2.3 in TS 38.214 --------------Text omitted---------------- - Number of DM-RS CDM groups, DM-RS ports, SRS resource indication and DM-RS sequence initialization are determined as in Clause 7.3.1.1.2 of [5, TS 38.212], and the antenna port value, the bit value for DM-RS sequence initialization, precoding information and number of layers, SRS resource indicator are provided by antennaPort, dmrs-SeqInitialization, precodingAndNumberOfLayers, and srs-ResourceIndicator respectively; When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, precoding information and number of layers (applicable when higher layer parameter usage in SRS-ResourceSet set to ‘codebook’) associated with the first and second SRS resource set is provided by precodingAndNumberOfLayers and precodingAndNumberOfLayers2, respectively, and SRS resource indicators associated with the first and second SRS resource sets are provided by srs-ResourceIndicator and srs-ResourceIndicator2, respectively. - If when two SRS resource sets with usage set to ‘codebook’ or ‘noncodebook’ are configured in srs-ResourceSetToAddModList, the two SRS resource sets are used to determine the SRS resource indications by srs-ResourceIndicator and srs-ResourceIndicator2. - otherwise, the two SRS resource sets with usage set to ‘codebook’ or ‘noncodebook’ configured in srs-ResourceSetToAddModListDCI-0-2 are used to determine the SRS resource indications by srs-ResourceIndicator and srs-ResourceIndicator2. --------------Text omitted----------------

| Company | Comments |
| --- | --- |
| ZTE | As we elaborated before, can any proponents(s) clarify the motivation and use case to configure the different SRS resource set number of srs-ResourceIndicator and srs-ResourceIndicator2? |
| QC | Support. @ZTE: As we commented in Email, it would be good if the concern can be clarified. “ configure the different SRS resource set number of srs-ResourceIndicator and srs-ResourceIndicator2” is not clear to us. |
| OPPO | Support |
| CATT | Support. We can change “ If when two SRS resource sets with usage set to ‘codebook’ or ‘noncodebook’ are configured in srs-ResourceSetToAddModList ” to “ If when two SRS resource sets with usage set to ‘codebook’ or ‘noncodebook’ are configured in srs-ResourceSetToAddModList” to avoid duplication. |

## Issue 15: TP in [18] to address CG PUSCH

The following agreement is not yet implemented in the existing specifications:

Agreement

When a DCI that includes the new 2-bits DCI field for dynamic switching activates a type 2 CG or schedules a retransmission of a type 1 or type 2 CG, and the CG configuration is RRC-configured with only one set of power control parameters (one ‘p0-PUSCH-Alpha’ and ‘powerControlLoopToUse’):

- The UE expects the new DCI field for dynamic switching is set to “00”, and all PUSCH repetitions are associated with the first SRS resource set.

[18] has the following TP,

============TP for 38.214 Section 6.1.2.3 =========================== --Unchanged part omitted------------------------ For PUSCH transmissions with a Type 1 configured grant, when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, if configuredGrantConfig contains only one srs-ResourceIndicator and precodingAndNumberOfLayers (applicable when higher layer parameter usage in SRS-ResourceSet set to ‘codebook’), PUSCH repetitions are associated only with the first SRS resource set. If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’, and the UE is not provided p0-PUSCH-Alpha2 and powerControlLoopToUse2, for a retransmission of a configured grant Type 1 PUSCH, or for activation or retransmission of a configured grant Type 2 PUSCH, scheduled by a DCI format that includes an SRS resource set indicator field, the UE expects the value of the SRS resource set indicator field to be set to ‘00’, and PUSCH repetitions are associated only with the first SRS resource set. The UE shall not transmit anything on the resources configured by configuredGrantConfig if the higher layers did not deliver a transport block to transmit on the resources allocated for uplink transmission without grant. ===============================================================

Please provide the views on the TP.

| Company | Comments |
| --- | --- |
| Apple | OK |
| Lenovo | Support. |
| QC | Support. The agreement is not implemented yet in current spec. |
| Samsung | We are fine with TP |
| ZTE | Support. |
| MediaTek | Support. |
| OPPO | We are fine with the TP |
| vivo | OK. |
| LG | OK. |
| Futurewei | Support |
| CATT | Support |
| Ericsson | Support |
| Xiaomi | Support |
| Intel | Support the TP. |
| Huawei, HiSilicon | Support. |
| Nokia, NSB | Support |

### Update 1:

This is an offline agreement.

Offline agreement 1.15: Adopt the following TP in Clause 6.1.2.3 of TS 38.214,

Clause 6.1.2.3 in TS 38.214 --------------Text omitted---------------- For PUSCH transmissions with a Type 1 configured grant, when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, if configuredGrantConfig contains only one srs-ResourceIndicator and precodingAndNumberOfLayers (applicable when higher layer parameter usage in SRS-ResourceSet set to ‘codebook’), PUSCH repetitions are associated only with the first SRS resource set. If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’ or ‘nonCodebook’, and the UE is not provided p0-PUSCH-Alpha2 and powerControlLoopToUse2, for a retransmission of a configured grant Type 1 PUSCH, or for activation or retransmission of a configured grant Type 2 PUSCH, scheduled by a DCI format that includes an SRS resource set indicator field, the UE expects the value of the SRS resource set indicator field to be set to ‘00’, and PUSCH repetitions are associated only with the first SRS resource set. The UE shall not transmit anything on the resources configured by configuredGrantConfig if the higher layers did not deliver a transport block to transmit on the resources allocated for uplink transmission without grant. --------------Text omitted----------------

## Issue 16: Power control parameter sets in FR1

As highlighted in [19],  RAN2 agreed to the below:

RAN2#116e-bis agreements:

add a new IE for power control for mTRP FR1 operation and consult on the number of power control sets to be configured.

[060] Introduce the new MAC CE(s) to support PUCCH Power control set update (with power control) for FR1 cases. FFS, detail MAC CE design based on new RRC IE for FR1-dedicated power control set.

Also, there is a question from RAN2  on the number of power control parameter sets for PUCCH in case of FR1. To agree on number of power control parameter sets and adding relevant spec text to FR1 PUCCH repetition mode, the following proposal and TP can be used for discussion,

Proposal 1.16: Up to 8 sets of power control parameters are supported in FR1 for Rel-17 multi-TRP PUCCH.

----------------Start of TP1 for 38.213 V17.0.0---------------- 7.2.1 UE behaviour If a UE transmits a PUCCH on active UL BWP b of carrier f in the primary cell c using PUCCH power control adjustment state with index l, the UE determines the PUCCH transmission power PPUCCH,b,f,c(i,qu,qd,l) in PUCCH transmission occasion i as [dBm] where - PCMAX,f,c(i) is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [8-3, TS38.101-3] for carrier f of primary cell c in PUCCH transmission occasion i - PO_PUCCH,b,f,c(qu) is a parameter composed of the sum of a component PO_NOMINAL,PUCCH, provided by p0-nominal, or PO_NOMINAL,PUCCH=0 dBm if p0-nominal is not provided, for carrier f of primary cell c and, if provided, a component PO_UE_PUCCH(qu) provided by p0-PUCCH-Value in P0-PUCCH for active UL BWP b of carrier f of primary cell c, where 0≤qu<Qu. Qu is a size for a set of PO_UE_PUCCH values provided by maxNrofPUCCH-P0-PerSet. The set of PO_UE_PUCCH values is provided by p0-Set. If p0-Set is not provided to the UE, PO_PUCCH,b,f,cqu=0, 0≤qu<Qu - If the UE is provided PUCCH-SpatialRelationInfo, the UE obtains a mapping, by an index provided by p0-PUCCH-Id, between a set of pucch-SpatialRelationInfoId values and a set of p0-PUCCH-Value values. If the UE is provided more than one values for pucch-SpatialRelationInfoId and the UE receives an activation command [11, TS 38.321] indicating a value of pucch-SpatialRelationInfoId, the UE determines the p0-PUCCH-Value value through the link to a corresponding p0-PUCCH-Id index. The UE applies the activation command in the first slot that is after slot k+3⋅Nslotsubframe,μ where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the activation command and μ is the SCS configuration for the PUCCH - If the UE is not provided PUCCH-SpatialRelationInfo and is provided with a configuration of multiple sets of power control parameters for FR1, and the UE receives an activation command [11, TS 38.321] indicating one or two of the multiple sets of power control parameters, the UE determines the p0-PUCCH-Value value according to the corresponding one or two sets of power control parameters. The UE applies the activation command in the first slot that is after slot k+3⋅Nslotsubframe,μ where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the activation command and μ is the SCS configuration for the PUCCH - If the UE is not provided PUCCH-SpatialRelationInfo and a configuration of multiple sets of power control parameters for FR1, the UE obtains the p0-PUCCH-Value value from the P0-PUCCH with p0-PUCCH-Id value equal to the minimum p0-PUCCH-Id value in p0-Set - MRB,b,f,cPUCCH(i) is a bandwidth of the PUCCH resource assignment expressed in number of resource blocks for PUCCH transmission occasion i on active UL BWP b of carrier f of primary cell c and μ is a SCS configuration defined in [4, TS 38.211] - PLb,f,c(qd) is a downlink pathloss estimate in dB calculated by the UE using RS resource index qd as described in clause 7.1.1 for the active DL BWP b of carrier f of the primary cell c as described in clause 12 - If the UE is not provided pathlossReferenceRSs or before the UE is provided dedicated higher layer parameters, the UE calculates PLb,f,c(qd) using a RS resource obtained from an SS/PBCH block with same SS/PBCH block index as the one the UE uses to obtain MIB - If the UE is provided a number of RS resource indexes, the UE calculates PLb,f,c(qd) using RS resource with index qd, where 0≤qd<Qd. Qd is a size for a set of RS resources provided by maxNrofPUCCH-PathlossReferenceRSs. The set of RS resources is provided by pathlossReferenceRSs. The set of RS resources can include one or both of a set of SS/PBCH block indexes, each provided by ssb-Index in PUCCH-PathlossReferenceRS when a value of a corresponding pucch-PathlossReferenceRS-Id maps to a SS/PBCH block index, and a set of CSI-RS resource indexes, each provided by csi-RS-Index when a value of a corresponding pucch-PathlossReferenceRS-Id maps to a CSI-RS resource index. The UE identifies a RS resource in the set of RS resources to correspond either to a SS/PBCH block index or to a CSI-RS resource index as provided by pucch-PathlossReferenceRS-Id in PUCCH-PathlossReferenceRS - If the UE is provided pathlossReferenceRSs and PUCCH-SpatialRelationInfo, the UE obtains a mapping, by indexes provided by corresponding values of pucch-PathlossReferenceRS-Id, between a set of pucch-SpatialRelationInfoId values and a set of referenceSignal values provided by PUCCH-PathlossReferenceRS. If the UE is provided more than one values for pucch-SpatialRelationInfoId and the UE receives an activation command [11, TS 38.321] indicating a value of pucch-SpatialRelationInfoId, the UE determines the referenceSignal value in PUCCH-PathlossReferenceRS through the link to a corresponding pucch-PathlossReferenceRS-Id index. The UE applies the activation command in the first slot that is after slot k+3⋅Nslotsubframe,μ where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the activation command and μ is the SCS configuration for the PUCCH - If PUCCH-SpatialRelationInfo includes servingCellId indicating a serving cell, the UE receives the RS for resource index qd on the active DL BWP of the serving cell - If the UE is provided pathlossReferenceRSs and a configuration of multiple sets of power control parameters for FR1 and is not provided PUCCH-SpatialRelationInfo, and the UE receives an activation command [11, TS 38.321] indicating one or two of the multiple sets of power control parameters, the UE determines the corresponding referenceSignal value in PUCCH-PathlossReferenceRS indicated in the one or two sets of power control parameters - If the UE is provided pathlossReferenceRSs and is not provided PUCCH-SpatialRelationInfo and a configuration of multiple sets of power control parameters for FR1, the UE obtains the referenceSignal value in PUCCH-PathlossReferenceRS from the pucch-PathlossReferenceRS-Id with index 0 in PUCCH-PathlossReferenceRS where the RS resource is either on the primary cell or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking - If the UE - is not provided pathlossReferenceRSs, and - is not provided PUCCH-SpatialRelationInfo, and - is not provided a configuration of multiple power control parameter sets for FR1 - is provided enableDefaultBeamPL-ForPUCCH, and - is not provided coresetPoolIndex value of 1 for any CORESET, or is provided coresetPoolIndex value of 1 for all CORESETs, in ControlResourceSet and no codepoint of a TCI field, if any, in a DCI format of any search space set maps to two TCI states [5, TS 38.212] the UE determines a RS resource index qd providing a periodic RS resource configured with qcl-Type set to 'typeD' in the TCI state or the QCL assumption of a CORESET with the lowest index in the active DL BWP of the primary cell. If the CORESET has two activated TCI states, as described in clause 10.1, the UE determines the RS resource index qd based on the first activated TCI state. For a PUCCH transmission over multiple slots, a same qd applies to the PUCCH transmission in each of the multiple slots. ---unchanged text omitted --- - For the PUCCH power control adjustment state gb,f,ci,l for active UL BWP b of carrier f of primary cell c and PUCCH transmission occasion i - δPUCCH,b,f,c(i,l) is a TPC command value included in a DCI format associated with the PUCCH transmission for active UL BWP b of carrier f of the primary cell c that the UE detects for PUCCH transmission occasion i, or is jointly coded with other TPC commands in a DCI format 2_2 with CRC scrambled by TPC-PUCCH-RNTI [5, TS 38.212], as described in clause 11.3 - l∈0,1 if the UE is provided twoPUCCH-PC-AdjustmentStates and PUCCH-SpatialRelationInfo or a configuration of multiple sets of power control parameters, and l=0 if the UE is not provided twoPUCCH-PC-AdjustmentStates, or PUCCH-SpatialRelationInfo, and a configuration of multiple sets of power control parameters, ---unchanged text omitted --- - gb,f,ci,l=gb,f,ci-i0,l+m=0CDi-1δPUCCH,b,f,c(m,l) gb,f,ci,l=gb,f,ci-i0,l+m=0CCi-1δPUCCH,b,f,c(m,l) is the current PUCCH power control adjustment state l for active UL BWP b of carrier f of primary cell c and PUCCH transmission occasion i, where - The δPUCCH,b,f,c values are given in Table 7.1.2-1 - m=0CDi-1δPUCCH,b,f,c(m,l) m=0CCi-1δPUCCH,b,f,c(m,l) is a sum of TPC command values in a set Ci of TPC command values with cardinality CCi that the UE receives between KPUCCHi-i0-1 symbols before PUCCH transmission occasion i-i0 and symbols before PUCCH transmission occasion i on active UL BWP b of carrier f of primary cell c for PUCCH power control adjustment state, where i0>0 is the smallest integer for which KPUCCHi-i0 symbols before PUCCH transmission occasion i-i0 is earlier than KPUCCHi symbols before PUCCH transmission occasion i - If the PUCCH transmission is in response to a detection by the UE of a DCI format, KPUCCHi is a number of symbols for active UL BWP b of carrier f of primary cell c after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUCCH transmission - If the PUCCH transmission is not in response to a detection by the UE of a DCI format, KPUCCHi is a number of KPUCCH,min symbols equal to the product of a number of symbols per slot, Nsymbslot, and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP b of carrier f of primary cell c - If the UE has reached maximum power for active UL BWP b of carrier f of primary cell c at PUCCH transmission occasion i-i0 and m=0CDi-1δPUCCH,b,f,c(m,l)m=0CCi-1δPUCCH,b,f,c(m,l)≥0, then gb,f,ci,l=gb,f,ci-i0,l - If UE has reached minimum power for active UL BWP b of carrier f of primary cell c at PUCCH transmission occasion i-i0 and m=0CDi-1δPUCCH,b,f,cm,l m=0CCi-1δPUCCH,b,f,c(m,l)≤0, then gb,f,ci,l=gb,f,ci-i0,l - If a configuration of a PO_PUCCH,b,f,cqu value for a corresponding PUCCH power control adjustment state l for active UL BWP b of carrier f of primary cell c is provided by higher layers, - gb,f,ck,l=0, k=0,1,…,i If the UE is provided PUCCH-SpatialRelationInfo, the UE determines the value of l from the value of qu based on a pucch-SpatialRelationInfoId value associated with the p0-PUCCH-Id value corresponding to qu and with the closedLoopIndex value corresponding to l; Else if the UE is provided with a configuration of multiple sets of power control parameters for FR1, and if the UE receives an activation command for the PUCCH resource indicating one or two of the multiple sets of power control parameters, the UE determines the value of l based on the indicated the first or second sets of power control parameters where the closedLoopIndex value contained in the indicated set of power control parameters corresponds to l otherwise, l=0 - Else, ---unchanged text omitted --- ---------------------End of TP1 for 38.213 -------------------

Please provide the views on the proposal and the TP.

| Company | Comments |
| --- | --- |
| Apple | We think 2 sets should be sufficient. We failed to see necessity for other changes. |
| Lenovo | Support the proposal in principle, however, the maximum number of power control parameter set can be discussed in UE feature. Support the TP. |
| NTT Docomo | Support |
| QC | Support the proposal and the TP in principle. |
| Samsung | We are fine with this update. Additionally, RAN2 made new RRC parameter for the set of power control parameters as ‘pucch-PowerControlSet’. Therefore, with this new RRC parameter, we can update the specification. |
| ZTE | Do Not support. Given that it has been agreed that up to two sets of power control parameters are used for one PUCCH resource when MTRP PUCCH schemes in FR1 and PUCCH power control can be configured as UE-specific as in Rel-15/16, we think the maximum number should be 2. |
| MediaTek | Support in principle. |
| OPPO | Some modifications suggested as below to make it clearer: - If the UE is not provided PUCCH-SpatialRelationInfo and is not provided with a configuration of multiple sets of power control parameters for FR1 - If the UE is provided pathlossReferenceRSs and is not provided PUCCH-SpatialRelationInfo and is not provided with a configuration of multiple sets of power control parameters for FR1, Regarding to the maximum number, we tends to agree with Apple and ZTE |
| vivo | Support in principle. |
| LG | We are open for maximum number of configurable PC sets for FR1. We are agree the TP in principle but it can be further revised after discussion on the proposal. |
| Futurewei | Generally ok with the principle of the TP. The maximum number can be discussed in UE feature. |
| CATT | Support |
| Ericsson | Support the TP. @ZTE, note that RAN2 is currently discussing MAC CE design for indicating one or two power control sets per PUCCH resource. If maximum number of PUCCH power control sets is 2, then there is no need for this MAC CE, right? In FR1, there could be a cluster of TRPs and two TRPs may be selected for PUCCH mTRP reception per resource. Then, we need more than 2 PUCCH power control sets. The maximum number of PUCCH resource sets will be agreed as part of LS response to RAN2 this meeting. So we could come back to this proposal once we have converged on the LS response to RAN2. |
| Xiaomi | Support in principle. |
| Intel | We are open to discuss the maximum number of power control parameter sets and the TP. |
| Huawei, HiSilicon | To provide more flexibility for FR1 mTRP (same level of flexibility as that for FR2), we prefer to discuss this issue. |
| Nokia, NSB | Support the proposal and TP. |

### Update 1:

Good alignment among majority of companies, but there are some other views as well

- Majority of companies are ok with the direction of the proposal and the TP with minimal changes.

- Number of power control parameter sets creating some debate where ZTE and Apple think that 2 power control parameter sets are enough. But, RAN1 agreed a new MAC-CE to select the power control parameter sets and having two as the maximum does not fit our older discussions as highlighted by E///.

- Oppo has modification to the TP, and makes sense to include.

- RAN2 RRC LS reply also require us to agree on the exact max number of power control parameters. As it may not easy to agree in the LS discussion, we shall quickly converge on the exact number in this discussion.

Proposal 1.16-A: Up to 8 sets of power control parameters are supported in FR1 for Rel-17 multi-TRP PUCCH.

Proposal 1.16-A: Adopt the following TP in Clause 7.2.1 of TS 38.213,

Clause 7.2.1 in TS 38.213 --------------Text omitted---------------- If a UE transmits a PUCCH on active UL BWP b of carrier f in the primary cell c using PUCCH power control adjustment state with index l, the UE determines the PUCCH transmission power PPUCCH,b,f,c(i,qu,qd,l) in PUCCH transmission occasion i as [dBm] where - PCMAX,f,c(i) is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [8-3, TS38.101-3] for carrier f of primary cell c in PUCCH transmission occasion i - PO_PUCCH,b,f,c(qu) is a parameter composed of the sum of a component PO_NOMINAL,PUCCH, provided by p0-nominal, or PO_NOMINAL,PUCCH=0 dBm if p0-nominal is not provided, for carrier f of primary cell c and, if provided, a component PO_UE_PUCCH(qu) provided by p0-PUCCH-Value in P0-PUCCH for active UL BWP b of carrier f of primary cell c, where 0≤qu<Qu. Qu is a size for a set of PO_UE_PUCCH values provided by maxNrofPUCCH-P0-PerSet. The set of PO_UE_PUCCH values is provided by p0-Set. If p0-Set is not provided to the UE, PO_PUCCH,b,f,cqu=0, 0≤qu<Qu - If the UE is provided PUCCH-SpatialRelationInfo, the UE obtains a mapping, by an index provided by p0-PUCCH-Id, between a set of pucch-SpatialRelationInfoId values and a set of p0-PUCCH-Value values. If the UE is provided more than one values for pucch-SpatialRelationInfoId and the UE receives an activation command [11, TS 38.321] indicating a value of pucch-SpatialRelationInfoId, the UE determines the p0-PUCCH-Value value through the link to a corresponding p0-PUCCH-Id index. The UE applies the activation command in the first slot that is after slot k+3⋅Nslotsubframe,μ where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the activation command and μ is the SCS configuration for the PUCCH - If the UE is not provided PUCCH-SpatialRelationInfo and is provided with a configuration of multiple sets of power control parameters for FR1, and the UE receives an activation command [11, TS 38.321] indicating one or two of the multiple sets of power control parameters, the UE determines the p0-PUCCH-Value value according to the corresponding one or two sets of power control parameters. The UE applies the activation command in the first slot that is after slot k+3⋅Nslotsubframe,μ where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the activation command and μ is the SCS configuration for the PUCCH - If the UE is not provided PUCCH-SpatialRelationInfo and a configuration of multiple sets of power control parameters for FR1, the UE obtains the p0-PUCCH-Value value from the P0-PUCCH with p0-PUCCH-Id value equal to the minimum p0-PUCCH-Id value in p0-Set - MRB,b,f,cPUCCH(i) is a bandwidth of the PUCCH resource assignment expressed in number of resource blocks for PUCCH transmission occasion i on active UL BWP b of carrier f of primary cell c and μ is a SCS configuration defined in [4, TS 38.211] - PLb,f,c(qd) is a downlink pathloss estimate in dB calculated by the UE using RS resource index qd as described in clause 7.1.1 for the active DL BWP b of carrier f of the primary cell c as described in clause 12 - If the UE is not provided pathlossReferenceRSs or before the UE is provided dedicated higher layer parameters, the UE calculates PLb,f,c(qd) using a RS resource obtained from an SS/PBCH block with same SS/PBCH block index as the one the UE uses to obtain MIB - If the UE is provided a number of RS resource indexes, the UE calculates PLb,f,c(qd) using RS resource with index qd, where 0≤qd<Qd. Qd is a size for a set of RS resources provided by maxNrofPUCCH-PathlossReferenceRSs. The set of RS resources is provided by pathlossReferenceRSs. The set of RS resources can include one or both of a set of SS/PBCH block indexes, each provided by ssb-Index in PUCCH-PathlossReferenceRS when a value of a corresponding pucch-PathlossReferenceRS-Id maps to a SS/PBCH block index, and a set of CSI-RS resource indexes, each provided by csi-RS-Index when a value of a corresponding pucch-PathlossReferenceRS-Id maps to a CSI-RS resource index. The UE identifies a RS resource in the set of RS resources to correspond either to a SS/PBCH block index or to a CSI-RS resource index as provided by pucch-PathlossReferenceRS-Id in PUCCH-PathlossReferenceRS - If the UE is provided pathlossReferenceRSs and PUCCH-SpatialRelationInfo, the UE obtains a mapping, by indexes provided by corresponding values of pucch-PathlossReferenceRS-Id, between a set of pucch-SpatialRelationInfoId values and a set of referenceSignal values provided by PUCCH-PathlossReferenceRS. If the UE is provided more than one values for pucch-SpatialRelationInfoId and the UE receives an activation command [11, TS 38.321] indicating a value of pucch-SpatialRelationInfoId, the UE determines the referenceSignal value in PUCCH-PathlossReferenceRS through the link to a corresponding pucch-PathlossReferenceRS-Id index. The UE applies the activation command in the first slot that is after slot k+3⋅Nslotsubframe,μ where k is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the activation command and μ is the SCS configuration for the PUCCH - If PUCCH-SpatialRelationInfo includes servingCellId indicating a serving cell, the UE receives the RS for resource index qd on the active DL BWP of the serving cell - If the UE is provided pathlossReferenceRSs and is not provided with a configuration of multiple sets of power control parameters for FR1 and is not provided PUCCH-SpatialRelationInfo, and the UE receives an activation command [11, TS 38.321] indicating one or two of the multiple sets of power control parameters, the UE determines the corresponding referenceSignal value in PUCCH-PathlossReferenceRS indicated in the one or two sets of power control parameters - If the UE is provided pathlossReferenceRSs and is not provided PUCCH-SpatialRelationInfo and a configuration of multiple sets of power control parameters for FR1, the UE obtains the referenceSignal value in PUCCH-PathlossReferenceRS from the pucch-PathlossReferenceRS-Id with index 0 in PUCCH-PathlossReferenceRS where the RS resource is either on the primary cell or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking - If the UE - is not provided pathlossReferenceRSs, and - is not provided PUCCH-SpatialRelationInfo, and - is not provided a configuration of multiple power control parameter sets for FR1 - is provided enableDefaultBeamPL-ForPUCCH, and - is not provided coresetPoolIndex value of 1 for any CORESET, or is provided coresetPoolIndex value of 1 for all CORESETs, in ControlResourceSet and no codepoint of a TCI field, if any, in a DCI format of any search space set maps to two TCI states [5, TS 38.212] the UE determines a RS resource index qd providing a periodic RS resource configured with qcl-Type set to 'typeD' in the TCI state or the QCL assumption of a CORESET with the lowest index in the active DL BWP of the primary cell. If the CORESET has two activated TCI states, as described in clause 10.1, the UE determines the RS resource index qd based on the first activated TCI state. For a PUCCH transmission over multiple slots, a same qd applies to the PUCCH transmission in each of the multiple slots. ---unchanged text omitted --- - For the PUCCH power control adjustment state gb,f,ci,l for active UL BWP b of carrier f of primary cell c and PUCCH transmission occasion i - δPUCCH,b,f,c(i,l) is a TPC command value included in a DCI format associated with the PUCCH transmission for active UL BWP b of carrier f of the primary cell c that the UE detects for PUCCH transmission occasion i, or is jointly coded with other TPC commands in a DCI format 2_2 with CRC scrambled by TPC-PUCCH-RNTI [5, TS 38.212], as described in clause 11.3 - l∈0,1 if the UE is provided twoPUCCH-PC-AdjustmentStates and PUCCH-SpatialRelationInfo or a configuration of multiple sets of power control parameters, and l=0 if the UE is not provided twoPUCCH-PC-AdjustmentStates, or PUCCH-SpatialRelationInfo, and a configuration of multiple sets of power control parameters, ---unchanged text omitted --- - gb,f,ci,l=gb,f,ci-i0,l+m=0CDi-1δPUCCH,b,f,c(m,l) gb,f,ci,l=gb,f,ci-i0,l+m=0CCi-1δPUCCH,b,f,c(m,l) is the current PUCCH power control adjustment state l for active UL BWP b of carrier f of primary cell c and PUCCH transmission occasion i, where - The δPUCCH,b,f,c values are given in Table 7.1.2-1 - m=0CDi-1δPUCCH,b,f,c(m,l) m=0CCi-1δPUCCH,b,f,c(m,l) is a sum of TPC command values in a set Ci of TPC command values with cardinality CCi that the UE receives between KPUCCHi-i0-1 symbols before PUCCH transmission occasion i-i0 and symbols before PUCCH transmission occasion i on active UL BWP b of carrier f of primary cell c for PUCCH power control adjustment state, where i0>0 is the smallest integer for which KPUCCHi-i0 symbols before PUCCH transmission occasion i-i0 is earlier than KPUCCHi symbols before PUCCH transmission occasion i - If the PUCCH transmission is in response to a detection by the UE of a DCI format, KPUCCHi is a number of symbols for active UL BWP b of carrier f of primary cell c after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUCCH transmission - If the PUCCH transmission is not in response to a detection by the UE of a DCI format, KPUCCHi is a number of KPUCCH,min symbols equal to the product of a number of symbols per slot, Nsymbslot, and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP b of carrier f of primary cell c - If the UE has reached maximum power for active UL BWP b of carrier f of primary cell c at PUCCH transmission occasion i-i0 and m=0CDi-1δPUCCH,b,f,c(m,l)m=0CCi-1δPUCCH,b,f,c(m,l)≥0, then gb,f,ci,l=gb,f,ci-i0,l - If UE has reached minimum power for active UL BWP b of carrier f of primary cell c at PUCCH transmission occasion i-i0 and m=0CDi-1δPUCCH,b,f,cm,l m=0CCi-1δPUCCH,b,f,c(m,l)≤0, then gb,f,ci,l=gb,f,ci-i0,l - If a configuration of a PO_PUCCH,b,f,cqu value for a corresponding PUCCH power control adjustment state l for active UL BWP b of carrier f of primary cell c is provided by higher layers, - gb,f,ck,l=0, k=0,1,…,i If the UE is provided PUCCH-SpatialRelationInfo, the UE determines the value of l from the value of qu based on a pucch-SpatialRelationInfoId value associated with the p0-PUCCH-Id value corresponding to qu and with the closedLoopIndex value corresponding to l; Else if the UE is provided with is not provided with a configuration of multiple sets of power control parameters for FR1, and if the UE receives an activation command for the PUCCH resource indicating one or two of the multiple sets of power control parameters, the UE determines the value of l based on the indicated the first or second sets of power control parameters where the closedLoopIndex value contained in the indicated set of power control parameters corresponds to l otherwise, l=0 - Else, --------------Text omitted----------------

| Company | Comments |
| --- | --- |
| ZTE | Thanks for E///’s further explanation, we support proposal 1.16-A and this TP now. |
| QC | In the updated TP, it is not clear to us why we have “ is not provided with a configuration of multiple sets of power control parameters for FR1” in multiple instances. Should it be if UE is provided with a configuration of …? For the first proposal, we suggest the following addition. Proposal 1.16-A: Up to 8 sets of power control parameters are supported in FR1 for Rel-17 multi-TRP PUCCH. - The number of sets of power control parameters are indicated by UE capability for FR1 |
| Lenovo | Agree with QC’s comments, we also have the concern about “ is not provided with a configuration of multiple sets of power control parameters for FR1” in multiple instances. And we support the updated proposal 1.16-A from QC. |
| OPPO | The “is no provided with” seems be put in wrong places. I copied our suggestion as below: - If the UE is not provided PUCCH-SpatialRelationInfo and is not provided with a configuration of multiple sets of power control parameters for FR1 - If the UE is provided pathlossReferenceRSs and is not provided PUCCH-SpatialRelationInfo and is not provided with a configuration of multiple sets of power control parameters for FR1, Reply to QC/Lenovo: “is not provided with ..” is just used to differentiate configurations/cases. Let’s take the part of PO_PUCCH,b,f,c(qu) for example - If the UE is provided PUCCH-SpatialRelationInfo, … (Case 1) - If the UE is not provided PUCCH-SpatialRelationInfo and is provided with a configuration of multiple sets of power control parameters for FR1, …. (Case 2) - If the UE is not provided PUCCH-SpatialRelationInfo and is not provided with a configuration of multiple sets of power control parameters for FR1, (case 3) For our side, we are open to any better wording. |
| CATT | Support Mod’s proposal 1.16-A and ok with the TP. How to change “configuration of multiple sets of power control parameters” can wait until the RRC parameter for the configuration in the endorsed TS38.331 is available. |

## Issue 17: M-TRP PUCCH operation for all UCI types

mTRP PUCCH repetition is supported when HARQ is carried in the PUCCH.

Conclusion

For Multi-TRP PUCCH Scheme 1/3 at least containing HARQ ACK, supporting dynamic switching between multi-TRP PUCCH scheme and single-TRP PUCCH transmission is not restricted, and can be done by associating,

- a PUCCH resource activated with one or two spatial-relation-info and PRI bit-field indicating a PUCCH resource,

- or a PUCCH resource with one or two power control parameter sets and PRI bit-field indicating a PUCCH resource

FFS: Support of dynamic switching for Scheme 2 (if the schemes supported)

As highlighted in [21], 38.213 text capturing the M-TRP PUCCH repetition operation doesn’t seem to restrict such operation to a specific UCI type(s). Clarification on whether m-TRP PUCCH operation also applicable to PUCCH carrying SPS HARQ-ACK (without a corresponding PDCCH), SR, and/or CSI may be needed.

Proposal 1.17: M-TRP PUCCH repetition (with schemes 1 and 3) operation is not applicable to PUCCH carrying SPS HARQ-ACK (without a corresponding PDCCH), SR, and/or CSI.

Please provide the views on the proposal.

| Company | Comments |
| --- | --- |
| Apple | We do not think it is needed, but we are open to discuss. |
| Lenovo | Not support. The M-TRP PUCCH repetition does not have the restriction according to R17 agreements. |
| NTT Docomo | Do not support In our understanding, M-TRP PUCCH repetition is also applicable to PUCCH carrying SPS HARQ-ACK, SR, CSI. The intention of above conclusion is dynamic switching is supported at least for HARQ ACK. |
| QC | We do not see the need for adding this restriction. The conclusion above is related to dynamic switching between mTRP and sTRP which can be achieved by PRI in DCI. It does not say that mTRP schemes should not be applicable to other UCIs. |
| Samsung | We are open to discuss. Is there any technical reason to restrict mTRP operation to a certain UCI type(s)? |
| ZTE | We are open to further discuss. |
| MediaTek | Open to discuss. So far, we do not see a need to impose such restriction. |
| OPPO | Do not support. We have similar question as Samsung. |
| vivo | We fail to see the need for the restriction. |
| LG | We can apply MTRP PUCCH repetition regardless of UCI types if two spatial relation or two PC sets are configured in the PUCCH resource. There is no need to restrict UCI types for MTRP PUCCH repetition. |
| Futurewei | Open for discussion, but leaning towards not to impose the restriction. |
| CATT | We have similar view that the dynamic switching mechanism should be applied to PUCCH carrying HARQ/ACK, SR, and/or CSI. |
| Ericsson | These restrictions may not be needed. |
| Xiaomi | Seems no need to restrict UCI type for mTRP operation, not sure about the motivation. |
| Intel | We do not support this proposal. |
| Huawei, HiSilicon | We fail to see the reason to have these restrictions. |
| Nokia, NSB | We would be fine with the majority view. |

### Update (discussion closed)

Majority of companies think that the discussion is not needed.

## Issue 18: S-TRP PUCCH operation

As highlighted in [21], there can be cases where a PUCCH resource is activated with two spatial relation info / power control parameter sets, while the S-TRP PUCCH operation is applicable (e.g. if the corresponding number of PUCCH repetitions is 1, or if M-TRP operation is finally not supported for some UCI types). [21] discuss two ways to handle such cases,

- Alt.1: If a PUCCH resource is associated/activated with two spatial relation info / power control parameter sets (via MAC CE), and the S-TRP PUCCH operation using this PUCCH resource is applicable (e.g. if the corresponding number of PUCCH repetitions is 1, or if M-TRP operation is finally not supported for some UCI types), the first indicated spatial relation info / power control parameter set (via MAC CE) could be simply selected and used.

- Alt.2: The UE doesn’t expect a PUCCH resource to be associated/activated with two spatial relation info / power control parameter sets (via MAC CE) and to have the S-TRP PUCCH operation using this PUCCH resource to be applicable, i.e., this case is treated as an error case.

Proposal 1.18: For the case where a PUCCH resource is associated/activated with two spatial relation info / power control parameter sets (via MAC CE), and S-TRP PUCCH operation using this PUCCH resource is applicable (e.g. if the corresponding number of PUCCH repetitions is 1, or if M-TRP operation is not supported for some UCI types), down-select between the following alternatives:

- Alt.1: The case is allowed and the first indicated spatial relation info / power control parameter set (via MAC CE) is selected and used.

- Alt.2: This is treated as an error case.

Please provide your views on the below proposal,

| Company | Comments |
| --- | --- |
| Apple | With regard to dynamic number of repetition indication, it seems Alt1 is better. |
| Lenovo | Support Alt.2 |
| NTT Docomo | we support Alt.2 |
| QC | We prefer Alt2. |
| Samsung | Support Alt.2 |
| ZTE | Support Alt.2, which can be avoid by gNB implement. If RRC configure that the PUCCH repetition number is 1 or MTRP operation is not supported, why MAC CE will activate the PUCCH resource with two spatial relation info/ power control parameter sets? |
| MediaTek | Support Alt.2. |
| OPPO | Alt.2 |
| vivo | Fine with Alt2. We also think this is somewhat similar to PUSCH repetition as given in Issue 21 summarized by FL. |
| LG | - Regarding UCI types, issue 17 needs to be discussed first. - If repetition number is dynamically indicated as 1, UE can freely select one of two spatial relation infos considering UL channel quality, e.g., depending on PLRS. As a result, channel selection gain still can be achieved for reliability enhancement. Therefore, we don’t need to fix it as first spatial relation info. We suggest the following revision. - Alt.1: The case is allowed and the first one of indicated spatial relation infos / power control parameter sets (via MAC CE) is selected and used. |
| Futurewei | Tend to agree with Apple |
| CATT | Support Alt 2. |
| Ericsson | We prefer Alt 1. |
| Xiaomi | We prefer Alt.2. |
| Intel | We do not support this proposal. |
| Huawei, HiSilicon | We support Alt.2. |
| Nokia, NSB | Alt.1 would provide better flexibility. |

### Update (discussion closed)

Majority of companies think that the Alt.2 can be assumed. FL do not think any conclusion is needed for the error case.

## Remaining issues (19-32)

Please provide your inputs (mainly the proponents) regarding issues 19-32 mentioned in Section 1. Please do not copy the TPs in your comments (it is easier to copy the TP in Section 1.20 if you have a suggestion).

| Issue 19 | LG: We support the TP. Agreed default beam operation should be applied for STRP case as well. Mod: need further inputs to highlight the issue. |
| --- | --- |
| Issue 20 | vivo: Let’s try to further describe Issue 20 again. Firstly, the bullet level is wrong for the highlighted part in yellow; secondly, a separate bullet is required for the highlighted part in blue. ------------------------------------------------------------------------------------------------------------------------------------------ If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', is provided SRI-PUSCH-PowerControl, and a DCI format scheduling the PUSCH transmission includes two SRI fields, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for - the first SRI field value and the l values provided by sri-PUSCH-ClosedLoopIndex, and determines the l value that is mapped to the first SRI field value corresponding to the first SRS resource set with usage set to 'nonCodebook', and - the value, associated with the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value, and the l value(s) provided by sri-PUSCH-ClosedLoopIndex, and determines the l value that is mapped to the value corresponding to the second SRS resource set with usage set to 'nonCodebook'- If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', is provided SRI-PUSCH-PowerControl, and a DCI format scheduling the PUSCH transmission does not include an SRI field ============================== The above text should be =========================== Text proposal: - If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', is provided SRI-PUSCH-PowerControl, and a DCI format scheduling the PUSCH transmission includes two SRI fields, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for - the first SRI field value and the l values provided by sri-PUSCH-ClosedLoopIndex, and determines the l value that is mapped to the first SRI field value corresponding to the first SRS resource set with usage set to 'nonCodebook', and - the value, associated with the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value, and the l value(s) provided by sri-PUSCH-ClosedLoopIndex, and determines the l value that is mapped to the value corresponding to the second SRS resource set with usage set to 'nonCodebook' - If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', is provided SRI-PUSCH-PowerControl, and a DCI format scheduling the PUSCH transmission does not include an SRI field - If the UE is provided twoPUSCH-PC-AdjustmentStates twoPUSCH-PC-AdjustmentStates - the UE determines l=0 for the PUSCH transmission corresponding to the first SRS resource set with usage set to 'codebook' or 'nonCodebook', and l=1 for the PUSCH transmission corresponding to the second SRS resource set with usage set to 'codebook' or 'nonCodebook' - else - the UE determines l=0 for the PUSCH transmission ====================== Futurewei: Support the updated version. Mod: Which spec version you refer here. There seems nothing wrong and what you suggest is already in the spec (at least in the word format). … |
| Issue 21 | LG: We are fine with the TP to exclude the error case. Mod: Error case does not need a TP to my reading. |
| Issue 22 | QC: We actually could not identify the quoted conclusion in Chairman’s notes. vivo: There is no such conclusion in RAN1#107-e. Mod: Correct, there is no such conclusion after all. I was coping it directly from ZTE contribution. Anyways, the suggested TP in [4] seems already covered. |
| Issue 23 | LG: agree with FL’s assessment. - … |
| Issue 24 | CATT: In TS38.214, it is specified that “For PUSCH transmissions with a Type 1 configured grant, when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, if configuredGrantConfig contains only one srs-ResourceIndicator and precodingAndNumberOfLayers (applicable when higher layer parameter usage in SRS-ResourceSet set to 'codebook’), PUSCH repetitions are associated only with the first SRS resource set.” It is possible that N SRS resources are configured for each SRS resource set in srs-ResourceSetToAddModList and M SRS resources are configured for each SRS resource set in srs-ResourceSetToAddModListDCI-0-2, wherein N does not equal to M. We prefer to change “PUSCH repetitions are associated only with the first SRS resource set” to “PUSCH repetitions are associated only with the first SRS resource set in srs-ResourceSetToAddModList” to make the spec more clear. Mod: I assume this is somewhat issue 2 Besides, in the above paragraph, srs-ResourceIndicator and precodingAndNumberOfLayers are used to indicate the Type 1 CG PUSCH transmission is an S-TRP transmission. However, in TS38.331, srs-ResourceIndicator is a optional parameter, and when one SRS resource is configured per SRS resource set, srs-ResourceIndicator is absent. Therefore, srs-ResourceIndicator can not be used to identify whether the Type 1 CG PUSCH transmission is an S-TRP transmission. We suggest to use pathlossReferenceIndex instead of srs-ResourceIndicator, i.e. the TP is as follows: “For PUSCH transmissions with a Type 1 configured grant, when two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2, if configuredGrantConfig contains only one pathlossReferenceIndex (applicable when higher layer parameter usage in SRS-ResourceSet set to 'codebook’), PUSCH repetitions are associated only with the first SRS resource set.” Company 2: Mod: We can also discuss this with Issue 2. srs-ResourceIndicator is not used to identify the mTRP operation. Please check issue 2. … |
| Issue 25 | CATT: In TS38.213, it is specified that If a UE - is configured with two UL carriers for a serving cell, and - determines a Type 1 power headroom report and a Type 3 power headroom report for the serving cell the UE - provides the Type 1 power headroom report if both the Type 1 and Type 3 power headroom reports are based on respective actual transmissions or on respective reference transmissions - provides the power headroom report that is based on a respective actual transmission if either the Type 1 report or the Type 3 report is based on a respective reference transmission. It can be seen from the spec that when a Type 1 PHR and a Type 3 PHR is determined, whether the Type 1 PHR is reported or not is related to whether the Type 1 PHR is an actual PHR or not. When PUSCH is configured with mTRP repetition, it is possible that two Type 1 PHRs are calculated, and the UE determines two Type 1 PHRs and a Type 3 PHR for the serving cell. When 2 Type 1 PHRs are calculated, it is possible that one of them is an actual PHR and the other one is a virtual PHR. For such case, whether Type 1 PHR is reported or Type 3 PHR is reported could be determined according to the first Type 1 PHR of the two Type 1 PHRs, i.e., For a UE configured with mTRP PUSCH repetition, when determining whether a Type 1 PHR or a Type 3 PHR is reported, - the Type 1 PHR is treated as a PHR based on actual transmission if the first PHR of PUSCH is an actual PHR; - the Type 1 PHR is treated as a PHR based on reference transmission if the first PHR of the Type 1 PHR is a virtual PHR. ASUSTeK: We also discuss this issue in our contribution [20], and similar issue indeed was discussed in RAN1 #107e, but group tends to decide it in maintenance phase. We share same view as CATT, in case of twoPHRMode and mTRP PUSCH, as there are two Type 1 PHRs, it’s not clear for us “the determined Type 1 power headroom report” in current spec is associated to which Type 1 PHR. In addition to CATT’s solution, we propose another solution to clarify “the determined Type 1 power headroom report” is one of two Type 1 PHR which is based on PUSCH transmission with same QCL Type-D RS as SRS transmission for the Type 3 PHR. Company 2: Mod: to my reading, we discussed this before. Need other’s inputs to proceed again. |
| Issue 26 | CATT: In previous meetings, we don’t have an agreement that the bitwidth of SRI is determined according to the number of configured SRS resources in the SRS resource set indicated by SRS resource set indicator field. In RAN1 #106b-e meeting, it was agreed that the number of SRS resources configured in the two SRS resource sets for mTRP PUSCH repetition is the same. It implies that NSRS in the formula for bit width of SRI fields in DCI format 0_1/0_2 can the number of configured SRS resources in an SRS resource set, where the SRS resource set can be any one of SRS resource sets configured by higher layer parameter srs-ResourceSetToAddModList/ResourceSetToAddModListDCI-0-2 and associated with the higher layer parameter usage of value 'codeBook' and 'nonCodeBook', respectively. Current TS38.212 specified that is the number of configured SRS resources in the SRS resource set indicated by SRS resource set indicator field, we think it is not inline with the agreement and suggest to change “ is the number of configured SRS resources in the SRS resource set indicated by SRS resource set indicator field” to “NSRS is the number of configured SRS resources in an SRS resource set configured by higher layer parameter srs-ResourceSetToAddModList(ResourceSetToAddModListDCI-0-2)” Company 2: Mod: to be honest, I still do not think there is any ambiguity of the reading. SRS resource indicator field has certain bit width and 38.212 associate the SRS resource set based on the resource set indicator field. May be further inputs needed from others. |
| Issue 27 | Lenovo: According to T338.213 V17.0.0, only M-TRP Type 1 CG PUSCH retransmission or Type 2 CG PUSCH transmission or retransmission by DCI format 0_1/0_2 are drafted for power control parameter set determination. While for Type 1 CG PUSCH transmission, Type 1 CG PUSCH retransmission or Type 2 CG PUSCH transmission or retransmission by DCI format 0_0 in M-TPR operation, the power control parameter set determination is not drafted in the specification. Therefore, we need the TP to cover these cases. Company 2: Mod: Let’s further seek for other’s input on this. … |
| Issue 28 | Company 1: Company 2: … |
| Issue 29 | LG: agree with FL’s assessment. … |
| Issue 30 | LG: agree with FL’s assessment. … |
| Issue 31 | Samsung: We can understand FL’s general comment but at least for CB based type1 CG PUSCH, clarification is needed because only sTRP case is covered in current version of 38.214 (V17.0.0) as follow: “or configured by higher layer parameter precodingAndNumberOfLayers:” LG: support this TP. According to current specification, 'Second Precoding information' field is not mentioned and not used to determine actual number of PTRS port. Mod: ok. let’s wait for other inputs prior triggering a discussion on this. … |
| Issue 32 | Company 1: Company 2: … |

## Section to include TP suggestions

Please provide your input on TP changes with the issue number (1-32) for the issues mentioned in Section 1. Refer a common format as illustrated by example below.

### TP for Issue 1.1: CATT

Section 7.1.1 of TS 38.213

< Unchanged parts are omitted >

-	if the DCI format scheduling the PUSCH transmission includes two SRI fields and the SRS resource set indicator is “10” or “11” and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the two SRI fields and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values, and determines first and second values of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId values that are mapped to the values of the first and second SRI fields, respectively.

-     if the DCI format scheduling the PUSCH transmission includes two SRI fields and the SRS resource set indicator is “00” or “01” and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to ‘codebook’, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the first SRI field and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values, and determines the value of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId values that are mapped to the values of the first SRI field.

-	if the DCI format scheduling the PUSCH transmission includes two SRI fields and the SRS resource set indicator is “10” or “11” and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between

-	a set of values for the first SRI field value and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values, and determines the first value of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId value that is mapped to the first SRI field value, and

- a set of values associated with the second SRI field value for a same number of layers as indicated by the first SRI field [5, TS 38.212], and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values, and determines the second value of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId value that is mapped to the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212].

-	if the DCI format scheduling the PUSCH transmission includes two SRI fields and the SRS resource set indicator is “00” or “01” and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between

-	a set of values for the first SRI field value and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values, and determines the first value of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId value that is mapped to the first SRI field value.

< Unchanged parts are omitted >

-	If the UE is provided by SRI-PUSCH-PowerControl more than one values of p0-PUSCH-AlphaSetId

-	if a DCI format scheduling the PUSCH transmission includes two SRI fields and the SRS resource set indicator is “10” or “11” and an open-loop power control parameter set indication field and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook'

-	if a value of the open-loop power control parameter set indication field is '0', the UE determines two values of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId values in SRI-PUSCH-PowerControl that are mapped to the two SRI values corresponding to each SRS resource set with usage set to ‘codebook’.

-	if a value of the open-loop power control parameter set indication field is '1', the UE determines two values of PO_UE_PUSCH,b,f,cj from first values in P0-PUSCH-Set in P0-PUSCH-SetList and P0-PUSCH-Set in P0-PUSCH-SetList2 with p0-PUSCH-SetId values mapped to the two SRI values corresponding to each SRS resource set with usage set to ‘codebook’, respectively.

-	if a DCI format scheduling the PUSCH transmission includes two SRI fields and the SRS resource set indicator is “00” or “01” and an open-loop power control parameter set indication field and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook'

-	if a value of the open-loop power control parameter set indication field is '0', the UE determines a value of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId values in SRI-PUSCH-PowerControl that are mapped to the first SRI value corresponding to the first SRS resource set with usage set to ‘codebook’.

- 	if a value of the open-loop power control parameter set indication field is '1', the UE determines a value of PO_UE_PUSCH,b,f,cj from first value in P0-PUSCH-Set in P0-PUSCH-SetList with p0-PUSCH-SetId value mapped to the first SRI values corresponding to the first SRS resource set with usage set to ‘codebook’.

-	if a DCI format scheduling the PUSCH transmission includes two SRI fields and the SRS resource set indicator is “10” or “11” and an open-loop power control parameter set indication field and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook',

-	if a value of the open-loop power control parameter set indication field is '0', the UE determines two values of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId values in SRI-PUSCH-PowerControl that are mapped to the first SRI field value corresponding to the first SRS resource set with usage set to ‘nonCodebook’ and to a second value, that is associated with the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value, corresponding to the second SRS resource set with usage set to ‘nonCodebook’.

-     if a value of the open-loop power control parameter set indication field is '1', the UE determines two values of PO_UE_PUSCH,b,f,cj from first values in P0-PUSCH-Set in P0-PUSCH-SetList and P0-PUSCH-Set in P0-PUSCH-SetList2 with p0-PUSCH-SetId values mapped to the first SRI field value corresponding to the first SRS resource set with usage set to 'nonCodebook, and a second value, that is associated with the second SRS field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value, corresponding to the second SRS resource set with usage set to 'nonCodebook', respectively.

-   -	if a DCI format scheduling the PUSCH transmission includes two SRI fields and the SRS resource set indicator is “00” or “01” and an open-loop power control parameter set indication field and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook',

-	if a value of the open-loop power control parameter set indication field is '0', the UE determines a value of PO_UE_PUSCH,b,f,cj from the p0-PUSCH-AlphaSetId values in SRI-PUSCH-PowerControl that are mapped to the first SRI field value corresponding to the first SRS resource set with usage set to ‘nonCodebook’.

-     if a value of the open-loop power control parameter set indication field is '1', the UE determines a value of PO_UE_PUSCH,b,f,cj from first values in P0-PUSCH-Set in P0-PUSCH-SetList and P0-PUSCH-Set in P0-PUSCH-SetList2 with p0-PUSCH-SetId values mapped to the first SRI field value corresponding to the first SRS resource set with usage set to 'nonCodebook.

< Unchanged parts are omitted >

-	if a DCI format scheduling the PUSCH transmission includes two SRI fields and the SRS resource set indicator is “10” or “11” and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook', the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for

-	the two SRI fields and a set of indexes provided by P0-PUSCH-AlphaSetId that map to P0-PUSCH-AlphaSet values, and determines first and second values of αb,f,cj from the P0-PUSCH-AlphaSetID values that are mapped to the values of the first and second SRI field values, respectively.

-	if a DCI format scheduling the PUSCH transmission includes two SRI fields and the SRS resource set indicator is “00” or “01” and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook', the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for

-	the first SRI field and a set of indexes provided by P0-PUSCH-AlphaSetId that map to P0-PUSCH-AlphaSet values, and determines a value of αb,f,cj from the P0-PUSCH-AlphaSetID values that are mapped to the values of the first SRI field values.

-	if a DCI format scheduling the PUSCH transmission includes two SRI fields and the SRS resource set indicator is “10” or “11” and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for

-	the first SRI field and a set of indexes provided by P0-PUSCH-AlphaSetId that map to P0-PUSCH-AlphaSet values, and determines first value of αb,f,cj from the P0-PUSCH-AlphaSetID value that is mapped to the first SRI field value, and

-    the second value, associated with the second SRI field value corresponding to Tables 7.3.1.1.2-28/29/30/31 of [5, TS 38.212] for a same number of layers as indicated by the first SRI field value, and a set of indexes provided by p0-PUSCH-AlphaSetId that map to a set of P0-PUSCH-AlphaSet values, and determines the second value of αb,f,cj from the p0-PUSCH-AlphaSetId value that is mapped to the second SRI field value

- 	if a DCI format scheduling the PUSCH transmission includes two SRI fields and the SRS resource set indicator is “00” or “01” and the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'nonCodebook', the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for

-	the first SRI field and a set of indexes provided by P0-PUSCH-AlphaSetId that map to P0-PUSCH-AlphaSet values, and determines first value of αb,f,cj from the P0-PUSCH-AlphaSetID value that is mapped to the first SRI field value0

### TP for Issue Y: Company 2

# Reference

| [1] | R1-2200930 | Remaining issues on multi-TRP for reliability and robustness in Rel-17 | Huawei, HiSilicon |
| --- | --- | --- | --- |
| [2] | R1-2200992 | Multi-TRP/panel for non-PDSCH | FUTUREWEI |
| [3] | R1-2201079 | Maintenance on Multi-TRP for PDCCH, PUCCH and PUSCH enhancements | vivo |
| [4] | R1-2201186 | Remaining issues on multi-TRP enhancements for PDCCH, PUCCH and PUSCH | ZTE |
| [5] | R1-2201224 | Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH | OPPO |
| [6] | R1-2201329 | Discussion on remaining issues on multi-TRP/panel for PDCCH, PUCCH and PUSCH | CATT |
| [7] | R1-2201427 | Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH | Lenovo, Motorola Mobility |
| [8] | R1-2201464 | Remaining issues on MTRP for reliability | NTT DOCOMO, INC. |
| [9] | R1-2201535 | Discussion on enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH | Spreadtrum Communications |
| [10] | R1-2201568 | Enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH | LG Electronics |
| [11] | R1-2201683 | Maintenance of multi-TRP PDCCH, PUCCH and PUSCH enhancements | Intel Corporation |
| [12] | R1-2201759 | Views on Rel-17 multi-TRP reliability enhancement | Apple |
| [13] | R1-2201845 | Remaining issues of enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH | CMCC |
| [14] | R1-2201897 | Discussion on remaining issues on multi-TRP for PUCCH and PUSCH | NEC |
| [15] | R1-2201939 | Enhancements on Multi-TRP for PDCCH, PUSCH and PUCCH | Xiaomi |
| [16] | R1-2201959 | Remaining issues on Multi-TRP for PDCCH, PUCCH and PUSCH | TCL Communication Ltd. |
| [17] | R1-2201997 | Maintenance on Rel-17 Multi-TRP for PDCCH, PUCCH and PUSCH | Samsung |
| [18] | R1-2202123 | Remaining details of Multi-TRP for PDCCH, PUCCH and PUSCH | Qualcomm Incorporated |
| [19] | R1-2202266 | Remaining issues on PDCCH, PUSCH and PUCCH enhancements for multi-TRP | Ericsson |
| [20] | R1-2202284 | Remaining issues for mTRP PDCCH and PUSCH | ASUSTeK |
| [21] | R1-2202317 | Maintenance of enhancements for Multi-TRP URLLC schemes | Nokia, Nokia Shanghai Bell |