3GPP TSG RAN WG1 #108-e		                                   					       												                 R1-2202861

e-Meeting, February 21st – March 3rd, 2022

Source:	Ad-Hoc Chair (AT&T)

Title:	Session Notes of AI 8.16.4

Agenda Item:	8.16.4

Document for: 	Endorsement

#### 8.16.4	UE features for NR NTN

[108-e-R17-UE-features-NR-NTN-01] Email discussion on UE features for NR-NTN – Ralf (AT&T)

- 1st check point: February 25

- Final check point: March 3

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 26. NR_NTN_solutions | 26-1 | Uplink Time and Frequency pre-compensation and timing relationship enhancements | - UE specific TA calculation based on its GNSS-acquired position and the serving satellite ephemeris. - UE applies calculates common TA according to the parameters provided by the network [(UE considers common TA as 0 if the parameter is not provided)] - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - FFS: UE pre-compensates the calculated TA in its uplink transmissions - Support of estimating UE-gNB RTT and delaying the start of RAR window [by UE-gNB RTT] - Support of frequency pre-compensation to counter shift the Doppler experienced on the service link - Determining timing of the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset if indicated - Determining timing of the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated and determining the timing of PDCCH monitoring in recovery search space using K-mac during beam failure recovery procedure - UE receives cell-specific K_offset/K_mac in system information |  | No | No | Release 17 NR UE cannot communicate via access [NTN/satellite/HAPS/ATG] | [Per UE/per band] | No | No | No | An NTN UE is required to at least support UE specific TA and frequency calculation based at least on its GNSS-acquired position and the serving satellite ephemeris | Optional with capability signalling For UE supports NR communication via [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR cell for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; NR NTN cell and ATG cell, for terrestrial any other cell except for ARG cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26. NR_NTN_solutions | 26-3 | Enhancement on the timing relationship | - FFS: delaying the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset - the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated | [26-1][26-2] | No | No | Release 17 UE cannot access [NTN/ satellite/HAPS/ATG] | [Per UE/per band] | No | No | No | FFS: whether this FG gets merged with FG 26-1 | Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR cell for NR NTN cell and ATG cell, for terrestrial cell except for ARG cell this feature is not supported] |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 26. NR_NTN_solutions | 26-6 | [HARQ feedback disabling for] Type-2 HARQ codebook Enhancement | - [Support of HARQ feedback disabling] - Support of type-2 HARQ codebook enhancements [for feedback-disabled HARQ processes] FFS: UE supports HARQ disabling |  | Yes | No | Type-2 HARQ codebook enhancement is not supported for NR communication via satellite | [Per UE/per band] | No | No | No |  | Optional with capability signalling [Note: This UE feature group is applicable only for NR cell for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; NR NTN cell and ATG cell, for terrestrial any other cell except for ARG cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26. NR_NTN_solutions | [26-6a] | [[HARQ feedback disabling for] Type-1 HARQ codebook enhancement] with HARQ disabling | - [Support of HARQ feedback disabling] - Support of Enhancement on Type-1 HARQ codebook enhancements for feedback-disabled HARQ processes in NTN - FFS: HARQ disabling | [26-1, 26-2] | Yes | No | Type-1 HARQ codebook enhancement is not supported for NR communication via satellite | [Per UE/per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] No | FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported.] [Note: This UE feature group is applicable only for NR cell for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; NR NTN cell and ATG cell, for terrestrial any other cell except for ARG cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-6b | [HARQ feedback disabling for] Type-3 HARQ codebook enhancement with HARQ disabling | - [Support of HARQ feedback disabling] - Support of Type-3 HARQ codebook enhancements for feedback-disabled HARQ processes |  | Yes | No | Type-3 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No |  | Optional with capability signalling [Note: This UE feature group is applicable only for NR cell for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; for any other cell this feature is not supported] |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 26. NR_NTN_solutions | 26-5 | Increasing the number of HARQ processes | The maximal supported HARQ process number is X for UL and Y for DL |  | Yes | No | Increased number of HARQ processes is not supported for NR communication via satellite | [Per band or per FSPC or per UE] | No | No | No | Candidate component values for (X,Y): {(16,32),(32,16),(32,32)} | Optional with capability signalling [Note: This UE feature group is applicable only for NR cell for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; NR NTN cell and ATG cell, for terrestrial any other cell except for ARG cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 26. NR_NTN_solutions | 26-6 | [HARQ feedback disabling for] Type-2 HARQ codebook enhancement | - [Support of HARQ feedback disabling] - Support of type-2 HARQ codebook enhancements when there are for feedback-disabled HARQ processes | FFS | Yes | No | Type-2 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No |  | Optional with capability signalling [Note: This UE feature group is applicable only for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; for any other cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26. NR_NTN_solutions | 26-6a | [HARQ feedback disabling for] Type-1 HARQ codebook enhancement | - [Support of HARQ feedback disabling] - Support of Type-1 HARQ codebook enhancements when there are for feedback-disabled HARQ processes | FFS | Yes | No | Type-1 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No |  | Optional with capability signalling [Note: This UE feature group is applicable only for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; for any other cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-6b | [HARQ feedback disabling for] Type-3 HARQ codebook enhancement | - [Support of HARQ feedback disabling] - Support of Type-3 HARQ codebook enhancements when there are for feedback-disabled HARQ processes | FFS | Yes | No | Type-3 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No |  | Optional with capability signalling [Note: This UE feature group is applicable only for NR cell for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; for any other cell this feature is not supported] |

R1-2200940	Rel-17 UE features for NR NTN	Huawei, HiSilicon

R1-2201222	UE features for NR NTN	MediaTek Inc.

R1-2201303	Discussion on UE features for NTN-NR	OPPO

R1-2201411	On UE features for NR NTN	Nokia, Nokia Shanghai Bell

R1-2201504	Discussion on Rel.17 UE features for NR NTN	NTT DOCOMO, INC.

R1-2201729	On UE features for NR NTN	Intel Corporation

R1-2201794	Views on Rel-17 NR NTN UE Features	Apple

R1-2201807	On UE features for NR NTN	Ericsson Hungary Ltd

R1-2202041	UE features for NR NTN	Samsung

R1-2202168	UE features for NR NTN	Qualcomm Incorporated

R1-2202212	Discussion on UE feature for NR-NTN	ZTE

R1-2202290	Discussion on Rel-17 UE feature for NR NTN	LG Electronics