3GPP TSG RAN WG1 #108-e		                                   					       												                 R1-2202862

e-Meeting, February 21st – March 3rd, 2022

Source:	Ad-Hoc Chair (AT&T)

Title:	Session Notes of AI 8.16.5

Agenda Item:	8.16.5

Document for: 	Endorsement

#### 8.16.5	UE features for NR positioning enhancements

[108-e-R17-UE-features-ePos-01] Email discussion UE on features for NR positioning enhancements – Ralf (AT&T)

- 1st check point: February 25

- Final check point: March 3

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-6 | DL PRS processing capabilities in RRC inactive state | 1. DL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE 3. Max number of DL PRS resources that UE can process in a slot under it […] | FFS | No |  |  | Per band | FFS | FFS | FFS | Component 1 candidate values: {Type 1, Type 2} Component 2 candidate values: T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Component 3 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Need for location server to know if the feature is supported Note: Having the PRS processing capabilities in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state [, but instead LMF may set the response time assuming a specific RRC state during the PRS measurement and inform the gNB on the assumed RRC state, while the actual RRC state is still determined by UE/gNB that take the response time requirement and assumed RRC state into account.] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-16 | OLPC for positioning SRS in RRC_INACTIVE state | Same as LPP OLPC-SRS-Pos-r16 RRC OLPC-SRS-Pos-r16 |  | Yes |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-16a | OLPC for positioning SRS in RRC_INACTIVE state | Same as LPP OLPC-SRS-Pos-r16 |  | No |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Support of OLPC in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signaling |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-19 | Spatial relation for positioning SRS in RRC_INACTIVE state | Same as LPP SpatialRelationsSRS-Pos-r16 RRC SpatialRelationsSRS-Pos-r16 |  | Yes |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-19a | Spatial relation for positioning SRS in RRC_INACTIVE state | Same as LPP SpatialRelationsSRS-Pos-r16 |  | No |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Support of spatial relation in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signalling |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-18a | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA | Support of PRS measurement in RRC_INACTIVE state state for DL-TDOA |  | FFS |  |  | FFS | FFS | FFS | FFS | [Need for location server to know if the feature is supported.] Note: Applicable for both UE-assisted and UE-based DL-TDOA Note: PRS capabilities for DL-TDOA measurement and reporting described in FGs in 13-3, 13-3a, 13-3b, 13-6, 13-13 are the same for RRC Inactive. Support of PRS processing measurement in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-18b | Support of PRS measurement in RRC_INACTIVE state for DL-AoD | Support of PRS measurement RRC_INACTIVE state for DL-AoD |  | FFS |  |  | FFS | FFS | FFS | FFS | [Need for location server to know if the feature is supported.] Note: Applicable for both UE-assisted and UE-based DL-AoD Note: PRS capabilities for DL-AOD measurement and reporting described in FGs 13-2, 13-2a, 13-2b, 13-5, 13-13 are the same for RRC Inactive. Support of PRS processing measurement in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signaling. |
| 27. NR_pos_enh | 27-18c | Support of PRS measurement in RRC_INACTIVE state for Multi-RTT | 1. Support of PRS measurement in RRC_INACTIVE state for Multi-RTT [2. Support of positioning SRS transmission in RRC_INACTIVE state] |  | FFS |  |  | FFS | FFS | FFS | FFS | [Need for location server to know if the feature is supported.] Note: PRS capabilities for Multi-RTT measurement and reporting described in FGs in 13-4, 13-4a, 13-4b, 13-11, 13-11a, 13-14 are the same for RRC Inactive Support of PRS processing measurement in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signaling. |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-15 | Support of Positioning SRS transmission in RRC_INACTIVE state for initial UL BWP | 1. Max number of SRS Resource Sets for positioning supported by UE 2. Max number of [P/SP] SRS Resources for positioning 3. Max number of [P/SP] SRS Resources for positioning per slot 4. Max number of periodic SRS Resources for positioning 5. Max number of periodic SRS Resources for positioning per slot Note: OLPC for SRS for positioning based on SSB from the last serving cell (the cell that releases UE from connection) is part of this FG. No dedicated capability signaling is intended for this component |  | Yes |  |  | Per band | n/a | n/a | n/a | Component 1 candidate values: {1, 2, 4, 8, 12, 16} Component 2 candidate values: {1,2,4,8,16,32,64} Component 3 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Component 4 candidate values: {1,2,4,8,16,32,64} Component 5 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Note: OLPC for SRS for positioning based on SSB from the last serving cell (the cell that releases UE from connection) is part of this FG. No dedicated capability signaling is intended for this component Need for location server to know if the feature is supported FFS: outside initial BWP | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-15b | Positioning SRS transmission in RRC_INACTIVE state configured outside initial UL BWP | - Maximum SRS bandwidth supported for each SCS that UE supports within a single CC - Max number of SRS Resource Sets for positioning supported by UE - Max number of periodic SRS Resources for positioning - Max number of periodic SRS Resources for positioning per slot - [Different numerology between the SRS and the initial UL BWP is supported] - [SRS operation without restriction on the BW: BW of the SRS may not include BW of the CORESET#0 and SSB] - Max number of P/SP SRS Resources for positioning - Max number of P/SP SRS Resources for positioning per slot - FFS: center frequenecy | 27-15 | Yes |  |  | Per band |  |  |  | Component 1 candidate values: FFS Component 2 candidate values: {1, 2, 4, 8, 12, 16} Component 3 candidate values: {1,2,4,8,16,32,64} Component 4 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Component 5 candidate values: FFS Component 6 candidate values: FFS Component 7 candidate values: FFS Note 1: The SRS should have a locationAndBandwidth, SCS, CP, defined the same way as a legacy BWP. [Note 2: Based on other signalled UE capabilities, the UE supports at least one connected mode configuration where a hypothetical BWP defined by this SRS is the active BWP and switching between this active BWP and the initial BWP is supported.] [Note 3: If component 5 is not signaled, the UE only supports same numerology between the SRS and the initial UL BWP] [Note 4: If component 6 is not signaled, the UE supports only SRS BW that include the BW of the CORESET #0 and SSB.] [Need for location server to know if the feature is supported] | Optional with capability signaling |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-13a | First path RSRPP reporting for UE-assisted DL-TDOA | 1. Support of RSRPP reporting for first path | 13-1 | No |  |  | FFS: Per UE or per band | No n/a | No n/a | No n/a | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-14a | First path RSRPP reporting for Multi-RTT | 1. Support of RSRPP reporting for first path | 13-1 | No |  |  | FFS: Per UE or per band | No n/a | No n/a | No n/a | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-2-1 | DL PRS RSRPP measurement report of the first path for UE-assisted DL-AoD | 1.) Support of measuring and reporting the PRS RSRPP of the first path for DL-AoD positioning method 2.) The maximum number of first path PRS RSRPP per TRP | 13-5 or 27-2-2 | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | Component 2 candidate values: 1, 2,4,8,16,24 Need for location server to know if the feature is supported The maximum number of first path PRS RSRP per TRP should be less than or equal to the maximum number of PRS RSRP (27-2-2) [Note: Having FG 13-5 as the prerequisite FG does not imply that in a measurement report, reporting PRS-RSRP of a PRS resource should be the prerequisite of reporting PRS-RSRPP for the first path of the PRS resource] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-3-2 | DL PRS measurement outside MG and in a PRS processing window - processing types | 1. Supported PRS processing types subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG and in a PRS processing window 2. Support of priority handing options of PRS: Option1, Option2 or Option3 - Option 1: UE may indicates support of two priority states. - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 2: UE may indicate support of three priority states - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than PDCCH and URLLC PDSCH and higher priority than other PDSCH/CSI-RS - Note: The URLLC channel corresponds a dynamically scheduled PDSCH whose PUCCH resource for carrying ACK/NAK is marked as high-priority. - State 3: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 3: UE may indicate support of single priority state - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS Note: - Type 1A refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from all DL CCs (per UE) are affected across LTE and NR - Type 1B refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from a certain band are affected (FFS FR2) - Type 2 refers to the determination of prioritization between DL PRS and other DL signals/channels only in DL PRS symbols within the PRS processing window [The DL signals/channels from all DL CCs (per UE) are affected (FFS FR2)] - [For Type 2 PRS processing time, the PRS processing in one FR2 band may affect the downlink receiving in a second FR2 band] Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options Note: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP | 13-1 | Yes |  | DL PRS measurement outside MG and in a PRS processing window is not supported | per band | n/a | n/a | n/a | Component 1 candidate values: [One or more of] {Type 1A, Type 1B, Type 2} Component 2 candidate values: {option1, option2, option3} Need for location server to know if the feature is supported Note: Component 2 can be reported per supported band for each type supported by the UE, details left to RAN2 Note: A UE that supports FG 27-3-2 also needs to support FG 27-3-2a | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-3-2a | Support of priority handing of PRS when PRS measurement is outside MG | Support of priority handing options of PRS: Option1, Option2 or Option3 - Option 1: UE may indicates support of two priority states. - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 2: UE may indicate support of three priority states - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than PDCCH and URLLC PDSCH and higher priority than other PDSCH/CSI-RS - Note: The URLLC channel corresponds a dynamically scheduled PDSCH whose PUCCH resource for carrying ACK/NAK is marked as high-priority. - State 3: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 3: UE may indicate support of single priority state State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS | [27-3-3] | Yes |  |  | Per band | No | No | No | Candidate values: {option1, option2, option3} Note: A UE that supports FG 27-3-2a also needs to support FG 27-3-2 Note: if the FFS in FG 27-2a gets resolved as “per band’, FG 27-2a will be deleted and becomes a component of FG 27-3-2 | Optional with capability signaling |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-17 | Support of positioning in RRC_INACTIVE state PRS processing in RRC_INACTIVE | Support of PRS processing in RRC_INACTIVE | [13-1, 13-2, 13-3, 13-4] | FFS Yes |  | FFS PRS processing in RRC_INACTIVE is not supported | FFS per band | FFS n/a | FFS n/a | FFS n/a | [Need for location server to know if the feature is supported.] FFS: separate UE capability for location information reporting in RRC_INACTIVE state using SDT Note: UE supporting this feature may support at least one from DL RSTD, DL PRS-RSRP, or UE Rx – Tx time difference measurement | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 27. NR_pos_enh | 27-18a | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA |  | FFS No |  | FFS PRS measurement in RRC_INACTIVE state for DL-TDOA is not supported | FFS per band | FFS n/a | FFS n/a | FFS n/a | Need for location server to know if the feature is supported. Note: Applicable for both UE-assisted and UE-based DL-TDOA Note: PRS capabilities for DL-TDOA measurement and reporting described in FGs in 13-3, 13-3a, 13-3b, 13-6, 13-13 are the same for RRC Inactive. Support of PRS processing measurement in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-18b | Support of PRS measurement in RRC_INACTIVE state for DL-AoD | Support of PRS measurement in RRC_INACTIVE state for DL-AoD |  | FFS No |  | FFS PRS measurement in RRC_INACTIVE state for DL-AoD is not supported | FFS per band | FFS n/a | FFS n/a | FFS n/a | Need for location server to know if the feature is supported. Note: Applicable for both UE-assisted and UE-based DL-AoD Note: PRS capabilities for DL-AOD measurement and reporting described in FGs 13-2, 13-2a, 13-2b, 13-5, 13-13 are the same for RRC Inactive. Support of PRS processing measurement in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signaling. |
| 27. NR_pos_enh | 27-18c | Support of PRS measurement in RRC_INACTIVE state for Multi-RTT | 1. Support of PRS measurement in RRC_INACTIVE state for Multi-RTT |  | FFS No |  | FFS PRS measurement in RRC_INACTIVE state for for Multi-RTT is not supported | FFS per band | FFS n/a | FFS n/a | FFS n/a | Need for location server to know if the feature is supported. Note: PRS capabilities for Multi-RTT measurement and reporting described in FGs in 13-4, 13-4a, 13-4b, 13-11, 13-11a, 13-14 are the same for RRC Inactive Support of PRS processing measurement in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signaling. |

R1-2200925	Remaining issues of Rel-17 positioning UE feature	Huawei, HiSilicon

R1-2201123	Discussion on UE features for NR positioning enhancements	vivo

R1-2201200	UE features for NR positioning enhancements	ZTE

R1-2201245	UE features for NR positioning enhancements	OPPO

R1-2201347	Remaining issues on Rel-17 UE features for NR Positioning enhancements	CATT

R1-2201412	On UE features for NR positioning enhancements	Nokia, Nokia Shanghai Bell

R1-2201447	UE features for Rel-17 NR positioning enhancements	China Telecom

R1-2201505	Discussion on Rel-17 UE features for NR positioning enhancements	NTT DOCOMO, INC.

R1-2201730	UE features for Rel.17 NR positioning enhancements	Intel Corporation

R1-2201795	Views on UE features for NR positioning enhancements	Apple

R1-2201883	Discussion on UE features for NR positioning enhancement	CMCC

R1-2201953	Discussion on UE features for NR Positioning Enhancements	Xiaomi

R1-2202042	UE features for NR positioning enhancements	Samsung

R1-2202169	UE features for NR positioning enhancements	Qualcomm Incorporated

R1-2202388	Views on NR positioning enhancements UE features	Ericsson