3GPP TSG RAN WG1 #108-e																			R1-2202914

e-Meeting, February 21st – March 3rd, 2022

Source:	Intel Corporation

Title:	Agreed TPs to TS 38.213 and TS 38.214 for HST-SFN enhancements

Agenda item:	8.1.2.4

Document for:	Discussion and Decision

# Introduction

This document contains agreed TPs on enhancements for HST-SFN deployment.

# TP #2-5 to TS 38.213

Motivation

Capture agreement on default spatial relation and PL-RS of single-TRP PUCCH/PUSCH/SRS transmission for the case when SFN PDCCH transmission is configured.

TP 2-5 to TS 38.213

9.2.2 PUCCH Formats for UCI transmission <Unchanged part omitted> If a UE - is not provided pathlossReferenceRSs in PUCCH-PowerControl, - is provided enableDefaultBeamPL-ForPUCCH, and - is not provided PUCCH-SpatialRelationInfo, and - is not provided coresetPoolIndex value of 1 for any CORESET, or is provided coresetPoolIndex value of 1 for all CORESETs, in ControlResourceSet and no codepoint of a TCI field, if any, in a DCI format of any search space set maps to two TCI states [5, TS 38.212] a spatial setting for a PUCCH transmission from the UE is same as a spatial setting for PDCCH receptions by the UE in the CORESET with the lowest ID on the active DL BWP of the PCell, if the CORESET has two activated TCI states, as described in clause 10.1, the UE determines the spatial setting based on the first TCI state. For a PUCCH transmission over multiple slots, a same spatial setting applies to the PUCCH transmission in each of the multiple slots. <Unchanged part omitted>

# TP #2-7 and TP#2-1a (second part) to TS 38.214

Motivation

Remove BWP-DownlinkCommon since it is not actual BWP that can be used for SFN scheme configuration. Capture agreement on configuration restrictions of SFN scheme for different BWPs and CCs.

TP#2-7 and TP#2-1a (second part)

TS 38.214 <Unchanged parts are omitted> ## 5.1 UE procedure for receiving the physical downlink shared channel … When a UE is configured with sfnSchemePdsch and/or sfnSchemePdcch for a DL BWP, the UE shall expect that the sfnSchemePdschand/or sfnSchemePdcch configuration are the same in the other all DL BWPs within a CC other than initial BWP [and BWP-DownlinkCommon], and the UE shall expect that the sfnSchemePdsch and/or sfnSchemePdcch configuration are the same in all CCs in a same frequency band if the UE is configured with CA. < Unchanged parts are omitted >