3GPP TSG-RAN WG1 Meeting #108-e	R1-2202968

# e-Meeting, February 21 – March 3, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | 0102 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on NR Multicast and Broadcast Services in 38.212 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS |  |  |  |  |  | Date: |  |  | 2022-03-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Capture endorsed TPs and agreements from RAN1#107bis-e and RAN1#108-e. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Incorporate the TPs agreed in RAN1#107bis-e meeting, including the TP for Table 5.4.2.1-1 and the TP for clause 7.3.1.5.1. - Reflect the agreements achieved in RAN1#107bis-e meeting, including adding fields of MCS/NDI/RV for transport block 2, adding the field of “ZP CSI-RS trigger” in Clasue 7.3.1.5.3, and adding the description that the size of DCI format 4_2 is configurable. - Incorporate the TPs agreed in RAN1#108-e meeting, including the TP for clause 7.3.1.5.2 and the TP for clause 7.3.1.5.3. - Reflect the agreements achieved in RAN1#108-e meeting, including deleting the case of 4 bits for DAI field in DCI format 4_2 and deleting the case of UE configured with a PUCCH-SCell in DCI format 4_2 since only one cell is configured for multicast. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification for NR Multicast and Broadcast Services is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.1, 7.3.1.5.1, 7.3.1.5.2, 7.3.1.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.213, TS 38.214 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.4.2.1	Bit selection

The bit sequence after encoding  from Clause 5.3.2 is written into a circular buffer of length  for the -th coded block, where  is defined in Clause 5.3.2.

For the -th code block, let  if  and  otherwise, where, ,  is determined according to Clause 6.1.4.2 in [6, TS 38.214] for UL-SCH and Clause 5.1.3.2 in [6, TS 38.214] for DL-SCH/PCH, assuming the following:

For one TB for DL-SCH with PDSCH scheduled by DCI format 4_0/4_1/4_2,

-	if the PDSCH is scheduled by DCI format 4_1/4_2,

-	maximum number of layers is given by X, where

-	if the higher layer parameter maxMIMO-Layers-Multicast of PDSCH-Config-Multicast is configured, X is given by that parameter;

-	otherwise, X equals to 1;

-	if the higher layer parameter mcs-Table given by a PDSCH-Config-Multicast for at least one common frequency resource (CFR) is set to  'qam256', maximum modulation order Qm=8 is assumed for DL-SCH; otherwise a maximum modulation order Qm=6 is assumed for DL-SCH;

-	if the PDSCH is scheduled by DCI format 4_0,

-	maximum number of layers is 1;

-	if the higher layer parameter mcs-Table given by a PDSCH-Config-MCCH is set to 'qam256', maximum modulation order Qm=8 is assumed for DL-SCH; otherwise a maximum modulation order Qm=6 is assumed for DL-SCH;

-	if the higher layer parameter mcs-Table given by a PDSCH-Config-MTCH is set to 'qam256', maximum modulation order Qm=8 is assumed for DL-SCH; otherwise a maximum modulation order Qm=6 is assumed for DL-SCH;

-	nPRB=nPRB,LBRM is given by Table 5.4.2.1-1, where the value of nPRB,LBRM for DL-SCH is determined according to the size of the CFR if only one CFR is configured to the UE;

-	maximum coding rate of 948/1024;

-	NRE=156∙nPRB;

-	C is the number of code blocks of the transport block determined according to Clause 5.2.2.

For one TB for UL-SCH, or for one TB for DL-SCH/PCH except for DL-SCH with PDSCH scheduled by DCI format 4_0/4_1/4_2,

-	maximum number of layers for one TB for UL-SCH is given by X, where

-	if the higher layer parameter maxMIMO-Layers of PUSCH-ServingCellConfig of the serving cell is configured, X is given by that parameter

-	elseif the higher layer parameter maxRank of pusch-Config of the serving cell is configured, X is given by the maximum value of maxRank across all BWPs of the serving cell

-	otherwise, X is given by the maximum number of layers for PUSCH supported by the UE for the serving cell

-	maximum number of layers for one TB for DL-SCH/PCH is given by the minimum of X and 4, where

-	if the higher layer parameter maxMIMO-Layers of PDSCH-ServingCellConfig of the serving cell is configured, X is given by that parameter

-	otherwise, X is given by the maximum number of layers for PDSCH supported by the UE for the serving cell

-	if the higher layer parameter mcs-Table-r17 or mcs-TableDCI-1-2-r17 given by a pdsch-Config for at least one DL BWP of the serving cell is set to 'qam1024', maximum modulation order Qm=10 is assumed for DL-SCH, else if the higher layer parameter mcs-Table or mcs-TableDCI-1-2 given by a pdsch-Config for at least one DL BWP of the serving cell is set to 'qam256', maximum modulation order  is assumed for DL-SCH; otherwise a maximum modulation order  is assumed for DL-SCH;

-	if the higher layer parameter mcs-Table or mcs-TableTransformPrecoder or mcs-TableDCI-0-2 or mcs-TableTransformPrecoderDCI-0-2 given by a pusch-Config or the higher layer parameter mcs-Table or mcs-TableTransformPrecoder given by configuredGrantConfig for at least one UL BWP of the serving cell is set to 'qam256', maximum modulation order  is assumed for UL-SCH; otherwise a maximum modulation order  is assumed for UL-SCH

-	maximum coding rate of 948/1024;

-	 is given by Table 5.4.2.1-1, where the value of  for DL-SCH is determined according to the initial downlink bandwidth part if there is no other downlink bandwidth part configured to the UE;

-	;

-	 is the number of code blocks of the transport block determined according to Clause 5.2.2.

Table 5.4.2.1-1: Value of

| Maximum number of PRBs across all configured DL BWPs and UL BWPs of a carrier for DL-SCH and UL-SCH, respectively, or Maximum number of PRBs across all CFRs of a carrier for DL-SCH with PDSCH scheduled by DCI format 4_0/4_1/4_2 |  |
| --- | --- |
| Less than 33 | 32 |
| 33 to 66 | 66 |
| 67 to 107 | 107 |
| 108 to 135 | 135 |
| 136 to 162 | 162 |
| 163 to 217 | 217 |
| Larger than 217 | 273 |

< Unchanged parts are omitted >

7.3.1.5.1	Format 4_0

DCI format 4_0 is used for the scheduling of PDSCH for broadcast in DL cell.

The following information is transmitted by means of the DCI format 4_0 with CRC scrambled by MCCH-RNTI or G-RNTI for MTCH configured by MBS-SessionInfo:

-	Frequency domain resource assignment – log2(NRBDL,CFR(NRBDL,CFR+1)2 bits where NRBDL,CFR equals to NRBDL,BWP

- the size of  CORESET 0 if CORESET 0 is configured for the cell; and

- the size of initial DL bandwidth part if CORESTE 0 is not configured for the cell.

-	Time domain resource assignment – 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214]

-	VRB-to-PRB mapping – 1 bit according to Table 7.3.1.2.2-5

-	Modulation and coding scheme – 5 bits as defined in Clause 5.1.3 of [6, TS38.214]

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	MCCH change notification – 2 bits as defined in Clause x.x.x of [8, TS38.321] if the CRC of the DCI format 4_0 is scrambled by MCCH-RNTI. Otherwise, this bit field is reserved.

-	Padding bits, if required

Zeros shall be appended to DCI format 4_0 until the payload size equals that of DCI format 1_0 monitored in common search space in the same serving cell.

##### 7.3.1.5.2	Format 4_1

DCI format 4_1 is used for the scheduling of PDSCH for multicast in DL cell.

The following information is transmitted by means of the DCI format 4_1 with CRC scrambled by G-RNTI configured by G-RNTI-Config or G-CS-RNTI:

-	Frequency domain resource assignment – log2(NRBDL,CFR(NRBDL,CFR+1)2 bits where NRBDL,CFR equals to NRBDL,BWP as given by clause 7.3.1.0

-	Time domain resource assignment – 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214]

-	VRB-to-PRB mapping – 1 bit according to Table 7.3.1.2.2-5

-	Modulation and coding scheme – 5 bits as defined in Clause 5.1.3 of [6, TS38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number – 4 bits

-	Downlink assignment index – 2 bits as defined in Clause 9.1.3 of [5, TS 38.213], as counter DAI

-	PUCCH resource indicator – 3 bits as defined in Clause 9.2.3 of [5, TS38.213]

-	PDSCH-to-HARQ_feedback timing indicator – 3 bits as defined in Clause 9.2.3 of [5, TS38.213]

-	Reserved bits – 3 bits

7.3.1.5.3	Format 4_2

DCI format 4_2 is used for the scheduling of PDSCH in DL cell.

The following information is transmitted by means of the DCI format 4_2 with CRC scrambled by G-RNTI configured by G-RNTI-Config or G-CS-RNTI:

-	Frequency domain resource assignment – number of bits determined by the following, where NRBDL,CFR is the size of the common frequency resource as configured by higher layer parameter locationAndBandwidth-Multicast:

-	NRBG bits if only resource allocation type 0 is configured, where NRBG is defined in Clause 5.1.2.2.1 of [6, TS38.214],

-	log2(NRBDL,CFR(NRBDL,CFR+1)2 bits if only resource allocation type 1 is configured, or

-	maxlog2(NRBDL,CFR(NRBDL,CFR+1)2,NRBG+1 bits if resourceAllocation in PDSCH-Config-Multicast is configured as 'dynamicSwitch'.

-	If resourceAllocation in PDSCH-Config-Multicast is configured as 'dynamicSwitch', the MSB bit is used to indicate resource allocation type 0 or resource allocation type 1, where the bit value of 0 indicates resource allocation type 0 and the bit value of 1 indicates resource allocation type 1.

-	For resource allocation type 0, the NRBG LSBs provide the resource allocation as defined in Clause 5.1.2.2.1 of [6, TS 38.214].

-	For resource allocation type 1, the log2(NRBDL,CFR(NRBDL,CFR+1)2 LSBs provide the resource allocation as defined in Clause 5.1.2.2.2 of [6, TS 38.214]

-	Time domain resource assignment – 0, 1, 2, 3, or 4 bits as defined in Clause 5.1.2.1 of [6, TS 38.214]. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter pdsch-TimeDomainAllocationList if the higher layer parameter is configured; otherwise I is the number of entries in the default table.

-	VRB-to-PRB mapping – 0 or 1 bit:

-	0 bit if only resource allocation type 0 is configured or if vrb-ToPRB-Interleaver in PDSCH-Config-Multicast is not configured;

-	1 bit according to Table 7.3.1.2.2-5 otherwise, only applicable to resource allocation type 1, as defined in Clause 7.3.1.6 of [4, TS 38.211].

-	PRB bundling size indicator – 0 bit if the higher layer parameter prb-BundlingType is not configured in PDSCH-Config-Multicast or is set to 'staticBundling', or 1 bit if the higher layer parameter prb-BundlingType in PDSCH-Config-Multicast is set to 'dynamicBundling' according to Clause 5.1.2.3 of [6, TS 38.214].

-	Rate matching indicator – 0, 1, or 2 bits according to higher layer parameters rateMatchPatternGroup1 and rateMatchPatternGroup2 in PDSCH-Config-Multicast, where the MSB is used to indicate rateMatchPatternGroup1 and the LSB is used to indicate rateMatchPatternGroup2 when there are two groups.

-	ZP CSI-RS trigger – 0, 1, or 2 bits as defined in Clause 5.1.4.2 of [6, TS 38.214]. The bitwidth for this field is determined as log2(nZP+1) bits, where nZP is the number of aperiodic ZP CSI-RS resource sets configured in PDSCH-Config-Multicast.

For transport block 1:

-	Modulation and coding scheme – 5 bits as defined in Clause 5.1.3.1 of [6, TS 38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

For transport block 2 (only present if maxNrofCodeWordsScheduledByDCI equals 2):

-	Modulation and coding scheme – 5 bits as defined in Clause 5.1.3.1 of [6, TS 38.214]

-	New data indicator – 1 bit

-	Redundancy version – 2 bits as defined in Table 7.3.1.1.1-2

-	HARQ process number – 4 bits

-	Downlink assignment index – number of bits as defined in the following

-	2 bits if the higher layer parameter pdsch-HARQ-ACK-Codebook-Multicast=dynamic, where the 2 bits are the counter DAI;

-	0 bits otherwise.

If higher layer parameter priorityIndicatorDCI-4-2 is configured in PDSCH-Config-Multicast, if the bit width of the Downlink assignment index in DCI format 4_2 for one HARQ-ACK codebook is not equal to that of the Downlink assignment index in DCI format 4_2 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller Downlink assignment index until the bit width of the Downlink assignment index in DCI format 4_2 for the two HARQ-ACK codebooks are the same.

-	PUCCH resource indicator – 3 bits as defined in Clause 9.2.3 of [5, TS 38.213]

-	PDSCH-to-HARQ_feedback timing indicator – 0, 1, 2, or 3 bits as defined in Clause 9.2.3 of [5, TS 38.213]. The bitwidth for this field is determined as log2(I) bits, where I is the number of entries in the higher layer parameter dl-DataToUL-ACK in PUCCH-Config-Multicast1 if configured or PUCCH-Config-Multicast2 if configured; otherwise, I is the number of entries in the higher layer parameter dl-DataToUL-ACK in PUCCH-Config.

If higher layer parameter priorityIndicatorDCI-4-2 is configured in PDSCH-Config-Multicast, if the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 4_2 for one HARQ-ACK codebook is not equal to that of the PDSCH-to-HARQ_feedback timing indicator in DCI format 4_2 for the other HARQ-ACK codebook, a number of most significant bits with value set to '0' are inserted to smaller PDSCH-to-HARQ_feedback timing indicator until the bit width of the PDSCH-to-HARQ_feedback timing indicator in DCI format 4_2 for the two HARQ-ACK codebooks are the same.

-	Antenna port(s) – 4, 5, or 6 bits as defined by Tables 7.3.1.2.2-1/2/3/4, where the number of CDM groups without data of values 1, 2, and 3 refers to CDM groups {0}, {0,1}, and {0, 1,2} respectively. The antenna ports {p0,…,pv-1} shall be determined according to the ordering of DMRS port(s) given by Tables 7.3.1.2.2-1/2/3/4.

If a UE is configured with both dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB, the bitwidth of this field equals max⁡{xA,xB}, where xA is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeA and xB is the "Antenna ports" bitwidth derived according to dmrs-DownlinkForPDSCH-MappingTypeB. A number of xA-xB zeros are padded in the MSB of this field, if the mapping type of the PDSCH corresponds to the smaller value of xA and xB.

-	Transmission configuration indication – 0 bit if higher layer parameter tci-PresentInDCI in PDCCH-Config-Multicast is not enabled; otherwise 3 bits as defined in Clause 5.1.5 of [6, TS38.214].

-	DMRS sequence initialization – 1 bit.

-	Priority indicator – 0 bit if higher layer parameter priorityIndicatorDCI-4-2 is not configured in PDSCH-Config-Multicast; otherwise 1 bit as defined in Clause 9 in [5, TS 38.213].

-	Enabling/disabling HARQ-ACK feedback indication –1 bit if higher layer parameter harq-FeedbackEnabler-Multicast indicates dci-enabler; 0 bit, otherwise.

The size of DCI format 4_2 is configurable by higher layer parameter sizeDCI-4-2 from 20 bits and up to 140 bits.

< Unchanged parts are omitted >