3GPP TSG RAN WG1 #108-e			R1-2200913

e-Meeting, February 21st – March 3rd, 2022

3GPP TSG-RAN WG4 Meeting # 101-bis-e								R4-2202769

# Electronic Meeting, Jan. 17-25, 2022

Title:	LS on signalings for enabling RLM and BFD relaxation in R17 UE power saving

Response to:

Release:	Rel-17

Work Item:	NR_UE_pow_sav_enh-Core

Source:	RAN WG4

To:	RAN WG2

Cc:	RAN WG1

Contact Person:

#### Name:	Yanliang SUN

#### E-mail Address:	 yanliang.sun@vivo.com

Attachments:

1. Overall Description:

In RAN4 #100e, LS R4-2115439 on criteria for RLM/BFD relaxation is sent to RAN2. In RAN4 #101e, further agreements achieved in RAN4 are sent to RAN2 via LS R4-2120314. In RAN4 101-bis-e, RAN4 concludes all the remaining issues regarding the signalings for enabling RLM/BFD relaxation, and would like to inform RAN2 about the following conclusions.

- The RLM/BFD relaxation is enabled by explicit signaling. The signaling design is left for RAN2.

- The low mobility criterion is NOT mandatory to be configured

- Low mobility criterion is a per-UE configuration.

- The good serving cell quality criteria for RLM/BFD is based on an offset X dB and Qx, while Qx is derived from PDCCH transmission parameters.

  - Qx = Qin for RLM

  - Qx = [Qin] for BFD

    - Note: definition of Qin for BFD needs to be clarified

  - The offset X can be configured from a set of 4 values

    - Exact values are FFS

  - One pre-defined value is used for evaluation if the offset is not configured

    - Pre-defined value X = [0] dB

  - Signalling details are up to RAN2

- Introduce a UE capability to indicate the support of RLM/BFD relaxation in general in Rel-17 feature table.

RAN4 respectfully ask RAN2 take above agreements into considerations, further discuss and specify corresponding RRC signalling. In case there is anything unclear from RAN2 perspective, RAN2 may send a reply LS to RAN4 with clarification questions.

2. To RAN WG2 group.

ACTION: 	RAN4 respectfully asks RAN2 to take RAN4 agreements in consideration, discuss how to specify the corresponding signalings, and provide feedback, if needed.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 Meeting #102e	21, Feb. – 4, Mar, 2022	Online