3GPP TSG RAN WG1 #108-e		R1-2202591

e-Meeting, February 21st – March 3rd, 2022

Title:	Reply LS on MBS SPS

Response to:	R1-2200888 (R2-2202016) LS on MBS SPS

Release:	Rel-17

Work Item:	NR_MBS

Source:	RAN1

To:	RAN2

Cc:

Contact person:	Fei Wang

wangfei@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

RAN1 would like to thank RAN2 for the LS R1-2200888 (R2-2202016) on LS on MBS SPS. The following is the RAN1 response for the questions listed in R1-2200888 (R2-2202016).

Q1:RAN2 respectfully asks RAN1 to confirm RAN2’s understanding and the maximal number of G-CS-RNTI configured for UE? Can multiple G-CS-RNTIs be mapped to same MBS SPS-config and if so how that would work?

RAN1 answer: RAN1 made the following agreements for Q1:

Agreement

In the reply LS on MBS SPS to RAN2, capture the following for Q1:

- RAN1 confirms that RAN2’s understanding is correct.

- RAN1 thinks that the maximum number of G-CS-RNTI configured for UE should be subject to UE capability.

Agreement

RAN1 thinks that multiple G-CS-RNTIs cannot be mapped to same MBS SPS-config at the same time for a UE.

Q2: Clarify the handling of PTM retransmissions.

RAN1 answer: RAN1 made the following agreements for Q2:

Agreement

In the reply LS on MBS SPS to RAN2, capture the following for Q2:

- From RAN1 perspective, retransmission scheme (i.e. via PTM or PTP) can be changed per TB per transmission.

  - UE is not expected to receive PTM retransmission after PTP retransmission for the same multicast TB

  - There is no consensus in RAN1 to support PTM retransmission and PTP retransmission simultaneously for different UEs in the same MBS group

- A single CS-RNTI is used for PTP retransmissions of all G-CS-RNTIs.

# 2	Actions

To: RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into account.

# 3	Dates of next TSG RAN WG1 meetings

TSG RAN WG1 Meeting #109-e                       	16th May – 27th May, 2022			E-meeting

TSG RAN WG1 Meeting #110-e                       	22nd Aug – 26th Aug, 2022			E-meeting