3GPP TSG-RAN WG1 Meeting #108-e	R1-2202695

Electronic Meeting, Feb 21st - Mar 3rd, 2022

Title:	Reply LS on CORESET#0 impact of CBW narrower than 40MHz of n79

Response to:	LS on CORESET#0 impact of CBW narrower than 40MHz of n79 (R4-2202286)

Release:	Rel-17

Work Item:	NR_bands_R17_BWs

Source:	RAN1

To:	RAN4

Cc:	RAN2

Contact Person:

#### Name:	Qi Zhang

E-mail Address:	qiqi.zhang@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN1 would like to thank RAN4 for the LS (R1-2200907/R4-2202286) on CORESET#0 impact of CBW narrower than 40 MHz of band n79.

RAN1 has discussed the four alternatives (Alt-1, Alt-1a, Alt-1b, Alt-2) provided by RAN4, and also Alt-1a’ which adds new entries to table 13-5/13-6 based on Alt-1a. RAN1 understand the flexibility impact of Alt-1a, however, consider the less impact on both UE implementation and specification comparing with other alternatives, the following agreements were made:

- Alt-1a: Add narrower channel bandwidth to n79, and indicate the same table 13-5/13-6 to legacy UE and new UE for CORESET#0 configuration.

- Endorse the following text proposal for TS 38.213.

==========Text proposal for 38.213=================

<Unchanged part omitted>

Table 13-3: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {30, 15} kHz for frequency bands with minimum channel bandwidth 5 MHz or 10 MHz except for the frequency bands given in [5, TS38.101-1]

<Unchanged part omitted>

Table 13-4: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {30, 30} kHz for frequency bands with minimum channel bandwidth 5 MHz or 10 MHz except for the frequency bands given in [5, TS38.101-1]

<Unchanged part omitted>

Table 13-5: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {30, 15} kHz for frequency bands with minimum channel bandwidth 40MHz or for the frequency bands given in [5, TS38.101-1]

<Unchanged part omitted>

Table 13-6: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {30, 30} kHz for frequency bands with minimum channel bandwidth 40MHz or for the frequency bands given in [5, TS38.101-1]

<Unchanged part omitted>

==========End of Text proposal =================

- The above text proposal was agreed in RAN1 but the final CR is pending RAN plenary approval subject to approval of a WID update to include RAN1

RAN1 suggest RAN4 to add specific frequency band restriction (i.e., n79) to RAN4 specification to support Alt-1a.

2. Actions:

To RAN WG4

ACTION: RAN1 kindly requests RAN4 to take the above information into account in their future work.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #109-e	May 16 to May 27, 2022		E-meeting

TSG-RAN WG1 Meeting #110                            August 22 - 26, 2022                               Toulouse, FR