3GPP TSG-RAN WG1 Meeting #108-e	R1-2202843

e-Meeting, February 21st – March 3rd, 2022

Title:	Reply LS on NTN-specific SIB

Release:	Release 17

Work Item:	NR_NTN_solutions

Source:	RAN WG1

To:	RAN WG2

Cc:

Contact Person:     Jussi Kahtava

jussi.kahtava@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. 	Overall Description:

RAN1 thanks RAN2 for the liaison letter introducing the new NTN-specific SIB (SIBx) which is scheduled by SIB1. After reviewing the information RAN1 respectfully informs the following:

- RAN1 has no concern with the list of nine parameters of serving cell information

- RAN1 would like to inform RAN2 about two additional parameters (ntnPolarizationDL and ntnPolarizationUL) of serving cell information, which can be found in the following RAN1 agreements

RAN1#105-e meeting:

Agreement:

For explicit indication of polarization information for DL by the network, support indication in SIB

- FFS: Signaling details for indication in SIB

Agreement:

- Polarization information for UL may be indicated in SIB by the network

- UE assumes a same polarization for UL and DL, when the UL polarization information is absent.

- FFS: Signaling details for indication in SIB

RAN1#106-e meeting:

Agreement:

When polarization signalling is present in SIB

- SIB indicates DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linear

- FFS: whether polarization signalling is per SSB

RAN1 looked at the recent RAN2 agreement on system information update:

1. Update of ephemeris and common TA information does not affect the value tag and does not trigger SI modification procedure.
2. The ntnUlSyncValidityDuration applies to the whole SIBX. UE acquires the updated SIBX when the timer expires. FFS whether to also include it in the LS to RAN1. FFS if this applies only to Connected mode or to idle mode UE as well

With respect to the validity duration, RAN1 understanding is that it only applies to ephemeris and common TA parameters including the epoch time, and the other parameters follow the normal SI modification procedure.

Some of these other parameters (e.g. K_offset) will impact the UE’s UL transmit timing. For several meetings RAN1 has been discussing whether there is a need to resolve ambiguity of which cell-specific K_offset value to use during the SIB modification period and the majority of companies think that it can be handled by gNB implementation. According to some companies this potential ambiguity has to be resolved in the specifications. RAN1 respectfully asks RAN2 to share their understanding on whether there is a need to address this potential ambiguity.

2. 	Actions:

To RAN2 group:

ACTION: 	RAN1 respectfully requests RAN2 to take the above into account and provide feedback if needed.

3. 	Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #109-e 		16 – 27 May 2022			Electronic Meeting

TSG-RAN WG1 Meeting #110 			22 – 26 August 2022			Toulouse, France