3GPP TSG RAN WG1 #108-e		           R1-2202942

e-Meeting, February 21st – March 3rd, 2022

Title:	Additional LS response to RAN2 on beam management for multi-TRP

Response to: 	R2-2203876

Release:	Rel-17

Work Item:	NR_feMIMO-Core

Source:	3GPP TSG-RAN WG1

To:	3GPP TSG-RAN WG2

Cc:

Contact Person:

Name:	Xin Su

E-mail Address:	suxin@catt.cn

Background

This is a reply LS to RAN2 to answer question 3.3 in R2-2203876.

Answer to question 3.3 in R2-2203876

In RAN1 #107e meeting, the following agreement on the answer to question 3.3 in R2-2203876 has been reached.

Agreement

For Question 3.3 from RAN2, RAN1 response is as follows:

Question 3.3: When a serving cell use inter-cell mTRP, can the UE be configured with two BFD RS sets? If yes, please explain if there is any relation between a BFD RS set and a PCI (e.g. one set associated with RS of this serving cell and another associated with RS associated with an additional PCI).

Answer: Yes, when a serving cell is configured with inter-cell mTRP, the UE can also be configured with two BFD RS sets, each associated with one different PCI. Periodic CSI-RS which is QCLed with an SSB associated with additional PCI can be configured as BFD RS associated with an additional PCI

Actions:

To RAN2 group:

ACTION: 	RAN1 respectfully asks RAN2 to take the answer to question into account in your further work.

Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #109-e 	15 – 27 May 2022		Electronic Meeting

TSG-RAN WG1 Meeting #110 	22 – 26 August 2022		Toulouse, France