3GPP TSG RAN WG1 Meeting #109-e	R1-2203055

e-Meeting, May 9 – 20, 2022

Agenda Item:	9.6.2

Source:	Futurewei

Title:	Simulations for the Rel-18 RedCap SI

Document for:	Discussion/Decision

# Introduction

After extensive discussion in 2021 on a potential 5MHz RedCap work item, in December 2021 a 6-month (two WG meetings) Rel-18 RedCap study item was approved [1]. The main concern raised during the discussion on the work item was of a “bang for the buck” nature … i.e., is it worth the standards effort for a e.g. 10% additional cost reduction in Rel-18. In a similar vein, though the SID includes studying more techniques than just 5MHz RedCap, it also includes text to help reduce standards effort, such as reusing the evaluation methodology for the evaluation of the complexity reduction techniques and targeting a single new UE type for Rel-18.

Other concerns were of course also raised in the discussions, one of which was “network impacts”, related to deployment and “especially the compatibility with Rel-17 and coexistence of RedCap and non-RedCap UEs” [2]. It was also seen as “important to avoid the difficulties the WGs are encountering right now in Rel-17 with BW reduction and BWP operation”[3].

During the setting of the agenda for RAN1#109-e, the workload of required evaluations and simulations was discussed. All agreed on the main task of evaluating the complexity reduction. However, there was no agreement on which, if any, link and system simulations are required as part of the study. Two main views were provided:

- Study what the SID says to study

- “Redo” the Rel-17 study for the new techniques in Rel-18

The first view has the merit of not expanding the workload, the second has the merit of providing a familiar picture to RAN for WI setup for the new RedCap UE type. In this paper we discuss the two views, and provide some suggestions on how to handle simulations in Rel-18 RedCap.

# Discussion

For reference, the full objectives text of the Rel-17 [4] and Rel-18 [1] RedCap SIDs is provided in the Appendix. The Rel-17 study and the Rel-18 study both have an objective on studying complexity reduction techniques, but the Rel-17 study is noticeably bigger with additional objectives such as power savings and coverage recovery. Also, the Rel-18 study specifically did not include from Rel-17 the text “The study includes evaluations of the impact to coverage, network capacity and spectral efficiency”. So, following what the SID says to study would mean that, as in TR38.875 [5], the sections analyzing the complexity reduction techniques will include qualitative statements discussing “network impact, coexistence of Rel-17 and Rel-18 RedCap and non-RedCap UEs in a cell, UE impact, specification impact”. Network impact could for example discuss deployment and BWP configuration aspects. These complexity reduction technique sections may also include possible numeric results from simulations that companies elect to submit. This occurred for example in section 7.3.3 of the Rel-17 TR with 50MHz results from a minority of companies. Essentially, simulations for the complexity reduction objective / x.y.1 complexity reduction agenda item are neither prohibited nor required.

With the view to “redo” the Rel-17 study in Rel-18, this 9.6.2 agenda item would be used in a similar way as the x.y.3 agenda item from Rel-17 on Coverage Recovery and Capacity Impact, including aspects related to “evaluations of the impact to coverage, network capacity and spectral efficiency” that was removed from the Rel-18 SID. The argument for this view is to interpret “network impact” to mean that “impact to coverage, network capacity and spectral efficiency” is required for every technique, and that “based on Rel-17 evaluation methodology” means all of the methodology must be used, not just the complexity aspects. As there could easily be a dozen or more variants of complexity reduction techniques proposed in x.y.1, the workload for this view could be quite large.

However, the room for complexity reductions in Rel-18 is much smaller than in Rel-17 in part because these are further reductions on top of Rel-17. It may be that a dozen different variants are included in the Rel-18 TR, but the spread of additional complexity reduction is expected to be quite constrained as the techniques are very similar. From that perspective, it may not be worth the effort to “redo” the Rel-17 study for every technique. Instead, providing quantitative results for “UE bandwidth reduction to 5MHz” and the eventual “single Rel-18 RedCap UE type” (if not the 5MHz solution) may be sufficiently helpful for the WI discussion in RAN. Companies are always free to provide other results, particularly if they feel it would help their technique be selected for the single type, but they need not be required for every proposed solution. In addition, RAN1 can also focus efforts on certain types of simulations. For example, a single link budget calculation may be sufficient to determine that there are no coverage issues with low data rate Rel-18 RedCap UEs, while multiple spectral efficiency simulations could be used to compare techniques. And no network capacity simulations may be needed at all, as the network impact concern is mainly of a deployment nature and not on whether low data rate UEs are worth supporting in the network from a capacity perspective.

Observation: Simulations may be included in the Rel-18 TR but they are not required for the study to be concluded and a Rel-18 RedCap WI to begin.

Proposal 1: RAN1 to discuss whether/which simulation results to plan to include in the Rel-18 TR.

Proposal 2: Any planned simulations should include UE bandwidth reduction to 5MHz. Other simulations are not prohibited but including in the TR should be discussed later.

# Conclusion

We observe and propose:

Observation: Simulations may be included in the Rel-18 TR but they are not required for the study to be concluded and a Rel-18 RedCap WI to begin.

Proposal 1: RAN1 to discuss whether/which simulation results to plan to include in the Rel-18 TR.

Proposal 2: Any planned simulations should include UE bandwidth reduction to 5MHz. Other simulations are not prohibited but including in the TR should be discussed later.

# References

RP-213661, “Study on further NR RedCap UE complexity reduction”, Ericsson, Dec. 2021.

RP-212221, Moderator’s summary for discussion [RAN93e-R18Prep-07] RedCap evolution, Sept. 2021.

RP-212665, Moderator’s summary for discussion [RAN94e-R18Prep-05] RedCap Evolution, Dec. 2021.

RP-202704, “Study on support of reduced capability NR devices”, Ericsson, Dec. 2020.

TR 38.875, Study on support of reduced capability NR devices, v17.0.0, March 2021.

# Appendix

Rel-17 SID objectives from RP-202704:

The study item includes the following objectives:

Identify and study potential UE complexity reduction features, including [RAN1, RAN2]:

- Reduced number of UE RX/TX antennas

- UE Bandwidth reduction

Note: Rel-15 SSB bandwidth should be reused and L1 changes minimized

- Half-Duplex-FDD

- Relaxed UE processing time

- Relaxed UE processing capability

The study includes evaluations of the impact to coverage, network capacity and spectral efficiency

Note1: The work defined above should not overlap with LPWA use cases. The lowest data rate and bandwidth capability considered should be no less than an LTE Category 1bis modem.

Study UE power saving and battery lifetime enhancement for reduced capability UEs in applicable use cases (e.g. delay tolerant) [RAN2, RAN1]:

- Reduced PDCCH monitoring by smaller numbers of blind decodes and CCE limits [RAN1].

- Extended DRX for RRC Inactive and/or Idle [RAN2]

- RRM relaxation for stationary devices [RAN2]

Study functionality that will enable the performance degradation of such complexity reduction to be mitigated or limited, including [RAN1]:

- Coverage recovery to compensate for potential coverage reduction due to the device complexity reduction.

  - Note: For FR1, coverage analysis for wearables can include consideration of potential reduced antenna efficiency due to device size limitations as part of the antenna gains. The extent of additional recovery of coverage loss due to reduced antenna efficiency is to be limited to 3 dB

- The study includes evaluations of the impact to network capacity and spectral efficiency

Study standardization framework and principles for how to define and constrain such reduced capabilities – considering definition of a limited set of one or more device types and considering how to ensure those device types are only used for the intended use cases [RAN2, RAN1].

Study functionality that will allow devices with reduced capabilities to be explicitly identifiable to networks and network operators, and allow operators to restrict their access, if desired [RAN2, RAN1].

Note2: Potential overlap with coverage enhancements study is discussed and resolved in RAN#87 or later.

Note3: Coexistence with Rel-15 and Rel-16 UE should be ensured

Note4: This SI should focus on SA mode and single connectivity

Rel-18 SID objectives from RP-213661:

To further reduce the complexity of RedCap devices, the following should be studied, and the results should be captured in TR 38.8xx:

- Study further UE complexity reduction techniques based on Rel-17 evaluation methodology in TR 38.875 [RAN1]

  - Consider network impact, coexistence of Rel-17 and Rel-18 RedCap and non-RedCap UEs in a cell, UE impact, specification impact

  - Potential solutions, which may complement each other, for reducing device complexity are focusing on:

    - UE bandwidth reduction to 5MHz in FR1,

      - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI

    - reduced UE peak data rate in FR1,

      - Possibly including restricted bandwidth for PDSCH and/or PUSCH

      - Possibly in combination with relaxed UE processing timeline for PDSCH and/or PUSCH and/or CSI

- Notes:

  - Rel-15 SSB should be reused and L1 changes minimized.

  - Operation in BWP with/without SSB and without/with RF retuning should be considered.

  - It is not precluded that some solutions for FR1 can be applied to FR2 in WI stage.

  - Aim to define a single Rel-18 RedCap UE type for further UE complexity reduction.