3GPP TSG RAN WG1 Meeting #109-e	R1-2203056

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.2.4

Source:	Futurewei

Title:	Remaining Details in Channel access for Beyond 52.6 GHz

Document for:	Discussion/Decision

# Introduction

In RAN # 90 the extensions to WI [1] for NR operation up to 71GHz were approved. According to [1] RAN1 should define:

“Physical layer procedure(s) including [RAN1]:

Channel access mechanism assuming beam based operation in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6GHz and 71GHz.

Specify both LBT and No-LBT related procedures, and for No-LBT case no additional sensing mechanism is specified.

Study, and if needed specify, omni-directional LBT, directional LBT and receiver assistance in channel access.

Study, and if needed specify, energy detection threshold enhancement”

Multiple Agreements were reached in RAN1#104-e through RAN1#108-e based on the above principles. In this document we provide our views on the remaining details after recalling the relevant agreements.

# EDT Computation

In this section we address few open issues related to the independent per-beam LBT sensing.

## Multiple independent LBT Sensing Beams

The support for independent per-beam LBT sensing as a method to initiate a COT with TDM of beams with beam switching was agreed [RAN1#107, R1-2112820]:

Agreement (SDM Case)

For a COT with MU-MIMO (SDM) transmission, support both Alt 1 and Alt 2 below:

- Alt 1: Single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold

- Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT, if the node can perform simultaneous sensing in different beams

Note: On UE side, no UE capability will be introduced for this purpose.

Agreement (TDM Case)

Within a COT with TDM of beams with beam switching, Alt 2 is supported if the node has the capability to perform simultaneous sensing in different beams. Alt 3 is allowed as node implementation choice if the node also supports Cat 2 LBT. The use of Alt 2 or Alt 3 is based on node’s implementation.

- Alt 2 from previous agreement: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT

- Alt 3 from previous agreement: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT with additional requirement on Cat 2 LBT before beam switch

When the independent per-beam LBT sensing is performed at gNB the following condition for transmission on a particular beam was agreed [RAN1#108, R1-2202493]:

Agreement

When independent per-beam LBT sensing is performed at gNB, a transmission is allowed to occur on a beam if the corresponding LBT procedure has been successful before the channel occupancy start time.

- Note: For multi-beam transmission, channel occupancy start time corresponding to all Tx beams is aligned.

- FFS: When independent per-beam LBT sensing is performed at UE

The above agreements leave open for interpretation how some possible scenarios are handled. One possible scenario is when the independent per-beam LBT sensing is performed some of the per-beam LBT sensing succeed and some fail. Based on the previous agreement the gNB may start transmitting only on those directions where LBT succeeded.

Observation 1: When a node performs independent per-beam LBT sensing it does not know in advance which LBT will be successful and which one will fail.

The following agreement defines the Pout in EDT determination [RAN1#107, R1-2112820]:

Agreement

Confirm the WA with some clarifications

Working assumption:

- For Pout in EDT determination, define Pout as the maximum EIRP of the intended transmissions by the node determining EDT during a COT.

  - The node is not expected to transmit in the COT with higher Pout than the Pout used to determine the EDT used to acquire the COT

Each beam has potentially a different gain therefore the EIRP of transmissions along different beams may be substantially different. However, without knowing in advance which per-beam LBT sensing will be successful it makes difficult to decide which Pout will be used in transmissions during COT, and thus which EDT value should be used.

The TS 37.213 specifications the “per beam sensing” LBT Type 1 is supported by the second sub-bullet do not help with EDT value selection:

“If a channel occupancy includes transmission(s) in different beams that are multiplexed in spatial domain, one of the followings is applicable for the corresponding sensing to perform the transmission(s) within the channel occupancy:

-	Type 1 channel access procedure as described in Clause 4.4.1 is applied before the start of the channel occupancy using a single sensing beam where the single beam covers all the transmission beams within the channel occupancy. When the channel is accessed, the transmission(s) within the channel occupancy across different beams can occur.

-	Type 1 channel access procedure as described in Clause 4.4.1 is applied before the start of the channel occupancy simultaneously per sensing beam where each sensing beam covers a transmission beam within the channel occupancy. When the channel is accessed, the transmission(s) within the channel occupancy across different beams can occur. “

Observation 2: Clarifications of the EDT value(s) selection for independent per beam channel sensing are necessary.

We identified two possible solutions for EDT value selection for per beam sensing. One solution could be to select the largest Pout of all intended transmissions for that COT. This is the simplest solution.

When some LBT directions are successful and some not, a node may utilize spatial reuse opportunities and perform transmissions only on those beams (directions) where LBT sensing was successful. For such a scenario, it is evident that using a separate EDT for each sensing beam is beneficial. In particular, the Pout definition that is used in the EDT computed for each sensing beam must consider the transmit power and beamforming gain of the intended transmit beam (via a specific EIRP). If a single EDT value is used, that value may be (erroneously) calculated when it is based on the transmission power Pout intended some direction, which will not be used because the LBT for that direction failed.

Using a common maximum EIRP across all sensing beams may also degrade the spatial reuse possibilities thereby negating the main advantage of multiple per-beam LBT sensing.

Indeed, suppose that in one direction the power (Pout, EIRP) necessary to be used is 10 dB higher than the power (Pout, EIRP) to be used in a second direction (beam). In this case, using the same common EDT for both sensing beams will limit the communication opportunity for the second direction.

That is, a common EDT for both directions may lead to LBT failure on second beam (direction) even though given the Pout (EIRP) to be used on the second direction would allow a higher EDT value, when EDT formula is used.

Observation 3: Always using a common Pout (common EDT) for multiple sensing beams can limit spatial reuse, which may increase the latency and the number of transmissions.

The second solution for EDT value selection may be to allow different EDT values for each sensing beam, where the EDT values are corresponding to the intended transmit power for each beam.

Proposal 1: Adopt the changes proposed in TP1.

# No LBT and Contention Exempt Signaling

The following agreement has been made in Rel. 17 [RAN1#105, R1-2106193].

Agreement:

For regions where LBT is not mandated, gNB should indicate to the UE this gNB-UE connection is operating in LBT mode or no-LBT mode

- Support both cell specific (common for all Ues in a cell as part of system information or dedicated RRC signalling or both) and UE specific (can be different for different Ues in a cell as part of UE-specific RRC configuration) gNB indication.

One issue to be resolved is whether the UE should be directed to assume a certain mode for gNB’s operation (i.e., either LBT or no-LBT). We recall that when the gNB uses LBT for access there is a possibility of it being unable to access the channel and consequently being unable to transmit signals/channels such as periodic CSI-RS. Then, if the UE is directed to use no-LBT/LBT mode for its access using only cell-specific indication, it can assume that gNB will also use the indicated mode. On the other hand, if the UE is configured to use no-LBT mode for its access using UE-specific RRC indication it can assume that the gNB will also use the no-LBT mode in which case it assumes that all periodic-RS will always be transmitted by the gNB.

The remaining open case is if the UE is directed (in a UE-specific manner) to use LBT mode for accessing the channel, then there is an ambiguity on whether it should assume gNB to be using LBT mode as well. Note that the UE’s assumption could be different from the actual mode used by the gNB since that gNB would be catering to other users as well. This assumption can potentially be used by the UE to address impact of LBT failures in gNB transmission of periodic RS etc., for instance via an additional validation stage. Our preference in this case is to not specify any additional assumption for the UE. We recall that in Rel.17 no specific RS enhancement to deal with LBT failures was agreed (for regions with mandatory LBT). One key argument was that the UE can set appropriate thresholds for failure counters/timers that were already provisioned in Rel.16 to deal with beam failures. This was argued to be sufficient to deal with beam link failures interspersed with LBT failures. Extending the same argument, we prefer to rely on an implementation-based setting of thresholds instead of explicitly configuring UE assumption about gNB’s mode of operation.

Proposal 2: For regions where LBT is not mandated, gNB should indicate to the UE whether the gNB-UE connection is operating in LBT mode or no-LBT mode

- Support both cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) and UE specific (can be different for different UEs in a cell as part of UE-specific RRC configuration) gNB indication

When LBT mode or no-LBT mode is indicated to a UE, the mode applies to the UE for the operation between the gNB and UE.

Cell specific indication for LBT/No-LBT can be provided in SIB1. UE for the initial access phase uses the LBT-Mode signaled in SIB1.  After UE’s connection status changes to RRC_CONNECTED a dedicated configuration can also be provided to that UE to enable or disable LBT for its channel access.

Further, indication of LBT/No LBT mode of channel access per UE may also be provided in MSG2 during the initial random access, which can speed-up the UE’s initial access. For UE-specific gNB indication of LBT/No-LBT mode, (MAC-CE) configuration or RRC configuration can be used. Moreover, mode indication (LBT type indication) in scheduling DCI should be defined for FR2-2 as well.

Proposal 3: Priority or precedence rules should be defined to address the scenarios when UE receives multiple types of LBT/No-LBT mode indications.

In the RAN1 #108 the following proposal [RAN1 Chair notes] for LBT indication was considered and Alt 2 had the most support from companies, however a consensus was not achieved.

Proposal 2.15-1 (with GTW revisions)

Down-select between the next two alternatives

- Alt 1. Introduce one bit in SIB1 indicates whether LBT is required for all UL transmissions

  - If the bit is set to true, msg1 and msgA cannot be transmitted with Contention Exempt Short Control Signaling based transmission. If the bit is set to false, msg1 and msgA can be transmitted by with Contention Exempt Short Control Signaling based transmission if requirement of 10% over 100ms is satisfied

    - It is a separate discussion if the requirement of 10% over 100ms is per UE or per cell

  - For fallback DCI formats 0_0 and 1_0 and RAR UL grant, for FR2-2 operation, the ChannelAccess-Cpext field in DCI indicates the channel access type only. A new table similar to Table 7.3.1.1.1-4 is introduced with entries “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213 or Type 3 channel access in 4.4.3 of 37.213”. For the 2nd entry, if the bit is set to true, it will be interpreted as “Type 2 channel access in 4.4.2 of 37.213”. If the bit is set to false, it will be interpreted as “Type 3 channel access in 4.4.3 of 37.213”

  - For an UL transmission indicated or configured to use Type 1 channel access, if the UE later finds out, through DCI 2_0 detection, the transmission falls in a gNB COT, the UE can change the channel access type to Type 2 channel access if the bit is set to true and the UE can change the channel access type to Type 3 channel access if the bit is set to false

- Alt 1A Introduce one bit in SIB1 indicates whether LBT is required for all UL transmissions

  - If the bit is set to true, msg1 and msgA cannot be transmitted with Contention Exempt Short Control Signaling based transmission. If the bit is set to false, msg1 and msgA can be transmitted by with Contention Exempt Short Control Signaling based transmission if requirement of 10% over 100ms is satisfied

    - It is a separate discussion if the requirement of 10% over 100ms is per UE or per cell

  - For an UL transmission indicated or configured to use Type 1 channel access, if the UE later finds out, through DCI 2_0 detection, the transmission falls in a gNB COT, the UE can change the channel access type to Type 2 channel access if the bit is set to true and the UE can change the channel access type to Type 3 channel access if the bit is set to false

  - UE as initiating device can resume transmission within maximum COT without a type 2 LBT, no matter how long the gap is from the previous transmission from initiating device or responding device, if the bit is set to false. If the bit is set to true, UE as initiating device can resume transmission after a type 2 LBT if device is capable.

- Alt 1B. Introduce one bit in SIB1 indicates whether LBT is required for all UL transmissions

  - If the bit is set to true, msg1 and msgA cannot be transmitted with Contention Exempt Short Control Signaling based transmission. If the bit is set to false, msg1 and msgA can be transmitted by with Contention Exempt Short Control Signaling based transmission if requirement of 10% over 100ms is satisfied

    - It is a separate discussion if the requirement of 10% over 100ms is per UE or per cell

  - For fallback DCI formats 0_0 and 1_0 and RAR UL grant, for FR2-2 operation, the ChannelAccess-Cpext field in DCI indicates the channel access type only. A new table similar to Table 7.3.1.1.1-4 is introduced with entries “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213” and “Type 3 channel access in 4.4.3 of 37.213”, and “reserved”. If the bit in SIB is set to true, UE does not expect a DCI indicating “Type 3 channel access in 4.4.3 of 37.213”

  - For an UL transmission indicated or configured to use Type 1 channel access, if the UE later finds out, through DCI 2_0 detection, the transmission falls in a gNB COT, the UE can change the channel access type to Type 2 channel access if the bit is set to true and the UE can change the channel access type to Type 3 channel access if the bit is set to false

- Alt 2.

  - gNB provides RRC configuration in SIB1 to indicate if msg1 or msgA transmission with Contention Exempt Short Control Signaling based transmission is allowed.

    - It is a separate discussion if the requirement of 10% over 100ms is per UE or per cell

    - This RRC configuration is common for msg1 and msgA

    - This RRC configuration is separate from the ChannelAccessMode2-r17

  - Introduce one bit in SIB1 indicates whether LBT is required for all UL transmissions

  - For fallback DCI formats 0_0 and 1_0 and RAR UL grant, for FR2-2 operation, the ChannelAccess-Cpext field in DCI indicates the channel access type only. A new table similar to Table 7.3.1.1.1-4 is introduced with entries “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213” and “Type 3 channel access in 4.4.3 of 37.213”, and “reserved”.

    - Note: This option requires 2 bits in fallback DCI

    - TP 2.9-C

  - For an UL transmission indicated or configured to use Type 1 channel access, if the UE later finds out, through DCI 2_0 detection, the transmission falls in a gNB COT, the UE can change the channel access type to Type 2 channel access or Type 3 channel access.

    - FFS: RRC configuration is introduced to indicate either Type 2 channel access or Type 3 channel access will be used, subject to UE capability

  - FFS: For UE as initiating device, if additional RRC configuration is introduced to indicate if the UE can resume transmission within maximum COT without a type 2 LBT, no matter how long the gap is from the previous transmission from initiating device or responding device, or this is left for UE implementation

In this proposed agreement the following open issues are considered:

- Indication of LBT for all UL transmissions

- Indication of msg1 or msgA transmission with Contention Exempt Short Control Signalling based transmission is allowed

- Indication in fallback DCI formats 0_0 and 1_0 and RAR UL grant of channel access type via CAPC field

- The UL channel access type when UE is transmitting during a gNB initiated COT

- The UL channel access type after a gap when UE is transmitting during a UE initiated COT

In our opinion Alt 2 offers more flexibility for signaling with respect to Alt 1 therefore is preferable. We also note that the last FFS seems to be covered by the precedent FFS sub-bullet, therefore it is not necessary.

Proposal 4: Support Alt 2 of the above proposed agreement for LBT and Short Control Signaling indication (Proposal 2.15-1), without the last FFS bullet.

# Conclusion

Observation 1: When a node performs independent per-beam LBT sensing it does not know in advance which LBT will be successful and which one will fail.

Observation 2: Clarifications of the EDT value(s) selection for independent per beam channel sensing are necessary.

Observation 3: Always using a common Pout (common EDT) for multiple sensing beams can limit spatial reuse, which may increase the latency and the number of transmissions.

Proposal 1: Adopt the changes proposed in TP1.

Proposal 2: For regions where LBT is not mandated, gNB should indicate to the UE whether the gNB-UE connection is operating in LBT mode or no-LBT mode

- Support both cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) and UE specific (can be different for different UEs in a cell as part of UE-specific RRC configuration) gNB indication

When LBT mode or no-LBT mode is indicated to a UE, the mode applies to the UE for the operation between the gNB and UE.

Proposal 3: Priority or precedence rules should be defined to address the scenarios when UE receives multiple types of LBT/No-LBT mode indications.

Proposal 4: Support Alt 2 of the above proposed agreement for LBT and Short Control Signaling indication (Proposal 2.15-1), without the last FFS bullet.

# References

- RP-202925, Revised WID: Extending current NR operation to 71 GHz, CMCC, RAN#90e, 2020.

- Chairman's Notes RAN1#107-e.

- R1-2112820 – Email discussion summary for channel access for B52 -final (RAN1#107), October 2021

- R1-2202493 - FL summary of channel access mechanism for 52.6GHz-71GHz band (RAN1#108) February 2022

- R1-2106193 - FL summary for channel access of 52.6 GHz to 71GHz (RAN1#105), May 2021

- Chairman's Notes RAN1#108-e.

- ETSI EN 302 567 Multiple-Gigabit/s radio equipment operating in the 60 GHz band; Harmonized Standard for access to radio spectrum.

# Annex

TP #1

| Reason for change: | Lack of clear specifications to select EDT for LBT sensing when a node performs independent per-beam LBT sensing |
| --- | --- |
| Summary of change: | Add text to allow a separate EDT per each sensing beam LBT |
| Consequences if not approved: | The EDT selection text for per-beam sensing LBT it is not clear. |

TP#1

TS 37.213, 4.4 Channel access procedures for frequency range 2-2 *** Unchanged text omitted *** If a channel occupancy includes transmission(s) in different beams that are multiplexed in spatial domain, one of the followings is applicable for the corresponding sensing to perform the transmission(s) within the channel occupancy: - Type 1 channel access procedure as described in Clause 4.4.1 is applied before the start of the channel occupancy using a single sensing beam where the single beam covers all the transmission beams within the channel occupancy. When the channel is accessed, the transmission(s) within the channel occupancy across different beams can occur. - Type 1 channel access procedure as described in Clause 4.4.1 is applied before the start of the channel occupancy simultaneously per sensing beam where each sensing beam covers a transmission beam within the channel occupancy. The EDT value used per each sensing beam may be different. When the channel is accessed, the transmission(s) within the channel occupancy across different beams can occur. If a channel occupancy includes transmissions in different beams that are multiplexed in time domain, one of the followings is applicable for the corresponding sensing to perform the transmissions within the channel occupancy: - Type 1 channel access procedure as described in Clause 4.4.1 is applied before the start of the channel occupancy using a single sensing beam where the single beam covers all the transmissions beams within the channel occupancy. When the channel is accessed, the transmissions within the channel occupancy across different beams can occur following the procedures described in Clause 4.4.3. - When the gNB/UE can perform simultaneous sensing in different beams, Type 1 channel access procedure as described in Clause 4.4.1 is applied before the start of the channel occupancy per sensing beam where each sensing beam covers a transmission beam within the channel occupancy. The EDT value used per each sensing beam may be different. When the channel is accessed, the transmission within the channel occupancy across different beams can occur following the procedures described in Clause 4.4.3. - When the gNB/UE can perform simultaneous sensing in different beams, Type 1 channel access procedure as described in Clause 4.4.1 is applied before the start of the channel occupancy per sensing beam where each sensing beam covers a transmission beam within the channel occupancy. The EDT value used per each sensing beam may be different. When the channel is accessed, the transmission within the channel occupancy can occur following the procedures in Clause 4.4.2 before switching to a different beam within the channel occupancy. When the gNB intends to transmit a DL transmission(s) across multiple transmission beams, if the gNB performs sensing on the corresponding sensing beam(s) independently, the DL transmission(s) can occur on a transmission beam(s) among the multiple transmission beams if the channel access procedures on the corresponding sensing beam(s) have succeeded, and the channel occupancy would start at the same time across the multiple transmission beams. The EDT value used per each sensing beam may be different. *** Unchanged text omitted ***