3GPP TSG RAN WG1 Meeting #109-e	R1-2203057

e-Meeting, May 9th – 20th, 2022

Agenda Item:	9.5.1.1

Source:	Futurewei

Title:	Considerations on scenarios and target requirements for sidelink positioning

Document for:	Discussion/Decision

# Introduction

In RAN#94e the SID for “Study and improved NR positioning” was approved [RP-213588]. The first objective of the SI is to:

“Study solutions for sidelink positioning considering the following: [RAN1, RAN2]

- Scenario/requirements

  - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage

  - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104

  - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104)

  - Spectrum: ITS, licensed

- Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1]

- Define evaluation methodology with which to evaluate SL positioning for the use cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1].

- Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2]

  - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1]

  - Study of positioning methods (e.g., TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1]

  - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1]

  - Study of positioning architecture and signalling procedures (e.g., configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network-based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required]

Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.”

# SL Positioning Scenarios

According to the improved NR positioning SID [1], RAN1 should study sidelink (SL) positioning solutions for various scenarios and requirements.

In this contribution, for positioning purposes, we define the target UE as the UE that requires position determination, and the anchor nodes those devices such as UEs, RSU, or gNB that are used as references for position determination. The anchor nodes may be used to determine the relative or absolute position of the target node. In the scenarios analyzed we consider at least the PC5 interface that is present and used for positioning purposes.

Observation 1: A SL positioning scenario as the positioning scenario where at least one sidelink wireless link is used to carry signals for position determination of a target UE.

The solutions to be studied in this SID should be able to address multiple use cases such as V2X (TR38.845, TS 22.186), public safety (TR38.845), commercial (TS22.261), and IIOT (TS22.104).

Moreover, based on the coverage, the V2X positioning solutions should encompass scenarios when the target SL UE is in coverage of gNB and/or Road Side Units (RSUs), scenarios when target UE and/or anchor nodes (such SyncRef UEs) are in partial coverage and respectively scenarios when target UE and anchor nodes are out of coverage.

It was determined, in the previous study [TR 38.845], that most of positioning scenarios exhibit non-line of sight propagation (NLOS) conditions, which contribute substantially to the degradation of the accuracy of position determination. Thus, the positioning solutions should be able to operate in scenarios with NLOS propagation between the target and the anchor nodes.

In this contribution we will take a closer look at the scenarios and the associated requirements for sidelink positioning.

# V2X scenarios

The 3GPP system (TS22.261, TS 22.186) is expected to support various enhanced V2X scenarios such as:

- Vehicles Platooning,

- Advanced Driving (semi-automated, fully automated driving),

- Extended Sensors (exchange of raw or processed data gathered through local sensors or live video data among vehicles, RSUs, devices of pedestrians and V2X application servers)

- Remote Driving

- Vehicle Quality Service Support (timely notifications of expected or estimated change of quality of service before actual change occurs)

It is expected that some scenarios, such as outdoor dense urban, the NLOS propagation and limited GNSS access will pose more difficulties for location determination. On the other hand, other outdoor scenarios such as the highways will pose different type of challenges determined by the high vehicle speeds (Doppler phase shift).

## Coverage

Figure 1 shows various scenarios to be supported by the V2X positioning solutions.  The target UE may be in coverage of gNB (Uu link) or in the coverage of Roadside Units (RSU with PC5 link). When the target UE is in partial coverage there is no direct link to gNB or RSU, however the network is reachable through a SL relay. It expected that a Location Management Function (LMF) will orchestrate the positioning configurations of the reference signals and their measurements for the in coverage and partial coverage situations.

As Figure 1 suggests, it is expected that Uu interface (uplink and downlink), PC5 interface (sidelink), and their combinations may be considered as the radio link for positioning. In the Figure 1 S-PRS denotes the SL Positioning RS, necessary for positioning purposes and carried over PC5.

Uu interface is available for UEs under in-coverage scenario, and for UEs under network coverage in partial coverage scenario, while PC5 interface is available for UEs under in-coverage, partial coverage, out-of-coverage scenarios.

For the out of coverage situations, both target UE and anchor UEs do not have a direct link to the network (gNB, or RSU). For these scenarios, RAN1 needs to study and define how the management of SL positioning is implemented.

Figure 1, Examples of V2X scenarios in coverage, partial coverage and out of covverage

## Position Calculation Entity

5GAA [6,7,8] and 3GPP [TR 38.845] identify two configuration cases based on the calculation entity. The UE based solutions, where the target UE collects the measurements and estimates its location, and the UE assisted solutions, where the target UE collects some or all measurements and send them to the network where LMF estimates the location. We note that the type of the position calculation entity does not limit which interface(s) can be used for the radio link(s). The UE based solutions may not require the connectivity to an LMF entity.

## Spectrum

The Intelligent Transportation System (ITS) applications will deploy the V2X communications in dedicated spectrum.

- In November 2020, the FCC adopted a Report & Order (R&O) in docket number 19-138 [7] revising its rules for the 5850-5925 MHz band, which had been dedicated exclusively for DSRC. The R&O concluded that the United States should move forward with C-V2X in the 5895-5925 MHz portion of the band.

- In October 2018, in China, the MIIT decided to make 5905-5925 MHz available for use by LTE-V2X (PC5) in China on a technology specific basis, with the lower 10 MHz block for V2V communications, and the upper 10 MHz block for V2I/I2V communications.

- In Europe 5855-5875 MHz is designated for non-safety road-ITS, whereas 5875-5935 MHz is designated for safety related ITS.

Different V2X scenarios require the transport of V2X messages with different performance requirements for the 3GPP system. Depending on the requirements some of the services V2X may be deployed in the ITS speactrum while others can be deployed in the licensed spectrum. For instance, the services that require basic safety may be in ITS spectrum and advanced services using for example unicast communication may use licensed spectrum. The positioning accuracy is expected to be related to the amount of available spectrum.

In TR 38.855 the BW considered for positioning takes values of 5, 50 and 100 MHz (FR1), and 100, 400 MHz (FR2). These values may cover both the ITS and the licensed applications of the V2X positioning. A SL UE may be able to use for SL positioning references with different signaling and bandwidth sizes.

Proposal 1: The SL positioning solutions should be able to cover and combine a wide range of signaling BW sizes from 5, 10, and 20 MHz (ITS), 5, 50, and 100 MHz (FR1), to 100, 400 MHz (FR2).

## Absolute and Relative Positioning

Depending on the scenario and the available anchor position the SL positioning method may provide the absolute position or the relative position:

- Absolute position: the longitude and latitude of the SL UE target device are calculated. An anchor with known location (gNB, RSU or another vehicle) is required to calculate absolute position

- Relative position: the relative position such as the distance or the relative angle between the target UE and anchor nodes is calculated. This is also known as “sidelink ranging”, and it can be useful for collision warning for instance.

TS 22.104 defines the necessity of relative position determination for out-of-coverages situations.

- “The 5G system shall provide positioning information for a UE that is out of coverage of the network, with accuracy of < [1 m] relative to other UEs that are in proximity and in coverage of the network

Observation 2: The SL positioning scenarios will require solutions for absolute and relative position determination.

## V2X positioning requirements

5GAA [6] identifies three types of services over C-V2X: traffic safety, traffic efficiency and information services. Different positioning requirements are defined in each service scenario. While [6] offers an extensive list of use cases and requirements [Table 5.1-1], in general the services may be classified in three categories.

Examples of C-V2X scenarios that require tens of meters level accuracy:

- Traffic jam warning: 20m (one sigma)

- Software update: 30m (one sigma)

- HD content delivery: 30 to 50m (one sigma)

Examples of C-V2X scenarios that require meter level accuracy:

- Intersection Movement Assist:1.5m (three sigma)

- Lane changing warning :1.5m (one sigma)

- Vulnerable road user – awareness: 1m (three sigma)

- Vehicle platooning in steady state: 1.5m (three sigma)

- Cooperative lane merge: 1.5m (three sigma)

Examples of C-V2X scenarios that require sub-meter level accuracy:

- Infrastructure-based tele-operated driving: 0.1m (three sigma)

- Automated intersection crossing: 0.15m (three sigma)

- Cooperative manoeuvres for autonomous vehicles for emergency situations: 0.2m (three sigma)

- Vulnerable road user – collision warning: 0.5m (three sigma)

These requirements are defined for 3D/2D scenarios, for absolute and relative position determination including out-of-coverage scenarios.

The above constraints for the accuracy levels must be satisfied for various speeds of the vehicles (Doppler shift) and various densities of the vehicles. The vehicle velocities considered by 5GAA vary from stationary scenarios (0 km/h for Accident report) and very low speeds of 10-20 km/h (obstructed view assist via CCTV) to medium speeds of 50-70 km/h (tele-operated driving and group start, vulnerable road user urban collision warning) and to higher speeds (120km/h set-through for pass maneuver, 250km/h remote automated driving cancellation, real-time situational awareness and high-definition maps, or emergency break warning).

Proposal 2: For SL V2X scenarios consider three category of velocity levels: low velocity (less than 20km/h), medium velocity (20-100km/h) and high velocity (100-250km/h).

The vehicle density is another characteristic of the positioning scenarios. Examples of such densities are (in units/km^2): 1500 for cross-traffic left-turn assist, 12000 for intersection movement assists or emergency breaking, and 4500 for traffic jam warning. The unit density may influence the interference and latency of the SL positioning.

Observation 3: The SL V2X positioning solutions should be able to operate in multiple service areas and channel propagation conditions such as:

- Rural (9000 units/km^2)

- Urban (12000 units/km^2) both Micro and Micro

- Highway (4500 units/km^2)

# IoT

The high accuracy positioning requirements for factories are defined in TS22.104. Such applications for high accuracy positioning are for tracking of mobile devices as well as of mobile assets. The 5G system shall provide positioning information for a UE out of coverage with accuracy < [1m] relative to other UEs that are in its proximity. The scenarios and the requirements for horizontal and vertical accuracy are provided in Table 5.7.1-1 in TS 22.104 (Appendix A). The study will consider SL positioning solutions for the factory scenarios presented in TS 22.104. We note that the requirements for such scenarios vary from high accuracy of 0.5m horizontal and 0.3m vertical with very low latency (delay) of 150ms for drone control to lower accuracy and higher delay for emergency calls of 50 m horizontal, 3m vertical and 30 seconds latency.

Observation 4: IoT applications may be require positioning solutions that provide very high accuracy of 0.5m horizontal and 0.3m vertical with very low latency of 150ms in outdoor service areas.

We note that these requirements do not exclude other forms of positioning than 5G, therefore is expected that for very stringent requirements hybrid solutions will be considered (for instance usage of RAT-independent techniques such as radars, accelerometers, gyroscopes, etc.). Moreover, scenarios like remote UAV control may benefit of LOS propagation between target and anchor nodes.

# Public Safety

Public safety positioning requirements are defined in TS 22.261 and TS 22.280.

TS 22.261 provides positioning requirements for the mission critical organizations.

“Mission Critical Organizations require mission critical services to have accurate positioning such that first responders may be located at all times during normal and critical operations, indoors as well as outdoors. The level of positioning accuracy (and other KPIs) required is much more stringent than that required by local and regional regulatory requirements for commercial 5G users.”

The high accuracy positioning requirements are captured in TS 22.661 Table 7.3.2.2-1 (Appendix A)

The horizontal accuracy  for absolute precision based on the positioning service level is between 0.3m and 10 m, while the vertical accuracy is between 2m and 3m. For relative positioning the requirement is more stringent than the absolute positioning, and it is 0.2m both for horizontal and vertical.

Observation 5:  5G solutions for public safety scenarios may require very high accuracy of 0.2m horizontal and 0.2m vertical within 1s delay for relative indoor positioning, and 0.3m horizontal and 2m vertical within 1s delay for absolute high speed outdoor positioning scenarios.

# SL Positioning Target Performance Requirements

It is expected that most of SL UE will support the UE positioning as defined in Rel 16 and Rel 17 when conditions allow, i.e., when UE is in gNB coverage. Therefore, a good starting point for the positioning requirements are the requirements defined in TR 38.855.

At the same time, taking into consideration the requirements for V2X, IoT and public safety as presented above we suggest the following requirements as a starting point of the discussions for SL positioning scenarios:

Proposal 3: For SL positioning consider three category of horizontal position accuracy levels: tens of meters [10-20m], meters[1-3m], and sub-meter [0.2-0.5m]. Consider vertical position accuracy level of sub meter [<1m] and of above meter[1-3m].

Proposal 4: For SL positioning consider the latencies (delays) levels requirements [150ms, 1s].

Few clarification remarks should be considered together with above proposals:

Observation 6: It is understood that there is no single positioning technology or method that must meet all the target performance requirements for every scenario.

Observation 7: Some of the SL positioning target performance requirements may be satisfied through a combination of RAT independent and RAT dependent solutions.

# Conclusions

Observation 1: A SL positioning scenario as the positioning scenario where at least one sidelink wireless link is used to carry signals for position determination of a target UE.

Proposal 1: The SL positioning solutions should be able to cover and combine a wide range of signaling BW sizes from 5, 10, and 20 MHz (ITS), 5, 50, and 100 MHz (FR1), to 100, 400 MHz (FR2).

Observation 2: The SL positioning scenarios will require solutions for absolute and relative position determination.

Proposal 2: For SL V2X scenarios consider three category of velocity levels: low velocity (less than 20km/h), medium velocity (20-100km/h) and high velocity (100-250km/h).

Observation 3: The SL V2X positioning solutions should be able to operate in multiple service areas and channel propagation conditions such as:

- Rural (9000 units/km^2)

- Urban (12000 units/km^2) both Micro and Micro

- Highway (4500 units/km^2)

Observation 4: IoT applications may be require positioning solutions that provide very high accuracy of 0.5m horizontal and 0.3m vertical with very low latency of 150ms in outdoor service areas.

Observation 5:  5G solutions for public safety scenarios may require very high accuracy of 0.2m horizontal and 0.2m vertical within 1s delay for relative indoor positioning, and 0.3m horizontal and 2m vertical within 1s delay for absolute high speed outdoor positioning scenarios.

Proposal 3: For SL positioning consider three category of horizontal position accuracy levels: tens of meters [10-20m], meters[1-3m], and sub-meter [0.2-0.5m]. Consider vertical position accuracy level of sub meter [<1m] and of above meter[1-3m].

Proposal 4: For SL positioning consider the latencies (delays) levels requirements [150ms, 1s].

Observation 6: It is understood that there is no single positioning technology or method that must meet all the target performance requirements for every scenario.

Observation 7: Some of the SL positioning target performance requirements may be satisfied through a combination of RAT independent and RAT dependent solutions.

# References

RP-213588, Study on expanded and improved NR positioning, RAN #94e, December 2021.

TR 38.845, Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases (Release 17)

TS 22.261 Service requirements for the 5G system

TS 22.186 Enhancement of 3GPP support for V2X scenarios; Stage 1 (Release 17)

TS 22.104 Service requirements for cyber-physical control applications in vertical domains; Stage 1 (Release 18)

5GAA “System Architecture and Solution Development; High-Accuracy Positioning for C-V2X”, https://5gaa.org/news/system-architecture-and-solution-development-high-accuracy-positioning-for-c-v2x/ , 2021

5GAA “C-V2X-Use-Cases-Volume-II”, https://5gaa.org/wp-content/uploads/2020/10/5GAA_White-Paper_C-V2X-Use-Cases-Volume-II.pdf , 2020

5GAA, A-200134, “Draft reply LS to 3GPP RAN on requirements of in-coverage, partial coverage, and out-of- coverage positioning use cases”

TS 38.855, Study on NR positioning support (Release 16)

# Appendix A

Table 6.1-1, TR 22.872

| Use cases |  | Potential requirements per use cases |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Environment of Use | Position Accuracy | Velocity | Avail. | Update rate or interval | TTFF | Latency | Other KPI |
| 5.2.1 | Bike sharing | 5G positioning service area - Outdoor | 2m Horizontal |  | 90 % |  | 10s | 1s |  |
|  |  | Enhanced positioning area - Outdoor | 0.2m Horizontal |  | 99 % |  | 10s | 1s |  |
| 5.2.2 | Augmented Reality | Outdoor - 5G positioning service area | 1-3m Horizontal 0.1-3m Vertical | 2 m/s 10deg. | 80 % | 1 - 10 Hz | 10s | 1s | Low Energy |
| 5.2.3 | Wearables | 5G positioning service area - -Outdoor/Indoor | 2m Horizontal 1-3m Vertical |  | 90 % | 30s - 300s | 10s |  | Power saving mode |
|  |  | 5G positioning service area - -Outdoor/Indoor | 2m Horizontal 1-3m Vertical |  | 99 % | 1s - 30s | 10s | 1s | Normal mode |
| 5.2.4 | Advertisement push | 5G positioning service area - -Outdoor/Indoor | 3m Horizontal 3m Vertical |  | 90 % |  |  | 60s |  |
| 5.2.5 | Flow management | Enhanced positioning- Outdoor/Indoor | 10m Horizontal |  | 80 % | 10s | 10s |  |  |
| 5.3.1 | Person and medical equipment location in Hospital | Enhanced positioning- Outdoor/Indoor | 3m Horizontal 2m Vertical |  | 99 % |  |  | 60s |  |
| 5.3.2 | Patient location (outside Hospital) | 5G positioning service area Outdoor/Indoor | 10m Horizontal 3m Vertical (floor) |  | 99 % |  |  |  |  |
| 5.3.3 | Trolley | Enhanced positioning- Outdoor/Indoor | 0.5m Horizontal 1-3m Vertical |  | 99 % |  |  | 20ms |  |
| 5.3.4 | Waste management | 5G positioning service area - Outdoor | 3m Horizontal |  | 99 % | 2h - 1 day |  | 60s | Very low energy (15 years) |
| 5.4.1 | Emergency call | 5G positioning service area Outdoor/Indoor | 50m Horizontal 3m Vertical |  | 95 % |  | 30s | 60s | Reliability/ Confidence |
| 5.4.2 | Accurate Positioning for First Responders | Outdoor | 1m Horizontal, 0.3 m Vertical |  | 98 % |  | 10s | 5s | MCX Confidence Event-triggered report |
|  |  | Indoor | 1m Horizontal, 2 m Vertical |  | 95 % |  | 10s | 1s |  |
| 5.4.3 | Alerting nearby emergency responders | 5G positioning service area Outdoor/Indoor | 50m Horizontal 3m Vertical (floor) |  | 99% |  | 10s |  | Privacy, scalability, cross operator |
| 5.4.4 | Emergency equipment loc. outside hospitals | 5G positioning service area Outdoor/Indoor | 10m Horizontal 3m Vertical (floor) |  | 95% |  | 10s |  | Extended sleep periods |
| 5.5.1 | Traffic Monitoring & Control | 5G positioning service area - Outdoor | 1-3m Horizontal 2.5m Vertical |  | 95 % | 10 Hz | 10s | 30ms | Antispoofing Antitampering |
| 5.5.2 | Road User Charging | 5G positioning service area - Outdoor Enhanced positioning-Tunnels | <1m (across track) 3m (along track) | 2 m/s | 99 % | 1 Hz | 10s |  | Antispoofing Antitampering |
| 5.6.1 | Asset tracking and management | 5G positioning service area - Outdoor | 10-30m Horizontal | 5 m/s | 99 % | 300s-1day |  |  | 20 mJ/fix (average), Antispoofing, Antitampering, support for "out of coverage" positioning |
|  |  | Enhanced positioning - Outdoor | 1m Horizontal |  | 99 % | 1s | 1s in enhanced positioning area |  |  |
| 5.7.1 | UAV (Data analysis) | 5G positioning service area - Outdoor | 0.1m Horizontal 0.1m Vertical | 0.5 m/s 2 deg. | 99 % |  | 10s |  | Low Energy, Antispoofing, Antitampering |
| 5.7.2 | UAV (Remote control) | 5G positioning service area - Outdoor | 0.5m Horizontal 0.3m Vertical |  | 99 % |  |  | 150ms | Antispoofing Antitampering |
|  |  | Enhanced positioning area - Outdoor | 0.5m Horizontal 0.1m Vertical |  | 99.9 % |  |  | 150ms | Antispoofing Antitampering |
| 5.8.1 | Support multiple different location service | 5G positioning service area - Outdoor | 2m Horizontal |  | 90 % |  | 10s | 1s | Management of different KPI and positioning services |
|  |  | Enhanced positioning area - Indoor | 0.1m Horizontal |  | 99 % |  | 10s | 1s |  |
| 5.8.2 | Support location method negotiation | 5G positioning service area Outdoor/Indoor |  |  |  |  |  |  | Support + negotiation of positioning methods (incl. hybrid) |
| Note: most use cases also feature potential requirements on modes of operation, intended for the UE, the Network or for the 5G system. |  |  |  |  |  |  |  |  |  |