3GPP TSG RAN WG1 Meeting #109-e	R1-2203059

eMeeting, May 9 – May 20, 2022

Agenda Item:	8.11.1

Source:	FUTUREWEI

Title:	Remaining aspects of power consumption reduction for sidelink resource allocation

Document for:	Discussion and decision

# Introduction

At RAN#86, a WI on sidelink enhancements was agreed for Rel-17 [1], modified in [2], and further updated in [3]. In this WI, there is an objective on resource allocation enhancements to reduce power consumption:

2. Resource allocation enhancement:

- Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2]

  - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2.

  - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly.

  - This work should consider the impact of sidelink DRX, if any.

In the status report of Rel-17 SL WI for RAN#94-e [4], a list of remaining open issues on physical layer aspects on resource allocation to reduce UE’s power consumption were generated and discussed subsequently in RAN1#107bis-e and RAN1#108-e.

As summarized in the status report of Rel-17 SL WI for RAN#95-e [5], the Rel-17 WI on sidelink enhancement has been completed, with some remaining details to be discussed in the maintenance phase.

In this contribution, we discuss some remaining issues on power consumption reduction for sidelink resource allocation.

# Discussions

## CPS window for aperiodic traffic and CPS disabling

For CPS window for aperiodic traffic, the following was agreed in RAN1#107bis-e.

Agreement When UE performs at least contiguous partial sensing in a mode 2 Tx pool for a resource (re)selection procedure and re-evaluation/pre-emption checking triggered by aperiodic transmission (Prsvp_TX=0) in slot n, - For minimum size M of the CPS monitoring window [n+TA, n+TB]: - By default, M is 31 unless (pre-)configured with another value - The range of (pre-)configured M is from 0 (working assumption) to 30

The lower bound of minimum size M for CPS window 0 was a working assumption to be confirmed. The value depends on whether CPS can be disabled or not. It was discussed in RAN1#108-e. Because there was no consensus, the working assumption was not confirmed. We continue to discuss the issue.

For a UE that is capable of partial sensing, in general, it is not preferred to disable CPS for resource (re)selection as the minimum CPS window size can be small, which does not consume much power. However, it is better to keep it as an alternative option for additional power savings.

The condition to disable CPS and switch to random resource selection can be specified. Based on past CBR measurements, the UE can detect whether the system load is high or low. When the system load is low, there is no resource, or a small number of resources occupied by other nearby UEs. Under this condition, the UE can disable CPS and switch to random resource selection to reduce power consumption. The conditions can then be set by a threshold on CBR. To enable CPS, a predefined timer can be activated once the UE switches to random resource selection.

Another condition is for low power levels (e.g., battery levels). In such a case, when the battery is about to drain out or is below a certain level, to reduce power consumption, it is beneficial that UE disables CPS and perform random resource selection. For low power level UEs, they may also be configured to perform random resource selection all the time and not to switch between partial sensing and random resource selection.

If CPS can be disabled, the lower bound of M can be set to M=0. However, if CPS cannot be disabled, some other non-zero value should be used.

As in the agreement for CPS, the minimum number of M slots for CPS defined for aperiodic traffic includes the processing time, Tproc,0 and Tproc,1.  With a small M value, e.g., M=1, it is possible that the size of the sensing window [n+TA, n+TB] becomes zero, which is equivalent to CPS disabling. Therefore, to ensure that there is at least 1 logical slot for CPS sensing, the lower bound should be the number of logical slots within Tproc,0SL+Tproc,1SL plus one, which is then SCS dependent. Therefore, it is inappropriate to set M to a small value such as 1. If a single value is preferred for the lower bound of M, since for 15 kHz SCS, the number of physical slots in Tproc,0SL+Tproc,1SL is 4, we can set lower bound of M=5. Then for most cases, we guarantee to have at least one slot for sensing.

Proposal 1: The lower bound of minimum M slots can be set as 0 if CPS can be disabled, or 5 if CPS cannot be disabled.

## Update of resource exclusion in Step 6

In the resource exclusion in Step 6 of 8.1.4 in TS38.214 for Rel-16, which is copied below, the assumption for the parameter settings on the reference slot t'n'SL , Tscal , and subsequently, Q, is that the sensing is before the resource (re)selection triggering slot n. The assumption no longer holds for partial sensing when sensing is performed after triggering slot n. Therefore, for performance improvement, the parameters need to be updated.

6) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: ….. c) the SCI format received in slot t'mSL or the same SCI format which, if and only if the 'Resource reservation period' field is present in the received SCI format 1-A, is assumed to be received in slot(s) t'm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=TscalPrsvp_RX if Prsvp_RX< Tscal and n'-m≤Prsvp_RX', where t'n'SL = n if slot n belongs to the set t'0SL,t'1SL,...,t'T'max-1SL, otherwise slot t'n'SL is the first slot after slot n belonging to the set t'0SL,t'1SL,...,t'T'max-1SL; otherwise Q=1. Tscal is set to selection window size T2 converted to units of msec.

In RAN1#108-e, we have the following agreement on the update of t'n'SL  and Tscal for partial sensing.

Agreement In Step 6 c) of TS38.214 Section 8.1.4, when UE is configured with partial sensing by its higher layer, adopt the following changes: - t'n'SL=tyiSL-Tproc,1SL if slot tyiSL-Tproc,1SL belongs to the set t'0SL,t'1SL,...,t'T'max-1SL, otherwise, slot t'n'SL is the first slot after slot tyiSL-Tproc,1SL belonging to the set t'0SL,t'1SL,...,t'T'max-1SL. - Option D: Tscal=tyLSL-(tyiSL-Tproc,1SL) converted to milliseconds, where slot tyLSL is the last slot of the Y or Y’ candidate slots. - Slot tyiSL is the first slot of the selected/remaining set of Y or Y’ candidate slots.

With the update on Tscal being agreed, the parameter Q was later discussed in RAN1#108-e.  However, no consensus could be reached. We continue to discuss this remaining detail.  In RAN1#108-e, after rounds of discussions, a proposal with four options was proposed by the FL, which is copied below, where the first three options provided a change on Q, but option 4 proposed no change on the Q formula, i.e., reusing Rel-16 Q formula.

Proposal 4-2 (IV): In Step 6 c) of TS38.214 Section 8.1.4, when UE is configured with partial sensing by its higher layer, adopt the following changes: - When additionalPeriodicSensingOccasion is (pre-)configured, - Option 1: Q=TscalPrsvp_RX +1 if Prsvp_RX< Tscal and n'-m≤2·Prsvp_RX'; otherwise Q=2 - Option 2: - Case 1: if Prsvp_RX< Tscal and Prsvp_RX'<n'-m≤2·Prsvp_RX'; Q=TscalPrsvp_RX +1 - Case 2: if Prsvp_RX≥ Tscal and Prsvp_RX'<n'-m≤2·Prsvp_RX'; Q=2 - Case 3: if Prsvp_RX< Tscal and n'-m≤Prsvp_RX'; Q=TscalPrsvp_RX - Case 4: Otherwise Q=1 - Option 3: - if Prsvp_RX< Tscal and n'-m≤PrsvpRX', Q=TscalPrsvp_RX - else if Prsvp_RX'<n'-m≤2·Prsvp_RX', Q=TscalPrsvp_RX +1 - else Q=1 - Option 4: No update to the Q formular (i.e., existing R16 Q formular is reused)

Although the Rel-16 Q formula works, it misses some resource exclusion checks when additionalPeriodicSensingOccasion is (pre-)configured, which consequently impacts the performance for the periodic based partial sensing with additionalPeriodicSensingOccasion (pre-)configured. Therefore, it is preferable to have an update on Q. Among the first 3 options, option 2 and option 3 are essentially the same.  For option 1, there are some “unnecessary” checks because the condition in option 1 will result in some exclusion checks outside Y candidate resource slots. However, all the slots covered in option 2 will be covered in option 1, which is why there is no performance difference. Since option 1 and option 2 have the same performance, either option 1 or option 2 can be adopted. Although option 1 has some unnecessary checks, it may facilitate parallel processing in implementation due to less logical condition checking.

Proposal 2:  When additionalPeriodicSensingOccasion is (pre-)configured, adopt either option of the following on the update of Q for the resource exclusion in Step 6(c). Option 1 is slightly preferred due to fewer logical conditions which may facilitate parallel processing.

- Option 1: Q=TscalPrsvp_RX +1 if Prsvp_RX< Tscal and  n'-m≤2·Prsvp_RX'; otherwise Q=2

- Option 2:

  - Case 1: if Prsvp_RX< Tscal and  Prsvp_RX'<n'-m≤2·Prsvp_RX'; Q=TscalPrsvp_RX +1

  - Case 2: if Prsvp_RX≥ Tscal and  Prsvp_RX'<n'-m≤2·Prsvp_RX'; Q=2

  - Case 3: if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX'; Q=TscalPrsvp_RX

  - Case 4: Otherwise Q=1

# Conclusion

The remaining details for sidelink power saving were discussed. We propose the following with the corresponding TPs provided in the Appendix.

Proposal 1: The lower bound of minimum M slots can be set as 0 if CPS can be disabled, or 5 if CPS cannot be disabled.

Proposal 2:  When additionalPeriodicSensingOccasion is (pre-)configured, adopt either option of the following on the update of Q for the resource exclusion in Step 6(c). Option 1 is slightly preferred due to fewer logical conditions which may facilitate parallel processing.

- Option 1: Q=TscalPrsvp_RX +1 if Prsvp_RX< Tscal and  n'-m≤2·Prsvp_RX'; otherwise Q=2

- Option 2:

  - Case 1: if Prsvp_RX< Tscal and  Prsvp_RX'<n'-m≤2·Prsvp_RX'; Q=TscalPrsvp_RX +1

  - Case 2: if Prsvp_RX≥ Tscal and  Prsvp_RX'<n'-m≤2·Prsvp_RX'; Q=2

  - Case 3: if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX'; Q=TscalPrsvp_RX

  - Case 4: Otherwise Q=1

# References

RP-193257, “NR sidelink enhancement”, RAN#86, Dec. 09-12, 2019.

RP-201385, “WID revision: NR sidelink enhancement”, RAN#88-e, June 29-July 3, 2020.

RP-202846, “WID revision: NR sidelink enhancement”, RAN#90-e, Dec. 7-11, 2020.

RP-212880, “Status report of WI: NR Sidelink enhancement”, RAN1, Dec. 6-17, 2021.

RP-220520, “Status report of WI: NR Sidelink enhancement”, RAN#95, Mar. 14-17, 2022.

# Appendix

The list of TPs corresponding to the technical proposals are provided below.

TP for Proposal 1:

| Reason for change: | The lower bound [0] of CPS window for aperiodic traffic is still a working assumption. To confirm the WA, i.e., the lower bound 0, the meaning to set for setting contiguousSensingWindowAperiodi to 0 needs to be specified, i.e., CPS for aperiodic traffic is disabled. |
| --- | --- |
| Summary of change: | Include the description of CPS disabling with contiguousSensingWindowAperiodic configured to 0 in TS38.214. |
| Consequences if not approved: | Ambiguous specifications. |

## Text proposal for specify the behavior when contiguousSensingWindowAperiodic is configured to 0 in TS38.214 Section 8.1.4 --------------------------------- Text proposal begins ----------------------------------------------- ### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 ## *** Unchanged text is omitted *** When the UE performs contiguous partial sensing with periodic reservation for another TB (sl-MultiReserveResource) disabled and if Prsvp_TX=0, the sensing window is defined by the range of slots [n+TA, n+TB]. TA and TB are both selected such that the UE has sensing results starting at M consecutive logical slots before ty0SL and ending at Tproc,0SL+Tproc,1SL slots earlier than ty0SL. The value of M is (pre-)configured with the contiguousSensingWindowAperiodic. If contiguousSensingWindowAperiodic is not (pre-)configured, M equals to 31. When the minimum M slots for CPS cannot be guaranteed and when Prsvp_TX=0, it is up to UE implementation to either continue with step 3) or perform random selection. If contiguousSensingWindowAperiodic is configured to 0, contiguous partial sensing is disabled. ## *** Unchanged text is omitted *** --------------------------------- Text proposal ends -----------------------------------------------

TP for Proposal 2:

| Reason for change: | For PBPS, when additionalPeriodicSensingOccasion is (pre-)configured, according to the existing definition of Q formula in TS38.214, Sec. 8.1.4, step 6(c), for the periodic reservation detected in the second most recent sensing occasion, some of the corresponding reserved resources within the selected Y candidate resources may not be checked or excluded. To address this issue, this TP captures new formula agreed in RAN1#109-e. |
| --- | --- |
| Summary of change: | Add new Q formula for the case when additionalPeriodicSensingOccasion is (pre-)configured, if the UE is configured with partial sensing by its higher layer |
| Consequences if not approved: | Serious performance impact on PBPS when additionalPeriodicSensingOccasion is (pre-)configured, which may make the (pre-)configuration of additionalPeriodicSensingOccasion ineffective. |

## Text proposal for updating Q parameter in TS38.214 Section 8.1.4 --------------------------------- Text proposal begins ----------------------------------------------- ### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 ## *** Unchanged text is omitted *** c) the SCI format received in slot t'mSL or the same SCI format which, if and only if the 'Resource reservation period' field is present in the received SCI format 1-A, is assumed to be received in slot(s) t'm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=TscalPrsvp_RX if Prsvp_RX< Tscal and n'-m≤Prsvp_RX', where if the UE is configured with full sensing by its higher layer, t'n'SL = n if slot n belongs to the set t'0SL,t'1SL,...,t'T'max-1SL, otherwise slot t'n'SL is the first slot after slot n belonging to the set t'0SL,t'1SL,...,t'T'max-1SL; If UE is configured with partial sensing by its higher layer, tn''SL=tyiSL-Tproc,1SL if slot tyiSL-Tproc,1SL belongs to the set (t0'SL, t1'SL,⋯, tTmax'-1'SL), otherwise, slot tn''SL is the first slot after slot tyiSL-Tproc,1SL belonging to the set (t0'SL, t1'SL,⋯, tTmax'-1'SL). Otherwise Q=1. If the UE is configured with partial sensing by its higher layer, when additionalPeriodicSensingOccasion is (pre-)configured, Q=TscalPrsvp_RX +1 if Prsvp_RX< Tscal and n'-m≤2·Prsvp_RX'; otherwise Q=2. If the UE is configured with full sensing by its higher layer, Tscal is set to selection window size T2 converted to units of msec. If UE is configured with partial sensing by its higher layer, Tscal=tyLSL-(tyiSL-Tproc,1SL) shall be converted to milliseconds, where slot tyLSL is the last slot of the Y or Y' candidate slots. The slot tyiSL is the first slot of the selected/remaining set of Y or Y' candidate slots. ## *** Unchanged text is omitted *** --------------------------------- Text proposal ends -----------------------------------------------