3GPP TSG RAN WG1 Meeting #109-e	R1-2203061

e-Meeting, May 9th – May 20th, 2022

Agenda Item:	9.1.1.1

Source:	Futurewei

Title:	Unified TCI framework extension for multi-TRP

Document for:	Discussion and decision

# Introduction

In RAN#94-e meeting, the Rel. 18 NR_MIMO_evo_DL_UL WID [1] was approved. There are seven objectives in the new WID. The second and seventh objectives in [1] are:

- Specify extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states focusing on multi-TRP use case, using Rel-17 unified TCI framework. - Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

In this contribution, we present our views on extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states, as well as power control for UL single DCI for multi-TRP operation, and proposals for moving forward.

# Indication of Multiple DL and UL TCI States

In Rel-17, the unified TCI framework was defined to streamline beam indication/update for multiple DL and/or UL channels/signals.  During the development of Rel-17 unified TCI framework, there have been some discussions on the supported number of simultaneous active TCI states (e.g., M=1 or M>=1 for DL and N=1 or N>=1 for UL).  With multiple simultaneous active TCI states, we are referring to the cases where multiple DL, UL, or joint DL/UL TCI states are indicated in one instance of beam indication DCI. During the Rel-17 discussions, the group arrived at the following conclusions [2] on multiple DL and UL TCI states.

Conclusion On Rel.17 unified TCI framework, based on the agreements in RAN1#102-e and 103-e, the following terms are defined as follows (at least for the purpose of discussion and reaching agreements). For M=1: - DL TCI: The source reference signal(s) (analogous to Rel.15, two, if qcl_Type2 is configured in addition to qcl_Type1) in the DL TCI provides QCL information at least for UE-dedicated reception on PDSCH and all of CORESETs in a CC For N=1: - UL TCI: The source reference signal in the UL TCI provides a reference for determining UL TX spatial filter at least for dynamic-grant/configured-grant based PUSCH and all of dedicated PUCCH resources in a CC For M=N=1: - Joint DL/UL TCI: A TCI refers to at least a common source reference RS used for determining both the DL QCL information and the UL TX spatial filter. - Separate DL/UL TCI: The DL TCI and UL TCI are distinct (therefore, separate). For M>1: - DL TCI: Each of the M source reference signals (or 2M, if qcl_Type2 is configured in addition to qcl_Type1) in the M DL TCIs provides QCL information at least for one of the M beam pair links for UE-dedicated receptions on PDSCH and/or subset of CORESETs in a CC For N>1: - UL TCI: Each of the N source reference signals in the N UL TCIs provide a reference for determining UL TX spatial filter at least for one of the N beam pair links associated with dynamic-grant(s)/configured-grant(s) based PUSCH, and/or subset of dedicated PUCCH resources in a CC For M>1 and/or N>1: - Joint DL/UL TCI: A TCI refers to at least a common source reference RS used for determining both the DL QCL information and the UL TX spatial filter. In this case, M=N. - Separate DL/UL TCI: The M DL TCIs and N UL TCIs are distinct (therefore, separate). Note: Other TCI types/terms such as “common TCI” are not used.

However, due to lack of time, the support of indication of multiple DL and UL TCI states (e.g., M>1 and/or N>1) is not specified in Rel-17.  To support unified TCI framework extension for multi-TRP in Rel-18, mechanism to indicate multiple DL and UL TCI states is needed.

Activation and Indication of Multiple DL and UL TCI States:

Support of two active TCI states for mTRP has been defined in Rel-16; including both single DCI and multi-DCI cases.  In single DCI case, MAC CE [3] was enhanced to enable the mapping of two TCI states to one TCI codepoint such that one TCI codepoint in DCI TCI field can indicate two different TCI states, one for each TRP.  In multi-DCI case, the coresetPoolIndex [4] in the ControlResourceSet information element can be used to differentiate TCI states for different TRP.  However, since these mechanisms were defined in Rel-16 and unified TCI framework was only developed in Rel-17, the Rel-16 mechanisms cannot directly apply to the unified TCI framework.

To extend the unified TCI framework to support indication of multiple TCI states for the cases of M>1 and/or N>1 in one instance of beam indication DCI, the Rel. 16 “Enhanced TCI States Activation/Deactivation for UE-specific PDSCH MAC CE” [3] can be enhanced to achieve that goal.  Figure 1 shows an example of the enhanced UE-specific MAC CE for TCI states activation/deactivation for cases of M>1 and/or N>1.  In this example, the number of TCI states (e.g., the value of M and/or N) mapped to a single TCI codepoint is higher layer configured, e.g., through RRC configuration.  Alternatively, the number of TCI states mapped to each TCI codepoint could be different and those numbers can be indicated in the MAC-CE.  For example, a two-bit value can be associated with each of the TCI codepoint to indicate the number of TCI states (up to 4) mapped to that TCI codepoint.  Therefore a total of 16 bits (e.g., 8 two-bit fields) are required to indicate the number of TCI states mapped to 8 different TCI codepoints.  The function of the original bits “Ci” and “R” are modified to indicate whether the corresponding TCI state indicated in the same octet is from the DL/joint TCI state pool or the UL TCI state pool.  As shown in Figure 1, multiple TCI states are mapped to one TCI codepoint and the number of TCI states mapped to one TCI codepoint is higher layer configured.  This way, one MAC CE can activate multiple TCI states, and M (or N) TCI states are mapped to one of the 8 TCI codepoints.  If the UE is configured with separate DL/UL TCI, some of these TCI states can be DL-only TCI states, and some can be UL-only TCI states.  IF the UE is configured with joint TCI, the TCI states will be joint DL/UL TCI states.  The 3-bit TCI field in the DCI can then be used to indicate one TCI codepoint (and its mapped M/N TCI states) for beam indication.

Figure 1: Example of enhanced UE-specific MAC CE for TCI states activation/deactivation for cases of M>1 and/or N>1

Based on the above analysis, we propose the following:

Proposal 1: For extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states, support enhanced UE-specific MAC CE for TCI states activation/deactivation, where one TCI codepoint can be mapped to one or multiple TCI states.  Different TCI codepoint can be mapped to TCI state(s) from different TCI state pool.  TCI states mapped to one TCI codepoint can be from the same or different TCI state pool.

Association of TRP, Target Channels/Signals with Indicated Multiple DL and UL TCI States:

Next, we need to define the following: how each of the indicated multiple TCI states is associated with each of the multiple TRPs and to which channels/signals the TCI state applies. In our view, the following rule should apply.  For M-TRP with single DCI, each of the multiple DL/joint or UL TCI states mapped to one TCI codepoint is associated with each of the multiple TRPs according to the order of the TCI state among the TCI states mapped to the TCI codepoint, respectively.  For example, the first DL/joint TCI state mapped to the TCI codepoint indicated by the DCI is associated with the first TRP and is applied to the DL/UL channels/signals associated with the first TRP and configured to share the unified TCI state (e.g., using same rule as defined in Rel-17).  Similarly, the second DL/joint TCI state mapped to the TCI codepoint indicated by the DCI is associated with the second TRP and is applied to the DL/UL channels/signals associated with the second TRP and configured to share the unified TCI state.  Similar rule can be defined for the separate DL and UL TCI case as well.

For M-TRP with multiple DCIs, the “R” bit in the first octet in the MAC CE can be used to indicate the coresetPoolIndex to which the mapping between the TCI states and the TCI codepoint specified in the MAC CE should be applied.  This coresetPoolIndex effectively act as identification of different TRP.  The indicated TCI states can then be applied to the DL/UL channels/signals associated with different TRP (e.g., different coresetPoolIndex) and configured to share the unified TCI state, respectively.

Based on the above analysis, we propose the following:

Proposal 2: For M-TRP with single DCI, each of the multiple DL/joint or UL TCI states mapped to one TCI codepoint is associated with each of the multiple TRPs according to the order of the TCI state among the TCI states mapped to the TCI codepoint, respectively.  The indicated TCI state is applied to the DL/UL channels/signals associated with the corresponding TRP and configured to share the unified TCI state according to Rel-17 mechanism.

Proposal 3: For M-TRP with multiple DCI, each of the multiple DL/joint or UL TCI states mapped to one TCI codepoint is associated with each of the multiple TRPs according to the coresetPoolIndex indicated in the MAC CE activating the TCI states.  The indicated TCI state is applied to the DL/UL channels/signals associated with the corresponding TRP and configured to share the unified TCI state according to Rel-17 mechanism.

# Power Control for UL single DCI

Support of Multiple TPC Fields:

In Rel-17 enhancements on multi-TRP for PUCCH and PUSCH, to support per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2, and per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2, a second TPC field can be configured via RRC for DCI formats 1_1 / 1_2 and DCI formats 0_1 / 0_2, respectively.  Therefore, the mechanism to support two TPC fields in one UL DCI is already available in Rel-17 and should be reused in Rel-18 to support UL power control for single DCI.

Support of Multiple Power Control Parameter Settings:

The Rel-17 enhancements on multi-TRP and related power control settings are based on the Rel-16 TCI and spatial relation framework, instead of the Rel-17 unified TCI framework.  As stated in the 7th objective of the Rel. 18 NR_MIMO_evo_DL_UL WID, power control for UL single DCI for multi-TRP operation needs to be specified assuming the unified TCI framework extension in objective 2.  Therefore, UL power control parameter (P0, alpha, closed loop index, and PL RS) setting for multi-TRP need to be defined based on the unified TCI framework and its extension in Rel-18.

In Rel-17 unified TCI framework, it was agreed that it should be possible to associate the UL power control parameters settings with UL or joint TCI state for each of the PUSCH, PUCCH, and SRS.  In this case, multiple settings are configured.  Each setting can be associated with at least one TCI state. For each of the PUSCH, PUCCH, and SRS, one individual setting can be associated with each of the UL or joint TCI states.  As discussed in Section 2, each of the multiple DL/joint or UL TCI states mapped to one TCI codepoint is associated with each of the multiple TRPs.  In our view, if we reuse the Rel-17 unified TCI framework mechanism to associate the UL power control parameter settings with UL or joint TCI state, the indicated TCI state associated with different TRP can then indicate the UL power control parameter settings for different UL channels associated with different TRP through the UL power control parameter settings associated with the indicated TCI state.  For example, assuming UL TCI States #1 and #2 are associated with TRPs #1 and #2, respectively, and power control parameter settings #1 and #2 are associated with UL TCI States #1 and #2, respectively, for PUSCH.  When the DCI indicates UL TCI States #1 and #2, it also indicates that power control parameter settings #1 and #2 should be used for transmission of PUSCH associated with TRPs #1 and #2, respectively.

Based on the above analysis, we propose the following:

Proposal 4: For power control for UL single DCI, reuse the Rel-17 mechanism to support two TPC fields in one UL DCI and to associate the UL power control parameters (P0, alpha, closed loop index, and PL RS) settings with UL or joint TCI state for each of the PUSCH, PUCCH, and SRS.

# Conclusions

In this contribution, we present our views on extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states, as well as power control for UL single DCI for multi-TRP operation.  Based on the discussions in the previous sections we propose the following:

Proposal 1: For extension of Rel-17 Unified TCI framework for indication of multiple DL and UL TCI states, support enhanced UE-specific MAC CE for TCI states activation/deactivation, where one TCI codepoint can be mapped to one or multiple TCI states.  Different TCI codepoint can be mapped to TCI state(s) from different TCI state pool.  TCI states mapped to one TCI codepoint can be from the same or different TCI state pool.

Proposal 2: For M-TRP with single DCI, each of the multiple DL/joint or UL TCI states mapped to one TCI codepoint is associated with each of the multiple TRPs according to the order of the TCI state among the TCI states mapped to the TCI codepoint, respectively.  The indicated TCI state is applied to the DL/UL channels/signals associated with the corresponding TRP and configured to share the unified TCI state according to Rel-17 mechanism.

Proposal 3: For M-TRP with multiple DCI, each of the multiple DL/joint or UL TCI states mapped to one TCI codepoint is associated with each of the multiple TRPs according to the coresetPoolIndex indicated in the MAC CE activating the TCI states.  The indicated TCI state is applied to the DL/UL channels/signals associated with the corresponding TRP and configured to share the unified TCI state according to Rel-17 mechanism.

Proposal 4: For power control for UL single DCI, reuse the Rel-17 mechanism to support two TPC fields in one UL DCI and to associate the UL power control parameters (P0, alpha, closed loop index, and PL RS) settings with UL or joint TCI state for each of the PUSCH, PUCCH, and SRS.

# References

RP-213598, “New WID: MIMO Evolution for Downlink and Uplink”, Samsung, RAN#94-e.

3GPP RAN1, RAN1#104-e, Chairman Notes.

TS 38.321, “NR: Medium Access Control (MAC) protocol specification”.

TS 38.331, “Radio Resource Control (RRC) protocol specification”.