3GPP TSG RAN WG1 Meeting #109-e	R1-2203062

e-Meeting, May 9th – May 20th, 2022

Agenda Item:	9.1.1.2

Source:	Futurewei

Title:	Enhancements to support two TAs for multi-DCI

Document for:	Discussion and decision

# Introduction

In RAN#94-e meeting, the Rel. 18 NR_MIMO_evo_DL_UL WID [1] was approved. There are seven objectives in the new WID.  The seventh objective in [1] is:

- Study, and if justified, specify the following - Two TAs for UL multi-DCI for multi-TRP operation - Power control for UL single DCI for multi-TRP operation where unified TCI framework extension in objective 2 is assumed. For the case of simultaneous UL transmission from multiple panels, the operation will only be limited to the objective 6 scenarios.

In this contribution, we present our views on two TAs for UL multi-DCI for multi-TRP operation and proposals for moving forward.

# Analysis on Two TAs for UL multi-DCI multi-TRP Operation

For UL multi-DCI multi-TRP operation, one usage scenario is that UE is equipped with multiple panels and each panel is used for UL/DL transmission/reception to/from a different TRP.  Considering the impact of non-ideal backhaul, it is possible that different TRPs are not perfectly synchronized, especially for the multi-DCI case.  As different TRPs are not perfectly synchronized, the DL transmit timing of different TRP could be different.  Also, due to the fact that the links between different TRP and different UE panel will experience different paths, e.g., some may be LOS while some may be N-LOS, the propagation delays between different TRP and the UE in general are different.  The different DL transmit timing from different TRP, coupled with the different propagation delay between different TRP and the UE, triggers the need of multiple (e.g., 2) TAs for UL multi-DCI multi-TRP operation.

In the following we will have a more detailed analysis on this topic.  We will examine different options for TA offset settings.  For each option, we will also look at multiple examples, each with a different combination of DL transmit timing and propagation delay.

Different Options for TA Offset Settings:

- For multi-TRP operation, let’s assume that TRP1 and TRP2 are communicating with the UE.  The UE may have several options to determine its UL transmit timing. First, the UE may use only one TA offset, i.e., the TA offset is based on TRP1 and will be applied to UL transmissions to both TRP1 and TRP2. An alternative to this is that the UE may adopt TRP/panel-specific TA offsets, e.g., two TA offsets, one for each of the two TRPs, respectively. Second, the UE may need to determine a UL TA reference timing, i.e., a TA offset will be applied on top of a reference time such as the DL receive time / DL symbol starting time. The UL TA reference timing may be based on one of the TRPs or may be TRP-specific. The four possible combinations are listed below.

- Option 1: Only one TA offset (e.g., based on TRP1), and only one UL TA reference timing (e.g., based on TRP1).

- Option 2: Only one TA offset (e.g., based on TRP1), and two UL TA reference timings (e.g., TRP-specific).

- Option 3: two TA offsets (e.g., TRP/panel-specific), and one UL TA reference timing (e.g., based on TRP1).

- Option 4: two TA offsets (e.g., TRP/panel-specific), and two UL TA reference timings (e.g., TRP-specific).

- For each of the four options mentioned above, we will look at multiple possible cases to examine the resulting UL TRP receive timing offset, each case having different assumptions on DL transmit timing from different TRP and propagation delay between different TRP and the UE.

Analysis for Different Options:

- First, we look at Option 1 where only one TA offset (e.g., based on TRP1) and only one UL TA reference timing (e.g., based on TRP1) are utilized.  Table 1 shows the UL TA analysis for Option 1.  In Table 1 and following similar tables, the notations below are used to indicate cases with combination of different assumptions on DL transmit timing and propagation delay:

- TRP2-A: DL transmit timing of TRP2 is perfectly synchronized with that of TRP1.  The propagation delay between TRP2 and the UE is d us longer than that between TRP1 and the UE.

- TRP2-B: There exists a p us delay between DL transmit timing of TRP2 and that of TRP1.  The propagation delay between TRP2 and the UE is the same as that between TRP1 and the UE.

- TRP2-C: There exists a p us delay between DL transmit timing of TRP2 and that of TRP1.  The propagation delay between TRP2 and the UE is d us longer than that between TRP1 and the UE.

- As shown in Table 1, for Option 1 with the case of TRP2-C, with the assumption that p=-2 us and d=2 us (e.g., corresponding to the cell with red text), TRP2 may experience an UL receive timing offset that is twice as long as the TRP timing synchronization difference or propagation delay difference.  Figure 1 illustrates the timing diagrams for the case of TRP2-C for Option 1.  Please note that in Figure 1, “TRP2 Rx TO” denotes the UL receive timing offset at TRP2.

Table 1: UL TA analysis for Option 1: Only one TA offset (based on TRP1), and only one UL TA reference timing (based on TRP1)

|  | TRP1 | TRP2-A | TRP2-B | TRP2-C |
| --- | --- | --- | --- | --- |
| (a) TRP Tx timing (us) | T | T | T+p | T+p |
| (b) Propagation delay (us) | t | t+d | t | t+d |
| (c) UE Rx timing = (a) + (b) | T+t | T+t+d | T+t+p | T+t+d+p |
| (d) TA offset (based on TRP1) | s | s | s | s |
| (e) UL TA reference timing (based on TRP1) | T+t | T+t | T+t | T+t |
| (f) UE Tx timing = (e) – (d) | T+t-s | T+t-s | T+t-s | T+t-s |
| (g) TRP Rx timing = (f) + (b) | T+2t-s | T+2t-s+d | T+2t-s | T+2t-s+d |
| (h) TRP Rx timing offset = (g) – (a) | 2t-s | 2t-s+d | 2t-s-p | 2t-s+d-p |
| TRP Rx timing offset (us) Example 1: s=2t, d=2us, p=2us | 0 | 2 | -2 | 0 |
| TRP Rx timing offset (us) Example 2: s=2t, d=2us, p=-2us | 0 | 2 | 2 | 4 |

Figure 1: Timing diagrams for the case of TRP2-C for Option 1: Only one TA offset (e.g., based on TRP1), and only one UL TA reference timing (e.g., based on TRP1)

Next, we look at Option 2 where only one TA offset (e.g., based on TRP1), and two UL TA reference timings (e.g., TRP-specific) are utilized.  Table 2 shows the UL TA analysis for Option 2.  As shown in Table 2, for Option 2, for both the cases of TRP2-A and TRP2-C, TRP2 may experience an UL receive timing offset that is twice as long as the TRP timing synchronization difference or propagation delay difference (as indicated in the cells with red text).  Figure 2 illustrates the timing diagrams for the case of TRP2-A for Option 2.

Table 2: UL TA analysis for Option 2: Only one TA offset (based on TRP1), and two UL TA reference timings (e.g., TRP-specific)

|  | TRP1 | TRP2-A | TRP2-B | TRP2-C |
| --- | --- | --- | --- | --- |
| (a) TRP Tx timing (us) | T | T | T+p | T+p |
| (b) Propagation delay (us) | t | t+d | t | t+d |
| (c) UE Rx timing = (a) + (b) | T+t | T+t+d | T+t+p | T+t+d+p |
| (d) TA offset (based on TRP1) | s | s | s | s |
| (e) UL TA reference timing (TRP-specific) = (c) | T+t | T+t+d | T+t+p | T+t+d+p |
| (f) UE Tx timing = (e) – (d) | T+t-s | T+t+d-s | T+t+p-s | T+t+d+p-s |
| (g) TRP Rx timing = (f) + (b) | T+2t-s | T+2t+2d-s | T+2t+p-s | T+2t+2d+p-s |
| (h) TRP Rx timing offset = (g) – (a) | 2t-s | 2t-s+2d | 2t-s | 2t-s+2d |
| TRP Rx timing offset (us) Example 1: s=2t, d=2us, p=2us | 0 | 4 | 0 | 4 |
| TRP Rx timing offset (us) Example 2: s=2t, d=2us, p=-2us | 0 | 4 | 0 | 4 |

Figure 2: Timing diagrams for the case of TRP2-A for Option 2: Only one TA offset (e.g., based on TRP1), and two UL TA reference timings (e.g., TRP-specific)

Third, we look at Option 3 where two TA offsets (e.g., TRP/panel-specific), and one UL TA reference timing (e.g., based on TRP1) are utilized.  Table 3 shows the UL TA analysis for Option 3.  As shown in Table 3, for Option 3 with the case of TRP2-C, TRP2 may experience an UL receive timing offset that is twice as long as the TRP timing synchronization difference or propagation delay difference (as indicated in the cells with red text).  Figure 3 illustrates the timing diagrams for the case of TRP2-C for Option 3.

Table 3: UL TA analysis for Option 3: Two TA offset (e.g., TRP-specific), and one UL TA reference timing (based on TRP1)

|  | TRP1 | TRP2-A | TRP2-B | TRP2-C |
| --- | --- | --- | --- | --- |
| (a) TRP Tx timing (us) | T | T | T+p | T+p |
| (b) Propagation delay (us) | t | t+d | t | t+d |
| (c) UE Rx timing = (a) + (b) | T+t | T+t+d | T+t+p | T+t+d+p |
| (d) TA offset (TRP-specific) = 2*(b) | 2t | 2t+2d | 2t | 2t+2d |
| (e) UL TA reference timing (based on TRP1) | T+t | T+t | T+t | T+t |
| (f) UE Tx timing = (e) – (d) | T-t | T-t-2d | T-t | T-t-2d |
| (g) TRP Rx timing = (f) + (b) | T | T-d | T | T-d |
| (h) TRP Rx timing offset = (g) – (a) | 0 | -d | -p | -d-p |
| TRP Rx timing offset (us) Example 1: d=2us, p=2us | 0 | -2 | -2 | -4 |
| TRP Rx timing offset (us) Example 2: d=2us, p=-2us | 0 | -2 | 2 | 0 |

Figure 3: Timing diagrams for the case of TRP2-C for Option 3: two TA offsets (e.g., TRP/panel-specific), and one UL TA reference timing (e.g., based on TRP1)

Lastly, we look at Option 4 where two TA offsets (e.g., TRP/panel-specific), and two UL TA reference timings (e.g., TRP-specific) are utilized.  Table 4 shows the UL TA analysis for Option 4.  As shown in Table 4, for Option 4, the UL receive timing offset of TRP2 is 0 for all the cases considered.  Figure 4 illustrates the timing diagrams for the case of TRP2-C for Option 4.

Table 4: UL TA analysis for Option 4: Two TA offset (e.g., TRP-specific), and two UL TA reference timings (e.g., TRP-specific)

|  | TRP1 | TRP2-A | TRP2-B | TRP2-C |
| --- | --- | --- | --- | --- |
| (a) TRP Tx timing (us) | T | T | T+p | T+p |
| (b) Propagation delay (us) | t | t+d | T | t+d |
| (c) UE Rx timing = (a) + (b) | T+t | T+t+d | T+t+p | T+t+d+p |
| (d) TA offset (TRP-specific) = 2*(b) | 2t | 2t+2d | 2t | 2t+2d |
| (e) UL TA reference timing (TRP-specific) = (c) | T+t | T+t+d | T+t+p | T+t+d+p |
| (f) UE Tx timing = (e) – (d) | T-t | T-t-d | T-t+p | T-t-d+p |
| (g) TRP Rx timing = (f) + (b) | T | T | T+p | T+p |
| (h) TRP Rx timing offset = (g) – (a) | 0 | 0 | 0 | 0 |

Figure 4: Timing diagrams for the case of TRP2-C for Option 4: two TA offsets (e.g., TRP/panel-specific), and two UL TA reference timings (e.g., TRP-specific)

- One key observation from the above analysis is that for Options 1-3, there always exist some cases that a TRP will experience UL receive timing offset much longer (such as twice as long) than TRP timing synchronization difference or propagation delay difference. In other words, for Options 1-3, even if the multi-TRP signals in the DL are within one CP length, the UL receive timing offset may still be larger than one CP length.  This issue becomes even more severe in FR2 considering the fact that the CP length for FR2 is much shorter than that for FR1 due to the use of larger subcarrier spacing.  Utilizing Option 4 with two TA offsets (e.g., TRP/panel-specific), and two UL TA reference timings (e.g., TRP-specific), on the other hand, can remove the UL receive timing offset.

To investigate the impact of UL receive timing offset/TA error on UL performance, a set of PUSCH evaluations have been performed.  In the simulations, all cases are with carrier frequency 4 GHz, SCS 30 kHz, UE speed 3 km/h, and 4T4R for UL. In addition, codebook and non-codebook based PUSCH, with 1 layer and 2 layers, and on TDL-A 30 ns and TDL-C 300 ns channels have been simulated. The simulation results are presented in Figure 5 to Figure 9 in the Appendix.  In the simulations, the UE is served by 2 TRPs, i.e., TRP1 and TRP2, but its UL TA is adjusted only according to TRP1, so its UL TA to TRP 2 has an offset/error compared with other UL TA for UEs adjusted according to TRP2. We sweep the TA error from –100% CP to + 150% CP.

From the simulation results, we have the following observations:

- For TDL-A 30 ns

  - TA error of +30% CP leads to no performance degradation

  - All other cases (TA error of -100% CP ~ -30% CP, ≥ +70% CP) lead to performance degradation

    - TA error of -30% CP and +70% CP have about 1~2 dB performance degradation

    - TA error of -70% CP and more, and +100% CP have significant performance degradation

- For TDL-C 300 ns

  - TA error of +/-30% CP leads to no performance degradation

  - All other cases (TA error of larger than +/-70% CP) lead to performance degradation

    - TA error of +70% CP have about 1~2 dB performance degradation

    - TA error of -70% CP and more, and +100% CP have significant performance degradation

Thus, even if the UL receive timing offset/TA error is within a CP length, PUSCH performance degradation is possible and can be detrimental. What can be generally tolerable is [0,+50%] of CP length in UL receive timing offset/TA error.  However, based on our previous analysis on Options 1-4, using Options 1-3 can cause the UL receive timing offset/TA error falling out of the tolerable range, resulting in significant UL performance degradation.  Option 4, with two TA offsets (e.g., TRP/panel-specific), and two UL TA reference timings (e.g., TRP-specific), on the other hand, can make sure the UL receive timing offset/TA error is within the tolerable range, resulting in no UL performance degradation.

Based on the above analysis, we propose the following:

Proposal 1: For UL multi-DCI for multi-TRP operation, support two TA offsets (e.g., TRP/panel-specific), and two UL TA reference timings (e.g., TRP-specific).

# PRACH Enhancement to Support Two TAs for UL multi-DCI multi-TRP Operation

As we analyze and propose in Section 2, it is necessary to support two TA offsets (e.g., TRP/panel-specific) and two UL TA reference timings (e.g., TRP-specific) for UL multi-DCI multi-TRP Operation.  Consequently, two TA offsets need to be acquired, one for each of the two TRPs, and then maintained by the UE.

However, the Rel-15/16/17 design of one TA per carrier/serving cell is not sufficient for UL multi-DCI multi-TRP operation.  In Rel-15/16/17, TAGs are cell-based.  In one carrier there is only one serving cell, and that cell is assigned with one TAG.  For UL multi-DCI multi-TRP operation, if a TRP is co-channel with a serving cell but is not co-located with the serving cell, Rel-16/17 does not have a separate TA for it and the UE applies the TA of the co-channel serving cell for this TRP.  However, for UL multi-DCI multi-TRP operation, as we analyze in Section 2, utilizing only a single TA may result in much longer UL receive timing offset, which would significantly impact UL performance.  Therefore, it is necessary to support two TA offsets (e.g., TRP/panel-specific) in a carrier for UL multi-DCI multi-TRP operation.

To acquire TAs from multiple TRPs, the UE needs to transmit PRACH to multiple TRPs, most likely using different panel for different TRP with different UL TCI state.  Therefore, enhancements to support multiple PRACH and multiple TA configurations in a carrier are needed.

Based on the above analysis, we propose the following:

Proposal 2: For UL multi-DCI for multi-TRP operation, support to acquire and maintain Two TA values for multiple TRPs on the same carrier via PRACH enhancement and TA configuration enhancement.

# Conclusions

In this contribution, we present our views on two TAs for UL multi-DCI for multi-TRP operation.  Based on the discussions in the previous sections we propose the following:

Proposal 1: For UL multi-DCI for multi-TRP operation, support two TA offsets (e.g., TRP/panel-specific), and two UL TA reference timings (e.g., TRP-specific).

Proposal 2: For UL multi-DCI for multi-TRP operation, support to acquire and maintain Two TA values for multiple TRPs on the same carrier via PRACH enhancement and TA configuration enhancement.

# References

RP-213598, “New WID: MIMO Evolution for Downlink and Uplink”, Samsung, RAN#94-e.

# Appendix

- M-TRP PUSCH performance with UL receive timing offset/TA error:

Figure 5: M-TRP PUSCH performance with UL receive timing offset/TA error, TDL-A 30 ns, codebook, 1 layer

Figure 6: M-TRP PUSCH performance with UL receive timing offset/TA error, TDL-A 30 ns, codebook, 2 layers

Figure 7: M-TRP PUSCH performance with UL receive timing offset/TA error, TDL-A 30 ns, non-codebook, 2 layers

Figure 8: M-TRP PUSCH performance with UL receive timing offset/TA error, TDL-C 300 ns, codebook, 1 layer

Figure 9: M-TRP PUSCH performance with UL receive timing offset/TA error, TDL-C 300 ns, non-codebook, 2 layers