3GPP TSG RAN WG1 Meeting #109-e	R1-2203065

e-Meeting, May 9th – May 20th, 2022

Agenda Item:	9.11.2

Source:	Futurewei

Title:	XR Capacity Evaluation and Enhancements

Document for:	Discussion and decision

# Introduction

In RAN#95-e meeting, the updated Rel. 18 FS_NR_XR_enh SID [1] was approved. There are three groups of objectives in the new SID. The third group of objectives in [1] are:

The study is to be based on Release 17 TR 38.838, on corresponding Release 17 work from SA4 (as per SP-210043) and on Release 18 work from SA2 (as per SP-211166). … Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements; - Dynamic scheduling/grant enhancements.

As indicated in the new SID, the studied mechanisms comprise dynamic scheduling/grant enhancements, which includes cooperative MIMO schemes evaluated in [3] in the Rel-17 SID and captured in Release 17 TR 38.838 [4].  In this contribution, as in our previous contributions and in line with the new SID, two transmission schemes are evaluated: A baseline zero-forcing transmission scheme and a transmission scheme based on cooperative MIMO. It is shown that the XR system capacity is limited with fewer satisfactory UEs that satisfy the reliability and latency requirements with the assumption of the baseline zero-forcing scheme. A precoding technique based on DL interference probing based on flexible A-SRS triggering is then evaluated and compared to baseline zero-forcing precoding.

In this paper, simulation results different from those presented in our previous papers are presented based on enhanced data traffic handling and scheduling. We were able to determine that the inaccuracy of the instantaneous feedback of CQI is the key factor that causes aggressive MCS assignment, thus causing the XR packet error and poor performance. We implemented an IIR filtering of the CQI to enable smoother and more conservative MCS assignment, which lead to generally better performance than before. A brief description of the enhancement is included in Section 2.2.

In section 3 and 4, we present downlink simulation results for different configurations including the following: Dense Urban FR1 and Urban Macro FR1, downlink slot configuration of [DDDSU] and [DDDUU], all for data rate of 45 Mbps with MU-MIMO. It is shown that the gain in terms of XR system capacity with enhanced cooperative MIMO scheme with MU-MIMO is substantial in the range of 19.6~64.8% for various configurations. The trend is similar to what we observed in [3] which was adopted in [4].

# Simulation Configurations

## FR1 Simulation Assumptions

This section includes the simulation assumptions used for Dense Urban FR1 and Urban Macro FR1 scenarios which follow the agreements made in meeting RAN1 #104-e and RAN1 #104-bis-e. The corresponding results are presented in Section 3 and Section 4. Table 1 summarizes the System Level Simulation SLS assumptions while Table 2 summarizes the traffic model parameters assumptions that were made to obtain the capacity results in Section 3 and Section 4. We further discuss the precoding, scheduling, traffic arrival offset assumptions made to compute the capacity of the system.

In RAN1 #104-bis-e meeting, RAN1 agreed on the exact parameters of Truncated Gaussian for the packet size distribution as well as the parameters of the jitter distribution which are presented in Table 2. The air interface delay budget used for the evaluations is 10 msec and the delay is measured from the point when a packet arrives at gNB to the point when it is successfully delivered to UE.

Table 1: Simulation Assumptions

| Scenario | Dense Urban FR1 | Urban Macro FR1 |
| --- | --- | --- |
| Channel Model | UMa TR 38.901 | UMa TR 38.901 |
| Layout | 21 cells with wraparound | 21 cells with wraparound |
| ISD | 200 m | 500 m |
| Carrier | 4 GHz | 4 GHz |
| SCS | 30 kHz | 30 kHz |
| BS height | 25m | 25m |
| BS noise figure | 5 dB | 5 dB |
| UE noise figure | 9 dB | 9 dB |
| UE receiver | MMSE-IRC | MMSE-IRC |
| Channel estimation DMRS | Realistic | Realistic |
| UE speed | 3km/hr | 3km/hr |
| MCS | Up to 256 QAM | Up to 256 QAM |
| BS antenna pattern | 3 sector radiation pattern, 8 dBi | 3 sector radiation pattern, 8 dBi |
| UE antenna pattern | Omni-directional, 0 dBi, | Omni-directional, 0 dBi, |
| UE distribution | 80% indoor and 20% outdoor | 80% indoor and 20% outdoor |
| BS antenna | Option 1: 64 TxRUs (8,8,2,1,1;4,8) (0.5 lambda, 0.5 lambda) | Option 1: 64 TxRUs (8,8,2,1,1;4,8) (0.5 lambda, 0.5 lambda) |
| Downtilt | 12 degree | 6 degree |
| BW | 100 MHz | 100 MHz |
| UE antenna | Baseline: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ | Baseline: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |
| BS Tx power | 44 dBm per 20 MHz | 49 dBm per 20 MHz |
| UE Tx power | 23 dBm | 23 dBm |
| HARQ | Number of retransmission: 4 | Number of retransmission: 4 |
| TDD configuration | Options: DDDUU, DDDSU | Options: DDDUU, DDDSU |
| UE height in meters | hUT=3(nfl – 1) + 1.5 nfl=1 for outdoor UEs nfl ~ uniform (1,Nfl) where Nfl ~uniform (4,8) | hUT=3(nfl – 1) + 1.5 nfl=1 for outdoor UEs nfl ~ uniform (1,Nfl) where Nfl ~uniform (4,8) |

Table 2: Downlink Traffic Model Simulation Assumptions for Dense Urban FR1 and Urban Macro FR1 Scenarios

| PDB for DL video stream | 10 msec |
| --- | --- |
| Average Data Rates | 45 Mbps |
| Frame rate | 60 fps |
| Jitter | - Truncated Gaussian - Mean 0 msec - STD 2 msec - Range [-4, 4] ms |
| Packet Size Distribution | - Truncated Gaussian - Mean 45 Mbps x 16.667 ms/frame = 750 kbits/frame - STD 10.5% x mean - Max: 1.5 x mean - Min: 0.5 x mean |

Further assumption on DL traffic model

In RAN1 #104-bis-e the following agreement was made for the case of single stream per UE in DL.

Agreement:

In case of single stream per UE in DL, a UE is declared a satisfied UE if more than X (%) of packets are successfully delivered within a given air interface PDB.

- The baseline X value is 99.

- Other values of X can be optionally evaluated, e.g., X < = 95, X=99.9.

- Additional combinations of (X, PDB) values can be optionally evaluated, e.g.,

  - (99, 7), (95, 13) for VR/AR

  - (99, 12), (95, 18) for CG

- FFS: Different values for I-frame and P-frame if evaluation of them is agreed.

In this contribution, the single stream per UE in DL is assumed and a UE is declared as a satisfied UE if more than 99% of packets are successfully delivered within 10 msec PDB.

In RAN1#103-e meeting the following agreement was made regarding the system capacity

Agreement:

System capacity is defined as the maximum number of users per cell with at least X % of UEs being satisfied.

- X=90 (baseline) or 95 (optional)

- Other values of X can also be evaluated optionally

Note: The exact ‘satisfied’ requirements will be discussed separately

FFS: how to calculate the percentage of satisfied users across multiple drops of simulations

We focus on the exact value of X being 90%, that is the system capacity is defined as the maximum number of users per cell such that at least 90% of UEs being satisfied.

Assumption on the traffic arrival offset

An important consideration that may affect the XR system capacity is the consideration of whether the periodic traffic pattern of XR users is aligned or have an offset relative to each other. In this paper, to reduce the number of evaluation results, we only consider the latter case, which represents the better case with the anticipated better performance where the UEs have arrival offsets randomly spread out within the packet arrival period.

Assumption on the gNB precoding

The baseline performance assumes the gNB performs zero forcing precoding to null the MU-MIMO interference. The corresponding results are presented in Section 3. A cooperative MIMO scheme based on DL interference probing (by uplink SRS utilizing TDD channel reciprocity) is then evaluated with the corresponding results in section 4. The description of the enhanced precoding is described in detail therein.

Assumption on the uneven/even load

For “uneven load”, the reported number of UEs per cell means the average number of UE’s across all cells. In this case, the actual number of UE’s from cell to cell can be different, depending on the cell association. For “even load”, every cell has exactly same number of UE’s associated with it, which generally has better performance. In this contribution, we focus on the “uneven load” only, to reduce the number of simulations. In our “uneven load” simulation, we first physically drop users evenly across all sectors, i.e. all sectors physically has same number of users residing in them. After that, we perform cell association for each user with a sector. This will end up with sectors that have different number of users, sometimes drastically different.

## Enhancement Scheme in System Simulation

During the performance evaluations, we observed that XR packet errors and poor performance are often caused by aggressive MCS assignment at some instances. Due to the high MCS assignment, even with the HARQ retransmissions in the PDB time period, combined SINR would not meet required SINR for the MCS assigned.

The instantaneous feedback of CQI and time delay between CQI feedback and PDSCH transmission coupled with channel/interference variation cause the inaccuracy. We implemented an IIR filtering of the CQI values to enable smoother and more conservative MCS assignment. The process is summarized in figure below.

Figure 1 CQI Filtering

The CQI filtering scheme is used in all the results presented in this paper. Combining with the relatively slow OLLA adjustment, the MCS assigned is less sensitive to the instantaneous CQI feedback that is heavily based on interferences caused by bursts of data packets scheduled for users in all the sectors. This scheme gives better results than those reported in our previous SI paper [3].

# Capacity Results with Zero forcing Precoding

## Capacity Results with Zero forcing Precoding

Section 3.2 will present all different downlink simulation results based on various configurations. In this section, we present the downlink MU-MIMO simulation of data packet rate 45Mbps, with slot configuration of [DDDUU].

With the simulation assumptions made in Section 2 and while assuming zero-forcing precoding, the percentages of satisfied UEs with satisfaction rate of 99% for Dense Urban and Urban Macro results are shown in figure below. For each scenario, uneven load is investigated. With the threshold of 90%, the capacity of XR system is limited to ~8.0 users/cell for Dense Urban scenario, and ~5.2 users/cell for Urban Macro scenario.

Figure 2: Zero forcing Precoding Capacity Results

Table 3: Capacity of the XR system assuming ZF precoding (with offset), data packet rate of 45Mbps, with slot configuration of [DDDUU]

| Scheme | Capacity (users/cell) (Uneven UE Load) |
| --- | --- |
| Dense Urban FR1 | ~8.0 |
| Urban Macro FR1 | ~5.2 |

Observation 1: For the FR1 Dense Urban Scenario and with the assumption of zero forcing precoding, the capacity of the XR system is ~8.0 UEs/cell for uneven UE load, with data packet rate of 45Mbps, with slot configuration of [DDDUU].

Observation 2: For the FR1 UMa Scenario and with the assumption of zero forcing precoding, the capacity of the XR system is ~5.2 UEs/cell for uneven UE load, with data packet rate of 45Mbps, with slot configuration of [DDDUU].

Proposal 1:  Study enhancements to MU-MIMO with a large number of antennas in order to increase the system capacity of XR

## Capacity Results with Zero forcing Precoding for More Configurations

The following table shows the performance of various configurations according to the table format of standards meeting 107-e, all with Zero forcing Precoding.

Table 4: Capacity of the XR system assuming ZF precoding (with offset)

| Scenario | App | PDB | Bit rate | Fps | MIMO | Capacity |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | [DDDUU] | [DDDSU] |
|  |  |  |  |  |  | ZF | ZF |
| DU | AR/VR | 10ms | 45Mbps | 60 | MU | 8 | 12.2 |

| Scenario | App | PDB | Bit rate | Fps | MIMO | Capacity |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | [DDDUU] | [DDDSU] |
|  |  |  |  |  |  | ZF | ZF |
| UMa | AR/VR | 10ms | 45Mbps | 60 | MU | 5.2 | 7 |
|  | CG | 15 ms | 45Mbps | 60 | MU | 6.8 | 9.7 |

# Capacity Results with Cooperative MIMO

## Cooperative MIMO for TDD with Downlink Interference Probing via SRS

Massive MIMO is considered as the major technique to achieve very high SE performance. One of the main issues in Massive MIMO is how to obtain accurate knowledge of the channels to enable spatial multiplexing of transmissions to multiple users without strong inter-user interference. For FDD systems, a relatively smaller number of antennas in the system limits the potential of performance, and in addition, without channel reciprocity, CSI acquisition needs to rely on CSI feedback which can lead to higher overhead, longer latency, lower accuracy, and hence limited SE performance. For TDD systems, Massive MIMO works relatively well thanks to accurate CSI acquisition through channel UL-DL reciprocity and generally provides higher SE than FDD.

However, channel UL-DL reciprocity is so far only utilized for single-cell (non-cooperative) MU-MIMO for TDD system and the gain is still far from what is promised with the massive number of antennas.

On the other hand, cooperative MIMO (e.g., CoMP in LTE) has not been found successful yet in practice. Here are some insights on the causes:

- Reciprocity in TDD is utilized (only) for channel information estimation for desired signals.

- Intra-cell interference (from MU-pairing) is handled relatively well, but for most scenarios with Massive MIMO inter-cell interference becomes the dominant limiting factor.

- Handling inter-cell interference using channel information of neighbouring cells is not successful even for TDD system:

  - Each individual interfering link may not be strong enough to obtain reliable channel information, by CSI feedback or by SRS.

  - The number of interfering links to consider is generally large, resulting in overhead, complexity, and robustness issues.

  - Each cell (or cell group) uses a centralized approach, which leads to significant complexity, robustness, backhaul, and edge-effect issues.

To resolve these issues and obtain the promised gain from the massive number of antennas, a new approach is proposed here. Instead of trying to obtain channel state information of interfering links and then determine precodings jointly at a centralized entity, SRS is enhanced to directly reflect DL interference spatial information (utilizing UL-DL reciprocity). Each gNB measures the corresponding SRS resources to obtain such information and adjusts its precoding to achieve interference coordination/avoidance.

To utilize TDD UL-DL reciprocity for interference measurement, the key is to tie some sounding activities to scheduled DL transmissions, which may be referred to as DL Interference Probing or SRS Probing. Then by TDD reciprocity, a gNB seeing strong UL interference from a certain spatial direction on the SRS resources (for example, via estimating the UL spatial covariance of interference signal) implies that in DL transmission the gNB will cause strong interference to UE(s) in that direction. The gNB can then adjust the precoding for DL interference avoidance. The gNBs in the network can coordinate the probing SRS resources in a semi-static fashion a priori, but essentially no inter-gNB information exchange is required on the fly. To summarize, DL Interference Probing from UL is possible if the network controls the UEs to transmit SRS in a way that best reflects prospective DL interference.

The above approach is illustrated in figure below, in which the sounding from UEs is based on prescheduling and reflects UE receive beamforming capabilities. As a result, the gNB can acquire DL interference spatial information. The gNB adjusts the precoders and thus DL SINR and SE can be improved. Furthermore, depending on the network implementation, more users/layers may be paired for MU-MIMO (such as via less conservative scheduling), yielding even higher SE gains. This approach of interference probing and mitigation, namely SRS probing, can be used to enable a variety of communication schemes to be implemented, generally in the category of Cooperative Massive MIMO.

Figure 3: TDD DL SE performance enhancement via interference probing and mitigation. To convey spatial information about the interference (both intra-cell and inter-cell) to the network, flexible A-SRS triggering based on prescheduling may be used.

The above approach is effective to suppress both inter-cell and intra-cell interference. This approach also has the following advantages:

- Distributed across gNBs (or TRPs) in the network, with low computational complexity for each gNB

- No or little channel information exchange among gNBs

- Can adapt to inter-cell interference, including unknown interference from non-cooperative gNBs or outside of the network (e.g., from other service providers, small cells, etc.)

- No need to estimate element-wise channel, reducing the complexity and overhead.

One specific way to implement is called bi-directional training (BiT); see, e.g., [5][6][7] (and references therein) for details for algorithms for narrowband systems, wideband systems, and dynamic TDD systems. Alternatively, BiT without multiple iterations (i.e., only one shot of interference probing before the PDSCH transmission, as shown in figure above can be performed, in which beamformed sounding based on conventional DL channel measurement and interference measurement is used. The latter is more practical for typical wireless communications.

## Capacity Results with BiT precoding

In this section, the results herein are divided into Dense Urban and UMa while assuming the cooperative MIMO transmission scheme.

We present sample results of the downlink MU-MIMO simulation of data packet rate 45Mbps, with slot configuration of [DDDUU]. Capacity Results with BiT Precoding for various configurations will be presented in Section 4.4 and compared with results of Zero-Forcing precoding baseline.

Dense Urban Results

With the assumption of BiT precoding described in Section 4.1, the capacity of the XR system has increased from ~8 UEs to ~13.1 UEs with the assumption of having uneven users traffic arrive with random offset. The advantage of BiT comes from the fact that inter and intra cell interference is nulled which may be achieved as explained in Section 4.1.

Figure 4: Capacity Results with BiT precoding: Dense Urban FR1, Uneven loads

Urban Macro Results

The capacity of the UMa scenario assuming the BiT precoding increases from 5.2UEs to ~6.6 UEs with the assumption of having BiT precoding.

Figure 5: Capacity Results with BiT precoding: Urban Macro FR1, Uneven UE loads

Table 5: Capacity of the XR system assuming BiT precoding (with offset), data packet rate of 45Mbps, with slot configuration of [DDDUU]

| Scheme | Capacity (users/cell) (Uneven UE Load) |
| --- | --- |
| Dense Urban FR1 | ~13.1 |
| Urban Macro FR1 | ~6.6 |

Note that the BiT performance of ~13.1 UEs and ~6.6 UEs for DU and UMa are improved from those reported in [3] which was ~10.6 UEs and ~5.5 UEs in the same configuration of data packet rate of 45Mbps and slot configuration of [DDDUU].  The reason for this improvement was described in Section 1.

Observation 3: For the Dense Urban Scenario with the BiT precoding assumption, the gain is 64.8% for the XR system capacity with the assumption of having uneven, with data packet rate of 45Mbps, with slot configuration of [DDDUU].

Observation 4: For the UMa Scenario with the BiT precoding assumption, the gain is 26.9% for the XR system capacity with the assumption of having uneven load, with data packet rate of 45Mbps, with slot configuration of [DDDUU].

Observation 5: TDD ZF performance can be significantly improved by flexible A-SRS triggering with dynamically indicated partial frequency sounding.

Proposal 2: Support cooperative MIMO via DL interference probing based on SRS enhancements to improve XR system capacity for TDD.

## Further Performance Analysis for Dense Urban FR1 scenario

The performance analysis in this section include: Resource Utilization (RU), 5%ile UPT and SINR CDF. It is assumed that there are 11 UEs/cell on average. The assumptions include a traffic arrival offset for UEs. The simulations have demonstrated generally similarly high gains of BiT over baseline ZF. This section uses the data packet rate of 45 Mbps and downlink slot configuration of [DDDUU].

### Resource Utilization

In figure below, we also see that BiT can reduce the RU with the same assumption of traffic load as that of ZF. The gain of (~7.6%) results from the increased spectrum efficiency due to much higher SINR (as shown in Figure 8) resulting from better interference suppression with BiT. With the reduction of RU, the interference in the network is reduced, further improving the SINR and UPT in the system as shown in the section 4.3.2 and 4.3.3.

Figure 6: RU comparison of BiT (with one-shot interference probing) and ZF-based Massive MIMO

### 5%UPT

With one-shot BiT, significant UPT gains (42.4% for 5%ile UPT) are achieved over ZF as shown in figure below. The gains primarily come from increased DL SINR and the increased total number of paired layers due to effective inter-cell interference avoidance.

Figure 7: Performance comparison of BiT (with one-shot interference probing) and ZF-based Massive MIMO for 5%ile UPT

### SINR

In figure below, a comparison of BIT and ZF show an SINR improvement of about 6 dB at 50%ile due to the superb capability of interference mitigation by BiT. The gains on DL SINR are due to effective inter-cell interference avoidance which results in increase in UPT, decrease in RU and decrease in PER.

Figure 8: Performance comparison of BiT (with one-shot interference probing) and ZF-based Massive MIMO for DL SINR

## Capacity Results with BiT Precoding for More Configurations

The following table shows the performance of various configurations according to the table format of standards meeting 107-e, for both Zero-Forcing and BiT precodings.

From the performances of various configurations, we can observe the following phenomenon regarding the gains of BiT vs. ZF:

DU scenario has higher gains than Uma scenario:  This is due to the shorter inter-cell distance in DU scenario. In this case, interference is more dominating than noise. BiT can do good job to suppress interference, but not meant to suppress noise level.

Table 6: Capacity of the XR system assuming ZF and BiT precoding (with offset)

| Scenario | App | PDB | Bit rate | Fps | MIMO | Capacity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | [DDDUU] |  |  | [DDDSU] |  |  |
|  |  |  |  |  |  | ZF | BiT | BiT Gain | ZF | BiT | BiT Gain |
| DU | AR/VR | 10ms | 45Mbps | 60 | MU | 8 | 13.1 | 64.8% | 12.2 | 16.9 | 38.5% |

| Scenario | App | PDB | Bit rate | Fps | MIMO | Capacity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | [DDDUU] |  |  | [DDDSU] |  |  |
|  |  |  |  |  |  | ZF | BiT | BiT Gain | ZF | BiT | BiT Gain |
| UMa | AR/VR | 10ms | 45Mbps | 60 | MU | 5.2 | 6.6 | 26.9% | 7 | 9 | 28.6% |
|  | CG | 15 ms | 45Mbps | 60 | MU | 6.8 | 9.4 | 38.2% | 9.7 | 11.6 | 19.6% |

Observation 6: DU scenario experiences higher gains than Uma scenario with the BiT precoding relative to Zero-Forcing precoding, due to the shorter inter-cell distance, in which interference is more dominating than noise.

# Conclusions

In this contribution, we evaluate performance of XR system capacity for different configurations. Cooperative MIMO scheme via precoding technique based on DL interference probing is evaluated and compared to baseline MU-MIMO using zero forcing precoding. It is shown that the former increases the XR system capacity by 19.6~64.8% for the DU and UMa scenarios for various configurations, when UEs have traffic arrival offset and uneven UE loads. Based on the discussions in the previous sections we propose the following:

Proposal 1:  Study enhancements to MU-MIMO with a large number of antennas in order to increase the system capacity of XR

Proposal 2: Support cooperative MIMO via DL interference probing based on SRS enhancements to improve XR system capacity for TDD.

Observation 1: For the FR1 Dense Urban Scenario and with the assumption of zero forcing precoding, the capacity of the XR system is ~8.0 UEs/cell  for uneven UE load, with data packet rate of 45Mbps, with slot configuration of [DDDUU].

Observation 2: For the FR1 UMa Scenario and with the assumption of zero forcing precoding, the capacity of the XR system is ~5.2 UEs/cell  for uneven UE load, with data packet rate of 45Mbps, with slot configuration of [DDDUU].

Observation 3: For the Dense Urban Scenario with the BiT precoding assumption, the gain is 64.8% for the XR system capacity with the assumption of having uneven, with data packet rate of 45Mbps, with slot configuration of [DDDUU].

Observation 4: For the UMa Scenario with the BiT precoding assumption, the gain is 26.9% for the XR system capacity with the assumption of having uneven load, with data packet rate of 45Mbps, with slot configuration of [DDDUU].

Observation 5: TDD ZF performance can be significantly improved by flexible A-SRS triggering with dynamically indicated partial frequency sounding.

Observation 6: DU scenario experiences higher gains than Uma scenario with the BiT precoding relative to Zero-Forcing precoding, due to the shorter inter-cell distance, in which interference is more dominating than noise.

# References

RP-220285, “Revised SID : Study on XR Enhancements for NR”, Nokia, RAN#95e.

3GPP RAN1, RAN1#104-e, Chairman Notes.

R1-2110885, “XR evaluation results”, Futurewei, RAN1#107-e

TR 38.838, V17.0.0 (2021-12), "Study on XR (Extended Reality) Evaluations for NR" (Release 17)

H. Zhou, J. Liu, Q. Cheng, D. Maamari, W. Xiao, and A. C. K. Soong, "Bi-Directional Training with Rank Optimization and Fairness Control," in 2018 IEEE 88th Vehicular Technology Conference (VTC-Fall), 2018.

J. Liu, Q. Cheng, W. Xiao, D. Maamari, and A. C. K. Soong, "Bi-directional Training for Wideband Systems," in 2019 IEEE 89th Vehicular Technology Conference (VTC-Fall), 2019.

R1-2102765, “Enhancements on SRS flexibility, coverage and capacity”, FUTUREWEI, RAN1#104-bis-e.