3GPP TSG RAN WG1 Meeting #109-e	R1- 2203068

e-Meeting, May 9 – 20, 2022

Agenda Item:	9.2.2.1

Source:	Futurewei

Title:	Discussion on evaluation of AI/ML for CSI feedback enhancement use case

Document for:	Discussion and decision

# Introduction

In RAN#94-e, the Rel. 18 NR_AIML_Air study item was approved, and the scopes and objectives were agreed  [1]. The initial set of use cases includes:

- CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

- Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

- Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

For the use cases under consideration:

- Evaluate performance benefits of AI/ML based algorithms for the agreed use cases in the final representative set:

  - Methodology based on statistical models (from TR 38.901 and TR 38.857 [positioning]), for link and system level simulations.

    - Extensions of 3GPP evaluation methodology for better suitability to AI/ML based techniques should be considered as needed.

    - Whether field data are optionally needed to further assess the performance and robustness in real-world environments should be discussed as part of the study.

    - Need for common assumptions in dataset construction for training, validation and test for the selected use cases.

    - Consider adequate model training strategy, collaboration levels and associated implications

    - Consider agreed-upon base AI model(s) for calibration

    - AI model description and training methodology used for evaluation should be reported for information and cross-checking purposes

  - KPIs: Determine the common KPIs and corresponding requirements for the AI/ML operations. Determine the use-case specific KPIs and benchmarks of the selected use-cases.

    - Performance, inference latency and computational complexity of AI/ML based algorithms should be compared to that of a state-of-the-art baseline

    - Overhead, power consumption (including computational), memory storage, and hardware requirements (including for given processing delays) associated with enabling respective AI/ML scheme, as well as generalization capability should be considered.

For each use case under consideration, the following objectives were agreed [1]:

In this contribution, we present our views on the evaluation methodology and KPIs for performance and complexity evaluation pertinent to the AI/ML-based CSI feedback enhancement use case.

# Evaluation methodology and KPIs for AI/ML based CSI feedback enhancements

In [2], we have discussed potential sub use cases under AI/ML-based CSI feedback enhancements, and we propose treating “CSI feedback overhead reduction and reconstruction accuracy improvement” sub use case as the representative sub use case in the study phase. Thus, in this section, we discuss the evaluation methodology and needed KPIs to assess the performance and complexity of AI/ML-based CSI feedback overhead reduction and reconstruction improvement sub use case.

Unlike analytics-based method, AI/ML-based approach is data-driven and there are different characteristics we need to consider first before discussing the evaluation methodology and KPIs for AI/ML-based CSI feedback enhancement.

Typically, data used for training an AI/ML model should be close to the deployment scenario for which the model will be used, and data / observations used in the testing phase is also drawn from the same distribution as the training data. In some cases, it may be costly to build or deploy different models for different scenarios, instead, a generalized AI/ML model can be trained using data collected from various scenarios and this generalized model can work in multiple deployment scenarios.  Thus, we believe the Rel-18 study needs to consider / include the following:

- For each (sub) use case, discuss whether a scenario-based modelling/training approach is expected, or a generalized modelling/training approach is preferred, or both should be supported.

- Based on the above, discuss how the training / testing data should be generated. In the case that a generalized model is expected to support multiple scenarios, also discuss whether the testing data will be generated from the same scenario(s) / distribution as the training data, or it will be generated from different scenarios (in this case, the model trained using data from one scenario (or few scenarios) / distribution should generalize to another scenario or multiple scenarios during the inference / testing phase.

Observation 1: For a given use case, AI/ML-based approach can be designed as a scenario-specific solution, or it can be designed as a generalized solution across multiple scenarios.

Proposal 1: Discuss the following aspects in the Rel-18 study for each selected (sub) use case:

- Scenario-based vs. generalized AI/ML model training/inference

- Data generation methodology for scenario-based vs. generalized AI/ML modelling approach

  - In the case of generalized modelling, further discuss whether the testing data is generated from scenarios different than the scenarios from which the training data is generated.

## Existing evaluation methodology and KPIs

In Rel-16 the evaluation methodology (EVM) and assumptions for CSI enhancement have been discussed in details and document in [3]. As a starting point, we suggest adopting the EVM agreed in Rel-16 as the EVM for Rel-18 CSI feedback enhancements use case.

In addition, Rel-16 EVM for CSI enhancement has specified the following evaluation metrics:

Throughput and CSI feedback overhead as baseline metrics.

Additional metrics, e.g., ratio between throughput and CSI feedback overhead, can be used.

Maximum overhead (payload size for CSI feedback) for each rank at one feedback instance is the baseline metric for CSI feedback overhead, and companies can provide other metrics.

These evaluation metrics can be adopted as part of the evaluation metrics for the Rel-18 CSI feedback enhancements use case.

Proposal 2: Use the assumptions and EVM (including KPIs) agreed in Rel-16 for CSI enhancement study for the evaluation of Rel-18 CSI feedback enhancements.

## Other KPIs

Unlike analytics-based method, most AI/ML-based approaches learn to perform prediction from training data (during training phase). Typically, a subset of the available data is set aside for testing purpose to assess how well the trained model can generalize to data unseen during the training phase. After the AI/ML model training and testing phases are completed, then the model can be deployed to make prediction using new data as input (during inference phase).

For the CSI feedback overhead reduction and reconstruction improvement sub use case, the overall goal is to reduce CSI feedback overhead and improve system / network throughput. An AI/ML encoder can be built to learn to reduce the dimension/size of the CSI feedback (input) at the UE side, then an AI/ML decoder at gNB side will learn to reconstruct the original CSI feedback input. This learning process is based on the training data acquired. As discussed above, once trained, AI/ML model testing phase verifies how well the encoder-decoder type of AI/ML model works to recover the original CSI feedback, e.g., how close the predicted outcome is to the input, and/or how accurate the prediction result is.

Figure 2.2-1: Training and testing process of AI/ML for CSI feedback enhancement

From the above, we can see there are two sets of KPIs, one set is to ensure that the AI/ML model performs correctly and accurately, and the other set is to verify that the overall performance goal of the use case can be achieved. Figure 2.2-1 depicts the AI/ML model training and testing process for the CSI feedback enhancement and how the two sets of KPIs cooperate to achieve the final goal.

Observation 2: AI/ML model learns to perform prediction using training data (during training phase). A subset of the available data is set aside as testing data to evaluate whether the trained AI/ML model can generalize to unseen data (during testing phase).

Observation 3: The KPI(s) that are used to assess the performance of AI/ML model(s) and the KPI(s) that are used to ensure the system / network level performance are both needed to verify whether the overall performance goal of the use case can be achieved.

Proposal 3: In addition to evaluating system / network performance, include evaluation metrics to assess the performance of AI/ML model(s) for each identified (sub) use case.

In this sub section, we discuss other KPIs for CSI feedback overhead reduction and reconstruction accuracy enhancement from AI/ML modeling perspective.

### Performance related KPIs

For CSI feedback overhead reduction and reconstruction accuracy improvement sub use case, we leverage AI/ML techniques to recover the original input (CSI feedback from the UE side). Thus, direct indication of AI/ML model performance is to understand / measure how close the predicted CSI feedback is to the original input. We suggest using normalized mean square error (NMSE) and cosine similarity to evaluate the performance of AI/ML-based CSI compression and reconstruction algorithm. These two KPIs are widely adopted by numerous research papers for CSI prediction related problems like [5][6]. In the following, we describe NMSE and cosine similarity.

- Normalized Mean Square Error (NMSE)

  - NMSE measures normalized mean square error (MSE)

  - Definition:

NMSE =  E (∥h -h∥2∥h∥2) , where h is the prediction of h, and 𝔼( 𝅭 ) represents expected value.

Note: h represents the input to the AI/ML model for CSI-feedback enhancements, for example, h may be the full CSI feedback channel vector(s), or the channel vector(s) after the pre-processing step.  NMSE can also be measured/converted in/to dB.

- Cosine similarity:

  - Given two vectors of attributes, h and h, the cosine similarity, cos(θ), is represented using a dot product and magnitude.

  - Definition:

Cosine similarity or cos( ) = E (h ⋅ h∥h∥∥h∥)

Observation 4: When using AI/ML-based approach to compress/predict CSI feedback, normalized mean square error (NMSE) and cosine similarity are widely adopted as KPIs to evaluate the performance of AI/ML models.

Proposal 4: Include normalized mean square error (NMSE) and cosine similarity, cos () in the KPI list to assess the performance of AI/ML model(s) for CSI feedback overhead reduction and reconstruction accuracy improvement sub use case.

### Complexity related KPIs

Assumption: the discussion in this sub section assumes that a neural network (NN) based encoder-decoder type of AI/ML architecture is leveraged for CSI feedback compression and reconstruction.

In designing AI/ML-based approach, it is very useful and important to take into account its associated complexity to make sure the requirements for memory and computation can be supported by the hardware (HW) and software (SW) environments in which the AI/ML model will be hosted/used. Understanding the complexity also helps in designing more HW, SW and data efficient AI/ML models. The term “complexity” for neural network-based models may cover many aspects, like space/memory, computation, execution time, energy, etc. while some of the aspects may be HW or SW environment-dependent, e.g., execution time and energy.

For NN based AI/ML models, complexity mainly concerns two aspects that are not dependent on the HW or SW environment, the first is the amount of space or memory needed and the other aspect is the number of computational operations required to run the NN model.

- Space: this aspect mainly relates to amount memory usage during training and inference.

- Computational operations: this aspect mainly concerns the computation power and time needed to support or run the designed NN model.

In the following, we briefly describe space (or model) complexity and computational (or operation) complexity, and their components while the details can be found in [4].

- Model (or space) complexity can be characterized using the following:

  - Number of parameters in AI/ML model(s): this measures the number of coefficients of the NN model, which need to be optimized during training according to a given optimization strategy. If the AI/ML algorithm for CSI feedback enhancements utilizes an encoder and decoder type or architecture, then it is desirable to calculate the complexity for encoder and decoder separately.

  - Memory usage

    - Memory usage of NN-based AI/ML model consists of various components and number of parameters is one of the contributors. In general, memory usage is separately calculated for training and inference and includes the following aspects:

      - Training time:

        - Memory used to store outputs of the intermediate layers (forward propagation)

        - Memory used to store parameters

        - Memory used to store error signals

        - Memory used to store gradient of parameters

        - Additional memory used by the chosen optimizer, e.g., for momentum

      - Inference time:

        - Memory used to store outputs of the intermediate layers

        - Memory to store parameters

Figure 2.2.2-1: Memory usage during model Training vs. memory usage during Inference

Figure 2.2.2-1 depicts the memory usage during training time and inference time.

As shown in Figure 2.2.2-1, true memory usage depends on implementation. In addition, memory usage during training time can be significantly larger than memory usage during inference time; thus, it is desirable to calculate them separately. This is also very helpful when model training and inference are taken place in different entities. For example, in the scenario when model training is performed on gNB while UE side is doing the inference, UE only needs to consider the memory usage during inference time.

- Computational complexity: computations in (deep) neural network or deep learning-based AI/ML model is dominated by multiply-adds in fully connection layers and convolutional layers. Typically, number of floating-point operations (FLOPs) or number of multiplies and accumulates (MACs), is used to estimate the multiply-adds operations in the forward propagation, and any non-linearities, dropout, and normalization layers can be ignored as they have negligible computation cost.

  - Floating Point Operations (FLOPs): separate estimation for encoder and decoder is preferred.

    - For fully connected layer:

FLOPs = number_of_inputs  number_of_outputs (ignoring biases)

    - For convolution layer and variants of convolutional layer: Chapter 2 (“Model Efficiency: Metrics of model efficiency”) in [4] describes the formulas for calculating FLOPs for convolution layer and its variants.

Observation 5: When leveraging AI/ML-based approach, it is very important to understand the associated complexity to make sure the requirements for memory and computation can be supported by the hardware (HW) and software (SW) environments in which the AI/ML model will be hosted/used.

Observation 6: Complexity for NN-based AI/ML models can be measured using model (or space) complexity and computational complexity, which reflect the memory usage and computation power needed respectively.

Proposal 5: Include model/space complexity as one of the KPIs to evaluate the complexity of AI/ML-based approach for CSI feedback overhead reduction and reconstruction accuracy improvement sub use case. Model/space complexity measurements include number of parameters in the model and memory usage.

Proposal 6: Include computational complexity as one of the KPIs to evaluate the complexity of AI/ML-based approach for CSI feedback overhead reduction and reconstruction accuracy improvement sub use case. Computational/operation complexity can be estimated using number of floating-point operations (FLOPs) or number of multiplies and accumulates (MACs).

# Conclusions

In this contribution, we present our views on the evaluation methodology and KPIs for performance evaluation pertinent to the AI/ML-based CSI feedback enhancement use case.  Based on the discussions in the previous sections, our observations and proposals are as follows.

General aspects of evaluation methodology:

Observation 1: For a given use case, AI/ML-based approach can be designed as a scenario-specific solution, or it can be designed as a generalized solution across multiple scenarios.

Proposal 1: Discuss the following aspects in the Rel-18 study for each selected (sub) use case:

- Scenario-based vs. generalized AI/ML model training/inference.

- Data generation methodology for scenario-based vs. generalized AI/ML modelling approach.

  - In the case of generalized modelling, further discuss whether the testing data is generated from scenarios different than the scenarios from which the training data is generated.

Existing evaluation methodology and KPIs for CSI enhancement:

Proposal 2: Use the assumptions and EVM (including KPIs) agreed in Rel-16 for CSI enhancement study for the evaluation of Rel-18 CSI feedback enhancements.

Other KPIs from AI/ML modeling perspective:

Observation 2: AI/ML model learns to perform prediction using training data (during training phase). A subset of the available data is set aside as testing data to evaluate whether the trained AI/ML model can generalize to unseen data (during testing phase).

Observation 3: The KPI(s) that are used to assess the performance of AI/ML model(s) and the KPI(s) that are used to ensure the system / network level performance are both needed to verify whether the overall performance goal of the use case can be achieved.

Observation 4: When using AI/ML-based approach to compress/predict CSI feedback, normalized mean square error (NMSE) and cosine similarity are widely adopted as KPIs to evaluate the performance of AI/ML models.

Observation 5: When leveraging AI/ML-based approach, it is very important to understand the associated complexity to make sure the requirements for memory and computation can be supported by the hardware (HW) and software (SW) environments in which the AI/ML model will be hosted/used.

Observation 6: Complexity for NN-based AI/ML models can be measured using model (or space) complexity and computational complexity, which reflect the memory usage and computation power needed respectively.

Proposal 3: In addition to evaluating system / network performance, include evaluation metrics to assess the performance of AI/ML model(s) for each identified (sub) use case.

Proposal 4: Include normalized mean square error (NMSE) and cosine similarity, cos () in the KPI list to assess the performance of AI/ML model(s) for CSI feedback overhead reduction and reconstruction accuracy improvement sub use case.

Proposal 5: Include model/space complexity as one of the KPIs to evaluate the complexity of AI/ML-based approach for CSI feedback overhead reduction and reconstruction accuracy improvement sub use case. Model/space complexity measurements include number of parameters in the model and memory usage.

Proposal 6: Include computational complexity as one of the KPIs to evaluate the complexity of AI/ML-based approach for CSI feedback overhead reduction and reconstruction accuracy improvement sub use case. Computational/operation complexity can be estimated using number of floating-point operations (FLOPs) or number of multiplies and accumulates (MACs).

Note: For proposals 5 and 6, it is assumed that a neural network (NN) based encoder-decoder type of AI/ML architecture is leveraged for CSI feedback compression and reconstruction.

# References

RP-213599, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#94-e.

R1- 2203069, “Discussion on sub use cases of AI/ML for CSI feedback enhancement use case”, Futurewei.

3GPP RAN1, RAN1#94-bis, Chairman Notes.

B. Wu, Efficient Deep Neural Networks, PhD thesis, EECS Department, University of California, Berkeley, Aug 2019.

W. Utschick, V. Rizzello, M. Joham, Z. Ma and L. Piazzi, "Learning the CSI Recovery in FDD Systems," in IEEE Transactions on Wireless Communications, doi: 10.1109/TWC.2022.3149946.

P. Dong, H. Zhang and G. Y. Li, "Machine Learning Prediction Based CSI Acquisition for FDD Massive MIMO Downlink," 2018 IEEE Global Communications Conference (GLOBECOM), 2018, pp. 1-6, doi: 10.1109/GLOCOM.2018.8647328.