3GPP TSG RAN WG1 Meeting #109-e	R1-2203069

e-Meeting, May 9 – 20, 2022

Agenda Item:	9.2.2.2

Source:	Futurewei

Title:	Discussion on sub use cases of AI/ML for CSI feedback enhancement use case

Document for:	Discussion and decision

# Introduction

Study the 3GPP framework for AI/ML for air-interface corresponding to each target use case regarding aspects such as performance, complexity, and potential specification impact.

Use cases to focus on:

- Initial set of use cases includes:

  - CSI feedback enhancement, e.g., overhead reduction, improved accuracy, prediction [RAN1]

  - Beam management, e.g., beam prediction in time, and/or spatial domain for overhead and latency reduction, beam selection accuracy improvement [RAN1]

  - Positioning accuracy enhancements for different scenarios including, e.g., those with heavy NLOS conditions [RAN1]

- Finalize representative sub use cases for each use case for characterization and baseline performance evaluations by RAN#98

  - The AI/ML approaches for the selected sub use cases need to be diverse enough to support various requirements on the gNB-UE collaboration levels

Note: the selection of use cases for this study solely targets the formulation of a framework to apply AI/ML to the air-interface for these and other use cases. The selection itself does not intend to provide any indication of the prospects of any future normative project.

In RAN#94-e, the Rel. 18 NR_AIML_Air study was approved, and the scopes and objectives were agreed  [1]. The initial use cases to focus for the study are as follows:

In this contribution, we present our views on potential sub use cases under the AI/ML for CSI feedback enhancement use case.

# AI/ML for CSI feedback enhancements

In massive MIMO systems, the gNBs  are equipped with a large number of antennas, thereby mitigating the effects of fading, thermal noise, and intra-cell and inter-cell interference while supporting much better spectral efficiency by higher order MU-MIMO. These potential benefits are based on the assumption that the gNB has obtained the instantaneous CSI and the accuracy of the CSI directly affects the performance of the massive MIMO systems. The user equipment (UE) estimates the downlink channel by the reference signals sent by the gNB first. Then, the estimated downlink CSI is fed back to the gNB through the uplink. However, in massive MIMO systems, this feedback leads to a large overhead and occupies a lot of uplink bandwidth resources because the dimension of the CSI matrix is much higher due to the large number of antennas at the gNB. Traditional analytics-based channel estimation methods, even though with good performance, become not scalable as the estimation is performed repeatedly and the results have to be available almost instantaneously.

In recent years, thanks to the advances in artificial intelligence (AI) / machine learning (ML), and improvements in computing power, Deep Learning (DL)-based image compression methods have achieved superior performance compared to traditional algorithms. Motivated by this, many academic research works have leveraged DL-based algorithm to improve the accuracy of CSI prediction and reducing the overhead in FDD systems [2]. However, the results of most academic works are not considering realistic assumptions and these algorithms are usually not validated using practical configurations in real deployment communication networks either.

Considering the above, we believe it is beneficial for the standards to study the benefits of leveraging AI/ML techniques for physical layer functions and the associated impacts and complexity. CSI feedback enhancement is part of the initial set of use cases to be studied under the Rel. 18 NR_AIML_Air SID. During RAN#94-e NWM discussion, companies proposed several (sub) use cases related to CSI feedback enhancements, and they include:

- CSI Feedback overhead reduction and reconstruction accuracy improvement

- CSI-RS overhead reduction

- CSI prediction

In the following sub sections, we discuss each sub use case at high level and share our views on selecting representative sub use case for the Rel-18 study.

## Sub use case: CSI feedback overhead reduction and high accuracy reconstruction

Figure 2.1-1: A high level architecture diagram of AI/ML-based CSI feedback

A high-level architecture of AI/ML-based CSI feedback is depicted in Figure 2.1-1, in which an AI/ML encoder at the UE side will compress / encode the CSI feedback to a bit stream and send the feedback bits to the gNB. Once received, gNB will decode the feedback bits to recover / reconstruct the CSI feedback to its original form.

The above high-level architecture for CSI feedback may introduce impacts on the UE side for using a new AI/ML-based encoder, on the gNB side for using a new AI/ML-based decoder, and on the air-interface for exchanging needed information, including configurations and potential AI/ML models between the UE and gNB. Even though the actual information to be exchanged will be studied and discussed during this Rel-18 SI phase, it is expected the standards impact can be complicated for this sub use case.

Despite the potential standards impact, one important consideration in selecting (sub) use case(s) for the Rel-18 study is whether the (sub) use case can better help us study and support various requirements across various gNB-UE collaboration levels. As this use case involves potential model information exchange(s) on the air interface and joint AI/ML operations (learning, inference, and update), we expect this sub use will enable us to study the following:

- How to apply and support various AI/ML operations (e.g., during training/validation, testing) when involving air-interface.

- Proper handling of exchanging AI/ML model and supporting information and the associated formations between the UE and the gNB, and between neighbouring nodes/gNBs.

- Joint AI/ML operations (training, inference, and update) between the UE and the gNB and the corresponding AI/ML model life cycle management.

## Sub use case: CSI-RS overhead reduction

During RAN#94-e discussion, some companies also proposed to include CSI-RS overhead reduction as one of the sub-use cases to allow gNB to use less ports in sending CSI-RS while UE still can accurately estimate CSI for full antenna ports. A high-level architecture for this sub use case is depicted in Figure 2.2-1, in which less CSI-RS ports are used on the gNB side in sending CSI-RS to UE. Once received, the UE will leverage AI/ML-based approaches to predict CSI for full antenna ports.

Figure 2.2-1: A high level architecture diagram of AI/ML-based CSI-RS overhead reduction

This high-level architecture for CSI-RS overhead reduction that leverages AI/ML-based techniques to predict / recover CSI for full antenna ports may introduce impacts on the UE side for using a new AI/ML-based predictor and potentially on air-interface for exchanging needed information including configurations and AI/ML model downloading to UE if gNB performs AI/ML model training procedure. Note that the model downloading step is not needed if UE performs the AI/ML model training procedure. Even though the actual information to be exchanged and the associated air-interface enhancements will be studied and discussed during this Rel-18 SI phase, it is expected the standards impact may be relatively less extensive compared to the CSI feedback overhead reduction and high accuracy reconstruction sub use case discussed in sub section 2.1.

Note that similar diagram can be applied to overhead reduction in resource element (RE) level.

## Sub use case: CSI prediction

Another potential sub use case is CSI prediction as indicated in [1]. This sub use case leverages AI/ML techniques to predict current and future CSIs using the collected historical CSI and other relevant information. The goal for this sub use case is to reduce the number of channel estimations and save the overhead of reference signals and of uplink feedback reports. It may include different implementation flavors / options, e.g., the prediction procedure is performed on the gNB, or the prediction procedure is performed on the UE side which may incur different collaboration levels. The corresponding standards impact will also be different. For the option that gNB is predicting current/future CSI, depending on the algorithm design, there may not be standards impact assuming gNB also preforms model training function. For the option that UE is predicting current/future CSI, it may introduce standards impact on air-interface for configurations and/or information exchange between gNB and UE, including potential model downloading if gNB performs AI/ML model training procedure. Note that the model downloading step is not needed if UE performs the AI/ML model training procedure. Even though the actual information to be exchanged and the associated air-interface enhancements will be studied and discussed during this Rel-18 SI phase, we expect the standards impact may be relatively less extensive (e.g., no joint learning between gNB and UE) compared to the CSI feedback overhead reduction and high accuracy reconstruction sub use case discussed in sub section 2.1.

## Summary view of sub use cases

As discussed in the previous sub sections, the actual impacts to the Standards and benefits for the 3 sub use cases are yet to be studied and analysed during the study phase. At high level, some of the aspects are summarized in Table 2.4-1.

Table 2.4-1: Summary view of sub use cases

| Sub use case | Description | AI/ML Potentials | Potential Standards impact | Help to assess AI/ML operations |
| --- | --- | --- | --- | --- |
| CSI feedback overhead reduction and accuracy improvement | Leverage AI/ML-based approach to encode the CSI feedback on the UE side to achieve high compression ratio, followed by decoding the received bits and accurately reconstructing the CSI feedback at gNB. | - Reduce CSI feedback overhead - Improve reconstruction accuracy | Relatively higher, involving signaling enhancements and model (and other information) exchanges. | Greater |
| CSI-RS overhead reduction | Leverage AI/ML-based approach to reduce CSI-RS overhead, either at port-level or at RE-level. | - Reduce CSI-RS overhead (ports or RE) | Relatively lower, details may depend on design and implementation. | Less |
| CSI prediction | Predict current/future CSI by using historically received CSIs. | - Reduce channel estimation complexity - Improve estimation accuracy | Relatively lower, details may depend on which network entity performs AI/ML model training and inference. | Less |

From the above analysis, the sub use case of “CSI feedback overhead reduction and reconstruction accuracy improvement” will enable us to assess the standards impacts of various AI/ML operations on gNB, UE and air-interface. In addition, this sub use case involves more advanced collaboration level between gNB and UE. i.e., joint learning, which will help us gain more AI/ML experience through the study of this sub use case.

Observation 1: The CSI feedback overhead reduction and reconstruction accuracy improvement sub use case may have relatively more standards impact and involve more advanced collaboration between gNB and UE, e.g., joint learning.

Observation 2: The CSI feedback overhead reduction and reconstruction accuracy improvement sub use case will enable us to study the following:

- how to support various AI/ML operations when involving air-interface

- proper handling of exchanging AI/ML model and supporting information

- joint AI/ML operations (training, inference, and update) between UE and gNB and the corresponding AI/ML model life cycle management

Based on the above observations, we have the following proposal:

Proposal 1: Consider “CSI feedback overhead reduction and reconstruction accuracy improvement” as the representative sub use case for CSI feedback enhancement use case in the Rel-18 study.

# Conclusions

In this contribution, we present our views on potential sub use cases of the AI/ML for CSI feedback enhancement use case.  Based on the discussions in the previous sections our observations and proposal are as follows.

Observation 1: The CSI feedback overhead reduction and reconstruction accuracy improvement sub use case may have relatively more standards impact and involve more advanced collaboration between gNB and UE, e.g., joint learning.

Observation 2: The CSI feedback overhead reduction and reconstruction accuracy improvement sub use case will allow us to study the following:

- how to support various AI/ML operations when involving air-interface

- proper handling of exchanging AI/ML model and supporting information

- joint AI/ML operations (training, inference, and update) between UE and gNB and the corresponding AI/ML model life cycle management

Proposal 1: Consider “CSI feedback overhead reduction and reconstruction accuracy improvement” as the representative sub use case for CSI feedback enhancement use case in the Rel-18 study.

# References

RP-213599, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#94-e.

Tanveer, J.; Haider, A.; Ali, R.; Kim, A. Machine Learning for Physical Layer in 5G and beyond Wireless Networks: A Survey. Electronics 2022, 11, 121.