3GPP TSG-RAN WG1 Meeting #109-e	R1-2203070

e-Meeting, May 9 – May 20, 2022

Agenda Item:	8.12.1

Source:	Huawei, HiSilicon, CBN

Title:	Mechanisms to improve reliability for RRC_CONNECTED UEs

Document for:	Discussion and Decision

Introduction

This paper discusses the remaining issues for UL-DAI for multicast, multiplexing different codebook types, and NACK-only based HARQ-ACK feedback.

ACK/NACK based HARQ-ACK feedback

UL-DAI for multicast

The agreements achieved in the last meeting are as follows:

Agreement If Type-1 codebook is configured for both multicast and unicast, at least for single cell case for both unicast and multicast: - If the UE is configured to construct the HARQ-ACK codebooks for unicast and multicast jointly, a single UL DAI bit applies for unicast and multicast - Otherwise, 1 additional bit UL DAI is included for multicast in DCI format 0_1/0_2, in addition to the UL DAI for unicast. The 1-bit UL DAI for multicast is applied to all configured G-RNTIs. - FFS: additional restrictions Agreement For the following two cases, the fallback operation for the Type-1 HARQ-ACK codebook for multicast is applied to G-RNTI/G-CS-RNTI configured with HARQ-ACK enabled only, - a SPS PDSCH release indicated by DCI format 4_1 with counter DAI field value of 1, - a PDSCH reception scheduled by DCI format 4_1 with counter DAI field value of 1 on the PCell, - SPS PDSCH reception(s) associated with G-CS-RNTIs. - Note: If the UE receives two SPS PDSCH releases for two respective G-CS-RNTIs with DAI=1, or two PDSCHs by DCI 4_1 with DAI=1 for two respective G-RNTIs on the PCell, there is no fallback. Agreement If Type-2 codebook is configured for unicast and multicast, the following option2-2 (from the previous agreement) is adopted: - Option2-2: 2-bit UL DAI(s) are included in DCI for multicast, in addition to the 2-bit UL DAI for unicast. - The 2-bit UL DAI for multicast is applied for all configured G-RNTIs. - FFS: how to count the total number of HARQ-ACK bits for all configured G-RNTIs. - Alt1-1: UL-DAI indicates the sum of DL-DAIs - Alt1-2: DL-DAIs have the same value. - Alt1-3: largest DL DAI value among the configured G-RNTIs. - Other alternatives are not precluded Agreement When Type-1 codebook is configured for unicast and Type-2 codebook is configured for multicast, or when Type-2 codebook is configured for unicast and Type-1 codebook is configured for multicast, the UL-DAI for multicast is included in DCI format 0_1/0_2, in addition to the UL-DAI field for unicast. - The UL-DAI for multicast is 1-bit for Type-1 codebook - The 1-bit UL-DAI for multicast is applied to all configured G-RNTIs. - The UL-DAI for multicast is 2-bit for Type-2 codebook applied for all configured G-RNTIs. - FFS: how to count the total number of HARQ-ACK bits for all configured G-RNTIs. - Alt1-1: UL-DAI indicates the sum of DL-DAIs - Alt1-2: DL-DAIs have the same value. - Alt1-3: largest DL DAI value among the configured G-RNTIs. - Other alternatives are not precluded - FFS: details of the UL DAI field

It was agreed that an UL-DAI field for multicast is included in DCI format 0_1/0_2 to indicate whether or how many HARQ-ACK bits for multicast are multiplexed into the PUSCH. In addition, it was agreed that the UL-DAI field for multicast is applied to all configured G-RNTIs. The remaining issue is how to count the total number of HARQ-ACK bits for all G-RNTIs through the UL-DAI field for multicast for Type-2 codebook. Three alternatives were listed from the agreements for further discussion [1].

For Alt1-1, the UL-DAI indicates the total number of DL-DAIs in the scheduling DCIs. If multiple G-RNTIs are configured and the last DCI missing issue may happen to any one of G-RNTIs, the UE cannot determine which G-RNTI the last DCI missing happens to just according to the total number of DL-DAIs indicated by the UL-DAI, and cannot determine where to pad bits of NACK for the missed DCI among the HARQ sub-codebook of multiple G-RNTIs, as illustrated in Figure 1. Thus, Alt1-1 is not reliable.

Figure 1. Last DCI missing issue when UL-DAI indicates the total number of DL-DAIs

For Alt1-2, it is assumed up to gNB implementation to ensure the number of DL-DAI is the same for each of configured G-RNTIs. However, such restriction is undesired from network perspective.

For Alt1-3, the UL-DAI indicates the largest value of DL-DAI in the scheduling DCIs among multiple G-RNTIs if configured. If the last DCI missing issue happens, UE can pad bits to the HARQ sub-codebook whose payload size is smaller than the largest value of DL DAI, as shown Figure 2. In this way, last DCI missing issue is solved.

Figure 2. Last DCI miss issue solved when UL-DAI indicates the largest value of DL-DAI

Therefore, Alt1-3 is preferable comparing to Alt1-1 and Alt1-2. Moreover, if disabling HARQ-ACK is configured by RRC, the UL-DAI field for both Type-1 and Type-2 codebook could be absent in the UL-DCI.

Proposal 1: For Type-2 codebook configured for multicast, UL-DAI field for multicast indicates the largest value of DL-DAI in the scheduling DCI, and the number of HARQ-ACK bits on PUSCH is the product of the UL-DAI value with the number of G-RNTIs configured with HARQ-ACK feedback enabled.

Proposal 2: The UL-DAI field for multicast is absent in DCI format 0_1/0_2 when disabling HARQ-ACK is configured by RRC signaling for all configured G-RNTIs.

Multiplexing different codebook types

When UE is configured with Type-3, or enhanced Type-3 codebook for unicast, the question is how to support multiplexing the codebook for unicast and the codebook for multicast. These issues were discussed in RAN1#108-e meeting [1] but the latest proposal as follows was not agreed:

Proposal 2.1.5-1 When UE is configured to provide Type-1 or Type-2 HARQ-ACK codebook for multicast and is triggered to provide Type-3 codebook in DCI format 1_1/1_2 in the same PUCCH slot, the UE only generates the Type-3 codebook that consists of HARQ-ACK bits for all HARQ processes including the ones used for multicast. Proposal 2.1.5-2 When UE is scheduled to provide Type-1 or Type-2 HARQ-ACK codebook for multicast of a PHY priority and is triggered to provide enhanced Type-3 codebook of smaller size of the same PHY priority in DCI format 1_1/1_2 in the same PUCCH slot, the UE only generates the enhanced Type-3 codebook and the UE is not expecting HARQ-ACK information in a Type-1 or Type-2 HARQ-ACK CB to be transmitted that cannot be mapped to the enhanced Type 3 HARQ-ACK CB of smaller size as the HARQ process is not part of the codebook.

In Rel-16, construction of Type-3 HARQ codebook is related to all HARQ processes without considering priority.

In Rel-17, enhanced Type-3 is supported by configuring UE with some sub-sets of HARQ processes. Construction of enhanced Type-3 HARQ codebook is related to the indicated subset of HARQ processes by the triggering DCI.

The following agreement was achieved in section of URLLC enhancement in the last meeting:

Agreement The Rel-16 PHY prioritization operation and Rel-16 Type 3 with PHY priority operation & Rel-17 enhanced Type 3 HARQ-ACK CB with PHY priority operation is further clarified as: - For Rel-17 enhanced Type 3 HARQ-ACK CB operation, the restriction on the Type 1 / Type 2 HARQ-ACK CB mapping from the earlier agreement below is only applicable to the same PHY priority of the Type 1 / Type 2 CB as the PUCCH for the enhanced Type 3 CB re-transmission. A LP PUCCH (or PUSCH) carrying the R17 enh. Type 3 CB is dropped according to the Rel-16 PHY prioritization procedures in case of an overlapping HP channel. Agreement For the enhanced Type 3 HARQ-ACK CB of smaller size triggered in a PUCCH slot, the UE is not expecting HARQ-ACK information in a Type 1 or Type 2 HARQ-ACK CB to be transmitted that cannot be mapped to the enhanced Type 3 HARQ-ACK CB of smaller size as the HARQ process is not part of the codebook.. - For Rel-16 Type HARQ-ACK 3 HARQ-ACK CB operation, the UE creates the Rel-16 Type 3 HARQ-ACK CB for transmission on a PUCCH of the indicated priority. A LP PUCCH (or PUSCH) carrying the R16 Type 3 HARQ-ACK CB is dropped according to the Rel-16 PHY prioritization procedures in case of an overlapping HP channel.

One difference between legacy Rel-16 type-3 codebook and Rel-17 enhanced type-3 codebook is that Rel-17 enhanced type-3 codebook is related with the priority. That is, enhanced Type-3 codebook has the same priority as the PUCCH carrying the enhanced Type-3 HARQ-ACK codebook.

Thus, Rel-16 Type-3 HARQ codebook and Rel-17 enhanced Type-3 HARQ codebook can be applied to multicast and we propose:

Proposal 3: When UE is configured to provide Type-1 or Type-2 HARQ-ACK codebook for multicast and is triggered to provide Type-3 codebook in DCI format 1_1/1_2 in the same PUCCH slot, the UE only generates the Type-3 codebook that consists of HARQ-ACK bits for all HARQ processes (i.e. including the HARQ processes used for multicast and unicast). In case a HARQ process is used for broadcast, the UE should report NACK in Type-3 codebook for that HARQ process.

- Note for enhanced Type-3 codebook: When UE is scheduled to provide Type-1 or Type-2 HARQ-ACK codebook for multicast of a PHY priority and is triggered to provide enhanced Type-3 codebook of smaller size of the same PHY priority in DCI format 1_1/1_2 in the same PUCCH slot, the UE only generates the enhanced Type-3 codebook and the UE is not expecting HARQ-ACK information in a Type-1 or Type-2 HARQ-ACK CB to be transmitted that cannot be mapped to the enhanced Type 3 HARQ-ACK CB of smaller size as the HARQ process is not part of the codebook.

PUCCH determination when multiplexing multicast with unicast

As shown in following agreement, it was agreed that separate PUCCH-config can be configured for multicast. It was agreed that PRI of the last DCI for unicast is used to determine PUCCH resource when multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast.

Agreement: For a separate PUCCH-ConfigurationList for multicast that is optionally configured, at least for ACK/NACK based HARQ-ACK feedback, - The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority codebook and high priority codebook, respectively. - FFS other configurations Agreement: For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to the following Alt1 from the previous agreement: - Alt.1: The last DCI for unicast - FFS: Any details when last DCI is missed by the UE if it is necessary to make them different from current specifications for this case.

From the current TS38.213 v17.1.0, as shown in following highlighted green part, it is clear that PRI of last unicast DCI format is used to determine PUCCH resource when multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast. However, it is not clear whether the mapped PUCCH resource by PRI, as shown in following highlighted yellow part, is provided by PUCCH-config for unicast or PUCCH-config-Multicast, especially considering the names of the parameters included in PUCCH-config-Multicast are exactly the same as those included in PUCCH-config for unicast.

… ### 9.2.3 UE procedure for reporting HARQ-ACK For a PUCCH transmission with HARQ-ACK information, a UE determines a PUCCH resource on the cell of the PUCCH transmission, as described in clause 9.A, after determining a set of PUCCH resources for OUCI HARQ-ACK information bits, as described in clause 9.2.1. The PUCCH resource determination is based on a PUCCH resource indicator field [5, TS 38.212], if present, in a last DCI format, among the DCI formats that have a value of a PDSCH-to-HARQ_feedback timing indicator field, if present, or a value of dl-DataToUL-ACK, or dl-DataToUL-ACK-r16, or dl-DataToUL-ACKForDCIFormat1_2, or dl-DataToUL-ACK-r17, or dl-DataToUL-ACK-MulticastDciFormat4_1, indicating a same slot for the PUCCH transmission, that the UE detects and for which the UE transmits corresponding HARQ-ACK information in the PUCCH…. The PUCCH resource indicator field values map to values of a set of PUCCH resource indexes, as defined in Table 9.2.3-2 for a PUCCH resource indicator field of 3 bits, provided by resourceList for PUCCH resources from a set of PUCCH resources provided by PUCCH-ResourceSet with a maximum of eight PUCCH resources. If the PUCCH resource indicator field includes 1 bit or 2 bits, the values map to the first two values or the first four values, respectively, of Table 9.2.3-2. If the last DCI format does not include a PUCCH resource indicator field, the first value of Table 9.2.3-2 is used. … - 18 Multicast Broadcast Services … A UE determines a PUCCH resource for a PUCCH transmission with HARQ-ACK information as described in clauses 9.2 and 9.2.1 through 9.2.5. If the UE multiplexes HARQ-ACK information associated with unicast DCI formats and HARQ-ACK information associated with multicast DCI formats in a same PUCCH, the last DCI format that the UE uses to determine the PUCCH resource, as described in clause 9.2.3, is a last unicast DCI format. …

In light of this, it is worthwhile to make it clear that the PUCCH resource indexes/resourceList/PUCCH-ResourceSet that mapped by PRI is provided by PUCCH-config for unicast.

Considering the above issue, a text proposal is suggested for clarification:

Proposal 4: Adopt the following text proposal to TS38.213 v17.1.0:

- Reason for change:

  - The current TS38.213 v17.1.0 does not clarify the determined PUCCH resource for multiplexed HARQ-ACK of unicast and multicast is provided by PUCCH-config for unicast or PUCCH-config-Multicast.

- Summary of change:

  - Clarify the determined PUCCH resource for multiplexed HARQ-ACK of unicast and multicast is provided by PUCCH-config for unicast.

- Consequence if not approved:

  - It is not clear that the determined PUCCH resource is provided by PUCCH-config for unicast or PUCCH-config-Multicast, especially the names of the parameters included in PUCCH-config-Multicast are exactly the same as those included in PUCCH-config for unicast.

--------------------------------Text proposal to TS38.213 v17.1.0 Starts------------------------------------------ 18 Multicast Broadcast Services < Unchanged parts are omitted > A UE determines a PUCCH resource for a PUCCH transmission with HARQ-ACK information as described in clauses 9.2 and 9.2.1 through 9.2.5. If the UE multiplexes HARQ-ACK information associated with unicast DCI formats and HARQ-ACK information associated with multicast DCI formats in a same PUCCH, the last DCI format that the UE uses to determine the PUCCH resource, as described in clause 9.2.3, is a last unicast DCI format. Values of a set of PUCCH resource indexes that the PUCCH resource indicator field are mapped to, as defined in Table 9.2.3-2 for a PUCCH resource indicator field of 3 bits, are provided by resourceList in PUCCH-ResourceSet configured in PUCCH-config for unicast. < Unchanged parts are omitted > --------------------------------Text proposal to TS38.213 v17.1.0 Ends-------------------------------------------

NACK-only based HARQ-ACK feedback

The discussion in this section aims to solve the open issues for NACK-only feedback.

More than one NACK-only multiplexing

How to support more than one NACK-only transmissions in the same PUCCH slot has been discussed for several meetings with Alt1 and Alt4, and the following agreement was made during RAN1#108e.

Agreement For supporting more than one NACK-only feedback in the same PUCCH transmission, define RRC configuration to configure between Alt1 and Alt4 (from previous agreements): - Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits. - FFS: how to determine PUCCH resource - Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission. - define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback, - FFS: The PUCCH slot for the transmission is based on the K1 in the “last DCI” scheduling multicast. - FFS: The PUCCH resource for the transmission is from PUCCH-config configured for NACK-only based feedback according to the mapping between number of TBs with PUCCH resource ID. - FFS mapping details. - How to determine the number of TBs is discussed separately, e.g., Type-1-like and/or Type-2-like codebook. - FFS: whether this applies to a single G-RNTI or multiple G-RNTIs - Alt4 is not supported for more than 4 TBs - FFS: whether RRC configuration between Alt1 and Alt4 is per G-RNTI or per CFR - FFS: UE capability Agreement Regarding RRC configuring Alt1 or Alt4 (from the previous agreement) for multiplexing more than one NACK-only in the same PUCCH transmission, the configuration is per CFR.

The remaining issues regarding the above agreements are mainly about the PUCCH resource determination and whether it can be applied to the case of different G-RNTIs.

For Alt 1, how to determine PUCCH resource?

When Alt 1 is configured, UE generates a HARQ-ACK codebook by transforming the NACK-only into ACK/NACK bits and the PUCCH resource to be used for the transmission are based on the k1 and PRI included in the last DCI scheduling multicast, which is similar to the case of ACK/NACK based feedback for multicast but without separate PUCCH resources configured for multicast.

Proposal 5: For Alt 1 if more than one NACK-only feedback are available for transmission in the same PUCCH slot, UE determines the PUCCH resource for transmission from the PUCCH resources configured for unicast based on the k1 and PRI included in the last DCI scheduling multicast.

For Alt 4, how to map number of TBs with PUCCH resource ID?

Regarding the orthogonal PUCCH resources needed for combinations of TBs for NACK-only, UE does not report HARQ-ACK if all TBs are decoded correctly and the combinations of 4TBs corresponding to the PUCCH resources can be illustrated in Table 1.

Table 1 Combinations of 4TBs corresponding to the PUCCH resources

| TB combination PUCCH resource IDs | TB#1 | TB#1-TB#2 | TB#1-TB#2-TB#3 | TB#1-TB#2-TB#3-TB#4 |
| --- | --- | --- | --- | --- |
| PUCCH resource ID#0 | N | N-N/DTX | N-N-N | N-N-N-N |
| PUCCH resource ID#1 |  | A-N/DTX | A-N-N | A-N-N-N |
| PUCCH resource ID#2 |  | N-A | N-A-N | N-A-N-N |
| PUCCH resource ID#3 |  |  | A-A-N | A-A-N-N |
| PUCCH resource ID#4 |  |  | N-N-A | N-N-A-N |
| PUCCH resource ID#5 |  |  | A-N-A | A-N-A-N |
| PUCCH resource ID#6 |  |  | N-A-A | N-A-A-N |
| PUCCH resource ID#7 |  |  |  | A-A-A-N |
| PUCCH resource ID#8 |  |  |  | N-N-N-A |
| PUCCH resource ID#9 |  |  |  | A-N-N-A |
| PUCCH resource ID#10 |  |  |  | N-A-N-A |
| PUCCH resource ID#11 |  |  |  | A-A-N-A |
| PUCCH resource ID#12 |  |  |  | N-N-A-A |
| PUCCH resource ID#13 |  |  |  | A-N-A-A |
| PUCCH resource ID#14 |  |  |  | N-A-A-A |

The UE firstly sorts TBs to a TB combination based on the k1 and the C-DAI included in the DCI scheduling multicast as discussed in section 3.2. If a TB in the middle is missed, the status of the missed TB in the TB combination is considered as NACK, and then UE maps to the corresponding PUCCH resource ID according to the sorted TB status or sorted HARQ-ACK bits.

If unfortunately the last TB or even the last few TBs are missed, UE will choose the wrong TB combination with a smaller size. In this case, gNB can still retransmit the last TB or the last few TBs to the UE by the highlighted mapping rule between PUCCH resources and TB combinations in Table 1. For example, if gNB schedule TB#1, TB#2 and TB#3 with HARQ-ACK reported within the same PUCCH slot, UE1 received TB#1 correctly, TB#2 incorrectly and missed TB#3. From UE perspective, status of TB combinations will be N-N(TB#1-TB#2) and PUCCH resource ID#1 is chosen. From gNB perspective, once PUCCH resource ID#1 is detected, gNB acknowledges the status of TB combination reported from UE1 is A-N-N(TB#1-TB#2-TB#3). Therefore, the missed TB#3 will be retransmitted so will not be lost.

Proposal 6: For Alt 4 if more than one NACK-only feedback are available for transmission in the same PUCCH slot, support the following mapping between PUCCH resource IDs and decoding results of TBs combinations, where the TBs are ordered according to the sequence in which they are received:

| TB combination PUCCH resource IDs | TB#1 | TB#1-TB#2 | TB#1-TB#2-TB#3 | TB#1-TB#2-TB#3-TB#4 |
| --- | --- | --- | --- | --- |
| PUCCH resource ID#0 | N | N-N/DTX | N-N-N | N-N-N-N |
| PUCCH resource ID#1 |  | A-N/DTX | A-N-N | A-N-N-N |
| PUCCH resource ID#2 |  | N-A | N-A-N | N-A-N-N |
| PUCCH resource ID#3 |  |  | A-A-N | A-A-N-N |
| PUCCH resource ID#4 |  |  | N-N-A | N-N-A-N |
| PUCCH resource ID#5 |  |  | A-N-A | A-N-A-N |
| PUCCH resource ID#6 |  |  | N-A-A | N-A-A-N |
| PUCCH resource ID#7 |  |  |  | A-A-A-N |
| PUCCH resource ID#8 |  |  |  | N-N-N-A |
| PUCCH resource ID#9 |  |  |  | A-N-N-A |
| PUCCH resource ID#10 |  |  |  | N-A-N-A |
| PUCCH resource ID#11 |  |  |  | A-A-N-A |
| PUCCH resource ID#12 |  |  |  | N-N-A-A |
| PUCCH resource ID#13 |  |  |  | A-N-A-A |
| PUCCH resource ID#14 |  |  |  | N-A-A-A |

For Alt 4, whether it can be applied to multiple G-RNTIs case?

Some companies were concerned about the case of multiple G-RNTIs.

When the PUCCH resources for NACK-only are not shared with all UEs, gNB can easily differentiate which TB of which G-RNTI is not correctly decoded by which UE.

When the PUCCH resources for NACK-only are shared with all UEs and from UE perspective the TBs in the above table may come from different G-RNTIs, gNB can still differentiate which TB from which G-RNTI is failed in decoding because the mapping rule is aligned between UE and gNB, including for the case that different UEs may be configured with different G-RNTIs according to the proposed mapping rule.

For example, as shown in Figure 3, UE#1 is configured with G-RNTI1 and receives G-RNTI1/TB1/TB2 and UE#2 is configured with G-RNTI1 and G-RNTI2 and receives G-RNTI1/TB1/TB2 and G-RNTI2/TB3. gNB schedules the HARQ-ACK feedback of TB1, TB2 and TB3 in a same PUCCH slot and the PUCCH resource for NACK-only is shared. Assuming that TB1/TB2 are ACKed/NACKed by UE1 and TB1/TB2/TB3 are ACKed/NACKed/NACKed by UE2, UE1 will choose PUCCH resource ID#1 to represent ACK/NACK for TB1/TB2 and UE2 will choose PUCCH resource ID#1 to represent ACK/NACK/NACK for TB1/TB2/TB3, respectively. gNB will retransmit TB2 and TB3 according to the detected PUCCH resource ID#1 and UEs can obtain the concerned retransmission of the TBs, i.e., TB2 for UE1 and TB2/TB3 for UE2.

Figure 3: more than four TBs with NACK-only are transformed into ACK/NACK

Proposal 7: When more than one NACK-only feedbacks are available for transmission in the same PUCCH slot, Alt 4 can be applied to both single/multiple G-RNTIs case.

Codebook for NACK-only

For supporting more than one TB with NACK-only transmission in the same PUCCH slot, how to determine the number of TBs, there could be two options:

- Option1: based on the k1 set and TDRA table configured by higher layer signalling.

- Option2: based on C-DAI included in the scheduling DCI.

From the discussions in the last meeting [1], option2 based on C-DAI included in the scheduling DL DCI seems agreeable and whether the option 1 based on the k1 set and TDRA table configured by higher layer signaling is supported was controversial.

The proposal was updated as follows but no consensus was reached in the last meeting:

Proposal 3.4.2 For determining the HARQ-ACK feedback for NACK-only to be transmitted in the same PUCCH (for either Alt1 or Alt4), - based on C-DAI included in the scheduling DL DCI, the total number of HARQ-ACK bits for all G-RNTIs configured with NACK-only is the sum of C-DAI in the last DCI from each G-RNTI. - Note1: It is similar to Type-2 codebook size determination for ACK/NACK based feedback. - Note2: C-DAI is independently counted per G-RNTI configured with NACK-only. - Note3: Whether Alt4 is applied to a single G-RNTI or all configured G-RNTI can still be discussed further. - FFS: based on the k1 set and TDRA table configured by higher layer signaling, - Note4: It is similar to Type-1 codebook size determination for ACK/NACK based feedback. - The number of HARQ-ACK bits consists of all G-RNTIs configured with NACK-only based feedback.

NACK-only feedback is expected to be used for saving PUCCH resource overhead, and for the same reason, determining the number of TBs for NACK-only is expected to be based on the C-DAI as well. As analyzed in section 3.1, Alt 4 can be applied for both single/multiple G-RNTIs cases, so that determining the total number of TBs with NACK-only from multiple G-RNTIs in the same PUCCH slot can adopt the same mechanism of determining the number of HARQ-ACK bits for multiplexing on PUSCH as discussed in section 2.1. Moreover, given the mapping rule between combinations of TBs and PUCCH resources ID as proposed is robust to DCI missing for a given UE, it seems sufficient to determine the number of TBs with NACK-only based on C-DAI.

For Alt1 NACK-only being transformed to ACK/NACK based feedback which may also include the HARQ-ACK for multiple G-RNTIs, as agreed the codebook construction or concatenation is the same as when the feedback mode is ‘ACK/NACK’ when multiplexing NACK-only feedback with ACK/NACK for others (e.g., for unicast or for ACK/NACK based for some other G-RNTIs for which either Type-1 or Type-2 HARQ-ACK codebook could be configured), based on C-DAI (and adopt the same mechanism of determining the number of HARQ-ACK bits for multiplexing on PUSCH as discussed in section 2.1 for the case of multiple G-RNTIs scheduled) works as well in such case, i.e., the sub-codebook generated by Alt1 is concatenated to the other sub-codebook generated for unicast HARQ-ACK or ACK/NACK based HARQ-ACK for multicast, for which either Type-1 or Type-2 codebook is configured.

Therefore, we update the proposal as following:

Proposal 8: For determining the HARQ-ACK feedback for NACK-only to be transmitted in the same PUCCH (for either Alt1 or Alt4), support the following:

- based on C-DAI included in the scheduling DL DCI, the total number of HARQ-ACK bits for all G-RNTIs configured with NACK-only is the product of the largest value of DL-DAI in the scheduling DCI with the number of G-RNTIs configured.

  - Note1: It is similar to Type-2 codebook size determination for ACK/NACK based feedback.

  - Note2: C-DAI is independently counted per G-RNTI configured with NACK-only.

multiplex NACK-only with another PUCCH/PUSCH

During #108-e meeting, the following agreement is achieved:

Agreement For multiplexing NACK-only with HARQ-ACK feedback/CSI for unicast for the same priority or PUSCH transmission for the same priority or ACK/NACK based HARQ-ACK feedback for multicast with another G-RNTI for the same priority, the multiplexing does not depend on the decoding result. - Note: when multiplexing the NACK-only is transformed into ACK/NACK as agreed.

Based on the agreement, the multiplexing of NACK-only PUCCH with HARQ-ACK feedback/CSI for unicast or PUSCH does not depend on the decoding result to avoid network blind decoding, which means as long as the PUCCH resource for NACK-only transmission overlaps with other transmissions, UE will do multiplexing. However, for supporting more than one TB with NACK-only feedback, when configured with Alt4, UE chooses one PUCCH resource from a pre-configured PUCCH resource set based on the decoding result of corresponding PDSCH(s). When the multiplexing between NACK-only feedback and another PUCCH/PUSCH does not depend on the decoding result, the question is how to determine the resource overlapping.

To solve this issue, the nominal NACK-only PUCCH can be used for the determination based on the pre-configured NACK-only PUCCH resource set. Some options can be considered as listed as follows:

- Option 1: The nominal NACK-only PUCCH consists of all the symbols from PUCCHs of the NACK-only PUCCH resource set, as the yellow block in Figure 4 (the NACK-only PUCCH resource set includes PUCCH1~PUCCH3).

- Option 2: The nominal NACK-only PUCCH starts from the earliest starting symbol of PUCCHs in the NACK-only PUCCH resource set, and ends at the latest ending symbol of PUCCHs in the set, as the green block in Figure 4.

- Option 3: The nominal NACK-only PUCCH consists of all the symbols of the PUCCH slot, as the blue block in Figure 4.

Figure 4: nominal NACK-only PUCCH for multiplexing with other UL channels

For all of the options, the nominal NACK-only PUCCH is determined after the configuration of the NACK-only PUCCH resource set. When the nominal NACK-only PUCCH overlaps with other UL channels, UE multiplexes NACK-only feedback with the overlapping UL channels. Thus, the multiplexing does not depend on the decoding result but depends on the NACK-only PUCCH resource set. Among all three options, option 1 is preferred as it only consists of the symbols belonging to the PUCCHs of the NACK-only PUCCH resource set. Option 2 and option 3 may consist of  symbols not belonging to the PUCCHs in the set, and multiplexing may happen frequently especially for option 3.

Another way is to restrict the configuration of the NACK-only PUCCH resource set. All the NACK-only PUCCH candidates in the resource set starts from the same symbol and also ends at the same symbol. In another word, all the NACK-only resources within the set have the same starting symbol and the same ending symbol. Then no matter which NACK-only PUCCH resource is chosen by UE to send the HARQ-ACKs, there is no misalignment between UE and gNB about when the multiplexing happens. This can simplify UE’s behavior but introduce scheduling restrictions for gNB. This is also the special case for option 1 and option 2 by gNB’s implementation.

Observation: When UE multiplexes PUCCH transmission for Atl4 with other UCI/PUSCH for the same priorities, how to determine the resource overlapping needs to be defined.

Proposal 9: Define the nominal NACK-only PUCCH as a PUCCH consisting of all the symbols from PUCCHs of the NACK-only PUCCH resource set and the nominal NACK-only PUCCH is used for determining whether PUCCH for NACK-only overlaps with other UCI/PUSCH for multiplexing for the same priority.

PDSCH processing time for PDSCH with NACK-only feedback mode

Value for PDSCH processing time

In R15/R16, the UE processing time Tproc1[1] is defined as the minimum required gap between the end of the PDSCH reception and the earliest possible start of the PUCCH carrying the corresponding HARQ-ACK. UE is required to finish PDSCH reception and PUCCH preparation during the Tproc1 time.

If the first uplink symbol of the PUCCH which carries the HARQ-ACK information, as defined by the assigned HARQ-ACK timing K1 and Koffset, if configured, and the PUCCH resource to be used and including the effect of the timing advance, starts no earlier than at symbol L1, where L1 is defined as the next uplink symbol with its CP starting after after the end of the last symbol of the PDSCH carrying the TB being acknowledged, then the UE shall provide a valid HARQ-ACK message. - N1 is based on µ of table 5.3-1 and table 5.3-2 for UE processing capability 1 and 2 respectively, where µ corresponds to the one of (µPDCCH, µPDSCH, µUL) resulting with the largest Tproc,1, where the µPDCCH corresponds to the subcarrier spacing of the PDCCH scheduling the PDSCH, the µPDSCH corresponds to the subcarrier spacing of the scheduled PDSCH, and µUL corresponds to the subcarrier spacing of the uplink channel with which the HARQ-ACK is assumed to be transmitted regardless of whether or not the PDSCH reception provides a transport block for a HARQ process with disabled HARQ-ACK information as indicated by HARQ-feedbackEnabling-disablingperHARQprocess, if provided, and κ is defined in clause 4.1 of [4, TS 38.211].

For a R15/R16 UE, PUCCH resource is determined by PDSCH slot, K1 value and PRI. All these information can be determined based on the DCI scheduling the corresponding PDSCH, which means UE can  do the preparation of PUCCH resource right after the decoding of DCI.

However, for a UE configured with Alt4 for more than one TB with NACK-only reported in the same slot, the PUCCH resource depends on the decoding of PDSCHs. Thus, UE can only start the preparation of PUCCH resource after the decoding of PDSCHs. Compared to the UE in ACK/NACK reporting mode, UE in NACK-only mode requires more time to do the PDSCH reception and PUCCH preparation. Therefore, Tproc1 as specified needs to be extended to include the case for NACK-only based feedback, e.g., by adding d3 in the formula.

Figure 5: Extending Tproc1 for NACK-only based feedback

Two options are listed for the value of d3:

- Option 1: The value of d3 equals to the time gap between PDCCH ending symbol and PDSCH ending symbol plus N1.

- Option 2: The value of d3 depends on the UE capability and can be reported by UE.

With option 1, given the PUCCH determination is postponed as shown in Figure 5, the value of d3 can be set to the postponed time. According to Figure 5, the PUCCH determination is postponed as

SPDSCH - SPDCCH + TPDSCH_decod - TPDCCH_decod

where SPDSCH is the ending symbol of PDSCH, SPDCCH is the ending symbol of PDCCH, TPDSCH_decod is the decoding time for PDSCH, and TPDCCH_decod is the decoding time for PDCCH. As the decoding time for PDCCH is always shorter than the one for PDSCH, TPDSCH_decod - TPDCCH_decod can be set to N1 for simplicity. Thus the value of d3 equals to

SPDSCH - SPDCCH + N1

where N1 is the same as the N1 in Tproc1.

With option 2, UE can choose one value from the pre-configured candidate value set and report it as d3. But the candidate values needs more discussions.

We prefer to use option 1 as this can guarantee enough PDSCH processing time for UE in NACK-only mode and requires no more discussions about the candidate values as in option 2.

Proposal 10：Extend Tproc1 by adding d3 in NACK-only reporting mode and set the value of d3 as,

SPDSCH - SPDCCH + N1

where SPDSCH is the ending symbol of PDSCH, SPDCCH is the ending symbol of PDCCH, and N1 is the same as the N1 in Tproc1.

Reference point for PDSCH processing time

In R15/R16, the UE processing time Tproc1 is defined as the minimum required gap between the end of the PDSCH reception and the earliest possible start of the PUCCH carrying the corresponding HARQ-ACK. But for a UE configured in NACK-only mode, the PUCCH resource depends on the decoding of PDSCH, which means gNB has no knowledge about which PUCCH is used by UE to report NACK-only HARQ-ACK feedback and blind decoding through the whole PUCCH resource set is required. Thus it is hard for gNB to guarantee the time gap between the end of the PDSCH reception and the earliest possible start of the chosen PUCCH is always larger than Tproc1.

To solve this issue, one possible solution is use the starting symbol of the NACK-only PUCCH resource set as the reference point for NACK-only feedback, where the starting symbol of the PUCCH resource set means the earliest starting symbols of all the PUCCH resources in the set. By using this new reference point, gNB can always guarantee the PDSCH processing timeline is satisfied.

Proposal 11: Define the reference point as the starting symbol of PUCCH resource set instead of the chosen PUCCH for NACK-only feedback, and use the new reference point for PDSCH processing time, where the starting symbol of the PUCCH resource set means the earliest starting symbols of all the PUCCH resources in the set.

Besides PDSCH processing time, another timeline related to PUCCH resource is N3[2] used in HARQ-ACK codebook construction. If a UE determines a first PUCCH resource for HARQ-ACK in a slot, and later receives a second DCI indicating another HARQ-ACK also in the slot, the time gap between the second DCI and the first PUCCH resource is required to be larger than N3. The details about N3 can be found in the follow spec.

If a UE determines a first resource for a PUCCH transmission with HARQ-ACK information corresponding only to a PDSCH reception without a corresponding PDCCH or detects a first DCI format indicating a first resource for a PUCCH transmission with corresponding HARQ-ACK information in a slot and also detects at a later time a second DCI format indicating a second resource for a PUCCH transmission with corresponding HARQ-ACK information in the slot, the UE does not expect to multiplex HARQ-ACK information corresponding to the second DCI format in a PUCCH resource in the slot if the PDCCH reception that includes the second DCI format is not earlier than from the beginning of a first symbol of the first resource for PUCCH transmission in the slot where, κ and Tc are defined in clause 4.1 of [4, TS 38.211] and corresponds to the smallest SCS configuration among the SCS configurations of the PDCCHs providing the DCI formats and the SCS configuration of the PUCCH. If processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell with the second DCI format and for all serving cells with corresponding HARQ-ACK information multiplexed in the PUCCH transmission in the slot, for , for , for ; otherwise , for , for , for , for .

Similar to the PDSCH processing time, the starting symbol of NACK-only PUCCH resource set can be used as the new reference point to N3.

- Proposal 12: Define the reference point as the starting symbol of PUCCH resource set instead of the chosen PUCCH for NACK-only feedback, and use the new reference point for N3, where the starting symbol of the PUCCH resource set means the earliest starting symbols of all the PUCCH resources in the set.

Codebook for the disabled HARQ-ACK

Whether UE should report HARQ-ACK when the HARQ-ACK is disabled and whether it makes differences for Type-1 or Type-2 codebook were discussed in the last meeting without progress due to controversy.

Type-1 CB for RRC disabled HARQ-ACK

The following agreements were achieved in the last meeting.

Agreement For Type-1 codebook generation, - if all configured G-RNTIs are with HARQ-ACK disabled by RRC signalling, UE does not report any HARQ-ACK information in the PUCCH slot; - For other cases, FFS between the 3 alternatives below: - Alt1: if at least one configured G-RNTI is with HARQ-ACK enabled, UE will report NACK for the G-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH. - Alt2: if at least one configured G-RNTI is with HARQ-ACK enabled, UE reports actual HARQ-ACK result for all G-RNTIs. - Alt3: UE is not expected to be configured with some G-RNTI with HARQ-ACK disabled by RRC signalling and some other G-RNTI with HARQ-ACK enabled by RRC signalling for all configured G-RNTI - Other alternatives are not precluded Agreement If a UE supports ACK/NACK based feedback for dynamic multicast scheduling and for multicast SPS, and if the UE is configured with ACK/NACK based feedback for multicast dynamic scheduling and is configured with disabled HARQ feedback for multicast SPS scheduling, for Type-1 codebook generation, - FFS between the 4 alternatives below: - Alt1: UE will report NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH. - Alt2: UE will report ACK/NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH. - Alt3: UE is not expected to be configured with G-RNTI with HARQ-ACK enabled by RRC signaling and G-CS-RNTI with HARQ-ACK enabled by RRC signalling. - Alt4: UE does not report any HARQ-ACK information for G-CS-RNTI with HARQ-ACK feedback disabled by RRC. - Other alternatives are not precluded.

The remaining issue is how to generate Type-1 codebook when at least one G-RNTI is with HARQ-ACK enabled that is to be multiplexed with HARQ-ACK for the G-RNTI with HARQ-ACK disabled by RRC signalling. In this case, UE needs to report something instead of reporting nothing because otherwise network need to do blind decoding due to different hypotheses as shown in Figure 4. In Figure 4, it is assumed (as proposed by some companies) that UE doesn’t report a bit for a successfully received DCI corresponding to a G-RNTI configured with disabled HARQ. However, if the DCI is missed the UE needs to report NACK as per the legacy procedure. This means the codebook size is variable and this won’t be known to the gNB in case of a missed DCI, therefore the codebook size network expected will not be aligned with the codebook size UE generated due to DCI missing.

Figure 6: Type-1 CB size misalignment if UE reports nothing for the disabled HARQ-ACK

Instead, the Type-1 codebook size should be fixed to solve the DCI missing issue and in this case UE should report something for the HARQ-ACK disabled for a successfully received DCI. Whether UE reports NACK or UE reports ACK or NACK based on the decoding result for the disabled HARQ-ACK was also debated in NR NTN topic. The potential benefit for only reporting NACK is improving network decoding UCI accuracy because it is fixed NACK known to network.

Finally, NR NTN topic agreed UE reports NACK for the feedback-disabled HARQ process regardless of decoding results of corresponding PDSCH, the same approach can be taken for NR MBS for this case.

Agreement For Type-1 HARQ codebook, the UE will consistently report NACK-only for the feedback-disabled HARQ process regardless of decoding results of corresponding PDSCH.

Proposal 13: For Type-1 codebook generation of dynamic multicast scheduling with multiple G-RNTIs,

- if at least one configured G-RNTI is with HARQ-ACK enabled, UE will report NACK for the G-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

Proposal 14: For Type-1 codebook generation of dynamic multicast scheduling configured with ACK/NACK based feedback and multicast SPS configured with disabled HARQ feedback,

- UE will report NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

Type-1 CB and DCI disabling HARQ-ACK

The following proposal was discussed in the last meeting but it was not agreed.

Proposal UE is not expected to be configured with Type-1 HARQ-ACK codebook and with presence of enabled/disabled HARQ-ACK feedback indicator in multicast DCI simultaneously. - Note: this restriction is per UE.

Type-1 codebook size in general should not be affected by dynamic scheduling. Regardless of whether the group-common DCI indicates enabling or disabling HARQ-ACK, the Type-1 codebook size should be kept the same as if the HARQ-ACK is always enabled. Hence, UE is not expected to be configured with Type-1 codebook and to be configured with HARQ-ACK disabled by group-common DCI simultaneously.

Proposal 15: UE is not expected to be configured with Type-1 HARQ-ACK codebook and with presence of enabled/disabled HARQ-ACK feedback indicator in multicast DCI simultaneously.

- Note: this restriction is per UE.

Conclusions

This contribution discusses the remaining issues UL-DAI, multiplexing different codebook types, and NACK-only based feedback. This discussion leads to the following proposals:

Proposal 1: For Type-2 codebook configured for multicast, UL-DAI field for multicast indicates the largest value of DL-DAI in the scheduling DCI, and the number of HARQ-ACK bits on PUSCH is the product of the UL-DAI value with the number of G-RNTIs configured with HARQ-ACK feedback enabled.

Proposal 2: The UL-DAI field for multicast is absent in DCI format 0_1/0_2 when disabling HARQ-ACK is configured by RRC signaling for all configured G-RNTIs.

Proposal 3: When UE is configured to provide Type-1 or Type-2 HARQ-ACK codebook for multicast and is triggered to provide Type-3 codebook in DCI format 1_1/1_2 in the same PUCCH slot, the UE only generates the Type-3 codebook that consists of HARQ-ACK bits for all HARQ processes (i.e. including the HARQ processes used for multicast and unicast). In case a HARQ process is used for broadcast, the UE should report NACK in Type-3 codebook for that HARQ process.

- Note for enhanced Type-3 codebook: When UE is scheduled to provide Type-1 or Type-2 HARQ-ACK codebook for multicast of a PHY priority and is triggered to provide enhanced Type-3 codebook of smaller size of the same PHY priority in DCI format 1_1/1_2 in the same PUCCH slot, the UE only generates the enhanced Type-3 codebook and the UE is not expecting HARQ-ACK information in a Type-1 or Type-2 HARQ-ACK CB to be transmitted that cannot be mapped to the enhanced Type 3 HARQ-ACK CB of smaller size as the HARQ process is not part of the codebook.

Proposal 4: Adopt the following text proposal to TS38.213 v17.1.0:

- Reason for change:

  - The current TS38.213 v17.1.0 does not clarify the determined PUCCH resource for multiplexed HARQ-ACK of unicast and multicast is provided by PUCCH-config for unicast or PUCCH-config-Multicast.

- Summary of change:

  - Clarify the determined PUCCH resource for multiplexed HARQ-ACK of unicast and multicast is provided by PUCCH-config for unicast.

- Consequence if not approved:

  - It is not clear that the determined PUCCH resource is provided by PUCCH-config for unicast or PUCCH-config-Multicast, especially the names of the parameters included in PUCCH-config-Multicast are exactly the same as those included in PUCCH-config for unicast.

--------------------------------Text proposal to TS38.213 v17.1.0 Starts------------------------------------------ 18 Multicast Broadcast Services < Unchanged parts are omitted > A UE determines a PUCCH resource for a PUCCH transmission with HARQ-ACK information as described in clauses 9.2 and 9.2.1 through 9.2.5. If the UE multiplexes HARQ-ACK information associated with unicast DCI formats and HARQ-ACK information associated with multicast DCI formats in a same PUCCH, the last DCI format that the UE uses to determine the PUCCH resource, as described in clause 9.2.3, is a last unicast DCI format. Values of a set of PUCCH resource indexes that the PUCCH resource indicator field are mapped to, as defined in Table 9.2.3-2 for a PUCCH resource indicator field of 3 bits, are provided by resourceList in PUCCH-ResourceSet configured in PUCCH-config for unicast. < Unchanged parts are omitted > --------------------------------Text proposal to TS38.213 v17.1.0 Ends-------------------------------------------

Proposal 5: For Alt 1 if more than one NACK-only feedback are available for transmission in the same PUCCH slot, UE determines the PUCCH resource for transmission from the PUCCH resources configured for unicast based on the k1 and PRI included in the last DCI scheduling multicast.

Proposal 6: For Alt 4 if more than one NACK-only feedback are available for transmission in the same PUCCH slot, support the following mapping between PUCCH resource IDs and decoding results of TBs combinations, where the TBs are ordered according to the sequence in which they are received:

| TB combination PUCCH resource IDs | TB#1 | TB#1-TB#2 | TB#1-TB#2-TB#3 | TB#1-TB#2-TB#3-TB#4 |
| --- | --- | --- | --- | --- |
| PUCCH resource ID#0 | N | N-N/DTX | N-N-N | N-N-N-N |
| PUCCH resource ID#1 |  | A-N/DTX | A-N-N | A-N-N-N |
| PUCCH resource ID#2 |  | N-A | N-A-N | N-A-N-N |
| PUCCH resource ID#3 |  |  | A-A-N | A-A-N-N |
| PUCCH resource ID#4 |  |  | N-N-A | N-N-A-N |
| PUCCH resource ID#5 |  |  | A-N-A | A-N-A-N |
| PUCCH resource ID#6 |  |  | N-A-A | N-A-A-N |
| PUCCH resource ID#7 |  |  |  | A-A-A-N |
| PUCCH resource ID#8 |  |  |  | N-N-N-A |
| PUCCH resource ID#9 |  |  |  | A-N-N-A |
| PUCCH resource ID#10 |  |  |  | N-A-N-A |
| PUCCH resource ID#11 |  |  |  | A-A-N-A |
| PUCCH resource ID#12 |  |  |  | N-N-A-A |
| PUCCH resource ID#13 |  |  |  | A-N-A-A |
| PUCCH resource ID#14 |  |  |  | N-A-A-A |

Proposal 7: When more than one NACK-only feedbacks are available for transmission in the same PUCCH slot, Alt 4 can be applied to both single/multiple G-RNTIs case.

Proposal 8: For determining the HARQ-ACK feedback for NACK-only to be transmitted in the same PUCCH (for either Alt1 or Alt4), support the following:

- based on C-DAI included in the scheduling DL DCI, the total number of HARQ-ACK bits for all G-RNTIs configured with NACK-only is the product of the largest value of DL-DAI in the scheduling DCI with the number of G-RNTIs configured.

  - Note1: It is similar to Type-2 codebook size determination for ACK/NACK based feedback.

  - Note2: C-DAI is independently counted per G-RNTI configured with NACK-only.

Observation: When UE multiplexes PUCCH transmission for Atl4 with other UCI/PUSCH for the same priorities, how to determine the resource overlapping needs to be defined.

Proposal 9: Define the nominal NACK-only PUCCH as a PUCCH consisting of all the symbols from PUCCHs of the NACK-only PUCCH resource set and the nominal NACK-only PUCCH is used for determining whether PUCCH for NACK-only overlaps with other UCI/PUSCH for multiplexing for the same priority.

Proposal 10：Extend Tproc1 by adding d3 in NACK-only reporting mode and set the value of d3 as,

SPDSCH - SPDCCH + N1

where SPDSCH is the ending symbol of PDSCH, SPDCCH is the ending symbol of PDCCH, and N1 is the same as the N1 in Tproc1.

Proposal 11: Define the reference point as the starting symbol of PUCCH resource set instead of the chosen PUCCH for NACK-only feedback, and use the new reference point for PDSCH processing time, where the starting symbol of the PUCCH resource set means the earliest starting symbols of all the PUCCH resources in the set.

Proposal 12: Define the reference point as the starting symbol of PUCCH resource set instead of the chosen PUCCH for NACK-only feedback, and use the new reference point for N3, where the starting symbol of the PUCCH resource set means the earliest starting symbols of all the PUCCH resources in the set.

Proposal 13: For Type-1 codebook generation of dynamic multicast scheduling with multiple G-RNTIs,

- if at least one configured G-RNTI is with HARQ-ACK enabled, UE will report NACK for the G-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

Proposal 14: For Type-1 codebook generation of dynamic multicast scheduling configured with ACK/NACK based feedback and multicast SPS configured with disabled HARQ feedback,

- UE will report NACK for the G-CS-RNTI with HARQ-ACK disabled regardless of decoding results of corresponding PDSCH.

Proposal 15: UE is not expected to be configured with Type-1 HARQ-ACK codebook and with presence of enabled/disabled HARQ-ACK feedback indicator in multicast DCI simultaneously.

- Note: this restriction is per UE.

- References

R1-2202883, FL summary#5 on improving reliability for MBS for RRC_CONNECTED UEs, Moderator (Huawei), RAN1#108-e, Feb 21-Mar 3, 2022.

TS 38.213, NR; Physical layer procedures for control (Release 17), V17.1.0 (2022-03).