3GPP TSG-RAN WG1 Meeting #109-e	R1-2203071

e-Meeting, May 9– 20, 2022

Agenda Item:	8.16.12

Source:	Huawei, HiSilicon

Title:	Rel-17 UE features for NR MBS

Document for:	Discussion and Decision

# Introduction

After RAN1#108-e discussion, the RAN1 UE features for Rel-17 NR is updated in [1]. This contribution provides our views for Rel-17 NR MBS UE features.

# UE feature related discussion

## FG33-1 for MBS Broadcast

Since DCI format 4_0 is defined in TS 38.212 for scheduling broadcast, DCI format 1_0 in the 3rd component needs to be updated to DCI format 4_0.

MBS broadcast includes MCCH and MTCH, both of which will be scheduled by G-RNTI. The 6th component needs to clarify that group-common PDSCH including MCCH and MTCH will be TDM-ed or either one will be TDM-ed with unicast in different slots.

It was proposed to have two additional components to be included in FG33-1 regarding rate matching

- Support of semi-static rate-matching resource set configuration.

- Support of rate-matching around LTE CRS.

It was concerned to directly include such two bullets but instead was suggested to add “A UE supporting FG 33-1 must indicate support of FGs 5-26 and 5-28 for broadcast” in the column of note to clarify the rate matching capabilities are the ones mandatory support in Rel-15. This suggestion can be taken for approval in this meeting.

One more leftover issue for FG33-1 from the last meeting was whether merging DCI indicating slot-level repetition up to 16 for MTCH into FG 33-3-1. We think it could be merged and ends up a general capability with either FG33-1 or FG33-2 as prerequisite FG. The update FG33-3-1 refers to section 2.4

Overall, the FG33-1 for broadcast can be updated as in the following proposal.

Proposal 1: Updating FG33-1 as follows in red:

| 33. NR_MBS | 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 4_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and MCCH group-common PDSCH or MTCH group-common PDSCH, or between MCCH group-common PDSCH and MTCH group-common PDSCH, or among unicast PDSCH and MCCH group-common PDSCH and MTCH group-common PDSCH in different slots. - Support MCCH change notification indication via DCI. - support of higher layer configured slot-level repetition up to 8 for MTCH. |  | Up to RAN2 |  |  | Up to RAN2 | Up to RA-N2 | Up to RAN2 |  | Note: A UE supporting FG 33-1 must indicate support of FGs 5-26 and 5-28 for broadcast. | Up to RAN2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## FG33-2 for MBS multicast

To align with TS 38.212 CR, the DCI format 1_0 in the 4th component of FG33-2 should be updated to 4_1 (based on DCI format 1_0).

It was discussed in the last meeting that whether FG33-1 is the basic feature group for Rel-17 NR MBS and if it is then FG33-1 for broadcast will be the prerequisite FG for any other FGs defined. We clarified in the last meeting that broadcast and multicast are separate FGs demanded in different cases and there are still some differences for developing such two features from UE implementation aspect although the basic feature sets for them are strived to be common. Therefore, for early commercialization, it is preferred to keep FG33-1 and FG33-2 as separate independent FGs.

When considering UE capability for supporting both multicast and broadcast, it would be preferred to describe it in FG for multicast instead of defining additional FG for supporting both. For example, for the 5th component of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots where the group-common PDSCH is intended for multicast, it could be updated to consider broadcast group-common PDSCH as well if UE supports FG33-1.

In addition, the following was agreed in the last meeting:

Agreement

- FG 33-2 is separated to two FGs: One for PCell case, the other for SCell case

  - Type of FG for PCell case is [per band or per FSPC]

  - Type of FG for SCell case is per FSPC

  - FG for PCell is added as a Prerequisite FG for FG for SCell

As FG33-2 for SCell is reported per FSPC, FG33-2 for PCell can also be reported per FSPC. Alternatively, it can be reported per BC considering UE reporting larger CA bandwidth class for a BC then UE may not report the support of FG33-2.

For FG33-2a, considering support of shared PUCCH resource configurations with unicast has been the component for this FG, support of separate PUCCH resource configurations from unicast can be separate FG from FG 33-2a if separate PUCCH resources configurations may take extra UE buffer.

Proposal 2: Updating FG33-2, FG33-2a, FG33-2b, , and adding FG33-2c as follows in red:

| 33. NR_MBS | 33-2 | Dynamic scheduling for multicast for PCell | - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI for PCell. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 4_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH for multicast, or between group-common PDSCH for multicast and group-common PDSCH for broadcast (if UE supports FG33-1), or among unicast PDSCH and group-common PDSCH for multicast and group-common PDSCH for broadcast (if UE supports FG33-1) in different slots. - Support {2, 4, 8} times semi-static slot-level repetition for group-common PDSCH for multicast. | FFS | Yes |  |  | Per BC or per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-2a | Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | 1) Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling 2) Support of PTM retransmission for multicast 3) support of Type-1 and Type-2 HARQ-ACK CB for multicast feedback only 4) Support of shared PUCCH resource configurations with unicast FFS: Whether to define separate PUCCH resource configurations from unicast as separate FG from FG 33-2a | 33-2 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2b | DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling | 33-2a | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2c | Support of separate PUCCH resource configurations from unicast for ACK/NACK based feedback for dynamic scheduling for multicast | Support of separate PUCCH resource configurations from unicast for ACK/NACK based feedback for dynamic scheduling for multicast | 33-2a | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |

## FG33-2e/FG33-5-1h for supported RNTIs

RAN2 has agreed to define UE capability for the number of simultaneous G-RNTIs / G-CS-RNTIs reception for multicast [2] and companies were invited to provide the views on the actual maximum number of G-RNTIs/G-CS-RNTIs (or their combination) for UE to simultenously monitor, for which RAN2 discussions seem to leave it to RAN1 [3].

In the updated UE feature list[1], FG33-2e and FG33-5-1h defines the number of G-RNTIs and G-CS-RNTIs UE supports, respectively as follows:

| 33. NR_MBS | 33-2e | Multiple G-RNTIs for group-common PDSCHs | Capability on number of G-RNTI for groupcast FFS details. | [33-2] | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-5-1h | Multiple G-CS-RNTIs for SPS group-common PDSCHs | Max number of G-CS-RNTIs for SPS multicast [per slot per CC] | 33-5-1 | Yes |  |  | [Per UE] | [No] | [No] |  | FFS max number | Optional with capability signalling |

FG33-2e should be revised as the maximum number of G-RNTIs supported for multicast dynamic scheduling and the candidate values can be {2, 3, 4} or keep the max number is 4.

FG33-5-1h should be revised as the maximum number of G-CS-RNTIs for SPS multicast per CC, because such number is from the configuration-wise instead of scheduling-wise.

In addition, FG33-2e and FG33-5-1h are currently worded for the number of supported G-RNTIs/G-CS-RNTIs for multicast. In light of avoiding device hardware impact to facilitate implementation and deployment of the feature, receiving multicast and broadcast may take the resources built for unicast from UE implementation perspective without changing the existing hardware, e.g., the maximum number of RNTIs UE supports that was dimensioned for unicast will be shared with multicast as well when to support MBS multicast. Therefore, in addition to the value of the maximum number of G-RNTIs/G-CS-RNTIs supported for multicast in FG33-2e/FG33-5-1h, UE reporting the total number of all RNTIs (e.g., as introduced in FG33-10) that UE can support can also help network schedule unicast properly.

Additional reporting the total number of all RNTIs was discussed in the last meeting and the comment is mainly about the relation with other features besides NR MBS. As explained, the motivation was not changing UE hardware to support multicast/broadcast. Since the RNTIs UE supports as capability should be pre-dimensioned before UE implementation and when building multicast/broadcast on top of the current UE without changing the hardware, in addition to reporting the number of G-RNTIs or G-CS-RNTIs, reporting the total number of RNTIs (including all cases, e.g., PS-RNTI, IMT-RNTI, ….) can help network decide which feature can be configured to the UE. Without this reporting, UE may have to not report the capability of supporting some UE feature (but actually UE does have such capability) so as to control the features corresponding to the RNTIs not exceeding UE capability. It is per cell. The deficit of RNTs is originated from supporting multiple G-RNTI/G-CS-RNTI. If UE does not support MBS, this number may not be needed to be reported as legacy. From this sense, we can discuss it under MBS and even can have MBS FG33-2e/FG33-5-1h as the prerequisite FG.

There are two alternative that can be considered as following proposal:

Proposal 3: Consider the following two alternatives to modify the FG33-2e/FG33-5-1h:

- Alt1: Updating FG33-2e/FG33-5-1h and adding FG33-10 as follows in red:

| 33. NR_MBS | 33-2e | Multiple G-RNTIs for dynamic group-common PDSCHs | The maximum number of G-RNTIs supported for multicast dynamic scheduling per CC. FFS details. | 33-2 | Yes |  |  | Per UE | No | No |  | Candidate values for X is: {2, 3, 4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-5-1h | Multiple G-CS-RNTIs for SPS group-common PDSCHs | The maximum number of G-CS-RNTIs for SPS multicast per CC | 33-5-1 | Yes |  |  | Per UE | No | No |  | Candidate values for X is: {2, 3, 4} | Optional with capability signalling |
| 33. NR_MBS | 33-10 | Total number of RNTIs | The maximum number of all RNTIs UE can support per CC. | 33-2e and/or 33-5-1h | Yes |  |  | Per UE | No | No |  | Candidate values for X is: {16, 32} | Optional with capability signalling |

- Alt2: Updating FG33-2e/FG33-5-1h as follows in red:

| 33. NR_MBS | 33-2e | Multiple G-RNTIs for dynamic group-common PDSCHs | - The maximum number of G-RNTIs supported for multicast dynamic scheduling per CC. - Indication of whether the maximum number of G-RNTIs supported for multicast is to be added to the maximum number of supported RNTIs without multicast operation, or to be subtracted from the maximum number of supported RNTIs without multicast operation. | 33-2 | Yes |  |  | Per UE | No | No |  | For component 1：Candidate values for X is: {2, 3, 4} For component 2: candidate value is {added, subtracted} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-5-1h | Multiple G-CS-RNTIs for SPS group-common PDSCHs | - The maximum number of G-CS-RNTIs for SPS multicast per CC. - Indication of whether the maximum number of G-CS-RNTIs supported for multicast is to be added to the maximum number of supported RNTIs without multicast operation, or to be subtracted from the maximum number of supported RNTIs without multicast operation. | 33-5-1 | Yes |  |  | Per UE | No | No |  | For component 1：Candidate values for X is: {2, 3, 4} For component 2: candidate value is {added, subtracted} | Optional with capability signalling |

## FG33-2f for DCI format 4_2 monitoring

As discussed in section 2.9, support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling can be a separate FG in FG33-5-1 and can be merged into FG33-2f, which can be updated with G-CS-RNTI included.

Proposal 4: Updating FG33-2f as follows in red:

| 33. NR_MBS | 33-2f | Dynamic multicast and SPS multicast with DCI format 4_2 | Support of DCI format 4_2 with CRC scrambled with G-RNTI/G-CS-RNTI for multicast | 33-2 or 33-5-1 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## FG33-3-1 for dynamic slot-level repetitions

As discussed in section 2.1, dynamic slot-level repetition is proposed to be a separate FG from FG33-1 for broadcast and it can be merged into FG33-3-1 with FG33-1 as prerequisite FG for broadcast. The FG33-3-1 can be updated as the following proposal.

Proposal 5: Updating FG33-3-1 as follows in red:

| 33. NR_MBS | 33-3-1 | Dynamic Slot-level repetition for group-common PDSCH | - Support up to X times dynamic slot-level repetition for group-common PDSCH for multicast or for broadcast MTCH. | 33-1 or 33-2 | Yes |  |  | Per UE | No | No |  | Candidate values for X is: {8, 16} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## FG33-3-2 for FDMed unicast and group-common PDSCH

The IE FeatureSetDownlinkPerCC indicates a set of features that the UE supports on the corresponding carrier of one band entry of a band combination including supportedBandwidthDL, maxNumberMIMO-LayersPDSCH and supportedModulationOrderDL that can determine an maximum TB size in a slot. For UE supporting FDM-ed unicast PDSCH and one group-common PDSCH in a slot, the maximum TBS size is supposed to be the total maximum TB size for both unicast and group-common PDSCH, which should be clarified if it is not reported.

It should be noted that two cases (i.e., FDM-ed unicast and multicast and FDM-ed unicast and broadcast) can be considered but separate UE capabilities indications should be set. MBS broadcast may be received by RRC_IDLE/INACTIVE UEs in  addition to RRC_CONNECTED UEs, so even UE reports the support of FDM-ed unicast and broadcast, from network perspective, the scheduling for broadcast would take precedence over the scheduling for unicast because UEs in RRC_IDLE/INACTIVE UEs may not support FDM-ed unicast and broadcast. However, multicast is transmitted to UEs in RRC_CONNECTED state and then network has the full information of UEs regarding whether support FDM-ed unicast and multicast, so scheduling of unicast and multicast can be more flexibly adjustable based on the capability reporting.

In addition, it should be reported per FSPC.

Proposal 6: Updating FG33-3-2 and adding FG33-3-2a as follows in red:

| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and one group-common PDSCH for broadcast in a slot | - Support of FDM between one unicast PDSCH and one group-common PDSCH in a slot. | 33-1 or 33-2 | Yes |  |  | Per FSPC | NA | NA |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-3-2a | FDM-ed unicast PDSCH and one group-common PDSCH for multicast in a slot | - Support of FDM between one unicast PDSCH and one group-common PDSCH in a slot. | 33-2 | Yes |  |  | Per FSPC | NA | NA |  |  | Optional with capability signalling |

## FG33-3-3 for intra-slot TDM-ed unicast and group-common PDSCH

Similar rationale for the update for FG33-3-2, the maximum TBS size supported in a slot should be clarified as the total maximum TB size for both unicast and group-common PDSCH when UE supports intra-slot TDM-ed unicast and group-common PDSCH.

It should be noted that two cases (i.e., FDM-ed unicast and multicast and FDM-ed unicast and broadcast) are included depending on FG33-1 or FG33-2 is prerequisite FG for FG33-3-3. In addition, it should be reported per FSPC. Since FG33-3-3 is reported separately from FG33-1, MBS broadcast transmission should be based on the assumption that there are UEs supporting FG33-1 but not support FG33-3-3. Therefore, the component 3 or component 4 with N/L TDMed group-common PDSCHs in a slot per CC should be only applied to multicast with FG33-1 as prerequisite FG.

Proposal 7: Updating FG33-3-3 as follows in red:

| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). | 33-1 or 33-2 | Yes |  |  | Per FSPC | NA | NA |  | Note: component 3 or component 4 with N/L TDMed group-common PDSCHs in a slot per CC is only applied to multicast with FG33-1 as prerequisite FG. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## FG33-4 for NACK-only based feedback

Similar to the support of ACK/NACK-only based feedback for multicast as in FG33-2a, the support of NACK-only based feedback and RRC based enabling/disabling NACK-only based feedback as well as PTM retransmission can be merged into a single FG33-4. Since UE does not feedback ACK when the TB is decoded correctly and the PUCCH resources might be shared among UEs for reporting NACK if the TB is failed in decoding, network cannot differentiate which UE is failed in decoding so UE does not need to support PTP retransmission for NACK-only based feedback. Moreover, considering support of shared PUCCH resource configurations with unicast has been the component for this FG, support of separate PUCCH resource configurations from unicast can be separate FG from FG 33-4 if separate PUCCH resources configurations may take extra UE buffer.

In addition, regarding supporting more than one NACK-only feedback in the same PUCCH transmission, the two Alternatives are supported taking into UE capability into consideration:

Agreement

For supporting more than one NACK-only feedback in the same PUCCH transmission, define RRC configuration to configure between Alt1 and Alt4 (from previous agreements):

- Alt1: Support UE multiplexing the HARQ-ACK bits by transforming NACK-only into ACK/NACK HARQ bits.

  - FFS: how to determine PUCCH resource

- Alt4: Define combination of NACK-only which corresponds to a specific sequence or a PUCCH transmission.

  - define up to 15 orthogonal PUCCH resources to select from according to combinations of up to 4 TBs with NACK-only feedback,

    - FFS: The PUCCH slot for the transmission is based on the K1 in the “last DCI” scheduling multicast.

    - FFS: The PUCCH resource for the transmission is from PUCCH-config configured for NACK-only based feedback according to the mapping between number of TBs with PUCCH resource ID.

      - FFS mapping details.

      - How to determine the number of TBs is discussed separately, e.g., Type-1-like and/or Type-2-like codebook.

    - FFS: whether this applies to a single G-RNTI or multiple G-RNTIs

  - Alt4 is not supported for more than 4 TBs

- FFS: whether RRC configuration between Alt1 and Alt4 is per G-RNTI or per CFR

- FFS: UE capability

With the above analysed, the FG33-4 can be updated as follows:

Proposal 8: Updating FG33-4 and adding FG33-4-1a as follows in red:

| 33. NR_MBS | 33-4 | Support of NACK-only based HARQ-ACK feedback and RRC-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast | - Support NACK-only based HARQ-ACK feedback for dynamic scheduling for multicast, including: - A single TB with NACK-only transmitted in PUCCH - Multiple TB with NACK-only transmitted in PUCCH by transforming into ACK/NACK - Multiple TB with NACK-only transmitted in PUCCH by select one PUCCH resource. - Support of shared PUCCH resource configurations with unicast - support of enabling/disabling NACK-only based HARQ-ACK feedback for dynamic scheduling for multicast per the configuration of RRC signalling. - Support PTM retransmission for dynamic scheduling for multicast. FFS: Whether to define separate PUCCH resource configurations from unicast as separate FG from FG 33-4 | 33-2 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-4-1a | Support of separate PUCCH resource configurations from unicast for NACK-only based feedback for dynamic scheduling for multicast | - Support of separate PUCCH resource configurations from unicast for NACK-only based feedback for dynamic scheduling for multicast, including: - A single TB with NACK-only transmitted in PUCCH - Multiple TB with NACK-only transmitted in PUCCH by transforming into ACK/NACK - Multiple TB with NACK-only transmitted in PUCCH by select one PUCCH resource. - support of enabling/disabling NACK-only based HARQ-ACK feedback for dynamic scheduling for multicast per the configuration of RRC signalling. - Support PTM retransmission for dynamic scheduling for multicast. | 33-4 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |

## FG33-5-1/1a/1d/2 for multicast SPS

Support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling can be a separate FG in FG33-5-1 and can be merged into FG33-2f.

Support of PTM retransmission can be grouped into FG33-5-1a.

Support of PTP retransmission can be separate in FG33-5-1d but is proposed to be separate from FG33-2d.

Support of FG33-5-2 can be reported per UE.

Proposal 9: Updating FG33-5-1/1a/1d/2 as follows in red:

| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast | - Support one SPS group-common PDSCH configuration for multicast - Support {2, 4, 8} times semi-static slot-level repetition for SPS group-common PDSCH FFS whether to separate the capability for support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling | 33-2 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-5-1a | Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast | - Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling, SPS group-common PDSCH activation, and SPS release PDCCH - Support of PTM retransmission associated with G-CS-RNTI for SPS multicast | 33-5-1 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1d | PTP retransmission for SPS group-common PDSCH for multicast | Support of PTP retransmission for SPS multicast FFS whether to merge with 33-5-1a | 33-5-1a | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-2 | Multiple SPS group-common PDSCH configuration | - Support [N>1] SPS group-common PDSCH configuration per CFR for multicast, and up to K SPS group-common PDSCH configuration in a cell group - Support M>=1 activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast | 33-2 | Yes |  |  | Per UE | No | No |  | FFS the value range for K FFS: Candidate values for M | Optional with capability signalling |

## FG33-6-1 for different priorities

For dynamic scheduling, the priority indication can be included in DCI format 4_2 for multicast. The priority index cannot be directly configured by RRC signalling, although it means low priority when the priority indication is not included in the DCI format. Therefore, separating UE capabilities of supporting priority configuration from supporting priority indication included in DCI format does not seem necessary.

For multicast SPS scheduling, the priority index is configured together with the configuration for SPS. There could be an UE capability of support of priority configuration for multicast SPS but it is not needed to have a capability defined for support of priority indicator field in the DCI format just for multicast SPS scheduling.

Proposal 10: Updating FG33-6-1 and adding FG33-6-1a as follows in red:

| 33. NR_MBS | 33-6-1 | DL priority indication in DCI for multicast dynamic scheduling | - Support of priority indicator field configured in DCI format 4_2 with CRC scrambled with G-RNTI for multicast dynamic scheduling. - Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast and multicast at a UE | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-6-1a | DL priority configuration for SPS multicast | Support of priority configured for multicast HARQ-ACK feedback of SPS multicast | 33-6-1 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |

# Conclusions

This contribution provides our views of UE features for Rel-17 NR MBS with the proposals summarized as follows:

Proposal 1: Updating FG33-1 as follows in red:

| 33. NR_MBS | 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 4_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and MCCH group-common PDSCH or MTCH group-common PDSCH, or between MCCH group-common PDSCH and MTCH group-common PDSCH, or among unicast PDSCH and MCCH group-common PDSCH and MTCH group-common PDSCH in different slots. - Support MCCH change notification indication via DCI. - support of higher layer configured slot-level repetition up to 8 for MTCH. |  | Up to RAN2 |  |  | Up to RAN2 | Up to RA-N2 | Up to RAN2 |  | Note: A UE supporting FG 33-1 must indicate support of FGs 5-26 and 5-28 for broadcast. | Up to RAN2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2: Updating FG33-2, FG33-2a, FG33-2b, , and adding FG33-2c as follows in red:

| 33. NR_MBS | 33-2 | Dynamic scheduling for multicast for PCell | - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI for PCell. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 4_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH for multicast, or between group-common PDSCH for multicast and group-common PDSCH for broadcast (if UE supports FG33-1), or among unicast PDSCH and group-common PDSCH for multicast and group-common PDSCH for broadcast (if UE supports FG33-1) in different slots. - Support {2, 4, 8} times semi-static slot-level repetition for group-common PDSCH for multicast. | FFS | Yes |  |  | Per BC or per FSPC | N/A | N/A |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-2a | Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | 1) Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling 2) Support of PTM retransmission for multicast 3) support of Type-1 and Type-2 HARQ-ACK CB for multicast feedback only 4) Support of shared PUCCH resource configurations with unicast FFS: Whether to define separate PUCCH resource configurations from unicast as separate FG from FG 33-2a | 33-2 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2b | DCI-based enabling/disabling ACK/NACK-based feedback for dynamic scheduling for multicast | Support of DCI-based enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling | 33-2a | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2c | Support of separate PUCCH resource configurations from unicast for ACK/NACK based feedback for dynamic scheduling for multicast | Support of separate PUCCH resource configurations from unicast for ACK/NACK based feedback for dynamic scheduling for multicast | 33-2a | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |

Proposal 3: Consider the following two alternatives to modify the FG33-2e/FG33-5-1h:

- Alt1: Updating FG33-2e/FG33-5-1h and adding FG33-10 as follows in red:

| 33. NR_MBS | 33-2e | Multiple G-RNTIs for dynamic group-common PDSCHs | The maximum number of G-RNTIs supported for multicast dynamic scheduling per CC. FFS details. | 33-2 | Yes |  |  | Per UE | No | No |  | Candidate values for X is: {2, 3, 4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-5-1h | Multiple G-CS-RNTIs for SPS group-common PDSCHs | The maximum number of G-CS-RNTIs for SPS multicast per CC | 33-5-1 | Yes |  |  | Per UE | No | No |  | Candidate values for X is: {2, 3, 4} | Optional with capability signalling |
| 33. NR_MBS | 33-10 | Total number of RNTIs | The maximum number of all RNTIs UE can support per CC. | 33-2e and/or 33-5-1h | Yes |  |  | Per UE | No | No |  | Candidate values for X is: {16, 32} | Optional with capability signalling |

- Alt2: Updating FG33-2e/FG33-5-1h as follows in red:

| 33. NR_MBS | 33-2e | Multiple G-RNTIs for dynamic group-common PDSCHs | - The maximum number of G-RNTIs supported for multicast dynamic scheduling per CC. - Indication of whether the maximum number of G-RNTIs supported for multicast is to be added to the maximum number of supported RNTIs without multicast operation, or to be subtracted from the maximum number of supported RNTIs without multicast operation. | 33-2 | Yes |  |  | Per UE | No | No |  | For component 1：Candidate values for X is: {2, 3, 4} For component 2: candidate value is {added, subtracted} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-5-1h | Multiple G-CS-RNTIs for SPS group-common PDSCHs | - The maximum number of G-CS-RNTIs for SPS multicast per CC. - Indication of whether the maximum number of G-CS-RNTIs supported for multicast is to be added to the maximum number of supported RNTIs without multicast operation, or to be subtracted from the maximum number of supported RNTIs without multicast operation. | 33-5-1 | Yes |  |  | Per UE | No | No |  | For component 1：Candidate values for X is: {2, 3, 4} For component 2: candidate value is {added, subtracted} | Optional with capability signalling |

Proposal 4: Updating FG33-2f as follows in red:

| 33. NR_MBS | 33-2f | Dynamic multicast and SPS multicast with DCI format 4_2 | Support of DCI format 4_2 with CRC scrambled with G-RNTI/G-CS-RNTI for multicast | 33-2 or 33-5-1 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 5: Updating FG33-3-1 as follows in red:

| 33. NR_MBS | 33-3-1 | Dynamic Slot-level repetition for group-common PDSCH | - Support up to X times dynamic slot-level repetition for group-common PDSCH for multicast or for broadcast MTCH. | 33-1 or 33-2 | Yes |  |  | Per UE | No | No |  | Candidate values for X is: {8, 16} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 6: Updating FG33-3-2 and adding FG33-3-2a as follows in red:

| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and one group-common PDSCH for broadcast in a slot | - Support of FDM between one unicast PDSCH and one group-common PDSCH in a slot. | 33-1 or 33-2 | Yes |  |  | Per FSPC | NA | NA |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-3-2a | FDM-ed unicast PDSCH and one group-common PDSCH for multicast in a slot | - Support of FDM between one unicast PDSCH and one group-common PDSCH in a slot. | 33-2 | Yes |  |  | Per FSPC | NA | NA |  |  | Optional with capability signalling |

Proposal 7: Updating FG33-3-3 as follows in red:

| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). | 33-1 or 33-2 | Yes |  |  | Per FSPC | NA | NA |  | Note: component 3 or component 4 with N/L TDMed group-common PDSCHs in a slot per CC is only applied to multicast with FG33-1 as prerequisite FG. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 8: Updating FG33-4 and adding FG33-4-1a as follows in red:

| 33. NR_MBS | 33-4 | Support of NACK-only based HARQ-ACK feedback and RRC-based enabling/disabling NACK-only based feedback for dynamic scheduling for multicast | - Support NACK-only based HARQ-ACK feedback for dynamic scheduling for multicast, including: - A single TB with NACK-only transmitted in PUCCH - Multiple TB with NACK-only transmitted in PUCCH by transforming into ACK/NACK - Multiple TB with NACK-only transmitted in PUCCH by select one PUCCH resource. - Support of shared PUCCH resource configurations with unicast - support of enabling/disabling NACK-only based HARQ-ACK feedback for dynamic scheduling for multicast per the configuration of RRC signalling. - Support PTM retransmission for dynamic scheduling for multicast. FFS: Whether to define separate PUCCH resource configurations from unicast as separate FG from FG 33-4 | 33-2 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-4-1a | Support of separate PUCCH resource configurations from unicast for NACK-only based feedback for dynamic scheduling for multicast | - Support of separate PUCCH resource configurations from unicast for NACK-only based feedback for dynamic scheduling for multicast, including: - A single TB with NACK-only transmitted in PUCCH - Multiple TB with NACK-only transmitted in PUCCH by transforming into ACK/NACK - Multiple TB with NACK-only transmitted in PUCCH by select one PUCCH resource. - support of enabling/disabling NACK-only based HARQ-ACK feedback for dynamic scheduling for multicast per the configuration of RRC signalling. - Support PTM retransmission for dynamic scheduling for multicast. | 33-4 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |

Proposal 9: Updating FG33-5-1/1a/1d/2 as follows in red:

| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast | - Support one SPS group-common PDSCH configuration for multicast - Support {2, 4, 8} times semi-static slot-level repetition for SPS group-common PDSCH FFS whether to separate the capability for support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling | 33-2 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-5-1a | Support of ACK/NACK based HARQ-ACK feedback and RRC-based enabling/disabling ACK/NACK-based feedback for SPS group-common PDSCH for multicast | - Support of ACK/NACK based HARQ-ACK feedback, and support of enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling, SPS group-common PDSCH activation, and SPS release PDCCH - Support of PTM retransmission associated with G-CS-RNTI for SPS multicast | 33-5-1 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1d | PTP retransmission for SPS group-common PDSCH for multicast | Support of PTP retransmission for SPS multicast FFS whether to merge with 33-5-1a | 33-5-1a | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-2 | Multiple SPS group-common PDSCH configuration | - Support [N>1] SPS group-common PDSCH configuration per CFR for multicast, and up to K SPS group-common PDSCH configuration in a cell group - Support M>=1 activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast | 33-2 | Yes |  |  | Per UE | No | No |  | FFS the value range for K FFS: Candidate values for M | Optional with capability signalling |

Proposal 10: Updating FG33-6-1 and adding FG33-6-1a as follows in red:

| 33. NR_MBS | 33-6-1 | DL priority indication in DCI for multicast dynamic scheduling | - Support of priority indicator field configured in DCI format 4_2 with CRC scrambled with G-RNTI for multicast dynamic scheduling. - Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast and multicast at a UE | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-6-1a | DL priority configuration for SPS multicast | Support of priority configured for multicast HARQ-ACK feedback of SPS multicast | 33-6-1 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |

# References

R1-2202928, Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#108-e, February 21- March 3, 2022.

RAN2 chair notes, RAN2 Chair (MediaTek), RAN2#116bis-e meeting, January 17-25, 2022.

R2-2201865, [AT116bis-e] [026][MBS] UE capabilities (MediaTek), MediaTek, RAN2#116bis-e meeting, January 17-25, 2022.