3GPP TSG-RAN WG1 Meeting #109-e	R1-2203073

e-Meeting, May 9-20, 2022

Agenda Item:	8.3.2

Source:	Huawei, HiSilicon

Title:	Intra-UE multiplexing enhancements

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss the remaining issues for intra-UE multiplexing and prioritization of Rel-17 URLLC, including those for the intra-UE multiplexing on PUCCH and PUSCH. In addition, some suggestions for text proposals are also provided.

Remaining issues for UCI multiplexing on PUCCH

2.1 HP SPS HARQ-ACK provided by n1PUCCH-AN collides LP HARQ-ACK

A specific case was raised by companies in the last meeting that HP SPS HARQ-ACK provided by n1PUCCH-AN collides LP HARQ-ACK. According to the current spec, the HP PUCCH resource determined by n1PUCCH-AN in SPS-config is either PF0 or PF1 eligible to carry up to 2 bits. Hence, if the total HARQ-ACK bits become more than 2 bits as a result of merging the additional LP HARQ-ACK payload, the total HARQ-ACK cannot be accommodated by the HP SPS PUCCH. Thus, a straightforward way is to drop LP HARQ-ACK under such collision.

Proposal 1: When a PUCCH carrying HP HARQ-ACK provided by n1PUCCH-AN overlaps with a PUCCH carrying LP HARQ-ACK with total number of HARQ-ACK bits more than 2 bits, the LP HARQ-ACK is dropped.

2.2 HP PUCCH with HARQ-ACK and SR collides LP HARQ-ACK

In the last meeting [1], the handling of overlapped HP PUCCH with PF0/1 including HARQ-ACK and SR with LP PUCCH with HARQ-ACK was discussed, and there are two options for further down selection as shown in below.

Proposal 1: When a PUCCH carrying explicit/implicit HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, where the original PUCCH carrying the HP HARQ-ACK before Step 1 overlaps with K HP SRs which are all negative or include at least one positive, - In Step 1, 1-bit HP SR is multiplexed to HP HARQ-ACK bits (based on Rel-15 rules) for determining the resultant PUCCH resource. - Note: The description of Step 1 here is only for information purpose, which has no spec impact. - In Step 2, down-select from the two options: - Option 0: LP HARQ-ACK is dropped. When a PUCCH carrying positive HP SR and HP HARQ-ACK with PUCCH format 0/1 overlaps with a PUCCH carrying LP HARQ-ACK, the PUCCH carrying LP HARQ-ACK is dropped. - Option 2: If HP HARQ-ACK is dynamic HARQ-ACK or SPS HARQ-ACK with sps-PUCCH-AN-List, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1 - The resultant PUCCH resource for multiplexing HP SR + HP HARQ-ACK + LP HARQ-ACK is either of PF2, PF3, or PF4. If HP HARQ-ACK is SPS HARQ-ACK with n1PUCCH-AN, LP HARQ-ACK is dropped.

Between these 2 options, we slightly prefer Option 2, because the unnecessary dropping of LP HARQ-ACK should be avoided. Moreover, for Option 0, dropping LP HARQ-ACK may cause additional ambiguity issues. Considering the case of HP PUCCH PF0 including 1 bit HP HARQ-ACK (and HP SR if any) overlaps with 1 bit LP HARQ-ACK, the gNB cannot distinguish HP SR from LP HARQ-ACK under some certain cyclic shifts.

An example of the ambiguity situation is shown in Figure 1, and Table 1 shows the interpretation of each case. If all HP SRs are negative (Case 1 in the figure), the LP HARQ-ACK will be treated as 1 bit HP HARQ-ACK, and the 2 bits HP+LP HARQ-ACKs will use CS 0/3/6/9 for HP PUCCH PF0; on the other hand, if at least one SR is positive (Case 2 in the figure), the LP HARQ-ACK is dropped, and the 1bit HP HARQ-ACK and 1bit HP SR is transmitted on the same HP PUCCH PF0 with CS 3/9. That means, if UE reports CS 3/9, gNB cannot distinguish whether the 2 bits feedback information represent HP HARQ-ACK + HP SR or HP HARQ-ACK + LP HARQ-ACK.

Figure 1 The ambiguity case for Option 0

Table 1 The possible interpretations of the same CS

| Case |  | CS = 3 | CS = 9 |
| --- | --- | --- | --- |
| Case 1 | HP A/N + LP A/N (negative SR) | NACK, ACK | ACK, NACK |
| Case 2 | HP A/N + HP SR | NACK, positive | ACK, positive |

As another issue, we need to discuss the case when HP SR has been dropped in Step 1. In Rel-15, when HARQ-ACK of PF1 collides with SR of PF0, the SR should be dropped, and only HARQ-ACK is transmitted on PUCCH. When that case occurs in Step 1 of Rel-17, the UE has no information of the status of SR which has been discarded from the buffer at Step 1. So the HP PUCCH under this case should be regarded as the PUCCH with HP HARQ-ACK only, and consequently the handling of the HP PUCCH including HARQ-ACK and SR described in the above proposal should not be applied to this case. As a comparison case of channel selection between HP HARQ-ACK PF1 and HP SR PF1 at Step 1, the HP SR status is still stored in buffer, so the above proposal should be applied to this case which is regarded as ‘PUCCH carrying implicit SR’.

Proposal 2: When a PUCCH carrying HP SR and HP HARQ-ACK with PF0/1 overlaps with a PUCCH carrying LP HARQ-ACK:

- Option 2 is adopted, i.e., if HP HARQ-ACK is dynamic HARQ-ACK or SPS HARQ-ACK with sps-PUCCH-AN-List, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1.

Proposal 3: For the case of HP SR with PF0 dropped due to collision with HP HARQ-ACK PF1 in Step 1, the resultant HP PUCCH after Step 1 is regarded as HP PUCCH with HARQ-ACK only when resolving the overlapping with LP HARQ-ACK in Step 2.

Remaining issues for UCI multiplexing on PUSCH

3.1 Insufficient resources of HP PUSCH multiplexed with LP HARQ-ACK

In the last meeting, the multiplexing of LP HARQ-ACK onto HP PUSCH has been discussed. One remaining issue is that HP PUSCH has no enough resource to carry the LP HARQ-ACK. The candidate options were raised as shown below.

Proposal 2: For the scenarios where multiplexing low-priority HARQ-ACK onto high-priority PUSCH, down-select from the options: - Option 1: In case of insufficient resource for LP HARQ-ACK, LP HARQ-ACK is entirely dropped - Option 2: LP HARQ-ACKs are mapped to the rest REs of the PUSCH based on the rate matching equation, if HP HARQ-ACK and/or HP CSI have been mapped in prior on the PUSCH. - Option 3: UE does not expect insufficient resource for multiplexing low-priority HARQ-ACK.

In our view, the gNB is in full flexibility to schedule the resource of the HP PUSCH: if the LP HARQ-ACK is important, gNB will ensure there is sufficient resource; otherwise the gNB can choose not to receive the LP HARQ-ACK by implementation. The same principle with multiplexing HP HARQ-ACK and LP HARQ-ACK on PUCCH as shown in the agreement below should be adopted as a unified design.

Agreement For determining the PUCCH resource to carry the multiplexed high-priority and low-priority HARQ-ACKs, if OHP_UCI+OCRC, HP_UCIrHP_UCI∙Qm+OLP_UCI+OCRC, LP_UCIrLP_UCI∙Qm >MRBPUCCH∙Nsc, ctrlRB∙Nsymb-UCIPUCCH - The number of RBs is MRBPUCCH. Then follow Rel-15 procedure, i.e., LP HARQ-ACK is mapped to the rest REs after HP HARQ-ACK.

Proposal 4: For the scenario of multiplexing low-priority HARQ-ACK onto high-priority PUSCH, support Option 2, i.e. LP HARQ-ACKs are mapped to the rest REs of the PUSCH based on the rate matching equation, if HP HARQ-ACK and/or HP CSI have been mapped in prior on the PUSCH.

3.2 Multiplexing of CG-UCI on PUSCH of a different priority

In the last meeting, the agreement has been achieved for LP HARQ-ACK without HP HARQ-ACK transmitted on HP PUSCH. The remaining issue is the potential behavior of how to multiplex CG-UCI.

Agreement If LP HARQ-ACK without HP HARQ-ACK would be transmitted on HP PUSCH, down-select from the options: - Option 2: UE follows the same behaviour as that in case of PUSCH with HP HARQ-ACK assuming two bits - FFS for CG-UCI PUSCH.

For the issue when CG-UCI is present, the CG-UCI should be considered as the same priority with the PUSCH, and the joint encoding of CG-UCI with the HARQ-ACK of the same priority can be reused, while the CG-UCI with the HARQ-ACK of a different priority should be separately encoded. In particular, we can consider the following scenarios.

CG-UCI on HP PUSCH:

- LP HARQ-ACK without HP HARQ-ACK. In this case, CG-UCI can be regarded as HP HARQ-ACK, and the agreed rule of multiplexing HP HARQ-ACK and LP HARQ-ACK on HP PUSCH can be directly reused.

- LP HARQ-ACK with HP HARQ-ACK. In this case, CG-UCI can be jointly encoded with HP HARQ-ACK, and the agreed rule of multiplexing HP HARQ-ACK and LP HARQ-ACK on HP PUSCH can be directly reused.

CG-UCI on LP PUSCH:

- HP HARQ-ACK without LP HARQ-ACK. In this case, CG-UCI is regarded as LP HARQ-ACK, and the agreed rule of multiplexing HP HARQ-ACK and LP HARQ-ACK on LP PUSCH can be directly reused.

- HP HARQ-ACK with LP HARQ-ACK. In this case, CG-UCI is jointly encoded with LP HARQ-ACK, and the agreed rule of multiplexing HP HARQ-ACK and LP HARQ-ACK on LP PUSCH can be directly reused. Assuming the HP HARQ-ACK is highly reliable, the position of the CG-UCI would be deterministic.

Proposal 5: Adopt the following rules for multiplexing CG-UCI with HARQ-ACK of a different priority:

- For CG-UCI on HP PUSCH,

- LP HARQ-ACK without HP HARQ-ACK: CG-UCI is regarded as HP HARQ-ACK to reuse the rule of HP HARQ-ACK and LP HARQ-ACK on HP PUSCH that has been agreed.

- LP HARQ-ACK with HP HARQ-ACK: CG-UCI is jointly encoded with HP HARQ-ACK to reuse the rule of HP HARQ-ACK and LP HARQ-ACK on HP PUSCH that has been agreed.

- For CG-UCI on LP PUSCH,

- HP HARQ-ACK without LP HARQ-ACK: CG-UCI is regarded as LP HARQ-ACK to reuse the rule of HP HARQ-ACK and LP HARQ-ACK on LP PUSCH that has been agreed.

- HP HARQ-ACK with LP HARQ-ACK: CG-UCI is jointly encoded with LP HARQ-ACK to reuse the rule of HP HARQ-ACK and LP HARQ-ACK on LP PUSCH that has been agreed.

3.3 Prioritizations for transmission power reductions

For the current power control mechanism, UE should reduce the transmission power of PUSCH/PUCCH/PRACH/SRS according to the following order if the maximum power  would be exceeded. In particular, for PUCCH and PUSCH transmissions, the Rel-16 power allocation prioritization order is as follows, where the power reduction is performed in descending order.

- HP PUCCH/PUSCH with HARQ-ACK and/or SR

- HP PUSCH with HP CSI

- HP PUSCH only

- LP PUCCH/PUSCH with HARQ-ACK and/or SR

- LP PUCCH/PUSCH with CSI

- LP PUSCH only

Two basic principles can be observed from the list: 1) Any type of HP signal/channel always has higher priority than any type of the LP signal/channel. 2) For the signals/channels with the same priority index, the priority order is: PUCCH/PUSCH with HARQ-ACK and/or SR>PUCCH/PUSCH with CSI>PUSCH only, as HARQ-ACK is generally more important than CSI and data only.

In Rel-17, as intra-UE multiplexing of inter-priority UCI on PUSCH is introduced, new cases arise such as HP HARQ-ACK on LP PUSCH, and LP UCI on HP PUSCH. It is observed that the current priority list could not give the proper order for UCI-on-PUSCH with mixed priority.

As one example, for LP HARQ-ACK on HP PUSCH, this channel would be categorized to HP PUCCH/PUSCH with HARQ-ACK, which is the same as HP PUCCH/PUSCH with HP HARQ-ACK and/or HP SR and higher than HP PUSCH with HP A-CSI or HP PUSCH only. However, the LP HARQ-ACK should be obviously with lower priority than HP HARQ-ACK, HP SR, and HP A-CSI as per the above principle 1). As another example, for HP HARQ-ACK on LP PUSCH, this channel would be categorized to LP PUCCH/PUSCH with HARQ-ACK, which has lower priority than other HP channels during power allocation, but HP HARQ-ACK is obviously more critical than HP PUSCH only as per the above principle 2).

Considering the PUCCH/PUSCH with mixed priorities due to multiplexing, the power allocation principle should also be updated. Firstly, the power allocation order of LP PUSCH with HP HARQ-ACK should be the same as HP PUCCH/PUSCH with HARQ-ACK, i.e., higher than HP PUSCH with CSI as well as HP PUSCH only. Secondly, the power allocation order of HP PUSCH with LP HARQ-ACK should be the same as HP PUSCH only, i.e., lower than HP PUCCH/PUSCH with HP HARQ-ACK and/or HP SR, as well as HP PUSCH with CSI.

Proposal 6: Update the transmission power allocation order for Rel-17 by considering inter-priority UCI-on-PUSCH cases:

- LP PUSCH with HP HARQ-ACK should be of the same priority as HP PUSCH with HP HARQ-ACK, i.e., higher than HP PUSCH with CSI, as well as HP PUSCH only.

- LP HARQ-ACK on HP PUSCH should of the same priority as HP PUSCH only, i.e., lower than HP PUSCH with HP HARQ-ACK, as well as HP PUSCH with CSI.

Text Proposals

In this section, the following text proposals are provided for 38.213 v17.1.0 [2].

4.1 Multiplexing/prioritization when simultaneousPUCCH-PUSCH configured

The agreements on simultaneous transmission of PUCCH and PUSCH are shown in below.

Agreements: Support simultaneous PUCCH/PUSCH transmissions on different cells at least for inter-band CA. Agreement If the simultaneous PUCCH/PUSCH transmission is enabled, a PUSCH that can be simultaneously transmitted with a PUCCH is excluded from overlapping channels for multiplexing the UCI of the PUCCH and for intra-UE prioritization with the PUCCH. - Note: For intra-UE multiplexing, above is for step 2-2. For intra-UE prioritization, above is applied after step 1. - FFS: How to capture this in the specifications Conclusion If the simultaneous PUCCH/PUSCH transmission is enabled, the timeline conditions of intra-UE multiplexing/prioritization of PUCCHs and PUSCHs with different priorities is not applicable to a PUSCH that can be simultaneously transmitted with a PUCCH.

From the agreement and conclusion, it is agreed that the PUCCH/PUSCH simultaneous transmission is only performed for inter-band cells, and the timeline condition is only required to be met among the PUCCHs and PUSCHs within the band(s) that do not support simultaneous transmission. In addition, it has been agreed in the UE feature discussion that the capability of simultaneous PUCCH/PUSCH transmission is reported per band combination [3].

Agreement Type of FG 25-18 is per BC

From the UE perspective, it is straightforward to perform the inter-priority multiplexing/prioritization only within a group of bands that does not support simultaneous transmission and ignore the band(s) that support the simultaneous transmission from the multiplexing/prioritization procedure. However, the current spec requires the UE to first add all PUSCHs from all bands, then exclude the PUSCH on the band that does not support simultaneous transmission with the PUCCH. This ‘adding’ and ‘excluding’ behavior is unnecessary and complex for UE.

So a text proposal is given as follows.

Proposal 7: Adopt the following TP for TS38.213

------------------ Text Proposal for 38.213 v17.1.0 Clause 9 ------------------ - 9 UE procedure for reporting control information *** Unchanged text is omitted *** If a UE - is provided simultaneousPUCCH-PUSCH and would transmit a PUCCH with a first priority index and PUSCHs with a second priority index that is different than the first priority index, where the PUCCH and the PUSCHs overlap in time - can simultaneously transmit the PUCCH on the first cell and the PUSCHs on a set of second cells [16, TS 38.306], the UE excludes the PUSCHs for resolvesing the time overlapping between the PUCCH and PUSCHs on cells that do not belong to the set of second cells if any in the remaining of this clause, where the timeline conditions are not required for the PUCCH on the first cell and excluded PUSCHs on the set of second cells. *** Unchanged text is omitted ***

4.2 Multiplexing for Step 2.1

Issue 1. It has been agreed that the LP PUCCH is associated with the first overlapped subslot of HP PUCCH, and if the LP PUCCH is not multiplexed/dropped after the association with the first overlapped subslot, LP PUCCH would associate to the next HP PUCCH subslot. The current text separates this procedure into disconnected sub-bullets, which impacts its readability. Hence some changes are given by adjusting the order of the sub-bullets to make the procedure more clearly addressed.

Issue 2. If the UE does not multiplex or drop the LP PUCCH at the current HP time unit, it will be associated to the next HP time unit. In the current text, it says ‘the UCI of the first PUCCH transmission is not multiplexed in a second PUCCH transmission of larger priority index’, which is not clear what type of UCI it is. E.g., if the LP PUCCH includes HARQ-ACK and CSI, and LP HARQ-ACK is multiplexed into the current HP time unit while the CSI is dropped, then it is ambiguous whether the UCI is multiplexed or not. The correction is to replace this part with ‘no UCI of the first PUCCH transmission is multiplexed…’.

Issue 3. For the second but last sub-bullet, it is not clear whether ‘when the second priority index is larger than the first priority index’ is applied to only the latter part of the sentence (after ‘, or’) or both parts. If it is applied to only the latter part, then the two parts would redundantly include the case of overlapping with LP PUCCH with HARQ-ACK. The editorial suggestion is to change ‘first’/‘second’ priority index to ‘smaller’/‘larger’ priority index so that the former part and the latter part can describe orthogonal cases.

So a text proposal is given as follows

Proposal 8: Adopt the following TP for TS38.213

------------------ Text Proposal for 38.213 v17.1.0 Clause 9 ------------------ - 9 UE procedure for reporting control information *** Unchanged text is omitted *** - second, the UE resolves the overlapping for PUCCH transmissions of different priority indexes, and - if the UE is provided subslotLengthForPUCCH in the second PUCCH-Config, a first PUCCH transmission of smaller priority index is associated with the first overlapping slot with subslotLengthForPUCCH symbols of larger priority index; otherwise, the PUCCH transmission of smaller priority index is associated with the overlapping slot with Nsymslot symbols [4, TS 38.211] of larger priority index. - if the UE determines that a the first PUCCH transmission of the smaller priority index is not dropped and the no UCI of the first PUCCH transmission is not multiplexed in a second PUCCH transmission of larger priority index in an overlapping slot with subslotLengthForPUCCH symbols, the first PUCCH transmission is associated with the next overlapping slot with subslotLengthForPUCCH symbols for PUCCH transmissions with the larger priority index - for each overlapping slot after the association of the first PUCCH transmission, - the UE first resolves the overlapping for PUCCH transmissions, where at least one of the PUCCH transmissions is with NPUCCHrepeat>1 repetitions, within a slot of larger priority index as is subsequently described in this clause, if any, and then the UE resolves the overlapping for PUCCH transmissions without repetitions within the slot using the pseudo-code in clause 9.2.5 - if the UE determines that a first PUCCH transmission of the smaller priority index is not dropped and the UCI of the first PUCCH transmission is not multiplexed in a second PUCCH transmission of larger priority index in an overlapping slot with subslotLengthForPUCCH symbols, the first PUCCH transmission is associated with the next overlapping slot with subslotLengthForPUCCH symbols for PUCCH transmissions with the larger priority index - the UE does not expect a PUCCH transmission that includes UCI of different priority indexes to overlap with a PUCCH transmission with NPUCCHrepeat>1 repetitions after resolving the overlapping for PUCCH transmissions without repetitions within a slot - the UE does not expect a PUCCH transmission with UCI of first smaller and second larger priority indexes to overlap with a PUCCH transmission with HARQ-ACK information of the first smaller priority index, or with a PUCCH transmission or with a PUSCH transmission of the second larger priority index when the second priority index is larger than the first priority index - the UE does not expect a PUCCH transmission with HARQ-ACK information of larger priority index to overlap with more than one PUCCH transmissions with HARQ-ACK information of smaller priority index *** Unchanged text is omitted ***

4.3 Multiplexing for Step 2.2

In Step 2.2, the text “the UE drops PUSCH transmissions of smaller priority index that overlap with a PUCCH transmission with NPUCCHrepeat>1 repetitions of larger priority index prior to multiplexing UCI in a PUSCH transmission of smaller priority index, if any” addresses the behavior of dropping of repetition, which is redundant with a later paragraph as below.

- else - if the UE would transmit the following channels that would overlap in time where, if a channel transmission is with repetitions, the following are applicable per repetition - a first PUCCH transmission of larger priority index and a second PUCCH transmission of smaller priority index - a first PUCCH transmission of larger priority index and a second PUSCH transmission of smaller priority index when the UE cannot simultaneously transmit the first PUCCH and second PUSCH - a first PUCCH transmission of smaller priority index and a second PUSCH transmission of larger priority index when the UE cannot simultaneously transmit the first PUCCH and second PUSCH the UE - transmits the PUCCH or the PUSCH of the larger priority index, and - does not transmit a PUCCH or a PUSCH of smaller priority index

Thus a text proposal is given as follows.

Proposal 9: Adopt the following TP for TS38.213

------------------ Text Proposal for 38.213 v17.1.0 Clause 9 ------------------ - 9 UE procedure for reporting control information *** Unchanged text is omitted *** - third, the UE resolves the overlapping for PUCCH and PUSCH transmissions of different priority indexes - the UE drops PUSCH transmissions of smaller priority index that overlap with a PUCCH transmission with positive SR of larger priority index prior to multiplexing UCI in a PUSCH transmission of smaller priority index, if any - the UE drops PUSCH transmissions of smaller priority index that overlap with a PUCCH transmission with NPUCCHrepeat>1 repetitions of larger priority index as subsequently described in this clause prior to multiplexing UCI in a PUSCH transmission of smaller priority index, if any - the UE multiplexes HARQ-ACK information in a PUSCH transmission, as is subsequently described in this clause for multiplexing HARQ-ACK information from a PUCCH transmission in a PUSCH transmission of a same priority index, if a PUCCH transmission with HARQ-ACK information of a first priority index overlaps with one or more PUSCH transmissions of a second priority index that is different than the first priority index *** Unchanged text is omitted ***

Conclusions

According to the discussion, following proposals are provided:

Proposal 1: When a PUCCH carrying HP HARQ-ACK provided by n1PUCCH-AN overlaps with a PUCCH carrying LP HARQ-ACK with total number of HARQ-ACK bits more than 2 bits, the LP HARQ-ACK is dropped.

Proposal 2: When a PUCCH carrying HP SR and HP HARQ-ACK with PF0/1 overlaps with a PUCCH carrying LP HARQ-ACK:

- Option 2 is adopted, i.e., if HP HARQ-ACK is dynamic HARQ-ACK or SPS HARQ-ACK with sps-PUCCH-AN-List, 1-bit HP SR is appended to HP HARQ-ACK bits. The number of HP UCI bits is OUCI=OACK,1+1.

Proposal 3: For the case of HP SR with PF0 dropped due to collision with HP HARQ-ACK PF1 in Step 1, the resultant HP PUCCH after Step 1 is regarded as HP PUCCH with HARQ-ACK only when resolving the overlapping with LP HARQ-ACK in Step 2.

Proposal 4: For the scenario of multiplexing low-priority HARQ-ACK onto high-priority PUSCH, support Option 2, i.e. LP HARQ-ACKs are mapped to the rest REs of the PUSCH based on the rate matching equation, if HP HARQ-ACK and/or HP CSI have been mapped in prior on the PUSCH.

Proposal 5: Adopt the following rules for multiplexing CG-UCI with HARQ-ACK of a different priority:

- For CG-UCI on HP PUSCH,

- LP HARQ-ACK without HP HARQ-ACK: CG-UCI is regarded as HP HARQ-ACK to reuse the rule of HP HARQ-ACK and LP HARQ-ACK on HP PUSCH that has been agreed.

- LP HARQ-ACK with HP HARQ-ACK: CG-UCI is jointly encoded with HP HARQ-ACK to reuse the rule of HP HARQ-ACK and LP HARQ-ACK on HP PUSCH that has been agreed.

- For CG-UCI on LP PUSCH,

- HP HARQ-ACK without LP HARQ-ACK: CG-UCI is regarded as LP HARQ-ACK to reuse the rule of HP HARQ-ACK and LP HARQ-ACK on LP PUSCH that has been agreed.

- HP HARQ-ACK with LP HARQ-ACK: CG-UCI is jointly encoded with LP HARQ-ACK to reuse the rule of HP HARQ-ACK and LP HARQ-ACK on LP PUSCH that has been agreed.

Proposal 6: Update the transmission power allocation order for Rel-17 by considering inter-priority UCI-on-PUSCH cases:

- LP PUSCH with HP HARQ-ACK should be of the same priority as HP PUSCH with HP HARQ-ACK, i.e., higher than HP PUSCH with CSI, as well as HP PUSCH only.

- LP HARQ-ACK on HP PUSCH should of the same priority as HP PUSCH only, i.e., lower than HP PUSCH with HP HARQ-ACK, as well as HP PUSCH with CSI.

Proposal 7: Adopt the following TP for TS38.213

------------------ Text Proposal for 38.213 v17.1.0 Clause 9 ------------------ - 9 UE procedure for reporting control information *** Unchanged text is omitted *** If a UE - is provided simultaneousPUCCH-PUSCH and would transmit a PUCCH with a first priority index and PUSCHs with a second priority index that is different than the first priority index, where the PUCCH and the PUSCHs overlap in time - can simultaneously transmit the PUCCH on the first cell and the PUSCHs on a set of second cells [16, TS 38.306], the UE excludes the PUSCHs for resolvesing the time overlapping between the PUCCH and PUSCHs on cells that do not belong to the set of second cells if any in the remaining of this clause, where the timeline conditions are not required for the PUCCH on the first cell and excluded PUSCHs on the set of second cells. *** Unchanged text is omitted ***

Proposal 8: Adopt the following TP for TS38.213

------------------ Text Proposal for 38.213 v17.1.0 Clause 9 ------------------ - 9 UE procedure for reporting control information *** Unchanged text is omitted *** - second, the UE resolves the overlapping for PUCCH transmissions of different priority indexes, and - if the UE is provided subslotLengthForPUCCH in the second PUCCH-Config, a first PUCCH transmission of smaller priority index is associated with the first overlapping slot with subslotLengthForPUCCH symbols of larger priority index; otherwise, the PUCCH transmission of smaller priority index is associated with the overlapping slot with Nsymslot symbols [4, TS 38.211] of larger priority index. - if the UE determines that a the first PUCCH transmission of the smaller priority index is not dropped and the no UCI of the first PUCCH transmission is not multiplexed in a second PUCCH transmission of larger priority index in an overlapping slot with subslotLengthForPUCCH symbols, the first PUCCH transmission is associated with the next overlapping slot with subslotLengthForPUCCH symbols for PUCCH transmissions with the larger priority index - for each overlapping slot after the association of the first PUCCH transmission, - the UE first resolves the overlapping for PUCCH transmissions, where at least one of the PUCCH transmissions is with NPUCCHrepeat>1 repetitions, within a slot of larger priority index as is subsequently described in this clause, if any, and then the UE resolves the overlapping for PUCCH transmissions without repetitions within the slot using the pseudo-code in clause 9.2.5 - if the UE determines that a first PUCCH transmission of the smaller priority index is not dropped and the UCI of the first PUCCH transmission is not multiplexed in a second PUCCH transmission of larger priority index in an overlapping slot with subslotLengthForPUCCH symbols, the first PUCCH transmission is associated with the next overlapping slot with subslotLengthForPUCCH symbols for PUCCH transmissions with the larger priority index - the UE does not expect a PUCCH transmission that includes UCI of different priority indexes to overlap with a PUCCH transmission with NPUCCHrepeat>1 repetitions after resolving the overlapping for PUCCH transmissions without repetitions within a slot - the UE does not expect a PUCCH transmission with UCI of first smaller and second larger priority indexes to overlap with a PUCCH transmission with HARQ-ACK information of the first smaller priority index, or with a PUCCH transmission or with a PUSCH transmission of the second larger priority index when the second priority index is larger than the first priority index - the UE does not expect a PUCCH transmission with HARQ-ACK information of larger priority index to overlap with more than one PUCCH transmissions with HARQ-ACK information of smaller priority index *** Unchanged text is omitted ***

Proposal 9: Adopt the following TP for TS38.213

------------------ Text Proposal for 38.213 v17.1.0 Clause 9 ------------------ - 9 UE procedure for reporting control information *** Unchanged text is omitted *** - third, the UE resolves the overlapping for PUCCH and PUSCH transmissions of different priority indexes - the UE drops PUSCH transmissions of smaller priority index that overlap with a PUCCH transmission with positive SR of larger priority index prior to multiplexing UCI in a PUSCH transmission of smaller priority index, if any - the UE drops PUSCH transmissions of smaller priority index that overlap with a PUCCH transmission with NPUCCHrepeat>1 repetitions of larger priority index as subsequently described in this clause prior to multiplexing UCI in a PUSCH transmission of smaller priority index, if any - the UE multiplexes HARQ-ACK information in a PUSCH transmission, as is subsequently described in this clause for multiplexing HARQ-ACK information from a PUCCH transmission in a PUSCH transmission of a same priority index, if a PUCCH transmission with HARQ-ACK information of a first priority index overlaps with one or more PUSCH transmissions of a second priority index that is different than the first priority index *** Unchanged text is omitted ***

- References

- R1-2202830, Summary#3 of email thread [108-e-R17-IIoT-URLLC-04], Moderator (OPPO), RAN1#108-e, March 2022.

- TS 38.213, v17.1.0, Physical layer procedures for control, March 2022.

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #108-e, March 2022.