3GPP TSG-RAN WG1 Meeting #109-e			R1-2203077

e-Meeting, May 9th – 20th, 2022

Title:	Draft reply LS on PDCCH Blind Detection in CA

Response to:	R1-2202901(R2-2204055),

Release:	Rel-16

Work Item:	NR_L1enh_URLLC-Core

Source:	Huawei [TSG RAN WG1]

To:	TSG RAN WG2

Contact Person:

#### Name:	Yan Cheng

E-mail Address:	chengyan.cheng@huawei.com

1. Overall Description:

RAN1 thanks RAN2 for the Reply LS on PDCCH Blind Detection in CA. RAN1 discussed the questions in the LS and would like to provide feedback as below:

Question 1: Is UE required to report both pdcch-BlindDetectionMCG-UE-Mixed-r16 for MCG and pdcch-BlindDetectionSCG-UE-Mixed-r16 for SCG?

Answer to Question 1:

RAN1 confirms that for NR-DC operation with a mix of Rel-15 and Rel-16 PDCCH monitoring capability on different carriers, the UE is required to report both pdcch-BlindDetectionMCG-UE-Mixed-r16 for MCG and pdcch-BlindDetectionSCG-UE-Mixed-r16 for SCG.

- Note: This is also in-line with the Rel-15 NR-DC operation, in which both pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE in pdcch-BlindDetectionNRDC shall be reported.

Question 2: How to interpret it if the UE reports both of FG 11-2c and FG 11-2g, or both of FG 11-2a and FG 11-2f?

Answer to Question 2:

RAN1 would like to inform RAN2 the following:

- The UE is only required to report either FG 11-2a or FG11-2f.

- The UE is only required to report either FG 11-2c or FG11-2g.

Note that the above is aligned with RAN1 previous agreements also. Some further clarification as below is provided for RAN2 information:

- If UE only supports aligned span for all cases, UE can just report FG 11-2c with “aligned spans only”.

- If UE supports aligned span and non-aligned span with restriction, then UE can just report FG 11-2g.

- If UE supports aligned span and non-aligned span without restriction, then UE can just report FG 11-2c with “aligned spans and non-aligned spans”.

Based on the above answers to question 1 and question 2, RAN1 respectively ask RAN2 to update RAN2 specification accordingly.

2. Actions:

To RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into account and update RAN2 specification accordingly.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #110	22 – 26 August 2022			Meeting

TSG-RAN WG1 Meeting #111	14 – 18 November 2022	        Meeting