3GPP TSG-RAN WG1 Meeting #109-e	R1-2203079

e-Meeting, May 9-20, 2022

Agenda Item:	8.2.1

Source:	Huawei, HiSilicon, SIA

Title:	Remaining issue of initial access signals and channels for 52-71GHz spectrum

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss our views on the remaining issues of initial access including CORESET#0 configuration according to the agreements and feature leader summary [1][2] in RAN1#108e.

CORESET#0 configuration for 120/480/960 kHz SCS

The following Working Assumption related to CORESET#0 configuration for 120/480/960 kHz SCS has been achieved in RAN1#108-e.

Working assumption (RAN1 #108-e) - The following table is used for set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120}, {480, 480}, and {960, 960} kHz for FR2-2. - FFS: whether/how to define X, if defined, candidate values {56, 76} - Text Proposal #4-2D (for 38.213, Section 13) in section 3 of R1-2202503 is endorsed - Note: this working assumption can be revisited once RAN4 finalizes the further details of the channelization. Index; SS/PBCH block and CORESET multiplexing pattern; Number of RBs NRBCORESET; Number of Symbols NsymbCORESET; Offset (RBs). 0; 1; 24; 2; 0. 1; 1; 24; 2; 4. 2; 1; 48; 1; 0. 3; 1; 48; 1; 14. 4; 1; 48; 1; 28. 5; 1; 48; 2; 0. 6; 1; 48; 2; 14. 7; 1; 48; 2; 28. 8; 1; 96; 1; 0. 9; 1; 96; 1; X. 10; 1; 96; 2; 0. 11; 1; 96; 2; X. 12; 3; 24; 2; -20 if kSSB=0 -21 if kSSB > 0. 13; 3; 24; 2; 24. 14; 3; 48; 2; -20 if kSSB=0 -21 if kSSB > 0. 15; 3; 48; 2; 48.

However, in the latest specification of TS37.213v17.1.0, the RB offset values is -24 and -48 in the 13th and 15th rows which is not aligned with the agreement. Thus, we propose to adopt the following TP#1.

#### TP#1 for TS38.213

## ===========Start of TP#1 for TS 38.213 =========== - 13 UE procedure for monitoring Type0-PDCCH CSS sets ## *** Unchanged text is omitted *** Table 13-10A: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120} kHz, {480, 480} kHz, or {960, 960} kHz for FR2-2 Index; SS/PBCH block and CORESET multiplexing pattern; Number of RBs NRBCORESET; Number of Symbols NsymbCORESET; Offset (RBs). 0; 1; 24; 2; 0. 1; 1; 24; 2; 4. 2; 1; 48; 1; 0. 3; 1; 48; 1; 14. 4; 1; 48; 1; 28. 5; 1; 48; 2; 0. 6; 1; 48; 2; 14. 7; 1; 48; 2; 28. 8; 1; 96; 1; 0. 9. 10; 1; 96; 2; 0. 11. 12; 3; 24; 2; -20 if kSSB=0 -21 if kSSB>0. 13; 3; 24; 2; 24. 14; 3; 48; 2; -20 if kSSB=0 -21 if kSSB>0. 15; 3; 48; 2; 48. ## *** Unchanged text is omitted *** ===========End of TP#1 for TS 38.213 ===========

Proposal 1: Adopt TP#1 in Section 13 of TS 38.213.

In TS38.213 v17.1.0, the rows with indices of 9 and 11 in Table 13-10A are empty because the 2nd RB offset value for 96RB CORESET0 with multiplexing pattern 1 is not decided in RAN1#108e. The candidate values of RB offset X are {56, 76}. For CORESET#0 with 96 PRB with multiplexing pattern 1, it would be beneficial for gNB to transmit portion of discovery burst (e.g. SSB, Type0-PDCCH, RMSI PDSCH) with the same beam and as compact as possible in time and frequency domain to save LBT and beam switching overheads. Considering only frequency domain resource allocation type 1 is allowed for RMSI PDSCH, aligning SSB with the two extremities of CORESET#0 in the frequency domain frees up more contiguous RBs and allows a more flexible and potentially a larger allocation for RMSI PDSCH. Aligning SSB with the two extremities of CORESET#0 in the frequency domain is even more important in 480/960 kHz as the beam switching delay may be too large to be absorbed in CP in 480/960 kHz SCS and, therefore, it is critical to have the possibility of transmitting SSB and its associated RMSI PDCCH/PDSCH in contiguous set of time/frequency resources without a need for a switching gap. Thus, in addition to the RB offset value of “0” in rows with indices of 8 and 10, we propose to choose 76 RB as the 2nd RB offset value for 96RB CORESET0.

In RAN1 108-e, some companies expressed a concern that, assuming the RB offset of {0 RB, 76 RB}, it is not able to accommodate 96RB COREST0 within the channel bandwidth corresponding to some ARFCNs of the assumed channel raster.  Therefore, the smaller RB offset value such as 56 RB is recommended. According to latest sync raster (GSCN) and channel raster (ARFCN) for band n264 endorsed in RAN4#102-e [4][5], we analyze the percentage of channel raster in n264 on which there is at least one valid GSCN and at least one RB offset value allowing 96RB COREST0 fully contained within the channel bandwidth. An illustrative example of valid GSCN and COREST0 combination with RB offset value is provided in Figure 1. The channel raster and sync raster agreed in RAN4 are reproduced in Table 1 and Table 2. We assume spectral utilization of 90% and 95% due to lack of consensus in RAN4. We do not evaluate 960kHz SSB because it is not expected in initial access. From the analysis result shown in Table 3, we observe that 96RB CORESET0 can be accommodated with RB offset value of either {0, 56} or {0,76} for all valid ARFCNs in the frequency range from 66GHz to 71GHz. Only 8% outage occur for RB offset value of {0, 76} when 480KHz SSB is configured within 800MHz channel bandwidth and 90% SU is assumed.

Figure 1 Example of {56RB, 76RB} configuration for {SS/PBCH block, PDCCH}={480, 480} kHz when 96 RB CORESET#0 is configured.

Table 1	Applicable NR-ARFCN for band 66~71 GHz

| Operating Band | ΔFRaster (kHz) | Uplink and Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- |
| n264 | 120 | 2563333 <2> 2794999 |
|  | 480 | 2565835 <8> 2792499 |
|  | 960 | 2565835 <16> 2792491 |

Table 2	Applicable SS raster entries for 66~71 GHz

| NR Operating Band | SS Block SCS | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- |
| n264 | 120 kHz | 24674 <3> 24959 |
|  | 480 kHz | 24677 <12> 24953 |
|  | 960 kHz | 24680 <6> 24950 |

Table 3. Percentage of channel raster able to accommodate 96 RB COREST0

| SCS | Bandwidth | CORSET#0 | spectral utilization | RB RB offsets | Percentage of channel raster |
| --- | --- | --- | --- | --- | --- |
| 120 kHz | 400 MHz | (96 RB, Multiplexing pattern 1) | 95% | 0 RB, 56RB | 100% (38500/38500 cases) |
|  |  |  |  | 0 RB, 76RB | 100% (38500/38500 cases) |
|  |  |  | 90% | 0 RB, 56RB | 100% (38667/38667 cases) |
|  |  |  |  | 0 RB, 76RB | 100% (38667/38667 cases) |
| 480 kHz | 800 MHz | (96 RB, Multiplexing pattern 1) | 95% | 0 RB, 56RB | 100% (8834/8834 cases) |
|  |  |  |  | 0 RB, 76RB | 100% (8834/8834 cases) |
|  |  |  | 90% | 0 RB, 56RB | 100% (8917/8917 cases) |
|  |  |  |  | 0 RB, 76RB | 91.75% (8182/8917 cases) |
|  | 1600 MHz | (96 RB, Multiplexing pattern 1) | 95% | 0 RB, 56RB | 100% (7250/ 7250 cases) |
|  |  |  |  | 0 RB, 76RB | 100% (7250/ 7250 cases) |
|  |  |  | 90% | 0 RB, 56RB | 100% (7417/ 7417 cases) |
|  |  |  |  | 0 RB, 76RB | 100% (7417/ 7417 cases) |

Considering the scheduling efficiency for SIB1 and deployment flexibility of SSB and component carrier, we propose to adopt 76 RB as the 2nd RB offset value for 96 RB CORESET0 and multiplex pattern 1.

Proposal 2: For CORESET#0 with 96 RBs and multiplex pattern 1, support RB offset of 76 RB for multiplexing pattern 1 when {SSB, CORESET#0} SCS={120, 120}, {480,480}, {960,960} kHz.

Conclusions

In this paper, we discuss the issues related to initial access. The following proposals are made.

Proposal 1: Adopt TP#1 in Section 13 of TS 38.213.

Proposal 2: For CORESET#0 with 96 RBs and multiplex pattern 1, support RB offset of 76 RB for multiplexing pattern 1 when {SSB, CORESET#0} SCS={120, 120}, {480,480}, {960,960} kHz.

- References

- R1-2202502, “Summary #2 of email discussion on initial access aspect of NR extension up to 71GHz”, Moderator (Intel Corporation)

- R1-2202683, “Summary #3 of email discussion on initial access aspect of NR extension up to 71GHz”, Moderator (Intel Corporation)

- Chair’s Notes RAN1#108-e

- R4-2206607, “Draft CR to TS 38.101-2: Channel arrangement”, Ericsson

- R4-2206582, “Draft CR to TS 38.104: Channel arrangement”, Ericsson