3GPP TSG-RAN WG1 Meeting #109-e                                                                                    R1-2203081

e-Meeting, May 9th – May 20th, 2022

Agenda Item:	8.2.3

Source:	Huawei, HiSilicon, SIA

Title:	Remaining issues of PDSCH/PUSCH enhancement for 52-71GHz spectrum

Document for:	Discussion and Decision

Introduction

This contribution discusses the remaining issues of multiple PDSCH/PUSCH scheduling and HARQ enhancement left after RAN1#108-e[1][2]. Furthermore, the discussion on the down-selection of application of TCI states during the span of multiple PDSCHs leftover in the beam management agenda item [3] is also covered in this paper.

Remaining issues on multiple PDSCH/PUSCH scheduled by single DCI

Clarification on “scheduling” PDSCH/PUSCH in early agreements

After RAN1#108-e meeting, the following cases are pending on whether “scheduled PDSCH” or “scheduled PUSCH” in early agreement corresponding to valid SLIV or configured SLIV:

Case 5: For out-of-order scheduling, is the rule for OOO scheduling determined based on configured SLIVs or valid SLIVs? Case 6: For a first DCI scheduling multiple PDSCHs and providing an inapplicable value of k1 in its PDSCH-to-HARQ_feedback timing indicator filed, to multiplex the corresponding HARQ-ACK information in a PUCCH or PUSCH in a slot indicated by the PDSCH-to-HARQ_feedback timing indicator filed in a second DCI.

We prefer a common solution for both case 5 and case 6 because they are all related to out of order issue. The former is for the DCI to PDSCH while the latter is for the PDSCH to HARQ. In previous meetings, we preferred to interpret the “scheduled” PDSCH/PUSCH as “valid” PDSCH/PUSCH. It provides more scheduling flexibility considering that the invalid SLIV(s) does not impact the order of scheduling.

At the same time, we also acknowledged the issue raised in [4][5] on the type 1 HARQ codebook generation. How UE determines the valid associated PDSCHs with the candidate PDSCH reception occasion corresponding to the last configured SLIV is not clear, when more than one row of TDRA have overlapped last SLIV and they are not valid. There are several alternatives mentioned during the discussion

Alt 1: Allow AND operation across valid PDSCHs scheduled by different DCI

Alt 2: Determine DCI-to-PDSCH OOO based on configured SLIV

Alt 3: Disallow TDRA rows with last SLIV invalid

Alt 4: Prune the rows in TDRA table based on the last valid SLIV

For Alt 1, it requires change of the current pseudo code for type 1 HARQ codebook generation because it is written assuming that the binary AND operation is across the PDSCH(s) scheduled by single DCI. It is also the common understanding when RAN1 reached agreement on the time bundling for type 1 HARQ codebook in RAN1#107-e. If the HARQ-ACK for eMBB PDSCHs scheduled by DCI 1-1 are bundled with HARQ-ACK feedback of URLLC PDSCH scheduled by DCI 1-2, it may cause unnecessary retransmission of URLLC PDSCH(s) due to transmission failure of eMBB PDSCH. So, it is beneficial to clarify the early agreement on the time bundling for type 1 HARQ codebook in RAN1#107-e that logical AND operation is applied across all valid PDSCHs scheduled by the same DCI associated with a determined candidate PDSCH reception occasion.

Proposal 1: It should be clarified that logical AND operation is applied across all valid PDSCHs scheduled by the same DCI associated with a determined candidate PDSCH reception occasion when time bundling is applied for type 1 HARQ codebook generation.

For Alt 2 and Alt 3, both of the alternatives resolve the ambiguity of determination of PDSCH receptions association with the candidate PDSCH reception occasion at the cost of scheduling flexibility. Alt 3 has been discussed in RAN1#107b-e. A conclusion was made to remove the FFS point that there is no requirement for gNB to ensure the last configured SLIV should be always valid. For Alt 4, it can resolve the issue without sacrificing the flexibility. However, it needs to revert agreement in RAN1#107-e that “Pruning procedure in Rel-16 is performed based on the last configured SLIV of each row index.” Taking all above into account, we can accept to specify the OOO and timeline of NNK1 based on the configured SLIV due to its less standard impact and no need to revert early agreements.

Agreement

- The case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) schedulings have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV, is considered as out-of-order scheduling and is not expected by UE.

  - This applies also when one of two DCIs is single-PDSCH (or single-PUSCH) scheduling DCI, including the case that one DCI schedules multi-slot PDSCH (or PUSCH repetition type A or B).

  - Note: This doesn’t apply when each of two DCIs schedules multi-slot PDSCH (or PUSCH repetition type A or B) as in Rel-15/Rel-16

  - Note: This doesn’t apply when each of the two DCIs schedules single PDSCH (or single PUSCH) as in Rel-15/Rel-16

- Note: It is separately discussed whether the scheduled SLIV is based on configured SLIV or valid SLIV.

Agreement

- The case where two multi-PDSCH (or multi-PUSCH) scheduling DCIs end in the same symbol but two multi-PDSCH (or multi-PUSCH) schedulings have overlapping spans, where the span is defined from the beginning of the first scheduled SLIV till the end of the last scheduled SLIV, is considered as out-of-order scheduling and is not expected by UE.

  - This applies also when one of two DCIs is single-PDSCH (or single-PUSCH) scheduling DCI, including the case that one DCI schedules multi-slot PDSCH (or PUSCH repetition type A or B).

  - Note: This doesn’t apply when each of two DCIs schedules multi-slot PDSCH (or PUSCH repetition type A or B) as in Rel-15/Rel-16

  - Note: This doesn’t apply when each of the two DCIs schedules single PDSCH (or single PUSCH) as in Rel-15/Rel-16

- Note: It is separately discussed whether the scheduled SLIV is based on configured SLIV or valid SLIV.

Moreover, there was another agreement achieved in RAN1#108-e on multiple PDSCH/PUSCH scheduling which leaves the issue of configured SLIV vs. valid SLIV to be separated discussed. We think this issue can be categorized into OOO issue and the scheduled SLIV in the agreement is based on the configured SLIV

Proposal 2: Support to clarify that “scheduled” PDSCH/PUSCH is the “configured” PDSCH/PUSCH in DCI-to-PDSCH OOO (case 5) and timeline of NNK1 (case 6).

Application of TCI states within the span of multi-PDSCH

When unified TCI framework is taken into consideration in multi-PDSCH scheduling, the following two alternatives in RAN1 108-e should be discussed regarding the applied TCI states:

Agreement TP#1 below is endorsed for TS 38.214 - RAN1 continues discussion to down-select between the following Alt1 and Alt2. - Alt1) The applied TCI states can be updated using unified TCI framework within the span of multi-PDSCH - Alt2) The applied TCI states cannot be updated using unified TCI framework within the span of multi-PDSCH - Note: coordination with Rel-17 MIMO WI is allowed as necessary TP#1 5.1.5 Antenna ports quasi co-location *** Unchanged text omitted *** When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH or the UE is configured with higher layer parameter [pdsch-TimeDomainAllocationListForMultiPDSCH-r17], the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH(s), and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH(s). When the UE is configured with CORESET associated with a search space set for cross-carrier scheduling and the UE is not configured with enableDefaultBeamForCCS, the UE expects tci-PresentInDCI is set as 'enabled' or tci-PresentDCI-1-2 is configured for the CORESET, and if one or more of the TCI states configured for the serving cell scheduled by the search space set contains qcl-Type set to 'typeD', the UE expects the time offset between the reception of the detected PDCCH in the search space set and a corresponding PDSCH is larger than or equal to the threshold timeDurationForQCL. *** Unchanged text omitted ***.

In Alt2, the unified TCI state cannot be applied during the time span of a multi-PDSCH scheduling. However, as the unified TCI state is not applied to the scheduled PDSCH, but applied after a certain period, such unnecessary restriction should not be imposed to gNB. The gNB can change or keep the applied beam flexibly by updating the corresponding TCI states for Rel-17 unified TCI framework without any limitation. Therefore, we propose the following:

Proposal 3: The applied TCI states can be updated using unified TCI framework within the span of multi-PDSCH (Alt1).

Remaining issues on HARQ enhancement

According to the current specification, the type 1 HARQ codebook generation procedure does not cover the case of PDSCH repetition scheduled by DCI 1-2 when time domain bundling is configured. According to agreement in RAN1#107e, it is possible to configure multiple PDSCH scheduled by single DCI and pdsch-AggregationFactor for DCI format 1_2. During the email discussion in RAN1#108e, most of the companies still hope to maintain such flexible when time domain bundling is configured. The divergence is between the yellow highlighted part of following TPs captured in feature leader summary of [2].

---------------------------------start of TP#1 for TS 38.213 Clause 9.1.2.1-------------------------------------------

9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

****Unchanged Text Omitted****

“elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot n0,k+nD- ∆K0,r(d), at least one symbol of the PDSCH time resource derived by row r of set R' is configured as UL, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r and for each slot from n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r of set R is configured as UL if the row r of set R belongs to time domain resource allocation table configured for DCI format 1_2”

****Unchanged Text Omitted****

---------------------------------end of TP#1 for TS 38.213 Clause 9.1.2.1-------------------------------------------

---------------------------------start of TP#2 for TS 38.213 Clause 9.1.2.1-------------------------------------------

9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

****Unchanged Text Omitted****

“elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot n0,k+nD- ∆K0,r(d), at least one symbol of the PDSCH time resource derived by row r of set R' is configured as UL, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r and for each slot from n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r of set R is configured as UL if the row r of set R includes a single SLIV,”

****Unchanged Text Omitted****

---------------------------------end  of TP#2 for TS 38.213 Clause 9.1.2.1-------------------------------------------

Agreement

- If a UE is configured with a TDRA table in which one or more rows contain multiple SLIVs for PDSCH for DCI format 1_1, the UE does not expect to be configured with repetitionNumber for the TDRA table, and if pdsch-AggregationFactor is configued in PDSCH-config, it does not apply to DCI format 1_1.

  - Note: repetitionNumber cannot be configured with pdsch-TimeDomainAllocationListDCI-1-2 as in Rel-16.

  - Note: Under agenda item 8.2.4, in RAN1#106-bis, it was already agreed that within the TDRA table for multi-PDSCH scheduling, the UE does not expect to be configured with the higher layer parameter repetitionNumber.

  - Note: These does not preclude pdsch-AggregationFactor can be configured and applies to DCI format 1_2

- If a UE is configured with a TDRA table in which one or more rows contain multiple SLIVs for PUSCH for DCI format 0_1, the UE does not expect to be configured with numberOfRepetitions for the TDRA table, and if pusch-AggregationFactor is configued in PUSCH-config, it does not apply to DCI format 0_1.

  - Note: These does not preclude numberOfRepetitions is configured for TDRA table corresponding to DCI format 0_2

  - Note: These does not preclude pusch-AggregationFactor can be configured and applies to DCI format 0_2

Agreement

- If a UE is configured with a TDRA table in which one or more rows contain multiple SLIVs for PDSCH for DCI format 1_1, the UE does not expect to be configured with repetitionNumber for the TDRA table, and if pdsch-AggregationFactor is configued in PDSCH-config, it does not apply to DCI format 1_1.

  - Note: repetitionNumber cannot be configured with pdsch-TimeDomainAllocationListDCI-1-2 as in Rel-16.

  - Note: Under agenda item 8.2.4, in RAN1#106-bis, it was already agreed that within the TDRA table for multi-PDSCH scheduling, the UE does not expect to be configured with the higher layer parameter repetitionNumber.

  - Note: These does not preclude pdsch-AggregationFactor can be configured and applies to DCI format 1_2

- If a UE is configured with a TDRA table in which one or more rows contain multiple SLIVs for PUSCH for DCI format 0_1, the UE does not expect to be configured with numberOfRepetitions for the TDRA table, and if pusch-AggregationFactor is configued in PUSCH-config, it does not apply to DCI format 0_1.

  - Note: These does not preclude numberOfRepetitions is configured for TDRA table corresponding to DCI format 0_2

  - Note: These does not preclude pusch-AggregationFactor can be configured and applies to DCI format 0_2

We think TP#2 implies that it is possible to schedule PDSCH with slot aggregation by the single SLIV entry in DCI 1-1 configured with PDSCH-TimeDomainResourceAllocationListForMultiPDSCH, which is already excluded by the highlighted part of the agreement in RAN1#107e.

Proposal 4: Support configuring enableTimeDomainHARQ-Bundling for type 1 HARQ codebook feedback for multiple PDSCHs scheduled by single DCI 1-1 and pdsch-AggregationFactor for PDSCH scheduled by DCI 1-2 at same time. TP#1 is preferred.

Conclusion

According to the discussion in the paper, we had following proposals.

Proposal 1: It should be clarified that logical AND operation is applied across all valid PDSCHs scheduled by the same DCI associated with a determined candidate PDSCH reception occasion when time bundling is applied for type 1 HARQ codebook generation.

Proposal 2: Support to clarify that “scheduled” PDSCH/PUSCH is the “configured” PDSCH/PUSCH in DCI-to-PDSCH OOO (case 5) and timeline of NNK1 (case 6).

Proposal 3: The applied TCI states can be updated using unified TCI framework within the span of multi-PDSCH (Alt1).

Proposal 4: Support configuring enableTimeDomainHARQ-Bundling for type 1 HARQ codebook feedback for multiple PDSCHs scheduled by single DCI 1-1 and pdsch-AggregationFactor for PDSCH scheduled by DCI 1-2 at same time. TP#1 is preferred.

Reference

- R1-2202560, “Discussion summary #1 of [108-e-NR-52-71GHz-05]”, Moderator (vivo)

- R1-2202796, “Summary #3 of PDSCH/PUSCH enhancements (Scheduling/HARQ)”, Moderator (LG)

- R1-2201036, “Discussion Summary #1 for Beam Management for new SCSs”, Moderator (InterDigital)

- R1-2202074, “Remaining discussion on multi-PDSCH scheduling design for 52.6-71 GHz NR operation”, Mediatek

- R1-2201436, “Remaining issues of multi-PDSCH scheduling via a single DCI”, Fujitsu