3GPP TSG-RAN WG1 Meeting #109-e	R1-2203082

e-Meeting, May 9 – May 20, 2022

Agenda Item:	8.2.4

Source:	Huawei, HiSilicon, SIA

Title:	Remaining issues of channel access mechanism for 60 GHz unlicensed operation

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss the remaining issues for the channel access mechanisms for operation in the 60 GHz unlicensed band in light of the latest agreements and email discussions. Some TPs are provided targeting respective clauses of the latest version of Rel-17 specifications.

Discussion

Higher layer parameters

Value ranges for cg-COT-Sharing-r17 and cg-COT-SharingList-r17

In RAN1#107-e [1], it was agreed to extend the duration and offset range to {1, …, 319} for CG-PUSCH to DL COT sharing as follows. The agreed value range accounted for the maximum COT duration of 5 ms and the largest SCS of 960 kHz.

Agreement For CG-PUSCH to DL COT sharing, extend the duration and offset range to {1, …, 319}.

Although the agreed value range was provided to RAN2 among the consolidated higher layer parameters list for Rel-17 NR in R1-2112976, the latest RAN2 running CR on the RRC parameters in R2-2404126 does not capture the agreed value range for the duration-r17 and offset-r17 in cg-COT-Sharing-r17 as highlighted below

ConfiguredGrantConfig information element

-- ASN1START

-- TAG-CONFIGUREDGRANTCONFIG-START

…

cg-COT-SharingList-r16                  SEQUENCE (SIZE (1..1709)) OF CG-COT-Sharing-r16             OPTIONAL,   -- Need R

…

cg-COT-SharingList-r17                  SEQUENCE (SIZE (1..1709)) OF CG-COT-Sharing-r17             OPTIONAL,   -- Need R

…

CG-COT-Sharing-r16 ::= CHOICE {

noCOT-Sharing-r16                   NULL,

cot-Sharing-r16                     SEQUENCE {

duration-r16                       INTEGER (1..39),

offset-r16                         INTEGER (1..39),

channelAccessPriority-r16          INTEGER (1..4)

}

}

CG-COT-Sharing-r17 ::=  CHOICE {

noCOT-Sharing-r17                   NULL,

cot-Sharing-r17                     SEQUENCE {

duration-r17                       INTEGER (1..139),

offset-r17                         INTEGER (1..139)

}

}

…

-- TAG-CONFIGUREDGRANTCONFIG-STOP

-- ASN1STOP

Furthermore, the value range for the size of the cg-COT-SharingList-r17 is neither aligned with the current value range for duration-r17 and offset-r17 (139) nor is it aligned with the RAN1 agreed value range (319). Considering the current value range for duration-r17 and offset-r17 (139), the value range for the size of the cg-COT-SharingList-r17 would be 139*138/2 +1 = 9592. Whereas considering the RAN1 agreed value range for duration-r17 and offset-r17 (319), the value range for the size of the cg-COT-SharingList-r17 would be 319*318/2 +1 = 50722. Nevertheless, RAN1 has not discussed the value range for the size of the cg-COT-SharingList-r17 and thus has not included it in the table of parameters in R1-2112976 or its subsequent updates.

Since the April ASN.1 review meeting was not expected to correct WI-specific issues, we propose to send an LS to RAN2 raising these issues.

Proposal 1: RAN1 to send an LS to RAN2 to correct the value range for duration-r17 and offset-r17 in cg-COT-Sharing-r17 to 319 as per the agreement in RAN1#107-e, and to correct the value range for the size of the cg-COT-SharingList-r17 to 50,722.

TCI state for inter-frequency L3-RSSI measurement

In RAN1#108-e [2], the following agreement was achieved regarding the L3-RSSI measurement beam for unlicensed operation in FR2-2:

Agreement: For the QCL Type-D of L3-RSSI measurement for unlicensed operation in FR2-2, if explicit TCI state is configured, use the TCI state. - Use the QCL type-D of the latest PDSCH reception or latest CORESET monitoring for RSSI measurement, if the explicit TCI state is not configured. - A dynamic update mechanism for TCI-State in RMTC-Config is not further considered in Rel.17 - The explicit TCI state is configured at least in RMTC-Config - Note: For inter-frequency L3-RSSI measurement, the TCI state configured is with respect to the target frequency TCI state - Note2: For a given L3-RSSI measurement occasion, the UE needs to identify the last PDSCH reception or last configured CORESET monitoring (which ever is later) before the L3-RSSI measurement occasion, and use the QCL Type-D of that for L3-RSSI monitoring

It is understood that using the QCL type-D of the latest PDSCH reception or latest CORESET monitoring is similar to the procedure for the CLI-RSSI measurements which are only applicable for the intra-frequency case defined as “when CLI-RSSI measurement resource is configured within active BWP” [9.7.1, TS 8.133] as captured below.

9.7.1 Introduction The UE capable of performing CLI measurements shall be able to measure SRS-RSRP and CLI-RSSI which are defined in TS38.215 [4] within active DL BWP. The measurements requirements in this clause apply for TDD mode only. CLI measurements are only applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is fully confined within BW of DL active BWP - when CLI-RSSI measurement resource is configured within active BWP

However, such a definition of intra-frequency measurement is different from the following definition of intra-frequency RSSI/CO measurements in Rel-16 which implies that even for intra-frequency RSSI, it is still possible that the measurement resource is outside of the active BWP.

9.2A.7 Intra-frequency RSSI and Channel occupancy measurements 9.2A.7.1 Intra-frequency RSSI measurements An RSSI measurement is defined as an intra-frequency measurement provided that the RSSI measurement bandwidth is fully contained within the current carrier bandwidth of the UE.

Therefore, the definition of ‘inter-frequency’ for L3-RSSI measurement in FR2-2 should be confirmed as applicable when the RSSI measurement bandwidth is fully contained within the current carrier bandwidth of the UE, unlike CLI-RSSI measurement.

Furthermore, in the case of ‘inter-frequency’ L3-RSSI measurement, if the measurement resource is not within any active BWP of any serving cell, then what would be the proper interpretation of ‘target frequency TCI state’? Given the fact that TCI states are currently defined per BWP of a serving cell and indicated within the BWP via the DL RS index, we propose that the spatial domain filter for the L3-RSSI measurement on the target frequency in such a case should be the same as the spatial domain filter associated with the TCI state of the active BWP in which the RMTC configuration is provided.

Finally, if the TCI state is not configured in RMTC-Config when the measurement resource is not within any active BWP of any serving cell, it is not clear whether the UE shall use the same spatial domain filter for the latest PDSCH or latest CORESET reception across all serving cells or a specific serving cell. In our view, it makes more sense in such a case to consider the serving cell on which the RMTC configuration is provided.

As such, the highlighted note on configuring the TCI state in the case of ‘inter-frequency’ L3-RSSI measurement in the above RAN1 agreement remains ambiguous, and we thus propose the following:

Proposal 2: RAN1 to consider the following for L3-RSSI measurement

- Confirm that the ‘inter-frequency L3-RSSI measurement’ applies when the RSSI measurement bandwidth is fully contained within the current carrier bandwidth

- For the ‘target frequency TCI state’ if the measurement resource is not within any active BWP of any serving cell,

  - the spatial domain filter for the L3-RSSI measurement on the target frequency should be the same as the spatial domain filter associated with the TCI state of the BWP in which the RMTC configuration is provided,

  - if no TCI state is provided in the RMTC configuration, the spatial domain filter for the L3-RSSI measurement on the target frequency should be the same as the spatial domain filter for one of the latest PDSCH reception and the latest CORESET monitoring in the active BWP of the carrier on which the RMTC configuration is provided.

Proposal 3: Adopt following TP#0 into TS 38.133 v15.7.0.

Proposal 4: RAN1 to send an LS to RAN4 capturing the clarifications and TP#1 if agreed.

## *** < Beginning of TP#0 for TS 38.133 v17.5.0> *** ### 9.2A.7 Intra-frequency RSSI and Channel occupancy measurements #### 9.2A.7.1 Intra-frequency RSSI measurements An RSSI measurement is defined as an intra-frequency measurement provided that the RSSI measurement bandwidth is fully contained within the current carrier bandwidth of the UE. The UE physical layer shall be capable of performing the RSSI measurements, defined in TS 38.215 [4] on one or more serving carriers operating with CCA, TS 37.213 [33], if the carrier(s) are indicated by higher layers [2], and report the RSSI measurements to higher layers. The UE physical layer shall provide to higher layers a single RSSI sample for each OFDM symbol within each configured RSSI measurement duration [2] occurring with a configured RSSI measurement timing configuration periodicity [2], rmtc-Periodicity. For performing RSSI measurement in FR2-2, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration. If no TCI state is provided in the RMTC configuration, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of the current carrier. *** < Unchanged parts are ommitted> *** ### 9.3A.8 Inter-frequency RSSI measurements An RSSI measurement is defined as an inter-frequency measurement provided that the RSSI measurement bandwidth is not contained within the current carrier bandwidth of the UE. The UE physical layer shall be capable of performing the RSSI measurements, defined in TS 38.215 [4] on one or more inter-frequency carriers operating with CCA, TS 37.213 [33], if the carrier(s) are indicated by higher layers [2], and report the RSSI measurements to higher layers. The UE physical layer shall provide to higher layers a single RSSI sample for each OFDM symbol within each configured RSSI measurement duration [2] occurring with a configured RSSI measurement timing configuration periodicity [2], rmtc-Periodicity. The requirements apply if rmtc-SubframeOffset [2] is configured. For performing RSSI measurement in FR2-2, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state provided in the RMTC configuration. If the configured RSSI measurement resources are not confined within the bandwidth of any serving cell, UE can assume that the measurement resources are QCL-ed with TypeD to the DL RS associated with the TCI state of the active BWP of the carrier on which the RMTC configuration is provided. If no TCI state is provided in the RMTC configuration, UE can assume the configured RSSI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET in the active BWP of the carrier on which the RMTC configuration is provided. *** < End of TP#0 for TS 38.133 v17.5.0> ***

Clarifications to the energy detection threshold

In RAN1#107-e [1], the following agreement was made to clarify the WA agreed in RAN1#104bis-e [3]:

Agreement Confirm the following working assumption with some clarifications - For Pout in EDT determination, define Pout as the maximum EIRP of the intended transmissions by the node determining EDT during a COT. - The node is not expected to transmit in the COT with higher Pout than the Pout used to determine the EDT used to acquire the COT

Our understanding is that ‘the node determining EDT’ is not restricted to an initiating node. The EDT determination mechanism can be used by a gNB/UE as an initiating node or as a responding node, i.e., if Type 2 is used during the COT. This is the reason that the main bullet of the above agreement on Pout definition recites “by the node determining EDT during a COT”.

The agreement has been captured as follows in the latest version of TS 37.213 v17.1.0

### 4.4.7 Energy detection threshold adaptation procedures A gNB/UE accessing a channel on which transmission(s) on beam(s) are performed within a channel occupancy, shall set the energy detection threshold XThresh to be less than or equal to the maximum energy detection threshold XThresh_max that is determined as follows: XThresh_max=-80dBm+Pmax- Pout+ 10⋅log10(BW) where: - Pmax is the RF output power limit in dBm. - Pout is the maximum EIRP of the intended transmission(s) by the gNB/UE to acquire a channel occupancy in dBm where Pout≤Pmax. The maximum EIRP used for the transmission(s) by the initiating gNB/UE during the channel occupancy is limited to Pout. - BW is the channel bandwidth in MHz.

However, the current wording “to acquire a channel occupancy” restricts the use of the EDT to an initiating gNB/UE only. Therefore, we propose to clarify the definition of Pout as shown in following TP#1.

Proposal 5: For operation in FR2-2, clarify in the current specifications that the EDT determination mechanism is not restricted to a node initiating a COT. The node determining EDT could be also a responding node.

The case of determining the EDT when initiating a multi-beam COT was also discussed in the previous RAN1#107bis-e meeting. Two alternatives were identified for defining Pout for each sensing beam when independent per-beam LBTs are performed as captured by the following Discussion 2.2-1 in [4]

Discussion 2.2-1

For a COT with MU-MIMO (SDM) transmission or TDM transmission of beams with beam switching, when independent per-beam LBT is performed at the start of the COT, for Pout in EDT determination of LBT for each sensing beam:

- Alt 1: For Pout in EDT determination for a sensing beam, define Pout as the maximum EIRP of all intended transmissions by the node determining EDT during a COT

- Alt 2: For Pout in EDT determination for a sensing beam, define Pout as the maximum EIRP of the intended transmissions “covered” by the sensing beam by the node determining EDT during a COT

Aligned with the majority views, we think that Alt 2 can realize the benefits of directional LBT and justifies the additional complexity of performing multiple independent per-beam LBTs. In contrast, Alt1 is unnecessarily more conservative by accounting for the EIRP of transmission(s) not intended for the sensing beam in consideration for EDT determination.

Proposal 6: For operation in FR2-2, when independent per-beam LBT is performed at the start of the COT, define Pout for each sensing beam as the maximum EIRP of the intended transmissions “covered” by the sensing beam by the node determining EDT during a COT.

Proposal 7: For operation in FR2-2, adopt following TP#1 for TS 37.213 v17.1.0

## *** < Beginning of TP#1 for TS 37.213 v17.1.0> *** ### 4.4.7 Energy detection threshold adaptation procedures A gNB/UE accessing a channel on which transmission(s) on beam(s) are performed within a channel occupancy, shall set the energy detection threshold XThresh to be less than or equal to the maximum energy detection threshold XThresh_max that is determined as follows: XThresh_max=-80dBm+Pmax- Pout+ 10⋅log10(BW) where: - Pmax is the RF output power limit in dBm. - Pout is the maximum EIRP of the intended transmission(s) by the gNB/UE to access the channel in dBm where Pout≤Pmax. The maximum EIRP used for the transmission(s) by the gNB/UE during the channel occupancy is limited to Pout the gNB/UE used to initiate the channel occupancy. - BW is the channel bandwidth in MHz. ## *** < End of TP#1 for TS 37.213 v17.1.0> ***

Clarification to gNB indication of LBT/No-LBT mode

It was agreed in RAN1#105-e to support both cell specific and UE specific gNB indication of the LBT/No-LBT mode as captured in the following agreement [4]:

Agreement: For regions where LBT is not mandated, gNB should indicate to the UE this gNB-UE connection is operating in LBT mode or no-LBT mode - Support both cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) and UE specific (can be different for different UEs in a cell as part of UE-specific RRC configuration) gNB indication

The above agreement has been captured in TS 37.213 v17.1.0 as follows

## 4.4 Channel access procedures for frequency range 2-2 When a gNB/UE(s) is required by regulations to sense channel(s) for availability for performing transmission(s) on the channel(s) or when a gNB provides UE(s) with higher layer parameters ChannelAccessMode2-r17 by SIB1 or dedicated configuration indicating that the channel access procedures would be performed for performing transmission(s) on channel(s), the channel access procedures described in this clause for accessing the channel(s) on which the transmission(s) are performed by the gNB/UE(s), are applied.

However, using “or” in the above specification language is in contrast to the following descriptions of higher layer parameter ChannelAccessMode2-r17 set out by RAN2 in the latest running CR on RRC parameters in R2-2404126 which clearly specifies that the whether the channel access procedures are applied or not is based on the presence of absence of the higher layer parameter. As such, the current specification language implies that, the channel access procedures are applied in a region where LBT is mandated even if ChannelAccessMode2-r17 is not provided.

ServingCellConfigCommonSIB field descriptions

channelAccessMode2 If present (‘enabled’), this field indicates that the UE shall apply channel access mode procedures for operation with shared spectrum channel access in accordance with TS 37.213 [48], clause 4.4 for FR2-2. If absent, the UE shall not apply any channel access procedure.

ServingCellConfigCommon field descriptions

channelAccessMode2 If present (‘enabled’), the UE shall apply channel access mode procedures for operation with shared spectrum channel access in accordance with TS 37.213 [48], clause 4.4 for FR2-2. If absent, the UE shall not apply any channel access procedure.

ServingCellConfig field descriptions

channelAccessMode2 If present, this field indicates that the UE shall apply channel access mode procedures for operation with shared spectrum channel access in accordance with TS 37.213 [48], clause 4.4 for FR2-2. If absent, the UE does not apply these channel access procedures. Overwrites the corresponding field in ServingCellConfigCommon or ServingCellConfigCommonSIB for this serving cell.

We therefore propose to align the conditions for applying the channel access procedures in TS 37.213 with the corresponding descriptions of the higher layer parameter ChannelAccessMode2-r17 set out by RAN2. Following TP#2 captures our proposed changes.

Proposal 8: For operation in FR2-2, align the conditions for applying the channel access procedures in TS 37.213 with the corresponding descriptions of the higher layer parameter ChannelAccessMode2-r17 set out by RAN2.

- Adopt following TP#2 for TS 37.213 v17.1.0

## *** < Beginning of TP#2 for TS 37.213 v17.1.0> *** ## 4.4 Channel access procedures for frequency range 2-2 When a gNB provides UE(s) with higher layer parameters ChannelAccessMode2-r17 by SIB1 or dedicated configuration indicating that the channel access procedures would be performed by the gNB/UE(s) to sense channel(s) for availability for performing transmission(s), the channel access procedures described in this clause for accessing the channel(s) on which the transmission(s) are performed by the gNB/UE(s), are applied. *** < Unchanged parts are ommitted> *** ## *** < End of TP#2 for TS 37.213 v17.1.0> ***

Indication of LBT type in fallback DCI formats and RAR UL grant

It was agreed in the previous RAN1#107-e meeting [1] that for FR2-2 operation, gNB indicates the channel access type only without CP extension in the non-fallback DCI formats and new tables are introduced with entries corresponding to Type 3/Type 2/Type 1 channel access procedures as follows:

Conclusion Rel.16 NR-U style Cyclic Prefix extension is not supported for FR2-2 at least for DCI scheduled UL transmission - FFS: If CP extension is supported for CG-PUSCH in FR2-2 Agreement For Non-Fallback DCI formats, for FR2-2 operation, for the configuration of the ChannelAccess-CPext field in DCI to indicate the channel access type only, new tables are introduced indicating channel access types for FR2-2, with entries “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213” and “Type 3 channel access in 4.4.3 of 37.213”.

However, for the fallback DCI formats and RAR UL grant, it has not been agreed yet whether a fixed field size of 2 bits (indicating one of Type1/2/3 channel access procedures) or 1 bit (indicating one of Type1/3 channel access procedures) is supported.

In our view, the use of the fallback DCI formats is not limited to the cases before RRC connection establishment and UE reporting of its capability of Type 2 channel access procedure. Therefore, we support using 2 bits for the ChannelAccess-CPext field in the non-falback DCI formats 0_0/0_1 as well as the RAR UL grant if ChannelAccessMode2-r17 is provided as captured by following FL’s Proposal 2.9-2a in RAN1#107bis-e email discussion [4]:

Proposal 2.9-2a

For fallback DCI formats 0_0 and 1_0 and RAR UL grant, for FR2-2 operation, the ChannelAccess-Cpext field in DCI indicates the channel access type only. A new table similar to Table 7.3.1.1.1-4 is introduced with entries “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213” and “Type 3 channel access in 4.4.3 of 37.213”, and “reserved”.

- Note: This option requires 2 bis in fallback DCI

- For a UE not capable of supporting Type 2 LBT If UE has not signalled that is capable of supporting Type2 LBT, select between the following alternatives

  - Alt 1. An indication for Type 2 LBT for UL transmission will be treated as an indicate for Type 1 LBT

  - Alt 2. The UE does not expect fallback DCI indicating Type 2 LBT for UL transmission to be received

Proposal 9: For operation in FR2-2 and ChannelAccessMode2-r17 is provided, the ChannelAccess-CPext field size in fallback DCI formats 0_0/1_0 and RAR UL grant is 2 bits; 0 bit otherwise

-	Adopt following TP#3 for TS 38.212 v17.1.0 and TP#4 for TS 38.213 v17.1.0

In terms of down selecting between Alt 1 and Alt 2 above, in RAN1#108-e, following related agreement was achieved:

Agreement Before the UE reports it LBT capability, gNB is allowed to schedule UL transmission with Type 1 channel access - If the UE does not support Type 1 channel access, the UE should not transmit

Although the following conclusion was also achieved in RAN1#107-e [1] that the UE does not expect the gNB to schedule UL transmission with LBT type the UE does not support after the UE reports its LBT capability, so far the case before the UE reports its capability of supporting Type 2 LBT is NOT covered.

Conclusion After the UE reports it LBT capability, UE does not expect the gNB to schedule UL transmission with LBT type it does not support

Therefore, we support Alt 2 if Proposal 2.9-2a is agreed, and propose to capture in the specifications that the UE does not expect to be indicated with Type 2 channel access procedure if it has not indicated the capability to support it.

Proposal 10: For operation in a cell with shared spectrum access in FR2-2, support Alt 2 if Proposal 2.9-2a from RAN1#107bis-e is agreed and capture that the UE does not expect to be indicated with Type 2 Channel access procedure if it has not indicated the capability to support it.

-	Adopt following TP#5 for TS 37.213 v17.1.0

## *** < Beginning of TP#3 for TS 38.212 v17.1.0> *** ##### 7.3.1.1.1 Format 0_0 DCI format 0_0 is used for the scheduling of PUSCH in one cell. The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: *** < Unchanged parts are ommitted> *** - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access; 2 bits indicating channel access type as defined in Table 7.3.1.1.1-4B if ChannelAccessMode2-r17 is provided for operation in a cell in frequency range 2-2; 0 bit otherwise. *** < Unchanged parts are ommitted> *** The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by TC-RNTI: *** < Unchanged parts are ommitted> *** - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access; 2 bits indicating channel access type as defined in Table 7.3.1.1.1-4B if ChannelAccessMode2-r17 is provided for operation in a cell in frequency range 2-2; 0 bit otherwise. *** < Unchanged parts are ommitted> *** Table 7.3.1.1.1-4B: Channel access type for DCI format 0_0 and DCI format 1_0 if ChannelAccessMode2-r17 is provided in frequency range 2-2 Bit field mapped to index; Channel Access Type. 0; Type 1 channel access defined in clause 4.4.1 of 37.213. 1; Type 2 channel access defined in clause 4.4.2 of 37.213. 2; Type 3 channel access defined in clause 4.4.3 of 37.213. 3; Reserved. *** < Unchanged parts are ommitted> *** ##### 7.3.1.2.1 Format 1_0 DCI format 1_0 is used for the scheduling of PDSCH in one DL cell. The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: - Identifier for DCI formats – 1 bits - The value of this bit field is always set to 1, indicating a DL DCI format - Frequency domain resource assignment – bits where is given by clause 7.3.1.0 If the CRC of the DCI format 1_0 is scrambled by C-RNTI and the "Frequency domain resource assignment" field are of all ones, the DCI format 1_0 is for random access procedure initiated by a PDCCH order, with all remaining fields set as follows: *** < Unchanged parts are ommitted> *** - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access; 2 bits indicating channel access type as defined in Table 7.3.1.1.1-4B if ChannelAccessMode2-r17 is provided for operation in a cell in frequency range 2-2; 0 bits otherwise. - Reserved bits – 2 bits when the DCI format is monitored in common search space for operation in a cell in frequency range 2-2 and the number of bits for the field of 'ChannelAccess-CPext' is 0; 0 bits otherwise *** < Unchanged parts are ommitted> *** The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by TC-RNTI: *** < Unchanged parts are ommitted> *** - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access; 2 bits indicating channel access type as defined in Table 7.3.1.1.1-4B if ChannelAccessMode2-r17 is provided for operation in a cell in frequency range 2-2; otherwise 0 bit. - Reserved bits – 2 bits when the DCI format is monitored in common search space for operation in a cell in frequency range 2-2 and the number of bits for the field of 'ChannelAccess-CPext' is 0; 0 bits otherwise *** < Unchanged parts are ommitted> *** ## *** < End of TP#3 for TS 38.212 v17.1.0> ***

## *** < Beginning of TP#4 for TS 38.213 v17.1.0> *** ## 8.2 Random access response - Type-1 random access procedure *** < Unchanged parts are ommitted> *** The ChannelAccess-CPext field indicates a channel access type and CP extension for operation with shared spectrum channel access [15, TS 37.213] as defined in Table 7.3.1.1.1-4 in TS 38.212 or Table 7.3.1.1.1-4A in TS 38.212 if ChannelAccessMode-r16 = "semistatic" is provided. For operation in a cell in frequency range 2-2 [15, TS 37.213], the ChannelAccess-CPext field indicates a channel access type as defined in Table 7.3.1.1.1-4B in TS 38.212 if ChannelAccessMode2-r17 is provided. Table 8.2-1: Random Access Response Grant Content field size RAR grant field; Number of bits. Frequency hopping flag; 1. PUSCH frequency resource allocation; 14, for operation without shared spectrum channel access 12, for operation with shared spectrum channel access. PUSCH time resource allocation; 4. MCS; 4. TPC command for PUSCH; 3. CSI request; 1. ChannelAccess-CPext; 0, for operation without shared spectrum channel access, or for operation in FR2-2 when ChannelAccessMode2-r17 is not provided. 2, for operation with shared spectrum channel access in FR-1, or in FR2-2 if ChannelAccessMode2-r17 is provided.. *** < Unchanged parts are ommitted> *** ## 8.2A Random access response - Type-2 random access procedure *** < Unchanged parts are ommitted> *** - for operation with shared spectrum channel access, a channel access type and CP extension [15, TS 37.213] for a PUCCH transmission is indicated by a ChannelAccess-CPext field in the successRAR as defined in Table 7.3.1.1.1-4 in TS 38.212 or Table 7.3.1.1.1-4A in TS 38.212 if ChannelAccessMode-r16 = "semistatic" is provided. For operation in a cell in frequency range 2-2 [15, TS 37.213], the ChannelAccess-CPext field indicates a channel access type as defined in Table 7.3.1.1.1-4B in TS 38.212 if ChannelAccessMode2-r17 is provided. - the PUCCH transmission is with a same spatial domain transmission filter and in a same active UL BWP as a last PUSCH transmission *** < Unchanged parts are ommitted> *** ## *** < End of TP#4 for TS 38.213 v17.1.0> ***

*** < Begining of TP#5 for TS 37.213 v17.1.0> *** ### 4.4.2 Type 2 channel access procedures This clause describes channel access procedures to be performed by a gNB/UE where the time duration spanned by sensing slots that are sensed to be idle before a DL/UL transmission(s) is deterministic. A gNB/UE may transmit a transmission(s) on a channel immediately after Td which includes a sensing slot with a duration Tsl=5μs where the channel is sensed to be idle. The UE does not expect to be indicated with Type 2 channel access procedures before the UE indicates the corresponding capability. *** < End of TP#5 for TS 37.213 v17.1.0> ***

Indication of LBT type in non-fallback DCI formats 0_2/1_2

It was agreed in the RAN1#106bis-e meeting [5] under the WI on enhanced IIoT/URLLC to extend the ChannelAccess-CPext(-CAPC) fields in the non-fallback DCI formats 0_1 and 1_1 to the DCI formats 0_2 and 1_2, respectively, for dynamic channel access. Therefore, for operation in a cell with shared spectrum access in FR2-2, whether or not to extend the indication of the corresponding LBT types to the ChanneAcces-CPext(-CAPC) field in the non-fallback DCI formats 0_2/1_2 should be discussed.

For instance, it could be agreed that the ChannelAccess-CPext(-CAPC) field size in the non-fallback 0_2/1_2 is configurable to 0, 1 or 2 bits based on the number of entries configured from the existing tables of channel access types, Table 7.3.1.1.2-35A and Table 7.3.1.2.2-6A, respectively; and 0 bit otherwise.

Proposal 11: For operation in a cell with shared spectrum access in FR2-2, it should be discussed whether to extend the indication of the corresponding LBT types to the ChannelAccess-CPext(-CAPC) field in the non-fallback DCI formats 0_2/1_2 as done in Rel-17 WI on enhanced IIoT/URLLC for FR1.

Short control signaling in UL

In RAN1#105-e [5], contention-exempt short control signalling (CESCS) eligibility for UL was further discussed and the following agreement was made for transmission of msg1 for the 4-step RACH, and MsgA for the 2-step RACH:

Agreement: - Contention Exempt Short Control Signaling rules apply to the transmission of msg1 for the 4 step RACH and MsgA for the 2-step RACH for all supported SCS. - Note restriction for short control signalling transmissions apply (10% over any 100ms intervals) - Alt 1: The 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell - Alt 2: The 10% over any 100ms interval restriction is applicable to the msg1/msgA transmission from one UE perspective - FFS: Other UL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as msg3, SRS, PUCCH, PUSCH without user plain data, etc

The FFS in the agreement above was discussed in the previous RAN1#107-e meeting and the following conclusion was reached:

Conclusion In Rel.17, there is no consensus to apply contention exemption short control signalling to UL transmissions other than msg1 and msgA.

However, the issue of how to apply the 10% over 100ms interval restriction for msg1/msgA remains open though further discussed in RAN1#108-e. In Alt 2 in the above agreement, if it is left to each individual UE to use exemption for msg1 for the 4-step RACH and MsgA for the 2-step RACH, then the total time resources at which at least one UE within the cell transmits msg1/msgA can easily far exceed the 10% occupancy time for short control signaling exemption. In our view, this is a misuse of the exemption that is introduced in regulations for “short control signaling”. Therefore, our preference is to support Alt 1.

Proposal 12: In regions where channel sensing is mandated and short control signaling exemption is allowed by regulations, contention-exempt short control signaling rules apply to the transmission of msg1 for 4 step RACH and msgA for 2-step RACH such that the 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured in a cell (Alt 1).

- Adopt following TP#6 for TS 37.213 v17.1.0

## *** < Beginning of TP#6 for TS 37.213 v17.1.0> *** ### 4.4.5 Exempted transmissions from sensing In regions where channel sensing is required to access a channel for transmission and short control signalling exemption is allowed by regulation, a gNB/UE may transmit the following transmission(s) on a channel without sensing the channel: - Transmission(s) of the discovery burst by the gNB - Transmission(s) of the first message in a random access procedure by the UE When the gNB/UE transmits the above transmission(s) without sensing on a channel by utilizing the exemption above, the total duration of such transmission(s) by the gNB/UE shall not occupy the corresponding channel more than 10ms over any 100ms interval. The limit of 10ms over any 100ms interval is applied to all available resources configured in the cell for transmitting the first message in a random access procedure by the cell UEs. ## *** < End of TP#6 for TS 37.213 v17.1.0> ***

It was also discussed in RAN1#107bis-e whether gNB should provide an RRC configuration, e.g., in SIB1, to indicate if msg1 or msgA transmission as Contention Exempt Short Control Signaling is allowed. We think it is up to UE to transmit these UL transmissions based on short control signaling or with LBT. That is, when the UL transmissions based on short control signaling reach the 10% limits over 100ms, the UE should normally switch to LBT. Therefore, we do not see the need for such additional configuration.

Proposal 13: Providing an additional RRC configuration to indicate whether or not msg1 or msgA is transmitted based on Contention Exempt Short Control Signaling is not supported.

Misalignment of channel access procedure definitions in TS 37.213

It can be observed that some general definitions in Section 4.0 of TS 37.213 v17.1.0 are not applicable to the channel access procedures in FR2-2 captured in Section 4.4 while there are no indications of such exceptions. Therefore, we propose to clarify these general definitions in Section 4.0 in the context of the channel access procedures in FR2-2.

Proposal 14: Adopt following TP#7 into TS 37.213 v17.1.0.

## *** < Beginning of TP#7 for TS 37.213 v17.1.0> *** ## 4.0 General Unless otherwise noted, the definitions below are applicable for the following terminologies used in this specification: - A channel refers to a carrier or a part of a carrier consisting of a contiguous set of resource blocks (RBs) on which a channel access procedure is performed in shared spectrum. - A channel access procedure is a procedure based on sensing that evaluates the availability of a channel for performing transmissions. Unless otherwise specified, the basic unit for sensing is a sensing slot with a duration Tsl=9us. The sensing slot duration Tsl is considered to be idle if an eNB/gNB or a UE senses the channel during the sensing slot duration, and determines that the detected power for at least 4us within the sensing slot duration is less than energy detection threshold XThresh. Otherwise, the sensing slot duration Tsl is considered to be busy. - A channel occupancy refers to transmission(s) on channel(s) by eNB/gNB/UE(s) after performing the corresponding channel access procedures in this clause. - A Channel Occupancy Time refers to the total time for which eNB/gNB/UE and any eNB/gNB/UE(s) sharing the channel occupancy perform transmission(s) on a channel after an eNB/gNB/UE performs the corresponding channel access procedures described in this clause. Except for operating in FR2-2, for determining a Channel Occupancy Time, if a transmission gap is less than or equal to 25us, the gap duration is counted in the channel occupancy time. A channel occupancy time can be shared for transmission between an eNB/gNB and the corresponding UE(s). - A DL transmission burst is defined as a set of transmissions from an eNB/gNB without any gaps greater than16usa specific duration. Transmissions from an eNB/gNB separated by a gap of more than 16us the specific duration are considered as separate DL transmission bursts. An eNB/gNB can transmit transmission(s) after a gap within a DL transmission burst without sensing the corresponding channel(s) for availability. The specific duration is 16us for operation in FR-1 and 8us for operation in FR2-2. - A UL transmission burst is defined as a set of transmissions from a UE without any gaps greater than 16usa specific duration. Transmissions from a UE separated by a gap of more than the specific duration are considered as separate UL transmission bursts. A UE can transmit transmission(s) after a gap within a UL transmission burst without sensing the corresponding channel(s) for availability. The specific duration is 16us for operation in FR-1, and 8us for operation in FR2-2. *** < Unchanged parts are ommitted> *** *** < End of TP#7 for TS 37.213 v17.1.0> ***

Conclusions

Based on the discussions, the following proposals were made:

Proposal 1: RAN1 to send an LS to RAN2 to correct the value range for duration-r17 and offset-r17 in cg-COT-Sharing-r17 to 319 as per the agreement in RAN1#107-e, and to correct the value range for the size of the cg-COT-SharingList-r17 to 50,722.

Proposal 2: RAN1 to consider the following for L3-RSSI measurement

- Confirm that the ‘inter-frequency L3-RSSI measurement’ applies when the RSSI measurement bandwidth is fully contained within the current carrier bandwidth

- For the ‘target frequency TCI state’ if the measurement resource is not within any active BWP of any serving cell,

  - the spatial domain filter for the L3-RSSI measurement on the target frequency should be the same as the spatial domain filter associated with the TCI state of the BWP in which the RMTC configuration is provided,

  - if no TCI state is provided in the RMTC configuration, the spatial domain filter for the L3-RSSI measurement on the target frequency should be the same as the spatial domain filter for one of the latest PDSCH reception and the latest CORESET monitoring in the active BWP of the carrier on which the RMTC configuration is provided.

Proposal 3: Adopt following TP#0 into TS 38.133 v15.7.0.

Proposal 4: RAN1 to send an LS to RAN4 capturing the clarifications and TP#1 if agreed.

Proposal 5: For operation in FR2-2, clarify in the current specifications that the EDT determination mechanism is not restricted to a node initiating a COT. The node determining EDT could be also a responding node.

Proposal 6: For operation in FR2-2, when independent per-beam LBT is performed at the start of the COT, define Pout for each sensing beam as the maximum EIRP of the intended transmissions “covered” by the sensing beam by the node determining EDT during a COT.

Proposal 7: For operation in FR2-2, adopt following TP#1 for TS 37.213 v17.1.0

Proposal 8: For operation in FR2-2, align the conditions for applying the channel access procedures in TS 37.213 with the corresponding descriptions of the higher layer parameter ChannelAccessMode2-r17 set out by RAN2.

- Adopt following TP#2 for TS 37.213 v17.1.0

Proposal 9: For operation in FR2-2 and ChannelAccessMode2-r17 is provided, the ChannelAccess-CPext field size in fallback DCI formats 0_0/1_0 and RAR UL grant is 2 bits; 0 bit otherwise

-	Adopt following TP#3 for TS 38.212 v17.1.0 and TP#4 for TS 38.213 v17.1.0

Proposal 10: For operation in a cell with shared spectrum access in FR2-2, support Alt 2 if Proposal 2.9-2a from RAN1#107bis-e is agreed and capture that the UE does not expect to be indicated with Type 2 Channel access procedure if it has not indicated the capability to support it.

-	Adopt following TP#5 for TS 37.213 v17.1.0

Proposal 11: For operation in a cell with shared spectrum access in FR2-2, it should be discussed whether to extend the indication of the corresponding LBT types to the ChannelAccess-CPext(-CAPC) field in the non-fallback DCI formats 0_2/1_2 as done in Rel-17 WI on enhanced IIoT/URLLC for FR1.

Proposal 12: In regions where channel sensing is mandated and short control signaling exemption is allowed by regulations, contention-exempt short control signaling rules apply to the transmission of msg1 for 4 step RACH and msgA for 2-step RACH such that the 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured in a cell (Alt 1).

- Adopt following TP#6 for TS 37.213 v17.1.0

Proposal 13: Providing an additional RRC configuration to indicate whether or not msg1 or msgA is transmitted based on Contention Exempt Short Control Signaling is not supported.

Proposal 14: Adopt following TP#7 into TS 37.213 v17.1.0.

- References

- Chairman notes, RAN1#107-e

- Chairman notes, RAN1#108-e

- Chairman notes, RAN1#104bis-e

- R1-2200753, FL summary#2 for channel access for 52.6 to 71 GHz band, Moderator (Qualcomm)

- Chairman notes, RAN1#105-e