3GPP TSG-RAN WG1 Meeting #109-e	         R1-2203083

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.16.2

Source:	Huawei, HiSilicon, SIA

Title:	Rel-17 UE features for extension to 71 GHz

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss our views on the UE features for extension to 71GHz according to Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e[1].

Discussion on the feature group table

- FG24-1b/1c, FG24-4b/4c and FG24-5c

Note:

- Final Proposal 5 (agreed): replace the notes under FGs 24-1c, 24-4c and 24-5c for multi-RB PUCCH, and replace the bracketed notes under FGs 24-1b and 24-4b for wideband PRACH, with “This FG is only applicable when PSD limitation applies within FR2-2 based on the regional regulations”

- Final Proposal 5 (agreed): replace the notes under FGs 24-1c, 24-4c and 24-5c for multi-RB PUCCH, and replace the bracketed notes under FGs 24-1b and 24-4b for wideband PRACH, with “This FG is only applicable when PSD limitation applies within FR2-2 based on the regional regulations”

In RANP#95-e, the following agreement had been reached [2].

The UE feature list prepared by RAN1 should be updated accordingly.

For 24-1b, it is still pending on “[A UE that supports FG 24-2 must indicate this FG is supported]”. To our understanding, it is not necessary because all basic uplink capabilities to fulfil the initial access procedures have been captured in FG24-1a, which is already the prerequisite of 24-2. Moreover, FG24-2 is applied for both licensed band unlicensed band while FG24-1b is only for unlicensed band from our perspective. The sentence of “[A UE that supports FG 24-2 must indicate this FG is supported]” should be deleted from the note column of FG24-1b.

For 24-1c, the issue on “[A UE that supports [24-1a/24-2/FR2-2] must indicate this FG is supported]” is not resolved.   According to the agreement in RANP#95-e, “This FG is only applicable when PSD limitation applies within FR2-2 based on the regional regulations”. However, even in unlicensed bands, PSD limitations do not apply in all regions. However, FG24-1a, FG24-2 or even the whole FR2-2 would include both licensed band and unlicensed band regardless of the region. In our view, in practice, it is not necessary for a UE that only supports licensed band to report this capability. For the UE operating on the unlicensed band without PSD limitation, this FG is also redundant. Thus, we propose to delete the sentence of “[A UE that supports [24-1a/24-2/FR2-2] must indicate this FG is supported]”in the note column.

Proposal 1: Reflect the RANP#95-e agreement in the RAN1 UE feature list, i.e. “replace the notes under FGs 24-1c, 24-4c and 24-5c for multi-RB PUCCH, and replace the bracketed notes under FGs 24-1b and 24-4b for wideband PRACH, with “This FG is only applicable when PSD limitation applies within FR2-2 based on the regional regulations”

Proposal 2: Delete the sentence of “[A UE that supports FG 24-2 must indicate this FG is supported]” in the note column of FG24-1b.

Proposal 3: Delete the sentence of “[A UE that supports [24-1a/24-2/FR2-2] must indicate this FG is supported]” in the note column of FG24-1c.

- FG24-1f and FG24-1g: multiple PDSCH/PUSCH scheduling by single DCI for 120kHz in FR2-1

Features:

In RAN#108-e, the multiple PDSCH/PUSCH scheduling by single DCI has been extended to FR2-1 for 120kHz SCS due to its scheduling efficiency to achieve high throughput. During the discussion and also noted in the Chair’s note, whether such FGs can be further extended to other SCS and FR can be further discussed. For UL, it is already supported for all SCSs in FR1 and FR2-1 when multiple PUSCHs scheduled by single DCI are consecutive. Extending the support of non-contiguous allocation to the 15/30/60kHz SCSs in FR1 and 60kHz SCS in FR2-1 same as those agreed for FR2-1 and FR2-2 for 120kHz/480kHz/960kHz could reduce scheduling overhead and PDCCH monitoring complexity without introducing additional standardization effort. Due to the same reason, we also support to extend multiple PDSCH scheduling by single DCI to 15/30/60kHz SCS in FR1 and 60kHz SCS in FR2-1.

Proposal 4: Define new feature groups to support multiple PDSCH scheduling by single DCI and multiple PUSCH scheduling by single DCI for other SCSs in FR1 and FR2-1.

- FG24-4 and FG24-5: 480/960 kHz SCS support for DL and multi slot PDCCH monitoring capability

Component:

The 7th component of “[7. For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of each slot of the slot group, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within each slot of the slot group.]”on the MO configuration for group 2) CSS is still pending because whether to further restrict monitoring occasions for the group 2) CSS is under discussion.

  - Working assumption: For Group (2) SSs

    - For Type0/0A/2 CSS

      - The slots indicated in monitoringSlotsWithinSlotGroup-r17 are not restricted to be consecutive

      - The number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 can be up to L

    - For Type1 CSS without dedicated RRC

      - The number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 per slot group of Xs slots should be no larger than M, where M is FFS

  - Working assumption: For Group (2) SSs

    - For Type0/0A/2 CSS

      - The slots indicated in monitoringSlotsWithinSlotGroup-r17 are not restricted to be consecutive

      - The number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 can be up to L

    - For Type1 CSS without dedicated RRC

      - The number of slots configured for multi-slot PDCCH monitoring in monitoringSlotsWithinSlotGroup-r17 per slot group of Xs slots should be no larger than M, where M is FFS

In RAN1 108-e, the following working assumption on group 2) CSS was reached.

It is observed that no additional restriction is added for type0/0A/2 CSS. Moreover, for Type1 CSS without dedicated RRC, we do not think it should be treated differently from the other CSSs in group 2) because it restricts maximum number of MOs within the RAR window. The detail discussion can be found in our companion paper [6] submitted to AI8.2.2. Thus, we propose to remove the bracket on the component 7 for FG24-4 and FG24-5.

Proposal 5: Support to remove bracket on component 7 in FG24-4 and FG24-5.

- FG24-6 and FG24-7: type 1/2 channel access procedure in uplink for FR2-2

Component:

In the RAN1#108-e, the following agreement on the LBT bandwidth was reached. As a compromise, the LBT bandwidth was defined as channel which includes at least the active BWP.  Thus, the component of “[2. Support LBT performed per carrier/BWP bandwidth]” should be updated as “Support LBT performed per channel at least includes the active UL BWP bandwidth for single carrier UL transmission”.

Agreement

For LBT for single carrier UL transmission, UE performs LBT over a BW that at least includes the active UL BWP bandwidth

- The BW that at least includes the active UL BWP bandwidth is captured as “channel” in 37.213

Agreement

For LBT for single carrier DL transmission to a UE, gNB performs LBT over a bandwidth that at least includes the active DL BWP bandwidth configured for that UE.

- This does not rule out gNB implementation to perform LBT over a wider bandwidth

- The BW that at least includes the active DL BWP bandwidth is captured as “channel” in 37.213

For LBT for single carrier DL transmission to multiple UEs, from each UE point of view, gNB performs LBT over a bandwidth that at least includes the active DL BWP bandwidth configured for that UE.

- This does not rule out gNB implementation to perform LBT over a wider bandwidth that includes the active DL BWP of multiple UEs

Agreement

For LBT for single carrier UL transmission, UE performs LBT over a BW that at least includes the active UL BWP bandwidth

- The BW that at least includes the active UL BWP bandwidth is captured as “channel” in 37.213

Agreement

For LBT for single carrier DL transmission to a UE, gNB performs LBT over a bandwidth that at least includes the active DL BWP bandwidth configured for that UE.

- This does not rule out gNB implementation to perform LBT over a wider bandwidth

- The BW that at least includes the active DL BWP bandwidth is captured as “channel” in 37.213

For LBT for single carrier DL transmission to multiple UEs, from each UE point of view, gNB performs LBT over a bandwidth that at least includes the active DL BWP bandwidth configured for that UE.

- This does not rule out gNB implementation to perform LBT over a wider bandwidth that includes the active DL BWP of multiple UEs

Proposal 6： The component 2 in FG24-6 and FG24-7 should be updated as “Support LBT performed per channel that at least includes the active UL BWP bandwidth for single carrier UL transmission”.

- FG24-8 and FG24-9: 32 HARQ processes

Features:

In RAN1#108-e, it was agreed to further discuss whether 32 HARQ processes can be extended for FR2-1 [5]. In FR2-2, the reason to introduce 32 HARQ processes for FR2-1 is to avoid HARQ processes starvation when multiple PDSCH is scheduled by a single DCI. It is also beneficial when gNB configuring small number of DL/UL switching point in a long periodicity, as discussed in NR-U. Considering multiple PDSCH scheduling by single DCI is extended to FR2-1 for 120kHz SCS, it would be straightforward to extend 32 HARQ processes to FR2-1 as well. In order to avoid ambiguity on the number of HARQ processes during potential BWP switching between 60kHz SCS and 120kHz SCS，we support to extend 32 HARQ processes for 60kHz SCS in FR2-1 as well. UE should report support of 32 HARQ processes in FR2-1 for 60kHz and 120kHz together in a single FG.

Proposal 7: Introduce FGs to support 32 UL HARQ and 32 DL HARQ processes for both 60kHz/120kHz SCS in FR2-1.

New capabilities on PDCCH monitoring across multiple carriers

In RAN1#108-e, additional 4 capability reports on the number of CCs with Rel-17 PDCCH monitoring only or combination of PDCCH monitoring with earlier releases in different serving cells are introduced as follows.

- Case 4: Capability on the number of CCs with Rel-17 monitoring capability only

- Case 5: Capability on the number of CCs with Rel-15 monitoring capability and Rel-17 monitoring capability on different serving cells

- Case 6: Capability on the number of CCs with Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells

- Case 7: Capability on the number of CCs with Rel-15 monitoring capability, Rel-16 monitoring capability and Rel-17 monitoring capability on different serving cells

Similar to Rel-16, separate FGs should be introduced correspondingly. The detail text for each FG are in appendix.

Proposal 8: Introduce separate FGs corresponding to the additional 4 cases on the capability on the number of CCs with different monitoring capability combinations as agreed in RAN1#108-e. The definition of the FGs are provided in appendix.

In RAN1#108-e, the PDCCH monitoring capabilities in NR-DC scenario are still pending. When a UE is configured for NR-DC operation with a total of NNR-DCDL,cells downlink cells on both the MCG and the SCG, UE capability signaling and the corresponding new RRC parameters for the following 4 additional cases should be supported:

- Case   4: All the downlink cells have SCS configuration μ∈{5,6}.

- Case 5: NNR-DC,r17DL,cells downlink cells have SCS configuration μ∈{5,6}, the UE is provided monitoringCapabilityConfig = r15monitoringcapability for NNR-DC,r15DL,cells downlink cells, the UE is not provided monitoringCapabilityConfig = r16monitoringcapability for any downlink cell.

- Case 6: NNR-DC,r17DL,cells downlink cells have SCS configuration μ∈{5,6}, the UE is provided monitoringCapabilityConfig = r16monitoringcapability for NNR-DC,r16DL,cells downlink cells, and the UE is not provided monitoringCapabilityConfig = r15monitoringcapability for any downlink cell.

- Case 7: NNR-DC,r17DL,cells downlink cells have SCS configuration μ∈{5,6}, the UE is provided monitoringCapabilityConfig = r15monitoringcapability for NNR-DC,r15DL,cells downlink cells and monitoringCapabilityConfig = r16monitoringcapability for NNR-DC,r16DL,cells downlink cells.

The detail discussion can be found in our companion paper [6].

Proposal 9: Introduce separate FGs corresponding to the additional 4 cases on the capability on the number of CCs with different monitoring capability combinations when a UE is configured with NR-DC. The definition of the FGs are provided in appendix.

Conclusions

Based on the discussion above, we had following proposals.

Proposal 1: Reflect the RANP#95-e agreement in the RAN1 UE feature list, i.e. “replace the notes under FGs 24-1c, 24-4c and 24-5c for multi-RB PUCCH, and replace the bracketed notes under FGs 24-1b and 24-4b for wideband PRACH, with “This FG is only applicable when PSD limitation applies within FR2-2 based on the regional regulations”

Proposal 2: Delete the sentence of “[A UE that supports FG 24-2 must indicate this FG is supported]” in the note column of FG24-1b.

Proposal 3: Delete the sentence of “[A UE that supports [24-1a/24-2/FR2-2] must indicate this FG is supported]” in the note column of FG24-1c.

Proposal 4: Define new feature groups to support multiple PDSCH scheduling by single DCI and multiple PUSCH scheduling by single DCI for other SCSs in FR1 and FR2-1.

Proposal 5: Support to remove bracket on component 7 in FG24-4 and FG24-5.

Proposal 6： The component 2 in FG24-6 and FG24-7 should be updated as “Support LBT performed per channel that at least includes the active UL BWP bandwidth for single carrier UL transmission”.

Proposal 7: Introduce FGs to support 32 UL HARQ and 32 DL HARQ processes for both 60kHz/120kHz SCS in FR2-1.

Proposal 8: Introduce separate FGs corresponding to the additional 4 cases on the capability on the number of CCs with different monitoring capability combinations as agreed in RAN1#108-e. The definition of the FGs are provided in appendix.

Proposal 9: Introduce separate FGs corresponding to the additional 4 cases on the capability on the number of CCs with different monitoring capability combinations when a UE is configured with NR-DC. The definition of the FGs are provided in appendix.

- References

- R1-2202929, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e　including remaining RAN1 issues”, Moderators (AT&T, NTT DOCOMO, INC.)

- RP-220951, “Moderator's conclusion for discussion [95e-29-R17-52.6-71GHz-WI]”, RAN1 Vice-Chair (Huawei)

- RP-213540, “Revised WID:  Extending current NR operation to 71 GHz”, Qualcomm

- R1-2200781, “LS on updated Rel-17 RAN1 UE features list for NR”, RAN1#107bis-e, Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2202860, “Session Notes of AI 8.16.2”, RAN1#108-e, Ad-Hoc Chair (AT&T)

- R1-2203080, “Remaining issues of PDCCH monitoring enhancement for 52-71GHz spectrum”, Huawei, HiSilicon

- Appendix: proposed changes to the UE feature list corresponding to [1]

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24. NR_ext_to_71GHz | 24-1b | Wideband PRACH for 120 kHz in FR2-2 | Enhanced PRACH design for operation by adopting a single long ZC sequence, with ZC sequence equal to 1151 for 120kHz and ZC sequence equal to 571 for 120kHz | 24-1a | Yes | N/A | Wideband PRACH for 120 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | This FG is only applicable when PSD limitation applies within FR2-2 based on the regional regulations | Optional with capability signalling |
|  | 24-1c | Multi-RB support PUCCH format 0/1/4 for 120 kHz in FR2-2 | 1. Support multi-RB PUCCH format 4 for 120 kHz 2. Support multi-RB PUCCH format 0/1 for 120 kHz | 24-1a | Yes | N/A | Multi-RB support PUCCH format 0/1/4 for 120 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | This FG is only applicable when PSD limitation applies within FR2-2 based on the regional regulations | Optional with capability signalling |
|  | 24-1f | Multiple PDSCH scheduling by single DCIin FR2-1 | 1. Multi-PDSCH scheduling by single DCI 2. HARQ enhancements for both type 1 and type 2 HARQ codebook for supporting multi-PDSCH scheduling with singe DCI |  | Yes | N/A | Multiple PDSCH scheduling by single DCI is not supported in FR2-1 | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
|  | 24-1g | Multiple PUSCH scheduling by single DCI in FR2-1 | 1. Multi-PUSCH scheduling by single DCI with non-contiguous allocation |  | Yes | N/A | Multiple PUSCH scheduling by single DCI is not supported in FR2-1 with non-contiguous allocation | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
|  | 24-1h | Multiple PDSCH scheduling by single DCI in FR1 | 1. Multi-PDSCH scheduling by single DCI 2. HARQ enhancements for both type 1 and type 2 HARQ codebook for supporting multi-PDSCH scheduling with singe DCI |  | Yes | N/A | Multiple PDSCH scheduling by single DCI for 120kHz is not supported in FR1 | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
|  | 24-1i | Multiple PUSCH scheduling by single DCI in FR1 | 1. Multi-PUSCH scheduling by single DCI with non-contiguous allocation |  | Yes | N/A | Multiple PUSCH scheduling by single DCI is not supported in FR1 with non-contiguous allocation | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
|  | 24-4 | 480KHz SCS support for DL | 1. 480KHz SCS for DL data and control channels, SSB, and reference signal reception in FR2-2 for non-initial access 2. Multiple-slot PDCCH monitoring for 480KHz with (Xs,Ys) = (4,1) 3. Multi- PDSCH scheduling by single DCI for the operation with 480 kHz SCS and corresponding HARQ enhancements 4. Within the Ys = 1 slot (with Xs=4), monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS with a maximum of two monitoring spans per slot with a span duration of Y symbols and a minimum gap of X symbols between the start of two spans, where (X,Y) = (4, 3) and (7, 3) are supported 5. Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for FDD 6. Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for TDD 7. For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of each slot of the slot group, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within each slot of the slot group. | 24-1 | Yes | N/A | 480KHz SCS for DL is not supported | Perband | N/A | N/A | N/A |  | Optional with capability signalling |
|  | 24-4b | Wideband PRACH for 480 kHz in FR2-2 | PRACH with 480KHz and length 571 | 24-4a | Yes | N/A | Wideband PRACH for 480 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | This FG is only applicable when PSD limitation applies within FR2-2 based on the regional regulations | Optional with capability signalling |
|  | 24-4c | Multi-RB PUCCH format 0/1/4 for 480 kHz in FR2-2 | Support multi-RB PUCCH format 0/1/4 for 480 kHz | 24-4a | Yes | N/A | Multi-RB PUCCH format 0/1/4 for 480 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | This FG is only applicable when PSD limitation applies within FR2-2 based on the regional regulations | Optional with capability signalling |
|  | 24-5 | 960KHz SCS support for DL | 1. 960KHz SCS for DL data and control channels, SSB, and reference signal reception in FR2-2 for non-initial access 2. Multiple-slot PDCCH monitoring for 960KHz with (Xs,Ys)=(8,1) 3. MultiPDSCH scheduling by single DCI for the operation with 960 kHz SCS and corresponding HARQ enhancements 4. Within the Ys = 1 slot (with Xs=8), monitoring of type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS with a span duration of Y symbols and a minimum gap of X symbols between the start of two spans, where (X,Y)= (7, 3) is supported 5. Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for FDD 6. Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of Xs slots per scheduled CC for TDD 7. For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of each slot of the slot group, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within each slot of the slot group. | 24-1 | Yes | N/A | 960KHz SCS support for DL is not supported | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
|  | 24-5c | Multi-RB PUCCH format 0/1/4 for 960 kHz in FR2-2 | Support multi-RB PUCCH format 0/1/4 for 960 kHz | 24-5a | Yes | N/A | Multi-RB PUCCH format 0/1/4 for 960 kHz in FR2-2 is not supported | Per band | N/A | N/A | N/A | This FG is only applicable when PSD limitation applies within FR2-2 based on the regional regulations | Optional with capability signalling |
|  | 24-6 | Type 1 channel access procedure in uplink for FR2-2 with shared spectrum channel access | 1. Support Type 1 channel access procedure 2. Support LBT performed per channel at least includes active UL BWP bandwidth] | 24-1a | Yes | N/A | Type 1 channel access procedure in uplink for FR2-2 with shared spectrum channel access is not supported | per band | N/A | N/A | N/A | A UE that supports FR2-2 must indicate this FG is supported when required by regulation | Optional with capability signalling |
|  | 24-7 | Type 2 channel access procedure in uplink for FR2-2 with shared spectrum channel access | 1. Support Type 2 channel access procedure 2. Support LBT performed per channel at least includes active ULBWP bandwidth] | 24-1a, 24-6 | Yes | N/A | Type 2 channel access procedure in uplink for FR2-2 with shared spectrum channel access is not supported | per band | N/A | N/A | N/A | A UE that supports FR2-2 must indicate this FG is supported when required by regulation | Optional with capability signalling |
|  | 24-8a | 32 DL HARQ processes for FR 2-1 | Support 32 HARQ processes in DL for 60/120 kHz |  | Yes | N/A | 32 DL HARQ processes for FR 2-2 is not supported | Per band | N/A | N/A | N/A | A UE supporting 32 maximum number of HARQ processes for 120 kHz SCS for DL shall support 32 as the maximum number of HARQ processes for 60 kHz SCS for DL in FR2-1 | Optional with capability signalling |
|  | 24-9a | 32 UL HARQ processes for FR 2-1 | Support 32 HARQ processes in UL for 60/120 kHz |  | Yes | N/A | 32 DL HARQ processes for FR 2-2 is not supported | Per band | N/A | N/A | N/A | A UE supporting 32 maximum number of HARQ processes for 120 kHz SCS for DL shall support 32 as the maximum number of HARQ processes for 60 kHz SCS for DL in FR2-1 | Optional with capability signaling |
|  | 24-Xa | Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs when configured with DL CA with Rel-17 PDCCH monitoring capability on all the serving cells | Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs when configured with DL CA with Rel-17 PDCCH monitoring capability on all the serving cells - Candidate value for the component: {[4, 5, …, , 16]} | 24-4, 24-5 | Yes | N/A |  |  | N/A | N/A | N/A |  | Optional with capability signaling |
|  | 24-Xb | Mix of Rel. 17 PDCCH monitoring capability and Rel.15 and/or Rel.16 PDCCH monitoring capability on different carriers | Support Rel-15 monitoring capability, Rel-16 monitoring capability and/or Rel-17 monitoring capability on different serving cells | 24-4, 24-5 | Yes |  |  |  |  |  |  |  | Optional with capability signaling |
|  | 24-Xc | Number of carriers for CCE/BD scaling with DL CA with mix of Rel. 17 and Rel. 15 PDCCH monitoring capabilities on different carriers | 1. Supported combination(s) of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R17) - Candidate values for pdcch-BlindDetectionCA-R15 is 1 to 15 - Candidate values for pdcch-BlindDetectionCA-R17 is 1 to 15 - Range of pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R17: {[4, 5, …, 16]} | 24-Xb | Yes | N/A |  |  | N/A | N/A | N/A |  | Optional with capability |
|  | 24-Xd | Number of carriers for CCE/BD scaling with DL CA with mix of Rel. 17 and Rel. 16 PDCCH monitoring capabilities on different carriers | 1. Supported combination(s) of (pdcch-BlindDetectionCA-R16, pdcch-BlindDetectionCA-R17) - Candidate values for pdcch-BlindDetectionCA-R16 is 1 to 15 - Candidate values for pdcch-BlindDetectionCA-R17 is 1 to 15 - Range of pdcch-BlindDetectionCA-R16 + pdcch-BlindDetectionCA-R17: {[3,4, 5, …, 16]} | 24-Xb | Yes | N/A |  |  | N/A | N/A | N/A |  | Optional with capability |
|  | 24-Xe | Number of carriers for CCE/BD scaling with DL CA with mix of Rel. 17, Rel. 16 and Rel. 15 PDCCH monitoring capabilities on different carriers | 1. Supported combination(s) of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16, pdcch-BlindDetectionCA-R17) - Candidate values for pdcch-BlindDetectionCA-R15 is 1 to 15 - Candidate values for pdcch-BlindDetectionCA-R16 is 1 to 15 - Candidate values for pdcch-BlindDetectionCA-R17 is 1 to 15 - Range of pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R16+ pdcch-BlindDetectionCA-R17: {[4, 5, …, 16]} | 24-Xb | Yes | N/A |  |  | N/A | N/A | N/A |  | Optional with capability |
|  | 24-Xf | Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs for MCG and for SCG when configured for NR-DC operation with Rel-17 PDCCH monitoring capability on all the serving cells | Supported combination of (pdcch-BlindDetectionMCG-UE-r17, pdcch-BlindDetectionSCG-UE-r17) | 24-4, 24-5 | Yes | N/A |  |  | N/A | N/A | N/A |  | Optional with capability |
|  | 24-Xg | Number of carriers for CCE/BD scaling for MCG and for SCG when configured for NR-DC operation with mix of Rel. 17 and Rel. 15 PDCCH monitoring capabilities on different carriers | Supported combination(s) of (pdcch-BlindDetectionMCG-UE-r15, pdcch-BlindDetectionSCG-UE-r15, pdcch-BlindDetectionMCG-UE-r17, pdcch-BlindDetectionSCG-UE-r17) | 24-Xb | Yes | N/A |  |  | N/A | N/A | N/A |  | Optional with capability |
|  | 24-Xh | Number of carriers for CCE/BD scaling for MCG and for SCG when configured for NR-DC operation with mix of Rel. 17 and Rel. 16 PDCCH monitoring capabilities on different carriers | Supported combination(s) of (pdcch-BlindDetectionMCG-UE-r16, pdcch-BlindDetectionSCG-UE-r16, pdcch-BlindDetectionMCG-UE-r17, pdcch-BlindDetectionSCG-UE-r17) | 24-Xb | Yes | N/A |  |  | N/A | N/A | N/A |  | Optional with capability |
|  | 24-Xj | Number of carriers for CCE/BD scaling for MCG and for SCG when configured for NR-DC operation with mix of Rel.17 , Rel.16 and Rel. 15 PDCCH monitoring capabilities on different carriers | Supported combination(s) of (pdcch-BlindDetectionMCG-UE-r15, pdcch-BlindDetectionSCG-UE-r15, pdcch-BlindDetectionMCG-UE-r16, pdcch-BlindDetectionSCG-UE-r16, pdcch-BlindDetectionMCG-UE-r17, pdcch-BlindDetectionSCG-UE-r17) | 24-Xb | Yes | N/A |  |  | N/A | N/A | N/A |  | Optional with capability |