3GPP TSG-RAN WG1 Meeting #109-e	R1-2203085

e-Meeting, May 9 – 20, 2022

Agenda Item:	8.7.1

Source:	Huawei, HiSilicon

Title:	Remaining issues on UE power saving in IDLE/inactive mode

Document for:	Discussion and Decision

Introduction

In this contribution, the remaining issues on UE power saving in IDLE/inactive mode, including paging early indication and assistance TRS, are discussed.

Remaining issues of PEI

  - Discussion on monitoring occasion determination when SDT is configured for inactive mode

In RAN1#108-e, how to determine the monitoring occasion when SearchSpaceId=0 is configured for peiSearchSpace was discussed for the case when SSB and CORESET multiplexing pattern 2 or 3.

The following TP to Subclause 10.1 of TS 38.213 was proposed:

*** Unchanged text is omitted *** If a UE is provided a zero value for searchSpaceID in PDCCH-ConfigCommon for a Type0/0A/2-PDCCH CSS set, Type2A-PDCCH CSS with SS/PBCH block and CORESET multiplexing pattern 2 or 3, or is not provided searchSpaceBroadcast, the UE determines monitoring occasions for PDCCH candidates of the Type0/0A/2/2A-PDCCH CSS set as described in clause 13, and, if the UE is provided a C-RNTI, the UE monitors PDCCH candidates only at monitoring occasions associated with a SS/PBCH block, where the SS/PBCH block is determined by the most recent of - a MAC CE activation command indicating a TCI state of the active BWP that includes a CORESET with index 0, as described in [6, TS 38.214], where the TCI-state includes a CSI-RS which is quasi-co-located with the SS/PBCH block, or - a random access procedure that is not initiated by a PDCCH order that triggers a contention-free random access procedure *** Unchanged text is omitted ***

The motivation of the legacy specification for UE determining monitoring occasions is that when UE in connected mode, a single-beam UE-specific PDCCH with CRC scrambled by C-RNTI can be transmitted in a Type0/0A/2-PDCCH CSS set. Therefore, UE has to determine which SS/PBCH block the monitoring occasions of the PDCCH candidates of the Type0/0A/2-PDCCH CSS set are associated with before explicit TCI-state configuration. While as described in clause 13 of TS38.213, the monitoring occasions of PDCCH candidates for search space zero are associated with all the SS/PBCH blocks, so UE needs a mechanism to determine the beam direction for the monitoring occasions. However, the case is different for Type2A-PDCCH CSS set, because PEI is the only channel transmitted in Type2A-PDCCH set in Rel-17, and PEI is a cell-specific channel/signal which is broadcasted in all the beam directions associated with all the actual transmitted SSBs in the cell. Therefore, a UE can choose the proper monitoring occasions to receive PEI just based on its measurements results on SSBs in IDLE/IANCTIVE mode. So it is unnecessary to introduce additional restriction on the monitoring occasions for PDCCH-candidates in a Type2A-PDCCH CSS set, and we don’t see any necessity of clarification on current specification, which seems to be nothing to do with paging enhancements in Rel-17.

Also the justification of the TP in RAN1#109 is for the case when SDT is configured. However, we didn’t see any relationship for this TP with SDT.

- No further clarification on monitoring occasion determination of PEI is needed.

Remaining issues of TRS

  - Clarification on the reference point for MOs within the same PO/PEI-O

In RAN1 #108-e, the following agreement was made for further down-selection [1].

Agreement Down-select one of the TPs to clarify the current default DRX cycle used for determining the reference point for start of validity duration for an availability indication - Alt1: Adopt TP#1 - Alt2: Adopt TP#2 - Note: TRS availability indication transmitted in all PDCCH monitoring occasions of the same PEI-O or PO is same. - Note: Reference point for TRS availability indication in a given DCI is identical for all UEs regardless of UE specific DRX value. ============================= start of TP#1========================================== 10.4B Indication of TRS resources <omitted text> A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame within the DRX cycle that includes a the PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. ============================= end of TP#1========================================== ============================= start of TP#2========================================== 10.4B Indication of TRS resources <omitted text> A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame of the DRX cycle associated with the first PDCCH MO of the PEI-O or PO that includes a the PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. ============================= end of TP#2==========================================.

The key difference between the two alternatives is that whether the reference points for MOs within the same PO/PEI-O are aligned or not. And our preference is to get them aligned.

According to TS 38.304[5], one PO consists of S PDCCH monitoring occasions, while S is the number of actually transmitted SSBs. The starting position of a PO can be configured by firstPDCCH-MonitoringOccasionOfPO. Similar design is also applied to PEI according to the previous agreements. Per our understanding, it happens that the S monitoring occasions of a PEI occasion spans cross the border of two default DRX cycles, which has a start radio frame satisfying (SFN + PF_offset mod T =0) , as shown in the example of Figure 1.

Figure 1 One PEI-O spanning two default DRX cycles

Considering the configuration of PO/PEI-O, the definition of the reference point in the agreements of RAN1 #107-e [3] “SFN of the first PF from the current default DRX cycle where UE receives the availability indication” is not clear and shall lead to different start position of validity duration depending on which MO UE detects the L1 signaling. Due to the fact that one PEI-O or PO consists of S MOs, the reference point would be different for different UEs considering the UEs may detect the paging DCI/PEI DCI on different MOs among the S MOs.  For example, if UE1 and UE2 receive paging DCI or PEI DCI in MOs before and after the border of two consecutive default DRX cycles respectively, UE1 and UE2 have different referent points for validity durations as shown in Figure 2. However, gNB cannot know that which MO a UE detects paging DCI and PEI DCI. Thus the UE determined reference point is unknown by gNB.

- The description of “SFN of the first PF from the current default DRX cycle where UE receives the availability indication” in the agreement may cause different start point of validity durations for different UEs sharing the same PO/PEI-occasion, and needs further clarification to avoid ambiguity between gNB and UE.

Figure 2 Different reference point for different MO within the same PO/PEI-O

There were some comments that it is up to gNB to maintain consistent indication, i.e. gNB needs to trigger multiple validity durations for different MOs. However, it basically violates the motivation to introduce validity duration for TRS availability. Especially for the case where the validity duration length is configured not to be very large, e.g. 1 or 2 DRX cycle length, the ambiguity of the start point may cause the introduction of validity duration meaningless. Also, in FR2, 64 MOs of a PO/PEI-O could create complicated handling on gNB for each L1 availability indication, which is due to the ambiguity that could be easily resolved by standard work. We are concerning what is the point to introduce validity duration if availability indication would be leave to gNB implementation to handle the TRS availability.

- It violates the motivation to introduce validity duration of TRS availability if the ambiguity of the reference point caused by detected MOs on UE is left to gNB implementation handling.

To be a robust mechanism, it should be specified to have the same understanding on the validity duration between gNB and UE. An example was shown in Figure 3.

Figure 3 Aligned reference point for all the MOs within the same PO/PEI-O

Based on the above discussion, it is easier for gNB and UE to maintain the validity durations if the reference points for MOs within the same PO/PEI-O are aligned. Therefore we have the following proposal.

- Adopt TP#2 of the agreements on reference point made in RAN1 #108-e to define the reference point of validity duration based on the first MO of a PO/PEI-O where UE receives the availability indication.

  - How do CONNECTED UEs handle the TRS resources

In RAN1 #108-e, the following agreement was made [1].

Agreement - TRS-ResourceSetConfig IE is not accounted in determination of available resource elements for PDSCH.

However, it is still not determined to how to capture the above agreement in specification. According to TS38.211 [7], when the modulated complex-valued symbols are mapped to VRBs, the ‘available REs’ does not generally include the REs used for NZP-CSI-RS. But some special NZP-CSI-RS, e.g. the CSI-RS configured by CSI-RS-Resource-Mobility and AP-NZP-CSI-RS are not accounted (see the cited paragraph below). According to the description of TS38.214, TRS configured by SIB is also a kind of NZP-CSI-RS [6]. And based on the above agreement, TRS configured by SIB is also not accounted. So it is essential to update the TS 38.211 to capture the agreement. Therefore, we suggest the following TP in TS 38.211 to capture the above agreement.

- Adopt the following TP2 not to impact connected mode UE’s rate matching behavior due to TRS resource set(s) configured in SIB.

------------------------------------------ Start of Text Proposal 2------------------------------------------ < Unchanged parts are omitted > - 7.3.1.5 Mapping to virtual resource blocks The UE shall, for each of the antenna ports used for transmission of the physical channel, assume the block of complex-valued symbols yp0, …, yp(Msymbap-1) conform to the downlink power allocation specified in [6, TS 38.214] and are mapped in sequence starting with yp0 to resource elements k',lp,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission; - the corresponding physical resource blocks are declared as available for PDSCH according to clause 5.1.4 of [6, TS 38.214]; - the corresponding resource elements in the corresponding physical resource blocks are - not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled UEs as described in clause 7.4.1.1.2; - not used for non-zero-power CSI-RS according to clause 7.4.1.5 if the corresponding physical resource blocks are for a PDSCH scheduled by a PDCCH with the CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI, or a PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource or except if the non-zero-power CSI-RS is TRS resources configured via [TRS-ResourceSetConfig]; - not used for PT-RS according to clause 7.4.1.2; - not declared as 'not available for PDSCH according to clause 5.1.4 of [6, TS 38.214]. The mapping to resource elements (k',l)p,μ allocated for PDSCH according to [6, TS 38.214] and not reserved for other purposes shall be in increasing order of first the index k' over the assigned virtual resource blocks, where k'=0 is the first subcarrier in the lowest-numbered virtual resource block assigned for transmission, and then the index l. ------------------------------------------ End of Text Proposal 2------------------------------------------

Another issue is whether the TRS resource set(s) configured in SIB can be used for Rel-17 connected mode UE. It is up to UE implementation whether to utilize TRS resources configured in SIB when UE is in CONNECTED mode. No spec impact is expected.

- Conclude that it is up to UE implementation whether to utilize TRS resources for time/frequency tracking configured in SIB when UE is in CONNECTED mode.

  - Maximum number of configurable TRS resource sets

In RAN1 #107-e, it was agreed that the maximum number of configurable TRS resource sets is 64, and there is FFS regarding whether the number of configured TRS resource sets is not larger than the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 [3].

Agreement in RAN1#107 For the maximum number of TRS resource sets configured by higher layer, X, - X = 64 - FFS: the number of configured TRS resource sets is not larger than the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1

During RAN1 #107bis-e and #108-e, it is further discussed whether to reduce maximum number of configured TRS resource sets for FR1. In our view, limiting the configured TRS resource set(s) in FR1 is beneficial for the reduced memory size on UE to reduce the cost and power consumption of the UE, which would be critical for RedCap UEs. For FR1, it looks reasonable that gNB shall not configure 64 TRS resource set(s) in FR1 where at most 8 SSBs are transmitted in a SSB burst.  We support to clarify that the configured number of configurable TRS resource sets is smaller than or equal to 16, with the following summarized reasons:

- Impact on gNB in FR1: Attractive power saving gain is already provided by one or two TRS resource sets per beam direction, and in FR1 it seems there is no benefit to support such high number of TRS resource set(s) as that in FR2. Considering in FR1 there are at most 8 SSBs, 8~16 TRS resource sets are enough.

- Benefit on UE: The memory requirement for storing the configuration is reduced, which reduces the cost and power consumption of the UE, which would be critical for RedCap UEs. In RAN1 #108-e, some companies argued that the memory size for storing TRS configuration is also impacted by the supported number of TRS in CONNECTED mode, which is reported via maxConfiguredResourceSetsPerCC and maxConfiguredResourceSetsAllCC.  According to TS38.306, for FR1 UE will mandatorily report 8 for maxConfiguredResourceSetsPerCC and 16 for maxConfiguredResourceSetsAllCC. Thus for a UE only supporting FR1, it is allowed that the UE only support 16 TRS configurations. A larger number of TRS resource sets can put a higher requirement for such kind of UE, which in our view is unnecessary and unreasonable.

- It is beneficial to keep the maximum number of configurable TRS resource sets to 16 for FR1 to reduce UE memory size for low cost and power consumption, especially for RedCap UEs.

Considering it is still controversial to introduce an upper limit of the number of configured TRS resource set(s) depending on the number of actual transmitted SSBs, we propose to simply fix that the number of configured TRS resource set(s) is not expected to be larger than 16 in FR1.

- Number of configured TRS resource sets in SIB for idle/inactive UEs is not larger than 16 for FR1.

  - How to determine the number of bits of TRS availability indication field

One aspect is still open in RAN1 specification of TS 38.212, i.e. how the number of TRS availability indication field is determined. There is controversial view that whether the field length should be configured in RAN2 specification explicitly, or implicitly determined by other parameters. Based on it, RAN1 has the following agreement.

Agreement For L1 availability indication using a bitmap, the following is supported: - Number of bits in the bitmap, N, is up to 6 bits - a bit is associated with a group of TRS resource sets. The associated TRS resource sets for each bit can be based on - explicit configuration of TRS resource set group, where - each TRS resource set is configured with a ID i, with value from {0, …, N-1}, for the association with an indication bit in TRS availability indication field. - the ith bit maps to all the TRS resource set(s) associated with ID i. - start of the bitmap is the first bit of the reserved bits in paging PDCCH - Note: It is left to RAN2 decision on whether explicit parameter is used for N or it can be implicitly determined by the TRS resource set configurations.

Now in RAN2’s running CR, there is no explicit parameter provided to configure the TRS availability field. Considering this and also it is already agreed and captured in spec that gNB can configure indBitID within each TRS-ResourceSet. With all the configured TRS-ResourceSet in SIB-X, the UE can easily know how many bits in paging DCI or PEI are used to indicate TRS availability based on the configured indBitID by an implicitly way.

- Considering there is no explicit parameter to configure the TRS availability field size, the number of bits of TRS availability indication field is determined implicitly based on indBitID in all the configured TRS resource set(s) in SIB-X.

- Adopt the TP3 to determine TRS availability indication field length.

------------------------------------- Start of Text Proposal 3------------------------------------------ < Unchanged parts are omitted > - 7.3.1.2.1 Format 1_0 The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by P-RNTI: < Unchanged parts are omitted > - TRS availability indication – 1, 2, 3, 4, 5, or 6 bits according to indBitID(s) provided by the TRS-ResourceSetConfig if TRS-ResourceSetConfig is configured; 0 bits otherwise. - Reserved bits – (8 – M) bits for operation in a cell with shared spectrum channel access in frequency range 1 or for operation in a cell in frequency range 2-2; (6 – M) bits for operation in a cell without shared spectrum channel access, where the value of M is the number of bits for the field of 'TRS availability indication' as defined above. - 7.3.1.3.8 Format 2_7 The following information is transmitted by means of the DCI format 2_7 with CRC scrambled by PEI-RNTI: < Unchanged parts are omitted > - TRS availability indication – 1, 2, 3, 4, 5, or 6 bits according to indBitID(s) provided by the TRS-ResourceSetConfig if TRS-ResourceSetConfig is; 0 bits otherwise. ------------------------------------- End of Text Proposal 3------------------------------------------

  - Condition of TRS availability indication in paging PDCCH reception

In RAN1 #108-e, it was discussed whether or not to restrict transmission of TRS availability indication by Short Message indicator field, i.e. whether to re-strict that gNB can transmit TRS availability indication based on paging PDCCH only if paging PDCCH also carries scheduling information and/or short message.

In our view, paging DCI with short message indicator of value ‘00’ to indicate TRS availability implies ‘dedicated’ signaling for TRS availability indication. However, when we discuss the supporting of paging DCI to carry TRS availability indication, one important benefit is that it can reduce signaling overhead. Allowing ‘dedicated’ paging DCI for TRS availability indication seems to violate the motivation of introducing paging DCI based TRS availability indication. On the other hand, we also agreed on the validity duration, by which gNB is not required to send paging DCI only for TRS availability indication. If ‘dedicated’ paging DCI for TRS availability indication is allowed, it also violates the motivation of introducing validity duration of TRS availability.

- Allowing paging DCI with short message indicator of value ‘00’ to indicate TRS availability violates the motivation of introducing paging DCI based TRS availability indication and validity duration of TRS availability.

Besides, it is better for the network to keep this reserved DCI for other purpose in future. Therefore, we have the following proposal.

- Paging DCI with short message indicator of value ‘00’ to indicate TRS availability is not allowed.

With the above proposal, we are open to further discuss whether paging DCI with short message indicator of value ‘10’ to indicate TRS availability is allowed or not.

  - Clarify the “availability indication”

For the “TRS availability indication”, the corresponding agreements are:

Agreements in RAN1#104e For a cell with TRS/CSI-RS occasions configured for IDLE/Inactive UEs, IDLE/Inactive UE’s assumption on the availability of TRS/CSI-RS at the configured occasion(s) is informed to the idle/inactive UE based on explicit indication. - FFS details (e.g., the signalling, detailed information for the TRS/CSI-RS, etc.) - There is no intended blind detection of the presence/absence of TRS/CSI-RS at the UE side in this feature. That is, the UE assumes TRS/CSI-RS is not present if the network does not indicate it is available (or indicates it is unavailable).

It is clearly described that TRS availability means the TRS is presence on the configured TRS resource set(s) in TRS occasions. However, the current specification description in TS 38.213 uses the description of “A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets”.   This is different from what the agreement means. The configured TRS resource set(s) are always present, and the TRS of the TRS resource set(s) can be indicated presence or not presence.  Therefore, the following change is proposed to align with the agreement.

Another issue here is “0” means “no change to a current assumption for the availability or unavailability of associated TRS resource sets”. However, the current specification only specifies the case where UE assumes the TRS on the associated TRS resource is available, however, there is not any description regarding how UE obtain the “current assumption of unavailability”. Based on the discussion, a UE assumes the TRS on the associated TRS resource set unavailable if the UE has not received any TRS availability indication or the UE is not in validity duration of previously received TRS availability indication.

------------------------------------------ Start of Text Proposal 4------------------------------------------ < Unchanged parts are omitted > ## 10.4B Indication of TRS resources A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7, if peiSearchSpace is provided, and a DCI format 1_0 with CRC scrambled by P-RNTI includes a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap. The UE can be additionally provided a multiple, by validityDuration, for a number of frames provided by defaultPagingCycle for TRS resource sets with indicated presence; if validityDuration is not provided, the multiple is equal to 2. A value of '1' for a bit of the bitmap indicates presence of TRS on the associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the frame that includes a PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the presence of TRS on the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap indicates no change to a current assumption for the availability, or unavailability if the UE is not in a validity duration, of the TRS on the associated TRS resource sets. A UE can receive first and second PDCCHs that provide DCI format 2_7 or DCI format 1_0 with CRC scrambled by P-RNTI that indicate presence of TRS resource sets for the multiple of the number of frames, where the second PDCCH reception after the first PDCCH reception by a time that is smaller than the multiple of the number of frames. ------------------------------------------ Start of Text Proposal 4------------------------------------------

- Adopt TP#4 to align with the agreements.

Conclusions

According to the previous discussion, we have the following observations and proposals:

- No further clarification on monitoring occasion determination of PEI is needed.

- The description of “SFN of the first PF from the current default DRX cycle where UE receives the availability indication” in the agreement may cause different start point of validity durations for different UEs sharing the same PO/PEI-occasion, and needs further clarification to avoid ambiguity between gNB and UE.

- It violates the motivation to introduce validity duration of TRS availability if the ambiguity of the reference point caused by detected MOs on UE is left to gNB implementation handling.

- It is beneficial to keep the maximum number of configurable TRS resource sets to 16 for FR1 to reduce UE memory size for low cost and power consumption, especially for RedCap UEs.

- Allowing paging DCI with short message indicator of value ‘00’ to indicate TRS availability violates the motivation of introducing paging DCI based TRS availability indication and validity duration of TRS availability.

- Adopt TP#2 of the agreements on reference point made in RAN1 #108-e to define the reference point of validity duration based on the first MO of a PO/PEI-O where UE receives the availability indication.

- Adopt the following TP2 not to impact connected mode UE’s rate matching behavior due to TRS resource set(s) configured in SIB.

- Conclude that it is up to UE implementation whether to utilize TRS resources for time/frequency tracking configured in SIB when UE is in CONNECTED mode.

- Number of configured TRS resource sets in SIB for idle/inactive UEs is not larger than 16 for FR1.

- Considering there is no explicit parameter to configure the TRS availability field size, the number of bits of TRS availability indication field is determined implicitly based on indBitID in all the configured TRS resource set(s) in SIB-X.

- Adopt the TP3 to determine TRS availability indication field length.

- Paging DCI with short message indicator of value ‘00’ to indicate TRS availability is not allowed.

- Adopt TP#4 to align with the agreements.

- References

- Chairman's Notes RAN1#108-e.

- R1-2202651, “Moderator summary#3 on TRS/CSI-RS occasion(s) for idle/inactive UEs”, Samsung.

- Chairman's Notes RAN1#107-e.

- Chairman's Notes RAN1#107bis-e.

- TS 38.304, “NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state”.

- TS 38.214, “NR; Physical layer procedures for data”, v17.1.0.

- TS 38.211, “NR; Physical channels and modulation”, v17.1.0.

- TS 38.306, “NR; User Equipment (UE) radio access capabilities”, v17.0.0.