3GPP TSG-RAN WG1 Meeting #109-e	R1-2203086

e-Meeting, May 9 - 20, 2022

Agenda Item:	8.7.2

Source:	Huawei, HiSilicon

Title:	Remaining issues on the extensions to Rel-16 DCI-based power saving adaptation for an active BWP

Document for:	Discussion and Decision

Introduction

In this contribution, the remaining issues on DCI-based PDCCH monitoring adaptation are further analyzed and discussed.

Impact on PDCCH monitoring scrambled by C-RNTI/MCS-C-RNTI/CS-RNTI for Type 0/0A/1/2 CSS

It was agreed that PDCCH based monitoring adaptation is applied to USS and type-3 CSS. It is still open regarding whether PDCCH skipping impacts on PDCCH monitoring scrambled by C-RNTI/MCS-C-RNTI/CS-RNTI for Type 0/0A/1/2 CSS. The discussed proposal in RAN1#108-e was captured in the feature lead summary as following [1].

- [Medium] proposal 2-1 (v4) – skipping C-RNTI/... in Type 0/0A/1/2 CSS - UE also skips monitoring of PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI in any CSS and USS when UE is indicated skipping PDCCH monitoring for a duration. Note: it is previously agreed that PDCCH based monitoring adaptation is applied to USS and type-3 CSS.

For PDCCH skipping mechanism, if the UE still monitors PDCCH scrambled by C-RNTI, MCS-C-RNTI or CS-RNTI for Type 0/0A/1/2 CSS in a skipping duration, the power saving gain of PDCCH skipping would be reduced. Some companies raised that C-RNTI/MCS-C-RNTI/CS-RNTI for Type 0/0A/1/2 CSS is monitored in the slot where SI-RNTI, RA-RNTI, MsgB-RNTI, or P-RNTI is monitored, and SI-RNTI/RA-RNTI/MsgB-RNTI/P-RNTI is not always monitored by a UE. Therefore, they think the power consumption impact due to monitoring of C-RNTI/MCS-C-RNTI/CS-RNTI for Type 0/0A/1/2 CSS would be negligible.

------------------------------------------Description in TS 38.213----------------------------------------- If a UE is provided - one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, and - a C-RNTI, an MCS-C-RNTI, or a CS-RNTI the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, the MCS-C-RNTI, or the CS-RNTI in the one or more search space sets in a slot where the UE monitors PDCCH candidates for at least a DCI format 0_0 or a DCI format 1_0 with CRC scrambled by SI-RNTI, RA-RNTI, MsgB-RNTI, or P-RNTI.

However, it should be noticed that, according to following description in TS 38.331, UE shall monitor P-RNTI in any paging occasion. However, the gNB does not know the exact PO that a connected UE would monitor. So, gNB can only assume the P-RNTI is possible to be monitored on any PO and gNB may transmit a PDCCH scrambled by C-RNTI on any PO. To avoid miss-detection of DCI format 1_0 or 0_0 with CRC scrambled by a C-RNTI, UE needs monitoring PDCCH scrambled by C-RNTI on each PO even though the UE does not monitor PDCCH scrambled by P-RNTI on all of the POs. Therefore, the PDCCH monitoring of C-RNTI/MCS-C-RNTI/CS-RNTI for Type 0/1/1A/2 CSS has non-negligible power saving gain impact.

------------------------------------------Description in TS 38.304----------------------------------------- UEs in RRC_IDLE or in RRC_INACTIVE shall monitor for SI change indication in its own paging occasion every DRX cycle. UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

For the evaluation results provided by proponents, it seems the evaluation on PDCCH skipping always assumes the PDCCH can be all skipped including the PDCCH monitoring for DCI format 0_0 and 1_0. To guarantee the power saving gain as expected, it is proposed not to monitor PDCCH scrambled by C-RNTI, MCS-C-RNTI or CS-RNTI in Type 0/1/1A/2 CSS during the skipped duration. The following text proposal is provided.

Proposal 1: Do not monitor DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, MCS-C-RNTI, or the CS-RNTI in Type0/0A/1 or 2 PDCCH CSS during the PDCCH skipping duration.

Proposal 2: adopt the TP#1 to capture that monitoring of DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, MCS-C-RNTI, or the CS-RNTI in Type0/0A/1 or 2 PDCCH CSS is skipped during the PDCCH skipping duration.

-------------------------------------Start of Text Proposal 1 for TS 38.213 h10------------------------ < Unchanged parts are omitted > - 10.1 UE procedure for determining physical downlink control channel assignment < Unchanged parts are omitted > If a UE is provided - one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, and - a C-RNTI, an MCS-C-RNTI, or a CS-RNTI the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, the MCS-C-RNTI, or the CS-RNTI in the one or more search space sets in a slot where the UE monitors PDCCH candidates for at least a DCI format 0_0 or a DCI format 1_0 with CRC scrambled by SI-RNTI, RA-RNTI, MsgB-RNTI, or P-RNTI, - if the UE is not provided PDCCHSkippingDurationList; or, - if the UE is provided PDCCHSkippingDurationList and the slot is not overlapping with a duration, where the UE is indicated skipping PDCCH monitoring. < Unchanged parts are omitted > - 10.4 Search space set group switching and skipping of PDCCH monitoring < Unchanged parts are omitted > A UE can be provided a set of durations by PDCCHSkippingDurationList for PDCCH monitoring on an active DL BWP of a serving cell and, if the UE is not provided searchSpaceGroupIdList-r17 on the active DL BWP of the serving cell, a DCI format 0_1 and a DCI format 0_2 that schedule PUSCH transmission, and a DCI format 1_1 and a DCI format 1_2 that schedule PDSCH receptions can include a PDCCH monitoring adaptation field of 1 bit or of 2 bits. If the field has 1 bit and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on the active DL BWP of the serving cell - a '0' value for the bit indicates no skipping in PDCCH monitoring - a '1' value for the bit indicates skipping PDCCH monitoring for a duration provided by the first value in the set of durations If the field has 2 bits and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on the active DL BWP of the serving cell - a '00' value for the bits indicates no skipping in PDCCH monitoring - a '01' value for the bits indicates skipping PDCCH monitoring for a duration provided by the first value in the set of durations - a '10' value for the bits indicates skipping PDCCH monitoring for a duration provided by the second value in the set of durations - a '11' value for the bits indicates skipping PDCCH monitoring for a duration provided by the third value in the set of durations, if any; otherwise, if the set of durations includes two values, a use of the '11' value is reserved < Unchanged parts are omitted > A UE can be provided a set of durations by PDCCHSkippingDurationList and group indexes for a Type3-PDCCH CSS set or USS set by searchSpaceGroupIdList-r17 for PDCCH monitoring on an active DL BWP of a serving cell and, a DCI format 0_1 and a DCI format 0_2 that schedule PUSCH transmissions, and a DCI format 1_1 and a DCI format 1_2 that schedule PDSCH receptions can include a PDCCH monitoring adaptation field of 2 bits. If the set of durations includes one value and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on the active DL BWP of the serving cell - a '00' value for the bits indicates start of PDCCH monitoring according to search space sets with group index 0 and stop of PDCCH monitoring according to search space sets with group index 1, if any - a '01' value for the bits indicates start of PDCCH monitoring according to search space sets with group index 1 and stop of PDCCH monitoring according to search space sets with group index 0, if any, and the UE sets the timer value to the one provided by searchSpaceSwitchTimer-r17 - a '10' value for the bits indicates skipping PDCCH monitoring for a duration provided by the value in the set of durations - a '11' value is reserved If the set of durations includes two values and for PDCCH monitoring by the UE according to Type3-PDCCH CSS sets or USS sets on active DL BWP of the the serving cell - a '00' value for the bits indicates start of PDCCH monitoring according to search space sets with group index 0 and stop of PDCCH monitoring according to search space sets with group index 1, if any - a '01' value for the bits indicates start of PDCCH monitoring according to search space sets with group index 1 and stop of PDCCH monitoring according to search space sets with group index 0, if any, and the UE sets the timer value to the one provided by searchSpaceSwitchTimer-r17 - a '10' value for the bits indicates skipping PDCCH monitoring for a duration provided by the first value in the set of durations - a '11' value for the bits indicates skipping PDCCH monitoring for a duration provided by the second value in the set of durations When the UE is indicated skipping PDCCH monitoring for a duration on a serving cell, the UE skips monitoring of PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, the MCS-C-RNTI, or the CS-RNTI in the one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, in the duration on the serving cell. < Unchanged parts are omitted > -------------------------------------End of Text Proposal 1 for TS 38.213 h10--------------------------

Application delay and handling of HARQ retransmission

In RAN1#108-meeting, application delay and handling of HARQ retransmission for PDCCH monitoring adaptation was discussed. The following progress was achieved.

Working Assumption - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating SSSG switching (i.e., Beh 2/2A/2B), - the UE applies SSSG switching on an active BWP of the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception - FFS: a minimum applicable scheduling offset is configured in the BWP - Note: Pswitch is defined in Table 10.4-1 in TS38.213 Agreement - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating PDCCH skipping (i.e., Beh 1A), - Alt 1a: the UE applies Beh 1A on an active BWP of the serving cell at the first slot after the last OFDM symbol of the PDCCH reception - FFS: a minimum applicable scheduling offset is configured in the BWP - FFS: whether the UE will monitor PDCCH when DRX Retransmission timer is running

Application delay

The application delays of PDCCH skipping and SSSG switching have been agreed for the case when minimum scheduling offset restriction is not configured for the active BWP. The working assumption in RAN1#108 should be confirmed.

Proposal 3: confirm the working assumption of application delay of SSSG switching in RAN1#108:

Working Assumption - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating SSSG switching (i.e., Beh 2/2A/2B), - the UE applies SSSG switching on an active BWP of the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception - FFS: a minimum applicable scheduling offset is configured in the BWP Note: Pswitch is defined in Table 10.4-1 in TS38.213

It is still FFS on application delay for SSSG switching and PDCCH skipping when minimum scheduling offset is configured for the active BWP.

If cross slot scheduling is enabled, i.e., K0min>0, the PDCCH processing speed is allowed to be relaxed according to the discussion in Rel-16. A UE can only perform PDCCH skipping after the DCI is decoded, therefore the UE should only perform the PDCCH skipping at least after K0min slots, during which the UE may need to process the indicating DCI. Therefore, if DCI indicates the UE in slot n to skip PDCCH monitoring, the PDCCH skipping is applied from the slot n+X, where X is the the application delay of  max (applicable K0min, Z).

Similarly, if cross slot scheduling is enabled, K0min should be also considered for application delay of SSSG switching, i.e., if DCI indicates in slot n the UE to switch SSSG, the SSSG switching is applied from the slot n+X, where X is the application delay of max (applicable K0min, Z).

Proposal 4: If minimum scheduling offset is configured in the active BWP, the application delay of PDCCH skipping or SSSG switching is at least max (applicable K0min, Z), where K0min is the current applied minimum scheduling offset in the active BWP.

Proposal 5: Adopt the Text Proposal 2 for the application delay of PDCCH monitoring adaptation.

---------------------------------- Start of Text Proposal 2 for TS 38.213 h10--------------------------------- < Unchanged parts are omitted > - 10.4 Search space set group switching and skipping of PDCCH monitoring < Unchanged parts are omitted > When the PDCCH monitoring adaptation field indicates to a UE to start PDCCH monitoring according to search space sets with a first group index and stop PDCCH monitoring according to search space sets with a second group index, the UE applies the indication at the beginning of the earliest slot, - at the beginning of a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field when μ∈0, 1, 2, 3, - at the beginning of a first slot, of a slot group of Xs slots, that is at least Pswitch symbols after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field when μ∈5, 6, - that is at least max(K0minOld, Zµ) slots after the slot of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field if minimumSchedulingOffsetK0 is configured for the active DL BWP, where K0minOld is the currently applied K0min value of the active DL BWP and Zµ is determined by the subcarrier spacing of the active DL BWP in the scheduling cell in slot n, and given in Table 5.3.1-1 in TS 38.214. When the PDCCH monitoring adaptation field indicates to a UE to skip PDCCH monitoring for a duration on the active DL BWP of a serving cell, the UE starts skipping of PDCCH monitoring at the beginning of a first slot that is after the last symbol of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field if minimumSchedulingOffsetK0 is not configured for the active DL BWP; otherwise, the UE starts skipping of PDCCH monitoring at the beginning of the first slot that is max(K0minOld, Zµ) slots after the slot of the PDCCH reception providing the DCI format with the PDCCH monitoring adaptation field, where K0minOld is the currently applied K0min value of the active DL BWP and Zµ is determined by the subcarrier spacing of the active DL BWP in the scheduling cell in slot n, and given in Table 5.3.1-1 in TS 38.214. < Unchanged parts are omitted > ----------------------------End of Text Proposal 2 for TS 38.213 h10-----------------------------------

Handling of HARQ retransmission

It is FFS that whether the PDCCH skipping should consider the interaction with HARQ retransmission, which is desired by proponents.

One issue is the PDCCH skipping duration can be as long as 100ms. If the retransmission is not handled during the skipping duration, the overall data service latency shall be impacted significantly, and the UE perceived throughput shall be also impacted significantly.

There are some different views that gNB could always handle it by implementation, i.e. when indicating PDCCH skipping in a scheduling DCI, the gNB can schedule more resource or repetitions to use lower coding rate to improve the reliability of the initial transmission of the scheduled PDSCH/PUSCH. However, this would not really resolve the issue and may always pay the additional resource overhead to handle this potential issue, which would be not always achievable when the load is heavy in the cell. A more robust mechanism is always better.

Another issue is the UE may falsely detect a scheduling DCI which may also false indicating a PDCCH skipping for a duration as long as 100ms. Assuming the SCS is 30kHz and the UE blindly detects 36 PDCCH candidates in every slot, then in every second the UE may have about 0.00858 PDCCH candidates which are falsely detected as a detected DCI, which is calculated by 36*0.5*1000*2^-21. Therefore, for every 117 seconds, a false detected DCI shall happen on the UE. If a scheduling DCI is falsely detected, there are 50% and 75% percentage that the PDCCH adaptation field in the falsely detected DCI is indicating a PDCCH skipping, for the case of 1 bit and 2bits PDCCH adaptation fields, respectively. Although the probability of false detection rate of DCI is as low as 2^-21, a falsely detected PDCCH skipping still causes 100ms scheduling interruption for every 3.9 minutes (334 seconds) or 2.6 minutes (156 seconds) respectively. If the PDCCH skipping solution does not have any mechanism to work robustly when a falsely PDCCH skipping indication happens, the user experience shall be still impacted significantly in this case.

Retransmission handling can be taken as a mechanism in addition to handle the issue of falsely detected PDCCH skipping. If a UE is allowed to detect the DCI during the retransmission timer of C-DRX, the UE could find that the current skipped duration was indicated by a falsely detected DCI considering the UE could receive the scheduling that is really transmitted by the gNB during the retransmission timer.

According to discussions in previous meetings, the potential retransmission handling is listed as following.

- Alt 1: During PDCCH skipping duration, the UE still performs PDCCH monitoring (i.e., Beh 1) when drx-RetransmissionTimerDL/UL is running

- Alt 2:  For PDCCH skipping indication via 1-1/1-2, if the UE fails to decode the associated PDSCH and/or transmit a NACK, UE performs Beh 1 (i.e., the PDCCH skipping is stopped) at the first slot after the last OFDM symbol of the NACK transmission.

- Alt 3:  the UE applies Beh 1A next slot after the last OFDM symbol of ACK transmission, otherwise the indication is not applied.

Based on the discussion in RAN1#108bis, most companies seem fine with using the DL retransmission timer in Alt.1, i.e. during PDCCH skipping duration the UE still performs PDCCH monitoring (i.e., Beh 1) when drx-RetransmissionTimerDL is running, considering it can have the minimal standard effort to reuse the legacy timer in C-DRX. By this way UE monitors PDCCH after it feedbacks NACK, which is beneficial for reducing the latency of service and can handle DCI false alarm issue. When the retransmission timer expires, UE continues to skip PDCCH monitoring in the remaining PDCCH skipping duration as shown in Figure 1.

Figure 1 An example for overriding the ‘PDCCH skipping indication’

Proposal 6: Adopt the DL retransmission timer in Alt.1 to handle the retransmission of PDCCH skipping, i.e. the UE still performs PDCCH monitoring for HARQ retransmission when drx-RetransmissionTimerDL is running.

Certainly, the same mechanism can also be applied for uplink to ensure timely retransmission, i.e. UE can monitor PDCCH for potential UL HARQ retransmission when the uplink DRX retransmission timer is running within the skipped duration. However, there is no HARQ feedback for uplink. UE does not know when and whether there is retransmission, and will always start the UL retransmission timer after UL HARQ RTT timer. In the worst case, even the PUSCH is successfully received by gNB, UE still needs to perform PDCCH monitoring. So compared with downlink, PDCCH monitoring based on uplink retransmission timer has more power saving loss.

Proposal 7: Adopt the Text Proposal 3 on the handling of HARQ retransmission.

---------------------------------- Start of Text Proposal 3 for TS 38.213--------------------------------- < Unchanged parts are omitted > - 10.4 Search space set group switching and skipping of PDCCH monitoring < Unchanged parts are omitted > When a UE receives - a first PDCCH in a first slot that provides a DCI format with a PDCCH monitoring adaptation field having a first value indicating skipping PDCCH monitoring, or indicating start of PDCCH monitoring according to a search space sets with a first group index and stop of PDCCH monitoring according to search space sets with a second group index, for an active DL BWP and - a second PDCCH that provides a DCI format with a PDCCH monitoring adaptation field having a second value indicating skipping PDCCH monitoring, or indicating start of PDCCH monitoring according to search space sets with a first group index and stop of PDCCH monitoring according to search space sets with a second group index different than the first group index, for the active DL BWP where the second PDCCH is received - in the first slot if the first value indicates skipping PDCCH monitoring - before a slot that is at least Pswitch symbols after the first slot if the first value indicates start of PDCCH monitoring according to search space sets with a first group index the UE does not expect the second value to be different than the first value. If a UE is indicated skipping PDCCH monitoring for a duration by a DCI format 1_1 and/or DCI format 1_2 that schedules a PDSCH reception, during the duration the UE monitors PDCCH when the drx-RetransmissionTimerDL of any HARQ process is running. If a PDCCH monitoring adaptation field is received during the duration for skipping PDCCH monitoring, the indication of PDCCH monitoring adaptation field is ignored by the UE. < Unchanged parts are omitted > -------------------------------- End of Text Proposal 3 for TS 38.213-----------------------------------

Interaction with BWP switching

Monitored search space set group after a BWP switching triggered by timer

Based on the agreement in RAN1#107bis meeting, PDCCH skipping and SSSG switching is configured per BWP. When multiple BWPs are configured, the active BWP can be switched by DCI indication or bwp-InactivityTimer.

Agreement - The initial timer value for switching from SSSG#2 to SSSG#0 and from SSSG#1 to SSSG#0 is common and configured per BWP. - The PDCCHSkippingDurationList-r17 is configured per BWP.

For timer-based BWP switching, when the bwp-InactivityTimer expires, the UE will fall back to default BWP. When a scheduling DCI is received, the bwp-InactivityTimer shall restart. However, if a skipped duration is triggered and larger than bwp-InactivityTimer (the skipping duration can be as large as 100ms), there would be no PDCCH monitoring and UE will switch BWP before the end of PDCCH skipping duration. In this case, as shown in Figure 2, UE behavior on the new active BWP needs to be discussed. As PDCCH monitoring adaptation is configured per BWP, a straightforward way is when the UE switches to the new active BWP due to the expiration of bwp-InactivityTimer, the UE starts monitoring PDCCH on the active BWP. Therefore, we have the following proposal.

Figure 2  PDCCH skipping for a duration and timer-based BWP switching

Proposal 8: If the active BWP is switched due to expiration of bwp-InactivityTimer before the end of a PDCCH skipping duration, the default behavior on the new active BWP is

- UE starts monitoring PDCCH according to the configured SS sets if SSSG is not configured on the new active BWP;

- UE starts monitoring PDCCH according to the SS set(s) associated with SSSG#0 if SSSG is configured on the new active BWP.

Interpretation of PDCCH monitoring adaptation field in the DCI indicating a BWP switching

For DCI-based BWP switching, PDCCH monitoring adaptation and BWP switching can be indicated simultaneously in the same scheduling DCI. As PDCCH skipping durations and search space set groups are configured per BWP, the PDCCH monitoring adaptation field in the scheduling DCI indicating a BWP switching indicates the behaviors  on the indicated target BWP, according to the Rel-15 rule in cross-BWP scheduling. If a PDCCH skipping duration is configured on the target BWP, it is flexible for gNB to determine whether or not to indicate skipping PDCCH monitoring and BWP switching simultaneously. For example, for UE power saving, gNB can indicate the UE switching BWP from wide bandwidth to narrow bandwidth, an also indicate PDCCH skipping for a duration in the same scheduling DCI. In this case, the indicated value of the skipped duration is one of the values configured on the target BWP. Similarly, if SSSG switching is configured on the target BWP, the indicated SSSG is one of the SSSGs for the target BWP.

Observation 1: If PDCCH skipping/SSSG switching and BWP switching are indicated simultaneously by the same scheduling DCI, the PDCCH monitoring adaptation field in the DCI indicates the PDCCH monitoring adaptation on the target BWP, which has no specification impact reusing the same as R15 rule in cross-BWP scheduling.

Application delay on the target BWP

In the case of cross-BWP scheduling, there is a necessity to discuss when the PDCCH skipping or SSSG switching take effect when BWP switching is also triggered, which was also discussed for cross-slot scheduling based power saving in Rel-16 WI. Figure 7 shows an example of the BWP switching delay and application delay for PDCCH skipping, there are two options regarding the application time to start PDCCH skipping:

- Option 1: PDCCH skipping duration starts after the application delay of PDCCH skipping.

- Option 2: PDCCH skipping duration starts after BWP switching delay or from the slot indicated by the slot offset value of the time domain resource assignment field in the DCI format.

For BWP switching, the UE is not required to receive or transmit in the cell during the time duration from the end of the third symbol of a slot where the UE receives the PDCCH carrying the DCI format in a scheduling cell until the beginning of a slot indicated by the slot offset value of the time domain resource assignment field in the DCI format. From the perspective of UE power saving, it is preferred that PDCCH skipping duration is not overlapped with BWP switching delay. Hence, Option 2 is preferred.

Figure 3 BWP switching delay and application delay for PDCCH skipping

If SSSG switching and BWP switching are indicated simultaneously by the same scheduling DCI, to have a unified design, a simplest way is also to define the indicated SSSG takes effect from the slot indicated by the slot offset value of the time domain resource assignment field in the DCI format.

Proposal 9: In case of a scheduling DCI indicating PDCCH monitoring adaptation and BWP switching simultaneously,

- If PDCCH skipping is indicated, the skipping duration starts from the slot indicated by the slot offset value of the time domain resource assignment field in the DCI format;

- If SSSG switching is indicated, the indicated SSSG takes effect from the slot indicated by the slot offset value of the time domain resource assignment field in the DCI format.

Impacts on SR or RACH procedure

In RAN1#108-meeting, the following agreement was achieved.

Agreement Answer 2 for R1-2200884 (R2-2201960): RAN1 would discuss and conclude how to capture the above RAN2 agreements in RAN1 specification. - It is RAN1 understanding PDCCH skipping is not applied to perform PDCCH monitoring during RAR window/MsgB window/contention resolution timer or when SR is pending. - FFS: If the UE is skipping PDCCH monitoring for a duration and at the first slot after the last OFDM symbol of a positive SR transmission in PUCCH, the UE stops PDCCH skipping (i.e., PDCCH skipping is not activated ).

It is straight forward to understand that one important motivation for stopping PDCCH skipping by SR or RACH related procedure is to reduce the latency of UL traffic. After the UL traffic arrives, the UE should resume PDCCH monitoring as soon as possible. The remaining issue is what is the time point when the UE resumes PDCCH monitoring in SR procedure. It is possible that a SR is pending on a UE but the UE has not transmitted SR to gNB. Before the reception of SR, gNB assumes the UE is performing PDCCH skipping and does not transmit PDCCH in the PDCCH skipping duration. If UE immediately resumes PDCCH monitoring once SR is pending but has not been transmitted, the power consumption benefit of PDCCH skipping shall be eliminated. There is no motivation nor benefit for a UE to monitor PDCCH when the SR is pending but has not been transmitted. Considering this, we have the following proposal.

Proposal 10: If the UE is skipping PDCCH monitoring for a duration and at the first slot after the last OFDM symbol of a positive SR transmission in PUCCH, the UE stops PDCCH skipping in the duration.

The same issue also exists for SSSG switching. Assuming that the UE is monitoring PDCCH according to a first SSSG with large PDCCH periodicity, when UL traffic arrives, if the UE keeps monitoring current SSSG, the latency can be large. Therefore, to reduce the latency of UL traffic, similar mechanism can also be applied to SSSG switching, e.g. the UE will switch to another SSSG with short PDCCH periodicity during RAR window/MsgB window/contention resolution timer (i.e. after PRACH is transmitted), or after SR is transmitted. Specifically, for SR case, UE can switch to another SSSG with shorter PDCCH periodicity at the first slot after the last OFDM symbol of a positive SR transmission in PUCCH.

Proposal 11: Support SSSG switching to a SSSG with shorter PDCCH monitoring periodicity when RAR window/MsgB window/contention resolution timer is running or at the first slot after the last OFDM symbol of a positive SR transmission in PUCCH .

Interaction between PDCCH skipping and C-DRX

In RAN1#108-meeting, impacts on WUS was discussed and the following agreement was achieved.

Agreement It is RAN1 understanding that for DRX operation, - As DCP is monitored only outside active time, it cannot be missed due to PDCCH skipping as skipping applies only in active time. Note: RAN1 is discussing whether PDCCH skipping duration can apply to outside active time.

The remaining issue is whether the PDCCH skipping duration is counted outside C-DRX active time. In general, 3 options are listed as following.

- Alt 1: Suspend the PDCCH skipping duration outside active time and continue counting the PDCCH skipping duration in active time of the next C-DRX cycle;

- Alt 2: PDCCH skipping duration applies to outside active time, i.e., continue counting the PDCCH skipping duration outside active time;

- Alt 3: Stop the PDCCH skipping duration outside active time.

By Alt 2, PDCCH skipping duration is a consecutive duration regardless of the state of C-DRX, there is no additional specification impact.

By Alt 1, UE needs to suspend the counting of PDCCH skipping duration, which increase the UE implementation. , moreover it would increase the gNB complexity to predict how long the UE is assumed to be without scheduling, i,e, gNB indicates the PDCCH skipping duration by predicting that there would be no traffic in a duration, and in Alt.1 the duration that UE does not monitor PDCCH will be larger than the duration indicated by gNB.

By Alt 3, UE needs to be able to cancel of skipped duration when outside Active time, which increases the UE implementation complexity. The duration that UE does not monitor PDCCH may be smaller than the duration indicated by gNB. It will increase gNB’s complexity of prediction.

Based on the above analysis, we propose Alt 2 is adopted.

Proposal 12: Continue counting the PDCCH skipping duration outside active time.

Conclusion

In this contribution, remaining issues on PDCCH monitoring adaptation is discussed. Based on the discussion, we have the following observations and proposals.

Observation 1: If PDCCH skipping/SSSG switching and BWP switching are indicated simultaneously by the same scheduling DCI, the PDCCH monitoring adaptation field in the DCI indicates the PDCCH monitoring adaptation on the target BWP, which has no specification impact reusing the same as R15 rule in cross-BWP scheduling.

Proposal 1: Do not monitor DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, MCS-C-RNTI, or the CS-RNTI in Type0/0A/1 or 2 PDCCH CSS during the PDCCH skipping duration.

Proposal 2: adopt the TP#1 to capture that monitoring of DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, MCS-C-RNTI, or the CS-RNTI in Type0/0A/1 or 2 PDCCH CSS is skipped during the PDCCH skipping duration.

Proposal 3: confirm the working assumption of application delay of SSSG switching in RAN1#108:

Working Assumption - Upon detecting a scheduling DCI format 1-1/1-2/0-1/0-2 indicating SSSG switching (i.e., Beh 2/2A/2B), - the UE applies SSSG switching on an active BWP of the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH reception - FFS: a minimum applicable scheduling offset is configured in the BWP Note: Pswitch is defined in Table 10.4-1 in TS38.213

Proposal 4: If minimum scheduling offset is configured in the active BWP, the application delay of PDCCH skipping or SSSG switching is at least max (applicable K0min, Z), where K0min is the current applied minimum scheduling offset in the active BWP.

Proposal 5: Adopt the Text Proposal 2 for the application delay of PDCCH monitoring adaptation.

Proposal 6: Adopt the DL retransmission timer in Alt.1 to handle the retransmission of PDCCH skipping, i.e. the UE still performs PDCCH monitoring for HARQ retransmission when drx-RetransmissionTimerDL is running.

Proposal 7: Adopt the Text Proposal 3 on the handling of HARQ retransmission.

Proposal 8: If the active BWP is switched due to expiration of bwp-InactivityTimer before the end of a PDCCH skipping duration, the default behavior on the new active BWP is

- UE starts monitoring PDCCH according to the configured SS sets if SSSG is not configured on the new active BWP;

- UE starts monitoring PDCCH according to the SS set(s) associated with SSSG#0 if SSSG is configured on the new active BWP.

Proposal 9: In case of a scheduling DCI indicating PDCCH monitoring adaptation and BWP switching simultaneously,

- If PDCCH skipping is indicated, the skipping duration starts from the slot indicated by the slot offset value of the time domain resource assignment field in the DCI format;

- If SSSG switching is indicated, the indicated SSSG takes effect from the slot indicated by the slot offset value of the time domain resource assignment field in the DCI format.

Proposal 10: If the UE is skipping PDCCH monitoring for a duration and at the first slot after the last OFDM symbol of a positive SR transmission in PUCCH, the UE stops PDCCH skipping in the duration.

Proposal 11: Support SSSG switching to a SSSG with shorter PDCCH monitoring periodicity when RAR window/MsgB window/contention resolution timer is running or at the first slot after the last OFDM symbol of a positive SR transmission in PUCCH .

Proposal 12: Continue counting the PDCCH skipping duration outside active time.

- References

R1-2202895, Final FL summary of DCI-based power saving adaptation, RAN1#108-e meeting

Chairman’s Notes RAN1#108-e