3GPP TSG-RAN WG1 Meeting #109-e	R1-2203087

e-Meeting, May 9 – 20, 2022

Agenda Item:	8.16.7

Source:	Huawei, HiSilicon

Title:	Rel-17 UE features for UE power saving enhancements

Document for:	Discussion and Decision

- Introduction

This contribution includes our views on UE features for Rel-17 UE power saving enhancement based on RAN1 agreements/conclusions to address remaining issues for UE power saving features since RAN1#108 meeting.

- Discussion

- UE feature for paging enhancement (UE feature 29-1)

The following UE capability is agreed in RAN1#108 for FG 29-1.

| In RAN1#108, the following UE feature 29-1 was agreed to be introduced. Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support receiving paging early indication in DCI format 2_7 2. Support receiving UE subgroup indication in DCI format 2_7 [3. For type 2A CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, 2 or 2A CSS configurations within a single span of three consecutive OFDM symbols within a slot] |  | Yes |  | UE does not support paging enhancement | Per band | N/A | N/A | N/A |  | Optional with capability signalling |

Regarding the capability of monitoring PDCCH for DCI format 2_7, the common understanding is based on the legacy mandatory UE capability, i.e. FG 3-1 in 38.822.

The related description in FG 3-1 is as following:

“- For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within a slot”

There is no description about the new agreed Type2A CSS for PEI PDCCH in the FG3-1. To keep no touch on FG 3-1, which is stable from Rel-15, it is proposed to have a similar sentence to FG 29-1 to capture the above UE mandatory capability to support 29-1, i.e. the component 3 currently in the square brackets:

“- For type 2A CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, 2 or 2A CSS configurations within a single span of three consecutive OFDM symbols within a slot”

Therefore, the square brackets of component 3 should be removed.

Proposal 1: Remove the square brackets of component 3 in UE feature 29-1.

- TRS resources for idle/inactive UEs (UE feature 29-2)

The following UE capability is agreed in RAN1#108 for FG 29-2:

| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occasions for idle/inactive UEs 1. Support reading TRS configuration from SIB [2. Support receiving L1 indication for TRS availability] |  | N |  | UE cannot receive TRS resources for idle/inactive mode | N/A | N/A | N/A | N/A | [Receiving L1 indication via DCI format 2_7 is supported only if the UE supports receiving DCI format 2_7] | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

As discussed in previous meeting, we are fine to remove the square brackets of component 2, however, we don’t think the note is necessary in the table.

Proposal 2: Remove the square brackets of component 2, and delete all the content in the note column for FG 29-2.

- Conclusion

In this contribution, we have following proposals on UE features for Rel-17 UE power saving enhancement:

Proposal 1: Remove the square brackets of component 3 in UE feature 29-1.

Proposal 2: Remove the square brackets of component 2, and delete all the content in the note column for FG 29-2.

- References

- R1-2202928	Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e , RAN1#108

- R1-2201516, 	Summary on UE features for UE power saving enhancements, Moderator (NTT DOCOMO, INC.)

- Chair’s note of RAN1#107b-e.