3GPP TSG-RAN WG1 Meeting #109-e		                                   					       												           R1-2203090

e-Meeting, May 9 – May 20 , 2022

Agenda Item:	8.16.4

Source:	Huawei, HiSilicon

Title:	Rel-17 UE features for NR NTN

Document for:	Discussion and Decision

# Introduction

In RAN1#108-e, a list of Rel-17 UE features for NR NTN were agreed as baseline for further discussions [1]. In this contribution, we address the remaining aspects.

# Discussion

FG 26-1

- Comment 1: On the note in the last column, our view is that it is not strictly needed. The reason is that this FG is defined per band and it is an optional UE capability. A UE only needs to report what it supports on the particular band. The note would be needed for a per UE capability but not for per band UE capability. However, there is a similar situation in Rel-16 NR-U specific per-band FGs. For NR-U, a note “the signaling is per band but is only expected for a band where shared spectrum channel access must be used” is added even though it is NOT explicitly captured in 38.306. Overall, either way may be fine for this particular FG since it is a NTN-specific feature. To simplify the description, we suggest to remove note in last column.

| 26. NR_NTN_solutions | 26-1 | Uplink Time and Frequency pre-compensation and timing relationship enhancements | - UE specific TA calculation based on its GNSS-acquired position and the serving satellite ephemeris. - UE applies calculates common TA according to the parameters provided by the network [(UE considers common TA as 0 if the parameter is not provided)] - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - FFS: UE pre-compensates the calculated TA in its uplink transmissions - Support of estimating UE-gNB RTT and delaying the start of RAR window [by UE-gNB RTT] - Support of frequency pre-compensation to counter shift the Doppler experienced on the service link - Determining timing of the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset if indicated - Determining timing of the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated and determining the timing of PDCCH monitoring in recovery search space using K-mac during beam failure recovery procedure - UE receives cell-specific K_offset/K_mac in system information |  | No | No | Release 17 NR UE cannot communicate via access [NTN/satellite/HAPS/ATG] | [Per UE/per band] | No | No | No | An NTN UE is required to at least support UE specific TA and frequency calculation based at least on its GNSS-acquired position and the serving satellite ephemeris | Optional with capability signalling For UE supports NR communication via [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR cell for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; NR NTN cell and ATG cell, for terrestrial any other cell except for ARG cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 26-4

- Comment 1: On the note in the last column, same comment as for FG26-1.

| 26. NR_NTN_solutions | 26-4 |  | UE reporting of information about related to the UE specific TA pre-compensation | Support UE reporting of information about related to the UE specific TA pre-compensation [The exact content of UE reporting of information about the UE specific TA pre-compensation e.g., frequency of the reports, granularity of the reported conten, etc.] | [26-1] | Yes | No | UE does not support reporting of information related to TA pre-compensation for NR communication via satellite | [Per UE/per band] | No | No | No | Note: The exact content of UE reporting of information about the TA pre-compensation is up to RAN2 | Optional with capability signalling [Note: This UE feature group is applicable only for NR cell for NR NTN cell and ATG cell, for terrestrial cell except for ARG cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 26-5

- Comment 1: On the note in the last column, the situation is a bit different compared to other “NTN-specific” FGs. As discussed in [1], the support of 32 HARQ processes can be beneficial for other scenarios for a UE supporting this feature, e.g. used for FR1 licensed terrestrial bands and FR2-1. For example, for the support of FR1 + FR2 CA, if the PUCCH is transmitted on FR1 with small SCS, then it can be expected that a small number of slots on FR1 would correspond to large number of slots on FR2, which will result in the need of larger number of HARQ processes on FR2, otherwise it will degrade the system performance for FR2. Another example scenario that would be beneficial from the support of 32 HARQ processes is multi-TRP case, in which case the need of number of HARQ processes would be increased. As a matter of fact, the possibility to extend the 32 HARQ processes has been discussed before as summarized in [2]. Consensus cannot be reached due to different flavors of HARQ process ID indication. However, in NTN, the explicit HARQ process indication has already been agreed. Therefore, there seems no obstacles to extend this FG in other scenarios. Overall, we see a clear benefit of extending this FG to other scenarios and suggest to remove the note in the last column.

| 26. NR_NTN_solutions | 26-5 | Increasing the number of HARQ processes | The maximal supported HARQ process number is X for UL and Y for DL |  | Yes | No | Increased number of HARQ processes is not supported for NR communication via satellite | [Per band or per FSPC or per UE] | No | No | No | Candidate component values for (X,Y): {(16,32),(32,16),(32,32)} | Optional with capability signalling [Note: This UE feature group is applicable only for NR cell for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; NR NTN cell and ATG cell, for terrestrial any other cell except for ARG cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 26-6

- Comment 1: On the note in the last column, same comment as for FG26-1.

- Comment 2: Remove FFS in 5-th column.

| 26. NR_NTN_solutions | 26-6 | [HARQ feedback disabling for] Type-2 HARQ codebook enhancement | - [Support of HARQ feedback disabling] - Support of type-2 HARQ codebook enhancements when there are for feedback-disabled HARQ processes | FFS | Yes | No | Type-2 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No |  | Optional with capability signalling [Note: This UE feature group is applicable only for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; for any other cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 26-6a

- Comment 1: On the note in the last column, same comment as for FG26-1.

- Comment 2: Remove FFS in 5-th column.

| 26. NR_NTN_solutions | 26-6a | [HARQ feedback disabling for] Type-1 HARQ codebook enhancement | - [Support of HARQ feedback disabling] - Support of Type-1 HARQ codebook enhancements when there are for feedback-disabled HARQ processes | FFS | Yes | No | Type-1 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No |  | Optional with capability signalling [Note: This UE feature group is applicable only for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; for any other cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 26-6b

- Comment 1: On the note in the last column, same comment as for FG26-1.

- Comment 2: Remove FFS in 5-th column.

| 26. NR_NTN_solutions | 26-6b | [HARQ feedback disabling for] Type-3 HARQ codebook enhancement | - [Support of HARQ feedback disabling] - Support of Type-3 HARQ codebook enhancements when there are for feedback-disabled HARQ processes | FFS | Yes | No | Type-3 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No |  | Optional with capability signalling [Note: This UE feature group is applicable only for NR cell for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; for any other cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 26-8

- Comment 1: On the note in the last column, same comment as for FG26-1.

- Comment 2: Remove “[For UE supports  NR communication via [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported]”.

| 26. NR_NTN_solutions | 26-8 | Support of polarization [signalling/ information/reception] in NR NTN | - Support polarization indication reception in SIB indicating DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linear - Support polarization signalling for target serving cell in handover command message - Support polarization signalling for non-serving cell in RRM measurement configuration - FFS: polarization information for DL is indicated in SIB by the network |  | No | No | [It is assumed by the network that UE supports at least linear polarization] UE is not able to take the advantage of polarization information to save power | [Per UE/Per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] No | [Value range for component 1: {(RHCP, LHCP, Linear), (RHCP, LHCP), (RHCP), (LHCP), (Linear)}] FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR communication via [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR cell for NR NTN cell and ATG cell, for terrestrial cell except for ARG cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG 26-9

- Comment 1: On the note in the last column, same comment as for FG26-1.

| 26. NR_NTN_solutions | 26-9 | UE-specific K_offset | 1. Reception of UE-specific K_offset via MAC-CE 2. Determining the timing of PUSCH, PUCCH [and PDCCH ordered PRACH], CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with UE-specific Koffset | 26-1, 26-3, 26-4 | Yes | No | UE-specific K_offset reception is not supported for NR communication via satellite | FFS Per band | No | No | No |  | Optional with capability signalling [Note: This UE feature group is applicable only for NR cell for NTN cell and ATG cell, for terrestrial cell except for ARG cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

In addition, the following agreement on the K1 extension was made in RAN1 #104-e:

Agreement:

For unpaired spectrum, extend the value range of K1 from (0..15) to (0..31)

FFS: Whether there is an impact on the size of the PDSCH-to-HARQ_feedback timing indicator field in DCI.

The above featue has not been captured in the NTN UE capability. A Rel-17 UE should be able to support this feature as an optional UE capability.

Based on the discussion, an update to NTN UE feature groups can be found in Appendix.

# Conclusion

In this contribution, we discuss the Rel-17 UE features for NR NTN. The following proposals are presented:

Proposal: Agree on the updated FGs as listed in the Appendix.

- References

- R1-2202852, “Summary of UE features for NR NTN”.

- R1-1907706, “Summary of AI: 7.2.8.2 Enhancements on Multi-TRP/Panel Transmission of Offline Discussion”

# Appendix

| 26. NR_NTN_solutions | 26-1 | Uplink Time and Frequency pre-compensation and timing relationship enhancements | - UE specific TA calculation based on its GNSS-acquired position and the serving satellite ephemeris. - UE applies calculates common TA according to the parameters provided by the network [(UE considers common TA as 0 if the parameter is not provided)] - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops - FFS: UE pre-compensates the calculated TA in its uplink transmissions - Support of estimating UE-gNB RTT and delaying the start of RAR window [by UE-gNB RTT] - Support of frequency pre-compensation to counter shift the Doppler experienced on the service link - Determining timing of the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset if indicated - Determining timing of the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated and determining the timing of PDCCH monitoring in recovery search space using K-mac during beam failure recovery procedure - UE receives cell-specific K_offset/K_mac in system information |  | No | No | Release 17 NR UE cannot communicate via access [NTN/satellite/HAPS/ATG] | [Per UE/per band] | No | No | No | An NTN UE is required to at least support UE specific TA and frequency calculation based at least on its GNSS-acquired position and the serving satellite ephemeris | Optional with capability signalling For UE supports NR communication via [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR cell for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; NR NTN cell and ATG cell, for terrestrial any other cell except for ARG cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 26. NR_NTN_solutions | 26-3 | Enhancement on the timing relationship | - FFS: delaying the scheduling of PUSCH, PUCCH and PDCCH ordered PRACH, CSI reference resource, transmission of aperiodic SRS activation of TA command, first PUSCH transmission in CG Type 2 with cell-specific K_offset the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated | [26-1][26-2] | No | No | Release 17 UE cannot access [NTN/ satellite/HAPS/ATG] | [Per UE/per band] | No | No | No | FFS: whether this FG gets merged with FG 26-1 | Optional with capability signalling For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported. [Note: This UE feature group is applicable only for NR cell for NR NTN cell and ATG cell, for terrestrial cell except for ARG cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-4 | UE reporting of information about related to the UE specific TA pre-compensation | Support UE reporting of information about related to the UE specific TA pre-compensation [The exact content of UE reporting of information about the UE specific TA pre-compensation e.g., frequency of the reports, granularity of the reported conten, etc.] | [26-1] | Yes | No | UE does not support reporting of information related to TA pre-compensation for NR communication via satellite | [Per UE/per band] | No | No | No | Note: The exact content of UE reporting of information about the TA pre-compensation is up to RAN2 | Optional with capability signalling [Note: This UE feature group is applicable only for NR cell for NR NTN cell and ATG cell, for terrestrial cell except for ARG cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-5 | Increasing the number of HARQ processes | The maximal supported HARQ process number is X for UL and Y for DL |  | Yes | No | Increased number of HARQ processes is not supported for NR communication via satellite | [Per band or per FSPC or per UE] | No | No | No | Candidate component values for (X,Y): {(16,32),(32,16),(32,32)} | Optional with capability signalling [Note: This UE feature group is applicable only for NR cell for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; NR NTN cell and ATG cell, for terrestrial any other cell except for ARG cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-6 | [HARQ feedback disabling for] Type-2 HARQ codebook enhancement | - [Support of HARQ feedback disabling] Support of type-2 HARQ codebook enhancements when there are for feedback-disabled HARQ processes | FFS | Yes | No | Type-2 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No |  | Optional with capability signalling [Note: This UE feature group is applicable only for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; for any other cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-6a | [HARQ feedback disabling for] Type-1 HARQ codebook enhancement | - [Support of HARQ feedback disabling] - Support of Type-1 HARQ codebook enhancements when there are for feedback-disabled HARQ processes | FFS | Yes | No | Type-1 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No |  | Optional with capability signalling [Note: This UE feature group is applicable only for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; for any other cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-6b | [HARQ feedback disabling for] Type-3 HARQ codebook enhancement | - [Support of HARQ feedback disabling] Support of Type-3 HARQ codebook enhancements when there are for feedback-disabled HARQ processes | FFS | Yes | No | Type-3 HARQ codebook enhancement is not supported for NR communication via satellite | per band | No | No | No |  | Optional with capability signalling [Note: This UE feature group is applicable only for NR cell for communication via satellite/HAPS as specified in TS 38.101-5 or TS 38.104; for any other cell this feature is not supported] |
| 26. NR_NTN_solutions | [26-7] | [NTN Performance enhancement] | 1. The maximum number of supported aggregation factor (i.e., pdsch-AggregationFactor) for DL PDSCH is [X] FFS: X = 16 or 32 |  | Yes | No |  | [Per UE/per band/Per BC] | [No] | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] |  | Optional with capability signalling [Note: This UE feature group is applicable only for NR NTN cell, for terrestrial cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-8 | Support of polarization [signalling/ information/reception] in NR NTN | - Support polarization indication reception in SIB indicating DL and/or UL polarization information using respective polarization type parameters to indicate: RHCP or LHCP or linear - Support polarization signalling for target serving cell in handover command message - Support polarization signalling for non-serving cell in RRM measurement configuration - FFS: polarization information for DL is indicated in SIB by the network |  | No | No | [It is assumed by the network that UE supports at least linear polarization] UE is not able to take the advantage of polarization information to save power | [Per UE/Per band] | No | No | [support mixture of FDD/TDD (for HAPS and/or ATG) and/or FR1/FR2] No | [Value range for component 1: {(RHCP, LHCP, Linear), (RHCP, LHCP), (RHCP), (LHCP), (Linear)}] FFS: whether this FG gets merged with FG 26-1 if the note “For UE supports NR [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported” is confirmed in the positive | Optional with capability signalling [For UE supports NR communication via [NTN/ satellite/HAPS/ATG], UE must indicate this FG is supported] [Note: This UE feature group is applicable only for NR cell for NR NTN cell and ATG cell, for terrestrial cell except for ARG cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-9 | UE-specific K_offset | 1. Reception of UE-specific K_offset via MAC-CE 2. Determining the timing of PUSCH, PUCCH [and PDCCH ordered PRACH], CSI reference resource, transmission of aperiodic SRS, activation of TA command, first PUSCH transmission in CG Type 2 with UE-specific Koffset | 26-1, 26-3, 26-4 | Yes | No | UE-specific K_offset reception is not supported for NR communication via satellite | FFS Per band | No | No | No |  | Optional with capability signalling [Note: This UE feature group is applicable only for NR cell for NTN cell and ATG cell, for terrestrial cell except for ARG cell this feature is not supported] |
| 26. NR_NTN_solutions | 26-X | K1 range extension | 1. Support of extended K1 value range of (0..31) for unpaired spectrum. |  | Yes | No |  | Per band | No | No | No |  | Optional with capability signalling |