3GPP TSG-RAN WG1 Meeting #109-e                                                                                                                                                                                                           	                                                                                                                R1-2203091

e-Meeting, May 9 - 20, 2022

Agenda Item:	8.16.14

Source:	Huawei, HiSilicon

Title:	Rel-17 UE features for IoT over NTN

Document for:	Discussion and Decision

- Introduction

In RAN1#108-e, a list of Rel-17 UE features for IoT over NTN were agreed as baseline for further discussions [1]. In this contribution, we address some remaining aspects.

- Discussion

The UE capabilities, the deployment scenarios and the mobility assumptions for IoT over NTN UEs varies differently. Therefore it is no need for all the IoT UEs deploying in NTN band to support all the FGs. We think all the feature groups discussed in this paper should be supported on per UE basis.

FG2-1

- Comment 1: Regarding whether FG2-1 should be per UE or per band, it was agured in RAN1# 108-e that the motivation to define it as per band is to avoid support NTN functionality on both TN cell and NTN cell for a UE sharing the same spectrum between TN and NTN. Despite the fact that the scenario of spectrum sharing between TN and NTN is quite complicated (there are quite a lot of serious issues that needs to be solved, e.g. regulation aspects, system architecture, co-existence, etc.), a per UE capability definition is also sufficient from RAN1 point of view since TN band and NTN band will be defined by RAN4 separately. For UEs supporting both TN and NTN, the IoT-NTN specific features is only applicable to NTN but not TN and there is no need to differentiate among different NTN bands. Therefore, we suggest to define FG2-1 as a per UE capability.

| 2. LTE_NBIOT_eMTC_NTN | 2-1 | Basic IoT over NTN support for eMTC | 1. UE derives its position based on its GNSS measurements [1-1. UE reports the information associated with the validity duration of GNSS] 2. UE receives serving satellite ephemeris in either state vector format or orbital element format 4. UE calculates UE specific TA in RRC_IDLE and RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris 6. UE calculates common TA in RRC_IDLE and RRC_CONNECTED according to the parameters provided by the network (UE considers common TA as 0 if the parameter is not provided) 7. For TA update in RRC_CONNECTED state, UE uses a combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops 8. In RRC_IDLE and RRC_CONNECTED state, UE calculates frequency pre-compensation to counter shift the Doppler experienced on the service link [in DL] [and] [in UL] 10. UE supports a validity timer of UL synchronization that is configured by the network 13. UE applies cell specific K_offset in timing relationship enhancements 15. UE estimates UE-gNB RTT and delays the starts of ra-ResponseWindow by UE-gNB RTT 16. Delay the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated 17. In RRC_IDLE state and RRC_CONNECTED state, UE pre-compensates the calculated frequency offset and TA in uplink transmissions 18. UE receives cell-specific K_offset/K_mac in system information |  | Yes | N/A | Release 17 eMTC UE cannot communicate via satellite | [Per UE/per band] | No | No |  | Optional with capability signalling For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported Note: This UE feature group is applicable only for IoT-NTN cell for communication via satellite, for any other cell this feature is not supported |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG2-1a

- Comment 1: This is a per UE feature group. Same as comment 1 for FG2-1.

- Comment 2: Support the comment “[Release 17 eMTC UE cannot access communicate via [NTN/satellite] GEO and NGSO NTNs]”.

- Comment 3: Support the note “[For UEs support communicate via GEO and NGSO NTNs NSGO scenarios, it must indicate this FG is supported.]”.

| 2. LTE_NBIOT_eMTC_NTN | 2-1a | Segmented UL transmission duration configuration for eMTC | 1. Support UE applies segmented UL transmission according to duration configuration by the network | 2-1 | Yes | N/A | [Release 17 eMTC UE cannot access communicate via [NTN/satellite] GEO and NGSO NTNs] | [Per UE/per band] | No | No | [For UEs support communicate via GEO and NGSO NTNs NSGO scenarios, it must indicate this FG is supported.] | Optional with capability signalling [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG2-1b

- Comment 1: This is a per UE feature group. Same as comment 1 for FG2-1.

| 2. LTE_NBIOT_eMTC_NTN | 2-1b | Basic IoT over NTN support for NB-IoT | 1. UE derives its position based on its GNSS measurements [1-1. UE reports the information associated with the validity duration of GNSS] 2. UE receives serving satellite ephemeris in either state vector format or orbital element format 4. UE calculates UE specific TA in RRC_IDLE and RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris 6. UE calculates common TA in RRC_IDLE and RRC_CONNECTED according to the parameters provided by the network (UE considers common TA as 0 if the parameter is not provided) 7. For TA update in RRC_CONNECTED state, UE uses a combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops 8. In RRC_IDLE and RRC_CONNECTED state, UE calculates frequency pre-compensation to counter shift the Doppler experienced on the service link 10. UE supports a validity timer of UL synchronization that is configured by the network 13. UE applies cell specific K_offset in timing relationship enhancements 15. UE estimates UE-gNB RTT and delays the starts of ra-ResponseWindow by UE-gNB RTT 16. Delay the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated 17. In RRC_IDLE state and RRC_CONNECTED state, UE pre-compensates the calculated frequency offset and TA in uplink transmissions 18. UE receives cell-specific K_offset/K_mac in system information |  | Yes | N/A | Release 17 NB-IoT UE cannot communicate via satellite | [Per UE/per band] | No | No |  | Optional with capability signalling For UEs supporting NB-IoT NTN, it must indicate this FG is supported Note: This UE feature group is applicable only for IoT-NTN cell for communication via satellite, for any other cell this feature is not supported |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG2-1c

- Comment 1: This is a per UE feature group. Same as comment 1 for FG2-1.

| 2. LTE_NBIOT_eMTC_NTN | 2-1c | Segmented UL transmission for NB-IoT | 1. UE applies segmented UL transmission according to duration configuration by the network | 2-1b | Yes | N/A | Release 17 NB-IoT UE cannot communicate via NGSO NTNs | [Per UE/per band] | No | No | For UEs supporting communicate via NGSO NTNs, it must indicate this FG is supported. | Optional with capability signalling Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG2-2

- Comment 1: This is a per UE feature group. Same as comment 1 for FG2-1.

| 2. LTE_NBIOT_eMTC_NTN | 2-2 | Enhancing timing relationships using a time offset for eMTC | UE receives and applies UE specific K_offset, K_mac in timing relationship enhancements | 2-1 , 2-3 | Yes | N/A | eMTC UE does not know the offset to apply for UL transmission | [Per UE/per band] | No | No | The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG2-2a

- Comment 1: This is a per UE feature group. Same as comment 1 for FG2-1.

| 2. LTE_NBIOT_eMTC_NTN | 2-2a | Enhancing timing relationships using a time offset for NB-IoT | UE receives and applies UE specific K_offset, K_mac in timing relationship enhancements | 2-1b, 2-3a | Yes | N/A | NB-IoT UE does not know the offset to apply for UL transmission | [Per UE/per band] | No | No | The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG2-3

- Comment 1: This is a per UE feature group. Same as comment 1 for FG2-1.

| 2. LTE_NBIOT_eMTC_NTN | 2-3 | TA pre-compensation reporting for eMTC | Support reporting of information about the UE specific TA pre-compensation | 2-1 | Yes | No | UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD | [Per UE/per band] | No | No | UE-specific TA reporting is supported in IoT-NTN | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

FG2-3a

- Comment 1: This is a per UE feature group. Same as comment 1 for FG2-1.

| 2. LTE_NBIOT_eMTC_NTN | 2-3a | TA pre-compensation reporting for NB-IoT | Support reporting of information about the UE specific TA pre-compensation | 2-1b | Yes | No | UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD | [Per UE/per band] | No | No | UE-specific TA reporting is supported in IoT-NTN | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# Conclusion

In this contribution, we discuss the remaining issues of Rel-17 UE features for NB-IoT and eMTC over NTN. The discussion is presented in the following proposals, and all the changes are highlighted in green in the Appendix A.

Proposal 1: Agree on the updated UE features in the Appendix.

- References

[1] R1-2202856, “Summary of UE features for IoT over NTN”.

# Appendix

Proposed revision for Rel-17 UE features for IoT over NTN.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | [Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs)] | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. LTE_NBIOT_eMTC_NTN | 2-1 | Basic IoT over NTN support for eMTC | 1. UE derives its position based on its GNSS measurements [1-1. UE reports the information associated with the validity duration of GNSS] 2. UE receives serving satellite ephemeris in either state vector format or orbital element format 4. UE calculates UE specific TA in RRC_IDLE and RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris 6. UE calculates common TA in RRC_IDLE and RRC_CONNECTED according to the parameters provided by the network (UE considers common TA as 0 if the parameter is not provided) 7. For TA update in RRC_CONNECTED state, UE uses a combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops 8. In RRC_IDLE and RRC_CONNECTED state, UE calculates frequency pre-compensation to counter shift the Doppler experienced on the service link [in DL] [and] [in UL] 10. UE supports a validity timer of UL synchronization that is configured by the network 13. UE applies cell specific K_offset in timing relationship enhancements 15. UE estimates UE-gNB RTT and delays the starts of ra-ResponseWindow by UE-gNB RTT 16. Delay the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated 17. In RRC_IDLE state and RRC_CONNECTED state, UE pre-compensates the calculated frequency offset and TA in uplink transmissions 18. UE receives cell-specific K_offset/K_mac in system information |  | Yes | N/A | Release 17 eMTC UE cannot communicate via satellite | [Per UE/per band] | No | No |  | Optional with capability signalling For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported Note: This UE feature group is applicable only for IoT-NTN cell for communication via satellite, for any other cell this feature is not supported |
| 2. LTE_NBIOT_eMTC_NTN | 2-1a | Segmented UL transmission duration configuration for eMTC | 1. Support UE applies segmented UL transmission according to duration configuration by the network | 2-1 | Yes | N/A | [Release 17 eMTC UE cannot access communicate via [NTN/satellite] GEO and NGSO NTNs] | [Per UE/per band] | No | No | [For UEs support communicate via GEO and NGSO NTNs NSGO scenarios, it must indicate this FG is supported.] | Optional with capability signalling [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| 2. LTE_NBIOT_eMTC_NTN | 2-1b | Basic IoT over NTN support for NB-IoT | 1. UE derives its position based on its GNSS measurements [1-1. UE reports the information associated with the validity duration of GNSS] 2. UE receives serving satellite ephemeris in either state vector format or orbital element format 4. UE calculates UE specific TA in RRC_IDLE and RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris 6. UE calculates common TA in RRC_IDLE and RRC_CONNECTED according to the parameters provided by the network (UE considers common TA as 0 if the parameter is not provided) 7. For TA update in RRC_CONNECTED state, UE uses a combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops 8. In RRC_IDLE and RRC_CONNECTED state, UE calculates frequency pre-compensation to counter shift the Doppler experienced on the service link 10. UE supports a validity timer of UL synchronization that is configured by the network 13. UE applies cell specific K_offset in timing relationship enhancements 15. UE estimates UE-gNB RTT and delays the starts of ra-ResponseWindow by UE-gNB RTT 16. Delay the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated 17. In RRC_IDLE state and RRC_CONNECTED state, UE pre-compensates the calculated frequency offset and TA in uplink transmissions 18. UE receives cell-specific K_offset/K_mac in system information |  | Yes | N/A | Release 17 NB-IoT UE cannot communicate via satellite | [Per UE/per band] | No | No |  | Optional with capability signalling For UEs supporting NB-IoT NTN, it must indicate this FG is supported Note: This UE feature group is applicable only for IoT-NTN cell for communication via satellite, for any other cell this feature is not supported |
| 2. LTE_NBIOT_eMTC_NTN | 2-1c | Segmented UL transmission for NB-IoT | 1. UE applies segmented UL transmission according to duration configuration by the network | 2-1b | Yes | N/A | Release 17 NB-IoT UE cannot communicate via NGSO NTNs | [Per UE/per band] | No | No | For UEs supporting communicate via NGSO NTNs, it must indicate this FG is supported. | Optional with capability signalling Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported |
| 2. LTE_NBIOT_eMTC_NTN | 2-2 | Enhancing timing relationships using a time offset for eMTC | UE receives and applies UE specific K_offset, K_mac in timing relationship enhancements | 2-1 , 2-3 | Yes | N/A | eMTC UE does not know the offset to apply for UL transmission | [Per UE/per band] | No | No | The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. | Optional with capability signalling |
| 2. LTE_NBIOT_eMTC_NTN | 2-2a | Enhancing timing relationships using a time offset for NB-IoT | UE receives and applies UE specific K_offset, K_mac in timing relationship enhancements | 2-1b, 2-3a | Yes | N/A | NB-IoT UE does not know the offset to apply for UL transmission | [Per UE/per band] | No | No | The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. | Optional with capability signalling |
| 2. LTE_NBIOT_eMTC_NTN | 2-3 | TA pre-compensation reporting for eMTC | Support reporting of information about the UE specific TA pre-compensation | 2-1 | Yes | No | UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD | [Per UE/per band] | No | No | UE-specific TA reporting is supported in IoT-NTN | Optional with capability signalling |
| 2. LTE_NBIOT_eMTC_NTN | 2-3a | TA pre-compensation reporting for NB-IoT | Support reporting of information about the UE specific TA pre-compensation | 2-1b | Yes | No | UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD | [Per UE/per band] | No | No | UE-specific TA reporting is supported in IoT-NTN | Optional with capability signalling |