3GPP TSG-RAN WG1 Meeting #109-e	R1-2203095

e-Meeting, May 9th – May 20th, 2022

Agenda Item:	8.8.1

Source:	Huawei, HiSilicon

Title:	Discussion on PUSCH enhancements

Document for:	Discussion and Decision

- Introduction

In RAN1#108-e [1], several issues for PUSCH repetition type A enhancements, TB processing over multi-slot (TBoMS) PUSCH, and enhancements on type A PUSCH repetition for Msg3 were discussed, and following agreements were made:

Agreement The TP#B’’ (in R1-2202567) to TS38.214 is endorsed Agreement - For DG-PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, the legacy counting method applies when K=1, regardless of whether AvailableSlotCounting is enabled or not - Note: The legacy assumption on the K2 offset is applied, i.e., no RAN1 spec impact on the K2 offset is expected. - For CG-PUSCH repetition Type A, the legacy counting method applies when K=1, regardless of whether AvailableSlotCounting is enabled or not. - Note: No RAN1 spec impact is expected. - Note: No additional restriction on CG periodicity configurations, compared to Rel-16, is considered for CG-PUSCH with K=1. - For CG-PUSCH repetition Type A, when AvailableSlotCounting is enabled, and for K>1, a UE assumes that the slot (i.e., the first configured slot in a CG period) determined in 38.321 Section 5.8.2 can be the slot which is not counted in K available slots. - Note: No RAN1 spec impact is expected. Agreement For the bit selection of TBoMS: - G in Clause 5.4.2.1 [and/or Clause 6.2.6] of TS 38.212 is redefined for TBoMS as the total number of coded bits available for transmission of the transport block in a slot. - Note: no intention to change the legacy behavior For determining the index of the starting coded bit in the circular buffer for the n-th slot of a single TBoMS, RAN1 to down-select one of the following two options during RAN1 #108-e. Option 1 The index of the starting coded bit in the circular buffer for the n-th slot of a single TBoMS, i.e., sn, is calculated as sn=k0n=0(sn-1+H+τn-1)mod Ncbn=1, …,N-1 where: - ko is given by Table 5.4.2.1-2. - N is the number of slots allocated for TBoMS - Ncb is the length of circular buffer - H is the total number of coded bits available for transmission of the TB in a slot allocated for TBoMS, assuming no UCI multiplexing - τn-1 is the number of filler bits that would be skipped in the bit selection step, assuming no UCI multiplexing, in the n-1-th slot allocated for TBoMS, if any. Note: this equation describes the logic of the bit-selection for TBoMS; decision on where and how to capture this in TS 38.212 is up to the Editor.. Option 2 Adopt the following TP for TS 38.212 #### 5.4.2.1 Bit selection <Unchanged parts omitted> Denote by the redundancy version number for this transmission ( = 0, 1, 2 or 3), the rate matching output bit sequence , , is generated as follows, where is given by Table 5.4.2.1-2 according to the value of and LDPC base graph if numberOfSlotsTBoMS is not present in the resource allocation table or the value of numberOfSlotsTBoMS in the row indicated by the Time domain resource assignment field in DCI is 1, otherwise for the first allocated slot, that is the 0-th slot, is given by Table 5.4.2.1-2 according to the value of and LDPC base graph and [] for n-th slot, n=1,2,…numberOfSlotsTBoMS-1, is the bit next to the last selected bit by the bit selection in the previous slot assuming the UCI is not multiplexed:. Agreement The index of the starting coded bit in the circular buffer for the n-th slot of a single TBoMS, i.e., sn, is calculated as sn=k0n=0(sn-1+H+τn-1)mod Ncbn=1, …,N-1 where: - ko k0 is given by Table 5.4.2.1-2. - N is the number of slots allocated for TBoMS - Ncb is the length of circular buffer - H is the total number of coded bits available for transmission of the TB in a slot allocated for TBoMS, assuming no UCI multiplexing - τn-1 is the number of filler bits that would be skipped in the bit selection step, assuming no UCI multiplexing, in the n-1-th slot allocated for TBoMS, if any. Note: this equation describes the logic of the determining the starting coded bit in bit selection for TBoMS slot; decision on where and how to capture this or wording that is equivalent in TS 38.212 is up to the Editor. Conclusion There is no consensus in RAN1 to support TBoMS for CG-Type 1. Conclusion The decision on whether to modify the definition of G in Clause 6.2.6 of TS 38.212 such that this parameter is described as the total number of coded bits available for transmission of the transport block in a slot is left to the Editor’s discretion. Agreement (Excerpted from RAN1#107bis-e) Regarding how a UE should interpret MCS information field for indication of the number of repetitions for the case of CBRA, Option 1 is supported. - When a UE requests Msg3 repetition, the repurposed MCS information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition. - Repetition factor K=1 is included in the four candidate repetition factors used for repetition indication. - When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy MCS information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition. Agreement The above agreement (Excerpted from RAN1#107bis-e) is updated as follows. Regarding how a UE should interpret MCS information field for indication of the number of repetitions for the case of CBRA, Option 1 is supported. - When a UE requests Msg3 repetition, the repurposed MCS information field is applied. gNB schedules Msg3 with or without repetition for the UE requesting Msg3 repetition. - Whether Rrepetition factor K=1 is included in the four candidate repetition factors used for repetition indication is up to gNB configuration. - When the UE doesn’t request Msg3 repetition (including legacy UE), the legacy MCS information field is applied. gNB schedules Msg3 without repetition for the UE not requesting Msg3 repetition.. Agreement The Rel-17 collision rules among SSB and Msg3 PUSCH transmission for HD-FDD UEs, which is subject to the discussion on collision handling for Msg3 transmission in AI 8.6.1.2, can be reused for Msg3 PUSCH repetition for HD-FDD UEs. Agreement - RAN1 confirms to support separate RO for requesting of Msg3 PUSCH repetition in a BWP configured with ROs not for requesting of Msg3 PUSCH repetition. - Note: It expects RAN2 will continue the work on RACH partitioning based on separate RO for Rel-17 CE, and no further action in RAN1 on this aspect is pursued unless triggered by RAN2. Agreement [Draft] Reply LS on UL BWP with PRACH resources only for RACH with Msg3 repetition, R1-220xxxx (Reply LS_v02), is endorsed in principle Agreement Final LS on UL BWP with PRACH resources only for RACH with Msg3 repetition, R1-2202829, is endorsed. Agreement The following TP for Clause 6.1.2.1 of TS 38.214 h00 is endorsed. 6.1.2.1 Resource allocation in time domain <Unchanged parts are omitted> For PUSCH repetition Type A, when transmitting PUSCH scheduled by DCI format 0_1 or 0_2 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1, the number of repetitions K is determined as - if numberOfRepetitions is present in the resource allocation table, the number of repetitions K is equal to numberOfRepetitions; - elseif the UE is configured with pusch-AggregationFactor, the number of repetitions K is equal to pusch-AggregationFactor; - otherwise K=1. - the number of slots used for TBS determination N is equal to 1. For PUSCH repetition type A, when transmitting PUSCH scheduled by RAR UL grant, the 2 MSBs of the MCS information field of the RAR UL grant provide a codepoint to determine the number of repetitions K according to Table 6.1.2.1-1A, based on whether or not the higher layer parameter numberOfMsg3Repetitions is configured. The number of slots used for TBS determination N is equal to 1. For PUSCH repetition type A, when transmitting PUSCH scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI, the 2 MSBs of the MCS information field of the DCI format 0_0 with CRC scrambled by TC-RNTI provide a codepoint to determine the number of repetitions K according to Table 6.1.2.1-1A, based on whether or not the higher layer parameter numberOfMsg3Repetitions is configured. The number of slots used for TBS determination N is equal to 1. <Unchanged parts are omitted>.

Based on these agreements, this contribution continues to discuss remaining issues.

- Discussion on remaining issues of PUSCH enhancements

  - Repetition type A enhancement

Uplink Frequency Hopping

In RAN1#108-e [2], there is the discussion on frequency hopping. The following is the latest version of proposed conclusion on frequency hopping:

- It is RAN1’s common understanding that: - For PUSCH scheduled (with or without repetitions) by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, the UE is not required to read frequencyHopping in pusch-Config for the determination of frequency hopping type or for the determination of whether the frequency hopping is configured for the PUSCH and is required to read the frequency hopping flag information field of the RAR UL grant or the DCI format 0_0 with CRC scrambled by TC-RNTI for the determination of whether the frequency hopping is configured for the PUSCH or not. - For PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, the conclusion for R1-2003594 in RAN1#101-e is applied.

The following is the conclusion for R1-2003594 in RAN1#101-e:

Conclusion Draft CR in R1-2003594 is not pursued for Rel-15 with the following understanding - For a PUSCH scheduled by a DCI format 0_0 with CRC scrambled by C-RNTI, MCS-C-RNTI or CS-RNTI, the frequency hopping field in the DCI format 0_0 is not expected to be set to 1 if frequencyHopping in PUSCH-Config is not provided or set to interSlot. - For Type 2 CG PUSCH activated by DCI format 0_0 with CRC scrambled by CS-RNTI, the frequency hopping field in the DCI format 0_0 is not expected to be set to 1 if frequencyHopping in configuredGrantConfig is not provided or set to interSlot.

We support the latest version of proposed conclusion on frequency hopping.

DCI format 0_0 is a fallback DCI, which means its UE behavior is not supposed to depend on any UE dedicated RRC parameter. However, RRC parameter frequencyHopping is a UE dedicated parameter which is configured via BWP-UplinkDedicated->PUSCH-Config-> frequencyHopping.

Additionally, during initial access, a UE may be scheduled by DCI 0_0 but no UE dedicated RRC including frequencyHopping has been configured to the UE. In this case, it is quite clear that the UE has to do intra-slot hopping if the DCI 0_0 indicates so.

Thirdly, in TS 38.212, there is always the “Frequency hopping flag” field in DCI format 0_0.

Frequency hopping flag – 1 bit according to Table 7.3.1.1.1-3, as defined in Clause 6.3 of [6, TS 38.214] Table 7.3.1.1.1-3: Frequency hopping indication Bit field mapped to index; PUSCH frequency hopping. 0; Disabled. 1; Enabled.

So, we support the latest version of proposed conclusion on frequency hopping.

As to which release to apply, our view is since Rel-15. Because, the second bullet in the proposed conclusion was for Rel-15. As for the first bullet, RRC parameter pusch-Config is UE dedicated signaling, and may not be configured yet during initial access.

Proposal 1: Adopt the following conclusion for Rel-15:

- It is RAN1’s common understanding that:

  - For PUSCH scheduled (with or without repetitions) by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, the UE is not required to read frequencyHopping in pusch-Config for the determination of frequency hopping type or for the determination of whether the frequency hopping is configured for the PUSCH and is required to read the frequency hopping flag information field of the RAR UL grant or the DCI format 0_0 with CRC scrambled by TC-RNTI for the determination of whether the frequency hopping is configured for the PUSCH or not.

  - For PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, the conclusion for R1-2003594 in RAN1#101-e is applied.

In Rel-15 and Rel-16, a restriction on the scheduling of PUCCH with HARQ is introduced to guarantee the DAI mechanism works properly. As described in the Clause 9 of TS 38.213, the restriction is copied as below:

A UE does not expect to detect a DCI format scheduling a PDSCH reception or having associated HARQ-ACK information report without scheduling a PDSCH reception, and indicating a resource for a PUCCH transmission with corresponding HARQ-ACK information in a slot if the UE previously detects a DCI format scheduling a PUSCH transmission in the slot and if the UE multiplexes HARQ-ACK information in the PUSCH transmission.

For example, as shown in Figure 1, if a UL DCI format is received at slot n+1 scheduling a PUSCH transmission at slot n+k, the first DL DCI in slot n can schedule PDSCH_1 with corresponding HARQ feedback in slot n+k, but no HARQ feedback in slot n+k can be indicated for the second DL DCI format in slot n+2 scheduling PDSCH_2. As a result, the available PUCCH resource is limited by PUSCH slots that have been scheduled.

Figure 1. Timing restriction on UL/DL scheduling for HARQ information multiplexed on PUSCH.

However, in case of Rel-17 PUSCH repetition, due to the enlarged length of PUSCH transmissions in time, the restriction introduces too strict constrains on gNB scheduling. Especially for PUSCH repetition with counting on available slots, PUSCH repetitions may keep occupying more than one hundred contiguous UL slots and leave no available PUCCH for a long period for TDD system, which cost significant downlink throughput degradation. For example, as shown in Figure 2, for DDDDD DDSUU pattern, even with 6 repetitions and counting on available slots, 11 DL slots (before the green D slot) cannot be scheduled for PDSCH due to limited range of dL-DataToUL-ACK (up to 15). The larger repetitions number, the more degradation of DL throughput.

DDDDD DDSUU || DDDDD DDSUU || DDDDD DDSUU || DDDDD DDSUU

|						 	  |

PUSCH DCI				First available PDSCH

Figure 2. Impact of PDSCH due to PUSCH repetition with counting on available slots

TS 38.331: dl-DataToUL-ACK SEQUENCE (SIZE (1..8)) OF INTEGER (0..15) … dl-DataToUL-ACK-r16 SetupRelease { DL-DataToUL-ACK-r16 } … DL-DataToUL-ACK-r16 ::= SEQUENCE (SIZE (1..8)) OF INTEGER (-1..15)

Observation 1: For TDD configuration with DDDDD DDSUU, even with 6 repetitions and counting on available slots, 11 DL slots (before the green D slot) cannot be scheduled for PDSCH due to limited range of dL-DataToUL-ACK. The larger repetitions number, the more degradation of DL throughput.

Therefore, a solution is needed to mitigate the DL throughput degradation caused by the restriction.

To minimize specification impact, one solution is enlarge dL-DataToUL-ACK from current maximum value 15 to 31, which is subject to a UE capability. Since it is very simple UE implementation, the UE capability can be added to FG 10-1 as a prerequisite. Additionally, the maximum number of HARQ processes has to be enlarged to 32. Otherwise, the DL slots cannot be fully scheduled for PDSCH. Also, a UE capability can be added for it.

Proposal 2: In Rel-17, enlarge upper bound of RRC parameter dL-DataToUL-ACK to 31

- Add a UE capability for it. This UE capability is added to FG 10-1 as a prerequisite.

- Increase the max number of HARQ process to 32 at least for DL, add a UE capability for it.

With the above solution, for PUSCH repetition with counting on physical slots, the restriction on gNB scheduling can be alleviated and the specification impact is minimized. However, the issue for PUSCH repetition with counting on available slots is not completely solved. For example, the maximum value of dL-DataToUL-ACK has to be about 160 for 32 repetition in DDDDD DDSUU pattern, so does the maximum number of HARQ process which seems impractical. Thus additional solution on top of the solution above for PUSCH repetition counting on available slots is needed. Simply, we can revise the agreement that only up to 16 repetition is supported for PUSCH repetition counting on available slots.

Alternatively, the scheduling restriction in TS 38.213 can be revised. For example, a RRC parameter is introduced to turn off the scheduling restriction and allows that PDSCH_2 in Figure 1 can be scheduled after the UL DCI format transmitted in slot n+1.. Additionally, since the total DAI specified currently in a UL grant cannot reflect the number of scheduled PDSCH(s) after the UL grant, it should be indicated by the DAI received in the latest DL grant. It is the same DAI mechanism as that of the HARQ feedback piggybacked on CG PUSCH or a PUSCH scheduled by DCI format 0_0, which is following HARQ on PUCCH procedure and using the DAI in the last DL DCI for calculating HARQ bits on CG PUSCH. In summary, a TP for the solution can be found in Appendix.

Figure 3.  Update total DAI in UL DCI by the DAI in DL DCI

Proposal 3: In Rel-17, for PUSCH repetition counting based on available slot, either one alternative should be taken.

- Alt1: revise the agreement that only up to 16 repetition is supported for PUSCH repetition counting on available slots.

- Alt2: adopt the TP in Appendix.

  - TB processing over multiple slots

In RAN1#108-e [3], there are some remaining issues in the CR 38.214, as follows:

#### 6.1.2.1 Resource allocation in time domain <omitted text> For paired spectrum and SUL band: - In case K>1, the UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. - For the case of a reduced capability half-duplex UE, and when AvailableSlotCounting is enabled, the UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. <omitted text> For TB processing over multiple slots: - For the case of reduced capability half-duplex UE, and when AvailableSlotCounting is enabled, the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot.

For the first bullet, some companies suggest that “in case K>1” should be removed and added in the main bullet for both unpaired and paired spectrum. In our view, it should not be added in the main bullet, because the TBoMS PUSCH and Msg3 PUSCH have no restriction that they must be transmitted with repetition. In addition, “in case K>1” should only apply to PUSCH repetition type A but not TBoMS PUSCH in the first bullet for the same reason.

For other bullets, it is not clear whether or not take AvailableSlotCounting enabling as a prerequisite for TBoMS PUSCH. In our understanding, it should not be a prerequisite. The reason is that AvailableSlotCounting is introduced to indicate which slots counting method is used for PUSCH repetition type A, i.e., counted by available slots or counted by physical slots, but for TBoMS PUSCH, it counts on available slots by natural. So “when AvailableSlotCounting is enabled” in the bullets should not apply to the TBoMS PUSCH.

As discussed above, the following proposals are obtained.

Proposal 4: Adopt following TP.

#### 6.1.2.1 Resource allocation in time domain <omitted text> For paired spectrum and SUL band: - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2 when K>1, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. - For the case of a reduced capability half-duplex UE, the UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2 when AvailableSlotCounting is enabled and K>1, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. <omitted text> For TB processing over multiple slots: <omitted text> - For the case of reduced capability half-duplex UE, the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot.

- Conclusions

In this contribution, we provide our views on possible specification impact on PUSCH enhancements with following proposals:

Observation 1: For TDD configuration with DDDDD DDSUU, even with 6 repetitions and counting on available slots, 11 DL slots (before the green D slot) cannot be scheduled for PDSCH due to limited range of dL-DataToUL-ACK. The larger repetitions number, the more degradation of DL throughput.

Proposal 1: Adopt the following conclusion for Rel-15:

- It is RAN1’s common understanding that:

  - For PUSCH scheduled (with or without repetitions) by RAR UL grant or by DCI format 0_0 with CRC scrambled by TC-RNTI, the UE is not required to read frequencyHopping in pusch-Config for the determination of frequency hopping type or for the determination of whether the frequency hopping is configured for the PUSCH and is required to read the frequency hopping flag information field of the RAR UL grant or the DCI format 0_0 with CRC scrambled by TC-RNTI for the determination of whether the frequency hopping is configured for the PUSCH or not.

  - For PUSCH scheduled by DCI format 0_0 with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI, the conclusion for R1-2003594 in RAN1#101-e is applied.

Proposal 2: In Rel-17, enlarge upper bound of RRC parameter dL-DataToUL-ACK to 31

- Add a UE capability for it. This UE capability is added to FG 10-1 as a prerequisite.

- Increase the max number of HARQ process to 32 at least for DL, add a UE capability for it.

Proposal 3: In Rel-17, for PUSCH repetition counting based on available slot, either one alternative should be taken.

- Alt1: revise the agreement that only up to 16 repetition is supported for PUSCH repetition counting on available slots.

- Alt2: adopt the TP of TS 38.213 in Appendix.

Proposal 4: Adopt following TP

#### 6.1.2.1 Resource allocation in time domain <omitted text> For paired spectrum and SUL band: - The UE determines N∙K consecutive slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2 when K>1, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. - For the case of a reduced capability half-duplex UE, the UE determines N∙K slots for a PUSCH transmission of a PUSCH repetition type A scheduled by DCI format 0_1 or 0_2 when AvailableSlotCounting is enabled and K>1, or for a PUSCH transmission of TB processing over multiple slots scheduled by DCI format 0_1 or 0_2, based on the TDRA information field value in the DCI format 0_1 or 0_2. A slot is not counted in the number of N∙K slots if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. <omitted text> For TB processing over multiple slots: <omitted text> - For the case of reduced capability half-duplex UE, the same symbol allocation is applied across the N∙K slots determined for the PUSCH transmission and the PUSCH is limited to a single transmission layer. The UE shall transmit the TB across the N∙K slots determined for the PUSCH transmission, applying the same symbol allocation in each slot.

# References

- “RAN1 Chair’s Notes,” 3GPP TSG RAN WG1#108-e, Feb 21st – Mar 3th, 2022.

- R1-2202569, “FL Summary #4 on Enhancements on PUSCH repetition type A,” Feb 21st – Mar 3rd, 2022.

- R1-2202959, “Corrections on NR coverage enhancements,” Feb 21st – Mar 3rd, 2022.

Appendix

TPs of TS 38.213 for PUSCH repetitions with counting based on available slots.

9 UE procedure for reporting control information <Unchanged parts omitted> When [newRRC] is not configured, a UE does not expect to detect a DCI format scheduling a PDSCH reception or having associated HARQ-ACK information report without scheduling a PDSCH reception, and indicating a resource for a PUCCH transmission with corresponding HARQ-ACK information in a slot if the UE previously detects a DCI format scheduling a PUSCH transmission in the slot and if the UE multiplexes HARQ-ACK information in the PUSCH transmission. <Unchanged parts omitted>

9.1.2.2 Type-1 HARQ-ACK codebook in physical uplink shared channel If a UE is not provided pdsch-HARQ-ACK-Codebook = 'semi-static' for unicast or multicast HARQ-ACK information, the UE does not multiplex the unicast or multicast HARQ-ACK information in the PUSCH transmission, respectively. If a UE is provided pdsch-HARQ-ACK-Codebook = 'semi-static' for unicast and/or multicast HARQ-ACK information, and would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by a DCI format that does not include a DAI field or if a UE would multiplex HARQ-ACK information including that corresponding to the PDSCHs scheduled after a DCI format in the PUSCH scheduled by the DCI format when [newRRC] is configured, then - if the UE has not received any PDSCH or SPS PDSCH release or TCI state update that the UE multiplexes corresponding HARQ-ACK information in the PUSCH, based on a value of a respective PDSCH-to-HARQ_feedback timing indicator field in a DCI format scheduling the PDSCH reception or the SPS PDSCH release or the TCI state update, or on the value of dl-DataToUL-ACK or dl-DataToUL-ACK-r16 if the PDSCH-to-HARQ_feedback timing indicator field is not present in DCI format 1_1 or on the value of dl-DataToUL-ACK-DCI-1-2 if the PDSCH-to-HARQ_feedback timing indicator field is not present in DCI format 1_2 and the UE is provided pdsch-HARQ-ACK-Codebook = 'semi-static' for unicast HARQ-ACK information, or on the value of dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in DCI format 4_2 and the UE is provided pdsch-HARQ-ACK-Codebook = 'semi-static' for multicast HARQ-ACK information, in any of the Mc occasions for candidate PDSCH receptions by a DCI format or SPS PDSCH on any serving cell c, as described in clause 9.1.2.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission - else the UE generates the HARQ-ACK codebook as described in clause 9.1.2.1, except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH, unless the UE receives only a SPS PDSCH release, or only a SPS PDSCH reception, or only a TCI state update, or only a PDSCH that is scheduled by DCI format 1_0 with a counter DAI field value of 1 if the UE is provided pdsch-HARQ-ACK-Codebook = 'semi-static' for unicast HARQ-ACK information, or is scheduled by DCI format 4_1 with a counter DAI field value of 1 if the UE is provided pdsch-HARQ-ACK-Codebook = 'semi-static' for multicast HARQ-ACK information, on the PCell in the Mc occasions for candidate PDSCH receptions in which case the UE generates HARQ-ACK information only for the SPS PDSCH release or only for the PDSCH reception or only for the TCI state update as described in clause 9.1.2. A UE sets to NACK value in the HARQ-ACK codebook any HARQ-ACK information corresponding to PDSCH reception or SPS PDSCH release or TCI state update that the UE detects in a PDCCH monitoring occasion that starts after a PDCCH monitoring occasion where the UE detects a DCI format scheduling the PUSCH transmission. A UE does not expect to detect a DCI format switching a DL BWP within N2 symbols prior to a first symbol of a PUSCH transmission where the UE multiplexes HARQ-ACK information, where N2 is defined in [6, TS 38.214]. If a UE multiplexes HARQ-ACK information in a PUSCH transmission that is scheduled by DCI format that includes a DAI field when [newRRC] is not configured or does not detect any PDCCH scheduling PDSCH with HARQ-ACK to be multiplexed in the PUSCH scheduled by a previous PDCCH when [newRRC] is configured, and - is not provided fdmed-Reception-Multicast and is provided pdsch-HARQ-ACK-Codebook = 'semi-static' for both unicast and multicast HARQ-ACK information, or - is provided pdsch-HARQ-ACK-Codebook = 'semi-static' only for one of unicast and multicast HARQ-ACK information the UE generates the HARQ-ACK codebook as described in clause 9.1.2.1 when a value of the DAI field is VT-DAIUL=1 except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH. The UE does not generate a HARQ-ACK codebook for multiplexing in the PUSCH transmission when VT-DAIUL=0 unless the UE receives only a SPS PDSCH release, or only SPS PDSCH(s), or only a TCI state update, or only a PDSCH that is scheduled by a DCI format 1_0 if the UE is provided pdsch-HARQ-ACK-Codebook = 'semi-static' for unicast HARQ-ACK information, or scheduled by a DCI format 4_1 if the UE is provided pdsch-HARQ-ACK-Codebook = 'semi-static' for multicast HARQ-ACK information, with a counter DAI field value of 1 on the PCell in the Mc occasions for candidate PDSCH receptions in which case the UE generates HARQ-ACK information only for the SPS PDSCH release or only for the TCI state update or only for the PDSCH reception as described in clause 9.1.2. <Unchanged parts omitted>

9.1.3.2 Type-2 HARQ-ACK codebook in physical uplink shared channel If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by a DCI format that does not include a DAI field or if a UE would multiplex HARQ-ACK information including that corresponding to the PDSCHs scheduled after a DCI format in the PUSCH scheduled by the DCI format when [newRRC] is configured, then - if the UE has not received any PDCCH within the monitoring occasions for DCI formats scheduling PDSCH receptions, or providing a DCI format having associated HARQ-ACK information without scheduling a PDSCH reception, on any serving cell c and the UE does not have HARQ-ACK information in response to a SPS PDSCH reception to multiplex in the PUSCH, as described in clause 9.1.3.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission; - else, the UE generates the HARQ-ACK codebook as described in clause 9.1.3.1, except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH. If a UE multiplexes HARQ-ACK information in a PUSCH transmission that is scheduled by a DCI format that includes a DAI field when [newRRC] is not configured or does not detect any PDCCH scheduling PDSCH with HARQ-ACK to be multiplexed in the PUSCH scheduled by a previous PDCCH when [newRRC] is configured, the UE generates the HARQ-ACK codebook as described in clause 9.1.3.1, with the following modifications: - For the pseudo-code for the HARQ-ACK codebook generation in clause 9.1.3.1, after the completion of the c and m loops, the UE sets Vtemp2=VT-DAIUL where VT-DAIUL is the value of the DAI field according to Table 9.1.3-2 - For the case of first and second HARQ-ACK sub-codebooks, the DCI format includes a first DAI field corresponding to the first HARQ-ACK sub-codebook and a second DAI field corresponding to the second HARQ-ACK sub-codebook - harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH. <Unchanged parts omitted>