3GPP TSG-RAN WG1 Meeting #109-e	R1-2203097

e-Meeting, May 9 – 20, 2022

Agenda Item:     8.16.8

Source:	Huawei, HiSilicon

Title:	Rel-17 UE features for NR coverage enhancement

Document for:	Discussion and Decision

Introduction

In this contribution, our views on UE capabilities of NR coverage enhancement are provided.

Discussion on NR coverage enhancement UE capabilities

- Issues for FG 30-2

Any features to support repetition type A can be one of the prerequisite feature groups for FG 30-2. Thus it should be one of {5-14, 5-16, 5-17, 11-6, 30-1}.

Proposal 1: The prerequisite feature group for FG 30-2 is one of {5-14, 5-16, 5-17, 11-6, 30-1}.

- Issues for FG 30-3/3a

The features for TBoMS should not be associated with the feature of repetition type A. So it is sufficient that the prerequisite feature group for FG 30-3 is None and the prerequisite feature group for FG 30-3a is FG 30-3.

Proposal 2: For the features for TBoMS, it is sufficient that

- Prerequisite feature group for FG 30-3 is None.

- Prerequisite feature group for FG 30-3a is FG 30-3.

- Issues for FG 30-4/4a/4b/4c/4d/4e/4f/4g/4h

The LS reply on length of maximum duration is as follows [3].

RAN4 replied to LS R1-2106212 from RAN1 regarding maximum duration earlier with R4-2206537, which indicated that RAN4 concluded that for per band UE capability on length of maximum duration, the set of values of duration lengths for FDD are {4, 8, 16, 32} slots. Meanwhile, RAN4 will not define the requirements with 32 slots for FDD in Rel-17. RAN4 continued discussing the granularity of the UE capability and the length of maximum duration for TDD, and further concluded that: For the granularity of the UE capability on length of maximum duration: - Per band UE capability was agreed in RAN4 #101e-bis. RAN4 has not discussed requirements for combination of this feature with UL CA in Rel-17, and RAN4 suggests the final decision on the granularity to be made in RAN1. For the length of maximum duration for TDD: - Regarding the RAN1 request for maximum duration, RAN4 understands that the UE capability for TDD refers to the number of consecutively transmitted UL slots over which the UE can meet the phase consistency requirement, assuming no phase/power consistency violating events in between. - For the above UE capability (as defined), RAN4 has further concluded that the set of values of duration lengths should at least include {2, 4, 8} slots, and whether 16 slots can be included in the set of values for UE capability reporting is up to RAN1 to decide, but RAN4 does not intend to define requirements for 16 slots for TDD in Rel-17.

Considering the maximum number of repetition, 16 slots should be included in the set of maximum duration for TDD. And according to the above, a Note for FG 30-4 should include:

- Candidate numbers of slots for maximum duration for FDD are {4, 8, 16, 32}.

- Candidate numbers of slots for maximum duration for TDD are {2, 4, 8, 16}.

Proposal 3: A Note for FG 30-4 should be added as:

- Candidate numbers of slots for maximum duration for FDD are {4, 8, 16, 32}.

- Candidate numbers of slots for maximum duration for TDD are {2, 4, 8, 16}.

We summary the prerequisite feature groups of FGs for DMRS bundling as following table.

Table 1 Prerequisites

| Index | Feature group | Prerequisite feature groups |
| --- | --- | --- |
| 30-4 | The maximum duration for DM-RS bundling | FFS |
| 30-4a | DM-RS bundling for PUSCH repetition type A | [30-4], [30-1] or [30-2] one of {5-14, 5-16, 5-17, 11-6, 30-1} |
| 30-4b | DM-RS bundling for PUSCH repetition type B | [30-4], [11-5] [30-1] |
| 30-4c | DM-RS bundling for TB processing over multi-slot PUSCH | [30-4], [30-3] or 30-3a |
| 30-4d | DMRS bunding for PUCCH repetitions | [30-4], [4-23] or 25-2 |
| 30-4e | Enhanced Inter-slot frequency hopping with inter-slot bundling for PUSCH | [30-4a] or [30-4b] or [30-4c] |
| 30-4f | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [30-4d] |
| 30-4g | Restart DM-RS bundling after the events that violate power consistency and phase continuity | [30-4] |
| 30-4h | DM-RS bundling for non-back-to-back transmission | [30-4a, or 30-4b, or 30-4c or 30-4d] |

Proposal 4: Adopt the changes to prerequisites for FG 30-4a/4b/4c/4d/4e/4f/4g/4h as shown in the last column of Table 1.

First of all, for a given tolerance, the CFOs of different operating frequency are different, which may cause the “compensation leftover for frequency error” be different. And UE’s implementation and RF characteristics on different band to maintain the phase continuity could be different. So it is reasonable to define that the UE features of DMRS bundling is set to per band. Actually, FG 30-4 that has been set per band, should be a direct or indirect prerequisite to all other FGs of DMRS bundling. Therefore, per UE for FG 30-4a/4b/4c/4d/4e/4f/4g/4h is sufficient.

Proposal 5: The type of the granularities for FG 30-4a/4b/4c/4d/4e/4f/4g/4h are per UE.

For FG 30-4a/4b/4c/4d/4e/4f/4g/4h, FR1/FR2 differentiation and TDD/FDD differentiation are not necessary. And the need for FR1/FR2 differentiation and TDD/FDD differentiation should be N/A.

Proposal 6: For FG 30-4a/4b/4c/4d/4e/4f/4g/4h,

- No for the need for FR1/FR2 differentiation and TDD/FDD differentiation.

- N/A for the capability interpretation for mixture of FDD/TDD and/or FR1/FR2.

- Issues for FG 30-5

The prerequisite feature groups for FG 30-5 are FG 4-23 or 25-2.

Proposal 7: The prerequisite feature group for FG 30-5 is FG 4-23 or 25-2.

- Issues for FG 30-6

For the type of the granularity for FG 30-6, per UE is preferable. Besides, the need for FR1/FR2 differentiation and TDD/FDD differentiation are No, and the capability interpretation for mixture of FDD/TDD and/or FR1/FR2 is N/A.

Proposal 8: For FG 30-6,

- Per UE for the type of the granularity.

- No for the need for FR1/FR2 differentiation and TDD/FDD differentiation.

- N/A for the capability interpretation for mixture of FDD/TDD and/or FR1/FR2.

Proposal 9: In summary, adopt the changes of the table in Appendix.

Conclusions

In this contribution, we discussed the various aspects for the NR coverage enhancement UE capabilities for Rel-17. The proposals are listed in the following:

Proposal 1: The prerequisite feature group for FG 30-2 is one of {5-14, 5-16, 5-17, 11-6, 30-1}.

Proposal 2: For the features for TBoMS, it is sufficient that

- Prerequisite feature group for FG 30-3 is None.

- Prerequisite feature group for FG 30-3a is FG 30-3.

Proposal 3: A Note for FG 30-4 should be added as:

- Candidate numbers of slots for maximum duration for FDD are {4, 8, 16, 32}.

- Candidate numbers of slots for maximum duration for TDD are {2, 4, 8, 16}.

Proposal 4: Adopt the changes to prerequisites for FG 30-4a/4b/4c/4d/4e/4f/4g/4h as shown in the last column of Table 1.

Proposal 5: The type of the granularities for FG 30-4a/4b/4c/4d/4e/4f/4g/4h are per UE.

Proposal 6: For FG 30-4a/4b/4c/4d/4e/4f/4g/4h,

- No for the need for FR1/FR2 differentiation and TDD/FDD differentiation.

- N/A for the capability interpretation for mixture of FDD/TDD and/or FR1/FR2.

Proposal 7: The prerequisite feature group for FG 30-5 is FG 4-23 or 25-2.

Proposal 8: For FG 30-6,

- Per UE for the type of the granularity.

- No for the need for FR1/FR2 differentiation and TDD/FDD differentiation.

- N/A for the capability interpretation for mixture of FDD/TDD and/or FR1/FR2.

Proposal 9: In summary, adopt the changes of the table in Appendix.

- References

- R1-2201517, “Summary on UE features for NR coverage enhancement”, NTT DOCOMO, INC. Mar. 2022.

- R1-2202928, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e”, NTT DOCOMO, INC. Mar. 2022.

- R4-2206580, “Reply LS on Length of Maximum duration”, RAN4, Mar. 2022.

- Appendix

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-1 | Increased maximum number of PUSCH Type A repetitions | Maximum value of K (the number of repetitions) = 32 For DG PUSCH, the number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a DCI. For Type 1 CG PUSCH, the number of repetitions is indicated by repK-r17 For Type 2 CG PUSCH, the number of repetitions is indicated in a TDRA list or by repK-r17. | 5-14, 5-16, or 5-17 | Yes | N/A | UE does not support more than 16 repetitions PUSCH. | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-2 | PUSCH Type A repetitions based on available slots | Transmission occasions for [K] repetitions for dynamic and configured grant PUSCH are determined on the basis of available slots. | One of {5-14, 5-16, 5-17, [11-6, 30-1]} | Yes | N/A | UE does not support dynamic or configured grant PUSCH repetitions counted on the basis of available slots. | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH for DG and CG in RRC connected mode. | [11-6] | Yes | N/A | UE does not support TB processing over multi-slot PUSCH. | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-3a | Repetition of TB processing over multi-slot PUSCH | Support repetition of TB processing over multi-slot PUSCH in RRC connected mode. | TBD30-3 | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4 | The maximum duration for DM-RS bundling | The maximum duration during which UE is able to maintain power consisitency and phase continuity to support DM-RS bundling for PUSCH/PUCCH | FFS | Yes | N/A | UE does not support DM-RS bundling for PUSCH/PUCCH | Per band | N/A | N/A | N/A | Candidate numbers of slots for maximum duration for FDD are {4, 8, 16, 32}. Candidate numbers of slots for maximum duration for TDD are {2, 4, 8, 16}. | Optional with capability signalling |
| 30. NR_cov_enh | 30-4a | DM-RS bundling for PUSCH repetition type A | Support DM-RS bundling for PUSCH repetition type A | [30-4], [30-1] or [30-2] one of {5-14, 5-16, 5-17, 11-6, 30-1} | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type A | [Per UE] | FFSNo | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4b | DM-RS bundling for PUSCH repetition type B | Support DM-RS bundling for PUSCH repetition type B | [30-4], [11-5] [30-1] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type B | [Per UE] | FFSNo | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4c | DM-RS bundling for TB processing over multi-slot PUSCH | Support DM-RS bundling for TB processing over multi-slot PUSCH | [30-4], [30-3] or 30-3a | Yes | N/A | UE does not Support DM-RS bundling for TB processing over multi-slot PUSCH | [Per UE] | FFSNo | [No] | [N/A] | Note: If a UE reports support of FG 30-3a and 30-4c, the UE supports DMRS bundling for the repetitions of TBoMS | Optional with capability signalling |
| 30. NR_cov_enh | 30-4d | DMRS bunding for PUCCH repetitions | Support DM-RS bundling for PUCCH repetitions for PUCCH formats 1/3/4 | [30-4], [4-23] or 25-2 | Yes | N/A | UE does not support DMRS bunding for PUCCH repetitions | [Per UE] | FFSNo | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4e | Enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | Support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | [30-4a] or [30-4b] or [30-4c] | Yes | N/A | UE does not support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | [Per UE] | FFSNo | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4f | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [30-4d] | Yes | N/A | UE does not support Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [Per UE] | FFSNo | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4g | [Restart DM-RS bundling after the dynamic events that violate power consistency and phase continuity] | [Support restarting DM-RS bundling after the dynamic events that violate power consistency and phase continuity] | [30-4] | Yes | N/A | [UE does not support restarting DM-RS bundling after the events that violate power consistency and phase continuity] | [Per UE] | FFSNo | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4h | DM-RS bundling for non-back-to-back transmission | Support DM-RS bundling for [non-back-to-back transmission for consecutive slots] for PUSCH and PUCCH only for [corresponding supported back-to-back transmission FGs (30-4a, 30-4b, 30-4c, or 30-4d)] | [30-4a, or 30-4b, or 30-4c or 30-4d] | Yes | N/A | UE does not Support DM-RS bundling for non-back-to-back transmission | [Per UE] | FFSNo | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-5 | Slot based dynamic PUCCH repetition indication | Support slot based dynamic PUCCH repetition indication for PUCCH formats 0/1/2/3/4 support slot based dynamic PUCCH repetition for PUCCH formats 0/1/2/3/4 | [4-23 and/or 25-2] | Yes | N/A | UE does not support Dynamic PUCCH repetition indication | Per UE | No | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-6 | Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI | [Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI] |  | Yes | N/A | UE does not support repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI. | [Per UE] | [No] | [No] | [N/A ] |  | Optional with capability signalling |