3GPP TSG-RAN WG1 Meeting #109-e 	R1-2203099

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.5.1

Source:	Huawei, HiSilicon

Title:	Maintenance of Rel-17 positioning accuracy improvements

Document for:	Discussion and Decision

Introduction

In this paper, we discuss the maintenance of TEG features and DL-AoD enhancements.

TEG framework

As discussed in our companion paper [2], we think that what RAN4 agreed to Rx and RxTx TEG should be directly applied to Tx TEG reporting, in which case the TEG ID applies to all time instances (time stamps) within a single report.

- Proposal 1: Define the following framework for Tx TEG reporting

- The framework of UE/TRP Tx TEG: - Define multiple candidate timing error margin values {TE1, TE2, …, TEN} in the spec. - The number of candidate values (i.e. N) and the exact values of {TE1, TE2, …, TEN} will be decided in Perf part. - UE/TRP selects one value M from {TE1, TE2, …, TEN} based on its implementation and indicate to LMF per report. - For UE that supports multiple Tx TEGs (TEG#1, TEG#2, …), the associated timing error margin value of each Tx TEG is M, which means the timing error difference between the transmission within the same Tx TEG is within the margin M. - The applicability of reported UE Tx TEG is limited to all the SRS in a UEPositioningAssistanceInfo message in RRC or in a Multi-RTT-ProvideLocationInformation IE in LPP that are tagged with the corresponding TEG ID. - For TRP that supports multiple Tx TEGs, it is up to RAN3 to define the corresponding signaling if needed.

As discussed in the example in our companion paper [2], we think some reporting behaviour following the current ASN.1 should be considered by RAN1.

SRS#0 TEG#0

SRS#1 TEG#1

SRS#0 TEG#0

SRS#1 TEG#1

SRS#0 TEG#1

SRS#1 TEG#1

SRS#0 TEG#1

SRS#1 TEG#1

SRS#0 TEG#1

SRS#1 TEG#0

TEG#0 SRS#0 T=0

TEG#1 SRS#1 T=0

TEG#0 No resource T=2

TEG#1 SRS#0 SRS#1 T=2

TEG#0 SRS1 T=4

TEG#1 SRS#0  T=4

Report

No report

No report

Figure 1 SRS association change and report

In the example shown in Figure 1, some reporting behaviours should be clarified.

For the cases when the association does not change compared to the previous SRS transmission occasion, should UE skip the reporting for the occasion, e.g. T=1 and T=3?

Should the time stamp correspond to the slot when SRS is transmitted, or can be a slot when the association is valid, but may not contain any SRS transmission? For example, if two SRS resources are in two slots, should the TEG association at T=2 corresponds to two time stamps, or a single time stamp?

Should the change of the association for a SRS resource be reflected in both the original TEG and the target TEG? For example, if UE changes the TEG for a SRS, should the SRS be removed from the original TEG, when added to the target TEG?

How should the signaling design reflect the case when UE is not able to determine the association for a particular SRS?

- Our understanding on the above issues are summarized in the following proposal.

- Proposal 2: Define the following reporting principle and include it in the LS to RAN2.

UE is not required to report the same association of a TEG ID for an occasion (via the timestamp) compared with the previous occasion.

The timestamp corresponds to the slot when the association is valid, which may not be a slot containing the actual SRS transmission.

Both the original TEG and target TEG need to be included for an occasion via the timestamp in the report if UE switches SRS from the original TEG to the target TEG, e.g. delist the SRS from the original TEG and include the SRS to the target TEG.

For SRS not associated with any TEG in case UE is not able to determine the association, UE may not report the SRS resource ID in any of the TEG within the report

For SRS that has been previously associated with a TEG, but is no longer associated with any TEG in case UE is not able to determine the association after a given occasion, the UE may update the TEG by delisting the concerned SRS, without including the SRS to any target TEG.

For the current TS 38.331, we think the following changes are needed based on the above principle.

First, the description of the time stamp in TS 38.331 reads “the latest time instance”, which should be revised to “the earliest time instance” indicating the time from which the TEG association starts to be valid.

nr-TimeSTamp This field specifies the latest time instance at which the association is valid prior to the reporting.

Second, the srs-PosResSetAssociationList under the IE UE-TxTEG-Association should be optional, because it may be possible that during the reporting period, a Tx TEG has been initially associated with some SRS resources, but then at a given time instance, a Tx TEG no longer has any associated SRS resources at all.

- Proposal 3: Include the following change suggestion in the LS to RAN2.

The field description of nr-TimeStamp in the TEG association reporting should be the earliest time instance instead of the latest time instance.

The field srs-PosResSetAssociationList in the TEG association reporting should be optional to allow updating a TEG that is no longer associated with any SRS.

Receiver diversity for RSRPP reporting

There was discussion on receiver diversity in RAN1#108-e on the Rx branches for RSRPP. [3]

According to RAN4 agreement, they concluded that the same Rx branch as that for RSRP is used.

- Same Rx branches as applied for PRS-RSRP measurement are used for path PRS-RSRP measurement

We would like to note that when RAN4 made this agreement, they were intending to address the RSRPP diversity for DL-AoD, instead of DL-TDOA and Multi-RTT. There was also FL statement in [3] that RSRPP diversity for DL-TDOA and Multi-RTT belonged to the 8.5.1 agenda item of RAN1#108-e.

We can start the Rx Diversity discussion by focusing on the case of receiver diversity within a measurement for UL RSRPP. We note that there is a connection to the discussion on reporting PRS RSRPP with or without a relation to PRS RSRP. Regarding the proposal for multi-RTT and DL-TDOA, the discussion can be taken as part of the 8.5.1 agenda. If the 8.5.1 agenda is too busy, we can consider it for discussion in 8.5.3. Proposal 1.4: For path DL PRS RSRPP measurement reporting, when receiver diversity is in use, the reported path DL PRS RSRPP shall be corresponding to the same Rx branch associated with the reported DL PRS RSRP.

For the first path and additional path RSRPP reporting in DL-TDOA and Multi-RTT, especially when it is associated with the corresponding TOA reporting, the functionality should be interpreted as supporting PDP reporting. This is fundamentally different from the traditional signal strength-based power reporting, which is used to determine the coverage.

Having a flexible Rx branch selection for RSRPP in the case of DL-TDOA and Multi-RTT could even provide compatibility with the existing TEG reporting framework. For example, if TEG is associated with a UE panel, UE may use different Rx panels to receive the same PRS and report the corresponding TOA, in which case the first path RSRPP on both panels would be useful to determine whether the TOA reporting associated with Rx TEG ID is reliable or not.

- Observation 1: Reporting RSRPP value for the purpose of PDP reporting should not follow the same logic of coverage-motivated RSRP.

- Observation 2: When RSRPP reporting is associated with TEG reporting, it should be allowed to have Rx branch or Rx branch set specific power reporting.

Considering the late stage of Rel-17, we believe a quick fix should be to allow RSRPP reporting for DL-TDOA and Multi-RTT to be associated with any Rx branch set that is selected by the UE. UE may further indicate the Rx branch set ID associated with the RSRPP reporting to help network identify whether multiple RSRPP measurements correspond to the same Rx branch set, similar to the Rx beam index introduced for DL-AoD in Rel-16.

The Rx branch set ID can be locally defined within the measurement report for a TRP, unlike TEG ID, meaning that the same Rx branch set ID associated with the measurements from two TRPs should not be assumed to be same Rx branch set ID.

Proposal 4: Support the following Rx diversity scheme for RSRPP reporting for DL-TDOA and Multi-RTT

For both frequency range 1 and frequency range 2, if receiver diversity is in use by the UE, the reported RSRPP may be associated with an Rx branch set ID.

The RSRPP measurements associated with the same Rx branch set ID for a TRP correspond to the same set of Rx branches.

DL-AoD reference RSRPP

For DL-AoD, it was agreed that additional RSRPP is reported using relative mapping in reference to the first RSRPP. [1]

Agreement For the reporting of first path DL-PRS RSRPP in DL AOD, - For the 1st measurement, the report includes DL PRS RSRP and optionally DL PRS RSRPP using absolute reporting - For additional measurement, at least one of the two following measurement is reported: - First path DL PRS RSRPP can be optionally reported using differential reporting with the first measurement of DL PRS RSRPP, - DL PRS RSRP can be optionally reported using differential reporting with the first measurement of DL PRS RSRP. - Send the agreement in an LS reply to RAN2 as a reply regarding the question “For RAN1 agreements “The requested PRS measurement can be DL PRS RSRP and/or path PRS RSRP. ”, is there a need to request and provide only the RSRPP measurements for the additional measurements (without legacy RSRP)?”

Assuming for DL-AoD, only single-sided reporting is needed, which means that the first RSRPP should be the highest value. However, it may be possible that the resource with the highest RSRP may not exactly be the same resource with the highest RSRPP for the first path.

- Observation 3: The PRS resource with the highest RSRP may not be the PRS resource with the highest RSRPP for the first path.

To resolve it, the DL-AoD measurement reporting may additionally provide the DL-PRS resource set ID and resource ID for the first RSRPP measurement, if it is not the same as the resource for the first RSRP measurement.

- Proposal 5: For DL-AoD, introduce a separate DL PRS resource set ID and resource ID for the first RSRPP measurement when the resource is not same as that for the first RSRP measurement.

The additional RSRPP measurement still takes (0..30) as the reporting range in reference to the first RSRPP measurement.

RSRPP definition

RAN1#108-e made the following change to the RSRPP definition. [1]

Agreement The following definition for DL PRS RSRPP in 38.215 is endorsed: - DL PRS reference signal received path power (DL PRS-RSRPP), is defined as the power of the linear average of the channel response at the i-th path delay of the resource elements that carry DL PRS signal configured for the measurement, where DL PRS-RSRPP for the 1st path delay is the power contribution corresponding to the first detected path in time.

The calculation of RSRPP is using linear average of channel response at the i-th path delay, which is not quite clear.

Let’s assume the channel impulse response at the equivalent baseband is as

ht=p=0P-1hpδt-τp

The corresponding channel frequency response is as

Hf=p=0P-1hpexp-j2πfτp

On the RE index k, the channel response is

Hk=Hkfscs=p=0P-1hpexp-j2πfscsτpk=p=0P-1hpexp-j2πDpkNFFT

where Dp=NFFTfscsτp=τpTFFT is the delay τp in terms of time domain samples, and NFFT is the FFT size, while the TFFT=1NFFTfscs is the baseband sampling rate for the IFFT operation.

The current definition of linear average suggests the following equation for deriving the RSRPP, however we believe that the definition is still not quite clear because the right-handed side is not dependent on any path index p.

RSRPPp∝kHk2

Instead, another weighting factor related to path p should be added, so that the equation can be corrected as

RSRPPp∝kHkexpj2πDpkNIFFT2

This equation has its physical meanings, which corresponds to the oversampling of the IFFT output of Hk at n=Dp. The weight factor expj2πDpkNIFFT is to shift the delay Dp to 0, so that the channel response for the purpose of RSRPP “average” for path p should be as below:

hpRSRPPt=ht+τp

Based on the discussion, we have the following proposal.

- Proposal 6: Adopt the following modification to the RSRPP definition.

DL PRS reference signal received path power (DL PRS-RSRPP), is defined as the power of the linear average of the channel response at the i-th path delay on the resource elements that carry DL PRS signal configured for the measurement, where DL PRS-RSRPP for the 1st path delay is the power contribution corresponding to the first detected path in time.

The channel response at the i-th path delay on a resource element corresponds to the response on the resource element of a shifted version of the channel where the i-th path delay is shifted to 0.

DL-AoD first path alignment

For DL-AoD, there was discussion [3] whether the TOA should be provided along with the first path RSRPP, but no conclusion was made.

Reporting TOA for the first RSRPP measurement is problematic, because that effectively translates DL-AoD to timing based methods.

Reporting relative TOA for the additional RSRPP measurement may still require UE to do TOA estimate for DL-AoD, which is less desired.

However, if no TOA information is provided, network cannot even know if the additional RSRPP measurement corresponds to the same path as the first RSRPP measurement.

- Observation 4: It is not possible to support TOA reporting for the first RSRPP measurement or the additional RSRPP measurement, but network should be informed whether the first RSRPP measurement and additional RSRPP measurement correspond to the same (first) path.

One way to reduce the discussion while fixing the concerns of both sides can be that when UE reports the RSRPP measurement, UE may also indicate whether the RSRPP measurements belong to the same path, which is similar to the Rx beam index reporting.

The total number of path index can be set to e.g. 4, while for a typical case, all RSRPP measurements should be corresponding to the same path index.

- Proposal 7: For each first path RSRPP measurement of DL-AoD positioning, UE indicates a path index.

The path index may take a value from {0,1,2,3}.

First path RSRPP measurements associated with the same path index for a TRP correspond to the same path.

Determination of whether the first path RSRPP measurements belong to the same path or different paths is up to UE implementation.

Rx beam index

For the Rx beam index reporting, the current 38.214 description and 37.355 is no longer suitable considering the new features introduced in Rel-17.

When the UE reports DL PRS-RSRP measurements from one DL PRS resource set, the UE may indicate which DL PRS-RSRP measurements associated with the same higher layer parameter nr-DL-PRS-RxBeamIndex [17, TS 37.355] have been performed using the same spatial domain filter for reception if for each nr-DL-PRS-RxBeamIndex reported there are at least 2 DL PRS-RSRP measurements associated with it within the DL PRS resource set.

nr-DL-PRS-RxBeamIndex This field provides an index of the target device receive beam used for DL-PRS measurements. If the value of the receive beam index for two or more DL PRS measurements is the same, it indicates that the target device receive beam for the two or more DL PRS measurements were made with the same RX beam. The field is mandatory present if at least two DL-PRS RSRP measurements from the same DL-PRS Resource Set have been made with the same RX beam by the target device; otherwise it is not present..

First, it is possible that only a single RSRP is reported, while the remaining power reporting corresponds to RSRPP.

Second, it should be allowed that only one PRS resource is reported per PRS resource set, but on a positioning frequency layers, two PRS resource sets are used, resulting in two RSRP/RSRPP measurements per positioning frequency layer, e.g. via subset association between two PRS resource sets.

For the above reasons, we have the following TP to fix this.

- Proposal 8: Endorse the following TP to TS 38.214.

Reasons of change: The Rx beam index should consider the RSRPP reporting and the RSRP/RSRPP reporting across multiple resources, while the current restriction on condition when the Rx beam index is allowed to be indicated is too restrictive.

Summary of change: Relax the existing condition on Rx beam index reporting of two RSRP measurements contained within a single set, and add RSRPP as part of the condition to report the Rx beam index.

Consequences if not approved: Rx beam index reporting is not supported for RSRP reporting associated with two PRS resource sets and is not supported for RSRPP-based DL-AoD.

================== Start of TP ==================

The UE may be configured to measure and report, subject to UE capability, up to 24 DL PRS-RSRP measurements on different DL PRS resources associated with the same dl-PRS-ID. When the UE reports DL PRS-RSRP measurements from one DL PRS resource set, the UE may indicate which DL PRS-RSRP measurements associated with the same higher layer parameter nr-DL-PRS-RxBeamIndex [17, TS 37.355] have been performed using the same spatial domain filter for reception if for each nr-DL-PRS-RxBeamIndex reported there are at least 2 DL PRS-RSRP measurements associated with it within the DL PRS resource set. The UE may be configured to measure and optionally report via higher layer signaling nr-DL-PRS-FirstPathRSRP-Result, subject to UE capability, up to 24 DL PRS RSRPP for the first detected path on different DL PRS resources associated with the same dl-PRS-ID.

The UE may be configured to optionally report a differential DL RSRPP for a PRS resource with reference to [nr-DL-PRS-FirstPathRSRP-Result] and/or a differential DL PRS RSRP with reference to [nr-DL-PRS-RSRP-Result] via higher layer parameter [NR-DL-AoD-AdditionalMeasurementElement].

When the UE reports DL PRS-RSRP/DL PRS-RSRPP measurements, the UE may indicate which DL PRS-RSRP/DL PRS-RSRPP measurements associated with the same higher layer parameter nr-DL-PRS-RxBeamIndex [17, TS 37.355] have been performed using the same spatial domain filter for reception if for each nr-DL-PRS-RxBeamIndex reported there are at least 2 DL PRS-RSRP measurements or at least 2 DL PRS-RSRPP measurements associated with it within the DL PRS resource sets on a positioning frequency layer with the same dl-PRS-ID.

================== End of TP ==================

Proposal 9: Send an LS to RAN2 informing that the following change to the field description of nr-DL-PRS-RxBeamIndex is preferred from RAN1 perspective.

nr-DL-PRS-RxBeamIndex This field provides an index of the target device receive beam used for DL-PRS measurements. If the value of the receive beam index for two or more DL PRS measurements is the same, it indicates that the target device receive beam for the two or more DL PRS measurements were made with the same RX beam. The field is mandatory present if at least two DL-PRS RSRP measurements or at least two DL-PRS RSRPP measurements from the DL-PRS Resource Sets on a positioning frequency layer of the TRP have been made with the same RX beam by the target device; otherwise it is not present.

Number of RSRPP per TRP

The number of RSRPP measurement requested per TRP is missing from the higher layer parameter list. Note that in Rel-16, the number of RSRP measurements per TRP, and in Rel-17, the number of additional RSRP measurements per TRP are already included in the LPP specification.

#### 6.5.11.5 NR DL-AoD Location Information Request #### – NR-DL-AoD-RequestLocationInformation The IE NR-DL-AoD-RequestLocationInformation is used by the location server to request NR DL-AoD location measurements from a target device. -- ASN1START NR-DL-AoD-RequestLocationInformation-r16 ::= SEQUENCE { nr-AssistanceAvailability-r16 BOOLEAN, nr-DL-AoD-ReportConfig-r16 NR-DL-AoD-ReportConfig-r16, ... } NR-DL-AoD-ReportConfig-r16 ::= SEQUENCE { maxDL-PRS-RSRP-MeasurementsPerTRP-r16 INTEGER (1..8) OPTIONAL, -- Need ON ..., [[ maxDL-PRS-RSRP-MeasurementsPerTRP-r17 INTEGER (9..24) OPTIONAL, -- Need ON firstPathRSRP-MeasurementReq-r17 ENUMERATED { requested } OPTIONAL, -- Need ON nr-los-nlos-IndicatorRequest-r17 SEQUENCE { type-r17 ENUMERATED {hardvalue,softvalue}, granularity-r17 ENUMERATED {trpspecific,resourcespecific}, ... } OPTIONAL, -- Need ON requestedDL-PRS-ProcessingSamples-r17 ENUMERATED { m1, ... } OPTIONAL -- Need ON ]] } -- ASN1STOP

Since the reporting of RSRPP does not rely on reporting of RSRP, we believe that a separate RSRPP number should be included in the request.

Proposal 10: Add a parameter for the number of DL PRS-RSRPP measurements per TRP requested by the LMF to the UE for DL-AoD.

Conclusion

In this contribution, we have discussed remaining issues of TEG and DL-AoD, and have the following proposals.

- Observation 1: Reporting RSRPP value for the purpose of PDP reporting should not follow the same logic of coverage-motivated RSRP.

- Observation 2: When RSRPP reporting is associated with TEG reporting, it should be allowed to have Rx branch or Rx branch set specific power reporting.

- Observation 3: The PRS resource with the highest RSRP may not be the PRS resource with the highest RSRPP for the first path.

- Observation 4: It is not possible to support TOA reporting for the first RSRPP measurement or the additional RSRPP measurement, but network should be informed whether the first RSRPP measurement and additional RSRPP measurement correspond to the same (first) path.

- Proposal 1: Define the following framework for Tx TEG reporting

- The framework of UE/TRP Tx TEG: - Define multiple candidate timing error margin values {TE1, TE2, …, TEN} in the spec. - The number of candidate values (i.e. N) and the exact values of {TE1, TE2, …, TEN} will be decided in Perf part. - UE/TRP selects one value M from {TE1, TE2, …, TEN} based on its implementation and indicate to LMF per report. - For UE that supports multiple Tx TEGs (TEG#1, TEG#2, …), the associated timing error margin value of each Tx TEG is M, which means the timing error difference between the transmission within the same Tx TEG is within the margin M. - The applicability of reported UE Tx TEG is limited to all the SRS in a UEPositioningAssistanceInfo message in RRC or in a Multi-RTT-ProvideLocationInformation IE in LPP that are tagged with the corresponding TEG ID. - For TRP that supports multiple Tx TEGs, it is up to RAN3 to define the corresponding signaling if needed.

- Proposal 2: Define the following reporting principle and include it in the LS to RAN2.

UE is not required to report the same association of a TEG ID for an occasion (via the timestamp) compared with the previous occasion.

The timestamp corresponds to the slot when the association is valid, which may not be a slot containing the actual SRS transmission.

Both the original TEG and target TEG need to be included for an occasion via the timestamp in the report if UE switches SRS from the original TEG to the target TEG, e.g. delist the SRS from the original TEG and include the SRS to the target TEG.

For SRS not associated with any TEG in case UE is not able to determine the association, UE may not report the SRS resource ID in any of the TEG within the report

For SRS that has been previously associated with a TEG, but is no longer associated with any TEG in case UE is not able to determine the association after a given occasion, the UE may update the TEG by delisting the concerned SRS, without including the SRS to any target TEG.

- Proposal 3: Include the following change suggestion in the LS to RAN2.

The field description of nr-TimeStamp in the TEG association reporting should be the earliest time instance instead of the latest time instance.

The field srs-PosResSetAssociationList in the TEG association reporting should be optional to allow updating a TEG that is no longer associated with any SRS.

Proposal 4: Support the following Rx diversity scheme for RSRPP reporting for DL-TDOA and Multi-RTT

For both frequency range 1 and frequency range 2, if receiver diversity is in use by the UE, the reported RSRPP may be associated with an Rx branch set ID.

The RSRPP measurements associated with the same Rx branch set ID for a TRP correspond to the same set of Rx branches.

- Proposal 5: For DL-AoD, introduce a separate DL PRS resource set ID and resource ID for the first RSRPP measurement when the resource is not same as that for the first RSRP measurement.

The additional RSRPP measurement still takes (0..30) as the reporting range in reference to the first RSRPP measurement.

- Proposal 6: Adopt the following modification to the RSRPP definition.

DL PRS reference signal received path power (DL PRS-RSRPP), is defined as the power of the linear average of the channel response at the i-th path delay on the resource elements that carry DL PRS signal configured for the measurement, where DL PRS-RSRPP for the 1st path delay is the power contribution corresponding to the first detected path in time.

The channel response at the i-th path delay on a resource element corresponds to the response on the resource element of a shifted version of the channel where the i-th path delay is shifted to 0.

- Proposal 7: For each first path RSRPP measurement of DL-AoD positioning, UE indicates a path index.

The path index may take a value from {0,1,2,3}.

First path RSRPP measurements associated with the same path index for a TRP correspond to the same path.

Determination of whether the first path RSRPP measurements belong to the same path or different paths is up to UE implementation.

- Proposal 8: Endorse the following TP to TS 38.214.

Reasons of change: The Rx beam index should consider the RSRPP reporting and the RSRP/RSRPP reporting across multiple resources, while the current restriction on condition when the Rx beam index is allowed to be indicated is too restrictive.

Summary of change: Relax the existing condition on Rx beam index reporting of two RSRP measurements contained within a single set, and add RSRPP as part of the condition to report the Rx beam index.

Consequences if not approved: Rx beam index reporting is not supported for RSRP reporting associated with two PRS resource sets and is not supported for RSRPP-based DL-AoD.

================== Start of TP ==================

The UE may be configured to measure and report, subject to UE capability, up to 24 DL PRS-RSRP measurements on different DL PRS resources associated with the same dl-PRS-ID. The UE may be configured to measure and optionally report via higher layer signaling nr-DL-PRS-FirstPathRSRP-Result, subject to UE capability, up to 24 DL PRS RSRPP for the first detected path on different DL PRS resources associated with the same dl-PRS-ID.

The UE may be configured to optionally report a differential DL RSRPP for a PRS resource with reference to [nr-DL-PRS-FirstPathRSRP-Result] and/or a differential DL PRS RSRP with reference to [nr-DL-PRS-RSRP-Result] via higher layer parameter [NR-DL-AoD-AdditionalMeasurementElement].

When the UE reports DL PRS-RSRP/DL PRS-RSRPP measurements, the UE may indicate which DL PRS-RSRP/DL PRS-RSRPP measurements associated with the same higher layer parameter nr-DL-PRS-RxBeamIndex [17, TS 37.355] have been performed using the same spatial domain filter for reception if for each nr-DL-PRS-RxBeamIndex reported there are at least 2 DL PRS-RSRP measurements or at least 2 DL PRS-RSRPP measurements associated with it within the DL PRS resource sets on a positioning frequency layer with the same dl-PRS-ID.

================== End of TP ==================

Proposal 9: Send an LS to RAN2 informing that the following change to the field description of nr-DL-PRS-RxBeamIndex is preferred from RAN1 perspective.

nr-DL-PRS-RxBeamIndex This field provides an index of the target device receive beam used for DL-PRS measurements. If the value of the receive beam index for two or more DL PRS measurements is the same, it indicates that the target device receive beam for the two or more DL PRS measurements were made with the same RX beam. The field is mandatory present if at least two DL-PRS RSRP measurements or at least two DL-PRS RSRPP measurements from the DL-PRS Resource Sets on a positioning frequency layer of the TRP have been made with the same RX beam by the target device; otherwise it is not present.

Proposal 10: Add a parameter for the number of DL PRS-RSRPP measurements per TRP requested by the LMF to the UE for DL-AoD.

- Reference

- RAN1, Chair’s Notes RAN1#108-e v27, RAN1#108-e, e-Meeting, 21st Feb.  – 3rd Mar., 2022.

- R1-2204903, Discussion on UE/TRP TEG framework, Huawei, HiSilicon, RAN1#109-e, 9th – 20th May, 2022.

- R1-2202605, FL summary #3 for AI 8.5.3 Accuracy improvements for DL-AoD positioning solutions	Moderator (Ericsson), RAN1#108-e, e-Meeting, 21st Feb.  – 3rd Mar., 2022.