3GPP TSG-RAN WG1 Meeting #109-e	R1-2203102

e-Meeting, May 9 – 20, 2022

Agenda Item:	8.13

Source:	Huawei, HiSilicon

Title:	Discussion on NR Dynamic spectrum sharing

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss some remaining issues about NR DSS.

Discussion

SCell dormancy indication

In current specification, SCell dormancy indication is restricted to be only transmitted in PCell, including:

- DCI format 2_6 in Type3-PDCCH CSS set can only be configured to be monitored on the PCell when a UE is configured with DRX mode operation.

- With scheduling information, the SCell dormancy indication field is only present in a DCI format 0_1/1_1 when the DCI format is carried by PDCCH on the PCell within DRX Active Time and the UE is configured with at least two DL BWPs for an SCell.

- Without scheduling information, DCI format 1_1 indicating SCell dormancy can only be carried by PDCCH on the PCell within DRX Active Time and the UE is configured with at least two DL BWPs for an SCell.

The UE capability for supporting SCell dormancy indication received on PCell using DCI format 2_6 sent outside the active time and the UE capability for supporting SCell dormancy indication received on PCell with DCI format 0_1/1_1 sent within the active time are independently reported by the UE. That is, the UE may choose to support either of these two UE capabilities, or both. As long as the UE supports any one of the two UE capabilities, the gNB can set the SCells in the same cell group (MSG or SCG) as dormancy for saving power consumption.

When the PCell is scheduled by sSCell, the USS set(s) for scheduling the PCell can only be configured on the sSCell, or only the USS set(s) on the sSCell is available due to overbooking processing on the PCell. Under these cases, if the UE only supports SCell dormancy indication received on PCell with DCI format 0_1/1_1 sent within the active time and the SCell dormancy indication is still restricted to be transmitted in PCell, the gNB cannot set the SCells of the same cell group as dormancy to save power consumption. Therefore, it is recommended that the SCell dormancy indication can also be sent on the sSCell.

The dormancy indication on the PCell should be able to set the sSCell to dormant, but the dormancy indication on the sSCell is not suitable for setting itself to dormant. This problem can be solved by separate configurations for the dormancy indication on the PCell and on the sSCell.

Proposal 1: When PDSCH or PUSCH on a PCell can be scheduled by PDCCH of sSCell and the PCell, SCell dormancy indication can also be sent on the sSCell, and the dormancy indication on the PCell and on the sSCell are configured separately.

Proposal 2: Adopt the following TP in TS 38.212

## 7.3.1.1.2 Format 0_1 […] - SCell dormancy indication – 0 bit if higher layer parameter dormancyGroupWithinActiveTime is not configured; otherwise 1, 2, 3, 4 or 5 bits bitmap determined according to the number of different DormancyGroupID(s) provided by higher layer parameter dormancyGroupWithinActiveTime, where each bit corresponds to one of the SCell group(s) configured by higher layers parameter dormancyGroupWithinActiveTime, with MSB to LSB of the bitmap corresponding to the first to last configured SCell group in ascending order of DormancyGroupID. The field is only present when this format is carried by PDCCH on the primary cell or on the secondary cell that schedules a primary cell within DRX Active Time and the UE is configured with at least two DL BWPs for an SCell. […] ## 7.3.1.2.2 Format 1_1 […] - SCell dormancy indication – 0 bit if higher layer parameter dormancyGroupWithinActiveTime is not configured; otherwise 1, 2, 3, 4 or 5 bits bitmap determined according to the number of different DormancyGroupID(s) provided by higher layer parameter dormancyGroupWithinActiveTime, where each bit corresponds to one of the SCell group(s) configured by higher layers parameter dormancyGroupWithinActiveTime, with MSB to LSB of the bitmap corresponding to the first to last configured SCell group in ascending order of DormancyGroupID. The field is only present when this format is carried by PDCCH on the primary cell or on the secondary cell that schedules a primary cell within DRX Active Time and the UE is configured with at least two DL BWPs for an SCell. […]

Proposal 3: Adopt the following TP in TS 38.213

## 7.3.1.1.2 Format 0_110.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells […] If a UE is provided search space sets to monitor PDCCH for detection of DCI format 1_1, and if - the CRC of DCI format 1_1 is scrambled by a C-RNTI or a MCS-C-RNTI, and if - a one-shot HARQ-ACK request field is not present or has a '0' value, and if - the UE detects a DCI format 1_1 on the primary cell that does not include a carrier indicator field, or detects a DCI format 1_1 on the primary cell that includes a carrier indicator field with value equal to 0, or detects a DCI format 1_1 on the secondary cell that includes a carrier indicator field with value corresponding to a PCell and if - resourceAllocation = resourceAllocationType0 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0, or - resourceAllocation = resourceAllocationType1 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 1, or - resourceAllocation = dynamicSwitch and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0 or 1 […]

Minimum scheduling offset

When the UE is configured with minimumSchedulingOffsetK0/minimumSchedulingOffsetK2 in an active DL/UL BWP it applies a minimum scheduling offset restriction indicated by the 'Minimum applicable scheduling offset indicator' field in DCI format 0_1 or DCI format 1_1 between the scheduling cell and PDSCH/PUSCH of the active DL/UL BWP in the scheduled cell.

When the PCell can be scheduled by the PCell and the sSCell, both the PCell and the sSCell are scheduling cells. The DCI format 0_1/1_1 sent on both scheduling cells may include the 'Minimum applicable scheduling offset indicator' field. It needs to be clarified whether the 'Minimum applicable scheduling offset indicator' field in the DCI on one scheduling cell is only applied to the current scheduling cell carrying this DCI, or can be applied to both of the two scheduling cells. For simplicity, it is proposed that the 'Minimum applicable scheduling offset indicator' field in the DCI on one scheduling cell is only applied to this current scheduling cell carrying the corresponding PDCCH.

Proposal 4: When PDSCH or PUSCH on a PCell can be scheduled by PDCCH of sSCell and the PCell, the 'Minimum applicable scheduling offset indicator' field in the DCI on one scheduling cell is applied only to the scheduling cell carrying the DCI.

Conclusion

According to the above discussions, we have the following observation and proposals:

Proposal 1: When PDSCH or PUSCH on a PCell can be scheduled by PDCCH of sSCell and the PCell, SCell dormancy indication can also be sent on the sSCell, and the dormancy indication on the PCell and on the sSCell are configured separately.

Proposal 2: Adopt the following TP in TS 38.212

## 7.3.1.1.2 Format 0_1 […] - SCell dormancy indication – 0 bit if higher layer parameter dormancyGroupWithinActiveTime is not configured; otherwise 1, 2, 3, 4 or 5 bits bitmap determined according to the number of different DormancyGroupID(s) provided by higher layer parameter dormancyGroupWithinActiveTime, where each bit corresponds to one of the SCell group(s) configured by higher layers parameter dormancyGroupWithinActiveTime, with MSB to LSB of the bitmap corresponding to the first to last configured SCell group in ascending order of DormancyGroupID. The field is only present when this format is carried by PDCCH on the primary cell or on the secondary cell that schedules a primary cell within DRX Active Time and the UE is configured with at least two DL BWPs for an SCell. […] ## 7.3.1.2.2 Format 1_1 […] - SCell dormancy indication – 0 bit if higher layer parameter dormancyGroupWithinActiveTime is not configured; otherwise 1, 2, 3, 4 or 5 bits bitmap determined according to the number of different DormancyGroupID(s) provided by higher layer parameter dormancyGroupWithinActiveTime, where each bit corresponds to one of the SCell group(s) configured by higher layers parameter dormancyGroupWithinActiveTime, with MSB to LSB of the bitmap corresponding to the first to last configured SCell group in ascending order of DormancyGroupID. The field is only present when this format is carried by PDCCH on the primary cell or on the secondary cell that schedules a primary cell within DRX Active Time and the UE is configured with at least two DL BWPs for an SCell. […]

Proposal 3: Adopt the following TP in TS 38.213

## 7.3.1.1.2 Format 0_110.3 PDCCH monitoring indication and dormancy/non-dormancy behaviour for SCells […] If a UE is provided search space sets to monitor PDCCH for detection of DCI format 1_1, and if - the CRC of DCI format 1_1 is scrambled by a C-RNTI or a MCS-C-RNTI, and if - a one-shot HARQ-ACK request field is not present or has a '0' value, and if - the UE detects a DCI format 1_1 on the primary cell that does not include a carrier indicator field, or detects a DCI format 1_1 on the primary cell that includes a carrier indicator field with value equal to 0, or detects a DCI format 1_1 on the secondary cell that includes a carrier indicator field with value corresponding to a PCell and if - resourceAllocation = resourceAllocationType0 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0, or - resourceAllocation = resourceAllocationType1 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 1, or - resourceAllocation = dynamicSwitch and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0 or 1 […]

Proposal 4: When PDSCH or PUSCH on a PCell can be scheduled by PDCCH of sSCell and the PCell, the 'Minimum applicable scheduling offset indicator' field in the DCI on one scheduling cell is applied only to the scheduling cell carrying the DCI.

References

- RP-193260, “New WID on NR Dynamic spectrum sharing (DSS)”.