3GPP TSG-RAN WG1 Meeting #109-e	R1-2203103

e-Meeting, May 9 – 20, 2022

Agenda Item:	8.16.13

Source:	Huawei, HiSilicon

Title:	Rel-17 UE features for DSS and MR-DC

Document for:	Discussion and Decision

Introduction

A preliminary RAN1 NR UE feature list in [1] was submitted to RAN1 #106-e for information only. A formal baseline [2] is agreed during RAN1 #108-e. Some views for Rel-17 UE features for DSS and MR-DC are summarized in this contribution.

Discussion

The corresponding UE feature tables for DSS and MR-DC in [2] is cited in the appendix. We have the following views on the current UE features.

UE features for DSS

Regarding the component bullet 4 for FG 34-1 and the component bullet 4 for FG 34-2:

- The number of unicast DCI limits for PCell/PSCell scheduling

  - Processing K1 unicast DCI scheduling DL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s)

  - Processing K2 unicast DCI scheduling UL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s)

  - FFS: N is based on pair of (PCell/PSCell SCS, sSCell SCS): N=1 for(15,15), (30,30), (60,60) and N=2 for (15,30), (30,60) and N=4 for (15, 60)

  - Component 4 candidate values for FG 34-1: (K1, K2) = {(1,1) for FDD P(S)Cell; (K1, K2) = (1,2) for TDD P(S)Cell, [(K1, K2) = (2,2) for FDD P(S)Cell; (K1, K2) = (2,4) for TDD P(S)Cell]}

  - Component 4 candidate values for FG 34-2: (K1, K2) = {(1,1) for FDD P(S)Cell; (K1, K2) = (1,2) for TDD P(S)Cell}

The SCS 15 kHz, 30 kHz and 60 kHz are all typical SCSs in FR1, so the pairs that satisfy sSCell SCS is larger than or equal to P(S)Cell SCS should be supported. “FFS” can be removed.

The reason for introducing candidate value [(K1, K2) = (2,2) for FDD P(S)Cell; (K1, K2) = (2,4) for TDD P(S)Cell] for FG 34-1 needs to be further justified since FG 34-1 is assumed to be the basic one while the proposed candidate value is beyond some legacy UE capabilities. For example, a legacy UE does not support processing up to 4 unicast DCI scheduling UL on scheduling cell with SCS 15 kHz for a scheduled cell with SCS 15 kHz.

Proposal 1:  Remove “FFS” from bullet 4 for FG 34-1 and bullet 4 for FG 34-2.

Proposal 2:  Remove the candidate value [(K1, K2) = (2,2) for FDD P(S)Cell; (K1, K2) = (2,4) for TDD P(S)Cell] from bullet 4 for FG 34-1.

Regarding the component bullet 8 for FG 34-1:

- FFS: Support of monitoring DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s).

The design of R17 DSS appears to be the simplest UE implementation to allow SCell scheduling PCell for DSS carrier. Therefore, the need of also allowing USS also on PCell may not be very significant and can be considered in future releases if necessary.

Proposal 3: Remove bullet 8 for FG 34-1.

Regarding the component bullet 9 for FG 34-1 and the component bullet 7 for FG 34-2:

- PDCCH monitoring occasion(s).

  - Component 7 candidate values:

    - [Value 1: PDCCH monitoring occasion(s) on PCell/PSCell and on sSCell for cross-carrier scheduling to PCell/PSCell is within the first 3 OFDM symbols of a PCell/PSCell slot.

    - Value 2: PDCCH monitoring occasion(s) on PCell/PSCell and on sSCell for cross-carrier scheduling to PCell/PSCell is not restricted to the first 3 OFDM symbols of a PCell/PSCell slot]

There were some attempt to clarify the candidate Value 2. Given that R17 DSS enh. is designed with single cell PDCCH budget, the PDCCH MO restriction for a UE supporting FG 34-1 can generally follow the UE capability of MO reported as legacy. This can be the interpretation of Value 2 instead of no restriction at all.

Proposal 4: Adopt the Value 1 in Component 9 for FG 34-1 and Component 7 for FG 34-2.

Proposal 5: Clarify and adopt the Value 2 as PDCCH MO capability follows the UE capability about MO reported via FG 3-x (as legacy).

Regarding the component bullet 10 for FG 34-1 and the component bullet 8 for FG 34-2:

- FFS: frame boundary alignment between PCell/PSCell and sSCell.

The basic operation with 34-1 and 34-2 can assume that the frame boundary is aligned between P(S)Cell and sSCell, therefore “FFS” can be removed. If necessary, a separate UE capability can be introduced to allow unaligned frame boundary for both operations.

Proposal 6: Remove “FFS” from bullet 10 for FG 34-1 and bullet 8 for FG 34-2.

Regarding the Candidate value set for FG 34-1 and FG 34-2 in the “Note” column:

- Candidate value set: One or more of supported SCS combinations ({P(S)Cell SCS in kHz, sSCell SCS in kHz}) from following set are indicated by the UE: {15,15}, {15,30}, (15, 60), [{30,30}, {30,60},{60,60})] [Candidate value set 2: frequency band pair(s) for {PCell/PSCell, sSCell}]

[{30,30}, {30,60},{60,60})] can be adopted for expanding the application scenarios and aligned with proposal 1. While since this capability is to be reported per BC and the most relevant factor is the SCS configuration of a CC as in usual CA capability, we do not see the motivation to adopt Candidate value set 2. The possible signaling overhead is also too much compared to the potential benefit it could bring.

Proposal 7: Adopt [{30,30}, {30,60},{60,60})]  and remove candidate value set 2 for FG 34-1 and FG 34-2 in the “Note” column.

UE features for MR-DC

Regarding the note of FG 35-1, candidate values of aperiodic CSI-RS resource set configurations are further discussed. As presented in RRC CR from RAN2 [3], one resource set configuration contains the information of qcl-Info-r17 and gapBetweenBursts-r17. If only one aperiodic CSI-RS resource set for tracking can be reported by a UE, the available TRS resource configurations are greatly limited, which affects the flexibility of configuration. Therefore, it is inappropriate to take the value of 1 for aperiodic CSI-RS resource set configuration for fast activation. In addition, compared with the CSI-RS resource set configurations for fast activation in FR1, more CSI-RS resource sets are required for indication of beam information in FR2 to provide more configuration flexibility. Therefore, some small candidate values such as 1 and 8 may not be applicable in the FR2 scenarios. The following proposals are provided,

Proposal 8: For FG 35-1, remove the candidate value “1” for components 3 and 4 in the column “Note”.

Proposal 9: For FG 35-1, candidate values “1” and “8” for components 3 and 4 may not be applicable to FR2 and should be removed from the column “Note”.

Proposal 10: Adopt the changes in Appendix for FG 35-1.

Conclusion

Based on above analysis, we have the following proposal:

Proposal 1:  Remove “FFS” from bullet 4 for FG 34-1 and bullet 4 for FG 34-2.

Proposal 2:  Remove the candidate value [(K1, K2) = (2,2) for FDD P(S)Cell; (K1, K2) = (2,4) for TDD P(S)Cell] from bullet 4 for FG 34-1.

Proposal 3: Remove bullet 8 for FG 34-1.

Proposal 4: Adopt the Value 1 in Component 9 for FG 34-1 and Component 7 for FG 34-2.

Proposal 5: Clarify and adopt the Value 2 as PDCCH MO capability follows the UE capability about MO reported via FG 3-x (as legacy).

Proposal 6: Remove “FFS” from bullet 10 for FG 34-1 and bullet 8 for FG 34-2.

Proposal 7: Adopt [{30,30}, {30,60},{60,60})]  and remove candidate value set 2 for FG 34-1 and FG 34-2 in the “Note” column.

Proposal 8: For FG 35-1, remove the candidate value “1” for components 3 and 4 in the column “Note”.

Proposal 9: For FG 35-1, candidate values “1” and “8” for components 3 and 4 may not be applicable to FR2 and should be removed from the column “Note”.

Proposal 10: Adopt the changes in Appendix for FG 35-1.

- References

R1-2108679, “Preliminary RAN1 UE features list for Rel-17 NR”, Moderators (AT&T, NTT DOCOMO, INC.)

R1-2202929, “Updated RAN1 UE features list for Rel-17 NR after RAN1 108-e including remaining RAN1 issues”, Moderators (AT&T, NTT DOCOMO, INC.)

R2-2201714, “38331 CR Introduction of TRS based SCell activation”.

- Appendix

- NR_DSS

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 34. NR_DSS | 34-2 | Cross-carrier scheduling from SCell to PCell/PSCell (Type B) | Support of Cross-carrier scheduling (CCS) from sSCell to PCell/PSCell (Type B) - Cross-carrier scheduling from sSCell to PCell/PSCell with CIF - sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and search space sets on PCell/PSCell can be configured so that the UE monitors them in overlapping slot of PCell/PSCell and sSCell - Configuration of scaling factor α for BD and CCE limit handling and PDCCH overbooking handling on P(S)Cell - The number of unicast DCI limits for PCell/PSCell scheduling - Processing K1 unicast DCI scheduling DL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - Processing K2 unicast DCI scheduling UL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - FFS: N is based on pair of (PCell/PSCell SCS, sSCell SCS): N=1 for(15,15), (30,30), (60,60) and N=2 for (15,30), (30,60) and N=4 for (15, 60) - Same numerology between sSCell and P(S)Cell or sSCell SCS is larger than P(S)Cell SCS - USS set(s) for DCI format 0_1,1_1 configured on sSCell for CCS from sSCell to PCell/PSCell and USS set(s) for DCI format 0_2,1_2 configured on sSCell for CCS from sSCell to PCell/PSCell if UE supports FG 11-1 (dci-Format1-2And0-2-r16) - PDCCH monitoring occasion(s) - FFS: fFrame boundary alignment between PCell/PSCell and sSCell | 6-5 | Yes | N/A | Cross-carrier scheduling from SCell to PCell/PSCell (Type B) is not supported | Per BC | No | Applicable to FR1 only | No | Candidate value set: One or more of supported SCS combinations ({P(S)Cell SCS in kHz, sSCell SCS in kHz}) from following set are indicated by the UE: {15,15}, {15,30}, (15, 60), [{30,30}, {30,60},{60,60})] [Candidate value set 2: frequency band pair(s) for {PCell/PSCell, sSCell}] Component 4 candidate values: (K1, K2) = {(1,1) for FDD P(S)Cell; (K1, K2) = (1,2) for TDD P(S)Cell} Component 7 candidate values: [Value 1: PDCCH monitoring occasion(s) on PCell/PSCell and on sSCell for cross-carrier scheduling to PCell/PSCell is within the first 3 OFDM symbols of a PCell/PSCell slot. Value 2: PDCCH monitoring occasion(s) on PCell/PSCell and on sSCell for cross-carrier scheduling to PCell/PSCell is not restricted to the first 3 OFDM symbols of a PCell/PSCell slot (PDCCH MO capability follows the UE capability about MO reported via FG 3-x (as legacy))] Note: The CCS from sSCell to Pcell is applicable to FR1 only but there can be other Scells in FR2 configured for the UE Note: The SCell configured with Cross-carrier scheduling to PCell/PSCell is referred to as ‘sSCell’ | Optional with capability signalling |
| 34. NR_DSS | 34-1 | Cross-carrier scheduling from SCell to PCell/PSCell with search space restrictions (Type A) | Support of Cross-carrier scheduling from sSCell to PCell/PSCell with search space restrictions (Type A) - Cross-carrier scheduling from sSCell to PCell/PSCell with CIF - Search space restrictions: sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and at least following search space sets on PCell/PSCell can only be configured such that UE does not monitor them in overlapping slot of PCell/PSCell and sSCell - USS sets for DCI formats 0_1,1_1,0_2,1_2 - USS sets for DCI formats 0_0,1_0 - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI - Configuration of scaling factor α for BD and CCE limit handling and PDCCH overbooking handling on P(S)Cell - The number of unicast DCI limits for PCell/PSCell scheduling - Processing K1 unicast DCI scheduling DL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - Processing K2 unicast DCI scheduling UL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - FFS: N is based on pair of (PCell/PSCell SCS, sSCell SCS): N=1 for(15,15), (30,30), (60,60) and N=2 for (15,30), (30,60) and N=4 for (15, 60) - Same numerology between sSCell and P(S)Cell or sSCell SCS is larger than P(S)Cell SCS - USS set(s) for DCI format 0_1,1_1 configured on sSCell for CCS from sSCell to Pcell/PSCell and USS set(s) for DCI format 0_2,1_2 configured on sSCell for CCS from sSCell to PCell/PSCell if UE supports FG 11-1 (dci-Format1-2And0-2-r16) - sSCell USS set(s) (for CCS from sSCell to Pcell/PSCell) and Type0/0A/1/2 CSS sets on Pcell/PSCell can be configured so that the UE monitors them in overlapping slot of Pcell/PSCell and sSCell - no simultaneous monitoring between ‘USS sets (for P(S)Cell scheduling) on sSCell’ and ‘Type 0/0A/1/2/CSS sets on P(S)Cell for DCI formats with CRC scrambled by C-RNTI/MCS-C-RNTI/CS-RNTI’ - simultaneous monitoring of ‘USS sets (for P(S)Cell scheduling) on sSCell’ and ‘Type 0/0A/1/2/CSS sets on P(S)Cell for DCI formats with CRC not scrambled by C-RNTI/MCS-C-RNTI/CS-RNTI’ - FFS: Support of monitoring DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s) - PDCCH monitoring occasion(s) - FFS: fFrame boundary alignment between PCell/PSCell and sSCell | 6-5 | Yes | N/A | Cross-carrier scheduling from SCell to PCell/PSCell with search space restrictions (Type A) is not supported | Per BC | No | Applicable to FR1 only | No | Candidate value set: One or more of supported SCS combinations ({P(S)Cell SCS in kHz, sSCell SCS in kHz}) from following set are indicated by the UE: {15,15}, {15,30}, (15, 60), [{30,30}, {30,60},{60,60}]) [Candidate value set 2: frequency band pair(s) for {PCell/PSCell, sSCell}] Component 4 candidate values: (K1, K2) = {(1,1) for FDD P(S)Cell; (K1, K2) = (1,2) for TDD P(S)Cell, [(K1, K2) = (2,2) for FDD P(S)Cell; (K1, K2) = (2,4) for TDD P(S)Cell]} Component 9 candidate values: [Value 1: PDCCH monitoring occasion(s) on PCell/PSCell and on sSCell for cross-carrier scheduling to PCell/PSCell is within the first 3 OFDM symbols of a PCell/PSCell slot. Value 2: PDCCH monitoring occasion(s) on PCell/PSCell and on sSCell for cross-carrier scheduling to PCell/PSCell is not restricted to the first 3 OFDM symbols of a PCell/PSCell slot (PDCCH MO capability follows the UE capability about MO reported via FG 3-x (as legacy))] Note: The CCS from sSCell to PCell is applicable to FR1 only but there can be other SCells in FR2 configured for the UE Note: The SCell configured with Cross-carrier scheduling to PCell/PSCell is referred to as ‘sSCell’ | Optional with capability signalling |

- LTE_NR_DC_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 35. LTE_NR_DC_enh2 | 35-1 | Aperiodic CSI-RS for tracking for fast SCell activation | - Aperiodic CSI-RS for tracking for fast SCell activation is triggered by enhanced SCell activation/deactivation MAC CE - Aperiodic CSI-RS for tracking for fast SCell activation is triggered within the BWP indicated by firstActiveDownlinkBWP-Id for the SCell - Maximum number of aperiodic CSI-RS resource set configurations for tracking for fast SCell activation that can be configured to UE per CC in a reported band - Maximum number of aperiodic CSI-RS resource set configurations for tracking for fast SCell activation that can be configured to UE across CCs in a reported band | 6-5 | Yes | N/A | Aperiodic CSI-RS for tracking for fast SCell activation is not supported | Per band | N/A | N/A | N/A | Component 3 candidate values: {[1,] 8,16,32,48,64,128,255} Component 4 candidate values: {[1,] 8,16,32,64,128,256,512,1024} Note: In component 3, the candidate component values {FFS1,8} do not apply to FR2 Note: In component 4, the candidate component values {FFS1,8} do not apply to FR2 Note: component 3 and 4 candidate values refer to the number of RS configurations for fast SCell activation that can be indicated by the MAC CE The NZP-CSI-RS configured as RS for tracking for fast SCell activation are not considered when counting the maximum NZP-CSI-RS configurations of FG2-33 | Optional with capability signalling |