3GPP TSG-RAN WG1 Meeting #109-e																										R1-2203107

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.16.1

Source:	Huawei, HiSilicon

Title:	Rel-17 UE features for further NR MIMO enhancements

Document for:	Discussion and Decision

Introduction

In this paper, we provide our views on Rel-17 UE features for further NR MIMO enhancements, based on the updated version after RAN1#108-e in [1].

UE feature for <Enhancements on Multi-beam Operation>

Unified TCI framework

It was agreed in RAN#106-e that the maximum number of configured TCI state pools across BWPs and CCs in a band is reported as UE capability.

Agreement (RAN1#106-e)

On Rel.17 unified TCI framework, confirm the following working assumption as an agreement with a minor refinement highlighted in red

For common TCI state ID update and activation to provide common QCL information at least for UE-dedicated PDCCH/PDSCH and/or common UL TX spatial filter(s) at least for UE-dedicated PUSCH/PUCCH across a set of [configured] CCs/BWPs:

- RRC-configured TCI state pool(s) can be configured in the PDSCH configuration (PDSCH-Config) for each BWP/CC as in Rel-15/16

  - Note: Such RRC-configured TCI state pool(s) configuration doesn’t imply that separate DL/UL TCI state pool is excluded or supported

- RRC-configured TCI state pool(s) can be absent in the PDSCH configuration (PDSCH-Config) for each BWP/CC, and replaced with a reference to RRC-configured TCI state pool(s) in a reference BWP/CC

  - In the PDSCH configuration (PDSCH-Config) of the reference BWP/CC, RRC-configured TCI state pool(s) shall be configured

  - For a BWP/CC where the PDSCH configuration contains a reference to the RRC-configured TCI state pool(s) in a reference BWP/CC, the UE applies the RRC-configured TCI state pool(s) in the reference BWP/CC

- When the BWP/CC ID (i.e. bwp-Id or cell) for QCL-Type A/D source RS in a QCL-Info of the TCI state is absent, the UE assumes that QCL-Type A/D source RS is in the BWP/CC to which the TCI state applies

- Introduce a UE capability to report maximum number of TCI state pools it can support across BWPs and CCs in a band, and the candidate value at least includes 1

- FFS: Introduce a UE capability to report maximum number of configured TCI states that it can support across BWPs and CCs in a band

- FFS: How to define reference BWP/CC

In addition, to reduce memory consumption at UE, similar to legacy UE feature 2-4/2-59 in Rel-15/16, it should be supported to report the maximum number of configured unified TCI states across BWPs/CCs in a band in a band combination and the maximum number of configured unified TCI states per BWP per CC in a band in a band combination.

With above analysis, we propose the following:

Proposal 2-1: For FG 23-1-1, confirm the WA of component 2 with related brackets removed, and introduce a new component 2a

  - Component 2: The maximum number of configured joint TCI states per BWP per CC in a band in a band combination.

  - Component 2a: The maximum number of configured joint TCI states across all BWPs and all CCs in a band in a band combination

As one band can belong to more than one BC, it is more flexible to add “per BC” restriction in component 2 in FG 23-1-1e.

Proposal 2-2: Add “per BC” restriction in component 2 in FG 23-1-1e as follows,

  - Component 2: The maximum number of configured joint TCI state pools across all BWPs and all CCs in a band in a band combination.

It was agreed in RAN#105-e that the maximum number of activated UL TCI states or joint TCI state is reported as UE capability.

Agreement (RAN1#105-e)

On path-loss measurement for Rel.17 unified TCI framework, a PL-RS (configured for path-loss calculation) is either included in UL TCI state or (if applicable) joint TCI state or associated with UL TCI state or (if applicable) joint TCI state.

- Whether a UE supports “beam misalignment or not” (detailed definition FFS) between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is a UE capability

  - Note: The term “beam misalignment” is for discussion purpose only

- Whether it is ‘included in’ or ‘associated with’ (including the manner it is performed and the signaling) is up to RAN2

- The UE maintains the PL-RS of the activated UL TCI state or (if applicable) joint TCI state

- The maximum number of activated UL TCI states or (if applicable) joint TCI states per band per cell is a UE capability

- FFS: detailed aspects of PL-RS, e.g. CSI-RS type(s), restriction on configuration

- FFS: For the definition of “beam misalignment or not”, at least consider the case where the periodic DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication is configured/associated as the PL-RS

- Note: PL-RS is assumed to be periodic

Similarly, as one band can belong to more than one BC, it is more flexible to add “per BC” restriction in component 5 in FG 23-1-1. In addition, we also prefer per CC UE capability on maximum number of MAC-CE activated joint TCI states. So we have the following proposal.

Proposal 2-3: Add “per BC” restriction in component 5 and introduce a new component 5a in FG 23-1-1

  - Component 5: The maximum number of MAC-CE activated joint TCI states across all CCs in a band in a band combination.

  - Component 5a: The maximum number of MAC-CE activated joint TCI states per CC in a band in a band combination.

Similar to proposal 2-3, for FG 23-1-1b, we support component 3 and prefer to introduce a new component 3a as follows.

Proposal 2-4: Remove the brackets in component 3, and introduce a new component 3a in FG 23-1-1b

  - Component 3: The maximum number of MAC-CE activated joint TCI states per CC in a band in a band combination.

  - Component 3a: The maximum number of activated joint TCI states across all CCs in a band in a band combination

To reflect UE processing capability on number of CCs with beam failure recovery running, there needs to be a UE feature on maximum number of CCs on which spCell/SCell in Rel-15/16 under Rel-17 unified TCI framework can be configured. We then propose the following:

Proposal 2-5: Remove all the brackets in component 2 in FG 23-1-1c as follows

  - Component 2: Maximum number of CCs configured with SCell BFR with unified TCI framework in a band with SpCell BFR, with candidate values {1, 2, 3, 4, …, FFS}

Inter-cell measurement and reporting

As discussed in RAN1#106bis-e, regarding the number of configured additional PCIs (PCI different from serving cell PCI) for beam measurement, it is agreed that two independent X values (X1 and X2) are reported as UE capability for two different assumptions on additional SSB time domain position and periodicity with respect to serving cell SSB. In addition, it was agreed that this UE capability has FR1 and FR2 differentiation. It has been agreed that from RRC signaling perspective, the number of configured additional PCIs can be {1, 2, 3, 4, 5, 6, 7}.

Currently, UE capabilities (X1 and X2) corresponding to the number of configured additional PCIs are only included in FG23-4 (i.e., FG for inter-cell MTRP operation). There is a concern that if UE support inter-cell beam management but does not support inter-cell MTRP operation, UE will not report the value of X1 and X2 and hence gNB cannot know the number of configured additional PCIs for beam measurement. Hence, we suggest introducing two dedicated components in 23-1-2 for UE to reporting the number of configured additional PCIs for beam measurement.

For the number of configured additional PCIs for inter-cell beam management (component 10 and 11 in FG 23-1-2) and the number of configured additional PCIs for inter-cell MTRP operation (component 2 and 3 in FG 23-4), UE can report one of them or both of them. If only one of them is reported, it can be used for both inter-cell beam management and inter-cell MTRP operation.

Proposal 2-6: Introduce two components (component 10 and component 11 as follows) in FG 23-1-2 for the reporting of X1/X2 for inter-cell beam management.

  - Component 10: The maximum number of configured additional PCIs per CC is X1 (Case 1) when each configuration of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI;

  - Component 11: The maximum number of configured additional PCIs per CC is X2 (Case 2) when the configurations of SSB time domain positions and periodicity of the additional PCIs is different with SSB time domain positions and periodicity of the serving cell PCI;

Note: For the number of configured additional PCIs for inter-cell beam management (component 10 and 11 in FG 23-1-2) and the number of configured additional PCIs for inter-cell MTRP operation (component 2 and 3 in FG 23-4), UE can report one of them or both of them. If only one of them is reported, it can be used for both inter-cell beam management and inter-cell MTRP operation.

Regarding the number of SSBs with PCI(s) different from serving cell PCI for L1 beam measurement, to keep the memory consumption and computing complexity at UE under control, and also to put a limit on total number of SSB resources with PCI(s) same as or different from serving cell PCI, we propose the following:

Proposal 2-7: Remove the brackets in component 4/5 in FG 23-1-2 as follows

  - Component 4: The max number of SSB resources configured to measure L1-RSRP within a slot with PCI(s) same as or different from serving cell PCI across all CCs.

  - Component 5: The max number of SSB resources configured to measure L1-RSRP with PCI(s) same as or different from serving cell PCI across all CCs.

Inter-cell L1 measurement and inter-cell L3 measurement can be based the same SSB or different SSB. If they are based on different SSB, additional complexity will be introduced. Hence, it is reasonable to have a UE capability for this. Hence, we suggest supporting component 6 and remove the bracket.

Proposal 2-8: Remove the bracket in component 6 in FG 23-1-2.

  - Component 6: Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement.

In current spec, the total number of resources used for all kinds of DL measurements, including beam management, pathloss measurement, BFD, RLM and new beam identification is reported with UE capability FG 16-1g/16-1g-1. As inter-cell beam management is also a kind of DL measurement. So, the number of resources for inter-cell beam management should be counted into FG 16-1g/16-1g-1. Hence we suggest adding a note for this.

Proposal 2-9: Add a note in FG 23-1-2: Components 4 and 5 are also counted in FG 16-1g/16-1g-1.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-1-1 | Unified TCI with joint DL/UL TCI update for intra- [and inter-cell] beam management | - Joint DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals) - WA: The maximum number of configured joint TCI states [per BWP per CC] [in a band] [in a band combination] 2a. The maximum number of configured joint TCI states across all BWPs and all CCs in a band in a band combination - One MAC-CE activated joint TCI state per CC [in a band] [in a band combination] - TCI state indication [mode]: update and activation [in case of updates] a) MAC CE based TCI state indication [for one active TCI state] - The maximum number of MAC-CE activated joint TCI states across all CC(s) in a band in a band combination 5a. The maximum number of MAC-CE activated joint TCI states per CC in a band in a band combination |  | Yes |  | Unified TCI with joint DL/UL TCI update for intra- [and inter-cell] beam management is not supported | Per band | n/a | n/a | n/a | FFS: how to count the MAC-CE activated joint TCI | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-1b | Unified TCI with joint DL/UL TCI update for intra- [and inter-cell] beam management with more than one MAC-CE activated joint TCI state per CC | - TCI state indication [mode]: update and activation [in case of updates] b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) c) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) - [The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS] - The maximum number of MAC-CE activated joint TCI states per CC [in a band] [in a band combination] 3a. The maximum number of configured joint TCI states across all CCs in a band in a band combination - [The minimum time gap between the beam indication PDCCH and first slot where beam is applied] | 23-1-1 | Yes |  | Unified TCI with joint DL/UL TCI update for intra- [and inter-cell] beam management with more than one MAC-CE activated joint TCI state per CC is not supported | Per band | n/a | n/a | n/a | Component 3 candidate values: [{2, …}] Note: The maximum number of MAC-CE activated joint TCI states across all CC(s) in a band for more than one MAC-CE activated joint TCI state is signaled in 23-1-1, component 5 | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-1c | SCell BFR with unified TCI framework | 1. Support of SCell BFR with unified TCI framework [2. Maximum number of CCs configured with SCell BFR with unified TCI framework [in a band with SpCell BFR] |  | Yes |  | SCell BFR with unified TCI framework is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-1d | Per BWP TCI state pool configuration for CA mode | 1. Support of TCI state pool configuration per BWP for CA mode | 23-1-1 | Yes |  | Per BWP TCI state pool configuration for CA mode is not supported | Per band | n/a | n/a | n/a | FFS: A UE that supports 23-1-1 together with CA must indicate this FG is supported] | Optional with capability signaling |
| 23. NR_FeMIMO | 23-1-1e | TCI state pool configuration with TCI pool sharing for CA mode | 1. Support of reference BWP/CC configured with reference TCI state pool shared by a set of BWP/CC 2. The maximum number of configured joint TCI state pools across all BWPs and all CCs in a band in a band combination. | 23-1-1 | Yes |  | TCI state pool configuration with TCI pool sharing for CA mode is not supported | Per band | n/a | n/a | n/a | Component 2 candidate values: FFS FFS: A UE that supports 23-1-1 together with CA must support this FG] | Optional with capability signaling |
| 23. NR_FeMIMO | 23-1-1f | Common multi-CC TCI state ID update and activation | Common multi-CC TCI state ID update and activation | 23-1-1 | Yes |  | Common multi-CC TCI state ID update and activation is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 23. NR_FeMIMO | 23-1-1g | Beam misalignment between the DL source RS in the TCI state | Beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS | 23-1-1 | Yes |  | Beam misalignment between the DL source RS in the TCI state is not supported | Per band | n/a | n/a | n/a | FFS: FR2 only | Optional with capability signaling |
| 23. NR_FeMIMO | 23-1-1h | Association between TCI state and UL PC settings for PUCCH, PUSCH, and SRS | For PUCCH, PUSCH, and SRS, association between TCI state and UL PC settings except for PL RS | 23-1-1 | Yes |  | Association between TCI state and UL PC settings for PUCCH, PUSCH, and SRS is not supported | Per band | n/a | n/a | n/a | [Note: A UE that supports FG 23-1-1 must indicate this FG is supported] | Optional with capability signaling |
| 23. NR_FeMIMO | 23-1-1i | Indication/configuration of R17 TCI states for aperiodic CSI-RS, PDCCH, PDSCH [, and SRS] | Support of indication/configuration of R17 TCI states for aperiodic CSI-RS, PDCCH, PDSCH [, and SRS] (except for TRS and for CORESET #0 and the respective PDSCH reception) reusing the Rel-15/16 signaling/configuration design(s) | 23-1-1 | Yes |  | Indication/configuration of R17 TCI states for aperiodic CSI-RS, PDCCH, PDSCH [, and SRS] reusing the Rel-15/16 signaling/configuration design(s) is not supported | Per band | n/a | n/a | n/a | Note: This has no impact on detail signaling design for SRS TCI indication [A UE that supports 23-1-1 must indicate this FG is supported] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-1j | Indication/configuration of R17 TCI states for CORESET #0 | Support of indication/configuration of R17 TCI states for CORESET #0 and the respective PDSCH reception reusing the Rel-15/16 signaling/configuration design(s) | 23-1-1 | Yes |  | Support of indication/configuration of R17 TCI states for CORESET #0 and the respective PDSCH reception reusing the Rel-15/16 signaling/configuration design(s) is not supported | Per band | n/a | n/a | n/a | [A UE that supports 23-1-1 must indicate this FG is supported] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-2 | Inter-cell beam measurement and reporting [(for inter-cell BM [and mTRP])] | 1. Support of L1-RSRP measurement and reporting on SSB(s) with PCI(s) different from serving cell PCI 2. Support of up to K[=4] SSBRI-RSRP [pairs/beams] in one report [where at least one [pair/beam] associated with a PCI different from serving cell PCI can be reported] (FFS: if K is a component candidate value) 3. The maximum number of [RRC-configured] PCI(s) different from serving cell PCI for L1-RSRP measurement] (FFS: whether to split this for FR1 and FR2) (FFS: whether/how to capture different values/behaviors for periodic/aperiodic/semi-persistent L1-RSRP measurement) 4. The max number of SSB resources configured to measure L1-RSRP within a slot with PCI(s) same as or different from serving cell PCI [across all CC] [5. The max number of SSB resources configured to measure L1-RSRP with PCI(s) same as or different from serving cell PCI [across all CC]] [6. Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement] [7. Supported mode inter-cell measurement: {inside SMTC, both inside and outside SMTC}] [8. Supported mode of measurement over overlapped SSBs: {overlapped, both overlapped and non-overlapped}] [9. Maximum number of overlapped SSBs in one SSB resource for L1-RSRP measurement] 10. The maximum number of configured additional PCIs per CC is X1 (Case 1) when each configuration of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI 11. The maximum number of configured additional PCIs per CC is X2 (Case 2) when the configurations of SSB time domain positions and periodicity of the additional PCIs is different with SSB time domain positions and periodicity of the serving cell PCI | [2-24, 2-29] | Yes |  | Inter-cell beam measurement and reporting [(for inter-cell BM [and mTRP])] is not supported | per band | n/a | n/a | n/a | [Note: Whether component 4 and/or 5 are also counted in FG16-1g/16-1g-1] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-3 | MPE mitigation | 1. Support of enhanced [PHR] reporting which includes pairs of (P-MPR, SSBRI/CRI) 2. Maximum number of reported P-MPR and SSBRI/CRI pairs 3. Maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation |  | Yes |  | [MPE mitigation is not supported] | Per Band | n/a | n/a | n/a | 2. Candidate value of {1,2,3, 4} 3. Candidate value [{8, 12, 16, 28, 32, 48, 64}] Note: FR2 only | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-4 | MPUE support for UL | 1. Supported UE capability value [sets] and corresponding max number of SRS ports for each UE capability value [set] |  | Yes |  | [MPUE support for UL is not supported] | per band | n/a | n/a | n/a | Component 1 candidate values: Up to 4 value [sets] each with one value of {[0,]1,2,4} Note: the reported list contains only unique value [sets] This FG is a working assumption | Optional with capability signalling |

UE feature for <Enhancements for Multi-TRP Deployment >

Multi-TRP PDCCH repetition

Regarding the multi-TRP PDCCH repetition enhancement, three schemes have been agreed as SFN, TDM and FDM in Rel-17. For capability on SFN PDCCH, this has been captured as UE capability in HST-SFN feature. For FDM based PDCCH reception for FR2, as it requires two QCL-TypeD capability, it can be reported by FG 23-2-2. To make it clear, a note can be added to FG 23-2-1.

Proposal 3-1: Remove the brackets around the component 1 in FG 23-2-1,

  - 1. Support of intra-slot PDCCH repetition based on two linked SS sets associated with corresponding CORESETs with non-SFN scheme TDM and FDM (except FDM based PDCCH repetition for FR2) including PDCCH repetition for Type 3 CSS.

  - Add a note to FG 23-2-1: Note: This FG does not include supporting Two QCL-TypeD in time-domain overlapping CORESETs in FR2.

The following agreement has been agreed, but not captured yet.

Agreement

To handle UE complexity / memory requirements for linked PDCCH candidates, address the issue by UE capability, where UE indicates a limit (X) associated with the total number of linked candidates of which the first candidate is received and the second one has not been received at any given span.

- The limit X is indicated as a total count assuming count 1 for AL=1; 2 for AL=2; 4 for AL=4 or 8 or 16.

- The limit X is indicated per CC and also across all CCs

- Note: “received” and “not been received” is wrt the end of the corresponding span of PDCCH candidate.

Above is applicable at least for the inter-span case (FFS: intra-span case)

The value X related to AL was introduced to report the memory limit supported by UE at least for inter-span case. The remaining issue of whether the limit X can be applied for intra-span case is discussed in [2]. Therefore, we propose to add the new FG 23-2-3 and a candidate value for X indicated by UE.

Proposal 3-2: Add a new FG 23-2-3 as follows,

FG  23-2-3  UE memory requirements for linked PDCCH candidates

- 1. Support a limit (X) associated with the total number of linked candidates of which the first candidate is received and the second one has not been received at any given span.

- 2. The limit X is indicated as a total count assuming count 1 for AL=1; 2 for AL=2; 4 for AL=4 or 8 or 16.

- 3. The limit X is indicated per CC and also across all CCs

- The FG is applicable for the inter-span case and intra-span case.

Multi-TRP PUSCH repetition

In order to reduce the implementation complexity for a UE for the PHR calculation and report in MTRP across all CCs in a band, the max number of PHR report across CCs should be a UE capability.

Proposal 3-3: Support to add the component 2 in FG 23-3-1c,

  - 2. The maximum number of supported PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission) is a UE capability, FFS on candidate value, which is no more than 2*the number of supported CCs.

Inter-cell multi-TRP operation

It has been agreed that from RRC signaling perspective, the number of configured additional PCIs can be {1, 2, 3, 4, 5, 6, 7}. Therefore, to support more flexibility of gNB configuration and schedule, we propose the following:

Proposal 3-4: Support component 2 and 3 with candidate values {0, 1, 2, 3, 4, 5, 6, 7} in FG 23-4 and remove the corresponding brackets.

Beam Management for multi-TRP

A UE need simultaneous two panel activation to receive the simultaneous measurement on two CMR resource sets with two different QCL-TypeD in FR2, which would result in high UE implementation complexity. So we propose,

Proposal 3-5: Add a new FG 23-5-1b as follows,

FG 23-5-1b Two QCL TypeD for group based L1-RSRP reporting enhancement

  - Simultaneous reception of CSI-RS/SSB across two CMR sets with different QCL Type-D

The supported number of RSs for beam measurement/reporting for multi-TRP scenario in Rel-17 were agreed to be counted in FG 16-1g and 16-1g-1 in the last meeting. In addition, we propose the supported number of RSs for beam failure recovery for multi-TRP scenario in Rel-17 should be also be counted in FG 16-1g and 16-1g-1. We propose the following:

Proposal 3-6: Support adding the following note in FG 23-5-2: BFD-RS resources and NBI-RS resources for MTRP BFR are counted in FG 16-1g and 16-1g-1.

To reflect UE processing capability on number of CCs with beam failure recovery running, there needs to be a UE feature on maximum number of CCs on which spCell/SCell/MTRP BFR in Rel-15/16/17 can be configured. We then propose the following:

Proposal 3-7: Add a new component in FG 23-5-2 as follows

  - 4. The maximum number of CCs configured with BFR (including spCell/SCell/MTRP BFR in Rel-15/16/17), with candidate values {1, 2, 3, 4, 5, 6, 7, 8, 9}

HST multi-TRP deployment

It is proposed add a component of maximum number of codepoints configured for SFN data to FGs 23-6-1/1b/2/2b, to avoid the complexity of large number of combined TCI states for HST-SFN. Another option can be a new FG to maximal number of PDSCHs per slot. Either option can resolve the problem.

- Option 1: Add the following component to FGs 23-6-1/1b/2/2b

  - The maximal number of codepoints configured for SFN data, with candidate values {1, 2, 3, 4, 5, 6, 7, 8}

- Option 2: Add a new FG for PDSCH processing capability in HST

  - Component 1. The maximal number of PDSCHs per slot, with candidate values {1, 2, 3, 4 ,7}

With above analysis, we propose the following:

Proposal 3-8: down-select between the following options for HST-SFN FGs:

- Option 1: Add the following component to FGs 23-6-1/1b/2/2b

  - The maximal number of codepoints configured for SFN data, with candidate values {1, 2, 3, 4, 5, 6, 7, 8}

- Option 2: Add a new FG for PDSCH processing capability in HST

  - Component 1. The maximal number of PDSCHs per slot, with candidate values {1, 2, 3, 4 ,7}

In order to limit the UE implementation complexity in tracking activated TCI, similar to FG 2-4 in Rel-15 and FG 16-2a-7, the maximum number of activated TCI states should also be added in HST scenarios

Proposal 3-9: Support adding a new FG 23-6-5 for TCI tracking capability in HST as follow,

  - FG 23-6-5: Component 1. The maximal total number of activated TCI states per BWP per CC including data and control, with candidate values {1, 2, 4, 8 ,16};

  - FG 23-6-5: Component 2. The maximal total number of activated TCI states all BWPs all CCs including data and control, with candidate values {1, 2, 4, 8 ,16, 32};

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-2-1 | PDCCH repetition | 1. Support of intra-slot PDCCH repetition based on two linked SS sets associated with corresponding CORESETs [with [non-SFN scheme] TDM and FDM (except FDM based PDCCH repetition for FR2)] including PDCCH repetition for Type 3 CSS 2. Required number of BDs for the two PDCCH candidates 3. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier per slot |  | Yes |  | PDCCH repetition is not supported | Per FS | n/a | n/a | n/a | Component 2 candidate values: 2 or 3 Component 3 candidate values: {1,2,3, FFS more} Note: UE supports PDCCH repetition for the following (basic) PDCCH monitoring capability: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot Note: for component 3, each unique pair of overlaps is counted as one. | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-2 | Two QCL TypeD for CORESET monitoring in PDCCH repetition | 1.Support of determining two QCL-TypeD for time-domain overlapping CORESETs in the same CC or for intra-band CA when UE is configured with PDCCH repetition [with non-SFN TDM and/or FDM sheme] 2. Support of determining two QCL-TypeD for time-domain overlapping CORESETs in the same CC or for intra-band CA when UE is configured with PDCCH repetition with non-SFN FDM sheme | 23-2-1 | Yes |  | Two QCL TypeD for CORESET monitoring in PDCCH repetition is not supported | Per band | n/a | FR2 only | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-3 | UE memory requirements for linked PDCCH candidates | 1.Support a limit (X) associated with the total number of linked candidates of which the first candidate is received and the second one has not been received at any given span.2.The limit X is indicated as a total count assuming count 1 for AL=1; 2 for AL=2; 4 for AL=4 or 8 or 16. 3.The limit X is indicated per CC and also across all CCs The FG is applicable for the inter-span case and intra-span case. | 23-2-1 | Yes |  |  | Per band | n/a | n/a | n/a | Component 1 candidate value: {FFS} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1c | Two PHR reporting | 1. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion corresponding to each SRS resource set, and report two PHRs.) 2. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission) | 23-3-1 or 23-3-1-2 | Yes |  | Two PHR reporting is not supported | Per Band | n/a | n/a | n/a | FFS: on candidate value for component 2, […, 2*the number of supported CCs]. | Optional with capability signalling |
| 23. NR_FeMIMO | 23-4 | IntCell-mTRP | 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info 2. The maximum number of configured additional PCIs per CC is X1 (Case 1) when each configuration of SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI 3. The maximum number of configured additional PCIs per CC is X2 (Case 2) when the configurations of SSB time domain positions and periodicity of the additional PCIs is different with SSB time domain positions and periodicity of the serving cell PCI | 16-2a | Yes |  | IntCell-mTRP is not supported | Per band | n/a | n/a | n/a | Component 2 candidate values: {[0,]1,2,3,[4,5,6,]7} Component 3 candidate values: {0,1,2,3,[4,5,6,]7} Note: UE indicates a non-zero value for at least one of component 2 or component 3 FFS: how to count X1 and X2 [Note: case1 and case2 cannot be enabled simultaneously] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-5-1b | Two QCL TypeD for group based L1-RSRP reporting enhancements | 1. Simultaneous reception of CSI-RS/SSB across two CMR sets with different Type-D |  |  |  |  |  |  |  | FR2 only |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-5-2 | MTRP BFR based on two BFD-RS sets | 1. Maximum number of supported BFD-RS resources per set per BWP 3. Supported maximum number of BFD-RS resources across two BFD-RS sets per BWP 4. The maximum number of CCs configured with BFR (including spCell/SCell/MTRP BFR in Rel-15/16/17) |  | Yes |  | MTRP BFR based on two BFD-RS sets is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: {1, 2} Component 3 candidate values: {2,3,4} Component 4 candidate values: {1, 2, 3, 4, 5, 6, 7, 8, 9} Note:BFD-RS resources and NBI-RS resources for MTRP BFR are counted in FG 16-1g and 16-1g-1 | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-5 | Maximum number of activated TCI states for HST/[URLLC] | 1. The maximal number of activated TCI states per BWP per CC including data and control 2. The maximal number of activated TCI states all BWPs all CCs including data and control |  |  |  |  |  |  |  |  | Candidate values for Component 1: {1,2,4,8,16} Candidate values for Component 2: {1,2,4,8,16, 32} |  |
| 23. NR_FeMIMO | 23-6-6 | PDSCH processing capability for HST/[URLLC] | 1. The maximal number of PDSCH per slot |  |  |  |  |  |  |  |  | Candidate values for Component 1: {1,2,3,4,7} |  |

UE feature for <Enhancements on SRS flexibility, coverage and capacity >

In last meeting, 1 aperiodic SRS resource set was supported for 1T4R, which is UE optional.

Agreement

Support N = 1 for aperiodic SRS configuration for 1T4R

- This new configuration is UE optional.

According to the agreement, a new UE capability should be added for the new feature. Therefore, we propose

Proposal 4-1: Add a new FG 23-8-10 for 1 aperiodic SRS resource set for 1T4R.

For partial sounding, following agreement was achieved in last meeting:

Agreement

RPFS is applicable for both frequency hopping and non-frequency hopping cases, where support of RPFS for non-FH case is an optional UE feature for UEs supporting RPFS.

According to the agreement, a new UE feature is needed for RPFS for the non-frequency hopping cases. Therefore, we propose

Proposal 4-2: Add a new FG 23-8-11 for RPFS for non-FH case.

For 23-8-3, the candidate value of component 1 is “a combination from the set {t1r1, t2r2, t1r2, t4r4, t2r4, t1r4, t2r6, t1r6, t4r8, t2r8, t1r8}”, which means it can indicate ≤4R antenna switching configuration.  Notice that Rel-15/16 capability also can indicate ≤4R antenna switching configuration, then how to deal with the situation where Rel-15/16 capability and Rel-17 capability indicating same value for component 1 (e.g., 1t4r) and different value for component 2/3 should be further clarified. Therefore we propose to add the note below.

Proposal 4-2: Add following note for 23-8-3 to clarify the rule of dealing with the situation where Rel-15/16 capability and Rel-17 capability indicating same value for component 1 and different values for component 2/3:

Note: For the xTyR antenna switching configurations that are both reported here and in Rel-15/16 SRS antenna switching feature (2-55 and 14-4), the component 2 and 3 of this feature apply.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-8-1 | SRS triggering offset enhancement | The maximum number of configured available slots offsets for determining aperiodic SRS location based on available slot | 2-52 | Yes |  | SRS triggering offset enhancement is not supported | Per band | n/a | n/a | n/a | Candidate 1 component values: {1, 2, 4} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-2 | Triggering SRS only in DCI 0_1/0_2 | Support of triggering SRS in DCI 0_1/0_2 without data and without CSI | 2-52 | Yes |  | Triggering SRS only in DCI 0_1/0_2 is not supported | per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-3 | SRS Antenna switching for >4Rx | 1. Support of SRS antenna switching xTyR with y>4 2. Report the entry number of the first-listed band with UL in the band combination that affects this DL 3. Report the entry number of the first-listed band with UL in the band combination that switches together with this UL | 2-55 | Yes |  | SRS Antenna switching for >4Rx is not supported | Per FS | n/a | n/a | n/a | Component 1 candidate values: a combination from the set {t1r1, t2r2, t1r2, t4r4, t2r4, t1r4, t2r6, t1r6, t4r8, t2r8, t1r8} Note: For any indicated value, x shall be equal to or smaller than the one associated with the largest y Component 2 candidate values: {1 to 32} Component 3 candidate values: {1 to 32} Note: Component 2 and component 3 is not reported if component 1 is reported as xTyR with x=y. Note: For the xTyR antenna switching configurations that are both reported here and in Rel-15/16 SRS antenna switching feature (2-55 and 14-4), the component 2 and 3 of this feature apply. | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-4 | Maximum 2 SP and 1 periodic SRS sets for antenna switching | Support of maximum 2 SP SRS resource sets and maximum 1 periodic SRS resource set for antenna switching | 2-53 | Yes |  | Maximum 2 SP and 1 periodic SRS sets for antenna switching is not supported | Per FS | n/a | n/a | n/a | Note1: - Applies for all supported xTyR where y<=8 - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - For xTyR where y<=4, if UE does not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS - The two SP-SRS resource sets are not activated at the same time | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-5 | Increased repetition for SRS | Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS resource | 10-11, 2-52 | Yes |  | Increased repetition for SRS is not suported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-6 | Partial frequency sounding of SRS | Support of partial frequency sounding for SRS | 2-52 | Yes |  | Partial frequency sounding of SRS is not suported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-7 | Start RB location hopping for partial frequency SRS | Support of start RB location hopping in partial frequency SRS transmission across different SRS frequency hopping periods for periodic/semi-persistent/aperiodoc SRS | 23-8-6 | Yes |  | Start RB location hopping for partial frequency SRS is not suported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-8 | Comb-8 SRS | Support of comb-8 for SRS other than for positioning |  | Yes |  | comb-8 for SRS other than for positioning is not suported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-9 | Extension of aperiodic SRS configuration for 1T4R, 1T2R and 2T4R | Support of 4 aperiodic SRS resource sets for 1T4R and 2 aperiodic resource sets for 1T2R/2T4R. | 2-53, 2-55 | Yes |  | Extension of aperiodic SRS configuration for 1T4R, 1T2R and 2T4R is not supported | Per FS | N/A | N/A | N/A | [Note: When UE only supports part of {1T4R, 1T2R, 2T4R}, this FG is only applicable to the antenna switching configuration(s) that UE supports] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-10 | 1 aperiodic SRS resource set for 1T4R | Support of 1 aperiodic SRS resource sets for 1T4R. | 10-11, 2-55 | Yes |  | 1 aperiodic SRS resource set for 1T4R is not supported | Per FS | N/A | N/A | N/A |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-11 | Partial frequency sounding of SRS for non-frequency hopping case | Support of partial frequency sounding for SRS for non-frequency hopping case. | 23-8-6 | Yes |  | Partial frequency sounding of SRS for non-frequency hopping case is not supported | Per band | N/A | N/A | N/A |  | Optional with capability signalling |

UE feature for <CSI enhancements: MTRP and FR1 FDD reciprocity >

Since the number of CMRs associated with CMR pair for NCJT measurement is always even, it is unnecessary to include the odd candidate values for Component 4 c) of FG 23-7-1,.

Proposal: 5.1: Delate the odd candidate values for Component 4 c) of FG 23-7-1.

The UE is not mandated to support simultaneous reception with different QCL-TypeD for Rel-16 NCJT operation. Therefore UE FG 16-2c simultaneousReceptionDiffTypeD-r16 is used to indicate whether UE supports simultaneous reception with different QCL-TypeD reference signal, as 38.306.

However it is unclear whether simultaneousReceptionDiffTypeD-r16 can be applied CSI-IM resources, since one may argue that CSI-IM is not reference signal. Therefore if simultaneousReceptionDiffTypeD-r16 reports “not support” (i.e. such an IE is not reported by UE), NW may be able to configure two CMRs with different QCL-TypeD with a TDM manner, e.g., over different symbols in a slot.

Then according to following agreement in RAN1 #104bis-e:

Agreement

The UE may assume that QCL-Type D of CMRs associated with a NCJT measurement hypothesis are applied to the corresponding CSI-IM resource.

The UE still have to apply two different QCL-Type D of CMRs associated with a NCJT measurement hypothesis over the corresponding CSI-IM resource simultaneously. Therefore the UE needs to support two receive beams simultaneously which is contradictive to the spirit of FG 16-2c to report “not support”.

So we have the following proposal:

Proposal 5.2: Add a new FG 23-7-6 to indicate whether UE supports simultaneous reception with different QCL-TypeD for CSI-IM resource associated with NCJT measurement hypothesis.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-7-1 | Basic Features of CSI Enhancement for Multi-TRP | - Support of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Support of N=1 - Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max - CSI report mode selection of mode 1 with X=0 and/or mode 2 - A list of supported combinations, up to 16, across all CCs simultaneously, where each combination is - Maximum number of Tx ports in one NZP CSI-RS resource associated with an NCJT measurement hypothesis - Maximum total number of CMRs for NCJT measurement - Maximum total number of Tx ports of NZP CSI-RS resources associated with NCJT measurement hypotheses - [A list of (Y1,Y2): UE can process Y1 NCJT CSI and Y2 sTRP CSI measurement hypothesis simultaneously in a CC] - [A list of (X1,X2): UE can process X1 NCJT CSI and X2 sTRP CSI measurement hypothesis simultaneously across all CCs] |  | Yes |  | CSI Enhancement for Multi-TRP is not supported | Per band and per BC | n/a | n/a | n/a | Component 2 candidate value set: {2, 3, 4, 5, 6, 7, 8} Component 3 candidate value set: { mode 1 with X=0, mode 2, both} Component 4 candidate values: - {2, 4, 8, 12, 16, 24, 32} - {2,3,4 … : 2: 64} - {2,3,4, …, 256} [Component 5: The list can have maximum of 16 pairs. - Y1: {1 to 4} - Y2: {1 to 8}] [Component 6: The list can have maximum of 16 pairs. - X1: {1 to 16} - X2: {1 to 32}] Note: ‘NCJT’ is not used in RAN1 specifications and will be aligned with 38.214 | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-1b | Active CSI-RS resources and ports in the presence of multi-TRP CSI | 1. List of codebook combinations 2. List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination | 23-7-1 | Yes |  | [Active CSI-RS resources and ports in the presence of multi-TRP CSI is not supported] | Per band and per BC | n/a | n/a | n/a | Component 1 candidate values: Codebook 1 = {[‘NCJT’,] NCJT+Type 1 SP (for sTRP)} {Codebook 2, Codebook 3} = {(NULL, NULL}), {“Rel 16 combinations in FG 16-8”}, {“New Rel17 combinations in FG 23-9-5”}} Component 2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {2, 4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} Note 1: A CMR pair configured for NCJT will be counted as two activated resources, a CMR configured for sTRP will be counted as one activated resource for a triplet. Note2: This capability is relevant only when UE is configured with NCJT CSI in at least one CSI report setting in at least one CC in the band and/or band combination. | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-1a | Additional CSI report mode 1 | Maximum value of numberOfSingleTRP-CSI-Mode1 | 23-7-1 | Yes |  | CSI report mode 1 with X=1, or X=2 is not supported | Per Band | n/a | n/a | n/a | Component 1 candidate value set: { X=1, X=2} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-4 | Support of Nmax=2 for Multi-TRP CSI | Support of maximum number of CMR pairs Nmax=2 configured in NZP-CSI-RS-ResourceSet for a given CSI report setting | 23-7-1 | Yes |  | Nmax=2 for Multi-TRP CSI is not supported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-5 | CMR sharing | Support a NZP CSI-RS resource referred by both a CMR pair configured for Rel-17 Multi-TRP CSI enhancement and a single CMR configured for Single-TRP measurement in a CSI reporting setting | 23-7-1 | Yes |  | CMR sharing is not supported | Per band | n/a | FR2 only | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-6 | Simultaneous reception of CSI-IM with different Type-D | Supports simultaneous reception with different QCL Type-D over CSI-IM resources for Multi-TRP CSI | 23-7-1 |  |  |  | Per UE |  | FR2 only |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-1 | Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II) | - {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with M=1 and R=1 - Support rank 1,2 - Support parameter combinations with M=1 | 2-35 | Yes |  | Further Enhanced Port-Selection Type II Codebook (FeType-II) is not suported | Per band and per BC | n/a | n/a | n/a | Component 1 candidate values: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-5 | Active CSI-RS resources and ports for mixed codebook types in any slot | - List of codebook combinations - List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination | 23-9-1, 16-3a, 2-36, 2-40, 2-41, 23-9-2, 23-9-4 | Yes |  | Active CSI-RS resources and ports for mixed codebook types in any slot is not suported | Per band and per BC | n/a | n/a | n/a | Component 1 candidate values: Codebook 1 = {Type I SP, Type I MP} {Codebook 2, Codebook 3} = {{FeType II PS M=1, NULL},{FeType II PS M=2 R=1, NULL}, {FeType II PS M=2 R=2, NULL}, {Type II, FeType II PS M=1}, {Type II, FeType II PS M=2 R=1} ,{eType II R=1, FeType II PS M=1},{eType II R=1, FeType II PS M=2 R=1}} Component 2 candidate values: - Maximum 16 triplets for each codebook combination - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} Note 1：if a UE reports one or more codebook combinations in 23-9-5, then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations Note 2: For coexisting of mixed codebooks in any slot, gNB need to honor 16-8, 23-9-5 and per-codebook capability 2-36/40/41, 16-3a, and 23-9-1/23-9-2/23-9-4 Note 3: Up to 4 combinations for component 1 | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-2 | Support of M=2 and R=1 for FeType-II | 1. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with M=2 and R=1 2. Support parameter combinations with M=2 | 23-9-1 | Yes |  | M=2 and R=1 for FeType-II is not suported | per band and per BC | n/a | n/a | n/a | Component 1 candidate values - Maximum 8 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-3 | Support of rank 3, 4 for FeType-II | Support of rank 3, 4 for FeType-II | 23-9-1 | Yes |  | Rank 3, 4 for FeType-II is not suported | Per band | n/a | n/a | n/a |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-4 | Support of R = 2 for FeType-II | 1. Support of R = 2 for FeType-II 2. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with M=2 and R=2 | 23-9-2 | Yes |  | R = 2 for FeType-II is not suported | per band and per BC | n/a | n/a | n/a | Component 2 candidate values: • Maximum 8 triplets • Max # of Tx ports in one resource: {4,8,12,16,24,32} • Max # resources: {1 to 64} • Max # total ports: {4 to 256} | Optional with capability signalling |

Conclusions

This contribution presents our views on Rel-17 UE features for further NR MIMO enhancements, which can be found in above for detailed comments and suggestions. Further discussion and consolidation by RAN1 is required.

- References

R1-2202928, Updated RAN1 UE features list for Rel-17 NR after RAN1 #108-e, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#108-e, February 21 - March 3, 2022.

R1-2203106, Remaining issues on enhancements for multi-TRP deployment, Huawei, HiSilicon, RAN1#109-e, May 9 – May 20, 2022.