3GPP TSG-RAN WG1 Meeting #109-e 	R1-2203110

e-Meeting, May 9th – 20th, 2022

Agenda Item:	8.16.6

Source:	Huawei, HiSilicon

Title:	Rel-17 UE features for RedCap

Document for:	Discussion and Decision

Introduction

In this contribution, we provide our views on R17 RedCap UE features.

Discussion

2.1 CORESET number

For NR Rel-15/16 UE, the basic FG 3-1 for DL control channel is support of “one configured CORESET per BWP per cell in addition to CORESET0” [1]. So gNB can only configure one CORESET with ControlResourceSetId other than 0 in a BWP (initial/non-initial BWP), no matter if the BWP contains the bandwidth of CORESET0 or not. If RedCap UEs follow the same rule, only one CORESET can be used for both common and dedicated search spaces, when the BWP does not contain the bandwidth of CORESET0, which could significantly limit the flexibility of network scheduling, or even cause congestion problem compared with two separate CORESETs in legacy operation. Thus, we propose

Proposal 1: A RedCap UE should support one configured CORESET per BWP per cell in addition to CORESET0 if the BWP contains CORESET0, otherwise, support two configured CORESETs per BWP per cell in addition to CORESET0.

2.2 NCD-SSB

In TR 38.822, FG 6-1 and FG 6-1a are described as following:

| 6-1 | Basic BWP operation with restriction | 1) 1 UE-specific RRC configured DL BWP per carrier 2) 1 UE-specific RRC configured UL BWP per carrier 3) RRC reconfiguration of any parameters related to BWP 4) BW of a UE-specific RRC configured BWP includes BW of CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell |
| --- | --- | --- |
| 6-1a | BWP operation without restriction on BW of BWP(s) | BW of UE-specific RRC configured BWP may not include BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP may not include SSB for SCell |

Since the basic FG 6-1 also addresses CORESET#0, it is not preferable to use an existing FG with different interpretation and there are other related aspects associated with NCD-SSB, including number of SSBs, time domain periodicity and location, clarification of measurements etc.. Therefore for RedCap UEs, we suggest to introduce a new FG to report whether the UE expect NCD-SSB in the active BWP, e.g. to perform RRM/RLM/beam measurement of the serving cell. It is mandatory capability with signalling bit for proper test purpose.

Proposal 2: Define a new FG for a RedCap UE to report whether it expects NCD-SSB or not in the active BWP, and no more than one SSB within one active BWP, the periodicity of NCD-SSB is not expected to be shorter than CD-SSB, support of a configurable time-domain offset from CD-SSB, and support of measurements based on NCD-SSB including RRM, RLM and BM.

Conclusions

In this contribution, we presented our views on NCD-SSB for RedCap UEs:

Proposal 1: A RedCap UE should support one configured CORESET per BWP per cell in addition to CORESET0 if the BWP contains CORESET0, otherwise, support two configured CORESETs per BWP per cell in addition to CORESET0.

Proposal 2: Define a new FG for a RedCap UE to report whether it expects NCD-SSB or not in the active BWP, and no more than one SSB within one active BWP, the periodicity of NCD-SSB is not expected to be shorter than CD-SSB, support of a configurable time-domain offset from CD-SSB, and support of measurements based on NCD-SSB including RRM, RLM and BM.

- References

- 3GPP TR38.822, “User Equipment (UE) feature list”.