3GPP TSG-RAN WG1 Meeting #109-e 	R1-2203111

e-Meeting, May 9th – 20th, 2022

Agenda Item:	7.2.5

Source:	Huawei, HiSilicon

Title:	Remaining issues on SRS

Document for:	Discussion and Decision

Introduction

In previous meetings the SRS configurations with usage of value ‘codebook’, ’nonCodebook’ and ‘antennaSwitching’ applied for DCI format 0_2/1_2 were addressed. The case that has not been discussed yet is when the SRS resource set with the higher layer parameter usage is set to 'beamManagement'. Then, it is still unclear if this usage also is applicable to DCI formats 0_2/1_2 and, if yes, how to determine the maximum number of SRS resource sets. This is addressed in this paper and a potential solution is proposed.

The number of SRS resource set with usage of value ‘BeamManagement’

In Rel-15, the SRS resource set(s) configured in srs-ResourceSetToAddModList can be used for beam management. For SRS resource set(s) with usage set to ‘beamManagement’, FG 2-30 [1] was introduced to let the UE report the maximum number of SRS resource sets and the maximum number of SRS resources per resource set that can be supported. The related description in TS38.331 [2] is copied below.

uplinkBeamManagement SEQUENCE { maxNumberSRS-ResourcePerSet-BM ENUMERATED {n2, n4, n8, n16}, maxNumberSRS-ResourceSet INTEGER (1..8) } OPTIONAL,

In Rel-16, independent from srs-ResourceSetToAddModList used for DCI format 0_1/1_1, srs-ResourceSetToAddModListDCI-0-2 was introduced for DCI formats 0_2/1_2, and the SRS resource set(s) configured in srs-ResourceSetToAddModListDCI-0-2 can be set to usage ‘beamManagement’.

In our view, similar to the previous discussions for the other usages of SRS resource sets, the maximum number of SRS resource sets configured with usage ‘beamManagement’ and the maximum number of SRS resource per set should not be increased compared to Rel-15, since this would need a new UE capability which is not reasonable for this functionality at this late stage.

In Rel-15, the maximum number of SRS resource sets for usage ‘beamManagement’ and the maximum number of SRS resource per set is reported as a UE capability in FG 2-30. We propose that the maximum value for the number of SRS resource set(s) reported in this FG shall be applicable regardless if SRS resource set(s) are configured in srs-ResourceSetToAddModList or in srs-ResourceSetToAddModListDCI-0-2. Similar to ‘antennaSwitching’, for usage ‘beamManagement’, the SRS resource set(s) in srs-ResourceSetToAddModListDCI-0-2 should be the same or a subset of the SRS resource set(s) in srs-ResourceSetToAddModList.

Furthermore, we suggest that the value reported for the maximum number of SRS resource per set shall be applicable to any SRS resource set, regardless is the SRS resource set is configured in srs-ResourceSetToAddModList or in srs-ResourceSetToAddModListDCI-0-2.

We are therefore making the following proposal:

Proposal: In Rel-16, for SRS resource set(s) configured with usage ‘beamManagement’

- The supported maximum number of SRS resource set(s) reported in FG 2-30 shall be applicable regardless of if SRS resource set(s) with usage set to 'beamManagement' are configured in srs-ResourceSetToAddModList or in srs-ResourceSetToAddModListDCI-0-2.

- For usage ‘beamManagement’, the SRS resource set(s) in srs-ResourceSetToAddModListDCI-0-2 are the same or a subset of the SRS resource set(s) in srs-ResourceSetToAddModList.

- The supported maximum number of SRS resources per set reported in FG 2-30 shall be applicable to any SRS resource set with usage set to 'beamManagement', regardless if the SRS resource set is configured in srs-ResourceSetToAddModList or in srs-ResourceSetToAddModListDCI-0-2.

Conclusions

In this contribution, we discuss the limit of maximum number of SRS resource per set and SRS resource set(s) that can be configured for beam management in srs-ResourceSetToAddModList and in srs-ResourceSetToAddModListDCI-0-2. We are making the following proposal:

Proposal: In Rel-16, for SRS resource set(s) configured with usage ‘beamManagement’

- The supported maximum number of SRS resource set(s) reported in FG 2-30 shall be applicable regardless of if SRS resource set(s) with usage set to 'beamManagement' are configured in srs-ResourceSetToAddModList or in srs-ResourceSetToAddModListDCI-0-2.

- For usage ‘beamManagement’, the SRS resource set(s) in srs-ResourceSetToAddModListDCI-0-2 are the same or a subset of the SRS resource set(s) in srs-ResourceSetToAddModList.

- The supported maximum number of SRS resources per set reported in FG 2-30 shall be applicable to any SRS resource set with usage set to 'beamManagement', regardless if the SRS resource set is configured in srs-ResourceSetToAddModList or in srs-ResourceSetToAddModListDCI-0-2.

References

3GPP TR 38.822 V15.0.1 User Equipment (UE) feature list (Release 15)

3GPP TS 38.331 V16.7.0 Radio Resource Control (RRC) protocol specification (Release 16)