3GPP TSG-RAN WG1 Meeting #109-e	R1-2203112

e-Meeting, May 9 – 20, 2022

Agenda Item:	7.1

Source:	Huawei, HiSilicon

Title:	Discussion on PL-RS configuration of SCells

Document for:	Discussion and Decision

Introduction

In RAN1#107 meeting, an LS [1] from RAN4 is sent to RAN1. RAN4 has discussed the PL-RS configuration used for PUCCH transmission on target being-activated SCell during the activation procedure. RAN4 understands PL-RS assumption of PUCCH transmission for the active DL BWP of the carrier of the primary cell is specified in RAN1 TS38.213 section 7.2.1 because a UE is not required to obtain MIB of a SCell during the SCell activation procedure. However, RAN4 is not sure what the UE behavior is to determine the PL-RS for PUCCH of to-be-activated SCell. Thus, RAN4 has made following working assumption and asked RAN1 to confirm/clarify:

Working assumption: - RAN4 to agree that PL-RS assumptions defined in TS38.213 section 7.2.1 can be applied for the PUCCH of target being-activated SCell during the activation procedure. In FR2 if UE is not provided pathlossReferenceRSs but provided PUCCH-SpatialRelationInfo before receiving the PUCCH SCell activation command, as UE may not obtain MIB during activation procedure, UE shall use the associated DL-RS in PUCCH-SpatialRelationInfo as PL-RS. [Question to RAN1]: Is the above working assumption in line with RAN1’s understanding?

The RAN4 working assumption has surely RAN1 specification impact, thus the solution to the issue should be clarified in RAN1.

In this contribution, our views on this issue are provided.

Discussion

PL-RS used for pathloss estimation for PUCCH is specified in RAN1 TS38.213 section 7.2.1. According to the specification, it can be observed that:

- If pathlossReferenceRS is not provided, then UE RS resource obtained from an SS/PBCH block with same SS/PBCH block index as the one the UE uses to obtain MIB.

- If UE is provided pathlossReferenceRSs and PUCCH-SpatialRelationInfo, UE determine the PL-RS according to the pucch-PathlossReferenceRS-Id in UL spatial relation.

- If UE is provided with pathlossReferenceRSs and is not provided PUCCH-SpatialRelationInfo, then UE use the pucch-PathlossReferenceRS-Id with index 0.

- If UE is not provided with pathlossReferenceRSs and PUCCH-SpatialRelationInfo, then UE determine the PL-RS according to the CORESET with lowest index.

When PUCCH SCell is configured for UE, the UE shall apply the above procedures to determine a downlink pathloss. For the case if the pathlossReferenceRS is not provided, UE RS resource can be obtained from MIB in PCell. However, UE is not required to obtain MIB in SCell. Therefore, how to get the RS resource for SCell is unclear in the current specification.

The solution in RAN4 LS forces a gNB to configure  PUCCH-SpatialRelationInfo. the UE behavior is still not clear when pathlossReferenceRS is not configured and UE does not obtain MIB in FR1 PUCCH SCell. As a result, it cannot be well extended to FR1 cases because usually the PUCCH-SpatialRelationInfo may not be configured in FR1.

In addition to power control for PUCCH, the same issue is applied to PUSCH and SRS, i.e. the UE behavior for PUSCH/SRS in TS38.213 section 7.1.1 and 7.3.1 are also not clear when pathlossReferenceRS is not configured and UE not does not obtain MIB in FR1/FR2 SCell.

In the RAN1#92 meeting, an agreement for NR UL power control – non-CA aspects is achieved as follows:

Agreement: At least for the case of initial access - UE will use the SSB identified during the initial access as the DL RS/SSB for pathloss estimation for PUSCH (including MSG3) before DL RS(s) is explicitly configured for pathloss measurement. - UE will use the SSB identified during the initial access as the DL RS/SSB for pathloss estimation for PUCCH before DL RS(s) is explicitly configured for pathloss measurement.

The agreement is captured in current specification TS38.213 7.1.1/7.2/1/7.3.1 for PUSCH/PUCCH/SRS power control and does not distinguish between CA and non-CA.

Misalignment between RAN1 and RAN4 specification should be avoided. A unified solution is preferred for all PUSCH, PUCCH and SRS. With the RAN1 agreement in RAN1#92, it is quite natural to reuse the same SS/PBCH that has been used by UEs for SCell activation. Therefore, to clarify the UE behaviors in case of SCell, our proposals along with a companion CR [2] (i.e. R1-2203113) are,

Proposal 1: To determine a PUSCH/PUCCH/SRS transmission power for SCell, clarifications are proposed as follows:

- When determine a PUSCH transmission power, if the UE is not provided PUSCH-PathlossReferenceRS for SCell, UE calculates PLb,f,c(qb) using a RS resource obtained from an SS/PBCH block used for SCell activation.

- When determine a PUCCH transmission power, if the UE is not provided pathlossReferenceRSs for SCell, UE calculates PLb,f,c(qb) using a RS resource obtained from an SS/PBCH block used for SCell activation.

- When determine a SRS transmission power, if the UE is not provided pathlossReferenceRS or SRS-PathlossReferenceRS-Id for SCell, UE calculates PLb,f,c(qb) using a RS resource obtained from an SS/PBCH block used for SCell activation.

Proposal 2: Adopt the CR in R1-2203113 about the clarifications of determining PUSCH/PUCCH/SRS transmission power.

Conclusion

According to the above discussions, we have the following proposals:

Proposal 1: To determine a PUSCH/PUCCH/SRS transmission power for SCell, clarifications are proposed as follows:

- When determine a PUSCH transmission power, if the UE is not provided PUSCH-PathlossReferenceRS for SCell, UE calculates PLb,f,c(qb) using a RS resource obtained from an SS/PBCH block used for SCell activation.

- When determine a PUCCH transmission power, if the UE is not provided pathlossReferenceRSs for SCell, UE calculates PLb,f,c(qb) using a RS resource obtained from an SS/PBCH block used for SCell activation.

- When determine a SRS transmission power, if the UE is not provided pathlossReferenceRS or SRS-PathlossReferenceRS-Id for SCell, UE calculates PLb,f,c(qb) using a RS resource obtained from an SS/PBCH block used for SCell activation.

Proposal 2: Adopt the CR in R1-2203113 about the clarifications of determining PUSCH/PUCCH/SRS transmission power.

- References

R4-2202602, “LS on the PL-RS configuration of PUCCH SCell to be activated”.

R1-2203113, “Corrections on PL-RS configuration”.