3GPP TSG-RAN WG1 Meeting #109-e	R1-2203114

e-Meeting, 9th May – 20th May 2022

Agenda Item:	8.6.1

Source:	Ericsson

Title:	Maintenance issues for UE complexity reduction for RedCap

Document for:	Discussion, Decision

# 1	Introduction

We have identified the following RAN1 maintenance issues related to UE complexity reduction for RedCap:

- Issue #1: Clarification of UE behavior when separate initial DL BWP is not configured

  - Regarding clarification of the UE behavior when separate initial DL BWP is not configured, the three different options presented in Proposal 2-1-2b in the feature lead summary in [4] were discussed in RAN1#108e but it was not possible to reach consensus during the meeting for any of the options (nor the suggested compromises in Proposal 2-1-2c or Proposal 2-1-2d) due to lack of time.

- Issue #2: Presence of SSB transmission in separate initial DL BWP in connected mode for BWP#0 configuration option 1

  - Regarding presence of SSB transmission in separate initial DL BWP in connected mode for BWP#0 configuration option 1, the two options presented in Proposal 3-1h in the feature lead summary in [4] were discussed in RAN1#108e but it was not possible to reach consensus during the meeting for any of the options due to lack of time.

- Issue #3: Confirm working assumption for collision handling between SSB and Msg3 or PUCCH in response to Msg4/MsgB for HD-FDD UE

  - Regarding collision handling between SSB and Msg3 or PUCCH in response to Msg4/MsgB for HD-FDD UE, RAN1#108-e made a working assumption to reuse the same handling as for other dynamically scheduled UL transmission and prioritize SSB (with a note that it is up to the Rel-17 Coverage Enhancement WI whether to reuse this collision rule for Msg3 PUSCH repetition), but it was not possible to reach consensus during the meeting regarding how to resolve the working assumption due to lack of time.

- Issue #4: Corrections for RedCap BWP operation description in clause 17.1 of TS 38.213

  - Some of the RedCap BWP operation description captured in clause 17.1 of TS 38.213 were not fully consistent with the agreements made during the Rel-17 RedCap WI. Therefore, we provide text proposals for this clause.

# 2	Issue #1: UE behavior when separate initial DL BWP is not configured

One of the remaining open issues is regarding the UE behavior when a separate RedCap-specific initial DL BWP is not configured, and the non-RedCap initial DL BWP (DL BWP#0) is wider than the maximum RedCap UE bandwidth. Clearly, when the non-RedCap initial DL BWP is wider than the maximum RedCap UE bandwidth, it is not possible to share the same initial DL BWP between non-RedCap and RedCap UEs. In general, there are mainly two options possible in this case:

- A separate initial DL BWP is always configured for RedCap:

This is a straightforward solution and follows legacy behavior in the sense that an initial BWP is always configured, and hence, this option has a minimum specification impact.

- RedCap uses the MIB-configured CORESET #0 as its initial DL BWP:

In TDD, in some scenarios where non-RedCap initial DL BWP has a similar size as non-RedCap initial UL BWP, MIB-configured CORESET #0 may be configured near the carrier edge, as shown in the Figure 1. However, the non-RedCap initial DL BWP may not be often configured to be as wide as the non-RedCap initial UL BWP due to power saving considerations. Nevertheless, in scenarios as in Figure 1, one possibility could be to allow RedCap UEs to use, for example, the location and BW of CORESET #0 as its initial DL BWP without necessitating the configuration of a separate initial DL BWP. Although this solution may be beneficial for reducing some signaling overhead in SIB1 for some UL/DL BWP configurations, it requires defining a new behavior for RedCap and consequently more specification changes. Moreover, it is not desirable to pursue non-essential optimizations for Rel-17 RedCap during the maintenance stage.

Figure 1: An illustration of network configuration where CORESET #0 is configured near the carrier edge.

Based on the above considerations, we propose the following:

For a cell that allows access to RedCap UEs, a separate initial DL BWP is always configured for RedCap UEs if the initial DL BWP for non-RedCap UEs is wider than the maximum RedCap UE bandwidth.

| Reason for change: | To clarify that a separate initial DL BWP is always configured for RedCap UEs if the initial DL BWP for non-RedCap UEs is wider than the maximum RedCap UE bandwidth. |
| --- | --- |
| Summary of change: | Changes to RedCap UE procedures in Clause 17.1 of TS 38.213. |
| Consequences if not approved: | Unclear RedCap UE behaviour if the initial DL BWP for non-RedCap UEs is wider than the maximum RedCap UE bandwidth and a separate initial DL BWP for RedCap UEs is not configured. |

Therefore, we provide the following motivation and text proposal for TS 38.213 clause 17.1.

A UE expects the initial DL BWP and the active DL BWP after the UE (re)establishes dedicated RRC connection to be smaller than or equal to the maximum DL bandwidth that the UE supports. A UE can be provided a DL BWP by initialDownlinkBWP in DownlinkConfigCommonRedCapSIB, and an UL BWP by initialUplinkBWP in UplinkConfigCommonRedCapSIB. If initialUplinkBWP in UplinkConfigCommonSIB indicates an UL BWP that is larger than athe maximum UL BWP that a UE supports, the UE expects to be provided an UL BWP by initialUplinkBWP in UplinkConfigCommonRedCapSIB. If initialDownlinkBWP in DownlinkConfigCommonSIB indicates a DL BWP that is larger than the maximum DL BWP that a UE supports, the UE expects to be provided a DL BWP by initialDownlinkBWP in UplinkConfigCommonRedCapSIB.

The FFS in the RRC parameter list for RedCap will also need some updates (copied below [1]). The FFS may be updated as follows: [FFS: the case The parameter is always present when the SIB-configured initial DL BWP for non-RedCap UEs is wider than the RedCap UE maximum bandwidth.]

Update the FFS in the RRC parameter list for RedCap as follows: [FFS: the case The parameter is always present when the SIB-configured initial DL BWP for non-RedCap UEs is wider than the RedCap UE maximum bandwidth.]

# 3	Issue #2: SSB presence for BWP configuration option 1 in connected mode

There are two possible ways to configure a RedCap-specific separate initial BWP — either by BWP configuration option 1 or by BWP configuration option 2. In both options, common configuration (BWP-Down/UplinkCommon) is signaled in SIB1 (or by dedicated signaling using ServingCellConfigCommon), and hence, common/generic parameters for the initial BWP are common for all UEs and cannot be changed for individual RedCap UEs. However, option 1 and option 2 differs in the following aspects [2]:

- BWP configuration option 1: The network does not provide dedicated/UE-specific configurations (BWP-Down/UplinkDedicated) for the RedCap-specific initial BWP. Therefore, this initial BWP is not considered as an “RRC-configured” BWP. The network may configure a dedicated BWP (#1 - #4) which comprises both common and dedicated parameters, both of which can be changed by RRC reconfiguration.

- BWP configuration option 2: The network provides a dedicated configuration for the RedCap-specific initial BWP and is considered as an “RRC-configured” BWP. The dedicated parameters of this BWP can be changed individually per UE.

For BWP configuration options 1 and 2, based on the following agreements from RAN1#107-e [3], if the separate initial DL BWP is configured only for random access in idle/inactive mode, it does not need to contain an SSB:

Agreement: - For FR1, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - […] - […] Agreement: - For FR2, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - […] - […]

In fact, the capability to perform random access (in idle/inactive mode) in a separate initial DL BWP without SSB is one of the key components of the basic FG for RedCap UE, i.e., FG 28-1.

Agreement: - Following capabilities are added as components in FG 28-1 - a) Separate initial UL BWP for RedCap UEs - It includes the configuration(s) needed for RedCap UE to perform random access - b) Separate initial DL BWP for RedCap UEs - It includes CSS/CORESET for random access - FFS: For separate initial DL BWP used for paging, CD-SSB is included - For separate initial DL BWP only used for RACH, SSB may or may not be included

Furthermore, for BWP configuration option 2, it is clear from the agreements below from RAN1#107-e that the separate initial DL BWP should contain SSB (CD or NCD) while it is used in connected mode, at least for UEs supporting only baseline capabilities, i.e., FG 6-1.

Agreement: - For FR1, - […] - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - […] - […] Agreement: - For FR2, - […] - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - […] - […]

Now, for BWP configuration option 1, when the separate initial DL BWP is configured for paging in connected mode, it should contain a CD-SSB according to the agreement from RAN1#108-e.

Agreement: - […] - For BWP#0 configuration option 1, - For FR1, - For a separate initial DL BWP, for a RedCap UE in connected mode, paging can only be configured if it contains CD-SSB and the entire CORESET#0. - For FR2, - For a separate initial DL BWP, for a RedCap UE in connected mode, paging can only be configured if it contains CD-SSB and the entire CORESET#0. - […]

However, for BWP configuration option 1, where the separate initial DL BWP is not an “RRC-configured” BWP, there are no agreements related to the presence of SSB in connected mode when it is used only for random access. Recall that the separate initial DL BWP configured using Option 1 does not have dedicated configurations, and hence, it has a limited functionality which may not be suitable for connected mode Therefore, a UE typically switches to a non-initial RRC-configured DL BWP (e.g., BWP#1) after initial access. In principle, the separate initial DL BWP can act as a default DL BWP and be used for the purpose of power saving after the initial access. However, for RedCap UEs, the use of the separate initial DL BWP in connected mode for power saving purposes is quite limited. This is because the separate initial DL BWP is almost as large as the UE BW (e.g., 20 MHz in FR1), and therefore, the power saving gain by switching to this DL BWP is limited. For non-RedCap UEs with a large BW (e.g., 100 MHz), there is more motivation to switch to a significantly smaller initial DL BWP for power saving.

For BWP configuration option 1, the use of the separate initial DL BWP in connected mode is quite limited from both functionality and power saving perspectives.

Therefore, for option 1, the use of the separate initial BWP is not very likely in connected mode as the UE typically operates in a non-initial BWP. However, it is still possible that a UE in connected mode switches back to the separate initial DL/UL BWP for random access purposes. Note that random access procedure can be initiated by a PDCCH order from the gNB in connected mode, for example for uplink re-synchronization (i.e., when the status of UL synchronization status is non-synchronized). In this case, if RACH occasions are not configured for the active UL BWP, it is switched to the separate initial UL BWP, and potentially to its corresponding separate initial DL BWP in TDD.

In connected mode, it is possible that a UE switches back to the separate initial BWPs for random access purposes when RACH occasions are not configured for the active UL BWP of the serving cell.

It is also important to note that common/generic parameters for the separate initial DL BWP, including location and bandwidth, are cell-specifically configured, and cannot be changed for individual UEs while they are in connected mode. In addition, it could be interpreted from the RAN2 agreements copied below that NCD-SSB can only be configured in a non-initial DL BWP for option 1:

- A RedCap UE in idle/inactive mode monitors paging only in an initial BWP (default or RedCap specific) associated with CD-SSB and performs cell (re-)selection and measurements on the CD-SSB - In RRC connected mode NCD-SSB may be configured for a RedCap UE in dedicated DL BWP.

This implies that, for option 1, if the separate initial DL BWP does not contain an SSB during random access in idle/inactive mode (as already agreed in RAN1), it will not contain SSB if it is used during connected mode. At the same time, it is also not reasonable to expect the UE to operate in a DL BWP without presence of any SSB in connected mode for long periods of time. Therefore, we propose the following as a compromise (which is in-line option 2 in Proposal 3-1h in the FL summary from RAN1#108-e [4]):

For BWP configuration option 1, for a separate initial DL BWP during random access in connected mode, RedCap UE does not expect it to contain an SSB.

For BWP configuration option 1, upon successful completion of the random access procedure in connected mode, a RedCap UE supporting FG 6-1 but not FG 6-1a in connected mode is not required to operate on a separate initial DL BWP that does not contain SSB.

We do not expect specifications changes if the above proposals are agreed in RAN1. The reason is that Clause 17.1 (RedCap UE procedures) of TS 38.213 (V17.1.0) [5] does not differentiate between idle/inactive state and connected state with regards to whether SSB should be expected or not when performing random access in the separate initial DL BWP (see also Section 5 of this contribution).

RAN1 specification changes are not expected if proposals 2 and 3 described above are agreed.

# 4	Issue #3: Collision handling between SSB and Msg3 or PUCCH in response to Msg4/MsgB for HD-FDD

The only open issue left to be resolved on this topic is the handling of the following collision case:

- Collision handling between SSB and Msg3 or PUCCH in response to Msg4/MsgB for HD-FDD UE

With regards to the above issue, RAN1 made the following working assumption in RAN1#108-e [3]:

Working assumption: - For Case 5 of SSB overlapping with Msg3 (re)transmission or PUCCH for Msg4/MsgB, reuse the same handling as for other dynamically scheduled UL transmission and prioritize the SSB - Note: Whether the above collision rule is reused for Msg3 PUSCH repetition is up to the agreement in the CE WI

Our preference is to have a unified solution for all collision cases between SSB and UL transmission. For SSB vs. configured UL and SSB vs. dynamically scheduled UL cases, it was already agreed that SSB would be prioritized in case of collisions. Specifying the same rule also for the SSB vs. Msg3 or PUCCH in response to Msg4/MsgB case will lead to less specification impacts. That is, UE would receive SSB and does not transmit Msg3 or PUCCH in response to Msg4/MsgB. Also, in our view, the potential latency impact during random access procedure due to prioritizing SSB in case of no early HD-FDD capability indication may be limited, e.g., up to 1 slot, with typical SSB configuration in FDD. Therefore, we propose to confirm the WA from RAN1#108-e.

Prioritizing SSB for the collision case between SSB and Msg3 or PUCCH in response to Msg4/MsgB has the benefit of achieving a unified solution and less specification impacts. The potential latency impact may be limited.

Confirm the following working assumption from RAN1#108-e:

    - For Case 5 of SSB overlapping with Msg3 (re)transmission or PUCCH for Msg4/MsgB, reuse the same handling as for other dynamically scheduled UL transmission and prioritize the SSB

      - Note: Whether the above collision rule is reused for Msg3 PUSCH repetition is up to the agreement in the CE WI

In Clause 17.2 (Half-Duplex UE in paired spectrum) of TS 38.213 (V17.1.0), the specification text does not differentiate between dynamically scheduled PUSCH, configured grant PUSCH, and PUSCH scheduled by RAR UL grant, and has already specified that SSB should be prioritized when there is an overlap. Similarly, the specification text does not differentiate between PUCCH for Msg4/MsgB and other PUCCHs. Therefore, we do not expect specification changes if the WA from RAN1#108-e is agreed.

If a HD-UE would transmit a PRACH based on a detected DCI format, or PUSCH, or PUCCH, or SRS and the HD-UE is indicated presence of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon in a set of symbols, the HD-UE does not transmit PUSCH or PUCCH or PRACH if a transmission would overlap with any symbol from the set of symbols and the HD-UE does not transmit SRS in the set of symbols.

There is no need for RAN1 specification changes if the WA from RAN1#108-e on Case 5 of SSB overlapping with Msg3 (re)transmission or PUCCH for Msg4/MsgB is agreed.

# 5	Issue #4: Corrections for RedCap BWP operation description in clause 17.1 of TS 38.213

Some of the RedCap BWP operation description captured in clause 17.1 of TS 38.213 were not fully consistent with the agreements made during the Rel-17 RedCap WI.

| Reason for change: | The following RAN1 agreements have not been fully captured in 38.213: Agreement: - For FR1, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - […] - […] - Note: if a separate initial/RRC configured DL BWP is configured to contain the entire CORESET#0, CD-SSB is expected by RedCap UE. - Note: The network may choose to configure SSB or MIB-configured CORESET#0 or SIB1 to be within the respective DL BWP. - […] Agreement: - For FR2, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - […] - […] - Note: For SSB and CORESET#0 multiplexing pattern 1, if a separate initial/RRC configured DL BWP is configured to contain the entire CORESET#0, CD-SSB is expected by RedCap UE. - Note: The network may choose to configure SSB or MIB-configured CORESET#0 or SIB1 to be within the respective DL BWP. - […] Agreement: - […] - For BWP#0 configuration option 1, - For FR1, - For a separate initial DL BWP, for a RedCap UE in connected mode, paging can only be configured if it contains CD-SSB and the entire CORESET#0. - For FR2, - For a separate initial DL BWP, for a RedCap UE in connected mode, paging can only be configured if it contains CD-SSB and the entire CORESET#0. - […] Agreement: - A RedCap UE supports existing applicable mandatory feature(s) that are based on SSB using NCD-SSB (including NCD-SSB based measurements) as mandatory feature(s) in an RRC-configured DL BWP that does not include CD-SSB. - NCD-SSB is ‘QCL’-ed with CD-SSB when the NCD-SSB and CD-SSB share the same SSB index. - Note: RAN1 assumes that NCD-SSB is configured by higher layer |
| --- | --- |
| Summary of change: | Changes to RedCap UE procedures in Clause 17.1 of TS 38.213. |
| Consequences if not approved: | RedCap UE procedures that are not consistent with the agreements made in RAN1 during Rel-17. |

Therefore, we provide the following text proposal for TS 38.213 clause 17.1.

For an initial DL BWP provided by initialDownlinkBWP in DownlinkConfigCommonRedCapSIB, if a UE monitors PDCCH according to a Type1-PDCCH CSS set and does not monitor PDCCH according to Type2-PDCCH CSS set, the UE assumes that does not expect the initial DL BWP does not to include SS/PBCH blocks orand the CORESET with index 0. If the UE monitors PDCCH according to Type2-PDCCH CSS set, the UE assumes that the initial DL BWP includes a SS/PBCH block and, for SS/PBCH block and CORESET multiplexing pattern 1, the CORESET with index 0 if the UE used the SS/PBCH block to obtain SIB1 - includes a SS/PBCH block and does not include the CORESET with index 0 if the initial DL BWP does not include the SS/PBCH block the UE used to obtain SIB1 For an active DL BWP provided by BWP-DownlinkDedicated, a UE assumes that the active DL BWP includes a SS/PBCH block, unless the UE indicates a capability to operate in the DL BWP without receiving an SS/PBCH block, and does not expect the active DL BWP to include the CORESET with index 0. If the active DL BWP includes a different SS/PBCH block than the SS/PBCH block the UE used to obtain SIB1, the former and the latter SS/PBCH blocks have the same quasi-colocation properties when they have the same index.

Adopt the text proposal provided in this contribution for TS 38.213 clause 17.1.

# 6	Conclusion

In the previous sections we made the following observations:

Observation 1	For BWP configuration option 1, the use of the separate initial DL BWP in connected mode is quite limited from both functionality and power saving perspectives.

Observation 2	In connected mode, it is possible that a UE switches back to the separate initial BWPs for random access purposes when RACH occasions are not configured for the active UL BWP of the serving cell.

Observation 3	RAN1 specification changes are not expected if proposals 2 and 3 described above are agreed.

Observation 4	Prioritizing SSB for the collision case between SSB and Msg3 or PUCCH in response to Msg4/MsgB has the benefit of achieving a unified solution and less specification impacts. The potential latency impact may be limited.

Observation 5	There is no need for RAN1 specification changes if the WA from RAN1#108-e on Case 5 of SSB overlapping with Msg3 (re)transmission or PUCCH for Msg4/MsgB is agreed.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For a cell that allows access to RedCap UEs, a separate initial DL BWP is always configured for RedCap UEs if the initial DL BWP for non-RedCap UEs is wider than the maximum RedCap UE bandwidth.

Proposal 2	Update the FFS in the RRC parameter list for RedCap as follows: [FFS: the case The parameter is always present when the SIB-configured initial DL BWP for non-RedCap UEs is wider than the RedCap UE maximum bandwidth.]

Proposal 3	For BWP configuration option 1, for a separate initial DL BWP during random access in connected mode, RedCap UE does not expect it to contain an SSB.

Proposal 4	For BWP configuration option 1, upon successful completion of the random access procedure in connected mode, a RedCap UE supporting FG 6-1 but not FG 6-1a in connected mode is not required to operate on a separate initial DL BWP that does not contain SSB.

Proposal 5	Confirm the following working assumption from RAN1#108-e:

	For Case 5 of SSB overlapping with Msg3 (re)transmission or PUCCH for Msg4/MsgB, reuse the same handling as for other dynamically scheduled UL transmission and prioritize the SSB

	Note: Whether the above collision rule is reused for Msg3 PUSCH repetition is up to the agreement in the CE WI

Proposal 6	Adopt the text proposal provided in this contribution for TS 38.213 clause 17.1.

# References

R1-2202760, “LS on updated Rel-17 LTE and NR higher-layers parameter list”, RAN WG1, Ericsson, 3GPP TSG-RAN WG1 Meeting #108-e, February 2022.

TS 38.331 V17.0.0, “Radio Resource Control (RRC) protocol specification (Release 17)”, March 2022.

R1-2202535, “RAN1 agreements for Rel-17 NR RedCap”, Rapporteur (Ericsson), 3GPP TSG RAN1 #108-e, February 2022.

R1-2202532, “FL summary #5 on reduced maximum UE bandwidth for RedCap”, Moderator (Ericsson), 3GPP TSG RAN1 #108-e, March 2022.

TS 38.213 V17.1.0, “Physical layer procedures for control (Release 17)”, March 2022.