3GPP TSG RAN WG1 Meeting #109-e						R1-2203115

e-Meeting 9th – 20th May 2022

Agenda Item:		8.6.3

Source:			Ericsson

Title:				Draft summary of WI on support of reduced capability (RedCap) NR devices

WI code(s):			NR_redcap-Core

leading WG:		RAN1

Release:			Rel-17

### 1	Introduction

This Rel-17 work item introduces support for UE complexity reduction techniques and UE power saving techniques suitable for IoT use cases such as industrial wireless sensors, video surveillance, and wearables, with requirements on low UE complexity and/or low UE power consumption and with relatively relaxed data rate requirements. Following an initial study [1], this work item [2] specified support for a reduced capability (RedCap) UE type and two UE power saving techniques: Extended DRX in RRC idle/inactive state, and RRM measurement relaxation for neighbour cells.

This draft summary is submitted to RAN1 for information/discussion only. Comments are welcome and can be sent directly to the rapporteur (johan dot bergman at ericsson dot com). The final summary will be submitted to RAN#96.

### 2	Description

The following key functionalities are introduced as part of this work item:

Reduced capability (RedCap) UE type

The new reduced capability (RedCap) UE type enables reduced UE complexity through various UE complexity reduction techniques. A RedCap UE supports a maximum UE Rx/Tx bandwidth of 20 MHz in FR1 and 100 MHz in FR2 (whereas a normal NR UE supports at least 100 MHz in FR1 and 200 MHz in FR2). A RedCap UE cannot support larger Rx/Tx UE bandwidths and it cannot support features related to carrier aggregation (CA), dual connectivity (DC), more than 2 UE Rx/Tx antenna branches, or more than 2 DL/UL MIMO layers.

A RedCap UE can furthermore have a reduced antenna configuration and a reduced number of DL MIMO layers:

- For FR1, a RedCap UE supports 1 or 2 UE Rx branches and 1 or 2 DL MIMO layers. The supported number of DL MIMO layers is the same as the implemented number of Rx branches, and as a result, 2-Rx RedCap UEs have twice as high DL peak rate as 1-Rx RedCap UEs. The UE indicates to gNB how many branches/layers it supports. The gNB can allow or disallow access from 1-Rx and 2-Rx RedCap UEs separately per cell.

- For FR2, a RedCap UE can either support a legacy UE power class such as PC3 (e.g., with the reference UE Rx/Tx antenna configuration with 2 antenna panels with 4 elements each) or the new lower UE power class PC7 (with a reduced-complexity reference UE Rx/Tx antenna configuration with either 1 panel with 4 elements or 2 panels with 2 elements each). Furthermore, the UE indicates support for 1 or 2 DL MIMO layers (independent of the antenna configuration).

A RedCap UE in FDD mode can report per band whether it implements half-duplex FDD (HD-FDD) or full-duplex FDD (FD-FDD) support. In HD-FDD operation, the UE is not required to transmit and receive at the same time. The network indicates in SIB1 whether the cell supports HD-FDD RedCap UEs.

A RedCap UE can be implemented with or without support for DL 256QAM in FR1, making it optional in the same way as other 256QAM features are optional for ordinary NR UEs (UL 256QAM in FR1, DL/UL 256QAM in FR2). 256QAM support increases the peak data rate by ~33%.

Some higher layer features are optional for RedCap UEs: RedCap UEs can optionally support 16 DRBs (as normal NR UEs) but only have mandatory support of 8 DRBs. RedCap UEs can optionally support 18-bit PDCP/RLC sequence numbers (as normal NR UEs) but only have mandatory support for 12-bit sequence numbers. RedCap UEs have optional (but not mandatory) support for automatic neighbour relation (ANR) functionality.

Due to the reduced UE bandwidth, there are some modifications of the bandwidth part (BWP) operation. Separate initial DL/UL BWPs can be configured for random access for RedCap UEs, which may be required if one or both of the ordinary initial DL/UL BWPs in the cell are configured with a bandwidth which is too wide for RedCap UEs. A separate initial DL BWP can, but does not need to, contain SSB/CORESET#0/SIB. A DL BWP used in connected mode needs to contain (cell-defining or non-cell-defining) SSB but not necessarily CORESET#0/SIB.

The UE provides an early indication already during random access that it is a RedCap UE. If RedCap-specific PRACH resources are configured in the cell, the early indication is provided implicitly already by Msg1. In any case, an indication will be provided in Msg3 (or MsgA in case of 2-step RACH) in the form of a RedCap-specific LCID value for CCCH.

To minimize UL resource fragmentation for other UEs, the network can choose to disable frequency hopping for the PUCCH transmission carrying HARQ-ACK feedback for Msg4 (similar to how PUCCH frequency hopping can be disabled in connected mode).

Extended DRX in RRC idle/inactive state

Extended DRX cycles are introduced for RRC idle state (up to 10485.76 seconds, i.e., roughly 3 hours) and RRC inactive state (up to 10.24 seconds) as an optional feature for both RedCap and non-RedCap UEs. For use cases with relatively relaxed requirements on DL reachability/latency, the network may configure an extended DRX cycle, which may reduce the UE power consumption substantially during periods with large enough packet inter-arrival time.

RRM measurement relaxation for neighbour cells

RRM measurement relaxation for neighbour cells is introduced as an optional feature for RedCap UEs that can be enabled by the network. In RRC idle/inactive states, the UE is allowed to further relax neighbour-cell RRM measurements (compared to existing Rel-16 relaxation functionality) when an RSRP/RSRQ-based stationarity criterion is met for a period of time, or when both the stationarity criterion and a not-at-cell-edge criterion are met, to help reduce UE power consumption. In RRC connected state, the network may configure the RSRP/RSRQ-based stationarity criterion and in that case the UE shall report when the criterion is met or no longer met (and how to use the reporting information is up to the network implementation).

### 3	References

[1]	TR 38.875 V17.0.0, “Study on support of reduced capability NR devices”

[2]	RP-220966, “Revised WID on support of reduced capability NR devices”

[3]	RP-220964, “Status report for support of reduced capability NR devices”