3GPP TSG RAN WG1 #109-e	R1-2203123

e-Meeting, May 9th - 20th, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	On Physical Channel Design Framework for SL-U

Agenda item:	9.4.1.2

Document for:	Discussion and Decision

# Introduction

In RAN Meeting #94e, the WID on NR sidelink evolution was agreed including the following objective for operation of sidelink in unlicensed spectrum [1]:

- Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98.

In this contribution, we give an overview on physical channel structure considerations related to Listen-Before-Talk both in time and frequency, PSFCH and S-SS/PSBCH design .

# Discussion

## Physical channel structure

### Considerations of Physical Channel Structure in-Time

As described in TR37.985, in Rel.16 three slot format types that multiplex the different SL channels and signals in time within a slot have been defined for NR SL. As depicted in Figure 1, there are three formats which the aim is the following:

- PSCCH and PSSCH slot – The full slot is used for a UE’s SL PSCCH/PSSCH transmission;

- PSCCH, PSSCH and PSFCH slot – A fraction of the slot is used for a UE’s PSCCH/PSSCH transmission and another fraction used for (another) UE’s PSFCH transmission; and

- S-SS/PSBCH slot – Used by a UESyncRef to transmit a SL synchronization and PSBCH block;

As stated in the WID objectives, the sidelink physical channel structure from legacy NR SL should be used as much as possible in the NR SL design for operation in shared/unlicensed spectrum (here denoted as SL-U).

Figure 1: Example of sidelink slot formats: (a) Sidelink slot with PSCCH and PSSCH; (b) Sidelink slot with PSCCH, PSSCH and PSFCH; (c) Sidelink S-SS/PSBCH slot.

Proposal 1: RAN1 to consider in the SL-U design the same slot formats as defined for legacy NR SL.

In the targeted bands n46 and n96/n102 there is the requirement that a device successfully completes a Listen-Before-Talk (LBT) procedure (either Type 1 or 2, depending on if a valid COT is available) before it can access the channel and proceed with its transmission. With the exception of LBT Type 2C (where there is no actual LBT taking place), all remaining LBT procedures are concluded with a final Clear Channel Assessment (CCA) slot (i.e. a sensing slot) for which the duration is 9 µs. Therefore, in SL-U the challenge becomes where should this CCA slot be placed, since it needs to precede any SL transmission such as PSCCH/PSSCH, PSFCH or even S-SS/PSBCH.

Observation 1: The last CCA slot of an LBT procedure should precede any SL transmission type.

In Figure 2, we depict potential options on how to integrate the LBT procedure into the three different slot formats. According to the legacy slot formats shown in Figure-1, the last CCA slot of the LBT procedure can be conducted before the AGC symbol at the beginning of the SL slot, before the PSFCH transmission symbol of the SL slot, or before the first PSBCH symbol.

Figure 2: Example of sidelink slot formats and where to anchor the last CCA slot (i.e. LBT) in time: (a) Sidelink slot with PSCCH and PSSCH; (b) Sidelink slot with PSCCH, PSSCH and PSFCH; (c) Sidelink S-SS/PSBCH block.

Proposal 2: RAN1 to evaluate the placement of the last CCA slot of an LBT procedure in each SL slot format.

### Considerations of Physical Channel Structure in-Frequency

Considering of SL channel multiplexing in frequency, generally the legacy SL channels, i.e., S-SS/PSBCH, PSCCH, PSSCH, and PSFCH, are designed in a localized manner with consecutive resource allocations in frequency. For the case of PSCCH/PSSCH this occurs at the sub-channel level, the PSFCH at the PRB level, and S-SS/PSBCH at a predefined number of PRBs. However, the operation in unlicensed spectrum introduces several challenges, including: the frequency allocation of each of these channels; the relation between LBT and frequency multiplexing; among others. In the following we discuss each of these challenges.

Regarding frequency allocation of each channel in unlicensed bands, we note that this should take into account the regulatory Occupied Channel Bandwidth (OCB) requirements as stated in our companion contribution in [2]. Namely, in Europe, ETSI harmonized standards have explicit requirements on the occupied channel BW (OCB). It is required that the occupied channel BW shall be at least 80% of Nominal Channel BW for 5GHz bands, respectively. NR channels and signals that may be transmitted alone need to be able to fulfil the OCB requirement. This means that, for some of the NR channels / signals, new wide BW variants fulfilling the OCB requirement need to be designed.

Observation 2: Wide bandwidth versions need to be designed for some of the SL channels / signals to fulfil the 80% occupied channel BW (OCB) requirement. However, this requirement is not applicable in all regulatory regions and in those cases Rel-16 SL channels / signals may be used.

However, it should be also noted that, on the 5 GHz bands, the transmission BW may temporarily be less than 80% of the Nominal Channel BW during a channel occupancy time, with a minimum BW being 2 MHz. We see that the potential benefits from this allowance should be explored. For example, a wide BW variant of a SL signal may use resources less efficiently than the original SL signal. In such case, the wide BW variant could be used only when the OCB requirement needs to be met by the signal, and the more efficient Rel-16 SL signal would be used otherwise.

Another aspect to consider regarding the use of wide BW signals, is that there are PSD and/or EIRP density strict limitations in several regulations. For example, the regulatory limit on 5GHz band may be as low as 10 dBm/MHz, meaning that transmission power is severely limited for transmissions with narrow bandwidth. This presents a challenge for channels and signals with reduced spectrum allocation such as PSFCH, S-PSS, P-SSS, PSBCH and occasionally also PSSCH. These channels are critical for SL operation and therefore can limit the achievable communication range. Hence, it is evident that PSD limitation needs to be addressed during the work. The obvious challenge in supporting high transmission power under strict PSD limit for small payload signals is to find a design that uses resources efficiently. Hence, we see that efficient resource usage should be one of the key design criteria.

Observation 3: Wide bandwidth versions need to be considered for some of the SL channels / signals to support high transmission power under PSD limits.

In NR-U, the regulatory limitations in terms of OCB and PSD guided the design choices for the uplink channels of NR-unlicensed system and therefore an interlaced FDM scheme was adopted. In interlaced FDM, specified in TS38.214 as UL resource allocation type 2, the UL resources are allocated in interlaces of 10 equidistant PRBs. The number of interlaces is 10 for 15 kHz SCS and 5 for 30 kHz SCS.

Observation 4: In NR-U, the wide bandwidth of UL signals was achieved via an interlaced FDM scheme.

However, in SL-U not all channels/signals are necessarily suitable to be interlaced and might not even require it. For example, the PSCCH/PSSCH can be seen as suitable for interlaced FDM. On the other hand, applying interlace to the S-SS/PSBCH block creates some issues regarding the S-PSS and S-SSS for which the associated detection performance is degraded when an interlace is applied. Thus, it is proposed to discuss whether it is beneficial to utilize interlaced structure for operation of all SL channels or just some of the SL channels.

Proposal 3: RAN1 to evaluate the need and suitability of applying interlaced FDM to each SL channel.

In the legacy NR SL structure in frequency with single carrier operation, a single SL BWP is supported within a SL carrier. Furthermore, within the SL BWP multiple transmission and/or reception resource pools (RP) are supported. Each RP can then be further divided into sub-channels in frequency, where each sub-channel is composed by consecutively non-overlapping sets of (at least ten) PRBs.

For SL-U operation, and as discussed in our companion contribution [2], the LBT bandwidth is generally understood to be 20 MHz. If the legacy design regarding having a single SL BWP per SL carrier is adopted in SL-U, then it is important to establish the relation between LBT bandwidth and RP and sub-channels.

For example, the sub-channel configuration can be extended to support a single LBT bandwidth and then an RP is composed by at least one sub-channel and therefore can encompass multiple LBT bandwidths, as depicted in Figure 3(a). In this case, the LBT procedure would be applied at the sub-channel level.

Observation 5: In SL-U a resource pool configuration could be extended to support multiple LBT bandwidths, and a single sub-channel could support a single LBT bandwidth.

In another example, the RP configuration is extended to support a single LBT bandwidth and, if multiple sub-channels are configured, then each sub-channel has a bandwidth below 20 MHz, as depicted in Figure 3(b). In this case, the LBT procedure would be applied at the RP level.

Observation 6: In SL-U a resource pool configuration could be extended to support a single LBT bandwidth, and then the LBT bandwidth could be partitioned into multiple sub-channels.

In our view, both cases are valid options to be considered, where they have their own targeted scenarios that can be explored and discussed further. Thus, for SL-U, it is proposed to discuss on whether the LBT procedure could be operated at sub-channel level and/or resource pool level.

| (a) LBT at sub-channel level | (b) LBT at RP level |
| --- | --- |

Figure 3: Relation between LBT bandwidth and RP and sub-channel frequency allocation configuration.

Proposal 4: RAN1 to discuss the relation between LBT bandwidth and resource pool and sub-channel configuration.

Proposal 5: RAN1 to discuss on whether the LBT procedure can be applied at sub-channel level and/or resource pool level.

### Considerations on UE multiplexing and LBT

In the legacy SL, UEs can be multiplexed both in time (enabled by partitioning time into SL slots) and frequency (enabled by the partitioning of the SL resource pool resources into multiple sub-channels). However, the type of UE multiplexing allowed in SL-U will constraint the design of the LBT process integration into the different SL slot formats.

In the case that both, time and frequency multiplexing of multiple UEs are allowed within an LBT bandwidth, to ensure that all devices have a chance to successfully complete their LBT procedure then the last CCA slot of an LBT procedure should be placed at the same time across all sub-channels, as depicted in Figure 4. Furthermore, all UEs attempting to perform LBT in association with their transmission in a SL slot, should also initiate their transmissions at the same time in each of their associated sub-channels. In case the last CCA slots were placed at different times across the different sub-channels in a slot and therefore the UEs would initiate their transmissions also at a different time instant, then some UEs would be blocked from accessing their selected resources due to their LBT procedure failing due to activity from other UEs.

Observation 7: When time and frequency multiplexing is allowed in an LBT bandwidth for SL-U, the last CCA slot associated with an LBT procedure should be placed at the same time across all sub-channels.

Figure 4: Example of frequency multiplexing where all LBT procedures associated last CCA slot are placed at the same time across all sub-channels.

Proposal 6: RAN1 to evaluate if time and frequency multiplexing of multiple UEs within an LBT bandwidth is to be allowed in SL-U.

In the case that only time multiplexing of multiple UEs within an LBT bandwidth is allowed, then the placement of the last CCA slot associated with an LBT procedure becomes more flexible.

Observation 8: When time-only multiplexing is allowed in an LBT bandwidth for SL-U, the placement of the last CCA slot associated with an LBT procedure is more flexible.

For example, in NR-U the configuration of multiple placement positions for the last CCA slot was supported, which in conjunction with CP-extension, allowed a UE with higher-priority traffic to acquire the channel earlier and then via the CP-extension indicate the channel occupancy to other LBT UEs. This, in practice, allows for the establishment of multiple transmission starting points. The same principle could be applied to SL-U, to differentiate between different priority transmissions. As an example, depicted in Figure 5, the transmission starting point of SL transmission can be either before the AGC symbol with CP extension as NR-U, or the transmission starting point of SL transmission can be also flexibly placed within the AGC symbol.

Figure 5: Example of multiple transmission starting points (implemented by the placement of multiple CCA slots).

Observation 9: The LBT can be implemented in the SL slot as follows: (i) the transmission starting point of SL transmission can be either before the AGC symbol with CP extension as NR-U, or (ii) it can be also flexibly placed within the AGC symbol.

Proposal 7: For the case of time only multiplexing in an LBT bandwidth, RAN1 to evaluate the benefits of introducing multiple transmission starting points (each preceded by a CCA slot) in a SL slot.

In legacy NR SL, the last symbol of a slot is always a guard-symbol, where generally the SL UE performs the Tx/Rx switching, as well as handling of synchronization time misalignment. In the context of Channel Occupation Time (COT) sharing, where an initiating device can share its acquired COT with a responding device; it might be beneficial for either the initiating or responding device to be able to reduce the duration of the guard-symbol in order to enable the application of LBT Type 2 B/C. Depending on the SCS configuration in FR1, the time duration of the guard-symbol can be larger than 16µs, which are the maximum transmission gaps allowed for UE to perform the reduced LBT with Type-2C/2B in case the COT sharing is applied. Thus, for enabling the COT sharing with reduced LBT in SL-U, the CP extension transmission can be also considered as one candidate option to meet the transmission gap requirement for the desired LBT type before PSCCH/PSSCH transmission or/and before PSFCH transmission, as shown in Figure 6.

Figure 6: Example of CP extension when a valid COT is available.

Observation 10: For enabling the COT sharing with reduced LBT, the CP extension transmission can be also considered as one candidate option for reducing the transmission gap duration, either before PSCCH/PSSCH transmission or/and before PSFCH transmission.

## PSFCH design

In unlicensed spectrum, channel access relies on LBT (Listen-Before-Talk) to ensure fair coexistence of different wireless communication systems. LBT uncertainty in unlicensed spectrum may substantially reduce efficiency of sidelink communication and increase latency. Reliable transmission of PSFCH is critical because retransmission of PSSCH consumes much more resources than PSFCH. While a PSSCH occupies multiple PRBs (typically more than 10 PRBs) and most OFDM symbols of a slot, a PSFCH only occupies one PRB and 2 OFDM symbols of a slot. In unlicensed spectrum, PSFCH transmissions may not be initiated due to LBT failure. This will lead to retransmission of PSSCH consuming substantially more resources and increasing latency.

In the legacy NR SL, there is a one-to-one mapping between the PSCCH/PSSCH resource used and the corresponding PSFCH resource. Therefore, from the physical layer perspective, there is only one chance in time domain for transmission of the HARQ feedback. In a SL-U context, if the UE is not able to complete the LBT procedure successfully before the start of the PSFCH resource symbols, then it will not be able to access the channel and therefore will have to drop the HARQ feedback transmission.

Observation 11: In case the UE is not able to complete successfully the LBT procedure, then it will be unable to access the PSFCH resource symbols and will have to drop the transmission of the HARQ feedback.

A similar situation occurs in the legacy NR SL for in-device coexistence, where in some conditions (e.g. due to prioritization of transmission and reception) the UE has to drop its HARQ feedback transmission. However, there is currently no NR SL legacy mechanism that recovers from it. In SL-U, the cause for dropping HARQ feedback transmission is external to the UE, therefore it should be evaluated if such a recovery mechanism should be introduced for that purpose.

Proposal 8: RAN1 to study mechanisms that allow the recovery from the case where the UE drops its HARQ feedback transmission due to LBT related failure.

## SL-SS/PSBCH block design

To enable coexistence with other systems in unlicensed band, the S-SS/PSBCH design needs to consider the LBT channel access mechanism. More specifically, the transmission of a S-SS/PSBCH block needs to be preceded by an LBT slot. A UE acting as UESyncRef would then have to perform an LBT check to confirm if it can proceed with the transmission of the S-SS/PSBCH block.

However, whenever the LBT check fails, as depicted in Figure 7, the UE will be unable to perform the transmission of the S-SS/PSBCH block, for which the main consequence is the degradation of the synchronization performance.

Figure 7: S-SS/PSBCH block preceded by an LBT check, with example of S-SS/PSBCH block not being transmitted due to LBT failure.

A known design principle to increase robustness against LBT failures is to increase the number of transmission opportunities, such that if the UE suffers LBT failure in one opportunity, then the UE has the chance to attempt additional LBT checks in the remaining opportunities. An example of this design principle applied to the S-SS/PSBCH block design is depicted in Figure 8.

Figure 8: Introduction of multiple S-SS/PSBCH block transmission opportunities to mitigate LBT failures, where it is assumed that the SyncRefUE only transmits in the first opportunity where the LBT passes.

However, even though an S-SS/PSBCH block occupies only 11 PRBs of the SL BWP, all the remaining PRBs in the slot where the S-SS/PSBCH block takes place cannot be used by any other SL transmission, as depicted in Figure 9. The reason for this design is to ensure that, when UEs are performing their search for a synch source, they can detect the S-PSS and S-SSS signals more easily, since the S-SS/PSBCH block is the only signal present in the SL BWP.

Figure 9: S-SS/PSBCH block occupation of the SL-BWP.

The introduction of additional S-SS/PSBCH block opportunities to increase robustness towards LBT failures is done at the cost of the reduction of resources available for SL communication. There is, therefore, the need for alternative approaches that enable robustness towards LBT failures, without impacting the number of usable resources for SL communications.

Proposal 9: RAN1 to study mechanisms that make S-SS/PSBCH block transmissions robust against LBT failures, without impacting the number of usable resources for SL-U communications.

# Conclusions

In this contribution, we discussed physical channel design for SL-U and make the following observations and proposals:

Proposal 1: RAN1 to consider in the SL-U design the same slot formats as defined for legacy NR SL.

Observation 1: The last CCA slot of an LBT procedure should precede any SL transmission type.

Proposal 2: RAN1 to evaluate the placement of the last CCA slot of an LBT procedure in each SL slot format.

Observation 2: Wide bandwidth versions need to be designed for some of the SL channels / signals to fulfil the 80% occupied channel BW (OCB) requirement. However, this requirement is not applicable in all regulatory regions and in those cases Rel-16 SL channels / signals may be used.

Observation 3: Wide bandwidth versions need to be considered for some of the SL channels / signals to support high transmission power under PSD limits.

Observation 4: In NR-U, the wide bandwidth of UL signals was achieved via an interlaced FDM scheme.

Proposal 3: RAN1 to evaluate the need and suitability of applying interlaced FDM to each SL channel.

Observation 5: In SL-U a resource pool configuration could be extended to support multiple LBT bandwidths, and a single sub-channel could support a single LBT bandwidth.

Observation 6: In SL-U a resource pool configuration could be extended to support a single LBT bandwidth, and then the LBT bandwidth could be partitioned into multiple sub-channels.

Proposal 4: RAN1 to discuss the relation between LBT bandwidth and resource pool and sub-channel configuration.

Proposal 5: RAN1 to discuss on whether the LBT procedure can be applied at sub-channel level and/or resource pool level.

Observation 7: When time and frequency multiplexing is allowed in an LBT bandwidth for SL-U, the last CCA slot associated with an LBT procedure should be placed at the same time across all sub-channels.

Proposal 6: RAN1 to evaluate if time and frequency multiplexing of multiple UEs within an LBT bandwidth is to be allowed in SL-U.

Observation 8: When time-only multiplexing is allowed in an LBT bandwidth for SL-U, the placement of the last CCA slot associated with an LBT procedure is more flexible.

Observation 9: The LBT can be implemented in the SL slot as follows: (i) the transmission starting point of SL transmission can be either before the AGC symbol with CP extension as NR-U, or (ii) it can be also flexibly placed within the AGC symbol.

Proposal 7: For the case of time only multiplexing in an LBT bandwidth, RAN1 to evaluate the benefits of introducing multiple transmission starting points (each preceded by a CCA slot) in a SL slot.

Observation 10: For enabling the COT sharing with reduced LBT, the CP extension transmission can be also considered as one candidate option for reducing the transmission gap duration, either before PSCCH/PSSCH transmission or/and before PSFCH transmission.

Observation 11: In case the UE is not able to complete successfully the LBT procedure, then it will be unable to access the PSFCH resource symbols and will have to drop the transmission of the HARQ feedback.

Proposal 8: RAN1 to study mechanisms that allow the recovery from the case where the UE drops its HARQ feedback transmission due to LBT related failure.

Proposal 9: RAN1 to study mechanisms that make S-SS/PSBCH block transmissions robust against LBT failures, without impacting the number of usable resources for SL-U communications.

# References

- RP-220300, WID on NR sidelink evolution, RAN#95e

- R1-2203122, “On Channel Access Mechanism and Evaluation Methodology for SL-U”, RAN1#109-e.