3GPP TSG RAN WG1 #109-e	R1-2203124

e-Meeting, May 9th - 20th, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	On Co-channel Coexistence for LTE Sidelink and NR Sidelink

Agenda item:	9.4.2

Document for:	Discussion and Decision

# Introduction

In RAN Meeting #94e, the WID on NR sidelink evolution was agreed including the following objective for co-channel co-existence of NR sidelink and LTE sidelink operation in common spectrum [1]:

- Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] - Reuse the in-device coexistence framework defined in Rel-16 as much as possible

In this contribution, we discuss the scope of co-channel coexistence, the different modes of co-channel coexistence, their pros and cons, the aspects that need to be considered for dynamic channel coexistence and finally a NR-V2x device communication and synchronization capabilities.

# Discussion

## On the scope of Co-channel coexistence

One of the objectives is to enable LTE SL and NR SL coexistence in V2X deployments in ITS spectrum, with the following justification:

Another aspect to consider is the V2X deployment scenario where both LTE V2X and NR V2X devices are to coexist in the same frequency channel. For the two different types of devices to coexist while using a common carrier frequency, it is important that there is mechanism to efficiently utilize resource allocation by the two technologies without negatively impacting the operation of each technology. This requirement was also mentioned as part of the input from 5G Automotive Association to the Rel-18 RAN Workshop.

Before discussing co-channel coexistence mechanisms, it is important to clarify what is the motivation to introduce co-channel coexistence and what is the intended degree of coexistence between LTE SL and NR SL.

The main motivation to consider co-channel coexistence between LTE SL and NR SL, is associated to the expectation that most (if not all) of the ITS spectrum will be allocated by regulators to LTE SL, leaving limited (or no) dedicated spectrum for NR SL. The motivation to prioritize LTE SL in the ITS spectrum, is related to the need to enable the basic safety V2X use cases (such as the ones described in TS 22.885) in a relatively short term in as many vehicles as possible to minimize the occurrence of traffic related accidents and fatalities. As new vehicles (that support both LTE SL and NR SL, and farther in the future NR SL only) are introduced into the market then at some point in time there will be enough market penetration to enable the use of advanced V2X use cases (such as the ones described in TS 22.886). However, for these advanced V2X use cases to be feasible it is required that enough spectrum is made available for NR SL both in the ITS band and in other non-ITS bands. While the latter case is being tackled by the introduction in NR SL of features such as carrier aggregation, operation in unlicensed band and beam management at FR2, the former is to be enabled via LTE SL and NR SL co-channel coexistence.

The basic safety V2X use cases are enabled by the exchange of Cooperative Awareness Messages (CAM) or Basic Safety Messages (BSM) can be both sent using LTE SL and/or NR SL, then as we progress in time, it is expected that more and more vehicles will utilize NR SL and less LTE SL. Therefore, by enabling LTE SL and NR SL to coexist in a dynamic way in a common carrier, then this will enable a soft transition/re-farming of the LTE SL to NR SL.

The use of semi-static resource pool configurations is not feasible to enable co-channel coexistence since there are no reliable mechanisms to update the resource pool pre-configuration after it has been first established. The main difficulty arises from the need to ensure that all vehicles always have the same configuration, otherwise V2V connectivity in out of coverage areas will be impaired.

Observation 1: Semi-static resource pool re-configurations (TDM or FDM) may not always be reliable in LTE SL and NR SL due to the potential inability to update a V2X resource pool pre-configuration once it has been first established.

Observation 2: Dynamic co-channel coexistence enables the transition/re-farming of LTE SL spectrum resources towards NR SL.

Proposal 1: RAN1 to study mechanisms that enable dynamic co-channel coexistence.

## On modes of co-channel coexistence

In Figure 1, we consider the three main modes of co-channel coexistence of LTE-V2X and NR-V2X in a common carrier frequency and in the following we analyze their pros and cons.

Figure 1: Examples of LTE SL and NR SL co-channel coexistence in the same carrier: (a) TDM co-channel coexistence; (b) FDM co-channel coexistence; (c) Full overlap of LTE SL + NR SL in the same common carrier resources.

The use of TDM to enable co-channel coexistence between LTE SL and NR SL, as depicted in Figure 1.(a), has the advantage that it can be achieved via resource pool configuration (i.e. without specification impact). However, once a resource pool pre-configuration is established, then it is expected that this TDM configuration will remain static for several decades due to regulatory concerns associated with traffic safety. In other words, transition/re-farming of LTE SL spectrum resources to NR SL is very unlikely to occur if co-channel coexistence is implemented via TDM.

Observation 3: TDM based co-channel coexistence prevents transition/re-farming of LTE SL spectrum resources to NR SL due to the inability to update a V2X pre-configuration once it has been established.

The use of FDM to enable co-channel coexistence, as depicted in Figure 1.(b), has again the advantage that it can be achieved via resource pool configuration. In addition to the drawbacks associated with the inability to update the resource pre-configuration once it has been established (as discussed for the TDM case); it also has an additional drawback. In LTE SL frame and subframe structure design, the first symbol of each LTE SL subframe is the AGC symbol, which is used by LTE SL Rx UE to adjust the AGC gain of the incoming signal to minimize the quantization noise and prevent saturation at the ADC (Analog-to-Digital Converter). However, in the NR SL slot format there is the possibility to introduce PSFCH related symbols at the end of the NR SL slot. Therefore, as depicted in

Figure 2, the LTE’s AGC setting can be impacted by the power variations associated with the NR’s V2X transmission in the case of NR slots configured with PSFCH symbols.

Figure 2: Impact of NR transmissions inducing on the setting of the LTE Rx’s AGC, for the case when a NR slot with PSFCH overlaps an LTE SL subframe in time.

Additionally, when the NR SL applies a different SCS than the LTE SL, then additional problems can occur when the LTE RX is setting its AGC. For instance, take the example depicted in Figure 3, if NR SL supports 30 kHz SCS while LTE SL only supports 15 kHz SCS, there will be 2 NR SL slots corresponding to one LTE SL subframe. Even if the start of the NR SL slots and LTE SL subframe are aligned in time, it still causes serious AGC issues for a LTE SL Rx UE, when one of the following occurs:

- The NR transmissions in the first and second slots are from different NR UEs (as depicted in Figure 3.a);

- The NR transmission occurs only in the second slot (as depicted in Figure 3.b);

Taking a closer look at the case depicted in Figure 3.b, where a NR SL transmission happens in the second slot. In this case, the LTE Rx performs the AGC adjustment in the first symbol of an LTE SL subframe where a NR SL transmission is absent and the AGC’s gain is adjusted to a lower received signal power (LTE only). When the NR performs its transmission in the second slot then its signal will be added to the ongoing LTE SL transmission – which increases the total received power at the LTE Rx – while the AGC’s gain determined in the first slot is still the one being applied. The main consequence is that this can lead the ADC to become saturated and therefore impair the decoding of the LTE transmission at the LTE Rx.

Figure 3: Impact of NR transmissions inducing on the setting of the LTE Rx’s AGC: (a) NR transmissions from different NR Tx in both NR slots overlapping with the LTE sub-frame; (b) NR transmission from a NR Tx in the second slot overlapping with the LTE sub-frame. The issue persists no matter if PSFCH is transmitted in the second slot or not when NR uses a different SCS than LTE.

Observation 4: FDM based co-channel coexistence prevents transition/re-farming of LTE SL spectrum resources to NR SL due to the inability to update a V2X pre-configuration once it has been established and can induce AGC setting problems at the LTE Rx side.

When the LTE SL + NR SL are fully overlapped in the same common carrier resources, as depicted Figure 1.(c), then it becomes possible for the NR SL to utilize any resources not being used by LTE SL. This enables a future proof ITS deployment, as the resources allocated for LTE SL can be transitioned/re-farmed by NR-V2X as the use of LTE SL decreases. However, as in the FDM case, the NR transmissions can still induce AGC issues at the LTE RX. Therefore, there is the need to study dynamic co-channel coexistence mechanisms that can prevent the occurrence of these AGC setting issues at the LTE Rx.

Observation 5: Full overlap of the LTE SL and NR SL resources enables the transition/re-farming of LTE SL spectrum to NR SL, however there is the need to study dynamic co-channel coexistence mechanism that can prevent AGC setting issues at the LTE Rx.

We summarize the pros and cons of each co-channel coexistence approach in Table 1.

| Type of co-channel coexistence in a common carrier | Pros | Cons |
| --- | --- | --- |
| TDM | - Dedicated LTE and NR SL resources - No AGC issues induced at the LTE Rx - No specification impact | - Static resource allocation, without possibility for NR SL to use unused LTE SL resources - Additional latency due to TDM |
| FDM | - Dedicated LTE and NR SL resources - No latency impact - No specification impact | - Static resource allocation, without possibility for NR SL to use unused LTE SL resources - AGC issues induced by NR SL transmission at the LTE Rx |
| Full co-channel overlap | - Allow for dynamic resource allocation, with the possibility for NR SL to use unused LTE SL resources - No latency impact | - AGC issues induced by NR SL transmission at the LTE Rx - Dynamic co-channel co-existence has specification impact |

Table 1: Pros and cons comparison between modes of co-channel coexistence in a common carrier.

Proposal 2: RAN1 to study mechanisms that enable dynamic co-channel coexistence in full co-channel overlap.

## On dynamic co-channel coexistence

The highlighted AGC problems can in principle be addressed in part via the Rel-16 in-device coexistence framework principles, where a NR SL transmission could be dropped every time the conditions that cause the AGC issue at the LTE Rx UE are detected. Yet, further discussion is needed to determine if any NR SL transmission should be dropped when AGC issues at the LTE Rx are detected or if there are any conditions for this. For example, should the NR SL transmission be dropped by default or should the dropping decision be made based on the priority of the LTE SL transmission.

Observation 6: In a co-channel coexistence setting a NR SL transmission may degrade an LTE SL transmission/reception.

Proposal 3: RAN1 to study conditions where a NR SL transmission should be dropped (i.e. apply Rel-16 In-device coexistence framework) to protect an LTE SL transmission/reception.

However, from a NR SL performance and reliability perspective dropping NR SL transmissions as the sole co-channel coexistence mechanism is not desirable. Instead, a NR SL device should consider in its resource selection procedure on how to avoid these conflicts in the first place.

Proposal 4: RAN1 to study mechanisms (beyond applying the Rel-16 In-device coexistence framework). E.g.  allow a NR SL device to avoid resources that are expected to impact an LTE SL transmissions/reception.

## On NR-V2x device communication and synchronization capabilities

A NR-V2x device capabilities can impact on its ability to co-channel coexist with LTE SL devices. Different types of devices are expected which can include both dual RAT (LTE SL and NR SL), single RAT (LTE SL only or NR SL only), and partial dual RAT (e.g., Partial LTE SL support and full NR SL support).

For the case of partial dual RAT, it is important that RAN1 discusses the different set of capabilities allowed to ensure that any developed co-channel coexistence mechanism can address adequately all devices (or at least have contingences to address these devices). Therefore, we list below the following levels of capability for further discussion:

- Capability A: NR SL device capabilities to receive and transmit LTE synchronization signals

  - Capability A.1: NR SL device can receive LTE synchronization signal

  - Capability A.2: NR SL device can receive and transmit LTE synchronization signals

- Capability B: NR SL device capabilities to receive and transmit LTE data signals

  - Capability B.1: NR SL device can receive LTE PSCCH (SCI only) data signals

  - Capability B.2: NR SL device can receive LTE PSCCH and PSSCH data signals

  - Capability B.3: NR SL device can transmit LTE PSCCH and PSSCH data signals (full dual RAT)

Proposal 5: RAN1 to study the NR SL device capabilities assumptions when coexisting with LTE SL.

Proposal 6: RAN1 to clarify if NR SL only devices or devices which cannot exchange information between the LTE module and the NR module in a timely manner are allowed to use LTE SL spectrum.

All the co-channel coexistence modes presented in the Figure 1 require at least slot and subframe boundary alignment between LTE and NR SL transmissions.

Observation 7: Any co-channel coexistence mode requires slot and subframe boundary alignment between LTE and NR SL.

In the case of TDM-based resource pool configuration, as presented in Figure 1(a), also DFN/SFN alignment between LTE and NR SL is needed.

Observation 8: TDM based co-channel coexistence requires DFN/SFN alignment between LTE and NR SL.

It can be assumed that all the SL UEs operating in the same channel or carrier prioritize the same synchronization source. In the ITS spectrum GNSS is typically selected as the highest priority synchronization source. If the highest priority synchronization source cannot be detected, SLSS based synchronization needs to be used. Depending on UE capabilities, synchronization of NR SL UE may be based on SLSS/PSBCH received from LTE SL UEs operating in the same channel.

As an LTE SL UE cannot synchronize to a gNB, whereas a NR SL UE can synchronize to both an eNB and gNB, it is possible that only the NR SL UE can use the highest priority synchronization source. In this case, it would be desirable that the NR SL UE has the capability to send LTE SL synchronization signals and forward the synchronization from NR S-SSB to LTE SL UEs. RAN1 should check whether these synchronization options are supported and if synchronization procedures need to be updated.

Proposal 9: RAN1 should discuss whether NR SL synchronization based on LTE SLSS/PSBCH transmissions and/or LTE SL synchronization based on NR transmissions are supported and if synchronization procedures need to be updated.

# Conclusions

In this contribution, we have the following observations and contributions:

Observation 1: Semi-static resource pool re-configurations (TDM or FDM) may not always be reliable in LTE SL and NR SL due to the potential inability to update a V2X resource pool pre-configuration once it has been first established.

Observation 2: Dynamic co-channel coexistence enables the transition/re-farming of LTE SL spectrum resources towards NR SL.

Proposal 1: RAN1 to study mechanisms that enable dynamic co-channel coexistence.

Observation 3: TDM based co-channel coexistence prevents transition/re-farming of LTE SL spectrum resources to NR SL due to the inability to update a V2X pre-configuration once it has been established.

Observation 4: FDM based co-channel coexistence prevents transition/re-farming of LTE SL spectrum resources to NR SL due to the inability to update a V2X pre-configuration once it has been established and can induce AGC setting problems at the LTE Rx side.

Observation 5: Full overlap of the LTE SL and NR SL resources enables the transition/re-farming of LTE SL spectrum to NR SL, however there is the need to study dynamic co-channel coexistence mechanism that can prevent AGC setting issues at the LTE Rx.

Proposal 2: RAN1 to study mechanisms that enable dynamic co-channel coexistence in full co-channel overlap.

Observation 6: In a co-channel coexistence setting a NR SL transmission may degrade an LTE SL transmission/reception.

Proposal 3: RAN1 to study conditions where a NR SL transmission should be dropped (i.e. apply Rel-16 In-device coexistence framework) to protect an LTE SL transmission/reception.

Proposal 4: RAN1 to study mechanisms (beyond applying the Rel-16 In-device coexistence framework). E.g.  allow a NR SL device to avoid resources that are expected to impact an LTE SL transmissions/reception.

Proposal 5: RAN1 to study the NR SL device capabilities assumptions when coexisting with LTE SL.

Proposal 6: RAN1 to clarify if NR SL only devices or devices which cannot exchange information between the LTE module and the NR module in a timely manner are allowed to use LTE SL spectrum.

Observation 7: Any co-channel coexistence mode requires slot and subframe boundary alignment between LTE and NR SL.

Observation 8: TDM based co-channel coexistence requires DFN/SFN alignment between LTE and NR SL.

Proposal 9: RAN1 should discuss whether NR SL synchronization based on LTE SLSS/PSBCH transmissions and/or LTE SL synchronization based on NR transmissions are supported and if synchronization procedures need to be updated.

# References

- RP-220300, WID on NR sidelink evolution, 3GPP TSG RAN Meeting #95e.