3GPP TSG RAN WG1 #109-e	R1-2203127

e-Meeting, May 9th - 20th, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	SL positioning scenarios and requirements

Agenda item:	9.5.1.1

Document for:	Discussion and Decision

# Introduction

At RAN#94, a new study item “Study on expanded and improved NR positioning” (FS_NR_pos_enh2) was approved ‎[1]. One of the objectives is relevant for the present agenda item:

- Study solutions for sidelink positioning considering the following: [RAN1, RAN2] - Scenario/requirements - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104 - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104) - Spectrum: ITS, licensed - Identify specific target performance requirements to be considered for the evaluation based on existing 3GPP work and inputs from industry forums [RAN1] - Define evaluation methodology with which to evaluate SL positioning for the use cases and coverage scenarios, reusing existing methodologies from sidelink communication and from positioning as much as possible [RAN1]. - Study and evaluate performance and feasibility of potential solutions for SL positioning, considering relative positioning, ranging and absolute positioning: [RAN1, RAN2] - Evaluate bandwidth requirement needed to meet the identified accuracy requirements [RAN1] - Study of positioning methods (e.g. TDOA, RTT, AOA/D, etc) including combination of SL positioning measurements with other RAT dependent positioning measurements (e.g. Uu based measurements) [RAN1] - Study of sidelink reference signals for positioning purposes from physical layer perspective, including signal design, resource allocation, measurements, associated procedures, etc, reusing existing reference signals, procedures, etc from sidelink communication and from positioning as much as possible [RAN1] - Study of positioning architecture and signalling procedures (e.g. configuration, measurement reporting, etc) to enable sidelink positioning covering both UE based and network based positioning [RAN2, including coordination and alignment with RAN3 and SA2 as required] Note: When the bandwidth requirements have been determined and the study of sidelink communication in unlicensed spectrum has progressed, it can be reviewed whether unlicensed spectrum can be considered in further work. Checkpoint at RAN#97 to see if sufficient information is available for this review.

In this contribution, we provide our views on SL positioning scenarios and requirements.

# Discussion

## Use Cases and Requirements

The SID (FS_NR_pos_enh2) [1] identities V2X, public safety, commercial and IIOT as the key use cases for consideration in SL positioning. In Rel.17, 3GPP RAN conducted a study on "Scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases" in [2] focused on V2X and public safety use cases, where the V2X positioning requirements defined in [3] and [4] as well as the public safety requirements defined in [3] and [5] are taken into account. For commercial and IIOT use cases, the positioning requirements are defined in [3] and [6], respectively.

The positioning requirements of use cases are captured via KPIs such as horizontal and vertical accuracy (within certain confidence level) of the positioning estimate, positioning service availability, positioning service latency as well as TTFF, update rate, energy consumption, etc. [3]:

- horizontal accuracy

- vertical accuracy: altitude accuracy; determines e.g., floor for indoor use cases and to distinguish between superposed tracks for road and rail use cases (e.g. bridges).

- positioning service availability: percentage value of the amount of time the positioning service is delivering the required position-related data within the performance requirements, divided by the amount of time the system is expected to deliver the positioning service according to the specification in the targeted service area [3].

- positioning service latency: time elapsed between the event that triggers the determination of the position-related data and the availability of the position-related data at the system interface [3].

- time to first fix (TTFF): time elapsed between the event triggering for the first time the determination of the position-related data and the availability of the position-related data at the positioning system interface [3].

- UE heading accuracy

- UE velocity accuracy

- Energy consumption

In addition, the following service area and environment of use related requirements are defined for supporting positioning services:

- UE velocity

- Environment: Indoor (incl. offices, hospital, industrial buildings) or outdoor (incl. rural, urban, dense urban)

Furthermore, positioning service areas are defines as following:

- 5G positioning service area: a service area where positioning services would solely rely on infrastructures and positioning technologies that can be assumed to be present anywhere where 5G is present (e.g. a country-wide operator-supplied 5G network, GNSS, position/motion sensors) [3]. This includes both indoor and any outdoor environments.

- 5G enhanced positioning area: a subset of the 5G positioning service area that is assumed to be provided with additional infrastructure or deploy a particular set of positioning technologies to enhance positioning services [3]. The enhanced positioning service area represents for example a factory plant, a dense urban area, an area along a road or railway track, a tunnel and covers both indoor and outdoor environments.

It is noted that, the positioning requirements can be on the 3D/2D coordinates (absolute position) or on the distance and/or angle (relative position) to a reference point, e.g. another UE. Furthermore, positioning service levels are defined in [3] to capture different performance (e.g. accuracy, positioning service availability, positioning service latency) and service area and environment (e.g. rural, urban, indoor) requirements of positioning services. Table 1 shows the performance requirements for different positioning service levels. Moreover, 10(or less)-30s TTFF is expected to be supported for positioning services. Besides, power consumption aspects are to be considered to support, in particular, low power high accuracy positioning use cases involving for e.g. IIoT devices.

Table 1: Performance requirements for Horizontal and Vertical positioning service levels [3]

| Positioning service level | Absolute(A) or Relative(R) positioning | Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Coverage, environment of use and UE velocity |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Horizontal Accuracy | Vertical Accuracy (note 1) |  |  | 5G positioning service area | 5G enhanced positioning service area (note 2) |  |
|  |  |  |  |  |  |  | Outdoor and tunnels | Indoor |
| 1 | A | 10 m | 3 m | 95 % | 1 s | Indoor - up to 30 km/h Outdoor (rural and urban) up to 250 km/h | NA | Indoor - up to 30 km/h |
| 2 | A | 3 m | 3 m | 99 % | 1 s | Outdoor (rural and urban) up to 500 km/h for trains and up to 250 km/h for other vehicles | Outdoor (dense urban) up to 60 km/h Along roads up to 250 km/h and along railways up to 500 km/h | Indoor - up to 30 km/h |
| 3 | A | 1 m | 2 m | 99 % | 1 s | Outdoor (rural and urban) up to 500 km/h for trains and up to 250 km/h for other vehicles | Outdoor (dense urban) up to 60 km/h Along roads up to 250 km/h and along railways up to 500 km/h | Indoor - up to 30 km/h |
| 4 | A | 1 m | 2 m | 99,9 % | 15 ms | NA | NA | Indoor - up to 30 km/h |
| 5 | A | 0,3 m | 2 m | 99 % | 1 s | Outdoor (rural) up to 250 km/h | Outdoor (dense urban) up to 60 km/h Along roads and along railways up to 250 km/h | Indoor - up to 30 km/h |
| 6 | A | 0,3 m | 2 m | 99,9 % | 10 ms | NA | Outdoor (dense urban) up to 60 km/h | Indoor - up to 30 km/h |
| 7 | R | 0,2 m | 0,2 m | 99 % | 1 s | Indoor and outdoor (rural, urban, dense urban) up to 30 km/h Relative positioning is between two UEs within 10 m of each other or between one UE and 5G positioning nodes within 10 m of each other (note 3) |  |  |
| NOTE 1: The objective for the vertical positioning requirement is to determine the floor for indoor use cases and to distinguish between superposed tracks for road and rail use cases (e.g. bridges). NOTE 2: Indoor includes location inside buildings such as offices, hospital, industrial buildings. NOTE 3: 5G positioning nodes are infrastructure equipment deployed in the service area to enhance positioning capabilities (e.g. beacons deployed on the perimeter of a rendezvous area or on the side of a warehouse). |  |  |  |  |  |  |  |  |

Observation 1: Positioning requirements span a wide range e.g., 0.2-10m horizontal accuracy, 0.2-3m vertical accuracy, 95-99% positioning service availability, 0.01-1s positioning service latency, etc. depending on different positioning service levels of use cases.

### V2X

The positioning requirements in V2X depend on the service the UE operates (incl. criticality of the service). 5GAA, in its LS reply to 3GPP RAN in [7], listed different V2X use cases and grouped the use cases into 3 sets based on the positioning requirements (first group with tens of meters accuracy, the second with lane level accuracy, and the third with sub-meter level accuracy) namely:

- Group 1) Lax positioning requirement:

  - Traffic Jam Warning - Urban Scenario on Road Warning

  - Software Update - Conventional-Routine/Urgent, Autonomous-Routine, etc.

- Group 2) Lane level positioning requirement

  - Cross-Traffic Left-Turn Assist

  - Intersection Movement Assist, etc.

- Group 3) Below meter positioning requirement

  - High Definition Sensor Sharing

  - Vulnerable Road User - Collision risk warning, etc.

For each of the above set, the positioning requirements (both for absolute and relative positioning) are captured as [2]:

- Set 1: 10 – 50 m with 68 – 95 % confidence level. This includes Group 1 and Service level 1.

- Set 2: 1 – 3 m with 95 – 99 % confidence level. This includes Group 2, Service level 2, 3, 4.

- Set 3: 0.1 – 0.5 m with 95 – 99 % confidence level. This includes Group 3, Service level 5, 6, 7.

### Public safety

Mission Critical Organizations require mission critical services to have accurate positioning such that first responders can be located at all times during normal and critical operations, indoors as well as outdoors. The level of positioning accuracy (and other KPIs) required is much more stringent than that required by local and regional regulatory requirements for commercial 5G users. The positioning requirements for the "1st responders" use case includes 1 m horizontal accuracy, 2 m (absolute) or 0.3 m (relative) vertical accuracy, 95 – 98 % positioning service availability as captured in Table 2 [3].

Table 2: "1st responders" use case positioning requirements from [3]

| Use cases | Description | Main KPIs range and drivers |
| --- | --- | --- |
| 1st responders | Tracking and guidance of 1st responders, with requirements for high-accuracy in the horizontal domain and vertical domain, as well as accurate awareness of height variation to detect falls, combined to a high level of availability and reliability | Service area is both indoor and outdoor Accuracy: < 1m horizontal, < 2m vertical (indoor for floor detection) and < 0,3 m vertical (relative) to detect changes in height of the UE holder. Availability > 95 % (98% outdoor) Other KPI drivers include: MCX, confidence, event-triggered report |

### Commercial

Few examples on the commercial side include industrial control, industrial automation, UAV control, and AR services. In particular, UAVs have applicability in various commercial and government sectors and services such as UAV control will require more precise positioning information that includes altitude, speed, and direction, in addition to horizontal coordinates. As per [8], network-based 3D space positioning (e.g., with altitude 30~300m) of a UE onboard UAV is to be supported. The UAV scenarios include 8K video live broadcast, laser mapping/HD patrol, Remote UAV controller through HD video, etc., and the correspond to positioning service level 5 in [3].

### IIOT

The key scenario for IIOT use case is Factories of the Future (e.g. Industry 4.0), where the tracking of mobile devices as well as mobile assets is becoming increasingly important in improving processes and increasing flexibility in industrial environments and hence high accuracy positioning is becoming essential. For example, in motion control system that is responsible for controlling moving and/or rotating parts of machines in a well-defined manner, SL positioning can enable determination of current position and/or rotation of one or multiple components at the motion controller. Furthermore, the SL positioning can enable localization of mobile robots within the factory setup. The other potential scenarios in a factory setup include (low-high positioning requirements) [6]

- Mobile control panels with safety functions (non-danger zones): Service Level 2

- Process automation – plant asset management: Service Level 3

- Flexible, modular assembly area in smart factories (for tracking of tools at the work-place location) : Service Level 3

- Augmented reality in smart factories: Service Level 4

- Mobile control panels with safety functions in smart factories (within factory danger zones): : Service Level 4

- Flexible, modular assembly area in smart factories (for autonomous vehicles, only for monitoring purposes): : Service Level 5

- Inbound logistics for manufacturing (for driving trajectories (if supported by further sensors like camera, GNSS, IMU) of indoor autonomous driving systems)): : Service Level 6

- Inbound logistics for manufacturing (for storage of goods): : Service Level 7

Apart from high accuracy positioning, low power consumption is a key requirement for industrial applications. Here, the battery life-time or minimum operation time is expected to range from 6 months to 12 years with positioning service levels ranging from 1 to 5 [Table A.7.2-1: Low power high accuracy positioning use cases in [6]].

Observation 2: SL positioning can be a key enabler of V2X, public safety, commercial and IIoT use cases

Proposal 1: RAN1 to consider V2X, public safety, commercial and IIoT use cases for SL positioning

Proposal 2: RAN1 to define a set of fixed performance requirements for each use cases to study SL positioning solutions

## Coverage Scenarios

V2X use cases require positioning services in indoor, outdoor and tunnel areas. Similarly, public safety use cases too require positioning services in indoor and outdoor areas. Therefore, SL positioning requirements must be fulfilled when the UE is inside the network coverage as well as when it is outside the network coverage. In this regard, [2] concluded to consider the following network coverage options for V2X and public safety use cases.

Network coverage: - In-coverage, partial coverage, out of network coverage scenarios - When the UE operates a use case having the corresponding positioning requirements, the requirements should be fulfilled when the UE is inside the network coverage as well as when it is outside the network coverage.

Moreover, commercial use cases such as UAV control as well as IIoT use cases have applicability in both indoor and outdoor areas and hence their positioning services may need to be supported in both inside and outside the network coverage area.

Proposal 3: RAN1 to support all network coverage scenarios (i.e., in-coverage, partial-coverage, out-of-coverage)

Proposal 4: RAN1 to discuss whether to define same or different performance requirements (e.g., horizontal and vertical accuracy, positioning service availability and latency, etc.) for different network coverage scenarios.

## Spectrum

PRS in UL/DL are wide-band signals that span contiguously over potentially very large bandwidths. This is because the wider is the bandwidth, the better is the achievable positioning accuracy (as known from the Cramér-Rao bound for positioning processes). However, it is noted that the SL data transmissions have typically limited bandwidth of several subchannels. Since the accuracy requirements dictate the bandwidth requirements and hence the SL positioning spectrum requirements, at the first bandwidth requirement needs to be identified. Subsequently, the spectrum requirement may be discussed as per the SL positioning needs.

It is noted that 5GAA, in its LS reply to 3GPP RAN in [7], considered ITS-dedicated, the licensed (including mmWave band) and the unlicensed spectrum band for V2X positioning without any preference on a specific band. Moreover, TR 38.845 concluded to consider the following spectrum options for V2X and public safety use cases.

- Spectrum: - V2X: licensed, unlicensed spectrum for both Uu and PC5 air-interfaces and ITS for PC5 air-interface. NOTE: There is no mechanism corresponding to regulatory requirements to use unlicensed spectrum in Rel-17 NR sidelink. - Public safety: licensed spectrum for both Uu and PC5 air-interfaces.

On the consideration of unlicensed spectrum, as per the SID [1], RAN#97 is targeted as the checkpoint to see if sufficient information is available for the review. This is based on progress of the study of sidelink communication in unlicensed spectrum.

Proposal 5: RAN1 to study bandwidth requirements needed to meet the accuracy requirements to discuss the need for additional spectrum beyond ITS and licensed band in SL positioning.

Proposal 6: RAN1 to consider ITS, licensed and unlicensed spectrum depending on the bandwidth requirements in SL positioning.

# Conclusions

In this contribution we discussed SL positioning scenarios and requirements and made the following observations and proposals:

Observation 1: Positioning requirements span a wide range e.g., 0.2-10m horizontal accuracy, 0.2-3m vertical accuracy, 95-99% positioning service availability, 0.01-1s positioning service latency, etc. depending on different positioning service levels of use cases.

Observation 2: SL positioning can be a key enabler of V2X, public safety, commercial and IIoT use cases

Proposal 1: RAN1 to consider V2X, public safety, commercial and IIoT use cases for SL positioning

Proposal 2: RAN1 to define a set of fixed performance requirements for each use cases to study SL positioning solutions Proposal 3: RAN1 to support all network coverage scenarios (i.e., in-coverage, partial-coverage, out-of-coverage)

Proposal 4: RAN1 to discuss whether to define same or different performance requirements (e.g., horizontal and vertical accuracy, positioning service availability and latency, etc.) for different network coverage scenarios.

Proposal 5: RAN1 to study bandwidth requirements needed to meet the accuracy requirements to discuss the need for additional spectrum beyond ITS and licensed band in SL positioning.

Proposal 6: RAN1 to consider ITS, licensed and unlicensed spectrum depending on the bandwidth requirements in SL positioning.

# References

- RP-213588, “Study on expanded and improved NR positioning”

- TR 38.845, “Scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases”

- TS 22.261, “Service requirements for the 5G system”

- TS 22.186, “Service requirements for enhanced V2X scenarios”

- TS 22.280, “Mission Critical Services Common Requirements (MCCoRe); Stage 1”

- TS 22.104, “Service requirements for cyber-physical control applications in vertical domains”

- RP-210040, “LS reply to 3GPP RAN on requirements of in-coverage, partial coverage, and out-of-coverage positioning use cases”

- TS 22.125, “Unmanned Aerial System (UAS) support in 3GPP”